Source:	Bytedance

Title:	[DaCAS] Scene-based audio capturing example solution

Document for:	Discussion and Agreement

Agenda Item:	7.6

# 1 Introduction

In TR 26.933 - Study on Diverse Audio Capturing Systems, the necessity of an immersive audio capturing system in UEs has been mentioned. This is a challenging task because end-user devices carry different microphone geometries. This proposal includes an example solution for scene-based audio capturing with arbitrary microphone arrays and performs tests to validate the performance of our solution.

# 2 Example of SBA capturing

## 2.1 UE type and definition of device centre

Among UEs mentioned in Clause 4, 3GPP TR 26.933 [1] who carries SBA capturing abilities, we category UEs into 4 subsets:

(1) Headset UEs including headphones and XR devices.

(2) Handset UEs including mobile phones and tablets; and

(3) handhold UEs including watches; and

(4) table-mounted UEs including laptops.

UEs such as cars where the audio scenes potentially occur inside the microphone arrays, are not discussed in this proposal.

For headset UEs, such as headphones and XR headsets, we define the device centre as the middle point between the two ears when the user is wearing the device. For UEs in other categories, we define the device centre as the geometric centre of the microphone array that the UE carries.

## 2.2 SBA recording:

Consider a UE carrying Q microphones, each positioned at rqθqφq with respect to the device centre of the UE. Assume the recording scene can be reconstructed with maximum L plane waves with DOA θlφl, and each of them has source signal slk, l is wave number, k is frequency bin number. The recorded microphone signals can be expressed as

xk=Vksk,

where Vk is the matrix contains the frequency responses of each virtual source to the microphones on the UE, which is also known as the steering matrix of the given microphone array on the UE. Vk can be obtained from the directional response measurement in clause 4.1.2 of 3GPP TS 26.260 [2] or based on finite element method simulations.

For headset devices only, it is also acceptable to numerically simulate Vk with human head modelled as a rigid sphere when measurements are not available. The frequency response of the q-th microphone on a headset UE due to a plane-wave from θlφl with the array worn on a r_h = 0.08~m rigid sphere is given by:

Vqk,θl,ϕl=n=019m=-nn4π-inYnmθl,ϕlbnk,rq,rhYnmθq,ϕq,

where:

bnkrqrh=jnkrq-j'nkrhh'nkrhhnkrq,

Ynmθ,ϕ=2n+14πn-m!n+m!Pnmcosθ2cosmϕ for m≥0Ynmθ,ϕ=2n+14πn-m!n+m!Pnmcosθ2sinmϕ for m<0

jn: spherical Bessel function,  hn: spherical Hankel function,  Pnm: associate Legendre function.

Define the desired recording HOA signals as anmk, it can be calculated with a linear encoding matrix cnmkas:

anmk=cnmkxk,

with

cnmk=VkVkH+λI-1Vkynm†,

ynm=Ynmθ1,ϕ1,Ynmθ2,ϕ2,⋯,YnmθL,ϕL,

where:

λ=1/SNR,  ⋅H: conjugate transpose,  ⋅†: Moore-Penrose pseudoinverse.

The recorded scene-based audio is then given by the Inverse Fourier Transform (IFT) where:

anmt=IFT{anmk}

# 3 Test of the example method

Several test methods have been included in TS 26.260 [2] for the sending-side terminal audio quality performance. We conduct an example SBA capturing test with these test methods addressed in clause 5.6 of TS 26.260. In this section, we list the results of an example scenario with one source and one DUT.

## 3.1 Test Scenario

We simulate the acoustic environment to be an anechoic chamber that meets clause 4.0.3 of 3GPP TS 26.260 [2] and assume the test equipment meets clause 4.0.2 of 3GPP TS 26.260. The loudness of test signals is calibrated using the method described in clause 5.5.1 in 3GPP TS 26.260. One sound source with an azimuth angle of 45 degrees, elevation 0 degrees, and radius 1.5 m is set with respect to the centre of the DUT. The DUT is a pair of wearable glasses with 4 mics evenly distributed on a circle at the ear level.

We tested two DUTs: (a) a pair of wearable glasses with 4 mics evenly distributed on a circle at the ear level, and (b) a pair of wearable glasses with 4 mics on the left and right-side glass frames, similar to the Ray-Ban Meta AR glasses. The microphone shape of the two DUTs is shown as follows:

| Microphone array Geometry of DUT-a. Unit is millimetres | Microphone array Geometry of DUT-b. Unit is millimetres |
| --- | --- |

## 3.2 Frequency Response Result

| FR of DUT-a | FR of DUT-b |
| --- | --- |

## 3.3 Directional Information Result:

| True Source direction | (-90 0) | (-60 0) | (-30 0) | (0 0) | (30 0) | (60 0) | (90 0) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Estimated Source direction: DUT-a | (-95.06 -4.39) | (- 71.67 -11.37) | (-43.69 -16) | (-6.55 -19.91) | (25.29 -31) | (63.79 -41.97) | (96.23 -36.28) |
| Estimated Source direction: DUT-b | (-67.44 -25.72) | (-52.0 -32.14) | (- 43.17 -31.69) | (-4.27 -43.45) | (33.56 -35.20) | (43.31 -36.60) | (58.68 -33.13) |

# 4 Proposal

- For headset UEs, such as headphones and XR headsets, we define the device centre as the middle point between the two ears when the user is wearing the device (head center).

- We suggest the WG to accept our capturing method as a candidate for DaCAS SBA recording.

# References

[1] TS 26.933 Study on Diverse Audio Capturing systems

[2] TS 26.260, Objective test methodologies for the evaluation of immersive audio systems.