Source:	Xiaomi EV Technology

Title:	Templates for potential target devices and database

## Document for:	Discussion & Agreement

## Agenda Item:	7.6

# Introduction

Based on the objectives of the DaCAS WID[1], to define a set of target devices or device types in a very first step to progress the project, the source proposes templates for collecting potential target devices and corresponding database.

The source also proposes a potential target device and database samples for discussion. The proposed target device is a smartphone with four microphone inputs, and some microphone raw database samples were recorded from the proposed target device.

# Content

## Template for potential target devices

### Template table

There are 7 types of UE described in TR 26.933[2], based on that, here, it is to make a template to describe a certain type of UE which will be used as a potential target device for the DaCAS project. The proposed parameters of the potential target devices are listed in the following table:

| No | 1 | 2 | ...... |
| --- | --- | --- | --- |
| Type |  |  |  |
| Size |  |  |  |
| Shape |  |  |  |
| Microphone type |  |  |  |
| Number of microphones |  |  |  |
| Placement of microphones |  |  |  |

No: It is the sequence of the proposed potential target device.

Type: It is to define the type of a UE, that is modern mobile Phones, foldable mobile phone and headphone etc.

Size: It is the overall size of a proposed UE. Refer to Annex A.1 defined in TR 26.933. For example: for a modern mobile Phone, the Length(mm)*Width(mm)* Depth(mm) is used to define its size.

Shape: It is to describe the overall appearance of a proposed UE, it is recommended to draw a diagram to make the device more intuitive.

Microphone type: It is to specify the type of the microphone used in a proposed UE. Refer to clause 5 in TR 26.933.

Number of microphones: It is to specify the number of microphones used in a proposed UE, the microphone can be used to capture satisfied raw audio signals.

Placement of microphones: It is to specify the coordinate data of all the microphones in a proposed UE. For example, using cartesian coordinates of three-dimensional, there are:  mic1: (X1,Y1,Z1), mic2: (X2,Y2,Z3).

### Placement of microphones description on modern mobile phones

It takes a modern mobile phone as an example to describe the placement of microphones. The centre of the port for microphones is used to describe the placement of microphones.

The position of each microphone is identified from a set of cartesian coordinates for each microphone.

The origin of the coordinate system is the center of the screen.

The X-axis is perpendicular to the screen, with its positive direction toward the screen is facing.

The Y-axis is parallel to the length edge. Its positive direction is toward USB port.

The Z-axis is parallel to the width edge, the positive direction decided by right-hand coordinate system.

It refers to the follow diagram for the modern mobile phone example:

## Template for database of potential target devices

### Template for setup of recording scenarios.

The source proposes the template of environment of recording audio data with potential target devices, it is as follows:

| No | 1 | 2 | ... |
| --- | --- | --- | --- |
| Sound source |  |  |  |
| Language |  |  |  |
| Playback level |  |  |  |
| UE position |  |  |  |
| Acoustic environment |  |  |  |
| Detailed spacing |  |  |  |
| Descriptions |  |  |  |

No: It is the sequence of recording scenario used for capturing the dataset with a proposed UE.

Sound source: It is to specify the type of the sound source, whether it is playback sound by loudspeakers or  real-time sound.

Language: It is to specify language type.

Playback level： It is to specify the playback intensity if the sound source is played by loudspeakers.

UE position: It is to specify the position of the UE in the recording scenario.

Acoustic environment: It is to specify the type of acoustic condition of the of the recording scenario.

Detailed spacing: It is to specify the detailed setup of all the devices in the recording scenario.

Descriptions: It is to describe the recording process shortly.

### Template for database.

Requirements template for collecting database.

| No | 1 | 2 | ...... |
| --- | --- | --- | --- |
| Category |  |  |  |
| Language |  |  |  |
| Position |  |  |  |
| Sound source angle |  |  |  |
| Simple rate |  |  |  |
| Duration |  |  |  |
| Bit depth |  |  |  |
| Audio format |  |  |  |
| Number of channels |  |  |  |
| License |  |  |  |

No: It is the sequence of recorded database.

Category: This refers to the type or classification of the audio files in the database. For example, categories can include speech, music, environmental sounds, or specific types of sound effects

Language: It is to specify the type of language or music.

Position: The sound source is in horizontal or vertical or other plane

Sound source angle: The exact angle of the sound source in the plane

Simple rate: This is the number of samples of audio carried per second, measured in Hertz (Hz)

Duration: This is the length of the audio file, typically measured in seconds or minutes. It indicates how long the audio content is.

Bit depth: This refers to the number of bits used to represent each sample of audio. It determines the resolution and the dynamic range of the audio.

Audio format: This is the file format in which the audio is stored, it is WAV or PCM

Number of channels: This indicates the number of discrete audio channels in the audio file

License: It is to specify how to use the database and for which purpose to use it

## Proposal of a potential target device and database

### Introduction

Based on the upper templates, the source proposes an example modern mobile phone as a potential target device and corresponding database examples got from the potential target device.

### A proposed potential target device

The modern mobile phone features four microphones and has dimensions of 161.4mm in length, 75.3mm in width, and 9.2mm in depth.

| No | 1 |
| --- | --- |
| Type | Modern mobile phone |
| Size (mm) | 161.4*75.3*9.2 |
| Shape | Rectangular |
| Microphone type | MEMS |
| Number of microphones | 4 |
| Placement of microphones(mm) | Mic1: (-5.2, 80.7, -10.1) Mic2: (-4.9,80.7,9.2) Mic3: (-1.6, -80.2, 7.4) Mic4: (-15.7, -19.2, 0) |

## proposed database samples

### Introduction

The source uses the proposed modern mobile phone to record raw microphone signals, the details are in the following using the upper template.

### Recording setup

The recording setup is as follows:

| No | 1 |
| --- | --- |
| Sound source | Loudspeaker: Genelec 8030A DA: RME Fireface UCX set at 48kHz/24bit |
| Language | Mandarin chinese |
| Playback level | Use -18dBFs pink noise calibrate to 80dBA with 10 seconds windows |
| UE position | All record is recorded on horizontal positions. |
| Acoustic environment | Only speech sound in the room |
| Detailed spacing | TBC |
| Descriptions | The recordings featured one stable sound source repeated at all five positions. |

### Database

The details of the database is as follows:

| No | 1 |  |  |
| --- | --- | --- | --- |
| Category | Speech |  |  |
| Language | Mandarin chinese (female) |  | ........ |
| Position | Horizontal (USB port direct to right) |  |  |
| Sound source angle | -60°, -30°, 0°, 30°, 60° |  |  |
| Simple rate | 48000 |  |  |
| Duration | 15.9sec every file |  |  |
| Bit depth | 16bit |  |  |
| Audio format | WAV |  |  |
| Number of channels | 4 |  |  |
| License | It is only used for DaCAS standardization work |  |  |

# Conclusion

## Templates for potential target devices and database are proposed, they are for discussion and agreement

## A potential target device and corresponding database are proposed for discussion. Additional data may be included based on further discussion.

# References

[1] SP-241962: WID on Diverse audio Capturing System for UEs

[2] TS 26.933: Technical Report on Diverse Audio Capturing system

[3] S4aR220043 Applications of Microphone Description