Source:	Dolby Laboratories Inc

Title:	Multi-microphone capture to Ambisonics format conversion

Agenda item:	7.6

Document for:	Discussion

- Introduction

IVAS supports variety of input formats as mentioned in [1] and [4]. However, in order to do immersive coding with IVAS on a device with multi-microphone capture, for e.g., mobile phones, a suitable conversion process is required to convert the captured format into an IVAS immersive format. The capture processing and conversion to IVAS immersive formats has been studied in detail in [1]. An example signal chain to process and convert a mobile phone captured audio to an IVAS immersive format is mentioned in Figure 1.1 wherein the compensation, enhancements, audio format generation and post proc blocks are studied in detail in [1]. Furthermore, an example signal chain (Figure 1.2) to convert a three microphone captured signal to ambisonics signal, including the description of various components of the capture processing signal chain, is shown in [2].

Table 1.1: IVAS Encoding Modes

| Coding modes | Full Name |
| --- | --- |
| Mono | EVS/IVAS Mono |
| Stereo | Stereo Operation |
| SBA | Scene-based Audio (SBA, Ambisonics) Operation (1-3) |
| MASA | Metadata-assisted Spatial Audio (MASA) Operation (1 or 2 channels) |
| ISM | Objects (Independent Streams with Metadata, ISM) Operation |
| MC | Multi-Channel (MC) Operation (5_1, 7_1, 5_1_2, 5_1_4, 7_1_4) |
| OMASA | Combined Objects and MASA (OMASA) Operation |
| OSBA | Combined Objects and SBA (OSBA) Operation |

Figure 1.1: example process of generating audio signals.

Figure 1.2: example process for three mic to ambisonics conversion.

In this contribution, the source provides an overview of an ‘Ambisonics Upmixer’ solution to convert a mobile phone captured audio signal to a First order Ambisonics (FOA) signal based on the Ambisonics Upmix method highlighted below. The source ran the solution on a mobile phone with three microphones (phone dimensions and microphone placements are shown in Figure 2.2 and Table 2.1) and informal evaluation results are provided in subclause 2.2.

- Discussion

  - Ambisonics Upmix

The source proposes an Ambisonics Upmix method wherein the response per frequency band is generated across each microphone towards each ambisonics channel using: i) a model of how each microphone responds to the incoming sound field from a dense set of directions, based on Finite element method (FEM) simulations which account for the 3D placement of microphones across device body structure and resultant acoustic energy transfer; ii) a perceptually-motivated optimisation where the complex upmix matrix coefficients are determined based on the data computed at the simulation step, and a target perceptual spatial accuracy. These steps of analysis, simulation and upmix design are performed offline once per device by assuming a simpler 3D geometry of the device and microphones as point sinks.

The Upmixer utilizes the static matrix and applies per frequency band transformation to achieve ambisonics channel output.

To generate the upmix matrix, the phone dimensions and microphone coordinates are fed as an input to the FEM method. The FEM method uses a filterbank with Equivalent Rectangular Banding (ERB) and generates a upmix matrix U4xn(nb) in each band, where nb is the band index and n is the number of microphones. The banded upmix matrix is then converted into complex gains per bin, Um4xn(b) , as per the filterbank response, where total number of bins are 1025 and b is the bin index. This is an offline process that needs to be done once per device model.

In the steady state, the time domain input is windowed and converted into frequency domain by performing Fast Fourier Transform (FFT) outputting 1025 bins (FFT size 2048). The frequency domain signal is then upmixed to FOA using the Um4xn(b)matrix as shown below.

FOA4x1b=Um4xnb Innx1b

The FOA signal is then converted back to time domain using inverse FFT.

Figure 2.1: Ambisonics Upmix

8.26 mm

z

M3

x(up)

M2

74 mm

M1

y

163.6 mm

Figure 2.2: Example three microphone placement on a mobile phone

Table 2.1: Microphone co-ordinates

|  | X (mm) | Y (mm) | Z (mm) |
| --- | --- | --- | --- |
| M1 | 4.1 | 0 | 25 |
| M2 | 9.7 | 139.9 | 47.5 |
| M3 | 3.6 | 163.6 | 62.2 |

  - Evaluation

    - Subjective listening

A mobile phone with three microphones, design shown in Figure 2.2, was fixed to a tripod in landscape orientation. This tripod was then placed at the listening position of a 7.0.4 loudspeaker setup (in a quiet room with low reverberance). Multiple recordings were made by playing audio from each loudspeaker. These recordings were then converted to planar FOA using the upmix matrix generated with the FEM method. The upmixed FOA signal was then encoded and decoded with IVAS at 256 kbps and then rendered to binaural using IVAS renderer. Multiple expert listeners listened to the binaural output, focusing on the spatial aspect of the audio. The listeners concluded that the spatial location of the audio objects in the binaural audio was in sync with the physical locations of the 7.0.4 loudspeakers, except the height speakers which were mapped to the X-Y plane.

    - Plots of the upmixed FOA signal

Figure 2.3 shows planar FOA output plot, wherein the FOA signal is generated using the same setup as described above. In this segment, the input speech is coming from the side left speaker i.e., 90 degrees (Left). Figure 2.4 shows planar FOA output when input speech is coming from the side right speaker i.e., 90 degrees (Right).

Figure 2.3:  FOA output when input signal is recorded from side left speaker i.e., 90 degree (Left)

Figure 2.4:  FOA output when input signal is recorded from side right speaker i.e., 90 degrees (Right)

- Next steps

# The source will look into performing the evaluation of the proposed solution using a suitable test methodology mentioned in [3].

- Proposal

# A generic method to convert a multi-microphone capture signal with three or more microphones to a First Order Ambisonics signal is presented with informal subjective evaluation results. The initial results look promising and the source plans to investigate into evaluating the presented solution with a suitable test methodology mentioned in [3]. The source proposes to document the presented solution, along with the evaluation results based on [3], among the DaCAS [5] example solutions in the DaCAS specification.

# References

- 3GPP TR 26.933, “Study on Diverse Audio Capturing system”

- Tdoc 240231, “Example design of spatial audio capture for multi-microphone UE devices”

- 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems”

- 3GPP TS 26.253, “Codec for Immersive Voice and Audio Services (IVAS)”

- Tdoc 242262, “New WID on Diverse audio CApturing System for UEs (DaCAS)”