3GPP TSG-WG SA4 Meeting #131	S4-250145

# Geneva, CH, 17th – 21st Feb, 2025	                          	                   (revision of S4-25XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0008 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NOTE for MBS User Service over eMBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2025-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For MBS User Service and Delivery protocols over eMBMS, there is a NOTE in clause 5.10.6 as following. NOTE: whether the TMGI allocation in the MBS US Announcement is achievable via MB-SMF is FFS. The main issue here is that the NEF selects the MB-SMF based on the MBS service area when the MBS Application Provider requests the TMGI allocation. If there is no single available MB-SMF serving the target service area (of eMBMS), the NEF rejects the request. It is proposed to correct the NOTE for clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and clarify the NOTE for MB-SMF availability for MBS User Service and Delivery protocols over eMBMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of NOTE is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.10.6	Summary and conclusions

The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV (as used for LTE-based 5G Broadcast) support transparent delivery mode and group communication. Clause 5.2 of TS 23.247 [26] and clause 4.9 of TS 26.502 [29] define interworking between these two systems. However, the architecture does not address deeper integration on User Service level. In this Key Issue, different deployment architectures are shown that allow the user services of MBMS and MBS to beneficially converge. In particular:

-	A service provider can use MBS northbound reference points Nmb8 and Nmb10 to interface with both MBS and eMBMS delivery,

-	In the network, a common MBSTF supports user plane delivery for both MBS and eMBMS. In the latter system, either the group communication or transparent delivery mode is used.

-	In the UE, a common eMBMS-aware MBSTF Client can take advantage of User Service delivery via either MBS or eMBMS. This aspect is important also for future deployments and enhancements harmonisation between the MBMS and MBS delivery methods.

-	Rather than requiring the application to be both MBMS and MBS aware, an application that is only MBS-aware may be implemented while still being able to leverage eMBMS delivery at the radio layer.

Based on this summary, it is recommended to:

-	Fully specify support for the Joint BM-SC and MBSF Functionality. For this purpose, the gap identified in clause 5.10.4.1 of the present document needs to be addressed by documenting additional procedures and baseline parameters as required in TS 26.502 [29] and permitting the signalling of MBMS sessions.

-	Document in an informative annex to TS 26.502 [29] the deployment architectures, client architectures and high-level call flows in clauses 5.10.2.3 and 5.10.2.4.

-	Validate the approaches by implementation, for example in 5G-MAG Reference Tools, and identify if the functionality is fully supported or any further specification updates are needed.

NOTE: Whether a single MB-SMF serving the target service area of eMBMS services exists is FFS.

-	Going forward, ensure that enhancements to the MBSTF and delivery methods in MBS can also be leveraged and deployed for eMBMS.

# * * * * End of changes * * * *