3GPP TSG-SA WG4 Meeting #131						S4-250149

Geneva, Switzerland, 17 – 21 Feburuary 2025

Source:	Panasonic Holdings Corporation

Title:	Target Devices and Performance Requirements for DaCAS Standardization

Agenda item:	7.6

Document for:	DISCUSSION

- Introduction

The new WID on diverse audio capturing system for UEs (DaCAS) has been approved [1].  The source proposes a potential time plan for the WI in another contribution document for this meeting [2]. Given the tight schedule, it is recommended to concentrate on the most relevant areas when defining target devices and performance requirements. Proposals regarding the definition of target devices and performance requirements have been made by referencing TS 26.260 and TS 26.261.

- Target devices

According to the WID [1], a set of target devices or target device types need to be defined.  The largest market for UEs used in 3GPP systems is mobile phones.  These devices are predominantly plate-shaped, although some are foldable as described in TR 26.933 [3].  Even when foldable, they can be categorized as plate-shaped when fully opened or closed.  Similarly, tablets and laptops can be also classified as plate-shaped devices.  Unlike other device types such as headphones, watches, XR devices and cars, these plate-shaped devices share their similar form but vary in size.  We may define a set of target devices as these plate-shaped devices.  However, if it proves challenging to establish a common number of microphones and overall microphone configuration model for all plate-shaped devices, we may focus specifically on the modern mobile phones studied in TR 26.933.  In TS 26.260, the handheld hands-free UE and the table-mounted UE can be chosen as the UE types for the test arrangement [5].  A typical number of microphones and an overall microphone configuration model should be defined, ensuring that it is not specific to any particular model from any specific manufacturer.

- Performance requirements

Terminal audio quality performance requirements for immersive audio services are specified in TS 26.261 [4].  According to TS 26.261, the performance requirements are defined by four parameters: delay, loudness, frequency response, and directional information.  Additionally, complexity should be considered a performance requirement if the capturing solution is based on signal processing.  Performance requirements can be derived from Section 5 of the TS 26.261.  However, many of these requirements remain TBD in TS 26.261.

- Test methodology

Objective test methodologies for the evaluation of immersive audio systems are specified in TS 26.260 [5].  As outlined in TS 26.261, compliance shall be checked by the relevant tests described in the TS 26.260.  Regarding the directional information, TS 26.260 addresses only single source cases.  Considering teleconferencing scenarios where multiple talkers may speak from different directions to the device, such as the Ad-hoc spatial audio telco described in IVAS Usage Scenarios (IVAS-9) [6], testing two or more sources could provide valuable additional information.  That is also true for the Stereo and Immersive Telephony usage scenario, which is also described in IVAS-9.  A simple subjective test may be employed for testing such cases.  For complexity, both memory requirements and computational complexity should be measured using agreed-upon method.  Given the tight schedule, precise measurements may not be practical; therefore, rough estimates may be employed for the measuring method.

- Concluding summary

This contribution document discusses the definition of target devices and performance requirements, along with test methodology.  Plate-shaped devices are proposed as target devices, and performance requirements and objective test methodologies are proposed to be derived from TS 26.261 and TS 26.260, respectively.  In addition to them, complexity is proposed to be assessed, and the evaluation of directional information for more than one source is recommended, in addition to single source scenarios.

- References

- SA WG4, “WID on Diverse audio Capturing System for UEs,” SP-241962, 3GPP TSG Meeting #106, Madrid, Spain, 10th – 13th December 2024.

- Panasonic Holdings Corporation, “Consideration about the DaCAS time plan,” S4-250148, 3GPP TSG SA WG4 Meeting #131, Geneva, Switzerland, 17 – 21 February 2025.

- 3GPP TR 26.933, “Study on Diverse Audio Capturing system”

- 3GPP TS 26.261, “Terminal audio quality performance requirements for immersive audio services”

- 3GPP TS 26.260, “Object test methodologies for the evaluation of immersive audio systems”

- Editor, “IVAS Usage Scenarios (IVAS-9),” S4-231523, 3GPP TSG SA WG4#125 meeting, Gothenburg, Sweden, 21 – 25 August 2023.