3GPP TSG-SA WG4 Meeting #131	S4-250154

# Switzerland, Geneva, 17 – 21 February 2025

Source:	InterDigital France

Title:	Pseudo-CR on QoE and QoS for Haptics Media TR

Spec:	3GPP TR 26.854 v1.0.1

Agenda item:	10.8

Document for:	Agreement

1. Introduction

This contribution provides text to finalize section 10.3 and address additional comments received during the RTC ad-hoc calls.

2. Reason for Change

The bracket would remain, and the text would be uncomplete.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 10	QoS and QoE

## 10.1	Introduction

The QoS factors influencing the QoE for haptics media enhanced services as described in the use-cases of section 5 are mostly similar to those used for traditional AV and immersive AV services (jitter, delay, packet loss etc). The asynchronicity between haptics and other media is a prevalent parameter to be considered when using haptics along with other media. Another characteristic of parametric haptics media is the presence of silent units alongside temporal and spatial units; which can be taken in account when setting PDU Set Importance.

* * * Next Change * * * *

## 10.3 	QoS requirement for haptic media enhanced services

The following from [2] is adjusted to the use cases in section 5.

1. 	Users in an enhanced haptics communication services are equipped with multiple devices to receive and/or capture and transmit audio, video and haptics media.

2. 	When a session starts, multiple streams are established over the wireless network between the corresponding UEs. Table 10.3-1 depicts the typical QoS requirements that need be fulfilled for the users’ QoE to be satisfactory.

Table 10.3-1 Typical QoS requirements for haptic media enhanced services

|  | Haptics | Video | Audio |
| --- | --- | --- | --- |
| Jitter (ms) | ≤ 2 | ≤ 30 | ≤ 30 |
| Delay (ms) | ≤ 50 | ≤ 400 | ≤ 150 |
| Packet loss (%) | ≤ 10 | ≤ 1 | ≤ 1 |
| Update rate (Hz) | ≥ 2000 | ≥ 24 | ≥ 20k |
| Packet size (bytes) per channel | 60-350 compressed parametric 50-1500 time sampled | ≤ MTU | 160-320 |
| Throughput (kbit/s) per channel | n*(fe*16)1 bits/s time sampled 16-32kbps for compressed parametric2 | 2500 - 40000 | 64-128 |

1Note: where n is the number of channels and fe the sampling frequency synthetized in 16 bits.

2Note: throughput for compressed parametric vary based on density and placement of keyframe.

3. The first Media Units (MUs) of haptics media (MIHS), video and voice or audio are capture at the same time. In this case, the MUs have the same timestamp, which represents the generation time. Assuming that the sampling interval for haptics media is smaller than for video and audio, a pacer may be configured to transmit the haptic media MU earlier than the MUs of audio and video, which may result in different arrival time of the MUs of different modalities. Conversely if the destination outputs the MUs synchronously, the output of the haptics media MU needs to be delayed until the voice, audio and or video MUs arrive at the destination.

4. 	A Synchronization Unit is assumed to preserve the time relation of the original signal as steady as possible and synchronize the three media streams with each other. The synchronization unit may contain information such as the identification of the different media streams, a synchronization threshold among other data. Synchronization becomes increasingly challenging with the increasing demand from the application itself, for example immersive XR experience, as well as the inevitable jitter/delay issues (especially due to the nature of the wireless communication) in the network domain. Necessary information is exchanged with the 5G network for the assistance of the synchronization between different streams of a haptics media enhanced communication session. Support from the radio access network scheduling can ensure that interflow synchronization and dependencies are assured with suitable granularity.

NOTE: the information in the synchronization unit is for further study and will align with work in SA2 and RAN.

NOTE: whether the 5QIs defined in TS 23.501 for media streaming and communication services are sufficient to enable haptics media enhanced services may need to be studied further.

* * * End of Changes * * * *