Source:	Nokia

Title:	Characterization of IVAS spatial audio codec

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec_Ph2

# 1. Introduction

IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1] discusses specifics of IVAS Codec characterization testing.

In this document the source proposes test material and conditions for ACR (Absolute Category Rating) experiment covering both legacy mono conditions with several 3GPP mono codecs and all IVAS spatial operation modes. ACR test methodology and different input formats allow large amount IVAS operation modes to be tested and enable easy comparison of subjective voice quality between different signal bandwidths, and with and without spatial dimension.

# 2. Overview

In the current work in progress IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], one or two P.800 ACR [2] listening tests have been reserved. The source proposes specific test material and conditions that can be used to characterize IVAS codec with ACR test methodology. Since the subjective quality varies from low bitrate narrowband mono AMR to uncompressed binaural spatial audio conditions it is proposed to use extended ACR5 scale for quality evaluation [3][4].

# 3. Discussion

Some observations and considerations are made relating, e.g., to the size of experiment. In particular, the P.SUPPL800 [5] DCR experiments can have 36 test conditions. ACR test in principle can have the double number of conditions, i.e. 72 conditions, for the same amount of listenable audio, since there is no reference sample. However, for the listener there is still double number of votes to be casted, causing significant mental load. Thus, the number of conditions should be somewhat lower than 72 conditions. In this document the proposal is to have 60 ACR conditions.

- Currently one ACR-1 listening test is proposed for the IVAS Characterization Phase.

  - The objective is to cover varying signal bandwidths and sampling rates: 16, 32 and 48 kHz.

  - The assumption of this proposal is that this test will be conducted within Nokia listening facilities based on earlier indication of lab capacities for the Characterization listening test campaign.

  - It would be highly beneficial to conduct this test in two different listening laboratories with two different languages.

- It is proposed to test all conditions with either IVAS_dec internal binaural renderer or with IVAS_rend functionality for Direct references.

- Mono References and inputs for coded conditions are generated with IVAS_rend from spatial audio inputs

- For mono conditions diotic headphone playback is used.

- Fixed binaural playback, no head-tracking, for all format supporting BINAURAL

- Fixed point version of the IVAS codec and IVAS_rend should be used, if available.

- It is proposed to test all codecs up to the highest available bitrate or up to a bitrate, where quality saturation is expected.

- No FER, nor DTX conditions are tested.

- No floating point to fixed point interoperation is tested.

- Different IVAS supported spatial audio formats are put to separate input signal categories. The proposed six IVAS spatial signal categories for ACR-1 experiment are:

  - STEREO

  - FOA

  - [HOA2 / HOA3]

  - MASA 1TC

  - MASA 2TC

  - [ISM2 / ISM3]

- All spatial IVAS formats except STEREO are rendered with IVAS_dec or IVAS_rend BINAURAL renderer without head-tracking.

- STEREO and ISM2 support at maximum bitrate of 256kbit/s, ISM3 supports up to 384 kbit/s.

- ISM2 does not support bitrate of 13.2 kbit/s

- ISM3 does not support bitrates of 13.2-16.4 kbit/s

- If the bitrate is not supported with the input format the nearest supported bitrate for that IVAS coding mode is used instead. This duplicates some conditions for that specific input signal category. Those results can either be discarded or used for some additional statistical analysis.

- Listeners can be either experts or naïve. As results show in Chapter 4, expert listeners provide more discriminative results (e.g. separation of mono and spatial conditions, and bitrate quality scaling). While noting this, also naïve listeners had consistent results, when instructed properly and having enough preliminaries.

- Test samples

  - Since high order ambisonics and 2Dir MASA modes are included to the test conditions some of the source material at minimum should be HOA2

  - There are six sample categories. It is proposed to reuse some suitable signal categories from the IVAS selection testing. I.e. Clean speech or noisy speech samples, which have FOA/HOA2/MASA/ISM material available.

  - Some sample categories are clean speech, and some contain also background signal.

  - Some sample categories may consist of “mixed content” e.g. have also music and speech. The proposal is to have this kind of material for STEREO.

  - In addition to the script generated test material, some real recorded speech material should be used for the listening evaluation. E.g. if MASA format is tested with real recordings. Eigenmike or HOA2 signal can be converted to MASA format with MASA- analyzer software [6]-[9].

  - The length of the test samples should be around [8 seconds]. Too short or too varying length samples may cause surprises to the listeners. I.e. listeners are not mentally ready to cast a vote and playback has already ended. Listeners also have to form an opinion of signal bandwidth, spatial quality, signal distortions before forming an overall opinion and casting the vote.

- Enough coverage of the IVAS operation points should be achieved

  - Most common AMR / AMR-WB / EVS deployed bitrates 13.2 and 24.4 kbps should be well covered

  - Also presumed IVAS spatial audio deployment bitrates of 32- 80 kbps should be fully covered.

  - High bitrates up to quality saturation should be tested.

  - Effects of varying signal bandwidth and sampling rates.

# 4. Extended ACR5 voting scale

Nokia has conducted listening tests for a long time with 9-scale ACR test methodology. Recently, Nokia reported some comparison results made with ACR5, ACR9 and extACR5 voting scales for ITU-T [3].

In the proposed IVAS ACR-1 characterization listening test, there are multiple quality dimensions. At least following can be easily separated and mentioned:

•	Bandwidth loss (NB, WB, FB, low bitrate effects on bandwidth)

•	Signal distortion (generated MNRU noise or low bitrate codec introduced signal distortions)

•	Spatiality degradation (generated ESDRU spatial distortion, mono audio, low bitrate introduced spatial issues)

We note that there exists a lot of listening test results and data performed with standard ITU-T P.800 test methodology and ACR 5-scale grading scale. Thus, the proposed voting scale is mostly compatible with the standard P.800 ACR scale. Table 2 shows the proposed scale. We call this an extended ACR5 scale. It is quite similar to ITU-T P.910 11-point ACR scale [10], with the change of numbering from 0 - 10 to 0.5 – 5.5 with 0.5 step size. It should be noted that listeners did not see the decimal values. The user interface screen shot can be seen in Figure 1.

Table 2 Extended ACR5 voting scale

| 5.5 |  |  |
| --- | --- | --- |
| 5 |  | Excellent |
| 4.5 |  |  |
| 4 |  | Good |
| 3.5 |  |  |
| 3 |  | Fair |
| 2.5 |  |  |
| 2 |  | Poor |
| 1.5 |  |  |
| 1 |  | Bad |
| 0.5 |  |  |

Figure 1 Example user interface of using extended ACR 5 voting scale in the listening test software.

# 4.1 Preliminary listening test results

Nokia has performed a shortened version of the proposed extended ACR5 listening test. The main difference to the proposed test plan is the reduced number of conditions and allocation of IVAS spatial coding modes to separate conditions instead of sample categories [4]. Here the results in Figure 1 are shown so that all different IVAS spatial conditions are combined into a single entry per bitrate. Also, all different spatial format reference conditions are combined to a single “Spatial FB” refence condition.

Figure 2 Extended ACR5 scores of spatial audio listening test. Comparison of naïve and expert listeners.

As can be seen from the above listening test results, expert listeners easily distinguish between mono and spatial audio (For Direct FB references the absolute ACR MOS difference is 1,44 between mono and spatial audio). Different 3GPP audio codec generations are also easily distinguishable from each other. Quality improvement with increasing bitrate and bandwidth is very clear. In the listening test, the maximum test bitrate was set to 128 kbit/s, which is just about enough to saturate the results.

Naïve listeners results shown in Figure 2 also show clear difference between different codecs, bandwidths and bitrates. The absolute MOS score difference between spatial audio and mono is significantly smaller than with expert listeners (0,52 vs 1,44). However, even naïve listeners notice the benefits of spatial audio, and prefer spatial audio over mono audio at all tested bitrates.

In addition, the results were analyzed for the voting pattern. Histogram of the voting pattern for both naïve and expert listeners can be seen in Figure 3. As can be seen, naïve listeners in general use less of the extremes of the scale and especially seem to avoid using votes above 4. Our earlier experiments indicate that there may be some cultural differences. Finnish listeners are usually quite critical and often avoid using the “Excellent”-rating. The reasoning being, there could be an even better sample coming later. The difference is clear, and the experts use the full range of voting scale. This naturally leads to better accuracy with expert listeners. Both listener groups were instructed to use the full-scale while rating the samples.

Figure 3 Histogram of voting pattern between naïve and expert listeners

# 5. Proposal for specific test conditions for the ACR test

It is proposed for to have following conditions for the IVAS characterization listening test ACR-1.

Table 3 Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 6 | IVAS-WB spatial 13,2- 64 kbps |
|  | 6 | IVAS-SWB spatial 13,2- 64 kbps |
|  | 12 | IVAS-FB spatial 24,4-512 kbps |
| Codec references | 4 | AMR (narrowband) operated at 4.75, 7,95, 10,2 and 12,2 kbps |
|  | 4 | AMR-WB operated at 6.6, 8.85, 12.65, and 23.85 kbps |
|  | 4 | EVS-WB 7.2, 9.6, 13.2, and 24.4 kbps |
|  | 6 | EVS-SWB 9.6, 13.2, 16.4, 24.4, 32, and 64 kbps |
|  | 6 | EVS-FB 16.4, 24.4, 48, 96, and 128 kbps |
| Other references |  |  |
| Directs | 6 | NB mono, WB mono, FB mono, spatial WB, spatial SWB, spatial FB |
| P.50 MNRU | 3 | Q = 30, 25, and 20 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.8, 0.5, and 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 2 | 3 categories using model-based generation according to processing scripts. 3 categories using true recordings in Eigenmike, HOA3 or STEREO format. |
| Rendering | 3 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend). Diotic playback for mono conditions. Stereo conditions use stereo in and out. |
| Audio sampling frequency / bandwidth | 4 | 8, 16, 32, and 48 kHz, High quality resampling used for up-/down-sampling as needed. |
| Content types / categories | 6 | Scenes as described in Table 4 |
| Number of talkers | 3 | In each sample there may be 2, 3, or 4 different talkers |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) measured from each listenable sample (post-scaling). |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 36 | Naïve and experienced listeners (about equal amount, 3 experts and 3 naïve per panel) |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | ACR with instructions according to [ext] ACR5 |
| Languages | [2] | Finnish / [tbd another test conducted with other language/laboratory is seen beneficial] |
| Listening System | 1 | High-quality headphones (e.g. Sennheiser HD-650), diotic presentation. No head-tracking. |
| Listening Environment | 1 | No noise |

Table 4 Sample Categories

| Category | Format | Overall Level [dB] | Background signal type | Background Level | Overtalk [s] | Note |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | STEREO | -26 | Mixed Music | Na | No overtalk | [Reuse from Mixed Music DCR test? |
| cat 2 | FOA | -26 | Clean speech | Over 30 dB SNR | No overtalk | [Reuse from Clean FOA DCR test] |
| cat 3 | [HOA2/HOA3] | -26 | Outdoors | 15 db SNR | Overtalk | [Reuse from Noisy HOA DCR test] |
| cat 4 | MASA 1TC | -26 | Clean speech (real recording) | Over 30 dB SNR | Overtalk | 4 talkers |
| cat 5 | MASA 2TC | -26 | Outdoors (real recording) | Na | Overtalk | 2 or 3 talkers outdorrs |
| cat 6 | [ISM2/ISM3] | -26 | [Clean speech / audio objects] | Over 30 dB SNR | Overtalk | Static / moving? |

Table 5 Test conditions for extended ACR5 IVAS characterization

| Label | Condition | Bitrate / MNRU / ESDRU | Notes |
| --- | --- | --- | --- |
| C01 | Spatial FB |  | Spatial reference signal is created with |
| C02 | Spatial SWB |  | IVAS_rend BINAURAL |
| C03 | Spatial WB |  | Except stereo input category is played back as STEREO |
| C04 | Mono FB |  | Mono reference signal is created with IVAS_rend MONO |
| C05 | Mono WB |  | for all mono conditions from the spatial input signal |
| C06 | Mono NB |  |  |
| C07 | MNRU | Q= 30 dB |  |
| C08 | MNRU | Q= 25 dB |  |
| C09 | MNRU | Q= 20 dB |  |
| C10 | ESDRU | α=0,8 |  |
| C11 | ESDRU | α=0,5 |  |
| C12 | ESDRU | α=0,2 |  |
| C13 | AMR | 4,75 | Mono input signal is created with IVAS_rend MONO |
| C14 | AMR | 7,95 | for all mono conditions from the spatial input signal |
| C15 | AMR | 10,2 |  |
| C16 | AMR | 12,2 |  |
| C17 | AMR-WB | 6,6 |  |
| C18 | AMR-WB | 8,85 |  |
| C19 | AMR-WB | 12,65 |  |
| C20 | AMR-WB | 23,85 |  |
| C21 | EVS-WB MONO | 7,2 |  |
| C22 | EVS-WB MONO | 9,6 |  |
| C23 | EVS-WB MONO | 13,2 |  |
| C24 | EVS-WB MONO | 24,4 |  |
| C25 | EVS-SWB MONO | 9,6 |  |
| C26 | EVS-SWB MONO | 13.2 |  |
| C27 | EVS-SWB MONO | 16,4 |  |
| C28 | EVS-SWB MONO | 24,4 |  |
| C29 | EVS-SWB MONO | 32 |  |
| C30 | EVS-SWB MONO | 64 |  |
| C31 | EVS-FB MONO | 16,4 |  |
| C32 | EVS-FB MONO | 24,4 |  |
| C33 | EVS-FB MONO | 32 |  |
| C34 | EVS-FB MONO | 48 |  |
| C35 | EVS-FB MONO | 96 |  |
| C36 | EVS-FB MONO | 128 |  |
| C37 | IVAS-WB spatial | 13,2 | IVAS_dec BINAURAL is used for spatial IVAS rendering |
| C38 | IVAS-WB spatial | 16,4 | Except STEREO is used for STEREO input category |
| C39 | IVAS-WB spatial | 24,4 |  |
| C40 | IVAS-WB spatial | 32 |  |
| C41 | IVAS-WB spatial | 48 |  |
| C42 | IVAS-WB spatial | 64 |  |
| C43 | IVAS-SWB spatial | 13,2 |  |
| C44 | IVAS-SWB spatial | 16,4 |  |
| C45 | IVAS-SWB spatial | 24,4 |  |
| C46 | IVAS-SWB spatial | 32 |  |
| C47 | IVAS-SWB spatial | 48 |  |
| C48 | IVAS-SWB spatial | 64 |  |
| C49 | IVAS-FB spatial | 24,4 |  |
| C50 | IVAS-FB spatial | 32 |  |
| C51 | IVAS-FB spatial | 48 |  |
| C52 | IVAS-FB spatial | 64 |  |
| C53 | IVAS-FB spatial | 80 |  |
| C54 | IVAS-FB spatial | 96 |  |
| C55 | IVAS-FB spatial | 128 |  |
| C56 | IVAS-FB spatial | 160 |  |
| C57 | IVAS-FB spatial | 192 |  |
| C58 | IVAS-FB spatial | 256 |  |
| C59 | IVAS-FB spatial | 387 |  |
| C60 | IVAS-FB spatial | 512 |  |

# 6. Summary

We have presented some IVAS characterization test plan specifics and options for agreement in clauses 4 and 5 of this input. The specific proposal covers extended scale ACR5 characterization listening experiment for IVAS operation.

# References

[1]	S4-25014 “IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase”, Version v.0.4.1

[2]	ITU-T Recommendation P.800 : “Methods for subjective determination of transmission quality”, August 08/1996. Online:  P.800 : Methods for subjective determination of transmission quality

[3]	ITU-T T22-SG12-C-0033, “Evaluation of extended ACR voting scales for subjective speech and audio quality testing”, Q7/SG12, Geneve, 7-17 June 2022

[4]	A. Rämö and H. Toukomaa, “Subjective voice quality of the IVAS codec”, in Proc. of the 2025 IEEE International Conference on Acoustics, Speech, and Signal Processing, Hyderabad, India, 2025, to appear

[5]	ITU-T P Suppl. 29 (01/2023): “ITU-T P.800 – Use cases”, Geneva, Switzerland, 2023. Online: P.Sup29 : ITU-T Rec. P.800 use cases - Supplement 29

[6]	J. Paulus et al., “Metadata-assisted spatial audio (MASA) - an overview,” in Proc. of the IEEE International Symposium on the Internet of Sounds, Erlangen, Germany, 2024

[7]	S4-191167: Description of the IVAS MASA C Reference Software, Source: Nokia Corporation.

[8]	S4-210840: Updates to IVAS MASA C Reference Software, Source: Nokia Corporation.

[9]	MASA tools for IVAS: https://github.com/Nokia-Bell-Labs/MASA_tools_for_IVAS

[10] 	ITU-T Recommendation Recommendation ITU-T P.910 (10/2023): “Subjective video quality 	assessment methods for multimedia applications”, Online: https://www.itu.int/rec/t-rec-p.910