3GPP TSG-SA WG4 Meeting #131						S4-250174

Geneva, Switzerland, 17 – 21 February 2025

Source:	Ericsson LM, Fraunhofer IIS, Dolby Laboratories Inc., Nokia

Title:	Observations on IVAS fixed-point encoder release on January 31

Agenda item:	7.5

Document for:	Discussion and Agreement

# Introduction

On January 31, Ittiam shared a release of the fixed-point code implementation which includes the IVAS Encoder [1] as per the agreed time plan in [2]:

| 31st January, 2025 | - Delivery of the Encoder v2, targeting fulfilment of FL-to-FX requirements, as basis for subjective verification |
| --- | --- |

This document aims to present results based on objective measurements and informal evaluation/verification to assess the quality and suitability of the Encoder v2 for subjective evaluation.

# Assessment results

## Objective measurement results

In the following, plots of objective measurements consisting of the Maximum Loudness Difference (MLD), Objective Difference Grade (ODG) and the Segmental SNR (SSNR) are presented for some exemplary input formats. Similarly as for the IVAS Decoder/Render verification in [3], the selection test experiments and input samples were used with the BASOP encoder and Float decoder binaries. It should be noted that the ODG plots are showing the scores of Float Enc/Float Dec output compared to the original input signal minus BASOP Enc/Float Dec output compared to the original input signal. The tests include -16, -26 and -36 LKFS loudness level, different sampling rates (16, 32 and 48 kHz) and tests with DTX on and off.

As the MDCT-stereo mode and MCT serve as the core-coding modules for other input formats like SBA, MASA, the parametric multi-channel formats and the combined formats, the results for these two formats have been separated and shown below:

Overall, the data in the above plot indicates that while most metrics exhibit good performance, as the majority of the tests fall withing the requirements set from SA4 in [4], there are numerous outliers that suggest that the Encoder is not stable at all operating points and indicate underlying issues. For all three metrics there are outliers that show significant differences, especially for the MCT case with 7.1+4.

Some more example plots from other input formats our shown in Annex A. As illustrated in these additional plots, the general trend suggests that although the majority of tests meet the requirements, certain configurations, especially for the examples of Unified Stereo and SBA, exhibit a significant number of outliers that point to considerable underlying issues.

In Annex B additional objective measurements and analysis regarding OSBA, SBA and MC ParamUpmix is presented. The respective plots suggest the presence of systematic problems in the fixed-point encoder codebase related to saturation (due to the severe worsening of statistics with respect to increasing LKFS) as well as fixed-point implementation issues related to low-bitrate coding in SBA and OSBA (as seen by the differences with respect to number of transport channels, where 1-transport channel cases correspond to bit rates at or below 32 kbps).

Similarly, in Annex C, the plots shared for MASA and OMASA imply systematic issues. These issues are identified not only for the selection item database but also for the test cases with long test vectors (LTVs) used for the Continuous Integration (CI). As seen, there is a large number of outliers present for these operating points, including some with very high MLD scores. Additionally, a significant number of test cases fall below the 20 dB SSNR threshold, indicating substantial differences between the fixed-point and floating-point coded signals.

## Informal verification and outstanding issues

During the pre-testing phase outlined in [2] the IVAS Public Collaboration companies have been conducting informal verification of the Encoder and have been reporting issues to the GitLab issue tracker . The current status of the issues (at the time the document was prepared) related to the Encoder is as follows:

| Priority | Number |
| --- | --- |
| Blocker | 2 |
| Critical | 12 |
| Major | 38 |
| Medium | 10 |
| Minor | 2 |
| Total | 64 |

Highlighted outstanding issues:

- The core coder has classification issues, affecting the mode selection. This may lead to distortions, as noted in issues #1124, #1272 and #1276.

- For unified stereo many issues with serious distortions in bandwidth extension still exist, see issues #1195, #1146, #1223, #1243, #1246. There are also issues related to Stereo CNG, e.g. #1247 and #1277.

- For MDCT-stereo there are quite many cases with higher MLDs for the bitrates 48 kbps and 64 kbps and comparison of the fixed-point encoded signal to the floating-point encoded signal that can be seen below, shows significant differences between the two signals (see also #1078).

This is very likely due to differences with ITD estimation/compensation but also other stereo tools that are used for this bitrate, like stereo preprocessing and kernel switching.

Additionally, another major issue that has been detected in many test items, also for higher bitrates, is an attenuation of mid frequencies (see #issue 1175) that can produce major audible distortions.

- For multi-channel MCT there is a serious issue detected where the output signal is completely distorted for a long portion possibly due to saturation  (#issue 1262). This could explain the extremely high MLD cases detected for MCT in clause 2.1.

- For SBA there are several issues reported with the most dominant issue being at the very low bitrates and specifically 13.2kbps with different input signal levels as reported in #1280, #1181, #1207, #1249, #1216, #1196, #1115.

# Proposal

From the above analysis, it is clear that the Encoder v2 is not in a state suitable for subjective testing and formal verification. The presence of numerous outliers in objective measurements and the critical issues reported during informal verification highlight significant stability problems at various operating points. These concerns, particularly regarding distortion in Unified stereo, MDCT-stereo and MCT, indicate that further development and refinement are necessary.

Based on a rough estimation of the sources, an additional 4 weeks will be required to address these issues and ensure a stable state for the Encoder. Therefore, it is proposed that the group takes action to adjust the time plan in [2] accordingly.

# References

- https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/releases/20250131_Ittiam_fixed_point_encoder_release

- S4-242071- IVAS Permanent Document IVAS-10 “IVAS BASOP Verification” v.0.1.0

- S4-242054 – Verification of IVAS BASOP code, Decoder and Renderer, Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Nokia, Philips International B.V., VoiceAge Corporation

- SA4aA240038 - Organization of IVAS Characterization Phase – rev 1, 3GPP SA4 Audio SWG

- https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues

# Annex A – Example plots of objective measurements (Fraunhofer IIS)

# Annex B - OSBA, SBA and MC paramupmix encoder verification (Dolby Laboratories, Inc.)

As of 10 February 2025, the Encoder V2 release has 15 reported open issues related to SBA, 2 issues related to ParamUpmix and 1 related to OSBA. 17 of these issues are Critical or Major. Of these, 9 issues are related to SBA format at 13.2 kbps. The Critical and Major issues correspond to obvious and audible audio artifacts in the fixed-point outputs. These open issues are preventing the start of subjective listening verification of the fixed-point encoder for these modes.

The plots below show objective evaluation (MLD histogram) results on the IVAS selection test material. The material was normalized to -16, -26 and -36 LKFS loudness level and both CTX and DTX cases were evaluated at various bitrates. MLD histogram plots are shown below. Output mode was EXT in all cases.

# Annex C – MASA and OMASA encoder verification (Nokia)

MASA and OMASA encoder operation points were evaluated on a February 10th 2025 commit of development code (SHA b009c0f9). The evaluation consisted various input files including LTV test vectors (for MASA and OMASA) and selection test material (for MASA). All bitrates and bitrate switching were included. Input sample rate and coding bandwidth were also varied to include WB, SWB, and FB conditions. DTX option was also varied for MASA conditions.

The decoder output for MASA input was EXT and the decoder output for OMASA input was mono. The metadata encoding was not analyzed in detail in this evaluation.

The following figures show the MLD and SSNR values of the path BASOP encoder to BASOP decoder compared to the reference float encoder to float decoder.