3GPP TSG-SA WG4 Meeting #131 	S4-250177

# Geneva, Switzerland, 17th – 21th February 2025

Source:	Fraunhofer IIS

Title:	On Testing Input Signal Levels for IVAS Characterization

Document for:	Discussion and Agreement

Agenda Item:	7.5

- Introduction

In the latest version of the Test Plan for Characterization Phase [1], one of the assumptions listed in clause 5.4 is the following:

- “It is assumed that the subjective material in the P.800 experiments comprising fixed-point validation accommodates all three input signal levels. Levels can be integrated into signal categories or as different audio samples within categories.”

However, this point has not been covered until now in the planned P.800 experiments in Annex F of [1]. This contribution aims to address this topic that is still uncovered by proposing an approach that could be common for all experiments.

- Discussion

For the characterization testing of the IVAS fixed-point implementation, it is essential to evaluate input signals of varying levels to ensure comprehensive quality assessment across diverse audio conditions. Input signal levels can significantly influence the performance of the encoder, as different loudness levels may expose stability issues, distortion, overflows or other artifacts that may not be evident at a single reference level. By testing a range of loudness levels, such as -16, -26, and -36 LKFS, it is shown, how the fixed-point encoder reacts to dynamic changes in audio input. This testing is particularly important for characterizing the encoder’s response to both typical usage scenarios and edge cases, ultimately guaranteeing that the encoder performs reliably and maintains audio quality under real-world conditions.

In the assumption in [1] it is stated that the way one could integrate testing different input levels would be to either integrate into signal categories or as different audio samples. For the signal category approach, the drawback is, if there are failures in certain categories it is hard to distinguish if they are due to the general codec performance on a specific category or the input signal level. Therefore, the source proposes to select audio samples to integrate the input level testing in a predetermined manner. Then it is easier to isolate issues that may appear due to the input levels and facilitates the task of creating the respective processing scripts. It should be noted that this option too comes with the trade-off of possibly degrading the overall performance of a certain category.

- Proposal

According to the above it is therefore proposed to integrate in each experiment testing of the input levels by assigning to the samples a different input level in a circular manner as seen in Table 1.

Table 1 - Assignment of different input levels per experiment

| Samples | Input Level (in LKFS) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | cat 1 | cat 2 | cat 3 | cat 4 | cat 5 | cat 6 |
| s01 | -16 | -36 | -36 | -26 | -26 | -16 |
| s02 | -16 | -16 | -36 | -36 | -26 | -26 |
| s03 | -26 | -16 | -16 | -36 | -36 | -26 |
| s04 | -26 | -26 | -16 | -16 | -36 | -36 |
| s05 | -36 | -26 | -26 | -16 | -16 | -36 |
| s06 | -36 | -36 | -26 | -26 | -16 | -16 |

This ensures that all audio samples are tested across all input levels while maintaining an equal distribution of input level assignments within each category.

Subsequently, it is proposed to include Table-1 in clause 5.1 “General Consideration of Experiments” of [1] and update the experiments in Annex F for each Experiment P800-X:

Table F.X.1: Conditions for Experiment P800-X

| Main Codec Conditions |  |
| --- | --- |
| Candidate |  |
| Bitrates |  |
| DTX |  |
| Input level | As defined in Table (1) |
| Input frequency mask |  |
| Noise |  |
| Error Conditions |  |
| References |  |

In conclusion, it is requested to agree on this proposed approach to integrate input level testing in each experiment. As discussed, this method allows for uniform distribution of the input level testing throughout the experiment, enhancing the reliability of the IVAS encoder's performance evaluation.

References:

- 3GPP S4-241195 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v0.4.0