Title:		            On Enhancing Negotiations

Source:	Panasonic Holdings Corporation

Document for:	Discussion

Agenda item:	7.9

Introduction
At SA4-130, TR 26.933 [1] was concluded, yet certain aspects remain unaddressed. One key issue is the negotiation of UE audio capture capabilities, which was left for further study. Modern mobile devices support multiple audio interfaces, enabling dynamic switching between calling modes during an ongoing call. Additionally, external devices can further enhance communication quality. However, current SIP/SDP negotiation mechanisms discussed in 3GPP do not account for terminal status or capture capabilities, leading to suboptimal user experiences.

Discussion

- Evolution of Mobile Audio Hardware
Unlike traditional mobile devices, modern smartphones and communication devices integrate multiple loudspeakers and microphones, enhancing the user’s audio experience. However, existing market-adopted codecs primarily support mono audio. While IVAS, standardized recently, introduces immersive audio capabilities, its practical application in mobile communication remains unclear. IVAS supports the following coding modes at bitrates ranging from 13.2 kbps to 512 kbps.

Table 1: IVAS coding modes

| Coding Mode | Full Name |
| --- | --- |
| Stereo | Stereo |
| SBA | Scene-based Audio (Ambisonics) |
| MASA | Metadata-assisted Spatial Audio |
| ISM | Objects (Independent Streams with Metadata) |
| MC | Multi-Channel |
| OMASA | Combined Objects and MASA |
| OSBA | Combined Objects and SBA |
| SR | Split Rendering |

Note: IVAS also supports EVS compatible mono downmix operation

- Challenges in Current SIP/SDP Negotiation

Lack of Terminal Awareness: Current negotiation mechanisms do not adapt to terminal status (e.g., handheld, table-placed, or hands-free modes). Without this awareness, IVAS coding modes cannot be effectively negotiated, as IVAS supports multiple coding modes as listed in Table 1.

Diverse UE Configurations: Variations in hardware configurations (e.g., loudspeaker count, microphone placement, and frequency response) among manufacturers impact audio capture and playback, yet these factors are missed in the TS 26.114 [2].

Audio vs. Video Adaptability: While video codecs dynamically adjust based on device orientation using metadata and RTCP mechanisms, widely adopted audio codecs lack similar adaptability.

- Impact of Terminal Status on Audio Behaviour

Microphone Configuration:

Handheld Mode: Uses a primary microphone close to the user's mouth, prioritizing mono audio.

Hands-Free Mode: Activates multiple microphones for improved far-field voice capture.

Speaker Configuration:

Handheld Mode: Mono output via the earpiece speaker.

Hands-Free Mode: Uses loudspeakers or external devices, where immersive audio would be beneficial.

- Context-Aware Call Negotiation

Calls are typically received in handheld mode, where immersive audio features (e.g., spatial audio, object-based audio) provide little benefit due to the proximity of the device to the user.

If a user switches to hands-free mode during a call, immersive audio should be enabled dynamically for enhanced spatial sound.

Existing SIP/SDP negotiations lack the ability to adapt codec or coding modes based on such transitions.

Proposal

Source believes that device capabilities is an important element, and some mechanism must be standardized. As such to start the discussion on the important topic, source listed some possibilities for further discussion.

For enhancing negotiations

To optimize the user experience in modern audio and video-audio calls, terminal-aware negotiation should be considered. As such, following improvements are proposed for discussion:

- Calling Mode Change Detection

- New SDP Attributes for Audio Capture Configuration

  - Define additional SDP parameters for negotiating audio capture capabilities dynamically.

  - Alternatively, communicate terminal status and routing changes seamlessly via SDP and RTCP.

- Adaptive Codec and Coding Mode Negotiation

  - Allow real-time updates to codec based on detected terminal status changes.

Conclusion

Modern mobile hardware supports multiple audio interfaces, allowing users to switch calling modes dynamically. However, current SIP/SDP negotiations do not consider UE status changes during a call. Context-aware negotiations are needed to optimize audio capture and playback, codec selection, and immersive audio experiences. To achieve this, we propose integrating calling mode detection, new SDP attributes, and adaptive codec switching mechanisms into the negotiation framework.

References

- TR 26.933: Study on Diverse Audio Capturing System

- TS 26.114: IP Multimedia Subsytem (IMS); Multimedia Telephony; Media handling and interaction