Agenda item: 	10.6

Source: 	Qualcomm, Lenovo, Nokia

Title: 	[5G_RTP_Ph2] On the definition of Data Burst

Document for	Discussion and Agreement

# Introduction

The data burst was initially defined by SA2 in TS 23.501 [1]:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

However, it is not clear how short is “short”. SA4 then gave a second definition to resolve this issue. Furthermore, the new definition mandates that there is an idle period between two data bursts. The SA4 definition was given in TS 26.522 [2]:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

Although the SA4 definition resolves the issue with “short” in the SA2 definition, it has its own issue with the “idle period”. This is because, depending on the implementation of the congestion control algorithm, a time gap or idle period may always exist between any two adjacent RTP packets. This is true even when no paced sending is used. In case of paced sending, for example, for Self-Clocked Rate Adaptation for Multimedia (SCReAM2) [3], the associated packet pacing algorithm described in clause 4.2.5 of RFC 8298 [3] tries to evenly distribute packets in time. This means that any arbitrary grouping of packets into data bursts will satisfy the definition of the data burst, but obviously arbitrary grouping is not helpful for the network.

Even if the congestion control algorithm may allow packets to be sent back-to-back, the underlying network protocol stack may introduce an idle period, or gap in transmission, between adjacent packets. Take Ethernet as an example, there is always a time gap between any two Ethernet frames due to the Ethernet design, i.e., inter-frame gap (IGP) scaling with the link speed. Specifically, as pointed out in [4], for 10 Mbps Ethernet links

“all adaptors wait 9.6 µs after the end of one frame before beginning to transmit the next frame”,

this value decreasing linearly to 9.6 ns for 10 Gbps Ethernet links and so on.

Therefore, an idle period may always exist between two adjacent packets, and the wording “that there is an idle period between two data bursts” in the definition in 26.522 essentially says nothing special.

# Candidate revisions of the data burst definition

Given that an idle period may always exist between two adjacent packets, how do we modify the SA4 definition to make it meaningful? A natural attempt is to add a qualification on the duration of the idle period, for example as follows:

Definition 1:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts that is greater than a threshold. A Data Burst can be composed of one or multiple PDU Sets.

If we adopt a threshold, then a question is: how is the threshold selected? The simplest solution is to leave it to the application of the sender, but the decision by the sender may not align with how the data burst information is used by the network. A more sophisticated solution is to let the network tell the threshold to the sender, but this involves standard specification of the signaling between the network and the application.

The definition of the data burst should be conducive to the SA4 specification of data burst characteristic signaling and the implementation. Considering this, the adoption of a threshold has a problem, because it may make the signaling of the burst size impossible without excessive delay (as detailed in [6]) while constraining application behavior to network-imposed timing constraints, unfeasible in practical deployments.

Given the above reasons, it is beneficial to let the application at the sender to decide how to group the packets into data bursts based on the information available which may be implementation specific. Therefore, we proposed the following definition aligned with existing SA2 data burst definition in TS 23.501 [1], while clarifying the “short period of time” scope:

Definition 2:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time. .

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

NOTE 2:  The sender application determines “a short period of time” based on its implementation.

To further align data burst definition across SA2 and SA4, SA4 could notify and suggest SA2 modifying the SA2 definition to address SA4’s concern on the wording “short”.

An alternative definition was proposed in [5]:

Definition 3:

Data Burst: A data burst is a set of consecutive PDUs with shorter inter-PDU gaps than the PDUs sent before or after the data burst.

NOTE 1: A data burst can be composed of one or multiple PDU Sets.

NOTE 2: Meaning of a short inter-PDU gap needs to be determined by the application and can vary depending on different factors including the available bandwidth and link utilization.

While Definition 3 eliminates the need for an absolute threshold in Definition 1, it relies on a comparison of the time gaps within a data burst and the gaps to packets outside the data burst. This introduces additional complexity related to control and meaning of inter-PDU gaps, which may be outside the control and scope of the application at the sender, and in fact related to medium and transport network effects, as for Ethernet frames for example.

# Proposals

Proposal 1: Adopt the below definition of the data burst in TS 26.522

:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

NOTE 2:  The sender application determines the meaning of “a short period of time” based on its implementation.

Proposal 2: SA4 sends an LS to SA2 to notify the change in Proposal 1 and request SA2 consider any similar improvements, e.g., incorporating NOTE 2, to the definition in TS 23.501 [1] to address the scope of ‘short’.

References

[1]		TS 23.501, "System architecture for the 5G System (5GS)", Rel-19, V19.1.0, Sep 2024.

[2]		TS26.522, 5G Real-time Media Transport Protocol Configurations, Rel-18.0.0, March 2024.

[3]		RFC 8298, Self-Clocked Rate Adaptation for Multimedia, draft-johansson-ccwg-rfc8298bis-screamv2-00, 2024.

[4] 	L. L. Peterson, B. S. Davie, “Computer Networks – A systems Approach”, 6th Ed, 2022.

[5]		Nokia, S4aR250021, "On the definition of data burst", SA4-e (AH) RTC SWG post 130, 15 January 2025, online.

[6]		Qualcomm, S4aR250048, "On the definition of Data Burst", RTC Ad Hoc Meeting Post SA4 #130 Meeting, February 5, 2025.