Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] On the starting time of the Time To the Next Data Burst

Document for	Discussion and Agreement

# Introduction

In clause 6.25 of TR26.822 [1], the following two options on the definition for the time to the next data burst (TTNB) are proposed:

-	Option 1: the starting time is the departure time of the first packet of the first PDU Set.

-	Option 2: the starting time is the departure time of the last packet of the last PDU Set.

NOTE: This needs to be coordinated with SA2/RAN2.

and their respective pros and cons are analyzed therein.

However, the data bust is not necessarily related to PDU Sets [2], and either of the above options assumes on the contrary that a data burst consists of one or multiple PDU Sets. Therefore, the definition needs to be revisited.

# Proposed definitions

### 2.1 The first definition

To decouple PDU Sets from the data burst, a first definition is proposed as follows:

-	Option 1: the starting time is the departure time of the first packet of the current data burst.

-	Option 2: the starting time is the departure time of the last packet of the current data burst.

It is not clear which of the above two is necessarily better given the analysis of the pros and cons in [1], which apply to this new definition.

Moreover, if the first packet is lost, then with Option 1, there will be no TTNB information about the next data burst available to the UPF, considering the DL case. Similarly, the same issue occurs if the last packet is lost, and Option 2 is adopted.

### 2.2 The second definition

To increase the error resilience, we consider the following definition:

The definition of TTNB is one of the following options:

-	Option 1: the starting time is the departure time of the first packet of the current data burst.

-	Option 2: the starting time is the departure time of the last packet of the current data burst.

Additionally, one or more packets can carry the TTNB information. How many packets in a data burst carry the TTNB information can be based on packet loss rate, and left to RTP sender (e.g., the application server) implementation.

Figure 1 illustrates Option 1 for the second definition. Note that all packets carry the same TTNB value.

Figure 1: Option 1 of the 2nd definition.

However, this definition has a drawback. It is impossible for the UPF to know when the next data burst will arrive, if the reference packet (the first packet in Option 1 and the last packet in Option 2) is lost.

### 2.3 	The third definition

Proposal: For the definition of TTNB, the starting time is the departure time of the current packet which carries the TTNB information. Additionally, how many packets carry the TTNB information is up to RTP sender implementation. This definition includes the first definition as a special case.

This is illustrated in Figure 2:

Figure 2: The third definition when packets depart the traffic source.

With this definition, different packets generally carry different TTNB information based on when the respective packets depart from the traffic source.

Another benefit of this definition is that it allows the UPF to estimate the N6 jitter (see clause 5.37.8.2 of TS23.501[3], where the scenario is periodic traffic but the need applies equally to dynamic traffic). This is shown in Figure 3. The N6 jitter can be estimated as (|d1’-d1| + |d3’-d3| |)/2.

Figure 3: The third definition when packets arrive at the UPF.

NOTE 1: when the burst durations are very short, e.g., less than 1ms, the differences among the three definitions become negligible.

NOTE 2: There are congestion control algorithms that try to deliberately reduce the burstiness of the traffic, because such traffic pattern may cause jitter and packet losses. One example is the packet pacing of SCReAMv2, which tries to distribute packets evenly in time, as detailed in 4.2.5 of [4]:

Packet pacing is used in order to mitigate coalescing, i.e., when packets are transmitted in bursts, with the risks of increased jitter and potentially increased packet loss. Packet pacing is also recommended to be used with L4S and also mitigates possible issues with queue overflow due to key-frame generation in video coders.

While the packet pacing of SCReAMv2 is on the other extreme, it is conceivable that some packet pacing is moderate and in between the two extremes (meaning the scenario in NOTE 1, and the evenly distributed traffic pattern). For such moderate packet pacing schemes, the differences among the three definitions can be significant.

NOTE 3: A good definition of TTNB needs to be useful for a wide range of scenarios.

NOTE 4: the third definition includes the first definition as a special case.

NOTE 5: if the last packet of the current data burst is lost, even though the base station knows when to wake up the UE, it does not know when to let the UE go to sleep.

To address NOTE 5, we propose that the packets also indicate the respective time gaps to the end of the current data burst in addition to the TTNB. The additional time gaps tell the base station when the data burst is supposed to end.

# Proposal

To have an error-resilient and generic definition that is also flexible to implementation, we propose the following definition.

Proposal: For the definition of TTNB, the starting time is the departure time of the current packet which carries the TTNB information. Additionally, how many packets carry the TTNB information is up to RTP sender implementation. The packets that indicate TTNB also indicate the respective time gaps to the end of the current data burst.

# References

[1] 	TR26.822, Study on 5G Real-time Transport Protocol Configurations, V1.2.0, Rel-19, Nov 2024.

[2]		TS26.522, 5G Real-time Media Transport Protocol Configurations, Rel-18.0.0, March 2024.

[3]		TS 23.501, "System architecture for the 5G System (5GS)", Rel-19, V19.1.0, Sep 2024.

[4] 	Self-Clocked Rate Adaptation for Multimedia, https://datatracker.ietf.org/doc/draft-johansson-ccwg-rfc8298bis-screamv2, October 2024.