3GPP TSG-SA WG4 Meeting #131						S4-250205

Geneva, Switzerland, February 17 – 21 February 2024

Source:	Nokia

Title:	On testing UEs in ambience

Agenda item:	7.7 (ATIAS_Ph2)

Document for:	DISCUSSION

# 1. Introduction

During the ATIAS ph1 work, no test methods/requirements for assessing the capture performance of ambient sound field or in presence of background ambience were defined. Despite the effort on defining test methods comprising diffuse sound field (de-correlated pink noise) and multiple simultaneous talkers in a single-sound scene, no progress leading to the actual specification changes was made.

In this contribution, an overview on the expected User Equipment (UE) capture and transmission behaviour in the ambient sound field, possible approaches for simulating the ambient sound fields, and envisioned test methods for assessing the UE performance are presented and discussed.

## 2. Expected UE sending behaviour in ambience

While a sufficient speech quality is one of the key drivers on defining the test methods and requirements for IVAS based UEs, the quality and performance characterization of ambience capture should not be neglected. The main question regarding the capture and transmission of the ambience is how the UEs are expected to perform in such sound fields with and without active speech.

In the future, UEs with immersive capabilities will be used in a vast amount of different noise and ambience conditions. In addition, the applied signal processing may vary depending on, e.g., the utilized UE type(s), acoustical environment, or user preference. Thus, following items were identified to assist on defining necessary tests and requirements for assessing the ambient sound capture:

- Test methods and requirements should guide the manufactures to achieve expected UE capture performance in the ambient sound field with and without active speech.

- Speech quality should be sufficient in every circumstance, including spatial characteristics.

- If the ambience is not targeted to be removed, the transmitted ambience should have similar characteristics as the original sound field.

- Flexible amount of ambience attenuation may be allowed, e.g., based on the user preference, noise conditions, use case, etc.

- If ambience is targeted to be removed, the amount of applied attenuation should be sufficient, while the quality and spatial characteristics of the active speech should be maintained.

- Despite the targeted ambience attenuation, simultaneously received far-end signal should not cause interference with the captured and transmitted near-end signal.

To facilitate the assessment complying with the above aspects, three different capture type characteristics for sending direction can be defined:

- Transparent capture: Sound field is intended to be captured and transmitted “as is”. The receiver expects accurate representation of the captured sound field in terms of loudness, frequency response, temporal characteristics, and spatial characteristics. Minor attenuation and/or modification of the captured ambience is allowed, to facilitate robust speech transmission. If speech is present, target speech-to-ambience ratio is typically improved [0-6] dB.

- Nominal capture: All speech is intended to be captured and transmitted “as is”. Moderate attenuation and/or modifications of the captured ambience is expected in order to enhance the speech transmission. Improvement to speech-to-ambience ratio is typically at least [6-12] dB.

- Suppressed capture: All background is expected to be attenuated, while foreground speech (the main talker) is transmitted. Spatial characteristics of the transmitted speech are expected to be maintained. Target improvement on speech-to-ambience ratio is over [12] dB.

The expected performance of each capture type can be defined via sufficient set of test methods and respective requirements. Furthermore, to achieve reasonable testing efforts, a common set of tests could be defined, with different requirements associated with a different capture type. Further discussion on envisioned test methods is in the section 4.

It should be noted that the presented capture type definitions are considering transmission characteristics only in send direction. Similar considerations might be required also for receiving direction but are out of this document’s scope. In addition, the presented capture characteristics should not be understood as performance classes but are merely motivated by different usage scenarios.

# 3. Test setup for simulating background ambience

As varying background suppression characteristics might be anticipated, test methods comprising background ambience are considered to be increasingly important with immersive voice and audio services. To specify any test methods comprising simulated sound fields, a simulation method for generating the sound fields, and applied test signals should be specified. For reduced testing complexity and cost, a simple loudspeaker setup with robust and reproducible calibration procedure would be ideal, while providing compatibility with vast number of different test methods. In addition, the reproduction accuracy of spatial characteristics should be considered.

## 3.1. Loudspeaker layout

In the current specification TS 26.132 [1], two different test setups for simulating realistic ambient noises may be used with Hand-held Handsfree (HHHF) UEs. The specified test setups are defined in ETSI TS 103 224 [2] and ETSI ES 202 396-1 [3]. The main differences of the setups are the number of loudspeakers (8 vs. 4) and utilized calibration procedure (matrix inversion via mic array vs. iterative equalization via HATS). As both methods are well defined and commercial implementations are available, it should be investigated whether either or both simulation methods would be applicable for simulating the ambient sound fields also for immersive test cases.

Figure 1a: Setup using TS 103 224 method	Figure1b: Setup using ES 202 396-1 method

In technical report 3GPP TR 26.921 [4], an investigation on the accuracy of aforementioned sound field simulation setups is provided. As the assessment is mostly based on comparing the test results in the two different simulated sound fields, no definitive conclusions on the applicability for immersive testing can be made. However, due to the increased number of loudspeakers and more advanced calibration procedure, the source assumes that the simulation system described in TS 103 224 [2] would produce more consistent sound field in terms of spatial accuracy. Similar statement is made in TS 26.132 [1] section 5.1.5.2: “TS 103 224 [43] provides improved reproduction accuracy in terms of frequency and spatial characteristics, and so is recommended for testing hands-free UE.”. Thus, the applicability of test system specified in ES 202 396-1 [3] can be questioned, and further considerations should be targeted to the method described in TS 103 224 [2].

In addition to the already standardized background noise simulation methods, a new simulation method could be developed and standardized for specifically testing immersive aspects. E.g., a surround system comprising 7.1 or 7.1+4 loudspeaker configuration placed in a room with a well-defined characteristic could be considered as a sufficient test setup for assessing the test methods requiring simulated ambient sound field or complex sound scene. To ensure consistent reproduction across different conditions, the calibration procedure for such setup could be done according to the section 7 in TS 103 224 [2] by utilizing, e.g., artificial ears of HATS as the calibration microphones.

Figure 2: Setup using 7.1 loudspeaker configuration

In conclusion, during the definition of any test method comprising ambience or complex sound scenes, a suitable sound field simulation method needs to be identified. At least the applicability of simulation method described in TS 103 224 [2] as well as 7.1(+4) surround system in properly defined room in terms of acoustic properties should be investigated. In addition, it would be beneficial to determine a setup that could be used with all defined test methods comprising ambient sound field or complex sound scene.

## 3.2. Reference ambient recordings

When defining test methods for immersive communication comprising ambient sound field, the utilized ambient scene should be chosen carefully. In TS 26.132 [1], tests comprising simulated background ambience are targeted to assess both speech quality and noise intrusiveness, i.e., the UE’s capability to remove background noise while maintaining speech quality. Due to the tested features, the specified ambience scenes are representing mostly diffuse noise scenarios. Considering a immersive call, such background noise signals are not necessarily interesting or informative from the receiver’s point of view.

Furthermore, both TS 103 224 [2] and ES 202 396-1 [3] have attached databases comprising multiple different background ambience scenes. Unfortunately, most of the scenarios represents noise or noisy environments, thus only relaying on these signals may not be feasible. While the spatial capture and reproduction may reduce the annoyance caused by a background noise, user may not necessarily want to transmit full ambience in such environments. Thus, before using the test signals included in the aforementioned datasets, it should be examined whether those databases have any meaningful sound scenes  for immersive testing. In addition to the available databases, test scenes and recordings specifically targeted for immersive testing should be considered. E.g., relevant background sound fields for immersive communication and testing could represent:

- Natural sounds, e.g., flowing water or birds in the park

- Music, e.g., a concert or street performance,

- Family events, e.g., a birthday party.

Depending on the assessed feature and utilized capture scenario, the active speech source could be defined similarly as in TS 26.260 [5], i.e., a user as an active talker is simulated using (a HATS with) an artificial mouth, and talker specifically located in the spatial scene may be included to the background ambience signal or simulated with a loudspeaker or artificial mouth (of HATS). However, to reduce the number of potential test scenarios, the source considers that the work should target on defining ambience tests with only user’s foreground speech as an active speech. In any case, additional care on defining active foreground and background speech should be taken when specifying any test scenarios including other than user’s speech.

## 4. Test methods

Traditionally, background ambience has been seen as a disturbing and undesired signal component, which should be attenuated from the captured and transmitted signal. Due to this assumption, traditional tests assessing the send performance in presence of background ambience are merely developed for assessing how well the background noise is attenuated, and how well the speech quality is maintained.

With immersive communication, the background ambience can be seen as an (equally) important component of the transmitted sound field, i.e., the signal processing of UE should not necessarily attenuate or remove the background ambience completely. In addition, it is expected that some UEs should be able to transmit sound field even without active talker. Thus, following test method categories are identified:

- Speech capture tests

- Ambience capture tests

- Speech and ambience capture tests

Each capture test category can be considered to be (more or less) relevant for each capture type presented in the Section 2, while the requirements for tests in different capture test categories may vary depending on the capture type. For example, obtained loudness of pure ambient sound field capture should be higher with Transparent capture type than with Suppressed capture type. On the other hand, some test category requirements might be common for all the presented capture types, e.g., requirements for the speech capture tests might be the same despite the underlying capture type.

### 4.1 Speech capture tests

Test methods targeted to speech capture assessment are intended to evaluate relevant capture characteristics for speech transmission, typically involving only speech as a test signal. Most of the specified test methods in TS 26.260 [5] in send direction could be classified to this test method category. Any additions or further considerations on test methods for speech capture assessment are not discussed in this document.

4.2 Ambience capture tests

Ambient capture tests can be considered to comprise all tests targeted to assess capture and delivery of more complex sound fields comprising other sounds than speech. In Figure 3 an example framework on assessing the ambient sound field capture performance is illustrated. In addition, more detailed description regarding the envisioned analysis metrics is presented.

Figure 3: Common test framework for assessing the capture of ambient sound field

### Loudness

When considering the immersive communication, it is expected that some UEs can transmit complex sound fields comprising only sound components other than speech. Thus, to ensure that such UE can capture and transmit ambient sound fields without an active talker at a sufficient level (note that the level may depend on the underlying capture type assumption), a simple send loudness test can be enforced.

For testing the send loudness, the UE is positioned in the simulated ambient sound field. Sound field is then captured, encoded, and sent to the reference client, where the transmitted, decoded signal is analysed against the reference sound field. For analysis, Loudness Rating metric, as utilized with only speech, could be considered. In addition, more evolved metrics could be applied, e.g., ITU-T P.700 [6] with sufficient mapping to acoustic domain.

### Frequency response

In addition to the send loudness, a send frequency response can be considered as a basic characteristic for assessing the ambient sound capture performance. Depending on the expected capture type, the UE should be able to capture the frequency characteristics of the original ambient sound field to some degree.

For testing the send frequency response of ambient sound capture, the UE is placed in the simulated ambient sound field. Similarly, as for loudness, the captured audio signal is encoded and transmitted to the reference client, where the frequency response of the decoded audio signal is analysed against the reference sound field.

### Temporal characteristics

While modern UEs typically apply dynamic signal processing, it might be anticipated that the dynamic changes of the captured ambience should be preserved. E.g., too aggressive dynamic compression may not be desired/acceptable when sound scene comprising large dynamic variations is captured. In addition, encountered extreme temporal changes in terms of signal level could indicate improper behaviour of applied noise suppression processing.

Temporal characteristics of the transmitted ambient sound field could be assessed with a simple level vs time -analysis comparing the captured, transmitted signal from UE with the reference sound field. While some level of dynamic processing may be expected, at least certain similarities in level fluctuation patterns could be expected, i.e., acoustical sound pressure level changes in the ambient sound field should be reflected in the level changes of captured, transmitted signal.

### Spatial characteristics

To obtain accurate representation of the ambient sound field, it is expected that the spatial characteristics of the sound field should be captured and transmitted as accurately as possible. Although the method for assessing a directional information of speech is already described in TS 26.260 [5], a further consideration on how to assess the spatial characteristics of more complex sound fields might be required. Thus, more than one analysis approach may be needed as ambient sound is typically less directional but more diffuse compared to foreground signal components/dominant speech.

### 4.3 Speech and ambience capture tests

In addition to assessing speech-only and ambient-only test scenarios, it is important to assess speech capture in presence of background ambience. While the background ambience is considered as a valuable part of the captured sound field, the quality and speech characteristics should be maintained even when additional sound components are present (other than speech), to ensure robust communication. For assessing the speech characteristics in presence of background ambience, a HATS with artificial mouth could be placed in a simulated ambient sound field, as illustrated in Figure 4. With well-defined acoustical levels of reproduced speech and ambience, the assessed UE should capture the speech signal with a certain speech-to-ambience ratio. Such ratio would naturally depend on the applied signal processing, e.g., the amount of noise suppression. Furthermore, additional metrics for evaluating the captured speech characteristics in presence of ambience could be considered. For example, speech quality and directional accuracy of the speech may be of interest to ensure sufficient capture performance of speech in presence of ambience.

Figure 4: Common test framework for assessing the capture of speech in ambient sound field

# 5. Conclusion

In this document, general considerations on expected UE behaviour in presence of ambience sound field, on ambience sound field simulation and applied test scenes, as well as on potential, envisioned tests for assessing UE performance in varying acoustic conditions are presented. The document aims to initiate a discussion on how UEs are expected to behave in ambient sound fields with and without active speech, and on which test methods are seen to be relevant for the assessment.

In order to further define relevant test methods and requirements for assessing the UE performance in ambient sound fields and complex acoustic scenes, common understanding on how UEs should function in such environments is necessary. We present three different UE capture types, which should assist the work on defining relevant test methods, analysis metrics, and requirements in varying acoustic scenarios.

In addition, initial views regarding the potential sound field simulation methods, setups and test scenarios are shared for discussion. Especially the applicability of two discussed loudspeaker setups for simulating complex acoustic scenes (namely TS 103 224 [2] and surround 7.1(+4) setup) are suggested to be further investigated. Furthermore, for tests including speech and ambience, the work should target defining tests with only foreground user’s speech as an active speech.

Categorization into three different capture test categories is presented, to help identify important aspects on capture performance assessment in different acoustic scenes. Additionally, test methods for evaluating basic capture characteristics in ambient sound fields are presented for discussion.

## 6. References

- 3GPP TS 26.132, “Speech and video telephony terminal acoustic test specification,” Release-18, 2024.

- ETSI TS 103 224, “Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database”, v1.6.1, March 2022.

- ETSI ES 202 396-1, “Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database,” 2022.

- 3GPP TR 26.921, “Investigations on ambient noise reproduction systems for acoustic testing of terminals,” Release-18, 2024.

- 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems,” Release-18, 2024.

- ITU-T P.700, “Calculation of loudness for speech communication,” 06/2021