Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS Characterization - Room Acoustics Test Plan

Document for:	Agreement

Agenda Item:	7.5

# Introduction

The IVAS-8b test plan for the characterization phase [1] still requires providing details regarding room acoustics tests. The sources have initiated discussions on room acoustics testing in their previous contributions [3][4].

This contribution proposes updates to IVAS-8b including room acoustics test methodology, test equipment requirements, allocation of room acoustics experiments, as well as annexes containing listening test instructions and description of IVAS room acoustics synthesis related experiments.

# Methodology and tools

As discussed in the previous contributions, both P.800 and BS.1534 methodologies do not seem suitable for room acoustics testing. Therefore, the sources have proposed applying test methodologies similar to those used for quality testing of MPEG-I Immersive Audio [5]. The proposed test methodology is based on MuSCR (Multiple Stimulus Category Rating). This methodology resembles the MUSHRA test, however without providing either open or hidden reference. The listeners are requested to provide scoring based on quality rating.

For the room acoustics testing, a set of tools was prepared, including:

- Evaluation software package based on Unity gaming engine based on concepts of Audio Evaluation Platform (AEP) [6] as used for MPEG-I quality testing,

- A lightweight Python reference renderer allowing for real-time frame-based audio processing, and using TCP socket communication for pose and audio communication as proposed in [2],

- An updated version of the BinauralSDM tool [7] for generating BRIR data from HRIR and spatially recorded room impulse responses,

- An updated version of MATLAB scripts for room acoustics parameters computation based spatial impulse response recordings.

The evaluation software package has been already introduced in [4]. It supports 3DoF real-time head tracking, communication with audio renderers using TCP sockets, and audio streaming using NAudio library. The tool has been further updated to facilitate output level normalization, control means for lightweight Python renderer, etc.

The Python reference renderer as developed during IVAS codec public collaboration does not support frame-based processing necessary for real-time rendering. Therefore, a lightweight Python reference renderer has been developed featuring frame-based processing and a socket interface, identical to the one used in IVAS decoder/renderer. This renderer runs as a server, responding to packets containing the head rotation sent by the Unity evaluation software, generating subsequent frames of audio samples, and sending them back to Unity application.

As there are no golden reference signals available, the lightweight Python reference rendering is used as a quasi-reference. To reduce the impact of core-coding on quality rating, an IVAS encoded and pass-through decoded audio is used as input to the Python renderer. Both Python reference renderer and IVAS decoder in BINAURAL_ROOM_IR mode use the same BRIRs computed using BinauralSDM from default IVAS HRIR dataset and recorded room impulse responses. IVAS decoder in BINAURAL_ROOM_REVERB mode uses reverb parameters computed from recorded room impulse responses. Furthermore, binaural rendering with default IVAS HRIR is used as an anchor test condition.

The processing paths for the room acoustic synthesis tests are shown in Figure 1.

Figure 1. Processing paths for room acoustics synthesis testing

Preliminary tests were prepared to evaluate the proposed methodology and tools using 3DoF VR environment. These tests are limited to anechoic single-talker audio recordings using multi-channel (5.1) input format. The talker signal appears in the center channel, while the other channels remain silent. The number of tests performed is not yet sufficient for a conclusive presentation. The results will be presented during the meeting. Additional feedback is also gathered from the listeners that might lead to improvement of the tools and refining the process.

# Proposal

The sources propose the following updates to the relevant sections of IVAS-8b document. The formatting applied is intended to facilitate editing of the permanent document.

The test methodology is discussed in section 4.2 of IVAS-8b. It is proposed to modify the opening sentence as follows:

The following primary test methodologies shall be used in the IVAS Characterization test: P.800 [4] and BS.1534 [13]. Additionally, MuSCR methodology (Multiple Stimulus Category Rating) [NN] shall be used for testing room acoustics synthesis. High-level configuration of experiments for envisaged methodologies is outlined below.

The reference [NN] to MuSCR methodology refers to [5]. It is proposed to add clause 4.2.3 on MuSCR methodology as follows:

4.2.3 MuSCR

For room acoustics testing MuSCR (Multiple Stimulus Category Rating) test methodology shall be used. This methodology resembles MUSHRA test, without providing neither open nor hidden reference. The listeners are requested to provide scoring based on quality rating.

  - 4 different rooms

  - 4 audio items per room

  - 4 conditions per test (reference, rendering with BRIR, rendering with reverb, anchor)

  - Total: 64 trials.

Each testing experiment shall be preceded with a training phase in which the listeners should familiarize themselves with the environment and methodology. Unipolar quality scale should be used for quality rating.

Figure 1. Unipolar 100-point quality scale

For clause 4.5 Listening Systems and Listening Environments, it is proposed to add the following bullet points:

  - VR setup for 3DoF binaural room acoustics synthesis listening comprising of:

    - A gaming-category PC running Unity game engine, capable to also run 3 IVAS decoder instances and a lightweight Python renderer in parallel,

    - A VR headset, e.g., HTC Vive Pro or Meta Quest,

    - Headphones requirements for binaural tests apply.

In clause 5.4 Allocation of experiments, it is proposed to update Table 9 with four room acoustics synthesis experiments as follows:

Table 9: Allocation of experiments using different methodologies

| Exp | Objective | Input format | Source material | Listening environment | Bitrates [kb/s] | LL |
| --- | --- | --- | --- | --- | --- | --- |
| ACR-1 | 16, 32, 48 kHz |  |  | Headphones |  | Nokia |
| ROOM-1 | Room acoustics | FOA | Generic audio | Headphones | 80 | Philips |
| ROOM-2 | Room acoustics | FOA | Generic audio | Headphones | 96 | Philips |
| ROOM-3 | Room acoustics | MC – 5.1 | Generic audio | Headphones | 96 | Philips |
| ROOM-4 | Room acoustics | MC – 5.1.2 | Generic audio | Headphones | 128 | Philips |

The proposed allocation triggers all applicable rendering modes of the IVAS decoder, as shown below. It also balances the impact of the core coding by using similar bitrates for all input formats.

| Exp | Input format | Bitrate | Rendering mode |
| --- | --- | --- | --- |
| ROOM-1 | FOA | 80 | Parametric binaural renderer |
| ROOM-2 | FOA | 96 | FastConv binaural renderer |
| ROOM-3 | MC – 5.1 | 96 | Time-domain object renderer / Crend binaural renderer |
| ROOM-4 | MC – 5.1.2 | 128 | Crend binaural renderer |

Annex A: Sample Instructions to Subjects and Data Collection currently only contains instructions for P.800 DCR tests. It is proposed to provide separate sections containing instructions for P.800, BS.1534, and MuSCR tests. The following section on the latter should be considered.

MuSCR room acoustics test instructions

INSTRUCTIONS TO THE LISTENERS FOR THE IVAS ROOM ACOUSTICS SYNTHESIS EVALUATION TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio is an audio experience where you can perceive sound as if coming from the space around you, outside and around your head. Spatial audio can be enriched by synthetic reverberation and acoustic reflections that simulate the acoustics of a room, creating a more natural and immersive experience. You will perform this test while wearing a VR headset. The VR scene used in the test allows for three degrees of freedom (3DoF). That means that you can change your position only by rotation, and not by moving around. Please familiarize yourself with the equipment during the training session and adjust it appropriately. It is highly recommended to perform the tests while sitting on a swivel chair. It is advised not to take this test while standing. Your task is to compare different versions of reverberant spatial audio. More specifically, you must evaluate, on a scale from 0 to 100, the quality of the room acoustics synthesis, and whether it corresponds to the visual virtual environment shown. You can experience an interactive soundfield by rotating your head. You should move your head and verify. The following quality scale is used in the tests. One of the test samples presented in each trial does not contain synthesized room acoustics. Your judgement should at least consider the following factors: - Congruence of the duration and tonality of the reverberance to the presented virtual environment - Presence of unwanted audio artifacts - Spatial impression quality and responsiveness to head-tracking

Annex F and Annex G of IVAS-8b contain description of the P800 and BS.1534 experiments. It is proposed to add Annex H containing conditions of room acoustics testing.

Annex H: MuSCR room acoustics testing experiments

H.1 Experiment ROOM-1: Parametric binaural renderer

Table H.1.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | FOA |
| Bitrate | 80 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference | Lightweight Python renderer with BRIR support |
| Other references |  |
| Anchor | IVAS decoding with anechoic HRIR rendering |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | 100-point unipolar quality scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.1.2 Test conditions for Experiment ROOM-1

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference | 80 |
| c02 | Anchor | 80 |
| c03 | IVAS – BINAURAL_ROOM_IR | 80 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 80 |

H.2 Experiment ROOM-1: FastConv Binaural Renderer

Table H.2.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | FOA |
| Bitrate | 96 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference | Lightweight Python renderer with BRIR support |
| Other references |  |
| Anchor | IVAS decoding with anechoic HRIR rendering |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | 100-point unipolar quality scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.2.2 Test conditions for Experiment ROOM-2

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference | 96 |
| c02 | Anchor | 96 |
| c03 | IVAS – BINAURAL_ROOM_IR | 96 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 96 |

H.3 Experiment ROOM-3: Time-domain object renderer / Crend binaural renderer

Table H.3.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | Multichannel 5.1 |
| Bitrate | 96 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference | Lightweight Python renderer with BRIR support |
| Other references |  |
| Anchor | IVAS decoding with anechoic HRIR rendering |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | 100-point unipolar quality scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.3.2 Test conditions for Experiment ROOM-3

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference | 96 |
| c02 | Anchor | 96 |
| c03 | IVAS – BINAURAL_ROOM_IR | 96 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 96 |

H.4 Experiment ROOM-4: Crend binaural renderer

Table H.4.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | Multichannel 5.1.2 |
| Bitrate | 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference | Lightweight Python renderer with BRIR support |
| Other references |  |
| Anchor | IVAS decoding with anechoic HRIR rendering |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | 100-point unipolar quality scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.4.2 Test conditions for Experiment ROOM-4

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference | 128 |
| c02 | Anchor | 128 |
| c03 | IVAS – BINAURAL_ROOM_IR | 128 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 128 |

# Conclusions

With this contribution the sources propose the updates to IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase filling the gaps regarding IVAS room acoustics synthesis testing. The proposed methodology, based on concepts well known from MPEG-I quality testing, utilizes a VR setup for real-time immersive audio experience. Initial test results are provided. The detailed proposal includes room acoustics test methodology, test equipment requirements, allocation of room acoustics experiments, and related annexes.

# References

- S4-241195 – IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.0.4.0

- S4-240257 – IVAS Characterization Testing – Binaural Rendering, Philips International B.V., Qualcomm Incorporated

- S4-240261 – IVAS Room Acoustics Testing Methodologies, Philips International B.V., Qualcomm Incorporated

- S4-242001 – IVAS Characterization - Room Acoustics Testing, Philips International B.V., Qualcomm

- Thomas Sporer, Sam Jelfs, Jurgen Herre, “Quality testing for AR and VR in MPEG-I immersive audio”, DAGA 2023.
https://pub.dega-akustik.de/DAGA_2023/data/articles/000401.pdf

- MPEG-I Immersive Audio Documentation for the Audio Evaluation Platform (AEP), Version 2 (2021) https://www.mpeg.org/wp-content/uploads/mpeg_meetings/136_OnLine/w20920.zip

- Facebook Research, BinauralSDM GitHub repository: https://github.com/facebookresearch/BinauralSDM

Rendering mode also depends on output format. Time-domain object renderer is used in case of BINARUAL_ROOM_REVERB format, Crend renderer in case of BINAURAL_ROOM_IR format.