3GPP TSG-SA WG4 Meeting #131 	S4-250208

Geneva, Switzerland, 17th February–21st February 2025

Title:	[FS_MediaEnergyGREEN]: Clarifications and additions to Solution #5 on exposure of energy related information

Agenda Item:	8.8

Source:	Samsung Electronics Co Ltd.

Introduction

Document S4aI250043 was agreed during the SA4-e (AH) MBS SWG Post 130 meeting that proposed a solution to KI 1 related to exposure of energy information from the UE, network, and other entities to the application in the UE. Based on this agreed tdoc, a draft is being prepared for review during the upcoming SA4#131 meeting (draft TR 26942-102). This contribution proposes some clarifications to the text proposal in the above draft based on agreed text in parent specification TS 23.501.

Background and motivation

Document S4aI250043 described generic reference architecture for collection and expose of Energy information, and instantiation of the above in 5G Media Streaming architecture and the generalized media delivery architecture. A TR draft 26942-102 is being prepared with the above solution for formal review during the upcoming SA4#131 meeting. This contribution proposes an update to the text proposal in the above draft because of the following:

- Clause 5.51 of parent specification TS 23.501 describes support of energy efficiency and energy savings in 5G System. In sub-clause 5.51.2.3 of this clause in TS 23.501, it is specified that upon receiving energy related information from gNB, OAM, and UPF, the EIF calculates Energy Consumption information of different granularities (UE, PDU Session and/or QoS flow). This information is shared with authorized consumers such as AF/NEF or other 5GC NF (clause 5.51.2.1 of TS 23.501). Because of this specification text, Energy Consumption of different granularities is available with the Energy Information AF, and therefore is able to be shared with the Energy Information Collector, as described in document S4aI250043. The present contribution updates the agreements in S4aI250043 with this additional information.

- The Key Issue #1 description in clause 6.1.1 of TR 26942 on the topic of Energy-related Information exposure proposes that the analysis by this Key Issue should consider 4 problems. One of the problems that is not well addressed so far in the candidate solutions related to this Key Issue is the following:

3. Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way?

There is not much text in the solutions related to this Key Issue in the TR 26942 to describe how the energy information exposed from the network will help the Media Session Handler optimize its media session in an energy-efficient way.

Clause 5.51.4 of TS 23.501 describes Background Data Transfer based on network energy related information and specifies the following:

### 5.51.4 BDT based on network energy related information 5GS supports BDT (background data transfer) policy selection and re-negotiation process by introducing energy related information. The PCF may make policy decisions based on energy related information. The PCF may also trigger the re-negotiation of BDT policy with the AF.

Background Data Transfer using dynamic policy invocation procedures for downlink and uplink in 5G Media Streaming are described in clauses 5.7.8 and 6.9.7 respectively in TS 26.501. Based on high level specification in TS 23.501, the BDT procedures in TS 26.501 may be enhanced as well. So, in current contribution, the following note is proposed to be added (in step-9 of call flow in clause 7.6.3.1) in the generic high-level procedures for collection and exposure of Energy Information:

NOTE 1: Based on the received Energy Information report, the Media Session Handler may perform selection of appropriate Background Data Transfer Policy during the Downlink Background Data Transfer using dynamic policy invocation procedure described in clause 5.7.8 of TS 26.501 [23] and Uplink Background Data Transfer using dynamic policy invocation procedure described in clause 6.9.7 of TS 26.501 [23].

Text proposal

The attached document shows the above proposed clarifications and additions to the draft TR 26942-102.