3GPP TSG-SA WG4 Meeting #131	S4-250210

# Geneva, Switzerland, 17th – 21th February 2025

Source:	Fraunhofer IIS

Title:	[DaCAS] Baseline Target Device

Document for:	Agreement

Agenda Item:	7.6

- Introduction

This document addresses the first objective of the recently approved DaCAS WI (SP-241962 [1]), which is defined as:

  - Definition of a set of target devices or target device types that allow the development of immersive audio capture example solutions and evaluation of their performance. The target devices or target device types are characterized by various properties, including at least the number of microphones and the overall microphone configuration model (detailing, e.g., the microphone placement and the relative distances of the microphones).

The source has analyzed this and came to the conclusion that on the one hand it may be very desirable to have representative target devices or target device types, but that it may also be quite challenging to identify a common target device or target device type. The challenge arises from the fact that the number of smartphone models on the market is quite high and any particular device may just represent one vendor’s design and their microphone placement. This design would then only apply to a single model, where it is likely that microphone placement will change with the next product cycle.

While defining a representative and accessible reference target device may be the optimum solution, simpler solutions of target device types exist that are based on generic microphone array types, such as linear and circular microphone arrays. In the context of this contribution a four-microphone circular array target device is proposed that could serve as a baseline reference platform to define spatial capturing algorithms. This reflects typical smartphone form factors of bar style devices where microphones are placed mostly on a single plane, e.g. at the top and bottom of the phone, potentially complemented by an additional microphone on the back.

Circular microphone arrays can be easily defined and are also commercially available. As an example, the source has used in previous IVAS spatial capturing demos the commercially available “ReSpeaker Mic Array v2.0” [2]. It is a four-mic array with 45.7 mm microphone distance, a mic array size which would fit inside the back plane of the phone or in an accessory for spatial capturing, see Annex A of this Tdoc for further details.

In the experience of the source, such a relatively low-cost, USB-connected and world-wide accessible target device could easily serve as a reference target device for algorithm development and comparison.

- Conclusion

This contribution addresses the first objective of the DaCAS work, definition of target devices or target device types. The source outlined that representative devices based on smartphone microphone placement would be desirable, but it may take too much time to define those and make them accessible to all involved parties. On the other hand, there are solutions on the market based on circular mic arrays that could serve as baseline reference designs.

Therefore, the source proposes to consider such a simple four-mic microphone array, based on the ReSpeaker [2] dimensions, as a baseline target device to be able to conduct the algorithmic DaCAS work.

References:

[1] SP-241962, WID on Diverse audio Capturing System for Ues (DaCAS)

[2] https://wiki.seeedstudio.com/ReSpeaker_Mic_Array_v2.0/

Annex A: Example

The following figure depicts the back of a typical smartphone where the device of [2] would be attached, e.g. using magnetic docking mechanisms. The orange circles indicate the microphone ID, i.e. microphone 1 corresponds to the blue MEMS, microphone, microphone 2 would be the orange MEMS microphone, microphone 3 the cyan MEMS microphone, and microphone 4 the yellow MEMS microphone.

1

2

3

4