3GPP TSG-SA WG4 Meeting #131 	S4-250212

# Geneva, Switzerland, 17th – 21th February 2025

Source:	Fraunhofer IIS

Title:	Proposal on Test Material Selection for IVAS Characterization Testing

Document for:	Agreement

Agenda Item:	7.5

- Introduction

For the IVAS selection phase, the process for subjective test material submission, selection and processing is outlined in the IVAS Testplan [1]. Back then, a significant effort by the “Material Collection entity” has been performed, especially to check, whether all submitted material is conformant to the requirements (both, P.800 and BS.1534 aka MUSHRA tests) and to select a critical subset of the submitted material for the BS.1534 tests. For the characterization phase testing, even more tests are scheduled than during selection phase testing, increasing this effort even more.

This contribution analyses the key-factors impacting the amount of material for the material selection/processing and proposes a way to streamline the process for the characterization testing.

- Impacting Factors on Test Material Selection and Processing

For the IVAS characterization phase, the envisioned testing is sharing a significant number of commonalities, but also some differences with the selection phase testing. In the following, the impact on the test material submission, verification, selection and processing is analyzed.

### Test Methodologies

Each test methodology has its own demands on the test material. This includes aspects such as content, criticality, duration, language, etc. For the test methodologies BS.1534 and P.800 DCR, the requirements on the test material is listed in [1]. The following test methodologies are currently planned for the characterization testing:

- BS.1534: Testing according to BS.1534 was already used during selection phase. The same assumptions with respect to the characteristics of test material and number of conditions/items per experiment as in the selection testing. Thus, no impact on the test material submission, verification and selection is foreseen.

- P.800 DCR: Testing according to P.800 DCR was also used in the selection testing. In general, the same assumption with respect to the characteristics of test material and to number of categories, number of samples/category etc. as in the selection phase is foreseen.

- P.800 ACR: Testing according to P.800 ACR wasn’t used in the selection testing. In general, no different test material is required for P.800 ACR tests compared to P800 DCR. However, it is unclear, whether the ACR testing is limited to clean speech testing or is intended for other material. In addition, possibly aspects such as recording noise or other recording or speaker specific artefacts might need extra attention. Only limited impact on the test material submission, verification and selection is foreseen.

- Testing of room acoustics: This test methodology is still not fully defined. To the source, the impact on the test material is currently unknown.

Overall, the planned test methodologies have only a limited impact on test material selection and processing. For BS.1534 and P.800 DCR tests, in general the same test samples could be re-used – however, especially for P.800 ACR further considerations have to be also taken into account (see below). For P.800 ACR, possibly new test material is needed, taking into account the special needs of an ACR test. On testing of room acoustics, the exact requirements on the test material are unknown.

### Test Laboratories

In the selection phase testing, the testing was carried out in external labs only. The following labs participated in the selection testing:

- Mesaqin.com

- FORCE Technology

- HEAD acoustics GmbH/IKS

- Macquarie University

For the characterization phase, the testing is expected to be carried out in both, external and internal labs. The following labs are expected to participate in the characterization testing:

- Mesaqin.com

- FORCE Technology

- Dolby Laboratories, Inc.

- Ericsson LM

- Fraunhofer IIS

- Huawei Technologies Co Ltd.

- Nokia Corporation

- NTT

- Orange

- Panasonic Holdings Corporation

- Philips International B.V.

- Qualcomm Incorporated

- VoiceAge Corporation

The availability and assignment of test labs to experiments has a direct impact on the test material: For P.800 tests on speech content, the language of the samples has to match the native tongue of the listeners. During selection phase, the test labs have provided monaural sentence pairs to the material collection entity and the host lab, in order to check and process the material. In addition, also stereo mixed/music samples have been provided by the test labs.

If this process is kept, at least the labs which have not participated in the selection phase P.800 testing have to make new material available, which has to get checked by the material collection entity and processed by the host lab. The source sees no problem to re-use the selected BS.1534 test material in different labs.

### Audio Format and Categories under Test

Another factor on the test material is the format and corresponding categories of the material under test. In the selection phase testing, the following audio formats have been tested:

- P.800 DCR:

  - Stereo (clean speech, speech, + background mixed & music)

  - Ambisonics FOA (clean speech, speech + background)

  - Objects: 1 object, 2 objects (clean speech)

  - MASA (clean speech, speech + background)  MASA material was derived from FOA/HOA2 material

- BS.1534 (all generic audio):

  - Stereo

  - Multi-channel 5.1, 7.1+4

  - Ambisonics FOA, HOA2, HOA3

  - Objects: 3 objects, 4 objects

  - MASA  MASA material was derived from FOA/HOA2 material

For the characterization phase testing, testing of the following audio formats is planned:

- P.800 DCR

  - Stereo (all)

  - Ambisonics FOA, HOA2, HOA3 (all)

  - Multi-channel 5.1, 7.1, 5.1+4, 7.1+4 (clean-speech, mixed & music)

  - MASA 1TC, 2TC (all)  MASA material expected to be derived from FOA/HOA2 material

  - OSBA 1-4 objects (all)

  - OMASA 1-4 objects (all)

- BS.1534 (all generic audio)

  - Stereo

  - Ambisonics FOA, HOA2, HOA3

  - Multi-channel 5.1, 7.1, 5.1+2, 5.1+4, 7.1+4

  - Objects: 1-4 objects

  - MASA 1-2 TCs  MASA material expected to be derived from FOA/HOA2 material

  - OSBA 1-4 objects

  - OMASA 1-4 objects

- P.800 ACR

  - Unknown

- Testing of room acoustics

  - Unknown

Following up on the section about test laboratories, it is expected that at least the BS.1534 material (all generic audio) could in general be re-used. In terms of formats, only the following new formats or format configurations are not available from the selection phase testing:

- Multi-channel 7.1, 5.1+2, 5.1+4

- Objects 1-2 objects

- OSBA

- OMASA

Further on, the source proposes to derive the BS.1534 Multi-channel 7.1, 5.1+2, 5.1+4 samples from the already selected BS.1534 7.1+4 samples by removing the superfluous channels and ignore possible small differences in the channel layout for the remaining channels.

- Conclusion

In order to streamline the work of the material collection entity, it is firstly proposed to re-use the BS.1534 selection test samples also for the BS.1534 characterization tests as far as possible, i.e., especially for the following formats:

- Stereo

- Ambisonics FOA, HOA2, HOA3

- Multi-channel 5.1, 7.1, 5.1+2, 5.1+4, 7.1+4 (7.1, 5.1+2, 5.1+4 to be derived from 7.1+4)

- Objects: 1-4 objects

- MASA 1-2 TCs  MASA material expected to be derived from FOA/HOA2 material

In order to facilitate this, it is proposed to ask the material proponents to re-submit the identical material as for the selection phase testing.

For the remaining BS.1534 formats, the P.800 ACR/DCR tests and the testing of room conditions, it is secondly proposed to follow the same process as for the selection phase material selection and processing (performed by the material collection entity and host lab). Since it is expected that a significant amount of new material has to be checked and processed, test material proponents are encouraged to submit material identical to the selection testing if possible (i.e. if format, test methodology and test lab match the setup of the selection phase testing).

References:

[1] 3GPP S4-231115 - IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v1.1.0

[2] 3GPP S4-241195 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v0.4.0