Source:	SA4 Audio SWG Vice-Chair

Title:	3GPP SA4 Audio SWG report from teleconference on ATIAS_Ph2 (10 January  2025)

Document for:	Approval

Agenda item:	5.1

Executive summary

The meeting (20 participants, ~2 hours) covered five input Tdocs on ATIAS_Ph2 on several topics:

- Motion to sound latency (SA4aA250001 - agreed):The proposed test method with a continuous HATS movement is agreed to be included in the ATIAS-2 Pdoc as a candidate test method.

- Capture with moving source (SA4aA250002r->SA4aA250004r->SA4aA250005 - noted): This input extending proposal at SA4#130 was reviewed by the group. In particular, concerns were expressed on loudspeaker array testing in ATIAS and the validity of the VBAP assumption for some spatial audio formats. Offline discussions were invited to progress on this topic.

- Simultaneous UE type (SA4aA250003 - noted):Initial feedback was collected on this input covering complex setups, an updated proposal is expected at the next meeting;

A.I. 1 Audio SWG

A.I. 1.1 Opening of the session and registration of documents

Stéphane opens the meetings at 16:00 CET and displays the Tdoc allocation (see Agenda in Annex A).

Stéphane: Tdoc SA4aA250002 has been revised twice to two late Tdocs (SA4aA250004 then SA4aA250005).

Xuzhou: editorial errors, final proposal and some plot figures, fixed in two documents.

Stéphane: Is it agreeable to consider these late revisions in the meeting?

Answer: yes

Stéphane: The agenda in Annex A is approved with the Tdoc allocation.

A.I. 1.2 IPR and antitrust reminder

Stéphane reads the IPR reminder statement and Antitrust reminder at the beginning of this SWG telco.

"The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

- to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms

"

“The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG/SWG meetings was important to allow for full and fair consideration of such matters.”

Delegates are reminded of the IPR obligations and Antitrust compliance statements are available at

https://www.3gpp.org/about-us/legal-matters/call-for-ipr

and

https://www.3gpp.org/about-us/legal-matters/statement-regarding-competition-law

A.I. 1.3 Reports/Liaison from other groups/meetings

None.

A.I. 1.4 Release 18 and earlier matters

None.

A.I. 1.5 IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2)

None.

A.I. 1.6 ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2)

| SA4aA250001 | Test Method for Motion-to-Sound Latency | HEAD acoustics GmbH |
| --- | --- | --- |

Presenter: Maximilian Kentgens

Discussion:

- Stefan D: also worked on this for 26.261 and 26.260, method done in VRStream, giving few inputs on this method, many head tracking devices do motion prediction, if linear movement, they will catch up and reduce their latency, how does it behave ?

- Max: right, would do intelligent prediction, effect would be that motion to sound latency is not constant in such case, increased error if direction changes, as these tests are designed to give overall experience to user, we say it’s ok to do motion prediction, same as what user experiences, not punishing for this, biggest part of motion to sound latency, not sensor, but other parts like BT or rendering on smartphone, short effects like intelligent processing of sensor has rather lower impact on m2s latency

- Stefan D: if go to figure 4, you are measuring at zero crossing, at middle of linear movement, at edges of curve you would see unpredictable events, movement does not continue at same angular speed, those directly related to user head movements, even more important at zero crossings

- Max: took data from test sensors, overlaid wirth head trajectory, max error was always less than 2 degrees, sensor was really able to follow this precisely, would expect that at edges m2s latency would not increase much more, measurement at starting and stopping times, otherwise artificially threshold, can be more preciser and makes it simpler to determine timestamp when zero crossing, need to measurement 80 msn, if measurement tolerance is 20 ms, not enough

- Stefan D: your head tracking device did not do prediction?

- Max: did some, but really low, reason to use smartphone, assumed smartphone would give reasonable values

- Jan: method is comparable to delay measurement for terminals, why measurement at 0 crossing, ask for quality of rendering and head tracking

- Stefan D: doing this measurement to improve QoE for user, so m2s latency has no impact to user, in our experience, immediate reaction to user movement, if few 100ms lacking behind, users don’t like it, why we had hits some objects, hard stop, more realistic figure, then removing effect of prediction, more reaction time, hitting wall happens immediately, looking at how it reacts when you have an abrupt event may be more sensible

- Max: could do your test method, then other problems, when does audio change, you need some thresholding, if reproducible measurement, abrupt stop may depend on measurement apparatus, if lots of weight, how impulse looks like, to allow different apparatuses and for reproducibility, prefer continuous method, allows for more accurate

- Stefan D: precise when movement has stabilized, not in case when unexpected movement

- Xuzhou: both linear and abrupt movements are worth testing, different in numbers, think we need more reproducible methods and abrupt motion movements, you have a turn table?

- Max: motorized HATS, other possibilities, team in Qualcomm used artificial ears on swing apparatus

- Wuzhou: regardless of what is rotating, rotate for 5 degrees and stop, stepise movement, would give a way to measurement abrupt

- Max: then define how abrupt, hard to define, to do reproducible measurements with different equipments

- Lasse: on orientation path, continuous rotation or oscillating between L and R directions?

- Max: see figure 4, continuous

- Lasse: can this be altered to have turning point closer to zero crossing, to reduce prediction part, could have two different orientation paths?

- Max: reduced starting angle but got larger variance, not able to detect precisely zero crossing anymore, but good to have more impulse like movements

- Stefan D: just ILD, level, not phase analysis

- Max: for simplicity, using short frames, 2 ms, ILD analysis gives us reliable robust results

- Stefan D: other question, using IVAS renderer for this, did measurement shortcutting output from measurement, no device, no wireless, … ?

- Max: problem is you need physical motion, replacing T0?

- Stefan D: instead of using HATS, simulate trajectories, gives zero crossing, then use trajectory instead of motion sensor

- Max: then cannot have a reference because no physical measurement, data not related to physical time, compared motorized HATS to sensor, quite close

- Jan: use simulated head tracking, how real-time system was recorded, not analyzed

- Stefan D: good to understand effect of IVAS, effect is compensated

- Marek: in 3.2 mention using IVAS reference renderer, provide Python wrapper, which reference renderer?

- Jan: IVAS standalone renderer

- Marek: wrapper for decoding, using TCP or UDP? Reusing one from forge?

- Jan: just encoding and decoding in advance, for simplicity, decoded file as source

- Marek: you provide pose

- Jan: UDP over OSC

- Marek: might lead to hickups there, potential improvements, computing ILD with default HRTF sets

- Max: method robust to HRTFs, evaluating 0 degree is well suited for black box testing, ILD is louder for one side than other, true for all HRTFs

- Marek: possible improvement, IVAS support customs HRTF sets, having binary HRTF, very short and simple, for all angles below 0, L channel would have, ones, R would have 0, at moment of zero crossing switch from L to R and vice versa, it would simplify zero crossing detection, would make use custom HRTF support of IVAS

- Max: then cannot do black box, UE might provide one HRTF, manufacturer may decide to have fixed HRTF, if independent lab for testing, must test it as it is

- Arvi: on results, interpret that one measurement, first dot is first zero crossing, etc. experiment was repeated, how reproducible was this test?

- Max: no statistical analysis, only sometimes we had outliers, e.g. 200ms outlier, reason to have median, tried this for different test signals (HOA, 5.1)... quite reliable, always in range of 90 ms

- Arvi: assume refitting between each measurement?

- Max: fitting is compensated by measuring

- Arvi: you obtained different calibration values, 2 dB offset could lead to error

- Max: might experience drift, always do calibration before measurement

- Max: proposing as candidate test method

- Stéphane: this proposal will be integrated in next of the version of the Pdoc in brackets?

- Stefan D: plan to have pseudo code to crosscheck?

- Jan: for analysis?

- Stefan D: to check trajectories, devil in details

Decision: SA4aA250001 is agreed

Jan is tasked to prepare the integration of proposals in the ATIAS-2 Pdoc

| SA4aA250002 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance |
| --- | --- | --- |

Decision: SA4aA250002 is revised to SA4aA250004

| SA4aA250003 | On testing simultaneous SND-UE-types | Nokia |
| --- | --- | --- |

Presenter: Arvi Lintervo

Discussion:

- Jan: thank you for update, good to have complex setup covered, ref point in geometric center of both components, now to headset UE in both cases, correct?

- Arvi: that is correct, received feedback that not easy for test maker to make arbitrary points to measure distance, good to measure distance, might be longer distance, for instance with HHHF far way, other question is rotation of loudspeaker could be rotated, not written here, formulation is different

- Jan: if apply rotation, take HHHF case, if rotate device to evaluate different angles, hand held as center point?

- Arvi: agree, might make more sense

- Max: more relevant, use cases can be that a smartphone captures MASA and headset capture only voice of the user, then center of spatial capture should be smartphone, otherwise make large errors for all DoAs

- Arvi: we don’t know which device analyzes the field

- Max: one idea could be to define a primary device, then it depends on use case, don’t specify what primary device is, in future lots of UEs will emerge, define reference point is primary device, other is auxiliary device

- Arvi: might make sense

- Stéphane: you will resubmit an update

Decision: SA4aA250003 is noted

An update of this proposal is expected at the next meeting.

| SA4aA250004 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance |
| --- | --- | --- |

Decision: SA4aA250004 is revised to SA4aA250005

| SA4aA250005 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance |
| --- | --- | --- |

Presenter: Xuzhou (Mike) Ye

Discussion:

- Jan: interesting document and data, core proposal is to add a loudspeaker testing to 26.260 and 26.261, followed discussion when concluding ATIAS WI and draft Phase 2 WID, loudspeaker array testing is tricky, clause 4 of TS 26.260 was made separate, focus is different, there was discussion that we leave out any loudspeaker testing in send for first version of ATIAS, but will consider for next phase. Not first proposal, idea to capture to test from L to R sound, but also for complex sound scenes, should be aligned. Drawback is that should be carefully defined across labs, should not make it complicated, also lots of discussion on free-field. One technical comment: suggest HRTFs of HATS for binaural recording, might be able to configure HRTFs in UE, should assume black-box testing for UEs, has impact on performance and so, this cannot work for IVAS UE testing

- Xuzhou: can understand that cannot set HRTFs, for loudspeaker expect cannot have loudspeaker array?

- Jan: there is clause 4 that existed before ATIAS, test description of clause 4 were in specification, there were pros and cons, ATIAS is for commercial device testing, things in clause 4 are oversized or not completely specified, tried to refer to clause 4, just gentleman agreement in meeting, but can define a simple one for complex sound scenes

- Xuzhou; would be nice, interested in suggestion defining one loudspeaker for more general sound field synthesis, to use it for more scenarios, any plan on that?

- Jan: it is on the list of WI objectives, can check at next meeting, can submit considerations for that

- Stéphane: details legacy method ?

- Xuzhou: old one

- Stéphane: proposing inclusion in Pdoc?

- Xuzhou: will check offline with Jan on loudspeaker array

- Max: proposing using arc

- Stéphane: two methods

- Xuzhou: having br,

- Tomas: for OBA, mention difference for reference signal and candidate signal, how do you get the

- Nien Wu: in proposal, virtual sound source to produce moving souce, multi speaker, for human hearing, could be two real sound source, could be problem for algorithm, if real source

- Xuzhou: test method if multiple sound source

- Nien Wu: concern with virtual sound source, so two sounds in environments captured by UE

- Xuzhou: concerned that VBAP could not simulate a moving sound source good enough?

- Nien Wu: this is issue differs from what format you want to record, all coincidence assumption for SBA, when UE in geometric center of arc speakers, distance between loudspeaker and UE will not change, in this case sound field should be equivalent to a moving sound source, for binaural audio and for object audio, this might be an issue, for binaural audio suggest recording by HATS, if anything different in virtually planned, this would be cancelled, Isaac from Huawei has pointed out this issue

- Nien Wu: for ambisionic microphones, same place, for UEs, the distance between microphones is larger, when encoding ambisonic, assumption of coincide assumption is always there.

- Stéphane: will have to discuss how loudspeaker array testing will be considered, also discuss further the assumption that VBAP is valid

Decision: SA4aA250005 is noted

Offline discussions are invited to progress on this topic.

A.I. 1.7 DaCAS (Diverse audio CApturing system for Smartphone devices)

None.

A.I. 1.8 FS_ACAPI (Study on Audio Codec APIs)

None.

A.I. 1.9 Others Rel-19 matters including TEI

None.

A.I. 1.10 Close of the session

Stéphane thanked all delegates for participating, and he thanked HEAD acoustics for hosting the meeting.

Stéphane: Next meetings for the Audio SWG are scheduled on Jan 17 and Feb. 15 (latter one is joint with RTC).

The meeting was closed at 17:50 CET.

Annex A – Meeting agenda

Source:	SA4 Audio SWG Vice-Chair

Title:	Proposed agenda for Audio SWG teleconference on ATIAS_Ph2 (10 January  2025)

Document for:	Approval

Agenda item:	1.1

Agenda for this telco (keeping only relevant items from the unique agenda for 3GPP SA4 AH telcos post-130):

| 1 | Audio SWG |  |
| --- | --- | --- |
| 1.1 | Opening of the session and registration of documents |  |
| 1.2 | IPR and antitrust reminder |  |
| 1.3 | Reports/Liaisons from other groups/meetings |  |
| 1.4 | Release 18 and earlier matters |  |
| 1.5 | IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2) |  |
| 1.6 | ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2) | SA4aA250001a (HEAD acoustics, motion-to-sound latency) SA4aA250002r->SA4aA250004r->SA4aA250005n (Bytedance, capture with moving source) SA4aA250003n (Nokia, simultaneous SND-UE types) |
| 1.7 | DaCAS (Diverse audio CApturing system for Smartphone devices) |  |
| 1.8 | FS_ACAPI (Study on Audio Codec APIs) |  |
| 1.9 | Other Rel-19 matters including TEI |  |
| 1.10 | Close of the session |  |

Legend for Tdocs:

- Color: not-yet processed, processed, late, withdrawn, moved to a different A.I., under email agreement

- a agreed, app approved, n noted, pa partially agreed, np not pursued, pp postponed…

Annex B – List of participants (20)

Bytedance - June Sun

Bytedance - Xuzhou (Mike) Ye

CMCC - Jaiyi

Ericsson – Tomas Toftgard

Fraunhofer IIS – Stefan Döhla

Fraunhofer IIS – Markus Multrus

HEAD acoustics - Alexander Hallmann

HEAD acoustics - Maximilian Rentgens

HEAD acoustics – Jan Reimes

Huawei – Huan-Yu Su

Nokia - Lasse Laaksonen

Nokia – Arvi Lintervo

Nokia – Anssi Rämö

Orange – Stéphane Ragot

Panasonic – Hiroyuki Ehara

Panasonic – Srikanth Nagisetty

Philips – Marek Szczerba

VoiceAge – Milan Jelinek

Xiaomi – Nien Wu

Xiaomi – Wang Bin

Annex C - Documents status

| Tdoc | Title | Source(s) | Agenda Item(s) | Status |
| --- | --- | --- | --- | --- |
| SA4aA250001 | Test Method for Motion-to-Sound Latency | HEAD acoustics GmbH | 1.6 | Agreed |
| SA4aA250002 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance | 1.6 | Revised |
| SA4aA250003 | On testing simultaneous SND-UE-types | Nokia | 1.6 | Noted |
| SA4aA250004 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance | 1.6 | Revised |
| SA4aA250005 | Objective Test Methods for Capturing Immersive Audio with Moving Sound Source | Bytedance | 1.6 | Noted |

Mr. Stéphane Ragot, Orange

stephane [dot] ragot [at] orange [dot] com

M: +33 6 76 63 09 23

Mr. Stéphane Ragot, Orange

stephane [dot] ragot [at] orange [dot] com

M: +33 6 76 63 09 23