3GPP TSG-SA WG4 Meeting #131	S4-250217

# Geneva, Switzerland, 17th – 21th February 2025

Source:	Fraunhofer IIS

Title:	[IVAS_Codec_Ph2] Delay and Error Profile for Characterization Testing

Document for:	Agreement

Agenda Item:	7.5

- Introduction

This document is a follow-up to S4-230604 [1], where JBM processing for IVAS was introduced into IVAS processing. Due to limited test capacity in IVAS selection, but also because the question “are the existing JBM delay and error profiles relevant for 5G/IVAS” was not answered, such tests were not conducted.

Given that the JBM functionality is a key component to enable use of IVAS over jittery packet-based networks (such as MTSI) and all processing is already defined, it is desirable to characterize this functionality, despite the lack of a final answer on the most relevant delay and error profile for IVAS/5G.

The existing delay and error profiles were collected during early 3G days but also from real-world LTE environments. Generally, packet-based networks suffer from the same issues, they however differ in their likelihood for buffer build-up, packet loss, and end-to-end delay. Therefore, having an as high variety of delay and error profile characteristics would be most beneficial for verifying JBM solutions.

Since test capacity is also limited in IVAS characterization, the source created a single profile that contains many impairments from the existing profiles, collecting the characteristic parts of each profile.

- Profile creation script

The following shell script uses the existing delay and error profiles from EVS-7c [2] and extracts the most representative 1000 samples of each profile. Note that profile 1 and 5 were not considered: Profile 1 is an error-free low-jitter scenario which has shown to not exercise JBMs at all; Profile 5 is a high-error cases for bundled operation, bundled operation is however not used in any other profile and this profile is thus also excluded.

#!/bin/sh

#

# simple bash script to create a medley of several delay and error profile

# MTSI profile 1 has been added to all other profiles to increase jitter slightly

# total output length is 8000 packets (as profiles 7-10 from EVS-7c)

#

OFFSET="150"

sed -n "1,+999p" dly_error_profile_7.dat | paste - p1_1000.dat |\
   awk -v o=$OFFSET '{if ($1>-1)print $1+$2-o;else print $1;}'

sed -n "30,+999p" dly_error_profile_8.dat | paste - p1_1000.dat |\
   awk -v o=$OFFSET '{if ($1>-1)print $1+$2-o;else print $1;}'

sed -n "1,+999p" dly_error_profile_9.dat |paste - p1_1000.dat |\

awk -v o=$OFFSET '{if ($1>-1)print $1+$2-25-o;else print $1;}'

sed -n "17,+999p" dly_error_profile_10.dat | paste - p1_1000.dat |\

awk -v o=$OFFSET '{if ($1>-1)print $1+$2-50-o;else print $1;}'

sed -n "1,+999p" dly_error_profile_2.dat | paste - p1_1000.dat |\
   awk -v o=$OFFSET '{if ($1>-1)print $1+$2-o;else print $1;}'

sed -n "1,+999p" dly_error_profile_3.dat | paste - p1_1000.dat |\

awk -v o=$OFFSET '{if ($1>-1)print $1+$2-o;else print $1;}'

sed -n "4500,+999p" dly_error_profile_4.dat | paste - p1_1000.dat |\
   awk -v o=$OFFSET '{if ($1>-1)print $1+$2-32-o;else print $1;}'

sed -n "1,+999p" dly_error_profile_6.dat | paste - p1_1000.dat |\
   awk -v o=$OFFSET '{if ($1>-1)print $1+$2-o;else print $1;}'

- Characteristics of the new profile “I1”

The following figure illustrates the delay and packet losses over time. Note each block of 1000 samples that corresponds to a characteristic portion of the original input delay and error profiles.

- Conclusion

This contribution addresses the selection of a delay and error profile that is used for IVAS testing. The previously used profiles seem partially outdated, they however have proven to work well in the MTSI context (where no new profiles have ever been added for objective JBM testing).

The source proposes to agree on the delay and error profile and use this for the IVAS characterization tests.

References:

[1] S4-230258, IVAS Testing with delay and error profiles, Fraunhofer IIS

[2] S4‑141392, EVS Permanent Document EVS-7c: processing functions for characterization phase, v1.1.0