3GPP TSG-SA4 Meeting #131	S4-250225

# Geneva, , 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0010 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the IVAS detailed algorithmic description (TS 26.253) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Upon review of the specification certain ambiguities, errors, and unclear portions of the algorithmic description were identified concerning mainly the IVAS encoder (clause 5) but also the IVAS decoder (clause 6). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct: - error in azimuth quantization of spatial metadata, - ambiguities and improve clarity for TD-based stereo encoder, - ambiguities and improve clarity for DFT-based stereo encoder, - ambiguities and improve clarity for LSF parameters for primary and secondary channels in TD-based stereo decoder In addition, there are several editorial corrections for encoder and renderer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities and errors in algorithmic description will remain, which can lead to incorrect implementations. Readability of the specification will suffer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.3.2.2, 5.2.4.5.1.5, 5.3.2.3.1.2.8, 5.3.2.3.2, 5.3.2.4.4.1, 5.3.2.4.4.2, 5.3.2.4.4.3, 5.3.2.4.4.4, 5.3.2.4.4.5, 5.3.2.4.7.2, 5.3.2.4.12.2.1, 5.3.2.4.12.2.3, 5.3.2.4.12.2.4, 5.3.2.4.13, 6.3.2.2.3, 7.2.2.1, 7.4.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

5.2.4.3.2.2	Azimuth quantization

The azimuth codebook is uniform, and the number of points is given by the nϕcorresponding corresponding to the quantized elevation value. The azimuth is differently quantized if it corresponds to a direction from MASA input format or to a direction from multichannel input format in McMASA representation. Both variants are presented in this clause.

If the input format is MASA, the azimuth points for two consecutive circles in the spherical grid are shifted by the value shift such that for zero valued elevation the first azimuth codeword has value zero. For the following elevation value, the first azimuth codeword has value equal to half of the corresponding quantization step: Δϕidθ 2=  3602 nϕ[idθ]. If the elevation values are numbered from 0, the first azimuth point on circles corresponding to even numbered elevation values are zero, while those corresponding to odd elevation values are Δϕidθ 2.  The shifting is considered only when the number of azimuth points on the horizontal circles on the sphere is larger than 2. The value shift is thus defined by:

shift= 0, if idθ is oddΔϕidθ 2, if idθ is even

(5.2-265)

The azimuth quantization for MASA input format is performed as shown in the following equation:

idϕ= ϕ-180-shiftΔϕidθ, 0≤ϕ<360 	(5.2-266)

For obtaining the quantized value ϕ and the final quantized azimuth index, the index  idϕ goes through the following verifications that take into account that the azimuth values are cyclic:

if ( idϕ + (nϕ[idθ] >> 1) < 0 ) idϕ =  idϕ + 1;

if ( idϕ  - (nϕ[idθ]  >> 1) >= 0 ) idϕ = idϕ -( nϕ[idθ]  >> 1 );

In addition, the quantized azimuth index is transformed with the following pseudo code, such that it has positive values for directions pointing to the left side and negative values for directions pointing to the right side. The frontal direction has value zero.

if ( idϕ  == -((nϕ[idθ]  >> 1) + ( nϕ[idθ]  % 2 )))

{

idϕ  = idϕ + (nϕ[idθ]  % 2);

}

else

{

if ( idϕ  == ((nϕ[idθ]  >> 1) + (nϕ[idθ]  % 2)))

{

if (nϕ[idθ]  % 2)

{

idϕ = idϕ - 1;

}

else

{

idϕ = idϕ - idϕ;

}

}

}

The quantized azimuth value is  ϕ= idϕ*Δϕidθ . The domain of the quantized azimuth is consequently , -180≤ϕ<180 .  After the value of the quantized azimuth is calculated, an additional transformation is performed on the quantized azimuth index ordering the quantized values such that the first index is assigned to the zero quantized azimuth value followed by alternating positive and negative azimuth values.

If the input format is multichannel and the encoding is done through the McMASA representation (described in 5.7.3), the azimuth quantization is preceded by a companding function. The companding function definition depends on the multichannel input format type and on the quantized elevation value being larger than a threshold or not. The companding function is defined for the input domain [-180, 180) and is specified below:

CHANGE 2

Editorial

5.2.4.5.1.5	Decision for fully raw coding or entropy-coding

Once the optimal code is found for the entropy coded elevation and azimuth quantization indices, the sum of the bits used for the entropy encoded part and the bits used for the raw encoding part (for the indices of the quantized spatial audio parameters) is then calculated. This sum is then compared to the number of bits used when raw encoding is used to encode the quantization indices of the spatial audio parameters associated with all TF tiles in the frame. The method using the less bits is retained and signalled by the bit flags in table 5.2-38:

CHANGE 3

5.3.2.3.1.2.8	Delay estimation

In TD stereo coding, ITD is estimated by a cross-correlation coefficient of the current frame, a delay track estimation value of the current frame based on buffered ITD information of at least one past frame, and an adaptive window function of the current frame. Specifically, the ITD is calculated based on a weighted cross-correlation coefficient which is obtained by the delay track estimation value and the adaptive window function.

CHANGE 4

##### 5.3.2.3.2	TD stereo coder overview

The TD stereo coder is a low-rate low-complexity parametric stereo technology where the stereo input signal is encoded in time domain. It is especially effective for scenes where the correlation between the left and the right channel of the input audio signal is low, when the background noise is fluctuating, and when an interfering talker is present. The left channel and the right channel of the input audio signal are obtained after preprocessing (e.g., high-pass filtering) and inter channel alignment, described in 5.3.2.3.1. The classifier of uncorrelated input signals and the cross-talk detector, described in 5.3.2.5, are used to select frames suitable for the TD stereo coder. The TD stereo coder comprises a method for combining the left and the right channels of the input stereo signal to obtain the primary and the secondary channels based on a mixing ratio. The bitrate is adaptively distributed between the primary and the secondary channels to maximize the quality of the encoded stereo signal. The secondary channel usually requires a lower number of bits due to leveraging some parameters from the primary channel.

CHANGE 5

Editorial correction to harmonize ToC for 5.3.2.4.4.1 – 5.3.2.4.4.5:

H6 -> Heading 5

##### 5.3.2.4.4.1	Description of GCC-PHAT implementation

CHANGE 6

Editorial correction to harmonize ToC for 5.3.2.4.4.1 – 5.3.2.4.4.5:

H6 -> Heading 5

##### 5.3.2.4.4.2	DTX hangover adaptation

CHANGE 7

Editorial correction to harmonize ToC for 5.3.2.4.4.1 – 5.3.2.4.4.5:

H6 -> Heading 5

##### 5.3.2.4.4.3	Peak picking algorithm for ITD estimation

CHANGE 8

Editorial correction to harmonize ToC for 5.3.2.4.4.1 – 5.3.2.4.4.5:

H6 -> Heading 5

##### 5.3.2.4.4.4	ITD stabilization hangover

CHANGE 9

Editorial correction to harmonize ToC for 5.3.2.4.4.1 – 5.3.2.4.4.5:

Incorrect Heading 6 -> Heading 5

##### 5.3.2.4.4.5 	Refined ITD control mechanism

The ITD is critical to keep the stereo image stable. Sometimes ITD values calculated by the frequency domain correlation are not continuous. A difference parameter is used to represent a difference between the ITD of the current frame ITDm and the ITD of the previous frame ITDm-1. The refined ITD is determined based on the difference parameter and a characteristic parameter of the current frame. The characteristic parameter is calculated as follows.

Divide a low frequency part of the left-channel frequency-domain signal of the current frame into M sub-bands, where each sub-band includes N frequency domain amplitude values. Calculate a correlation parameter which represents the degree of correlation of the current frame and a previous frame according to

corii=j=0N-1L(i*N+j)∙L(-1)(i*N+j)j=0N-1L(i*N+j)∙L(i*N+j)∙j=0N-1L(-1)(i*N+j)∙L(-1)(i*N+j)

i=0,1,…,M-1, where L(i*N+j) represents a jth frequency domain amplitude value of an ith sub-band in the low frequency part of the left-channel frequency-domain signal of the current frame, L(-1)(i*N+j)represents a jth frequency domain amplitude value of an ith sub-band in a low frequency part of a left-channel frequency-domain signal of the previous frame, and corirepresents a normalized cross-correlation value corresponding to an ith sub-band in the M sub-bands.

Calculate a peak-to-average ratio of each sub-band of the current frame par_Li.

If the ITD value of the current frame and an ITD value of the previous frame meet at least of one of the preset conditions, determine whether to reuse the ITD value of the previous frame for the current frame. The preset conditions set to

-	condition_1: the absolute ITD value of the previous frame is greater than the absolute ITD value of the current frame,

condition_1 = | ITDm-1| > 0.2 * | ITDm|

-	condition_2: the average value of the normalized cross-correlation values of the sub-bands is greater than 0.85,

condition_2 = avrg(cori) > 0.85

-	condition_3: the average value of the normalized cross-correlation values of the sub-bands is greater than 0.7, and a normalized cross-correlation value of a sub-band is greater than 0.9,

condition_3 = avrg(cori) > 0.7 and (cor0 > 0.9 or cor1 > 0.9 or cor2 > 0.9)

-	condition_4: the average value of the peak-to-average ratios of the sub-bands is greater than 0.6,

condition_4 = avrg(par_Li) > 0.6

-	condition_5: the ITD value of the previous frame is not equal to 0,

condition_5 = ITDm-1≠ 0

-	condition_6_a: a product of the ITD value of the previous frame and the ITD value of the current frame is negative,

condition_6_a = itd * prev_itd < 0

-	condition_6_b: a product of the ITD value of the previous frame and the ITD value of the current frame is 0,

condition_6_b = itd * prev_itd = 0

-	condition_6_c: an absolute value of a difference between the ITD value of the previous frame and the ITD value of the current frame is greater than half of a target value, where the target value is an ITD value whose absolute value is larger in the ITD value of the previous frame and the ITD value of the current frame,

condition_6_c=itd, |itd - prev_itd| > 0.5 * (|itd| > |prev_itd|)previtd,itd - previtd≤ 0.5 * (|itd| > |prev_itd|)

condition_6_c=itd, |itd - prev_itd| > 0.5 * max(itd, |prev_itd|)previtd,itd - previtd≤ 0.5 * max(itd, |prev_itd|)

The ITD fine control result itd_fine sets to

itd_fine=previtd, conditiontotal is true)itd,others

where

conditiontotal=(condition_1234 and ((condition_5 and condition_6_b) or condition_6_c)) or (condition_1234 and condition_6_a), condition_1234 = condition_1 and (condition_2 or condition_3 or condition_4)

when the signal-to-noise ratio meets the signal-to-noise ratio condition, stopping reusing the ITD value of the previous frame as the ITD value of the current frame. The signal-to-noise ratio condition is set as the SNR value is less than 0.006 or the SNR value is greater than 2 000 000.

In addition, the ITD of the current frame could reuse the ITD of the previous frame ITD(m-1) based on the hangover counter and the initial ITD of the current frame The hangover counter is the quantity of target frames that are allowed to appear consecutively. A characteristic information which is described by the signal-to-noise ratio and the peak feature of cross correlation coefficients of the stereo signal is used to control the hangover counter. When the signal-to-noise ratio and the peak feature of the cross-correlation coefficients meet at least one of preset conditions, the hangover counter will be reduced by adjusting at least one of a target frame count representing the quantity of target frame that appears consecutively and a threshold of the target frame count which allows the target frame to appear consecutively. And, the hangover counter will be reduced by increasing the target frame count or reducing the threshold of the target frame count.

CHANGE 10

5.3.2.4.7.2	IPD encoding scheme

In addition, a reference parameter is used to determine the IPD parameter encoding scheme. The reference parameter includes at least one of the signal characteristic parameter and the signal characteristic parameters of previous frame. The signal characteristic parameter comprises the correlation between left channel and right channel of the current frame. The signal characteristic parameters of previous frame include at least one of the correlations between left channel and right channel of previous frame, an ITD parameter of previous frame, a signal type of previous frame.

CHANGE 11

5.3.2.4.12.2.1	Adaptive residual signal encoding parameter

For the 32 kbps WB coding mode, an adaptive residual signal encoding parameter is used to determine whether to encode the residual signals of the M sub-bands in the current frame. The adaptive residual signal encoding parameter res_dmx_ratio_lt is calculated based on downmixed signal energy and residual signal energy of each of M sub-bands in the current frame, wherein spectral coefficients of the current frame are divided to obtain N sub-bands, the M sub-bands are at least some of the N sub-bands, N is a positive integer greater than 1, M ≤ N, and M is a positive integer. The parameter res_dmx_ratio_lt of the current frame is specifically determined based on the dmx_res_all and frame_nrg_ratio. The res_cod_mode_flag is calculated according to

res_cod_mode_flag=1,res_dmx_ratio_lt > 0.01 0,else

when res_cod_mode_flag is equal to 1 means encode the residual signals of the M subbands, otherwise do not encode the residual signals of the M subbands. The res_dmx_ratio is a parameter describing relationship between the downmixed signal energy and the residual signal energy of each of the M sub-bands, the res_dmx_ratio is the energy parameter with a largest value in the M energy parameters. The res_dmx_ratio_lt is a parameter describing a long-term smoothing parameter of the current frame. The long-term smoothing parameter (long-term smoothing residual signal encoding parameter) res_dmx_ratio_lt is calculated according to

res_dmx_ratio_lt=res_dmx_ratio* + res_dmx_ratio_lt_prev * 1-

wherein res_dmx_ratio_lt_prev represents the long-term smoothing parameter of the previous frame of the current frame, wherein

α=0.2,  frame_nrg_ratio> 3.2  and  res_dmx_ratio < 0.1)  or  (frame_nrg_ratio< 0.21 and res_dmx_ratio > 0.4) 0.05,       else

The res_dmx_ratio is calculated according to

res_dmx_ratio[b] = res_cod_NRG_S[b]/(res_cod_NRG_S[b] + (1 – g(b))∙(1 – g(b)) res_cod_NRG_M[b] + 1)

wherein res_dmx_ratio[b] represents the energy parameter of the sub-band whose sub-band index number is b, b=0,1,2,…M-1, b is greater than or equal to 0 and is less than or equal to a  preset maximum sub-band index number, res_cod_NRG_S[b] represents residual signal energy of the sub-band whose sub-band index number is b, res_cod_NRG_M[b] represents downmixed signal energy of the sub-band whose sub-band index number is b, and g(b) represents a function of a side gain side_gain[b] of the sub-band whose sub-band index number is b. The frame_nrg_ratio is a parameter described relationship between a sum of residual signal energy and downmixed signal energy of the M sub-bands, and a sum of residual signal energy and downmixed signal energy of M sub-bands in a frequency-domain signal of a previous frame of the current frame.

CHANGE 12

5.3.2.4.12.2.3	Calculation of downmixed signal and residual signal during transitional frame

In stereo coding, obtaining an initial downmixed signal and an initial residual signal of a sub-band corresponding to a preset frequency band in a current frame. If the previous/current frame is a switching frame, the downmixed signal and the residual signal of the sub-band corresponding to the preset frequency band in the current frame are calculated based on a switch fade-in/fade-out factor of a current/previous frame, the initial downmixed signal, and the initial residual signal. The switch fade-in/fade-out factor of the current/previous frame is determined based on a residual signal coding parameter and an inter-frame energy fluctuation parameter. The residual signal coding parameter is used to represent an energy relationship between a downmixed signal and a residual signal of the current/previous frame, and the inter-frame energy fluctuation parameter is used to represent an energy or amplitude relationship between the current/previous frame and a frame previous to the current/previous frame. The switch fade-in/fade-out factor of the current/previous frame is determined according to

when frame_nrg_ratio>3.2 and res_dmx_ratio<0.1, switch_fade_factor=0.75; when frame_nrg_ratio<0.21 and res_dmx_ratio>0.4, switch_fade_factor=0.25;  in another case, switch_fade_factor=0.5; wherein

frame_nrg_ratio represents the inter-frame energy fluctuation parameter of the current frame/previous which is defined as a ratio of total energy of the downmixed signal and the residual signal of the current/previous frame to total energy of a downmixed signal of a frame previous to the current/previous frame and a residual signal of the frame previous to the current/previous frame; res_dmx_ratio represents the residual signal coding parameter of the current frame; switch_fade_factor represents the switch fade-in/fade-out factor of the current/previous frame.

The initial downmixed signal and the initial residual signal are calculated according to

DMXi,b(k)=DMXi,b(k)+(1-switch_fade_factor)*DMX_compi,b(k)

RESi,b(k)=switch_fade_factor*RESi,b'(k)

wherein DMXi,b(k) represents downmixed signal of a sub-band b in a subframe i in the current frame; DMXi,b(k) represents an initial downmixed signal of the sub-band b in the subframe i in the current frame; DMX_compi,b(k) represents a compensated downmixed signal of the sub-band b in the subframe i in the current frame; RESi,b'(k) represents an initial residual signal of the sub-band b in the subframe i in the current frame; RESi,b(k) represents residual signal of the sub-band b in the subframe i in the current frame; the sub-band b in the subframe i in the current frame is a sub-band in the at least one sub-band corresponding to the preset frequency band; k represents a frequency bin index of the sub-band b in the subframe i in the current frame; and 0≤i≤P-1, wherein P represents a quantity of subframes comprised in the current frame.

when residual coding flag is unequal to residual coding flag value of previous frame, and a modification flag of the residual coding flag of the previous frame is 0 which indicates that the residual coding flag value of the previous frame has not been modified, the residual coding switching flag set to 1 which indicates the frame is a switching frame and the residual coding flag set to 1 indicates the residual signal should be encoded.

CHANGE 13

5.3.2.4.12.2.4 	Adaptive downmix

The encoding mode indication information of the residual signal of the current frame is obtained by at least one of the following information: the residual signal encoding status of the previous frame, the updating manner flag for a long-term smooth parameter of the current frame, and the status change parameter relative to the previous frame. The encoding status of the previous frame is used to indicate at least one of the following cases: the quantity of consecutive frames whose residual signals are encoded before the current frame, the quantity of consecutive frames whose residual signals are not encoded before the current frame, and the encoding modes of residual signals of the previous frame. The status change parameter is a ratio of the energy of the current frame to the energy of the previous frame. The encoding mode used to indicate whether to encode the residual signal of the current frame is determined based on the encoding mode indication information and the initial encoding mode of the residual signal of the current frame. The initial encoding mode of the residual signal of the current frame is determined based on the downmixed signal energy and the residual signal energy of the current frame.

If the following conditions are met, the encoding mode of the residual signal of the current frame is the encoding mode of the residual signal of the previous frame. The conditions include the initial encoding mode that is different from the encoding mode of the previous frame which is closely adjacent in time to the current frame, and the encoding mode of the previous frame that indicates to encode the residual signal of the previous frame, and an additional condition. The additional condition is the quantity of consecutive frames whose residual signals are encoded before the current frame is less than a threshold or the updating manner flag for the long-term smooth parameter is 0, and the encoding mode of the residual signal of the previous frame is not modified. If the additional condition is not met, the encoding mode of the current frame is the initial encoding mode. If the initial encoding mode is the same as the encoding mode of the previous frame, the encoding mode of the residual signal of the current frame is the initial encoding mode.

If the following conditions are met, the encoding mode of the residual signal of the current frame is the encoding mode of the residual signal of the previous frame. The conditions include the initial encoding mode that is different from the encoding mode of the previous frame which is closely adjacent in time to the current frame, and the encoding mode of the previous frame that indicates not to encode the residual signal of the previous frame, and a second additional condition. The second additional condition is the quantity of consecutive frames whose residual signals are not encoded before the current frame is less than a threshold or the value of the status change parameter is not less than a second threshold, and not greater than a third threshold. If the second condition is not met, the encoding mode of the current frame is the initial encoding mode.

In addition, the encoding mode of the residual signal of the current frame can be modified based on the encoding mode indication information of the residual signal of the current frame. If the encoding mode of the residual signal of the current frame is different from the encoding mode of previous frame which is closely adjacent in time to the current frame, and the encoding mode of previous frame is not modified, the encoding mode of residual signal of the current frame is used to indicate the encoding mode of the residual signal of the current frame.

CHANGE 14

##### 5.3.2.4.13	Reverberation gain parameter determination

The left channel signal and the right channel signals are treated as the first channel signal and the second channel signal. Encoder quantizes the downmixed signal, the target reverberation gain parameter, and the identification information which enables reconstruction of the first channel signal and the second channel signal, and writes the quantized parameters into the bitstream.

The reverberation gain parameters correspond to different sub-bands of the first channel signal and the second channel signal. The target reverberation gain parameter indicates those reverberation gain parameters that needs to be encoded. The target reverberation gain parameter is determined based on at least one of coherence between energy of the first channel signal and energy of the downmixed signal and coherence between energy of the second channel signal and the energy of the downmixed signal, wherein each of the first channel signal and the second channel signal comprises a plurality of frequency bins. The identification information is used to indicate a sub-band corresponding to the target reverberation gain parameter and whether the initial reverberation gain parameter needs to be adjusted. The identification information is determined based on the target difference value which is the larger difference value in the first difference value and the second difference value. The identification information uses 1bit to indicate the first frequency band. A target attenuation factor used to adjust initial reverberation gain parameter of a target channel signal is calculated based on the first difference value and the second difference value. Each of the plurality of attenuation factors corresponds to at least one sub-band of the target channel signal, and any sub-band corresponds to only one attenuation factor. The target attenuation factor is quantized and then written into the bitstream. The first difference value is a sum of absolute values of difference values between energy of the first channel signal and energy of the downmixed signal at a plurality of frequency bins, and the second difference value is a sum of absolute values of difference values between energy of the second channel signal and energy of the downmixed signal at the plurality of frequency bins. When the first difference value or the second difference value is greater than 120, the reverberation gain parameter corresponding to a sub-band of a first frequency band is the target reverberation gain parameter, wherein the first frequency band is a part of all frequency bands of each of the first channel signal and the second channel signal, wherein a frequency of the first frequency band is less than a frequency of another frequency band different from the first frequency band in the first channel signal and the second channel signal. The plurality of frequency bins is in a second frequency band of each of the first channel signal and the second channel signal, and a frequency of the second frequency band is greater than a frequency of another frequency band, different from the second frequency band, in the first channel signal and the second channel signal.

CHANGE 15

##### 6.3.2.2.3	Reusing LP filter coefficients of the primary channel and dequantization of LSF parameters for the secondary channel

When the coherence between the coefficients of the LP filter of the primary channel and the coefficients of the LP filter of the secondary channel is high enough, as described in clause 5.3.3.3.4.3, the LP filter coefficients of the primary channel may be reused inside the decoder of the secondary channel. In such case one of the following scenarios occurs:

-	the quantized LSP vector of the primary channel from the previous frame is used to calculate the LP filter coefficients of the secondary channel in the current frame

-	the quantized LSP vector of the primary channel from the previous frame is resampled from the internal sampling frequency of 16 kHz down to 12.8 kHz and then used to calculate the LP filter coefficients of the secondary channel in the current frame

-	in case the coder type of the secondary channel is INACTIVE the LP filter coefficients of the primary channel in the current frame are re-used in the secondary channel

-	same as in the previous case and when the coherence between the LP coefficients of the primary and the secondary channel is low, then a pre-defined default (average) LSP vector is used to calculate the LP filter coefficients of the secondary channel in the current frame

In scenarios involving LSP vectors the conversion of LSP coefficients to LP filter coefficients is done using the procedure described in clause 5.1.9.7 of [3]. The interpolation of LSP coefficients is performed as described in clause 5.3.3.3.4.5 of [3].

In TD stereo decoding the quantized LSF parameters of the primary channel signal in the current frame are obtained from the bitstream. The IVAS decoder obtaining the quantized LSF parameters includes performing spectrum broadening on the quantized LSF parameters of the primary channel signal; obtaining the quantized prediction residual of LSF parameters of a secondary channel signal from the bitstream; determining the quantized LSF parameters of the secondary channel signal by using a sum of the quantized prediction residual of the LSF parameters of the secondary channel signal and the prediction for the secondary channel LSF parameters. Wherein, the prediction for the secondary channel LSF parameters is obtained by performing two-stage prediction on the spectrum-broadened LSF parameters of the primary channel signal. The spectrum-broadened LSF parameters of the primary channel signal are obtained by performing pull-to-average processing on the quantized LSF parameters of the primary channel signal, wherein the pull-to-average processing is performed according to the following formula:

LSFSB(i)=β⋅LSFP(i)+(1-β)⋅LSFS(i)

wherein LSFSB represents a vector of the spectrum-broadened LSF parameters of the primary channel signal, LSFP(i) represents a vector of the quantized LSF parameters of the primary channel signal, i represents a vector index, β represents a broadening factor, 0 < β < 1, for example β = 0.91, LSFS represents a mean vector of an original LSF parameter of the secondary channel signal, 1 ≤ i ≤ M , i is an integer, and M represents a linear prediction parameter. The second stage prediction is the tri-diagonal matrix prediction as in clause 5.3.2.3.9.

CHANGE 16

Editorial, remove empty line in Table 7.2-1

### 7.2.2	Rendering for binaural headphone reproduction

#### 7.2.2.1	Binaural rendering overview

The IVAS codec has several binaural renderers described in clauses 7.2.2.2 – 7.2.2.5. Depending on the input format, bitrate, IVAS mode and whether head-tracking is enabled different binaural renderer technology is used. Three binaural output modes are supported:

	Binaural output without room acoustic synthesis (no room), command line option BINAURAL,

	Binaural output with room acoustics synthesized using impulse responses (room with IR), command line option BINAURAL_ROOM_IR,

	Binaural output with room acoustics synthesized using parametric reverb, with or without early-reflections (room with reverb), command line option BINAURAL_ROOM_REVERB.

The table 7.2-1 provides an overview of which renderers are used at which operating point for the output formats as listed above.

Table 7.2-1: Overview of renderers used for binaural rendering output modes in the IVAS Codec

| IVAS input Format | Bitrate Range [kbps] | Mode (if applicable) | Binaural rendering output mode (if applicable) | Renderer Used |
| --- | --- | --- | --- | --- |
| SBA | 13.2 – 80 | - | - | Parametric Binaural Renderer |
| SBA | 96 – 512 | - | - | FastConv Binaural Renderer |
| MASA | 13.2 - 512 | - | - | Parametric Binaural Renderer |
| ISM | Cf. Table 5.6-1 | ParamISM | - | Parametric Binaural Renderer |
| ISM | Cf. Table 5.6-1 | DiscISM | No room or room with reverb | Time Domain Object Renderer |
| ISM | Cf. Table 5.6-1 | DiscISM | Room with IR | Crend Binaural Renderer |
| MC | Cf. Table 5.7-1 | McMASA | - | Parametric Binaural Renderer |
| MC | Cf. Table 5.7-1 | ParamMC | - | FastConv Binaural Renderer |
| MC | Cf. Table 5.7-1 | ParamUpmix | - | FastConv Binaural Renderer |
| MC | Cf. Table 5.7-1 | DiscMC | All except below | Crend Binaural Renderer |
| MC Planar Layouts (5.1 and 7.1) | Cf. Table 5.7-1 | DiscMC | Head tracking enabled for either no room or room with reverb | Time Domain Object Renderer |
| OMASA | Cf. Table 5.9-1 | DiscOMASA | No room | Parametric Binaural Renderer and Time Domain Object Renderer |
| OMASA | Cf. Table 5.9-1 | other modes | - | Parametric Binaural Renderer |
| OSBA | Cf. Clause 5.8.1 | - | - | Same as non-combined format |

CHANGE 17

Editorial, remove duplicate figure

#### 7.4.5.1	Overview

The external orientation input provides to the IVAS renderer any orientation information separate from the head orientation (rotation) data of the listener. The external orientation data, when available, is therefore processed and applied in addition to the head-tracking data, which is described in clause 7.4.3.

Figure 7.4-4 presents a block diagram describing the external orientation inputs at the IVAS renderer.

External orientation input handling

Orientation tracking

Head orientation / tracking data (cl. 7.4.2)

Orientation tracking mode (cl. 7.4.3)

Orientation tracking data (cl. 7.4.3)

Head orientation control input, Flaghead

External orientation

External orientation control input, Flagext

External orientation delay control input, Flagext,intrp, Nframes,intrp

Binaural rendering

Combined orientation

Figure 7.4-4: Block diagram of external orientation input handling for binaural rendering

END OF CHANGES