Agenda item: 	10.5

Source: 	Qualcomm Inc.

Title: 	[SR_IMS]  Advanced MF discovery

Document for	Discussion and Agreement

# Introduction

A few meetings back, we brought a contribution to outline the need for more advanced MF discovery to support IMS-based split rendering. As part of the discussion in FS_AI4Media, similar needs can also be identified. The MF selection for media processing can not be done arbitrarily but needs to take into account the desired media processing needs and requirements as well as the specific application that the MF is catering for. In the case of network/split rendering, the MF must be able to run the application that is selected by the user for rendering. These applications will usually be very demanding in terms of processing power, which is why the UE triggered network/split rendering in the first place.

In this contribution, we want to remind the group of this need and urge the exchange of an LS with SA2 and CT4 on the discovery of MF and how it should be extended.

# MF Discovery

The IP Multimedia Subsystem (IMS) is an architectural framework for delivering IP multimedia services. One crucial aspect of IMS is the discovery of Media Functions (MF), which are responsible for processing media streams in IMS.

The MF discovery procedures enable various network functions to discover and utilize media functions in an IMS network. These procedures include registration, update, deregistration, and discovery.

## 2.1	MF Registration/Update/Deregistration

The MF registration procedure allows an MF to register its capabilities with the Network Repository Function (NRF). The NRF maintains a repository of available network functions and their profiles, which include the capabilities and services offered by each MF.

The MF registration involves submitting a profile that includes the following key parameters:

- MF Identifier (MF ID): A unique identifier for the MF.

- MF Type: The type of media function (e.g., Media Resource Function, Media Resource Broker).

- Media Capabilities: A list of media processing capabilities supported by the MF (e.g., codec support, media stream types) as defined in [5]. These are expressed as strings.

- Service Endpoints: Information on how to reach the MF, including IP addresses and port numbers.

- Operational Status: The current operational status of the MF (e.g., active, inactive).

- Load Information: Current load and capacity details to assist in load balancing decisions.

MFs may need to update their registered profiles periodically to ensure that the NRF has the most current information. This procedure involves sending an update request to the NRF with the latest profile details.

When an MF is no longer available, it must deregister itself from the NRF. This helps in maintaining an accurate repository of active network functions.

## 2.2	MF Discovery by Consumer Network Functions

Consumer Network Functions (NFs), such as the Application Function (AF) or the Policy Control Function (PCF), can discover MFs by querying the NRF. This enables them to route media processing requests to the appropriate MF.

The discovery process involves querying the NRF with specific parameters to find suitable MFs. Typical query parameters include [5]:

- NF Type: the NF type is set to “MF”.

- Location information: the MF profile may include location information to help in selecting a local MF to reduce the media path latencies

- Service List: a text-based list of IMS media services that are offered by the MF.

- Capabilities: currently the supported capabilities as defined by [4] are Data Channel (DC) and Augmented Reality (AR).

## 2.3 Advanced MF Discovery Needs

In order to support service-agnostic split rendering and other applications such as split inference, the discovery procedures for the MF need to be extended to include more detailed descriptions of:

- Capabilities: media processing capabilities need to align with the level of capability description offered by edge media processing,

- Applications: the service list offered by the MF needs to be extended to contain a description of the applications that it is able to process,

- QoS parameters: on top of the media capabilities, split rendering may have requirements on the QoS, such as supported bitrates and latencies. It should allow for determining the latencies between the UE and the MF.

These parameters need to be part of the MF discovery procedures and it should be possible to signal these parameters as part of the IMS session management signaling.

# Proposal

We propose to send an LS to SA2 and CT4 to ask about the possibility of accommodating these advanced MF discovery parameters.

# References

[1]	 	SP-240492, WID on Split Rendering over IMS (SR_IMS)

[2]		3GPP TS 23.502, Procedures for the 5G System; Stage 2

[3]	 	3GPP TS 23.228, IP Multimedia Subsystem (IMS); Stage 2

[4]	 	3GPP TS 29.176, IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3

[5] 	3GPP TS 29.510, 5G System; Network Function Repository Services; Stage 3