3GPP TSG-SA4 Meeting #65 	S4-110626

# Kista, Sweden, 15 – 19th August, 2011

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0001 | rev | - | Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPD Changes to Support DASH over Broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HTTP_SDS |  |  |  |  |  |  | Date: |  | 28/07/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change: |  | As stated in section 5.6 of TS 26.346, it shall be possible to carry Adaptive HTTP Streaming content over the MBMS download delivery method. However, Media Presenation Description (MPD) specfied in TS 26.247 cannot indicate the slection criteria for Media Segments sent over broadcast if the broadcast channel is available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add a new attribute to Representation Elements in TS 26.247 to indicate slection criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no slection criteria for the UE to use for the DASH Segments that are sent over broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 8.4.3.4	Representation

Representations are described by the Representation element. A Representation is one of the alternative choices of the media content or a subset thereof typically differing by the encoding choice, e.g. by bitrate, resolution, language, codec, etc.

A Representation starts at the start of the Period PeriodStart and continues to the end of the Period, i.e. the start of the next Period or the end of the Media Presentation.

A Representation consists of one or more Segments.

Each Representation either contains an Initialisation Segment or each Media Segment in the Representation is self-initialising.

Each Representation includes one or more media components, where each media component is an encoded version of one individual media type of continuous media such as audio, video, or timed text. Media components are time-continuous across boundaries of consecutive Media Segments within one Representation. The timing within each Representation is relative to the PeriodStart time or the Period that contains this Representation.

The semantics of the attributes and elements within a Representation are provided in Table 10. The XML-syntax of the Representation element is provided in Table 11.

Table 10: Semantics of Representation element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Representation |  |  | This element contains a description of a Representation. |
|  |  |  | @id | M | specifies a unique identifier for this Representation within the Period. The string shall only contain characters that permit to form a valid HTTP-URL according to RFC 1738. |
|  |  |  | @bandwidth | M | specifies the minimum bandwidth of a hypothetical constant bitrate channel in bits per second (bps) over which the Representation (i.e. the collection of all Segments of a Representation) can be continuously delivered such that a client, after buffering for exactly @minBufferTime when accessing a Representation at any RAP can be assured of having enough data for continuous playout. |
|  |  |  | @qualityRanking | O | specifies a quality ranking of the Representation relative to other Representations in the Adaptation Set. Lower values represent higher quality content. If not present then the ranking is undefined. |
|  |  |  | @SelectionCriteria | O | Identifies the representation to belong to one of the following categories. Different values and description of SelectionCriteria is described below: SelectionCriteria = “BC”: DASH segment is transmitted over broadcast channel when broadcast channel is available. |
|  |  |  | CommonAttributesElements | - | specifies common Adaptation Set, Representation and Sub-Representation attributes and elements (see clause 8.4.3.2) |
|  |  |  | SubRepresentation | 0 … N | specifies rovides information about a sub-representation that is embedded in the containing Representation. For more details see clause 8.3.4.5. |
|  |  |  | SegmentInfo | 1 | specifies Segment information. For more details see clause 8.4.4. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @, List of elements and attributes is in italics bold |  |  |  |  |  |

Table 11: XML-Syntax of Representation element

<!-- A Representation of the presentation content for a specific Period --> <xs:complexType name="RepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="SubRepresentation" type="SubRepresentationType" minOccurs="0"/> <xs:element name="SegmentInfo" type="SegmentInfoType" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:attribute name="SelectionCriteria" type="xs:string"> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:extension> </xs:complexContent> </xs:complexType>