Source:	Qualcomm, Incorporated

Title:	Estimation of CMW500 network simulator delay in loopback mode

## Document for:	Approval

## Agenda Item:	8.5

# Introduction

In the definition of UE delay test points, the source has indicated about the possibility of estimating the network simulator delays in loopback mode by looking at the time difference of the RTP packets containing encoded speech in the sending and receiving direction. This contribution reports on observations of such delays and provides an estimate of the CMW network delays under these conditions.

# Procedure

A CMW500 network simulator and a UE with logging capability of RTP packets in the send and receiver directions are used for these tests. The network simulator is configured for loopback mode operation and a pure 1023Hz sine wave tone with -16dBoV is electrically injected in the UE through the headset jack connector to avoid alternation between speech and SID frames.

In loopback mode, the network simulator receives the transport blocks sent by the UE, performs demodulation of the RTP packets, modulates those packets and transmits them back to the UE. It is noted that in loopback mode, there is time synchronization of the RTP stream in the send and receive directions (as there is only one clock involved) and the derivation of the RTP2RTP delay is possible without having to resort to special time referencing schemes (e.g. using GPS). A schematic of the loopback transmission from/to the speech coding unit of the UE is illustrated in Figure 1.

The RTP2RTP_loopback delay is specified as:

RTP2RTP_loopback_dly = UE_LTE_stack_TX + NetSim_LTE_stack_RX + NetSim_RTP2RTP_dly + NetSim_LTE_stack_TX + UE_LTE_stack_RX

In order to estimate NetSim_RTP2RTP_dly in the CMW500, *.pcap logs are generated within the network simulator and the logs later analyzed with the Wireshark utility. The RTP2RTP_loopback_dly is analyzed by using a proprietary logging tool connected to the UE through USB interface.

# Results

Measurements were conducted under 0% BLER and no DRX cycle. Under these conditions, NetSim_RTP2RTP_dly results for the CMW500 are illustrated below (obtained with Wireshark as previously described).

Table 1 - Results of NetSim_RTP2RTP_dly measurements.

| As expected the NetSim_RTP2RTP_dly is very RTP time stamp | log time of RCV’d packet at CMW500 | log time of TX’d packet by CMW500 | NetSim_RTP2RTP_dly |
| --- | --- | --- | --- |
| … | … | … | … |
| 236640 | 21.522202 | 21.522124 | 0.0000779999999984682 |
| 236800 | 21.542093 | 21.542167 | 0.0000739999999979091 |
| 236960 | 21.561126 | 21.561211 | 0.0000849999999985585 |
| 237120 | 21.582102 | 21.582180 | 0.0000780000000020209 |
| 237280 | 21.602081 | 21.602145 | 0.0000640000000018404 |
| 237440 | 21.622107 | 21.622211 | 0.000104000000000326 |
| 237600 | 21.641073 | 21.641137 | 0.0000640000000018404 |
| … | … | … | … |
|  |  |  | 0.0000781428571429948 |

small. The averaged NetSim_RTP2RTP_dly delay is around 0,08ms.

RTP2RTP_loopback_dly results captured at the UE are presented in Table 2. The averaged RTP2RTP_loopback_dly is in the order of 15.7ms.

| RTP time stamp given by UE | RTP pkt log time TX in UE | RTP pkt log time RX in UE | RTP-RTP delay |
| --- | --- | --- | --- |
| 13080480 | 23:52:04.125 | 23:52:04.141 | 16ms |
| 13080640 | 23:52:04.145 | 23:52:04.160 | 15ms |
| 13080800 | 23:52:04.165 | 23:52:04.181 | 16ms |
| 13080960 | 23:52:04.185 | 23:52:04.200 | 15ms |
| 13081120 | 23:52:04.205 | 23:52:04.221 | 16ms |
| 13081280 | 23:52:04.225 | 23:52:04.241 | 16ms |
| … | … | … | … |
| 13086080 | 23:52:04.825 | 23:52:04.841 | 16ms |

Table 2 - Results of RTP2RTP_loopback_dly measurements.

For the particular UE tested, UE_LTE_stack_TX is estimated to be in the range of 1 to 3ms and UE_LTE_stack_RX estimated to be in the range of 4-8ms. Due to the ideal and stable physical layer conditions used in these tests it is likely that the actual delays are in the lower range of these estimates.

RTP2RTP_loopback_dly = UE_LTE_stack_TX + NetSim_loopback_dly + UE_LTE_stack_RX

15.7ms = 1 to 3ms + NetSim_loopback_dly + 4 to 8ms

15.7ms = 1 to 3ms + NetSim_loopback_dly + 4 to 8ms

NetSim_loopback_dly = 15.7ms - (5 to 11ms)

NetSim_loopback_dly = 4.7ms to 10.7ms (for CMW500 network simulator in loopback operation)

# Summary and Proposal

Derivation of an estimate for the CMW500 network simulator delay was presented. These results can be subtracted from the total loopback delay measurement (M2E) in order to derive the UE delay in ideal channel conditions.

The uncertainty in the estimation is small since few components are involved in the delay path and the synchronization between sending and receiving entities is guaranteed.

It is proposed that the loopback method is adopted for the measurement of UE delay in LTE call in the absence of jitter and packet loss (ideal channel conditions).