3GPP TSG SA4#86 meeting	S4-151500

San Jose del Cabo, Mexico, 26 - 30 October, 2015

Agenda item: 	16.8.2

Source: 	Huawei Technologies Co Ltd

Title: 	Text Proposal for Inclusion in Clause 5.1.3 of the MCPTT TR 26.879 based upon the NTIA Report 15-520 Results

Document for	Approval

___________________________________________________________________

### 5.1.2	Review of the Codec Alternatives and their Relative Speech Intelligibility in Clean and Low SNRs

The high noise intelligibility results in NTIA Report 15-520 [16] provide valuable information for the selection of the codec for MCPTT.

The report provides intelligibility scores, as measured with the Modified Rhyme Test (MRT) methodology, across a range of noise conditions for different codecs, bit rates and bandwidths but for SA4 only 3 codecs are really of direct interest; namely AMR (Sometimes referred to as AMR NB in [16]), AMR-WB and EVS although the benchmarks of Analogue FM and the P25 codec are of interest. These intelligibility results were conducted without radio channel impairments.

In clean channel conditions, in two cases out of 12 EVS-NB performed statistically better than AMR but in the other 10 cases there was no statistical difference.

In one case out of 18, EVS-WB performed statistically worse than AMR-WB, but in this case EVS was operating at its lowest bit rate with DTX/CNG enabled and at a lower bit rate than AMR-WB. In the other 17 cases there was no statistical difference.

In three cases out of 24, EVS-FB performed statistically better than AMR-WB or EVS-WB, in one case EVS-FB performed statistically worse than AMR-WB (Alarm noise -30dB SNR) and in the other 20 cases there was no statistical difference.

•	From the NTIA report [16] using a WB codec results in higher intelligibility than using a NB codec in noisy conditions.

•	In the NTIA report [16] for SNRs within the range 10dB to -5dB, FB is always equivalent to or better than WB and NB in noisy conditions from an intelligibility point of view.

•	The NTIA report [16] shows that the intelligibility increases (up to a saturation level) with coded bitrate within confidence limits for a given audio bandwidth.