Title:	DRAFT Reply LS on EVS over CS

Response to:	LS on EVS over CS (S4-160711/ C3-161205)

Release:	Rel-13

Work Item:	EVSoCS (EVS in 3G Circuit-Switched Networks)

Source:	SA4

To:	CT3

Cc:	CT4

Contact Person:

#### Name:		Imre Varga

#### E-mail Address:	Imre . Varga @ qti . qualcomm . com

Attachments:	--

1. Overall Description

SA4 would like to thank CT3 for their LS reply on EVSoCS.

SA4 gives the following answers to the questions from CT3:

- Q1: Do those recommendations apply for the Mb interface in general, or only if interworking with EVSoCS is encountered? CT3 assumes that those recommendations apply only for interworking, but would like to ask SA4 to confirm.

Answer: Those recommendations apply for the Mb interface in case of interworking with EVSoCS only. They aim for optimal voice quality with minimal implementation and interworking complexity.

- Q2: Do those recommendations apply only for the MGCF and IMS-MGW, or also for the peer UE on the IMS side? (The recommendations can probably only apply to the UE if the answer to Q1 is that the recommendations apply in general, as the UE cannot be aware whether interworking occurs)

Answer: Those recommendations apply for the Mb interface of the MGCF and IMS-MGW and apply naturally for both ends of the Mb interface and up to the UE on the IMS side if end-to-end TrFO with EVSoCS is desired, as a guideline for the SDP Offer Answer procedure.

- Q3: Are those recommendation meant for SDP negotiation procedures or for RTP usage once a payload type has been negotiated?

Answer: Those recommendations are meant as a guideline for SDP negotiation procedures. If the SDP Offer Answer negotiation does not follow these recommendations, then the User Plane interworking is less favorable and in the worst-case transcoding will be required. The call will not fail, but voice quality may be degraded and more MGW resources may be needed.

- Q4: If those recommendations are for Mb in general, what is the relationship of the above recommendations with the Rel-12 mandatory procedures and recommendations for EVS in TS 26.114, which are also applicable on the Mb interface?

Answer: Not relevant, see Q1.

However, note that the recommendations given in TS 26.454 are within the Rel-12 mandatory procedures and recommendations for EVS in TS 26.114. SA4 acknowledges that TS 26.114 also allows offer answer procedures that would not be in line with the recommendations and consequently could lead to less favorable interworking.

[some answer to the other part of the LS]

2. Actions:

To CT3:

ACTION: SA4 kindly asks CT3 to take above information into account.

3. Date of Next SA4 Meetings:

SA4#90	5 – 9 September 2016	Ljubljana