Source:	Samsung Electronics Co., Ltd.

Title:	Carriage of ATSC 3.0 Content over TMB2

Document for:	discussion and agreement

Agenda Item:	9.12

# Introduction

SA4 has started working on TV enhancements to address several use cases of TV content distribution to mobile devices. In this contribution, we present the use case of ATSC 3.0 broadcast content distribution over mobile networks.

# Backing SA1 Use Cases and Requirements

## Requirements

From [1], the following requirement is relevant to the use case proposed in this document:

An eMBMS transport layer should be able to transport TV streams formatted not compliant to 3GPP standards.

## Use Cases

From [2], the following use cases are related to the use case that is proposed in this document.

## 4.16	Use Case 16: Decoupled eMBMS content, service, and transport

### 4.16.1	Description

A content provider partners with a MNO to distribute as subset of its video/TV services offered by the content provider.

Rather than re-encode all existing encoded content for 3GPP-based distribution as per the profiles set out in the MBMS service layer, and transpose the service initialization metadata to the format provided by the MBMS service layer, the content provider requests to be able to access only the MBMS transport.

As a result, the MNO provides the facility for the content provider to feed metadata and streamed content into the MNO core network for transport to a population maintained by the content provider.

### 4.16.2	Pre-conditions

Subscribers of MNO B have eMBMS enabled UEs.

### 4.16.3	Service Flows

A TV Content Provider establishes a business agreement with an MNO to use the MNOs eMBMS transport network to carry content traffic with certain parameters (time of day, bandwidth, geographic locations, etc.).

Within those parameters, the TV Content Provider is able to insert metadata and content into the MNOs eMBMS distribution network directly. This content and metadata is not formatted according to the eMBMS service layer format, and is not encoded accorded to the formats enforced by the eMBMS service layer.

Subscribers to the TV Content Providers service are notified by some mechanism (e.g. over-the-air electronic service guide) that content is being made available and sufficient metadata is made available by the TV Content Provider to enable the receiving UEs to tune into the service.

Subscribers to the content service have sufficient UE capabilities to support the service (e.g. codecs, subtitle support, DRM, error correction, reporting mechanisms, etc.)

### 4.16.4	Post-conditions

Subscribers to the service of TV Content Provider obtain their service provided by the eMBMS transport network of the MNO.

### 4.16.5	Potential Requirements

An MNO should be able to make available the transport aspects of an eMBMS network to a third party content provider.

An eMBMS transport layer should be able to transport TV service streams formatted not compliant to 3GPP standards.

A UE should be able to access an eMBMS transport session with the support of only transport metadata.

Note: service layer metadata describing TV service is outside the scope of 3GPP for non 3GPP transports.

An eMBMS system should be able to be initiated by a UE with sufficient metadata provided by a mechanism other than User Service Description for non 3GPP transport services.

## 4.13	Use Case 13: Service Layer Function and Codec Support

### 4.13.1	Description

A TV Content Provider partners with an MNO to distribute a subset of the channels offered on its DTT service.

Rather than re-encode all existing encoded content for mobile distribution as per the profiles set out in the pre-Release 14 MBMS service layer, the broadcaster partners with an MNO that enables a formats, codecs and service layer functions suitable and necessary for the rebroadcast of existing content which can comply with the regulatory requirements of the region.

### 4.13.2	Pre-conditions

Subscribers of MNO B have eMBMS enabled handsets.

### 4.13.3	Service Flows

A TV content Provider makes a business agreement with an MNO to use the MNO’s eMBMS network to carry TV content traffic with certain parameters (time of day, bandwidth, geographic locations, etc.).

Within those parameters, the TV Content Provider provides content to the MNO’s eMBMS network and this content is provided in a format supported by the Release 14 eMBMS service layer, including functions covered by regulatory requirements, interactivity functions, security functions, and accessibility functions supported by the Release 14 eMBMS service layer.

### 4.13.4	Post-conditions

Subscribers to the service of the TV Content Provider obtain their TV service provided by the eMBMS network of the MNO.

### 4.13.5	Potential Requirements

The eMBMS service layer should support audio and video formats typically supported by TV Content Providers for HD TV services and UHD TV services.

The eMBMS service layer should support codecs typically supported by TV Content Providers for HD TV services and UHD TV services.

The eMBMS service layer should support accessibility functions typically supported by TV Content Providers (e.g. subtitling, closed captioning, audio descriptions, anonymous reception, reporting to support ratings, reporting enforcement, etc.).

The eMBMS service layer should support regulatory mandates typically supported by TV Content Providers (blackouts, emergency alerts, etc.).

The eMBMS service layer should support interactivity functions typically supported by TV Content Providers (interactive services, second screen, personalization, etc.).

The eMBMS service layer should support ad insertion use cases typically supported by TV Content Providers (targeted ad insertion, ad replacement, etc.),

The eMBMS service layer should support encryption, security and conditional access functions typically supported by TV Content Providers.

The eMBMS service layer should support efficient concurrent delivery of multiple separate services (service signalling, statistical multiplexing, etc.).

The eMBMS service layer should support random access and channel change times comparable to existing HD TV services.

The eMBMS service layer should support service delivery over broadcast only, unicast only and combinations of the two.

The eMBMS service layer should support delivery of real-time and non-real-time content.

The eMBMS service layer should enable extensibility and forward-compatibility to new requirements, formats, codecs and other functions to the extent possible.

# IP-Based Broadcast Use Case

## Use Case

A broadcaster that has TV services over IP-Based Broadcast (such as ATSC 3.0 or ARIB) is interested in extending its reach to mobile devices and in improving the in-door reception for the fixed receivers. The broadcaster reaches an agreement with a mobile operator to distribute some of the TV channels over mobile broadcast using eMBMS. However, the broadcaster wants to maintain control over the content components and workflow and reuse existing players for the playback of the content. It also needs to maintain the service aspects that are related to regulatory commitments of the broadcaster.

## Proposed Requirements

We propose to adopt the requirement from SA1 as follows:

- An eMBMS transport layer should be able to transport TV streams formatted not compliant to 3GPP standards.

To clarify this requirement and to address the IP-based broadcast use case, we propose the following additional requirement:

- It is possible to deliver IP-based terrestrial broadcast streams without modifications to the contents of the stream

## Working Assumption

Based on the previous use case and requirements, the following working assumptions are determined:

- The BM-SC will forward the IP-based broadcast stream as is as user plane multicast data over the TMB2 interface

- The BM-SC creates a minimal service announcement that serves the purpose of bootstrapping

- The MBMS API offers access to the raw IP-based broadcast stream

- The BM-SC may offer additional functionality to support delivery over MBMS. The technical and commercial viability of these features will be studied on case by case basis.

# Conclusion

We propose to adopt the IP-based broadcast use case and the proposed requirements.

# References

[1]	TS 22.101, Service aspects; Service principles (Release 14)

[2]	TR 22.816, 3GPP enhancement for TV service