Agenda Item:	10.7

Source:	Huawei Technologies Co. Ltd.,

Title:	Discussion on the decomposition of E2E delay

Document for:	Discussion

- Introduction

The source intends to discuss the necessity and possibility to decompose the E2E delay requirement, e.g. MTP, MTS.

- Discussion

The indicator MTP/MTS as discussed in [1] plays important role in creating immersive experience. Take MTP as an example, the recommended requirement is 20ms, or even 10ms. How to ensure this ultra-low E2E delay? All intermediate steps in the service chain would play a role, majorly movement detection and data process in VR client, data preparation in the content server, and data transmission via wireless network.

Though E2E latency information could provide a general view, the information can’t tell which part of the E2E service chain caused the problem, if the latency is long. In this case, it helps if more detailed information could be logged and presented, i.e. the decomposed delays in each segment in the E2E service chain, e.g. delay of data process in the server, the transmission delay and the data process in the client. The source proposes that such work be discussed and studied in 3GPP SA4.

Referring to Figure 1, the source tries to decompose MTP delay into 4 parts:

Table 1 Latency decomposition

| Time point | Delay segment | Major contributor | Observation point |
| --- | --- | --- | --- |
| t1 | T2-T1 user movement detection and relevant FoV data request prepared | sensor detection and data processing | Sensor in the client |
| t2 | T5-T2 round-trip delay FoV video request | Network transmission latency + data process in the server | Network interface |
| t3 | T4-T3 data process in the server | Compression and encoding | Server |
| t4 | T6-T5 data process in the client | Decompression/decoding/presentation | Sensor in the client |

Figure 1 E2E delay decomposition

According to the above analysis, t1 and t4 could be logged by the client, t2 could be monitored by network interface in the client, i.e. the time delay between the request is sent out and the requested data is received, t3 could be logged by the sever. And the delay of network transmission could be obtained by t2 – t3.

This decomposition would help all players in the service chain, i.e. the content provider, the network vendor and the VR device provider, get to know where the problem is located and design targeted solution to improve the product.

- Proposal

It is suggested to study the feasibility and detailed solution to log relevant information to track delay in the E2E service chain.

- Reference

[1] TR 26.918 Virtual Reality (VR) media services over 3GPP v15.0.0;