Source:	Samsung Electronics Co., Ltd.

Title:	Viewport-dependent VR streaming

Agenda Item:	10.6

Document for:	Discussion and Agreement

# Introduction

3GPP SA4 has started a work item on VR Streaming [1] support in PSS and MBMS. In this contribution, we propose changes to the PSS codec requirements and describe content preparation for viewport-dependent VR streaming.

# Media Codec Changes

# Introduction

PSS has support for AVC High Profile 3.1 as mandatory and HEVC Main Profile Level 3.1 as recommended. However, to meet the requirements of VR applications, a minimum of 4K@30Hz (and preferably @60Hz) support needs to be provided. The following section defines requirements on video decoders for VR streaming applications in PSS.

# Video Decoder Requirements for VR

If a PSS client supports omnidirectional VR video, the following applies:

- H.264 (AVC) Progressive High Profile Level 5.1 decoder [90] shall be supported, wherein maximum VCL Bit Rate is constrained to be 240Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1000 and 1200, respectively.

- H.265 (HEVC) Main Profile, Main Tier, Level 5.1 decoder [117] should be supported.

When H.265 (HEVC) Main Profile decoder is supported, the client is only required to process H.265 (HEVC) Main Profile bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, and general_frame_only_constraint_flag equal to 1.

In addition, the following applies:

- The H.264 (AVC) Stereo High Profile shall be supported

- The frame packing arrangement SEI message of H.264 (AVC) shall be supported

- If H.264 (HEVC) is supported, the the frame packing arrangement SEI message of H.265 (HEVC) shall be supported

# Viewport-dependent Streaming

Fig.1 shows an example of producing the viewport-dependent VR content. It consists of the following three steps: 1) partitioning of the High Resolution (HR) ERP video, 2) down-sampling and rotating of the full ERP in low resolution (LR), 3) packing the HR and the LR videos side-by-side with a guard interval separating the two. The tree steps are depicted in Fig. 1-(a), Fig. 1-(b), Fig. 1-(c), respectively.

In the Fig. 1-(a), the ERR video content is vertically partitioned into 8 overlapping partitions, each of which covers 8 contains the HR part of the EQR content. In this embodiment, under condition of 360 degree for the whole EQR width, each EQR partition covers the 135 degrees horizontal field of view. The shift between two successive partitions is set to be 45 degrees. In the Fig. 1-(b), the full ERP video is down-sampled and rotated by 90 degrees clockwise to produce the LR rotated ERP content. In the Fig. 1-(c), each of the 8 HR partitions is packed side-by-side with the rotated LR to produce the frame-packed video.

(a) Partitioning of HR EQR video step

(b) Downsampling-90 degree rotating step

(c) Packaging HR partition and LR ERP

Fig. 1 Packaging for Viewport-dependent VR Streaming

The 8 resulting videos are described as separate disjoint DASH AdaptationSets. The receiver will only have to fetch a single video from only 1 of the 8 AdaptationSets at any point of time. Depending on the targeted HMD characteristics, the shift between two consecutive HR partitions may need to be changed (e.g. an HMD of 100 degrees horizontal field of view would result in 35 degrees shift).

The overlap and the side-by-side packing of HR and LR achieve the following benefits:

- A single video stream and a single decoder is used at any point of time and provides a coverage of the full 360 degrees

- Synchronization between the LR and HR videos is guaranteed through the usage of a single video

- The current viewport is always covered by a single HR partition irrespective of the viewing direction

- An LR representation is always available to cover for sudden viewport changes and up until the corresponding HR video is received.

- The shader will only use texels from either the HR or the LR video data, thus preventing any distortion that would result from mixing the two at rendering.

# Evaluation of the AVC viewport dependent baseline media profile with packed streaming

# Experimental setup

In the experiment, the MPEG 4K (3840x1920) test video sequences are used for evaluation.

For the proposed viewport-dependent VR profile, the experimental conditions are set as follows.

- Partition 4K ERP into 12 overlapping partitions with 30 degree shift

- Generate 12 frame packed versions and encode them at 10 different bitrate steps starting from 2Mbps using x265

- Decode and split HR (High Resolution) from LR(Low Resolution), and rotate back the LR

- Upscale HR partition and LR rotated down-sampled ERP to 8K

- Upscale 4K ERP to 8K ERP using the same upscaling configuration

- Calculate PSNR and W-PSNR using the 8K original video sequence as reference for the up-scaled HR partition, up-scaled LR ERP, and for the up-scaled 4K ERP as used in the viewport independent VR streaming

- Calculate the composite PSNR and W-PSNR for HR+LR as follows

PSNR= i=011PSNRi∙Pi+(1-∙Pi)∙PSNRlow

where PSNRi denotes the PSNR of the up-scaled partition i and PSNRlowdenotes the up-scaled low resolution ERP and Pi denotes the probability of presenting the high resolution partition, respectively.

# Experimental results

The PSNR results of the Harbor and KiteLite video sequences are shown in Fig 2 and Fig. 3, respectively. In the PSNR results the viewport-dependent streaming and ERP are plotted with five operation points. The results show that the viewport-dependent streaming outperforms the ERP for all of the operation points for the two reported sequences.

Fig. 2 PSNR results of the viewport-dependent streaming with packaging for Harbor sequence

Fig. 3 PSNR results of the viewport-dependent streaming with packaging for KiteFlite sequence

### References

[1]	SP-170612, Virtual Reality Profiles for Streaming Media