Source:	Dolby Laboratories Inc.

Title:	Considerations on Design Constraints and Performance Requirements for IVAS Codec

Annex: Initial Draft of IVAS Codec Design Constraints

Editor’s note: This document is a suggested initial draft of IVAS codec design constraints. The document is a derivative from the EVS codec design constraints P-doc EVS-4. The changes are marked up. Columns 3 and 4 of the design constraints table contain informative language related to the EVS study phase TR 22.813. The text in these columns should be for revision and parts that are not applicable should be changed or removed.

# 1. Scope

This document presents the Immersive Voice and Audio Services (IVAS) codec design constraints within 3GPP TSG SA4). The development of the IVAS codec was initiated at SA#77 in September 2017. This document is based on the IVAD WID that was approved at that meeting. Section 2 of this document describes the set of design constraints of the IVAS candidate codecs.

# 2. IVAS Design Constraints

| Parameter | Design Constraint | TR 22.813 V10.0.0 | Note |
| --- | --- | --- | --- |
| Sampling Frequency and Audio Bandwidth | The encoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. The decoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. Note: Functions that are necessary only for achieving the arbitrary selection of encoder / decoder input / output sampling rate are envisioned to be an informative part of the IVAS standard. The encoder shall support input signals with different input signal bandwidth (WB, SWB, and FB) with frequency masks to be defined in the test plan. For a given input sampling rate, the encoder shall support an input indicating the maximum audio bandwidth (WB, SWB, and FB) to be encoded. This input may not be provided in all cases. | 6.1.1. | It is recommended that the EVS codec - have mandatory support of high-quality and high-efficiency operation of NB, WB, and SWB audio; - may support FB audio. Note: NB audio is typically sampled at 8 kHz with an audio bandwidth of 100 – 3500 Hz. WB audio is typically sampled at 16 kHz with an audio bandwidth of 50 – 7000 Hz. SWB audio is typically sampled at 32 kHz with an audio bandwidth of 50 – 14000 Hz. FB audio typically provides an audio bandwidth up to 20000 Hz. |
| Number of audio streams | The IVAS candidate codecs shall support - mono, stereo and 5.1 channel-based audio - scene-based audio with first to [3rd] order ambisonics - object-based audio with up to [n] individual objects and [m] embedded objects into scene-based audio. Editor’s note: The maximum HOA order (current suggestion [3]) and the number of [n] individual objects and [m] embedded objects should be for further discussion based on discussion inputs on use cases and relevant audio capture scenarios. The decoder shall offer the possibility to render a mono compatible signal from the received multi-stream audio signal bit stream. Note: The quality of the mono compatible signal shall be verified. | 6.1.2 | Stereo or multi-channel presentation is one way to realize significantly improved QoE. The codec may provide stereo/ multi-channel coding capability. Multiple monophonic coding, i.e. one monophonic coding per channel, can be realized by appropriate packetization techniques while stereo/multi-channel coding, i.e. joint coding of the channels, is part of the audio coder algorithm. The choice of whether using dedicated stereo/multi-channel coding or multiple monophonic codings depends on a trade-off between achievable quality, available bit rates, available delay, complexity and other implementation factors. If stereo/multi-channel coding capability is provided, it is necessary to specify how stereo or multi-channel capture/presentations could be achieved in mobile communications. |
| Bit Rates | Mono operation In the single frame case the net source coding bit rates (IVAS payload) shall be derived from the gross bit rates (IVAS payload and payload header) by subtracting non-negative integer multiples of 0.4 kb/s to account for the RTP payload header. In the single frame case the IVAS candidate codecs shall support fixed rate source coding at gross bit rates of 7.2 kb/s, 8, 9.6, 13.2, 16.4, and 24.4 kb/s. Further the IVAS candidate codecs shall support fixed rate source coding at net source coding bit rates of 32, 48, 64, 96, 128 kb/s. IVAS candidate codecs may support source controlled variable bit rate (VBR) coding; the peak per-frame gross bit rate shall not exceed 150 percent of the average bit rate. For average rates below 32 kb/s, VBR coding shall be composed of frames that are transported efficiently using LTE Transport Block Sizes (TBSs) defined in TS 36.213. SID frames shall not exceed a gross bit rate of 56 bits per frame. For multi-stream audio modes of IVAS: Editor’s note: For multi-stream audio modes the required bit rates should be set after further discussion on use cases and feasibility. | 6.1.3 | The codec shall span a large range of bit rates from low rates needed for high efficiency conversational speech services to high rates required for EVS with high quality operation. It is recommended that the offered span of bit rates shall be wide enough to allow for rate adaptation in response to available transmission resource. |

| Algorithmic Delay | The algorithmic delay shall be less than or equal to 32 ms, with the exception of multi-stream audio for which the algorithmic delay shall be less than or equal to 50 ms. Note: The algorithmic delay for IVAS is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing). The algorithmic delay of the IVAS candidate codecs excludes processing delay (e.g., runtime of the DSP to process the speech frame at encoder and decoder), and channel transmission delays. | 6.1.4 | The codec delay requirement for the EVS codec is recommended to be flexible within certain limits, allowing for overall optimizations of the system performance and considering that there is a trade-off between delay consumed by the speech codec and delay consumed by the PS transmission via the LTE air interface. The delay requirements should be the same regardless of the nature of the input content (e.g. speech, music and mixed content). The algorithmic delay of the EVS codec should be such that the overall end-to-end delay in an EVS-UE to EVS-UE connection meets or exceeds the preferred performance expectations in 3GPP TS 22.105. According to 3GPP TS 22.105 v9.0.0, conversational voice one way delay is <150 ms preferred and <400 ms limit where it is noted that the one way delay in the mobile network (from UE to PLMN border) is approximately 100 ms. In addition it is recommended to consult the relevant 3GPP working groups (SA1, SA2, RAN, CT) in order to get a technically relevant breakdown of the various possible delay contributing elements of the end-to-end EVS-UE to EVS-UE transmission chain, which in turn enables specifying the allowable limits for the algorithmic delay of the EVS codec. |
| --- | --- | --- | --- |
| Complexity | Complexity limits are applied according to the following categories. The computational complexity and program ROM (PROM) of the candidate codecs for each category shall be measured with ITU-T STL2009 [1] as the observed worst-case encoder + observed worst-case decoder complexity within the same category. - Required mono operation modes including - support for all mandatory bit rates - support for all mandatory sampling rates - support for rate switching - VAD/DTX/CNG - PLC - RTP packaging and optional source-controlled VBR operation modes, excluding JBM Computational: wMOPS 88 wMOPS, approximately 2 x wMOPS complexity of AMR-WB estimated with ITU-T STL2009. Memory: RAM 100 kwords ( 16 RAM of AMR-WB speech codec: 6.5 kwords) ROM 100 kwords ( 10 x ROM of AMR-WB speech codec: 9.9 kwords) Program ROM 10*Program ROM of AMR-WB speech codec (The AMR-WB PROM estimated with ITU-T STL2009 equals 5426 STL2009 operators) - All other operation modes (for multiple audio streams), excluding JBM Computational: wMOPS (88 + n * 47) wMOPS, with n being the number of audio streams, up to a total limit of [500] wMOPS for arbitrary n. Memory RAM n * 100 kwords, with n being the number of audio stream ROM 100 kwords, extra on top of ROM limit for mono operation modes Program ROM 10*Program ROM of AMR-WB speech codec, extra on top of PROM limit for mono operation modes Note: The amount of ROM and PROM should be combined into single figures and be compared to combined limits. How to obtain such combined figures is for further discussion. Note: The computational complexity and memory usage of the JBM must be reported. Note: Estimation of AMR-WB complexity using ITU-T STL2009 was done in [2] and [3]. Estimation of PROM using ITU-T STL2009 was done in [4]. | 6.1.5 | The EVS Codec should be implementable on a mobile device using today’s technology. The EVS codec should provide low computational complexity not significantly exceeding the design limits set during the AMR-WB codec standardization, and should have low memory usage. Increased computational complexity and memory usage should be commensurate with the gain in quality of user experience (e.g. higher audio bandwidth such as SWB or stereo if it is supported) or with increased efficiency (e.g. lower bit rate for same quality when compared to a reference codec). |
| Backward Interoperability | The candidate solution shall allow bit-exact EVS operation by means of choosing EVS though SDP. | 6.1.6 |  |
| Frame length | The candidate codecs shall operate with a frame size of 20 ms. |  |  |
| Jitter Buffer Management (JBM) | A JBM solution conforming to the requirements in TS 26.114, except for the functional requirement in sub-clause 8.2.2 of TS 26.114: “Speech JBM used in MTSI shall support all the codecs as defined in clause 5.2.1”, shall be provided with the candidate codecs. |  |  |
| Rate switching | The candidate codecs shall perform rate switching upon command to the encoder throughout the entire bit rate range at arbitrary frame boundaries. The rate switching may imply switching between different bandwidths and between mono coding modes and and coding modes for multiple audio streams. |  |  |
| Packet loss concealment (PLC) | A PLC solution shall be provided by the IVAS candidate codecs. |  |  |
| RTP payload format | Candidate codecs shall provide an RTP payload format specification supporting the full set of features and functionality of the IVAS candidate codecs. |  |  |
| DTX | The candidate codecs shall provide a complete VAD/DTX/CNG framework. It shall be possible to operate the codec with DTX on or DTX off. - DTX operation shall be supported for all operation modes with gross-rates ≤ 24.4 kb/s with a SID frame size not exceeding the limit defined in the bit rate section. - DTX operation may be supported for all operation modes with rates > 24.4 kb/s with a SID frame size not exceeding the limit defined in the bit rate section. - SID update frames shall be sent with a frequency not exceeding once per 8 frames |  |  |
| Output gain limitation | The IVAS candidate codecs shall not amplify the output signal relative to the input signal beyond limits. Output gain control for VR streaming use cases may have different requirements such as employing dynamic range control. Corresponding design constraints are for further discussion. The limits and methodology to measure the amplification is described in the [IVAS-7a,b] processing plan permanent document. |  |  |

# References

- ITU-T G.191 “Software tools for speech and audio coding standardization”, March 2010

- S4-110246  “On relative complexity estimation factor of the EVS codec”, February 2011

- S4-110279 “On the complexity of AMR-WB with STL2009”, February 2011

- S4-110479 “On Program ROM measurement for EVS standardization”, April 2011