3GPP TSG-SA4 Meeting #95 	S4-170993

# Belgrade, Serbia, Oct 9-13 2017											Revision of S4-170769

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0109 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for reportingserver |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei technologies CO. LTD, China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2017-10-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | @reportingsever is an element set as must in Quality Reporting Scheme informaiton. This element is used to find the receiving server when HTTP based QoE reporting is used. While for QMC based reporting, it is not necessary to set a specific server, and the UE only needs to send this report to the network, e.g. the eNB the UE connects with. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a quality reporting scheme for QMC based reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is problem to set the @reportingserver when QMC reporting is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************************  Start of change 1 *************************

Annex L (normative): 
1. QoE Measurement Collection Functionalities

As an alternative to configuration via MPD or OMA-DM, the QoE configuration can optionally be specified by the QoE Measurement Collection  (QMC) functionality. In this case the QoE configuration is received via specific RRC [53] messages over the control plane, and the QoE reporting is also sent back via RRC messages over the control plane.

If QMC is supported, the UE shall support the following QMC functionalities:

-	QoE Configuration: The QoE configuration will be delivered via RRC to the UE as a container according to "Application Layer Measurement Configuration" (see [53]). The container is an octet string with a maximum length of 1000 bytes, with gzip-encoded data (see [18]) stored in network byte order. The container shall be uncompressed, and is then expected to conform to XML-formatted QoE configuration data according to clause 10.4 in the current specification. This QoE Configuration shall be forwarded to the DASH client..

-	QoE Metrics: QoE Metrics from the DASH client shall be XML-formatted according to clause 10.6 in the current specification. The XML data shall be compressed with gzip (see [18]) and stored in network byte order into an octet string container with a maximum length of 8000 bytes. The container shall be delivered via RRC to the eNB according to "Application Layer Measurement Reporting" (see [53]). The behaviour if the compressed data is larger than 8000 bytes is unspecified in this version of the specification.

-	The UE shall also set the QMC capability "QoE Measurement Collection for streaming services" (see [53]) to TRUE.

The exact implementation is not specified here, but an example signalling diagram below shows the QMC functionality with a hypothetical "QMC Handler" entity.

Figure L-1: example signalling diagram

Note that the QMC Handler is only shown here as one possible implementation, and it need not be implemented as such. The corresponding QMC functionality could be built into the DASH client or into other UE entities. In this version of the specification the detailed implementation of the above functionalities is left to the UE vendor.

2. Quality Reporting Scheme for QMC based reporting in case of MPD based reporting

Table L-1: Semantics of Quality Reporting Scheme Information

| Element or Attribute Name |  | Use | Description |
| --- | --- | --- | --- |
|  | @samplepercentage | O | Percentage of the clients that should report QoE. The client uses a random number generator with the given percentage to find out if the client should report or not. |
|  | @reportinginterval | O | Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, @reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>…<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @ |  |  |  |

Table L-2: Syntax of Quality Reporting Scheme Information

<?xml version="1.0"?> <xs:schema targetNamespace="urn:3GPP:ns:PSS:AdaptiveHTTPStreaming:2009:qm" attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns="urn:3GPP:ns:PSS:AdaptiveHTTPStreaming:2009:qm"> <xs:annotation> <xs:appinfo>3GPP DASH Quality Reporting</xs:appinfo> <xs:documentation xml:lang="en"> This Schema defines the quality reporting scheme information for 3GPP DASH. </xs:documentation> </xs:annotation> <xs:element name="ThreeGPQualityReporting" type="SimpleQualityReportingType"/> <xs:complexType name="SimpleQualityReportingType"> <xs:attribute name="samplePercentage" type="xs:double" use="optional"/> <xs:attribute name="reportingInterval" type="xs:unsignedInt" use="optional"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:complexType> </xs:schema>

*************************  End of change 1 *************************