TSG SA4#96 meeting	Tdoc S4-171151

13-17 November 2017, Albuquerque, New Mexico, USA	Agenda Item: 6.2

3GPP TSG-RAN WG2 Meeting #99bis			R2-1712035

Prague, Czech, 9th – 13th Oct 2017

Title:	LS on RAN2 progress of QoE Measurement Collection in LTE

Response to:

Release:	Rel-15

Work Item:	LTE_QMC_Streaming

Source:	TSG RAN2

To:	TSG RAN3, TSG SA4, TSG SA5, TSG CT1

Cc:

Contact Person:

#### Name:	Jun Chen

Tel. Number:	(86) 159 007 97 199

E-mail Address:	jun.chen@huawei.com

Attachments:

1. Overall Description:

At RAN2#98, RAN2#99, and RAN2#99bis meetings, RAN2 discussed the WI QoE Measurement Collection in LTE and made the following agreements:

- Both signalling based QMC initiation and management based QMC initiation should be supported in E-UTRAN

- QMC is only active in E-UTRAN RRC_CONNECTED mode

- Introduce SRB4 and SRB4 can be configured via the RRCConnectionReconfiguration message

- Introduce a new uplink RRC message and the naming could be “application layer measurement report”, and this message uses SRB4. For this new RRC message, it includes a container of report with Octet string (1..8000)

- Introduce a new IE “application layer measurement configuration” in the RRCConnectionReconfiguration message, and this IE includes a container

- If UE AS layer receives the application layer measurement configuration, it shall forward the configuration to upper layers

- If UE AS layer receives a network command to release the application layer measurement configuration, it shall inform upper layers to release the stored application layer measurement configuration

- eNB can inform UE to release the application layer measurement configuration

- Introduce a new UE capability for QMC

2. Actions:

To TSG RAN3:

RAN2 kindly asks RAN3 to take the above information into account in their work on the WI QoE Measurement Collection in LTE.

To TSG SA4 and TSG SA5:

RAN2 kindly asks SA4 and SA5 to take the above information into account and check whether any specification impacts would be required.

To TSG CT1:

RAN2 kindly asks CT1 to take the above information into account and check whether any specification impacts would be required regarding the interaction between Access Stratum layer and upper layer.

3. Date of Next TSG RAN2 Meetings:

TSG RAN2 Meeting #100                        27 Nov – 1 Dec 2017                        Reno, Nevada, USA

TSG RAN2 Meeting #101                        26 Feb – 2 Mar 2018                        Athens, Greece