TSG SA4#97 meeting	Tdoc S4-180057

5-9 February 2018, Fukuoka, Japan	Agenda Item: 5.2

# 3GPP TSG CT WG 1 Meeting 108	TDoc C1-180636

# Gothenburg (Sweden), 22-26 January 2018

Title:	Service types for QMC reporting

Response to:

Release:	Release 15

Work Item:	EQoE_MTSI, UMTS_QMC_Streaming, LTE_QMC_Streaming, QOED

Source:	CT1

To:	SA4, RAN2, RAN6

Cc:	SA5

Contact person:	Atle Monrad

atle.monrad@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

As input to the discussion on service types, CT1 would like to inform that AT-commands and responses were approved to support a generic mechanism within the UE for application level measurement configuration and reporting.

The mechanism can be described as follows:

An AT-command that opens for application level measurement configuration from the lower layers.

+CAPPLEVMC=[<n>]

An unsolicited result code that provides the application level measurement configuration data from the lower layers towards the application.

CAPPLEVMC: <app-meas_service_type>,<start-stop_reporting>[,<app-meas_config_file_length>,<app-meas_config-file>]

An AT-command that provides the application level measurement report from the application in the handset towards the lower layers.

+CAPPLEVMR=<app-meas_service_type>,<app-meas_report_length>,<app-meas_report>

______

This framework assumes that the lower layers includes a service type in the configuration-request towards the TE, in order for the TE to direct the configuration file to the correct application in the UE. In the same way, the network can use the service type to link the measurement report to the correct application when provided from the application in the UE back to the lower layers.

This creates a simple framework where multiple applications (i.e. service types) can configure and report independently and in parallel. The only limitation is that each application (i.e. service type) can have only one measurement ongoing at the time.

# 2	Actions

To SA4, RAN6

ACTION: 	CT1 kindly asks the relevant groups to take the above into account in their future work.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 109	26 Feb - 2 March 2018	Montreal, Canada

TSG CT WG1 Meeting 110	16 - 20 April 2018		Kunming, P.R. of China