# Multi-SDO Project on Converged Management PM Interface Definitions

# Document number:	S5eCPM00XX

# Source:	TEOCO (TM Forum)

# Title:		Collection Method Types – Metadata Properties

# Meeting date:	2014-05-11

# Objective

# As part of a previous contribution, “Performance Collection Method Types”, the various collection method types as defined by several SDOs were analysed. The following contribution is about proposed metadata properties that will support a clear understanding of the nature of a measurement. The need for these properties is a result of the analysis of the collection method types. The agreed properties and values may be merged into a next version of “PM2: Metadefinitions for Measurements and KPIs”.

# References

# Following is a list of references:

- S5eCPM0077, PM2: Metadefinitions for Measurements and KPIs V0.3

- S5eCPM0080, Performance Collection Method Types 1.0, March 2014

# Detailed Proposal

# Based on section 4 of the “Performance Collection Method Types 1.0” contribution, several metadata properties are proposed.

## Collection method Type

The “collectionMethodType” property already appears in section 5 of the “PM2: Metadefinitions for Measurements and KPIs V0.3” document. Based on the recent analysis, the following changes to this property are proposed:

| Converged Property name | Description | Legal values |
| --- | --- | --- |
| collectionMethodType | This property indicates the form in which this measurement data is obtained. CUMULATIVE This literal defines an increasing counter. The NE maintains a running count of the event being counted. The counter is reset to a well-defined value (usually "0") at the beginning of each granularity period. CUMULATIVE_GLOBAL This literal defines an increasing counter. The NE maintains a running count of the event being counted. The value of the counter monotonically increases until it reaches a maximum possible value. Then, it wraps around and starts increasing again from zero. Discontinuities in the monotonically increasing value normally occur at re- initialization of the management system. GAUGE This literal defines a dynamic variable; used when data being measured can vary up or down during the period of measurement. If a gauge is required to produce a consecutive readout over multiple granularity periods (e.g. cabinet temperature), then it shall only be reinitialised at the start of a recording interval. DISCRETE_EVENT This literal defines when data related to a particular event are captured every nth event is registered, where n ≥ 1. STATUS_INSPECTION This literal defines a mechanism for high frequency samplings of internal counters at predefined rates; the output is an array of values. | CUMULATIVE, CUMULATIVE_GLOBAL GAUGE, DISCRETE_EVENT (*), STATUS_INSPECTION (*), (*) – Used for backward compatibility |

Two changes are proposed here

- The value: CUMULATIVE_GLOBAL is proposed. This addition will resolve the ambiguity of 3GPP “COUNTER” and IETF “COUNTER”

- It is suggested to keep “DISCRETE_EVENT” and “STATUS_INSPECTION” mainly for backward compatibility. The case of DISCRETE_EVENT having n > 1 as a sampling rate, might be covered by a different property

## Aggregation Formula

It is proposed to add a new optional property:  “aggregationFormula” that will be applicable only for a measurement that is of a “Gauge” collection method type.

| Converged Property name | Description | Legal values |
| --- | --- | --- |
| aggregationFormula | This property indicates the aggregation formula of a “gauge” collection type measurement in the case when such a formula is used. | MIN, MAX, AVG, FIRST, LAST, OTHER(?) |

## Sampling Rate

It is proposed to add a new optional property:  “samplingRate” that will be applicable only for a measurement that is of a “Gauge” collection method type.

| Converged Property name | Description | Legal values |
| --- | --- | --- |
| samplingRate | This property indicates the sampling rate of a “gauge” collection type measurement in the case when the measurement is generated by sampling. In this case the calculation of the gauge considers every Nth event where N>1. | Number(?) |

# Recommendation

# Proposed decision, action, way forward, etc. - TBD

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"