# Multi-SDO Project on Converged Management PM Interface Definitions

# Document number:	S5eCPM0108

# Source:	WIPRO (TM Forum)

# Title:		PM Data Collection Types

# Meeting date:	2014-09-15

# Objective

# The objective of this contribution is to list various PM Data Collection Types and provide examples and depict the same to further understanding. The basis for the list and description and sample depictions are basically from 3GPP 32.4XX series.   By this it is envisaged that it will be possible to provide better definition for the collection types.

# Also two more Collection Types are suggested for discussion.

# References

# Following is a list of references:

- S5eCPM0103, PM2: Meta definitions for Measurements and KPIs V0.7

- 3GPP TSG 32.425 Telecommunication management; Performance Measurement (PM) ; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)

- 3GPP TSG 32.426 Telecommunication management; Performance Measurement (PM) ; Evolved Packet Core (EPC) Network

- 3GPP TSG 12.04 Technical Specification Group Services and System Aspects; Performance data measurements

- 3GPP TS 32.404 V9.0.0 (2009-12) Technical Specification; Telecommunication management; Performance Management (PM); Performance measurements - Definitions and template

# Detailed Proposal

Reference 1 provides a list of PM data Collection Types and definitions for the same. Following is a list of the types and their descriptions. Subsequent sections depict these collection types. The examples are mostly taken from Ref 2.

PM Data Collection Types; Description; Section. COUNTER; As per Ref 1; 3.1. CUMULATIVE; As per Ref 1; 3.1. GAUGE; As per Ref 1; 3.1. DISCRETE_EVENT; As per Ref 1; 3.2. STATUS_INSPECTION; As per Ref 1; 3.3. VECTOR; A set of ordered values, signifying certain measurement results. The number of values is fixed, and each value is uniquely identifiable. The order is known to the producer and consumer.; 3.4. LIST; A bag of values in no specific order. The specified measurements are independent discrete values.; 3.5.

The last two proposed Collection Types ( VECTOR and LIST) are multi-value  unlike other Collection Types .

## Counters, Cumulative Counters and Gauges

The following figure depicts the above 3 types of counters. However a number of type’s terms Counters and Cumulative Counters are interchangeably used. For example the 3GPP specifies CC (Cumulative Counter) and does not Specify Counters.

Figure 1 Example : Collection Types Counter, Cumulative Counter and Gauge

## Discrete Event Register

The following extract form ref 4 illustrates DER Collection Type

4.2.3	Dedicated bearer set-up time by MME (Mean)

a)	The measurement provides the valid time per dedicated bearer set-up procedure by MME, (unit: second).

b)	DER (n=1)

c)	This measurement is obtained by accumulating the time intervals for every successful dedicated bearer setup by MME between the transmission by the MME of a "ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST" and the corresponding "ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT" message receipt by the MME the over a granularity period using DER. If the dedicated bearer setup procedure is beyond one granularity period, only the set-up time for procedures whose message “ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT” is received in the granularity period can be accumulated.  The end value of this time will then be divided by the number of successful dedicated bearer set-up procedures in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

d)	A single integer value

e)	SM.EstabActDedicatedEpsBearerTimeMean

f)	MMEFunction

g)	Valid for packet switching

h)	EPS

The following extract form ref 4 illustrates DER Collection Type

B.3.1.25	Mean time to provide the CALL SETUP service

A.	This measurement provides the arithmetic mean of the time to provide the CALL SETUP service to a requesting MS.

B.	DER.

C.	This measurement is obtained by accumulating the time intervals between "SETUP" and the corresponding "ASSIGNMENT COMPLETE" messages over a granularity period using DER. This end value of the time will then be divided by the number of call set-ups observed in the granularity period to give the arithmetic mean (3GPP TS 04.08 [2], 3GPP TS 08.08 [5]).

D.	meanTimeToCallSetupService.

E.	A single real value.

F.	MSC Measurement Function.

G.	Valid for circuit switching.

The following figure depicts possible DER counters.

Figure 2 Example : Collection Types Discrete Event Register and List

The above example shows different durations between CALL SETUP Event and ASSIGNMENT COMPLETE Event measured during a time interval. Each of these are independent values. The Producing application needs to keep track of these events as well as subsequently compute.

## Status Inspection

The following extract from ref 3 illustrates an SI Collection Type.

4.2.1	Number of dedicated EPS bearers in active mode (Mean)

a)	This measurement provides the mean number of dedicated EPS bearers.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of dedicated EPS bearer established by MME in active mode and then taking the arithmetic mean.

d)	A single integer value

e)	SM.MeanNbrActDedicatedBearer

f)	MMEFunction

g)	Valid for packet switching

h)	EPS

4.2.2	Number of dedicated EPS bearers in active mode (Maximum)

a)	This measurement provides the maximum number of dedicated EPS bearers in active mode.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of dedicated EPS bearer established by MME in active mode and then taking the.maximum

d)	A single integer value

e)	SM.MaxNbrActDedicatedBearer

f)	MMEFunction

g)	Valid for packet switching

h)	EPS

The following diagram depicts the above performance metric. (The values used in the sample may not be realistic)

Figure 3 Example : Collection Types Status Inspection

This Collection Types enables measuring of a continuously changing performance metric. The metric is computed by taking samples at a certain predetermined frequency and statistical computation is carried out. In the above Session Management performance metrics example the number of dedicated EPS bearers in active mode are sampled every minute over an interval of 1 hour . These set of values are then used compute Mean, Max, Min values .

## Vector

The following extracts from ref 2 illustrate motivation for a Vector collection type. This Collection Type is more of convenience (i.e it is possible to identify and extract PM information using existing Collection Types.

4.2.1.2 Number of initial E-RABs attempted to setup

This measurement provides the number of initial E-RABs attempted to setup. The measurement is split into subcounters per E-RAB QoS level (QCI).

CC

On receipt by the eNodeB/RN of an INITIAL CONTEXT SETUP REQUEST message, each requested E-RAB in the message is added to the relevant measurement per QCI, the possible QCIs are included in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs attempted to setup. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

The measurement name has the form ERAB.EstabInitAttNbr.QCI
where QCI identifies the E-RAB level quality of service class.

4.2.1.2	Number of initial E-RABs successfully established

This measurement provides the number of initial E-RABs successfully established. The measurement is split into subcounters per E-RAB QoS level (QCI).

CC

On transmission by the eNodeB/RN of an INITIAL CONTEXT SETUP RESPONSE message, each E-RAB successfully established is added to the relevant measurement per QCI, the possible QCIs are included in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs successfully setup. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

The measurement name has the form ERAB.EstabInitSuccNbr.QCI
where QCI identifies the E-RAB level quality of service class.

EUtranCellFDD
EUtranCellTDD

Valid for packet switched traffic

EPS

4.2.1.3	Number of initial E-RABs failed to setup

This measurement provides the number of initial E-RABs failed to setup. The measurement is split into subcounters per failure cause.

CC

On transmission by the eNodeB/RN of an INITIAL CONTEXT SETUP RESPONSE, or INITIAL CONTEXT SETUP FAILURE message, each E-RAB failed to establish is added to the relevant measurement per cause, the possible causes are included in TS 36.413 [9]. The sum of all supported per cause measurements shall equal the total number of E-RABs failed to setup. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

The measurement name has the form ERAB.EstabInitFailNbr.Cause
where Cause identifies the cause resulting in the initial E-RAB setup failure.

EUtranCellFDD
EUtranCellTDD

Valid for packet switched traffic

EPS

The following chart provides an example of the above three EUtran counters related E-RAB setup ( the values may not be realistic)..

Figure 3 Example : Collection Types Vector

The above diagram depicts the three counters ERAB.EstabInitAttNbr.QCI, ERAB.EstabInitSuccNbr.QCI and ERAB.EstabInitFailNbr.Cause Code . The last Counter is  a summation counter , rather than a raw counter. Due to this it is not possible for example to what was the major reason for failures corresponding to a specific QCI. (This may not be the best example). By using a vector counter it is possible to show specific values corresponding to each QCI. The number and order of QCI is fixed.

It is possible to express the individual values by having a counter like “ERAB.EstabInitFailNbr.QCI.Cause Code” but is cumbersome.

## List

A list collection type can be a bag of values in any order . These values are not uniquely identifiable also the number of values is not known and not limited.. Fig 2 depicts such a list. It is not possible to transfer such values using current Collection Types.

The performance metric listCallSetUpDurations contains a list of values, each value indicates the duration for setting up the call. The list is not limited; also it is not possible to identify specific duration values.

Statistical math functions like Mean, Max, Min, Standard Deviation / Variance might be used instead. However even with multiple of these there can be loss of information. Also the producer should have the ability to compute and provide resultant value, which may not be possible in simple devices.

# Recommendation

# It is proposed to redefine different collection types and possibly include some of the examples in PM2.

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"