Information Framework

Wireless Network Support and 3GPP Mapping

Release  0.1

TR160

Version Number 0.5

October, 2010

Notice

No recipient of this document shall in any way interpret this document as representing a position or agreement of TM Forum or its members. This document is a draft working document of TM Forum and is provided solely for comments and evaluation. It is not a Forum Approved Document and is solely circulated for the purposes of assisting TM Forum in the preparation of a final document in furtherance of the aims and mission of TM Forum.

Although it is a copyrighted document of TM Forum:

- Members of TM Forum are only granted the limited copyright waiver to distribute this document within their companies and may not make paper or electronic copies for distribution outside of their companies.

- Non-members of the TM Forum are not permitted to make copies (paper or electronic) of this draft document other than for their internal use for the sole purpose of making comments thereon directly to TM Forum.

- If this document forms part of a supply of information in support of an Industry Group Liaison relationship, the document may only be used as part of the work identified in the Liaison and may not be used or further distributed for any other purposes

Any use of this document by the recipient, other than as set forth specifically herein, is at its own risk, and under no circumstances will TM Forum be liable for direct or indirect damages or any costs or losses resulting from the use of this document by the recipient.

This document is governed, and all recipients shall be bound, by all of the terms and conditions of the Intellectual Property Rights Policy of the TM Forum (http://www.tmforum.org/Bylaws/1094/home.html) and may involve a claim of patent rights by one or more TM Forum members or by non-members of TM Forum.

Direct inquiries to the TM Forum office:

240 Headquarters Plaza,

East Tower – 10th Floor,

Morristown, NJ  07960 USA

Tel No.  +1 973 944 5100

Fax No.  +1 973 944 5110

TM Forum Web Page: www.tmforum.orgwww.tmforum.org

Important Disclaimers for use of this document

Certain tools were used to create the illustration of some of the proposed mappings in this document. The use of those tools does not constitute an endorsement of the tool by the TM Forum or its members.

Prior to official publication necessary release of copyright use of all diagrams and screen shots will be secured from the proper parties or the information will be removed from the document. 3GPP has been officially approached regarding the use of their material.

Table of Contents

Notice	2

Important Disclaimers for use of this document	3

Table of Contents	4

List of Figures	6

Executive Summary	8

Information Framework (SID) Support for Wireless Networks	9

1.1.	SID Support for Wireless Networks	9

1.2.	SID Enhancements	13

2.	Mapping SID and 3GPP SubNetwork Models	14

2.1.	SID and 3GPP SubNetwork Mapping	14

2.2.	3GPP Entities Excluded from Mapping	16

2.3.	Mapping Notes	16

2.4.	SID Enhancements	18

3.	Mapping SID to Selected 3GPP IRPs	19

3.1.	Mapping SID to InventoryNRM	19

3.2.	Mapping SID to eUTRAN NRM	21

3.2.1.	Elements in eUTRAN to be mapped:	22

3.2.2.	Detailed attribute mapping	25

3.3.	Mapping of EPC	25

3.4.	Mapping of EUTRAN-EPC S1 links	25

4.	Use case approach for resource modeling	26

4.1.	The Idea	26

4.2.	Alarm Correlation Use Case	26

4.3.	The simplified scenario	26

4.4.	The modeling starting point	29

4.4.1.	3GPP entities – the starting point for UC realization	29

4.4.2.	SID entities – the starting point for UC realization	30

4.5.	Alarm correlation UC realization	32

4.6.	Conclusion	33

5.	Appendix 1 – Detailed Mapping	34

5.1.	Appendix 1.a – Detailed SubNetwork Mapping	34

5.2.	Appendix 1.b: Inventory mapping	42

5.3.	Appendix 1.c: EUtran mapping	44

6.	Appendix B:  Terms and Abbreviations Used within this Document	45

6.1.	Terminology	45

6.2.	Abbreviations and Acronyms	45

7.	Appendix C: References	45

7.1.	References	45

7.2.	IPR Releases and Patent Disclosures	46

8.	Administrative Appendix	47

8.1.	Document History	47

8.1.1.	Version History	47

8.1.2.	Release History	48

8.2.	Company Contact Details	48

8.3.	Acknowledgments	48

List of Figures

Figure 1 – Wireless Network Example	10

Figure 2 – Cell Site Resource Specifications	11

Figure 3 – North London Cell Site	12

Figure 4 – Cell Site Explicit Model	13

Figure 4 – SID SubNetwork Model (Partial View)	15

Figure 5 – SID and 3GPP SubNetwork Model Mapping Example	16

Figure 6 –  Inventory Unit attributes	20

Figure 7 - A Snaphot of mapping	20

Figure 8: Cell view of E-UTRAN NRM	21

Figure 9: Transport view of E-UTRAN NRM	22

Figure 10: Radio equipment view of E-UTRAN NRM	22

Figure 11 Assumed simplified view of the network	27

Figure 12 Simplified layer modeling	28

Figure 13 A fiber outage – alarms being generated	29

Figure 14  3GPP Entities related to the assumed UC – class model	30

Figure 15  The instance diagram corresponding to the UC – no transport entities	30

Figure 16 SID Entities related to the assumed UC – class model	31

Figure 17  The instance diagram corresponding to the UC	31

Figure 18  Alarm correlation based on the resource model	33

Executive Summary

This document covers the TM Forum input to the Resource model alignment activity with 3GPP.  The document covers the current capability of the SID for wireless as well as identifying how the SID can be aligned with the 3GPP Wireless Network Resource Model. Worked use case examples are provided.

Support for wireless networks within the Information Framework (SID) Resource domain is not evident when looking at the detailed model.  This document demonstrates the built-in support provided by the SID to support wireless networks and also defines minor enhancements to the SID to fully provide this support.

There are also questions about the relationship between the SID and the 3GPP deliverables, specifically the Information Object Class model published in 3GPP TS 32.662 V9.1.0 (2010-03) and the relationship between the SID and the content of Integration Reference Points (IRPs).  This document demonstrates the relationships to the IOC model and to selected IRPs.

The Use Case approach is employed to prove the methodology employed and also to help align the outcome to address key aspects of the TOP 10 requirements for the Next Generation of Mobile Networks.

# Information Framework (SID) Support for Wireless Networks

This chapter demonstrates how wireless networks can be supported by the current Information Framework (SID) and identifies minor enhancements to the SID to fully provide this support.

## SID Support for Wireless Networks

While the SID does not ostensibly show components of wireless networks, it certainly can be used to model wireless networks.  This is supported by two patterns used in the SID that allow for the dynamic creation of new types of Resources.  In this case it is the dynamic creation of types of resources that make up wireless networks.

The combination of the Entity Specification / Entity and the Characteristic Specification / Characteristic Value patterns enable dynamic subclasses to be defined.  This capability is a critical property of an information model.  Without it, the model would have to be changed each time a new type of Resource is required.  This alternative, adding a new type of Resource as required to the SID model, is certainly another option for introducing.  It is often referred to as explicit modeling.  The focus of this paper is to show how the SID can be used as-is to support wireless networks, to identify any necessary changes that are needed to do this, and to show examples of explicitly modeling requirements based on the 3GPP models/IRPs.

The SID Resource model contains different types of Resources.  A Physical Resource represents tangible entities, such as equipment and cards.  A Logical Resource presents intangible entities, such as software and logical ports.  A Compound Resource presents an entity that is a combination of two or more Physical and Logical Resources.

An Entity Specification enables the definition of more detailed types of these Resources, such as different types of cell sites (Compound Resource Specification), towers (Physical Resource Specification), and cell site sectors (Logical Resource Specification).  These definitions are used to describe specific Entities, such as the Northeast London cell site (Compound Resource), its associated tower (Physical Resource), and three cell site sectors (Logical Resource).

In Figure 1 – Wireless Network Example the cell sites enclosed in the dashed box will be used to show support for these types of Resources.

Figure 1 – Wireless Network Example

Example mappings between the SID model and selected areas of the 3GPP models in later chapters of this document will show additional support for other parts of a wireless network.

SID Release 9.0 documents (addenda) that may be helpful in understanding the patterns and parts of the Resource areas of the SID referenced in this document are:

- SID Concepts and Principles (GB922CP) for an overview of the SID and its structure

- Root Business Entities (GB922-1R)

  - Section 1.4 – Specification

  - Section 1.13 – Root Business Entities/Characteristic ABE

- Physical Resource Business Entities (GB922-5PR) – Section 1.1 – Physical Resource Domain

- Logical Resource Business Entities (GB922-5LR)

  - Section 1.1 – subsection titled “Logical Resource in More Detail” through end of section

  - Section 1.2 – subsection titled “Introduction” through the subsection on CompoundResources

  - Section 1.2 - subsection titled “ManagedTransmissionEntities” to end of Section 1.2.

The Characteristic Specification entities enable the definition of attributes associated with a specific type of Resource, such as the transmission frequency of the cell site sector and the number of cells supported by a tower.  The Characteristic Value entity enables the value for a certain attribute to be defined.  For example, F1 is the transmission frequency for cell site sector one at Northeast London and the fact that the tower at Northeast London supports three cells.

Figure 1 – Cell Site Resource Specifications depicts the specification of a Cell Site, Cell, and Tower and the associated characteristics mentioned in the example above that allow attributes specific to each to be defined.  The entities (classes) in the figure are specification entities for defining the types of resources.  The next figure will show entities that are described by these specifications for this simple example.

Figure 2 – Cell Site Resource Specifications

The level of granularity shown in the figure is the lowest level of current granularity in the SID for Resource Specifications.  For example, there is no specification for a Network Element (called a Resource Element in the SID).

On the entity side there is more granularity.  For example, in the SID, a NetworkElement is called a Resource Element, which is shown in Figure 2 – North London Cell Site.  Also shown in the figure is a suitable level of granularity for the Tower - Equipment.  Note:  In the following figure the attributes that describe each entity are not shown, but do exist.  For example, Equipment is described by attributes such as height and weight.

Figure 3 – North London Cell Site

The alternative to using the SID as-is by employing its dynamic extensibility is to explicitly model the entities shown in the previous object diagrams.  The equivalent simplistic model is shown in Figure 2 – Cell Site Explicit Model.

Figure 4 – Cell Site Explicit Model

The figure does not show the fact that Cell and CellSiteSector are subclasses of LogicalResource.

It is expected that there will be a requirement that the SID be enhanced to include, as explicit subclasses, some of the subclasses modeled in the 3GPP model/IRPs.

## SID Enhancements

The support shown in the previous section identified some minor updates to the SID.  These are actually changes that should be made to the SID in general and are not specific to wireless support.

The updates include:

- Adding an association that enables one Resource to reference another Resource

- Changing Logical Resource, Equipment, and Resource Element from abstract to concrete to enable the dynamic creation of new types of these entities.

# Mapping SID and 3GPP SubNetwork Models

This chapter and the contents of Appendix 1 – Detailed SubNetwork Mapping demonstrate the close relationship between the two models.  This chapter also includes some minor enhancements to the SID that were identified as the mapping progressed.

## SID and 3GPP SubNetwork Mapping

3GPP TS 32.622.V9.1.0 (2010-03) was the source for the 3GPP SubNetwork model.  SID 9.0 was the source for the SID SubNetwork model.

…Permission from 3GPP is needed to include the figure from 32.622 in section 6.1.21. that shows the UML version of the 3GPP SubNetwork model...

The SID release 9.x working model was the source for the SID SubNetwork model.  A partial view of the model is shown in Figure 3 – SID SubNetwork Model (Partial View).  All related entities used in the mapping are not shown in the figure.

Figure 4 – SID SubNetwork Model (Partial View)

An XML schema mapping tool was used to perform the mapping.  The 3GPP model was developed using an XML schema (XSD) tool.  A modified version of SID 9.0 XSDs was used.  It was modified to support the mapping and to include the SID enhancements identified in this document.

An example of the detailed mapping is shown in Figure 4 – SID and 3GPP SubNetwork Model Mapping Example.  Appendix 1 – Detailed SubNetwork Mapping contains the detailed mapping.

Figure 5 – SID and 3GPP SubNetwork Model Mapping Example

## 3GPP Entities Excluded from Mapping

Five 3GPP entities (classes) were excluded from the mapping, although they do appear in the appendix.  They are Any, IPRAgent, ManagedFunction, ManagedGenericIRP, and EP_RP.

The Any and EP_RP entities was not included because they provide ways to add entities (subclasses) defined in an IRP.  Patterns used in the SID provide a way to dynamically add entities (subclasses) to the SID model as described in Chapter 1 of this document.

The IRPAgent and ManagedFunction entities were excluded because they appear to be more oriented to IRPs and not an information model.  This also appears to be the case for ManagedGenericIRP.  However, this definition of this entity was not included in 3GPP TS 32.622 V9.1.0 (2010-03)

## Mapping Notes

The SubNetwork UML model in 3GPP TS 32.622 V9.1.0 (2010-03) is not specific to wireless networks, but is general in nature, while the IRPs do support wireless networks.

3GPP TS 32.622 V9.1.0 (2010-03) page 12 contains a figure that shows how the vsDataContainer can be used to add vendor-specific attributes to selected entities in the 3GPP SubNetwork model.  The equivalent within the SID is what is called the CharacteristicSpecification/CharacteristicValue pattern.  To demonstrate this, the vsDataContainer associated to the 3GPP SubNetwork entity was mapped to the SID Characteristic entities as an example.  The Characteristic pattern entities are related to the SID Resource entity.  This relationship (association) is inherited by all SID Resource-related entities.

The 3GPP Top entity, which is a super-class for most of the 3GPP entities, was included in the mapping. However, there is a question about it that may need clarification from 3GPP.  The question is about the objectInstance attribute, which from its definition appears to be a duplicate of the existing xxxId attributes for entities in the 3GPP SubNetwork model.

The Link entity in the 3GPP model provides basic support for physical and logical connectivity.  The SID model has a separate entity for physical connectivity and a set of entities that represent logical connectivity.  The SID Pipe entity and its related entities were used to map to the 3GPP Link entity.  It is acknowledged that 3GPP IRPs contain more detailed entities/attributes to support connectivity.  However, these are not in model form, but in interface form.  Therefore, assistance from 3GPP will be needed in order to understand the additional details and to map the content of the IRPs to the SID.

The managedElements in the ManagementNode 3GPP was changed to a reference (association) to ManagedElement in the reverse-engineered schema.

In some cases there are multiple associations, typically one direct and one or more indirect between entities in the 3GPP SubNetwork model.  For example, there is a direct association between SubNetwork and ManagedElement and indirect associations between SubNetwork and ManagedElement via ManagementNode and MeContext.  In these cases, only one mapping was made between SID SubNetwork entities’ attributes and 3GPP SubNetwork entities’ attributes.  The primary SID entity, ResourceElement was mapped directly to ManagedElement for indirect associations, rather than repeat the detailed attribute mappings.

The attribute iRPId defined in 3GPP TS 32.622 V9.1.0 (2010-03) section 6.1.5.1 was not found in any 3GPP SubNetwork model entities.  Because of this it was not mapped to an equivalent SID SubNetwork model attribute.

## SID Enhancements

Some minor enhancements to the SID were identified during the mapping.  The SID XML schema were updated with them to perform the mapping.  They include:

- Adding an association named “ResourceReferences” between Resource and Entity Identification.  This association is necessary to support the MeContext entity in the 3GPP SubNetwork model

- Changing Protocol from abstract to concrete to enable a direct mapping to it and to support the dynamic addition of new types of Protocols to the SID.

Note that more enhancements may be identified when selected IRPs are mapped to the SID.  There is an option in the SID to explicitly model model these requirements as SID entities, attributes, and associations and/or to use SID patterns to dynamically extend the SID as shown in Chapter 1.

The setOfMcc (country codes) attributes was mapped to the SID Country entity Iso3Code attribute.  This may not be correct, requiring the addition of the MobileCountryCode (MCC) as an attribute of Country.

# Mapping SID to Selected 3GPP IRPs

TBD. Besides the TS 32.622 CM/Generic NRM (Subnetwork mapping) above, we may consider other IRP like:

- TS 32.626 Generic Network Resources IRP

- TS 32.692, 32.696 Inventory Management NRM

- TS 32.762 EUTRAN NRM

- TS 32.642 UTRAN relation, TmaFunction, AntenaFunction, UtranCellTDD, UtranCellFDD (for 3G support too)

- TS 32.768 Home eNodeB system NRM

- TS 32.675 Statemanagement IRP

- TS 32.752 for Core (EPC) network (MME, SGW, PGW, PCRF, etc)

- TS 36.410 for S1 link model, S1-U and S1-MME

Optional:

- TS 32.652 GSM relation for 2G support

## Mapping SID to InventoryNRM

3GPP InventoryUnit is mapped to SID’s PhysicalResources, not Resource (which can be logical resource like Cell, Frequency). It’s because we considered that InventoryUnit has such attributes as unitPosition, ManufactureDate… which usually characterizes a (tangible) physical resource.

Figure 6 –  Inventory Unit attributes

InventoryUnit has several attributes which have direct equivalence in SID:

- dateOfManufacture, serialNumber, versionNumber: to SID PhysicalResource’s similar attributes

- vendorUnitFamilyType, vendorUnitTypeNumber, vendorName: to similar attributes of ResourceSpecification of the PhysicalResource

The remaining attributes of InventoryUnit, which do not have equivalence in SID, like dataOfLastService, unitPosition, manufacturerData, are mapped to Characteristics of the PhysicalResource

Figure 7 - A Snaphot of mapping

For detailed mapping, please see Appendix 1.b: Inventory mapping

eference refined lateristicsattributes already exist in SID Characteristicse RAN only.

## Mapping SID to eUTRAN NRM

Here are the object model from 3GPP TS 32.762 V10.0.0

Figure 8: Cell view of E-UTRAN NRM

Figure 9: Transport view of E-UTRAN NRM

Figure 10: Radio equipment view of E-UTRAN NRM

### Elements in eUTRAN to be mapped:

We will consider EUtranNRM.xsd in 3GPP TS 32.762 V10.0.0 and SID’s BusinessViewResource.xsd (modified), and reconcile with Lukasz Mendyk’s work. There are some complexities related to the dynamic extendability approach of SID. But below we try to describe the mapping in a more neutral way, in case later it’s decided if we should create explicitly the subclasses of SID for 3GPP specific model.

The draft mapping need further study:

- SubNetwork: Mapped to SID’s Subnetwork.

A Subnetwork represents a network of one operator in a geographical area. It contains many managed elements, like eNodeB device, EtranCell, etc, which, as below, will be mapped to SID’ various types of logical and physical resources

Question: Should these 3GPP managed elements mapped to elements of SID’s Subnetwork, or LayeredNework?

- ManagedElement: to SID’s ResourceElement, as described in the chapter about Subnetwork mapping

- eNodeBFunction: to SID’s LogicalDeviceRole under ResourceEmement. A physical device can be component of a  containment call Logical Device. The Logical Device can have more than  1 functions, or roles in SID. For example, one device may function as eNodeB and RNC (3G) at the same time (check!)

  - ENBFunction contains references to EUtranCellFDD and TDD, which are not local to it (ENB), but to the Subnetwork. The references are mapped to CharacteristicValue of the LogicalDeviceRole that represents the EnodeBFunction. But the cell objects themselves are mapped to LogicalResource of Subnetwork

    - EUtranCellFDD, in turn, contains EutranRelation. These cell relation is local to the cell, so EutranRelation is mapped to ChracteristicValue of the LogicalResource that represents EUtranCellFDD

      - The attributes of EutranRelation, like tCI, are mapped to CharacteristicValueFor, of the CharacteristicValue above that represents EUtranRelation

  - ENBFunction contains also EP_RP_EPS, the representation of the far end point of its connection to MME. As this characteristic is local to the ENBFunction(which is not a link), we map it to SID’s CharacteristicValue of the LogicalDeviceRole that represents the ENBFunction

- Similarly, BSSFunction, RNCFunction, EPDGFunction, MMEFunction, PCRFFunction, SGWFunction, PGWFunction, ExternalMMEFunction, ExternalENBFunction, ExternalSGWFunction, ExternalRNCFunction, ExternalBSSFunction are mapped to SID’s LogicalDeviceRole under ResourceElement

- EutranGenericCell (actually, for its inherited eUtranCellFDD and eUtranCellTDD):  to SID’s LogicalResource (Lukasz maps it to LogicalDeviceRole, which we may need to discuss as cell is not a kind of device) in the SubNetwork. In direct meaning Cell is a geographical area with a same FrequencyBand). Cell can have indirect relationship with AntennaSector (M:M). As Cell seems not contain inside it other cells, we do not use Composite resource

- Similarly, ExternalEUtranCellFDD, ExternalEUtranCellTDD, ExternalUTranCellFDD, ExternalUTranCellTDDHcr ExternalUTranCellTDDLcr  (high and low chip rate) and ExternalGsmCell are mapped to SID’s LogicalResource

- EutranRelation: This entity describe the relationship between this cell and a set of neigboring cells. It is to SID’s CharacteristicValue of the LogicalResource that represents the Cell.

  - Question: Do we have in SID a particular entity that models the relation between 2 resources? This is also a directional relationship

Similarly to UtranRelation, CDMARelation and GSMRelation

- AntenaFunction: to SID’s PhysicalDeviceRole inside ResourceElement. It’s the management function of ResourceElement for an Antenna

- Similarly, TmaFunction: to SID’s PhysicalDeviceRole inside ResourceElement.

- SectorEquipmentFunction: to SID’s LogicalResource. Note - in 3GPP definition, sector is a set of cells with same Antena and Amplifier. So it’s similar to the concept of Sector in UMTS.

- Link_ENB_MME, Link_ENB_SGW, Link_EPDG_PCRF, Link_EPDG_PGW, Link_HSS_MME, LinkMME_MME, Link_MME_SGSN, Link_MME_SGW, Link_MME_PGW, Link_PCRF_SGW, Link_PCRF_PGW, Link_PGW_SGW, Link_SSGN_SGW: to SID’s Pipe

  - For Link_ENB_ENB, we have question about management purpose, as this link can be dynamic

- MMEPool: This is a list of MME Nodes associated with an area. The realtionship between MME and the node is M:1. Mapped to SID’s LogicalResource under the SubNetwork.

- MMEPoolArea: an area within which an UE may be served without the need to change the serving MME. It contains multiple tracking area (1:M relationship), and contain multiple MME (described by an MME Pool). Mapped to SID’s GeographicArea under the Subnetwork

- QCISet: List of QoS Class Identifier. Each identifier is associated with figure 4 performance characteristics: Resource Type (GBR or Non-GBR); Priority; Packet Delay Budget;	Packet Error Loss Rate. This is attribute of a subnetwork. So QCI set is a list of integer. Mapped to SID’s Logical Resource under the Subnetwork

### Detailed attribute mapping

Please the Appendix 1.c: EUtran mapping. Only mapping for critical elements, as examples, are shown. Mapping for other entities and attributes are similar.

## Mapping of EPC

See the section of EUTranMapping above for MMEFunction, EPDGFunction, SGWFunction, PGWFunction, etc

## Mapping of EUTRAN-EPC S1 links

See EUTranMapping above for Link_ENB_MME, Link_ENB_SGW .

# Use case approach for resource modeling

## The Idea

This concept can be treated as a supplementary approach to the activity based on comparing models class by class. Some benefits of the UC approach include:

- Concentration on the part of the model which serves real Use Cases – immediate demonstration of the added value of the modeling effort

- Easy location of gaps in the model (models) which prohibit effective UC realization

- It is a practical approach for modeling – enables avoiding going into too much detail, which is not necessary for UC realization

- Validates the model from a practical point of view – easier to justify standards amendments when benefits can be demonstrated

- Starting from Fault Management UC – enables coordinating work with FM Harmonization team

- Alarm Correlation FM UC – refers to top 10 NGMN requirements – demonstrating alignment with operator needs

- If this approach works for FM UC, it should be extended for the other top 10 NGMN requirements

## Alarm Correlation Use Case

The Alarm Correlation Use Case refers to the top 10 NGMN requirements defined by mobile operators. The UC presented here assumes the simplest realization as feasible, as the purpose of this document is to validate the UC approach to the modeling.  The outcome of this document does not intend to define the final model, but should be treated as a starting point.

## The simplified scenario

The scenario assumes eNodeBs being connected via mobile backhaul to the Serving Gateway (SGW). In the case of an outage at the transport layer (a fibber outage), several eNodeBs can be cut off from the SGW. This outage may result in alarms being generated by EMSs controlling eNodeBs, the SGW and the transport layer. The purpose of alarm correlation is to correlate these alarms in order to distinguish the root cause, indicating the alarm from symptoms detecting alarms. From the RAN and Radio Core perspective, it means root cause analyses. From a transport perspective, service impact calculation.

Alarm Correlation UC realization assumes leveraging a common network model which will enable automatic realization of this UC, and can also be seen in the context of SON self-healing (self-diagnosing) functions.

The simplified network topology is depicted in Figure 11

Figure 11 Assumed simplified view of the network

The simplified layer modeling is depicted in Figure 12. The layer modeling is an initial idea for aligning RAN and Radio Core entities with the transport layer. In a nutshell, a link_ENB_SGW (S1-U) can be modelled as a trail in the RAN-CORE Layer implemented over the transport layers, finally realised on the physical layer (radio line, fibber, wire). The trail concept seems attractive, as it enables controlling the desired level of details. A trail can be treated as a black box, or is decomposed down to span connections provided by the lower layer trails. Trails can also be used to model management sub-domains.

Figure 12 Simplified layer modeling

In the case of an outage of a fiber, the alarms can be generated by many management entities, both at the transport, RAN and Core Radio network, as symbolically presented in Figure 13

Figure 13 A fiber outage – alarms being generated

## The modeling starting point

This chapter presents only the starting point for modeling. The level of detail has been kept to a minimum -  to entities related to the assumed scenario. It should not be viewed as a completely comprehensive model.

### 3GPP entities – the starting point for UC realization

The starting point for 3GPP classes’ identification is depicted in Figure 14 and Figure 15   The model presented here contains the minimal number of entities which are relevant to the UC. It should not be treated as a full 3GPP model. For example, this model does not include entities referring to antenna, antenna sectors etc.

Figure 14  3GPP Entities related to the assumed UC – class model

Figure 15  The instance diagram corresponding to the UC – no transport entities

### SID entities – the starting point for UC realization

The starting point for SID classes’ identification is depicted in Figure 16. The model presented here contains the minimal number of entities which are relevant to the UC. It should not be treated as a full SID model. For example, this model does not include entities referring to antenna, antenna sectors etc.

Figure 16 SID Entities related to the assumed UC – class model

Figure 17  The instance diagram corresponding to the UC

## Alarm correlation UC realization

The model presented in Figure 16 and Figure 17  is simplified (the final model will need to be elaborated by merging 3GPP and SID entities), but can be used to demonstrate the idea of model versification by UC realization. The UC realization assumes that EMSs (controlling eNodBs, SGW and the transport layer) expose the Inventory API which use the assumed model, each refereeing the appropriate entities which refer to their domains (for example, eNodeB EMS referring to an eNodeB instance). Another assumption is that Fault Management API of EMS also uses the assumed model when generating alarms.

- eNodeB EMS generate alarms indicating that the eNodeB it manages is cut of from the SGW, as for probable cause the link_ENB_SRG  (S1-U:Trail instance) is indicated

- transport EMS generates alarms indicting a fibber outage: The alarm refers to Segment3:Connection instance

- the Alarm correlation function leverages the data Figure 17 i) and infers that Segment3:Connection, is used by S1-U:Trail as a segment in its path. Having this knowledge, the function can correlate the alarm issued by eNodeB EMS and the transport EMS.  That idea is depicted in Figure 18, where 2 alarms eNodeBAlarm and TranportAlarm (object marked in red)  are correlated via resource objects and the relations between them (objects and relation with read shadow): via eNodeb, TrailTerminationPoint, S1-U:Trail, and Segment

Figure 18  Alarm correlation based on the resource model

The aforementioned UC realization, in a nutshell, is planned to be elaborated by operating formally on FM API by modeling alarms strictly on the alarm interface simulating the concrete instances of alarms, with alarm attributes set according to the assumed model.

## Conclusion

The described simplified Alarm Correlation UC realization demonstrates the importance of aligning RAN, Radio Core and transport models for automatic alarm correlation. It also demonstrates the usefulness of the UC approach for validation and synchronization of works done by separate teams (here Resource model team, FM team and Inventory API team).

# Appendix 1 – Detailed Mapping

## Appendix 1.a – Detailed SubNetwork Mapping

This appendix includes the detailed mappings between the SID SubNetwork model and the 3GPP SubNetwork model.  The tool, Altova MapForce, was used to define the mappings and generate the documentation included here.

| Connections | Nodes |
| --- | --- |
|  | File: 3GPP Network Model.xml Type: string |
|  | SubnetworkModel Type: Subnetwork [0..1] |
| core.constant ("SubNetwork"); Direct. | SubnetworkModel/objectClass Type: xs:anyType |
|  | .../objectClass/text() Type: text() |
| .../SIDBusCm:CompositeEntityIdentification/SIDBusCm:value Type: xs:string [0..1]; Direct. | SubnetworkModel/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| SIDBusCm:Subnetwork/SIDBusCm:objectID Type: xs:string [0..1]; Direct. | SubnetworkModel/subNetworkID Type: xs:string |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:ComponentEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | SubnetworkModel/dnPrefix Type: xs:anyType |
|  | .../dnPrefix/text() Type: text() |
| .../SIDBusCm:CommonObjectInfo/SIDBusCm:userLabel Type: xs:string [0..1]; direct. | SubnetworkModel/userLabel Type: xs:anyType |
|  | .../userLabel/text() Type: text() |
| .../SIDBusRe:ResourceSpecification/SIDBusCm:commonName Type: xs:string [0..1]; direct. | SubnetworkModel/userDefinedNetworkType Type: xs:anyType |
|  | .../userDefinedNetworkType/text() Type: text() |
| .../SIDBusRe:Place xsi:type="SIDBusCm:Country"/SIDBusCm:Iso3Code Type: xs:anySimpleType [0..1]; direct. | SubnetworkModel/setOfMCC Type: xs:anyType |
|  | .../setOfMCC/text() Type: text() |
| SIDBusCm:Subnetwork/SIDBusRe:SubNetworkHas Type: SIDBusRe:SubNetwork [0..∞]; direct. | SubnetworkModel/Subnetwork Type: Subnetwork [0..∞] |
|  | .../Subnetwork/objectClass Type: xs:anyType |
|  | .../Subnetwork/objectInstance Type: xs:anyType |
|  | .../Subnetwork/subNetworkID Type: xs:string |
|  | .../Subnetwork/dnPrefix Type: xs:anyType |
|  | .../Subnetwork/userLabel Type: xs:anyType |
|  | .../Subnetwork/userDefinedNetworkType Type: xs:anyType |
|  | .../Subnetwork/setOfMCC Type: xs:anyType |
|  | .../Subnetwork/Subnetwork Type: Subnetwork [0..∞] |
|  | .../Subnetwork/objectClass Type: xs:anyType |
|  | .../Subnetwork/objectInstance Type: xs:anyType |
|  | .../Subnetwork/subNetworkID Type: xs:string |
|  | .../Subnetwork/dnPrefix Type: xs:anyType |
|  | .../Subnetwork/userLabel Type: xs:anyType |
|  | .../Subnetwork/userDefinedNetworkType Type: xs:anyType |
|  | .../Subnetwork/setOfMCC Type: xs:anyType |
|  | .../Subnetwork/Subnetwork Type: Subnetwork [0..∞] |
|  | .../Subnetwork/ManagementNode Type: ManagementNode [0..∞] |
|  | .../Subnetwork/IRPAgent Type: IRPAgent [0..∞] |
|  | .../Subnetwork/MeContext Type: MeContext [0..∞] |
|  | .../Subnetwork/ManagedElement Type: ManagedElement [0..∞] |
|  | .../Subnetwork/Link Type: Link [0..∞] |
|  | .../Subnetwork/Any Type: Any [0..∞] |
|  | .../Subnetwork/VsDataContainer Type: VSDataContainer [0..∞] |
|  | .../Subnetwork/ManagementNode Type: ManagementNode [0..∞] |
|  | .../Subnetwork/IRPAgent Type: IRPAgent [0..∞] |
|  | .../Subnetwork/MeContext Type: MeContext [0..∞] |
|  | .../Subnetwork/ManagedElement Type: ManagedElement [0..∞] |
|  | .../Subnetwork/Link Type: Link [0..∞] |
|  | .../Subnetwork/Any Type: Any [0..∞] |
|  | .../Subnetwork/VsDataContainer Type: VSDataContainer [0..∞] |
| SIDBusCm:Subnetwork/SIDBusRe:LogicalResource xsi:type="SIDBusRe:SoftwareFeatureSets" Type: SIDBusRe:SoftwareFeatureSets; direct. | SubnetworkModel/ManagementNode Type: ManagementNode [0..∞] |
| core.constant ("ManagementNode"); direct. | .../ManagementNode/objectClass Type: xs:anyType |
| .../SIDBusCm:CompositeEntityIdentification/SIDBusCm:value Type: xs:string [0..1]; direct. | .../ManagementNode/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| .../SIDBusRe:LogicalResource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusCm:objectID Type: xs:string [0..1]; direct. | .../ManagementNode/managementNodeID Type: xs:anyType |
|  | .../managementNodeID/text() Type: text() |
| .../SIDBusCm:CommonObjectInfo/SIDBusCm:userLabel Type: xs:string [0..1]; direct. | .../ManagementNode/userLabel Type: xs:anyType |
|  | .../userLabel/text() Type: text() |
| .../SIDBusCm:additionalInfo/SIDBusCm:attributeName Type: xs:string [0..1]; direct. | .../ManagementNode/vendorName Type: xs:anyType |
|  | .../vendorName/text() Type: text() |
| .../SIDBusRe:CommonResourceInfo/SIDBusRe:resourceState Type: SIDBusRe:ResourceState [0..1]; direct. | .../ManagementNode/userDefinedState Type: xs:anyType |
|  | .../userDefinedState/text() Type: text() |
| .../SIDBusRe:Place/SIDBusCm:id Type: xs:anySimpleType [0..1]; direct. | .../ManagementNode/locationName Type: xs:anyType |
|  | .../locationName/text() Type: text() |
| .../SIDBusRe:LogicalResource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusRe:majorVersion Type: xs:string [0..1]; direct. | .../ManagementNode/swVersion Type: xs:anyType |
|  | .../swVersion/text() Type: text() |
| .../SIDBusRe:LogicalResource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusRe:minorVersion Type: xs:string [0..1]; direct. | .../ManagementNode/swVersion (2) Type: xs:anyType |
|  | .../swVersion (2)/text() Type: text() |
| .../SIDBusRe:LogicalResource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusRe:Resource xsi:type="SIDBusRe:ResourceElement" Type: SIDBusRe:ResourceElement; direct. | .../ManagementNode/ManagedElement Type: ManagedElement [0..∞] |
|  | .../ManagedElement/objectClass Type: xs:anyType |
|  | .../ManagedElement/objectInstance Type: xs:anyType |
|  | .../ManagedElement/managedElementID Type: xs:anyType |
|  | .../ManagedElement/dnPrefix Type: xs:anyType |
|  | .../ManagedElement/managedElementType Type: xs:anyType |
|  | .../ManagedElement/userLabel Type: xs:anyType |
|  | .../ManagedElement/vendorName Type: xs:anyType |
|  | .../ManagedElement/userDefinedState Type: xs:anyType |
|  | .../ManagedElement/locationName Type: xs:anyType |
|  | .../ManagedElement/swVersion Type: xs:anyType |
|  | .../ManagedElement/ManagementNode Type: ManagementNode [0..∞] |
|  | .../ManagedElement/IRPAgent Type: IRPAgent [0..∞] |
|  | .../ManagementNode/IRPAgent Type: IRPAgent [0..∞] |
|  | .../IRPAgent/objectClass Type: xs:anyType |
|  | .../IRPAgent/objectInstance Type: xs:anyType |
|  | .../IRPAgent/iRPAgentID Type: xs:anyType |
|  | .../IRPAgent/systemDN Type: xs:anyType |
|  | SubnetworkModel/IRPAgent Type: IRPAgent [0..∞] |
|  | .../IRPAgent/objectClass Type: xs:anyType |
|  | .../IRPAgent/objectInstance Type: xs:anyType |
|  | .../IRPAgent/iRPAgentID Type: xs:anyType |
|  | .../IRPAgent/systemDN Type: xs:anyType |
| .../SIDBusRe:ReferencesEntityIdentification/SIDBusCm:Entity Type: SIDBusCm:Entity [0..1]; direct. | SubnetworkModel/MeContext Type: MeContext [0..∞] |
| core.constant ("MEContext"); direct. | .../MeContext/objectClass Type: xs:anyType |
| .../SIDBusCm:CompositeEntityIdentification/SIDBusCm:value Type: xs:string [0..1]; direct. | .../MeContext/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:CompositeEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../MeContext/meContextID Type: xs:anyType |
|  | .../meContextID/text() Type: text() |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:ComponentEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../MeContext/dnPrefix Type: xs:anyType |
|  | .../dnPrefix/text() Type: text() |
| .../SIDBusRe:ResourceCollection/SIDBusRe:CompoundResource xsi:type="SIDBusRe:ResourceElement" Type: SIDBusRe:ResourceElement; direct. | .../MeContext/ManagedElement Type: ManagedElement [0..1] |
|  | .../ManagedElement/objectClass Type: xs:anyType |
|  | .../ManagedElement/objectInstance Type: xs:anyType |
|  | .../ManagedElement/managedElementID Type: xs:anyType |
|  | .../ManagedElement/dnPrefix Type: xs:anyType |
|  | .../ManagedElement/managedElementType Type: xs:anyType |
|  | .../ManagedElement/userLabel Type: xs:anyType |
|  | .../ManagedElement/vendorName Type: xs:anyType |
|  | .../ManagedElement/userDefinedState Type: xs:anyType |
|  | .../ManagedElement/locationName Type: xs:anyType |
|  | .../ManagedElement/swVersion Type: xs:anyType |
|  | .../ManagedElement/ManagementNode Type: ManagementNode [0..∞] |
|  | .../ManagedElement/IRPAgent Type: IRPAgent [0..∞] |
| .../SIDBusRe:ResourceCollection/SIDBusRe:CompoundResource xsi:type="SIDBusRe:ResourceElement" Type: SIDBusRe:ResourceElement; direct. | SubnetworkModel/ManagedElement Type: ManagedElement [0..∞] |
| core.constant ("ManagedElement"); direct. | .../ManagedElement/objectClass Type: xs:anyType |
| .../SIDBusCm:CompositeEntityIdentification/SIDBusCm:value Type: xs:string [0..1]; direct. | .../ManagedElement/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| .../SIDBusCm:CompositeEntityIdentification/SIDBusCm:value Type: xs:string [0..1]; direct. | .../ManagedElement/managedElementID Type: xs:anyType |
|  | .../managedElementID/text() Type: text() |
| .../SIDBusCm:EntityIdentificationSpecification/SIDBusCm:name Type: xs:string [0..1]; direct. | .../ManagedElement/dnPrefix Type: xs:anyType |
|  | .../dnPrefix/text() Type: text() |
| .../SIDBusRe:ResourceSpecification/SIDBusCm:commonName Type: xs:string [0..1]; direct. | .../ManagedElement/managedElementType Type: xs:anyType |
|  | .../managedElementType/text() Type: text() |
| .../SIDBusCm:CommonObjectInfo/SIDBusCm:userLabel Type: xs:string [0..1]; direct. | .../ManagedElement/userLabel Type: xs:anyType |
|  | .../userLabel/text() Type: text() |
| .../SIDBusCm:additionalInfo/SIDBusCm:attributeName Type: xs:string [0..1]; direct. | .../ManagedElement/vendorName Type: xs:anyType |
|  | .../vendorName/text() Type: text() |
| .../SIDBusRe:CommonResourceInfo/SIDBusRe:resourceState Type: SIDBusRe:ResourceState [0..1]; direct. | .../ManagedElement/userDefinedState Type: xs:anyType |
|  | .../userDefinedState/text() Type: text() |
| .../SIDBusRe:Place/SIDBusCm:id Type: xs:anySimpleType [0..1]; direct. | .../ManagedElement/locationName Type: xs:anyType |
|  | .../locationName/text() Type: text() |
| .../SIDBusRe:Resource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusRe:majorVersion Type: xs:string [0..1]; direct. | .../ManagedElement/swVersion Type: xs:anyType |
|  | .../swVersion/text() Type: text() |
| .../SIDBusRe:Resource xsi:type="SIDBusRe:SoftwareFeatureSets"/SIDBusRe:minorVersion Type: xs:string [0..1]; direct. | .../ManagedElement/swVersion (2) Type: xs:anyType |
|  | .../swVersion (2)/text() Type: text() |
| .../SIDBusRe:CompoundResource xsi:type="SIDBusRe:ResourceElement"/SIDBusRe:Resource xsi:type="SIDBusRe:SoftwareFeatureSets" Type: SIDBusRe:SoftwareFeatureSets; direct. | .../ManagedElement/ManagementNode Type: ManagementNode [0..∞] |
|  | .../ManagementNode/objectClass Type: xs:anyType |
|  | .../ManagementNode/objectInstance Type: xs:anyType |
|  | .../ManagementNode/managementNodeID Type: xs:anyType |
|  | .../ManagementNode/userLabel Type: xs:anyType |
|  | .../ManagementNode/vendorName Type: xs:anyType |
|  | .../ManagementNode/userDefinedState Type: xs:anyType |
|  | .../ManagementNode/locationName Type: xs:anyType |
|  | .../ManagementNode/swVersion Type: xs:anyType |
|  | .../ManagementNode/ManagedElement Type: ManagedElement [0..∞] |
|  | .../ManagementNode/IRPAgent Type: IRPAgent [0..∞] |
|  | .../ManagedElement/IRPAgent Type: IRPAgent [0..∞] |
| SIDBusCm:Subnetwork/SIDBusRe:Pipe Type: SIDBusRe:Pipe [0..∞]; direct. | SubnetworkModel/Link Type: Link [0..∞] |
| core.constant ("Link"); direct. | .../Link/objectClass Type: xs:anyType |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:CompositeEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../Link/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:CompositeEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../Link/linkID Type: xs:anyType |
|  | .../linkID/text() Type: text() |
| .../SIDBusCm:CommonObjectInfo/SIDBusCm:userLabel Type: xs:string [0..1]; direct. | .../Link/userLabel Type: xs:anyType |
|  | .../userLabel/text() Type: text() |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:CompositeEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../Link/aEnd Type: xs:anyType |
|  | .../aEnd/text() Type: text() |
| .../SIDBusCm:EntityIdentification xsi:type="SIDBusCm:CompositeEntityIdentification"/SIDBusCm:value Type: xs:string [0..1]; direct. | .../Link/zEnd Type: xs:anyType |
|  | .../zEnd/text() Type: text() |
| .../SIDBusRe:ResourceSpecification/SIDBusCm:commonName Type: xs:string [0..1]; direct. | .../Link/linkType Type: xs:anyType |
|  | .../linkType/text() Type: text() |
| .../SIDBusRe:ProtocolDeviceInterfaceDetailsForDeviceInterface/SIDBusCm:commonName Type: xs:string [0..1]; direct. | .../Link/protocolName Type: xs:anyType |
|  | .../protocolName/text() Type: text() |
| .../SIDBusRe:ProtocolDeviceInterfaceDetailsForDeviceInterface/SIDBusCm:version Type: xs:string [0..1]; direct. | .../Link/protocolVersion Type: xs:anyType |
|  | .../protocolVersion/text() Type: text() |
|  | SubnetworkModel/Any Type: Any [0..∞] |
|  | .../Any/ManagedFunction Type: ManagedFunction |
|  | .../ManagedFunction/objectClass Type: xs:anyType |
|  | .../ManagedFunction/objectInstance Type: xs:anyType |
|  | .../ManagedFunction/userLabel Type: xs:anyType |
|  | .../ManagedFunction/EP_RP Type: EP_RP [0..∞] |
| .../SIDBusCm:CharacteristicValue/SIDBusCm:CharacteristicSpecification Type: SIDBusCm:CharacteristicSpecification [0..1]; direct. | SubnetworkModel/VsDataContainer Type: VSDataContainer [0..∞] |
| core.constant ("VsDataContainer"); direct. | .../VsDataContainer/objectClass Type: xs:anyType |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:ID Type: xs:string [0..1]; direct. | .../VsDataContainer/objectInstance Type: xs:anyType |
|  | .../objectInstance/text() Type: text() |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:ID Type: xs:string [0..1]; direct. | .../VsDataContainer/vsContainerId Type: xs:anyType |
|  | .../vsContainerId/text() Type: text() |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:valueType Type: xs:string [0..1]; direct. | .../VsDataContainer/vsDataType Type: xs:anyType |
|  | .../vsDataType/text() Type: text() |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:name Type: xs:string [0..1]; direct. | .../VsDataContainer/vsData Type: xs:anyType |
|  | .../vsData/text() Type: text() |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:CharacteristicSpecValue Type: SIDBusCm:CharacteristicSpecValue [0..∞]; direct. | .../VsDataContainer/vsDataFormatVersion Type: xs:anyType |
|  | .../vsDataFormatVersion/text() Type: text() |
| .../SIDBusCm:CharacteristicSpecification/SIDBusCm:CharacteristicSpecificationHas Type: SIDBusCm:CharacteristicSpecification [0..∞]; direct. | .../VsDataContainer/vsDataContainer Type: VSDataContainer [0..∞] |
|  | .../vsDataContainer/objectClass Type: xs:anyType |
|  | .../vsDataContainer/objectInstance Type: xs:anyType |
|  | .../vsDataContainer/vsContainerId Type: xs:anyType |
|  | .../vsDataContainer/vsDataType Type: xs:anyType |
|  | .../vsDataContainer/vsData Type: xs:anyType |
|  | .../vsDataContainer/vsDataFormatVersion Type: xs:anyType |
|  | .../vsDataContainer/vsDataContainer Type: VSDataContainer [0..∞] |

## Appendix 1.b: Inventory mapping

Mapping InventoryNrmToSIDMapping

(InventoryNrmToSIDMapping.mfd)

Output inventoryNrm (inventoryNrm.xsd)

| Connections | Nodes |
| --- | --- |
|  | File: inventoryNrm.xml Type: string |
|  | in:InventoryUnit Type: extension of xn:NrmClass [0..1] |
| SIDBusRe:Subnetwork/SIDBusRe:PhysicalResource/SIDBusCm:objectID Type: xs:string [0..1]; direct Annotation: where ResourceSpecification.commonName="InventoryUnit". | in:InventoryUnit/@id Type: xs:string |
|  | in:InventoryUnit/@modifier Type: restriction of xs:string [0..1] |
|  | in:InventoryUnit/in:attributes Type: restriction of xs:anyType [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusCm:CharacteristicValue/SIDBusCm:value Type: xs:string [0..1]; direct Annotation: where CharacteristicSpecification.name="inventoryUnitType". | in:InventoryUnit/in:attributes/in:inventoryUnitType Type: xs:string [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusRe:ResourceSpecification xsi:type="SIDBusRe:PhysicalResourceSpecAtomic"/SIDBusRe:modelNumber Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:vendorUnitFamilyType Type: xs:string [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusRe:ResourceSpecification xsi:type="SIDBusRe:PhysicalResourceSpecAtomic"/SIDBusRe:partNumber Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:vendorUnitTypeNumber Type: xs:string [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusRe:ResourceSpecification xsi:type="SIDBusRe:PhysicalResourceSpecAtomic"/SIDBusRe:vendorName Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:vendorName Type: xs:string [0..1] |
| SIDBusRe:Subnetwork/SIDBusRe:PhysicalResource/SIDBusRe:serialNumber Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:serialNumber Type: xs:string [0..1] |
| SIDBusRe:Subnetwork/SIDBusRe:PhysicalResource/SIDBusRe:manufactureDate Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:dateOfManufacture Type: xs:date [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusCm:CharacteristicValue/SIDBusCm:value Type: xs:string [0..1]; direct Annotation: where CharacteristicSpecification.name="dateOfLastService". | in:InventoryUnit/in:attributes/in:dateOfLastService Type: xs:date [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusCm:CharacteristicValue/SIDBusCm:value Type: xs:string [0..1]; direct Annotation: where CharacteristicSpecification.name="unitPosition". | in:InventoryUnit/in:attributes/in:unitPosition Type: xs:string [0..1] |
| .../SIDBusRe:PhysicalResource/SIDBusCm:CharacteristicValue/SIDBusCm:value Type: xs:string [0..1]; direct Annotation: where CharacteristicSpecification.name="manufacturerData". | in:InventoryUnit/in:attributes/in:manufacturerData Type: xs:string [0..1] |
| SIDBusRe:Subnetwork/SIDBusRe:PhysicalResource/SIDBusRe:versionNumber Type: xs:string [0..1]; direct. | in:InventoryUnit/in:attributes/in:versionNumber Type: xs:string [0..1] |
|  | in:InventoryUnit/in:InventoryUnit Type: extension of xn:NrmClass [0..∞] |
|  | in:InventoryUnit/in:InventoryUnit/@id Type: xs:string |
|  | in:InventoryUnit/in:InventoryUnit/@modifier Type: restriction of xs:string [0..1] |
|  | in:InventoryUnit/in:InventoryUnit/in:attributes Type: restriction of xs:anyType [0..1] |
|  | in:InventoryUnit/in:InventoryUnit/in:InventoryUnit Type: extension of xn:NrmClass [0..∞] |
|  | in:InventoryUnit/in:InventoryUnit/xn:VsDataContainer Type: extension of xn:NrmClass [0..∞] |
|  | in:InventoryUnit/xn:VsDataContainer Type: extension of xn:NrmClass [0..∞] |
|  | in:InventoryUnit/xn:VsDataContainer/@id Type: xs:string |
|  | in:InventoryUnit/xn:VsDataContainer/@modifier Type: restriction of xs:string [0..1] |
|  | in:InventoryUnit/xn:VsDataContainer/xn:attributes Type: restriction of xs:anyType [0..1] |
|  | .../xn:VsDataContainer/xn:attributes/xn:vsDataType Type: xs:anyType [0..1] |
|  | .../xn:VsDataContainer/xn:attributes/xn:vsDataFormatVersion Type: xs:anyType [0..1] |
|  | .../xn:VsDataContainer/xn:attributes/xn:vsData Type: xn:vsData [0..1] |
|  | in:InventoryUnit/xn:VsDataContainer/xn:VsDataContainer Type: extension of xn:NrmClass [0..∞] |

## Appendix 1.c: EUtran mapping

Please see the embedded table, generated from MapForce.

A snapshot of MapForce used for this mapping:

# Appendix B:  Terms and Abbreviations Used within this Document

## Terminology

| Term | Definition | TMF or Outside Source |
| --- | --- | --- |
| <<BA Specific Term 1>> | <<Definition 1>> | <<Source>> |
| <<BA Specific Term n>> | <<Definition n>> | <<Source>> |

## Abbreviations and Acronyms

To be completed on the next version.

## Appendix C: References

## References

| Reference | Description | Source | Brief Use Summary |
| --- | --- | --- | --- |
| Next Generation Wireless-Wireline Project Charter | Next Generation Wireless-Wireline Project Charter |  | This is one of the programs of the Managing New Technologies initiative, with the main purpose of the project being to deliver operation requirements for converged networks, taking into account existing standards. The next generation wireless-wireline program addresses the management aspects and challenges of hybrid and converged wireless-wireline networks within a single operator and single operations environment. The scope of the project is to define the requirements which are needed to operate a hybrid network end-2-end, with focused on the operations of Services, independent of how the services are realized or implemented. |
| Link To Models | << Hyperlinks to or location of associated models>> |  | <<summary>> |
| Information Framework(SID) Release 9.0 documents (addenda) | Release 9.0 documents (addenda) |  | Addenda of relevance: GB922CB GB9221R GB922PR GB922LR |
| <<Reference n>> | << Type, Title, Number, Revision, Date>> |  | <<summary>> |

## IPR Releases and Patent Disclosures

<<List IPR Releases and Patent Disclosures with any specific wording—show member company or organization and contact name. >>

This document may involve a claim of patent rights by one or more of the contributors to this document, pursuant to the Agreement on Intellectual Rights between the TM Forum and its members.  Interested parties should contact the TM Forum office to obtain notice of current patent rights claims subject to this document.

# Administrative Appendix

This Appendix provides additional background material about the TM Forum and this document. In general, sections may be included or omitted as desired, however a Document History must always be included.

## Document History

### Version History

<This section records the changes between this and the previous document version as it is edited by the team concerned. Note: this is an incremental number which does not have to match the release number and used for change control purposes only>

| Version Number | Date Modified | Modified by: | Description of changes |
| --- | --- | --- | --- |
| 0.1 | Draft inputs collated |  | Not distributed. |
| 0.2 | 17/09/2010 | Marie Murphy/John Reilly/Lukasz Mendyk/Anh Le/ Nigel Davis/ Maksim Karpylev | First Draft for internal team review |
| 0.3 | 29/09/2010 | Marie Murphy | Collated updates provided during review and agreed during review meeting and provided after meeting. |
| 0.4 | 05/10/2010 | Marie Murphy | Added watermark, addressed some errors in document; No substantive changes to the technical content. |
| 0.5 | 06/10/2010 | Marie Murphy/Nigel Davis | Added disclaimers, added some omitted text in section 1.1; Additional administrative updates. |

### Release History

< This section records the changes between this and the previous Official document release. The release number is the ‘Marketing’ number which this version of the document is first being assigned to >

| Release Number | Date Modified | Modified by: | Description of changes |
| --- | --- | --- | --- |
| <<Release Number >> | DD/MMM/YY | <<name>> | Description e.g. first issue of document |

## Company Contact Details

| Company | Team Member Representative |
| --- | --- |
| Include all involved companies adding lines as necessary. | Name Title Email Phone Fax |
|  | Name Title Email Phone Fax |

## Acknowledgments

This document was prepared by the members of a number of TM Forum teams :

Maksim Karpylev, Netcracker

Lukasz Mendyk, Comarch

John Reilly, TM Forum

Nigel Davis, Ciena

Anh Le, Netcracker