| 3GPP TSG-SA5 (Network Management) Meeting #8 Bonn, Germany, 07th – 10th December, 1999 |  | Tdoc S5-99301 |
| --- | --- | --- |

| Title: | Annex B, Section 2 |
| --- | --- |
| Source: | T-Mobil (Karl-Heinz Nenner, Email: karl-heinz.nenner@t-mobil.de) |
| Agenda item: | 9.6 S5.PM Rapporteur Group |
| Document for: | Discussion and Decision |
| Category: | Contribution to 32.104 |
| Document Summary: | The contribution proposes the text for the currently empty Annex B.2 of TS 32.104. |
| Specification(s) involved: | 32.104 |

## B.2.	File transfer procedure

Both push (i.e. triggered by the NE) and pull (triggered by the OS) transfer modes must be supported on the NM interface. Implementation specific means may be employed for the administration and control of the file transfer, concerning

- the time of the transfer (in push mode);

- the routing of the transfer to one or more OS(s) (in push mode);

- the storage/deletion of the files in the NE, particularly when the EM based approach is chosen (cf. subsection B.1.1.1 above).

Measurement result files shall be retained by the file generator (i.e. NE or EM) at least until they have been successfully transferred to or collected by the NM. The storage capacity and the duration for which the data can be retained at the NE or the EM will be Operator and implementation dependent.

The file transfer procedure implemented in the system (NE or EM) shall ensure that no data can get lost under normal operating conditions. The procedure shall also ensure that the files will be deleted after successful transfer to the NM. Depending on the exact implementation of the procedure, the NM may be responsible for deleting those files, or older files will be eventually overwritten by new ones by the file generator in a round robin fashion.

Each implementation must support all primitives of the selected protocol (e.g. put file, get file, inspect directory contents,  delete file) which are needed by the NM. These primitives depend on the details of the procedure, as defined by the manufacturer.