# 3GPP TSG SA WG5 (Telecom Management) Meeting #102	S5-154442

# 24-28 August 2015, Beijing (China)	revision of S5-154439

# Source:	NTT DOCOMO

# Title:	pCR TR 32.842 Add FM procedure flow for correlation in VNFM

# Document for:	Approval

# Agenda Item:	6.5.3 Study on Network Management of Virtualized Networks

# 1	Decision/action requested

Discuss and approve on the text proposal.

# 2	References

[REF-A]	TR 32.842 Telecommunication management; Study on network management of Virtualized Networks.

# 3	Rationale

Based on the use case of failure management when the alarm is generated by VNFC described in subclause 5.1.5 in TR 32.842 [REF-A], it is proposed to add a new FM procedure flow for fault correlation done in VNFM similar to clause 7.3.6 FM procedure flow for correlation done in EM.

# 4	Detailed proposal

It is proposed to make the following changes to TR 32.842 v121 [REF-A].

1st proposed change

### 7.3.X	FM procedure flow for correlation in VNFM

#### 7.3.X.1 	Introduction

In the present FM procedure flow, there are two categories of fault reports that can be correlated at the VNFM:

-	Virtualized resource (VR) fault report

-	VNF/VNFC fault report

The procedure shows correlation at another level other than correlation done in EM (see clause 7.3.6).

Note:	The producer of the VNF/VNFC fault report, i.e., VNF or EM, is to be determined. The following sub-clauses describe the procedure where VNF provides the VNF/VNFC fault report.

#### 7.3.X.2 	VR fault report procedure

1.	The NFVI detects VR failure(s) and reports it to the VIM.

2. 	When the VIM receives the failure report, it will supplement/amend the failure related VR information, create a VR fault report, and send it to the corresponding VNFM.

3. 	The VNFM identifies which VNFs are impacted based on the VR fault report.

#### 7.3.X.3 	VNF/VNFC fault report procedure

4.	The VNF detects failure(s) related to virtualization aspects (e.g., configuration of the VNF deployment specific parameters, interaction of a VNFC with another VNFC, etc) and reports the fault to the VNFM.

#### 7.3.X.4 	Fault correlation done in VNFM

5. 	Based on the VR fault report from the VIM (see step 2 in clause 7.3.X.2) and the VNF/VNFC fault report (see step 4 in clause 7.3.X.3) from the VNF, the VNFM performs a correlation and analysis of the reported faults and their impact. If necessary, the VNFM triggers corrective actions.

6. 	The VNFM creates a VNF virtualization related fault report and sends it to corresponding EM.

EM

NFVI

VNF

VNFM

2. VR fault report

VIM

1. NFVI failure report

6. VNF virtualizationrelated fault report

3. Identify related VNF

5. Fault correlation based on reports from 2 and 4

4. VNF/VNFC fault report

Figure 7.3.X-1: FM procedure flow for correlation done in VNFM.

End