# 3GPP TSG SA WG5 (Telecom Management) Meeting #102	S5-154142

# 24-28 August 2015, Beijing (China)	revision of S5-15xxxx

# Source:	NTT DOCOMO

# Title:	pCR TR 32.842 Add procedures for on-demand healing by EM and auto-healing by VNFM

# Document for:	Approval

# Agenda Item:	6.5.3 Study on Network Management of Virtualized Networks

# 1	Decision/action requested

Discuss and approve on the text proposal.

# 2	References

[REF-A]	TR 32.842 Telecommunication management; Study on network management of Virtualized Networks.

# 3	Rationale

Based on the use case of failure management when the alarm is generated by NFVI (see clause 5.1.3 in [REF-A]) and the use case on failure management when the alarm is generated by VNFC (see clause 5.1.5 in [REF-A]), and the availability of potential requirements related to VNF auto-healing, it is proposed to add the management procedures for on-demand VNF healing by EM and VNF auto-healing by VNFM.

# 4	Detailed proposal

It is proposed to make the following changes to TR 32.842 v121 [REF-A].

1st proposed change

### 7.3.X	FM procedure flows for on-demand healing and auto-healing

#### 7.3.X.1	Procedure flow for on-demand VNF healing by EM

The main steps for the FM procedure flow for on-demand VNF healing by EM are:

1.	VNFM has received VR and VNF-related fault notifications. The VNFM identifies which VNFs are impacted based on such failure reports. No VNF auto-healing by VNFM is performed, e.g., because of unavailability of auto-healing scripts, or because the policy for auto-healing is not active or not applicable.

2.	VNFM creates a VNF-related virtualised resource fault report and sends it to the corresponding EM.

3.	VNF detects the VNF application failure (e.g., virtualised resource failures may lead to a VNF application failure) and sends a fault report to EM.

4.	Based on failure reports from steps 2 and 3, the EM makes correlation and analysis of impact. The EM determines the corrective action to be performed by healing the VNF.

5.	EM requests the VNFM to execute the VNF healing, passing enough information to determine the VNFC(s) that shall be recovered.

6.	Start notifications of the VNF lifecycle operation are sent by the VNFM to subscribed functional blocks, including e.g., EM (refer to step 6a in Figure 7.3.X-1) and NFVO (refer to step 6b in Figure 7.3.X-1).

7.	VNFM, NFVO and VIM execute the steps to terminate the impaired VNFCs, re-create the affected VNFCs, and configure the VNF/VNFCs based on the re-deployed VNFC instances.

8.	End notifications of the VNF lifecycle operation are sent by the VNFM to subscribed functional blocks, including, e.g., EM (refer to step 8a in Figure 7.3.X-1), and NFVO (refer to step 8b in Figure 7.3.X-1).

NFVI

EM

VNFM

NFVO

VIM

2. VNF related VR fault report (VNF specific)

VNF

4. Fault correlation and determination of corrective action through healing

7. VNF healing procedure involving VNFC re-creation, activation and configuration of impaired VNFCs

3. VNF fault report

5. VNF healing request

6a. VNF lifecycle change start notification

6b. VNF lifecycle change start notification

8a. VNF lifecycle change end notification

8b. VNF lifecycle change end notification

1. VNFM handles VR and VNF fault notifications

Figure 7.3.X-1: On-demand VNF healing by EM.

#### 7.3.X.2	Procedure flow for VNF auto-healing by VNFM

The main steps for the FM procedure flow for VNF auto-healing by VNFM are:

1.	VNFM has received VR and VNF-related fault notifications, and it identifies which VNFs are impacted based on such failure reports.

2.	VNFM determines auto-healing to be executed, e.g., because of availability of auto-healing scripts, and the auto-healing policy is active.

3.	Start notifications of the VNF lifecycle operation are sent by the VNFM to subscribed functional blocks, including, e.g., EM (refer to step 3a in Figure 7.3.X-2), and NFVO (refer to step 3b in Figure 7.3.X-2).

4.	VNFM, NFVO and VIM execute the steps to terminate the impaired VNFCs and re-create the affected VNFCs and configure the VNF/VNFCs based on the new re-deployed VNFC instances.

5.	End notifications of the VNF lifecycle operation are sent by the VNFM to subscribed functional blocks, including, e.g., EM (refer to step 5a in Figure 7.3.X-2), and NFVO (refer to step 5b in Figure 7.3.X-2).

NFVI

EM

VNFM

NFVO

VIM

VNF

2. VNFM determines auto-healing to be executed

4. VNF healing procedure involving VNFC re-creation, activation and configuration of impaired VNFCs

3a. VNF lifecycle change start notification

3b. VNF lifecycle change start notification

5a. VNF lifecycle change end notification

5b. VNF lifecycle change end notification

1. VNFM handles VR and VNF fault notifications

Figure 7.3.X-2: Auto-healing of VNF by VNFM.

End