# 3GPP TSG SA WG5 (Telecom Management) Meeting #103	S5-155078

# 12 - 16 October, 2015, Vancouver, Canada	revision of S5-153abc

# Source:	Intel

# Title:	Add specification level PM use case and requirement

# Document for:	Approval

Agenda Item:	6.5.1.1

# 1	Decision/action requested

The group is asked to discuss and agree on the use case and requirements.

# 2	References

[1]	3GPP TS 28.500 “Management concept, architecture and requirements for mobile networks that include virtualized network functions“

# 3	Rationale

This document proposes the specification level PM use cases and requirements for TS 28.500 [1].

# 4	Detailed proposal

1st Modified Section

# 6	Specification level requirements

## 6.1 	Requirements

### 6.1.1 	FM requirements

### 6.1.2 	CM requirements

### 6.1.3 	PM requirements

REQ-MAMO-PM-FUN-1 EM shall be able to receive from VNFM the PM data of the Virtualization Resources used by the VNF managed by EM.

REQ-MAMO-PM-FUN-2 EM should be able to send VNF or VNF related PM data to NM.

REQ-MAMO-PM-FUN-3 EM should ne able to send the notification of VNF related PM data threshold crossing to NM.

2nd Modified Section

## 6.3 	Telecommunications resources

Editor’s note: This clause follows the 3GPP specification template, but it may not be applicable to this document.

## 6.4		Use cases

### 6.4.1 	FM use cases

### 6.4.2 	CM use cases

### 6.4.3 	PM use cases

#### 6.4.3.1 Use Case of VNF related VR performance measurements

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To show how VNF related VR performance measurements can be received. |  |
| Actors and Roles | NM receives the performance measurments. EM administrates, collects, and reports the performance measurements. VNFM produce the VR PM data. |  |
| Telecom resources | NM, EM, VNFM |  |
| Assumptions | - VR is allocated to a VNF. - The VNF is in operation, and is using the VR allocated to the VNF. |  |
| Pre-conditions | A measurement job has been scheduled by EM for the periods for which VR PM data collection is performed. |  |
| Begins when | VNFM generates a VNF related VR PM data. |  |
| Step 1 (M) | VNFM reports the VNF related VR PM data to EM that manages the VNF/VNFC. |  |
| Step 2 (M) | If EM processes the VNF related VR PM data, it may send the threshold crossing notification to NM if necessary; otherwise, it may forwards the VNF related VR PM data to NM. |  |
| Ends when (*) | NM receives the notification or VNF related VR PM data. |  |
| Exceptions |  |  |
| Post-conditions |  |  |
| Traceability (*) | REQ-MAMO-PM-FUN-1, 2, 3 |  |

End of Modified Section