3GPP TR 32.838 V13.1.0 (2016-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;
Telecommunication management;

Gap analysis between 3GPP SA5 specifications and NGMN Top Operational Efficiency (OPE) Recommendations

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Management, Top OPE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	11

3.1	Definitions	11

3.2	Abbreviations	11

4	Quality and quantity of alarms	12

4.1	Overall alarming concept	12

4.2	Alarm quantity	12

4.2.1	General	12

4.2.2	Alarm correlation	13

4.2.3	Number of alarms	13

4.3	Alarm quality	14

4.3.1	Focus on customer and service impact	14

4.3.2	Prioritization of alarms	14

4.3.3	Alarm maintenance manuals	15

4.3.4	Alarm text	15

4.4	Interfaces	15

5	Automatic software management	16

5.1	Short term recommendations	16

5.1.1	Network Element (NE) health-check	16

5.1.2	Automated software download	16

5.1.3	One-click NE software activation	17

5.1.4	Automatic rollback	17

5.2	Long term recommendations	18

6	Energy saving	19

6.1	Recommendations on NE / X2 interface	19

6.2	OSS/EMS recommendations	21

6.3	Change Requests (CRs) related to energy saving work items	28

7	Self Organizing Networks (SON)	30

7.1	OAM Support for SON	30

7.2	Generic optimization	34

7.3	Automatic Neighbour Relation (ANR)	40

7.4	Minimization of Drive Tests (MDT)	45

7.5	Handover (HO) optimization	47

7.6 	Load Balancing (LB)	52

7.7	Cell Outage Compensation (COC)	57

7.8	Common Channel Optimization	61

7.9	Interactions between Home and Macro BTS	63

7.10	SON in CN	68

7.11	QoS optimization	69

8	Performance Management (PM) enhancements	72

8.1	Free configurable measurement and delivery periods	72

8.2	Efficient data transfer mechanism	72

8.3	Enhance Management System (EMS) internal post-processing	72

8.4	Automated counter or counter group administration	73

8.5	Automated performance data quality management	73

8.6	Function for simple threshold based on counters and KPIs	73

8.7	Function for simple KPI calculation based on counters	74

8.8	Automatic identification of network problems and error correction	74

8.9	Standardization of Performance Management (PM)	74

8.10	Real time behaviour of Performance Management (PM)	75

8.11	UE-based measurements	75

9	Enhancement of Trace functionality	76

9.1	Trace on Uu, S1 and X2 interfaces	76

9.2	Support for trouble-shooting, optimization, network wide monitoring and trouble detection and customer specific trouble shooting	76

9.3	UE support for transparent collection and reporting of measurements / events	76

9.4	Interface to external 3rd party systems	77

9.5	Capture and store real-time data across technologies	77

9.6	Successful and unsuccessful calls	77

9.7	High number of users / events	78

9.8	Set call identifying parameter values	78

10	eNodeB Plug & Play - Self commissioning	79

11	OSS Standard Itf-N	84

12	OSS tool support for optimization and operation	89

13	Automatic inventory	89

14	Conclusion	91

Annex A:	Change history	92

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The Next Generation Mobile Networks (NGMN) Alliance is a mobile telecommunications association of mobile operators, vendors, manufacturers and research institutes. It was founded by major mobile operators in 2006 as an open forum to evaluate candidate technologies to develop a common view of solutions for the next evolution of wireless networks. The NGMN Alliance complements and supports standards organizations by providing a coherent view of what mobile operators require.

The NGMN project Operational Efficiency (OPE) has taken the task to elaborate solutions and recommendations for pushing the operational efficiency in NMGN networks and has produced recommendations on standards and implementations. The NGMN OPE project strongly influenced the Top OPE Recommendations [2] in the operational area.

# 1	Scope

The present document provides a gap analysis between NGMN Top OPE Recommendations [2] and OAM capabilities offered by 3GPP SA5 Integration Reference Points (IRPs).

Based on this gap analysis, it provides a support status of SA5 specifications against NGMN Top OPE Recommendations. For each Top OPE Recommendation, the SA5 solution is described when it exists. Otherwise, some action may be considered to address the Top OPE recommendation.

Based on that information, a support status is produced which can take the following values:

- 	Supported,

- 	Partially Supported,

- 	Not Supported,

- 	Not Applicable.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN Top OPE Recommendations V1.0 http://www.ngmn.org/uploads/media/NGMN_Top_OPE_Recommendations_1.0.pdf

[3]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[4]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements"

[5]	3GPP TS 32.102: "Telecommunication management; Architecture".

[6]	3GPP TS 32.111-1: "Telecommunication management; Fault Management; Part 1: 3G fault management requirements"

[7]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)"

[8]	3GPP TS 32.111-6: "Telecommunication management; Fault Management; Part 6: Alarm Integration Reference Point (IRP): Solution Set (SS) definitions"

[9]	3GPP TS 32.121: "Telecommunication management; Advanced Alarm Management (AAM) Integration Reference Point (IRP): Requirements"

[10] 	3GPP TS 32.122: "Telecommunication management; Advanced Alarm Management (AAM) Integration Reference Point (IRP): Information Service (IS)"

[11]	3GPP TS 32.126: "Telecommunication management; Advanced Alarm Management (AAM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[12]	3GPP TS 32.541: "Telecommunication management; Self-Organizing Networks (SON); Self-healing concepts and requirements"

[13]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements"

[14]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)"

[15]	3GPP TS 32.536: "Telecommunication management; Software management Integration Reference Point (IRP); Solution Set (SS) definitions"

[16]	3GPP TS 32.551: "Telecommunication management; Energy Saving Management (ESM); Concepts and requirements"

[17]	3GPP TS 32.522: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[18]	3GPP TS 32.526: "Telecommunication management; Self-Organizing Networks (SON); Policy Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[19]	3GPP TS 32.762: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[20]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[21]	3GPP TS 28.629: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[22]	3GPP TS 32.642: "Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)"

[23]	3GPP TS 32.652: "Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)"

[24]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)"

[25]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements"

[26]	3GPP TS 32.501: "Telecommunication management; Self-configuration of network elements; Concepts and requirements"

[27]	3GPP TS 32.502: "Telecommunication management; Self-configuration of network elements Integration Reference Point (IRP); Information Service (IS)"

[28]	3GPP TS 32.503: "Telecommunication management; Self-configuration of network elements Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)"

[29]	3GPP TS 32.505: "Telecommunication management; Self-configuration of network elements Integration Reference Point (IRP): eXtensible Markup Language (XML) file format definition"

[30]	3GPP TS 32.506: "Telecommunication management; Self-configuration of network elements Integration Reference Point (IRP); Solution Set (SS) definitions"

[31]	3GPP TS 32.507: "Telecommunication management; Self-configuration of network elements Integration Reference Point (IRP): SOAP Solution Set (SS)"

[32]	3GPP TS 28.627: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[33]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2"

[34]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)"

[35]	3GPP TS 32.301: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Requirements"

[36]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)"

[37]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[38]	3GPP TS 32.766: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[39]	3GPP TS 28.659: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[40]	3GPP TS 32.511: "Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements"

[41]	3GPP TS 28.667: "Telecommunication management; Radio Planning Tool Access (RPTA) Integration Reference Point (IRP); Requirements"

[42]	3GPP TS 28.668: "Telecommunication management; Radio Planning Tool Access (RPTA) Integration Reference Point (IRP); Information Service (IS)"

[43]	3GPP TS 28.669: "Telecommunication management; Radio Planning Tool Access (RPTA) Integration Reference Point (IRP); Solution Set (SS) definitions"

[44]	3GPP TS 25.484: "Automatic Neighbour Relation (ANR) for UTRAN; Stage 2"

[45]	3GPP TS 32.761: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[46]	3GPP TS 32.763: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)"

[47]	3GPP TS 32.765: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); eXtensible Markup Language (XML) definitions"

[48]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[49]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements"

[50]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements"

[51]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2"

[52]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification"

[53]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management"

[54]	3GPP TS 32.521: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[55]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS protocol (BSSGP)"

[56]	3GPP TS 28.701: "Telecommunication management; Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[57]	3GPP TS 28.702: "Telecommunication management; Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[58]	3GPP TS 28.703: "Telecommunication management; Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[59]	3GPP TS 28.704: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[60]	3GPP TS 28.705: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[61]	3GPP  TS 28.706: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[62]	3GPP TS 28.707: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[63]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[64]	3GPP TS 28.709: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[65]	3GPP TS 32.411: "Telecommunication management; Performance Management (PM) Integration Reference Point (IRP): Requirements"

[66]	3GPP TS 32.412: "Telecommunication management; Performance Management (PM) Integration Reference Point (IRP): Information Service (IS)"

[67]	3GPP TS 32.416: "Telecommunication management; Performance Management (PM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[68]	3GPP TS 32.431: "Telecommunication management; Performance measurement collection Integration Reference Point (IRP); Requirements"

[69]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition"

[70]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition"

[71]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition"

[72]	3GPP TS 32.341: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Requirements"

[73]	3GPP TS 32.342: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Information Service (IS)"

[74]	3GPP TS 32.346: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP): Solution Set (SS) definitions"

[75]	3GPP TS 32.410: "Telecommunication management; Key Performance Indicators (KPI) for UMTS and GSM"

[76]	3GPP TS 32.450: "Telecommunication management; Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN): Definitions"

[77]	3GPP TS 32.451: "Telecommunication management; Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements"

[78]	3GPP TS 32.454: "Telecommunication management; Key Performance Indicators (KPI) for the IP Multimedia Subsystem (IMS); Definitions"

[79]	3GPP TS 32.455: "Telecommunication management; Key Performance Indicators (KPI) for the Evolved Packet Core (EPC); Definitions"

[80]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements"

[81]	3GPP TS 32.403: "Telecommunication management; Performance Management (PM); Performance measurements; UMTS and combined UMTS/GSM"

[82]	3GPP TS 32.404: "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template"

[83]	3GPP TS 32.405: "Telecommunication management; Performance Management (PM); Performance measurements; Universal Terrestrial Radio Access Network (UTRAN)"

[84]	3GPP TS 32.406: "Telecommunication management; Performance Management (PM); Performance measurements; Core Network (CN) Packet Switched (PS) domain"

[85]	3GPP TS 32.407: "Telecommunication management; Performance Management (PM); Performance measurements; Core Network (CN) Circuit Switched (CS) domain; UMTS and combined UMTS/GSM"

[86]	3GPP TS 32.408: "Telecommunication management; Performance Management (PM); Performance measurements; Teleservice"

[87]	3GPP TS 32.409: "Telecommunication management; Performance Management (PM); Performance measurements; IP Multimedia Subsystem (IMS)"

[88]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements"

[89]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management"

[90]	3GPP TS 32.441: "Telecommunication management; Trace Management Integration Reference Point (IRP); Requirements"

[91]	3GPP TS 32.442: "Telecommunication management; Trace Management Integration Reference Point (IRP); Information Service (IS)"

[92]	3GPP TS 32.446: "Telecommunication management; Trace Management Integration Reference Point (IRP); Solution Set (SS) definitions"

[93]	3GPP TS 52.008: "Telecommunication management; GSM subscriber and equipment trace"

[94]	3GPP TS 32.508: "Telecommunication management; Procedure flows for multi-vendor plug-and-play eNode B connection to the network"

[95]	3GPP TS 32.509: "Telecommunication management; Data formats for multi-vendor plug and play eNode B connection to the network"

[96]	3GPP TS 32.690: "Telecommunication management; Inventory Management (IM); Requirements"

[97]	3GPP TS 32.691: "Telecommunication management; Inventory Management (IM) network resources Integration Reference Point (IRP): Requirements"

[98]	3GPP TS 32.692: "Telecommunication management; Inventory Management (IM) network resources Integration Reference Point (IRP); Network Resource Model (NRM)"

[99]	3GPP TS 32.696: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[100]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[101]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[102]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[103]	3GPP TS 36.921: "Evolved Universal Terrestrial Radio Access (E-UTRA); FDD Home eNode B (HeNB) Radio Frequency (RF) requirements analysis"

[104]	3GPP TS 36.922: "Evolved Universal Terrestrial Radio Access (E-UTRA); TDD Home eNode B (HeNB) Radio Frequency (RF) requirements analysis"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TS 32.150 [3], TS 32.101 [4], TS 32.102 [5] and TS 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TS 32.150 [3], TS 32.101 [4], TS 32.102 [5] and TS 21.905 [1], in that order.

NGMN	Next Generation Mobile Networks (NGMN) Alliance

# 4	Quality and quantity of alarms

## 4.1	Overall alarming concept

Table 4.1

| Top OPE Recommendations Clause 1.3.1: Overall alarming concept "The supplier and their R&D departments are the owner of the detailed system know-how and are responsible for the overall alarming concept. Today the supplier’s development departments for network elements and OSS are working often autonomously in that area. The basis to design an overall alarm concept needs to be established between the different vendor’s product lines before the start of development phase." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | It is an important concept but it does not impact Itf-N standardized interfaces. More product coordination may be needed within vendors but the SA5 standards already define clear rules for alarm semantics. | None | None | Closed |

## 4.2	Alarm quantity

### 4.2.1	General

Table 4.2.1

| Top OPE Recommendations Clause 1.3.2: General "Only alarms that fulfil the quality recommendations and which have an additional benefit to solve abnormal conditions should be forwarded by the NE. Meaningless events must be avoided." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.121 (clauses 4.1, 4.2) [9], 32.122 [10], 32.126 [11] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | It is a requirement on the NE and the EMS, not directly on the Itf-N standardized protocols. Nevertheless on the principle aspect of "Alarm Quantity", the Advanced Alarm Management (AAM) IRP is providing some related capabilities on this topic. | None | None | Closed |

### 4.2.2	Alarm correlation

Table 4.2.2

| Top OPE Recommendations Clause 1.3.2: Alarm correlation "To fulfil the quality requirements and to reduce the event number correlation must be implemented on all levels (network element & element manager). Correlation rules which cover the whole product including all composed components must be part of the product solution and should not be project specific. FM agents on NE level (e.g. IT-systems) should be used to reduce the number of unwanted events. A further correlation of these alarm data in combination with KPI/PM data is needed to give a clear overview of the service related to the whole system environment." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | NE and EMS have to support correlation. Requirements are already defined in the Alarm IRP, but the product may take it as far as they want to. SA5 has created a Rel-10 SI and a Rel-10 WI to add alarm correlation and root cause analysis in TS 32.111-x ( 480045 Study on Alarm Correlation and Alarm Root Cause Analysis and 510041 Alarm Correlation and Root Cause Analysis). | Add concepts for Alarm Correlation and Root Cause Analysis in TS 32. 111-1. Make corresponding CRs on TSs 32.111-2 and 32.111-6. | TS 32. 111-1 CR007 TS 32.111-2 CR0066R1, CR0067R1, CR0068, CR0069 TS 32.111-6 CR0003R1, CR0004R1, CR0005R1 | Closed |
| 2 | Alarm on counter threshold crossing is already supported by SA5 specifications. | None | None | Closed |

### 4.2.3	Number of alarms

Table 4.2.3

| Top OPE Recommendations Clause 1.3.2: Number of alarms "In general per incident there should not be more than 10 alarms on the instance which caused the failure. Alarm floods of instances which did not cause the failure need to be blocked in any case." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status: Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | EMSs should have built in control for alarm flood. Iterators can be used to control the alarms sent. In addition SA5 has created a Rel-10 WI to add alarm correlation and root cause analysis concepts in TS 32.111-1. | Add concepts for Alarm Correlation and Root Cause Analysis in TS 32. 111-1 | TS 32. 111-1 CR007 | Closed |

## 4.3	Alarm quality

### 4.3.1	Focus on customer and service impact

Table 4.3.1

| Top OPE Recommendations Clause 1.3.3: Focus on customer and service impact "In incident situations the following question need to be answered by the system alarms, without the need for any additional optional tooling: "What does this incident mean for the customer and the service at all?"" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status: Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | At DM or NM level, need to identify what is the alarmed entity’s capability in terms of providing service. | Option 1: NM internal function. Option 2: service impact sent on Itf-N and then included in alarm by DM. This item can be revisited at a later date. | None | Closed |
| 2 | Impact on customer could imply more SA5 work. More generally, SA5 has to look at customer experience management, not necessarily FM only. Potential SI. | Customer experience is in the scope of SA5 but there is currently no NM layer activity in SA5. This item can be revisited at a later date. | None | Closed |

### 4.3.2	Prioritization of alarms

Table 4.3.2

| Top OPE Recommendations Clause 1.3.3: Prioritization of alarms "Criteria for Critical alarms: • Total disturbance of the system or significant service impact for customers • Performance, capacity, throughput restrictions • Accounting disturbed Criteria for Major alarms: • Outage of a redundant component (e.g. outage of a redundant power supply) • Introduction of retaliatory actions required, to ensure the service availability" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status: Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Alarm Severity defined in Alarm IRP 32.111-2 follows the ITU-T alarm severity. The severity is currently focused on NE level. A customer level severity could be considered. | Write a CR to add criteria in TS 32.111-1. | TS 32. 111-1 CR0008R1 | Closed |
| 2 | The severity is currently focused on NE level. A customer level severity could be considered. This is linked to clause 4.3.1 item 2. | This item can be revisited at a later date. | None | Closed |

### 4.3.3	Alarm maintenance manuals

Table 4.3.3

| Top OPE Recommendations Clause 1.3.3: Alarm maintenance manuals "Alarm maintenance manuals must contain a clear repair action for the dedicated malfunction. Wherever possible event-based automated repair actions to solve standard error situations without manual interaction should be implemented, if not already implemented on the Network Element level." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8]TS 32.541 [12] |  |  | Support status: Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This is a documentation requirement. Repair action already in alarm definition ("proposedRepairActions"), vendors have to populate this alarm field. | Write a CR on TS 32.111-1 to include the requirement. | TS 32. 111-1 CR0010R1 | Closed |
| 2 | This is a product requirement. | Write a CR on TS 32.111-1 to include the requirement. | TS 32. 111-1 CR0010R1 | Closed |

### 4.3.4	Alarm text

Table 4.3.4

| Top OPE Recommendations Clause 1.3.3: Alarm text "Alarm text should contain description of abnormal condition, probable cause, service impact, root cause and a clear short repair action or reference to online maintenance manual. Meaningless events have to be avoided." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status: Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Additional text, additional info, specific problem, probable cause can be used to determine the root cause of the alarm. Maintenance manuals are provided. These fields are already in alarm definition, vendors have to populate those alarm fields. | LS to NGMN on FM-REQ 7 clarification sent from SA5#84 to NGMN NGCOR (S5-122111) | None | Closed |

## 4.4	Interfaces

See clause 11 "OSS Standard Itf-N".

# 5	Automatic software management

## 5.1	Short term recommendations

### 5.1.1	Network Element (NE) health-check

Table 5.1.1

| Top OPE Recommendations Clause 2.3: NE health-check "OSS system has to be able to verify automatically that network elements are ready for software upgrade. The health-check (e.g. faulty HW Modules, critical alarms, free disk space) has to be executed during the dayshift to ensure the correct behaviour and preconditions of the NE itself." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.531 [13], 32.532 [14], 32.536 [15] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This was not explicitly mentioned in TS 32.53x, but could be easily introduced. Possible solutions could be an additional step/stop point and an extension / addition of a notification. | Provide a CR against TS 32.532 to add a stop point for NE health check. | TS 32.532 CR0027R1 (Rel-10) TS 32.536 CR0033R1 (Rel-10) | Closed |

## 5.1.2	Automated software download

Table 5.1.2

| Top OPE Recommendations Clause 2.3: Automated software download "The software download to the NEs should work in parallel with a minimum of unavoidable manual steps. A result overview list must be provided." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.531 [13], 32.532 [14], 32.536 [15] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The profile configuration in TS 32.53x allows multiple network elements to download the software in parallel. For the network elements which meet the profile condition, they will initiate the software change accordingly. The availability of new software in NE can be monitored by the notification notifyNewSwAvailability (TS 32.532). The result list can be built by NM based on those notifications. Manual operations (downLoadNESw, installNESw, ActivateNESw are also supported (TS 32.532). | None | None | Closed |

## 5.1.3	One-click NE software activation

Table 5.1.3

| Top OPE Recommendations Clause 2.3: One-click NE software activation "Software activation should also work in parallel with a minimum of unavoidable manual steps. The NE health-check should support also the wrap-up activities for urgent issues." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.531 [13], 32.532 [14], 32.536 [15] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The profile configuration in TS 32.53x allows multiple network elements to activate the software in parallel automatically. The resumeSwMProcess operation can work for one-click NE software activation when a stop point has been configured in the profile. Manual operations (downLoadNESw, installNESw, ActivateNESw) can be used for urgent issues on individual NE (TS 32.532). | None | None | Closed |

## 5.1.4	Automatic rollback

Table 5.1.4

| Top OPE Recommendations Clause 2.3: Automatic rollback "Only if the software activations fail completely an automatic rollback should be initiated." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.531 [13], 32.532 [14], 32.536 [15] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | No automatic rollback supported. Manual operation (swFallback) has been designed for individual NE (TS 32.532). Automatic rollback is not desirable due to difficulty to define a valid trigger mechanism and risk of service disruption. | None | None | Closed |

## 5.2	Long term recommendations

Table 5.2

| Top OPE Recommendations Clause 2.3: Long term recommendations "SW package is made available on OSS, and NEs are tagged on OMC for upgrade. Policies for software activation are set. All necessary activities (NE-health check, SW download, SW activation, corrective actions) are carried out policy controlled by the software management application. A final upgrade report is provided that will be used as basis for the final wrap up phase. It is understood that with the long term approach the operator looses detailed control of each single step necessary for a software upgrade. A policy controlled bulk software upgrade is expected to be less error prone than today’s solutions." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.531 [13], 32.532 [14], 32.536 [15] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The profile configuration in TS 32.53x allows operators configured software management policies. | None | None | Closed |
| 2 | The availability of new software in NE can be monitored by the notification notifyNewSwAvailability (TS 32.532). The result list can be built by NM based on those notifications. | None | None | Closed |

# 6	Energy saving

## 6.1	Recommendations on NE / X2 interface

Table 6.1.1

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "The network element shall provide an energy saving mode with minimum power consumption allowing a restart of the network element in less than 5 min triggered via the OAM or X2 interface." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 32.762 [19], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The IOC EnergySavingProperties defined in TS 32.522 allows to control via OAM the energy saving state of a network element supporting the Energy Saving Management functionality. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |
| 2 | X2 is not under SA5 responsibility. | An LS to NGMN is needed | None | Closed |

Table 6.1.2

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "In case of loss of connection of the X2 or OAM interface is detected, the node shall restart without any further trigger." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | NE implementation related, not applicable to OAM standard interface. | An LS to NGMN is needed | None | Closed |

Table 6.1.3

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "The network elements shall be informed about the status of neighbour sites. If additional capacity is needed, neighbour sites in energy saving mode shall be restarted via X2 interface immediately (less than 5 min)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | X2 is not under SA5 responsibility. | An LS to NGMN is needed | None | Closed |

Table 6.1.4

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "Energy saving features shall be considered in other SON use cases (load balancing, cell/service outage detection & compensation, mitigation of unit outage)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Coordination among Cell Outage Compensation, Capacity and Coverage Optimization, and Energy Saving Management is described in TS 32.522. See clause 4.7 about SON coordination. | Rel-11 WI LTE Self-Organizing Networks (SON) coordination management (UID 530051) | See clause 6.3. | Closed |

Table 6.1.5

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "Sites in energy saving mode shall be considered in automatic HO adjustment (via X2)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | X2 is not under SA5 responsibility. | An LS to NGMN is needed | None | Closed |

Table 6.1.6

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "Non-availability of sites due to energy saving mode (node itself and associated NEs) should not be alarmed by the NE." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.551 [16] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-09: When a NE is in energySaving state the IRPAgents shall not consider the NE as a fault, and no alarms shall be raised to the IRPManager for any condition that is a consequence of an energySaving NE (TS 32.551). | None | None | Closed |

Table 6.1.7

| Top OPE Recommendations Clause 3.3.1: Recommendations on NE / X2 interface "The energy saving functionality shall be adequately expandable to 2G / 3G technologies." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Energy Saving use cases and solution allow switching on/off E-UTRAN cells. GERAN and UTRAN networks may only be used for compensation. The IOC interRatEsPolicies defined in TS 32.522 allows to control via OAM inter-RAT energy saving functionality. | Rel-11 WI Inter-RAT Energy Saving Management (UID 540031) | See clause 6.3. | Closed |

## 6.2	OSS/EMS recommendations

Table 6.2.1

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The energy saving functionality shall be controlled by the element manager. A GUI shall be available to de-/activate a static / dynamic energy saving mode of single or groups of nodes incl. definition of time frames when the feature shall be active." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-08: The IRPAgent shall support a capability allowing the IRPManager to initiate energy saving activation/deactivation on one or multiple cells or network elements in the network (TS 32.551). REQ-DIES-FUN-09: The IRPManager shall be able to switch on (= enable) and switch off (= disable) energy saving for a selected part of the network. Switch off energy saving by the IRPManager shall be possible at any time, even if the selected NEs are in energySaving state. REQ-DIES-FUN-10: The IRPAgent should support a capability allowing the IRPManager to configure the time period during which energy saving is allowed. The IOC EnergySavingProperties defined in TS 32.522 allows to control via OAM the energy saving state of a network element supporting Energy Saving Management functionality. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.2

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The system supports automatic detection of low-load periods as basis for operator or automatic decisions on definition of time frames when the feature shall be active." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-14: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes into energySaving state (TS 32.551). REQ-32.551-CON-15: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes out of energySaving state (TS 32.551). This IOC ES Policies from TS 32.522 represents the energy saving policies information. This object class is valid in a distributed ES architecture or in an EM-centralized ES architecture. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.3

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The energy saving functionality shall be supported completely on the Itf-N (BulkCM and / or CLI)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The following IOCs defined in TS 32.522 provide a complete solution for energy saving management on Itf-N: SONControl, ESPolicies, EnergySavingProperties, and interRatEsPolicies. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) Rel-11 WI Inter-RAT Energy Saving Management (UID 540031) | See clause 6.3. | Closed |

Table 6.2.4

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "It shall be possible to configure thresholds and rules of conditions to "switch-on/off" a site automatically." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-14: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes into energySaving state (TS 32.551). REQ-32.551-CON-15: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes out of energySaving state (TS 32.551). This IOC ES Policies from TS 32.522 represents the energy saving policies information. This object class is valid in a distributed ES architecture or in an EM-centralized ES architecture | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.5

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The element management system shall have the actual status of the network element at all time." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-11: The IRPAgent shall allow the IRPManager to query which cells in the network under its domain are in the energySaving state (TS 32.551). REQ-32.551-CON-12: The IRPAgent shall support a capability to notify the IRPManager when a cell goes into or out of energySaving state (TS 32.551). REQ-32.551-CON-13: The IRPAgent shall notify the IRPManager when a cell fails to re-start as a result of going out of energySaving state (TS 32.551). The IOC EnergySavingProperties defined in TS 32.522 allows querying or be notified of the status of a network element supporting Energy Saving Management functionality. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.6

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The de-/activation of other SON features associated with automatic "switch-off" shall be configurable (automatic HO adjustment of neighbour sites, load balancing, cell/service outage detection & compensation, mitigation of unit outage)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Coordination among Cell Outage Compensation, Capacity and Coverage Optimization, and Energy Saving Management is described in TS 32.522. See clause 4.7 about SON coordination. | Rel-11 WI LTE Self-Organizing Networks (SON) coordination management (UID 530051) | See clause 6.3. | Closed |
| 2 | Generic SON coordination solution is described in TS 32.522. However, the models and solution sets are not complete. Whether the generic SON coordination solution can support the coordination between Energy Saving and other SON functions (i.e., automatic HO adjustment of neighbour sites, load balancing, cell/service outage detection) is for future study, which can be considered, depending on proposals from 3GPP members. | None | None | Closed |

Table 6.2.7

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "A failed re-start of a network element shall be alarmed." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-13: The IRPAgent shall notify the IRPManager when a cell fails to re-start as a result of going out of energySaving state (TS 32.551). | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.8

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "Non-availability alarms of sites due to energy saving mode shall be suppressed. This applies not only for alarms of the NE in energy saving mode itself but also alarms of connected NEs shall be avoided (e.g. neighbour-nodes, switches, etc., preferably at NE level)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24], 32.111-1 [6] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-ComES-FUN-05: When a NE is in energySaving state the IRPAgents shall not consider the NE as a fault, and no alarms shall be raised to the IRPManager for any condition that is a consequence of an energySaving NE (TS 32.551). | None | None | Closed |

Table 6.2.9

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "It shall be possible to identify the designated energy saving mode in performance data (preferably at NE level) in order to consider this in KPI calculations (e.g. cell availability)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | A specific sub-cause has been introduced for the definition of performance measurements to differentiate if the cell unavailability or the failure of the RRC connection establishment happens because of Energy Saving. Since Energy Saving Management feature is done by the Operator on purpose, such failures should be distinguishable from other network failures and should be counted separately. With the separate cell unavailability counter due to Energy Saving it is possible to deduct the cell downtime due to Energy Savings from the total cell outage. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.10

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "Under the assumption that the "switch off" of cells is only done if a redundant coverage is given by other cells (of e.g. eventually other collocated RAT) the system supports the import and export of traffic indicators from cells to understand the traffic situation in the cells doing the backup. If the traffic is exceeding a certain operator defined load the system ensures that cells in energy saving mode are activated at once to ensure best customer experience with respect to performance and quality." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | REQ-32.551-CON-14: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes into energySaving state (TS 32.551). REQ-32.551-CON-15: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes out of energySaving state (TS 32.551). REQ-NCES-FUN-04: The IRPAgent shall provide a capability allowing the IRPManager to monitor the network load (TS 32.551). There are related Use Cases defined in TS 32.551: eNB overlaid use case, Capacity-limited network use case. For NM centralized architecture, collection of traffic indicators is done using PM IRP. For distributed or EM centralized architecture, corresponding traffic threshold attributes are defined in 32.522. | Rel-10 WI OAM aspects of Energy Saving in Radio Networks (UID 470037) | See clause 6.3. | Closed |

Table 6.2.11

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "The delivery of these traffic indicators shall be in more real-time than PM to ensure a quick reaction on changed traffic situation in the backing cells." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | It is more efficient not to transport these traffic indicators via Itf-N, but only define policies based on their values. SA5 has defined two corresponding requirements in TS 32.551. REQ-32.551-CON-14: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes into energySaving state. REQ-32.551-CON-15: The IRPAgent should support a capability allowing the IRPManager to configure a cell traffic load threshold to be used for the decision if a network element goes out of energySaving state. | None | None | Closed |

Table 6.2.12

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "Provision of capability indicators indicating which RAT active UE in a cell is capable to support. Idea behind is to identify if upcoming traffic in backing layer is dedicated to a UE which can benefit by switching on the LTE cell once again." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TBD |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There is no cell switch off if the UE cannot handover. See ES probing procedure in TS 32.551. | None | None | Closed |

Table 6.2.13

| Top OPE Recommendations Clause 3.3.2: OSS/EMS recommendations "Energy saving features shall be considered in other SON use cases (Load Balancing, cell/service outage detection & compensation, mitigation of unit outage)" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.551 [16], 32.522 [17], 32.526 [18], 28.628 [20], 28.629 [21], 32.642 [22], 32.652 [23], 32.762 [19], 32.425 [24] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Coordination among Cell Outage Compensation, Capacity and Coverage Optimization, and Energy Saving Management is described in TS 32.522. See clause 4.7 about SON coordination. | Rel-11 WI LTE Self-Organizing Networks (SON) coordination management (UID 530051) | See clause 6.3. | Closed |
| 2 | Generic SON coordination solution is described in TS 32.522. However, the models and solution sets are not complete. Whether the generic SON coordination solution can support the coordination between Energy Saving and other SON functions (i.e., Load Balancing, cell/service outage detection) may be studied depending on proposals from 3GPP members. | None | None | Closed |

## 6.3	Change Requests (CRs) related to energy saving work items

Table 6.3.1

| UID | Name | Acronym | Release | Hyperlink | TSs |
| --- | --- | --- | --- | --- | --- |
| 470037 | OAM aspects of Energy Saving in Radio Networks | OAM-ES | Rel-10 | SP-110128 | 32.425, 32.522, 32.526, 32.626, 32.762, 32.766, new TS 32.551 |

Table 6.3.2

| WID | TS | CR | Rev | Release | Title |
| --- | --- | --- | --- | --- | --- |
| 470037 | 32.425 | 0026 | - | Rel-10 | Modifying RRC establishment failure and Cell unavailability measurements due to Energy Saving |
| 470037 | 32.522 | 0004 | 3 | Rel-10 | Adding NRM for Energy Saving Management policies and ESM switch |
| 470037 | 32.522 | 0013 | 3 | Rel-10 | Use ESPolicies also for EM centralized architecture |
| 470037 | 32.522 | 0014 | - | Rel-10 | Correct cardinality of ESPolicies |
| 470037 | 32.526 | 0002 | 1 | Rel-10 | Network Resource Model (NRM) for Energy Saving Management (ESM) Policies and Switch - Align with 32.522 SON NRM IRP Information Service |
| 470037 | 32.551 | 0001 | 1 | Rel-10 | Modify the energy saving compensation related use cases |
| 470037 | 32.551 | 0002 | 1 | Rel-10 | Add energy saving compensation deactivation requirement |
| 470037 | 32.551 | 0003 | 1 | Rel-10 | Modify the requirements about traffic threshold and time duration for Energy Saving Management (ESM) |
| 470037 | 32.551 | 0004 | - | Rel-10 | Modify errors in state name and in section title |
| 470037 | 32.551 | 0005 | 1 | Rel-10 | Clarify the description of energy saving compensation activation and deactivation over Itf-N |
| 470037 | 32.551 | 0006 | - | Rel-10 | Correct requirements for EM-centralized Energy Saving Architecture |
| 470037 | 32.626 | 0002 | - | Rel-10 | Add containment for object class ESPolicies - Align with 32.522 SON NRM IRP Information Service |
| 470037 | 32.762 | 0038 | 1 | Rel-10 | Introduction of attributes to reflect the status of Energy Saving |
| 470037 | 32.762 | 0042 | 2 | Rel-10 | Adding IOC for energy saving properties |
| 470037 | 32.762 | 0044 | - | Rel-10 | Adding NRM for "candidate cells" in Energy Saving Management (ESM) |
| 470037 | 32.762 | 0050 | 3 | Rel-10 | Add a new attribute into EUtranGenericCell object class to define a cell as not changeable by Energy Saving Management - Align with 32.551 ESM Concepts and requirements |
| 470037 | 32.762 | 0054 | 1 | Rel-10 | Correct ambiguous value usage on energySavingState |
| 470037 | 32.766 | 0013 | 4 | Rel-10 | Add Energy Saving Management (ESM) items for Itf-N - Align with 32.762, 32.551 |

Table 6.3.3

| UID | Name | Acronym | Release | Hyperlink | TSs |
| --- | --- | --- | --- | --- | --- |
| 540031 | Inter-RAT Energy Saving Management | OAM-IRAT-ESM | Rel-11 | SP-110701 | 32.405, 32.551, 32.522, 32.526, 32.642, 32.646, 32.762, 32.766 |

Table 6.3.4

| WID | TS | CR | Rev | Release | Title |
| --- | --- | --- | --- | --- | --- |
| 540031 | 32.405 | 0069 | 1 | Rel-11 | Add energy saving cause to UTRAN measurements |
| 540031 | 32.551 | 0008 | 5 | Rel-11 | Relocate misplaced text for energy saving management concept |
| 540031 | 32.551 | 0009 | 2 | Rel-11 | Add Inter-RAT Energy Saving Management requirements from 32.834 |
| 540031 | 32.551 | 0010 | 1 | Rel-11 | Add Inter-RAT Energy Saving Management concepts from TR 32.834 |
| 540031 | 32.551 | 0011 | 1 | Rel-11 | Add Inter-RAT Energy Saving Management use cases |
| 540031 | 32.522 | 0092 | 1 | Rel-11 | CR R11 32.522 Add time period configuration to allow inter-RAT energy saving |
| 540031 | 32.526 | 0016 | 3 | Rel-11 | CR R11 32.526 Add time period configuration to allow inter-RAT energy saving -Align with 32.522 |
| 540031 | 32.551 | 0019 | 3 | Rel-11 | Rel-11 CR 32.511 Clarify Inter-RAT Energy Saving Management requirement |
| 540031 | 32.522 | 0103 | - | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.526 | 0023 | - | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.551 | 0025 | 2 | Rel-11 | eNB overlaid use case |
| 540031 | 32.642 | 0078 | 1 | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.646 | 0045 | 1 | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.652 | 0038 | 1 | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.656 | 0007 | 1 | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.762 | 0081 | - | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.766 | 0039 | - | Rel-11 | Add support for Inter-RAT Energy Saving Management |
| 540031 | 32.522 | 0120 | 5 | Rel-11 | Add load threshold parameters to Inter-RAT energy saving policy |
| 540031 | 32.522 | 0131 | - | Rel-11 | Enhance description of esNotAllowedTimePeriod |
| 540031 | 32.526 | 0020 | 5 | Rel-11 | Add missing load thresholds for Inter-RAT ES |
| 540031 | 32.551 | 0028 | - | Rel-11 | Cleanup of energy saving management requirements |
| 540031 | 32.551 | 0029 | - | Rel-11 | Correction of references related to inter-RAT energy saving management |
| 540031 | 32.626 | 0003 | 1 | Rel-11 | Add containment for Inter-RAT ES policies |
| 540031 | 32.646 | 0047 | 1 | Rel-11 | Add containment for Inter-RAT ES policies |
| 540031 | 32.656 | 0008 | 1 | Rel-11 | Add containment for Inter-RAT ES policies |
| 540031 | 32.762 | 0084 | 1 | Rel-11 | Remove eNB partial overlay constraint |
| 540031 | 32.766 | 0042 | 1 | Rel-11 | Add containment for Inter-RAT ES policies |

Table 6.3.5

| UID | Name | Acronym | Release | Hyperlink | TSs |
| --- | --- | --- | --- | --- | --- |
| 530051 | LTE Self-Organizing Networks (SON) coordination management | OAM-SON-LTE_COORD | Rel-11 | SP-110637 | 32.425, 32.500, 32.521, 32.522, 32.526, 32.762, 32.766 |

Table 6.3.6

| WID | TS | CR | Rev | Release | Title |
| --- | --- | --- | --- | --- | --- |
| 530051 | 32.522 | 0078 | 1 | Rel-11 | CR R11 TS 32.522 Adding coordination use case among CCO, COC and ESM |
| 530051 | 32.522 | 0107 | 1 | Rel-11 | Coordination between ES and cell compensation |

# 7	Self Organizing Networks (SON)

## 7.1	OAM Support for SON

Abstract of Top OPE Recommendations Clause 4.1: OAM Support for SON:

"SON and related benefits are seen as an essential economical characteristic of LTE strongly asked for by all operators. As a consequence it has to be ensured that operator keep the control on all new SON functionality by implementation of appropriate policy control functions. 
An effective OAM support provides operator with network control in SON trust building and learning phase. It allows in all situations that a very good network quality can be assured."

Table 7.1.1

| "SON functionality / capability shall have controlled implementation in order to build trust and confidence in automation and avoid massive operational impact." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.522 [17] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Controlled implementation, for MRO, in the part of HO parameters modification. The OAM parameters "maximumDeviationHoTrigger" and "minimumTimeBetweenHoTriggerChange" can be used to control modification of the HO parameters during operations of the distributed MRO SON function. | None | TS 32.522, CR0066R2 | Closed |
| 2 | Controlled implementation of the Frequency Domain ICIC For Frequency Domain ICIC, the parameter isICICInformationSendAllowed allows to enable/disable sending ICIC load information message from source cell to target cell. | None | TS 32.762, CR0005 | Closed |
| 3 | Controlled implementation of the Frequency Domain ICIC Whether other parameters for controlling frequency domain ICIC are needed is for future study, which can be considered, depending on proposals from 3GPP members. | None | None | Closed |
| 4 | Controlled implementation of the Time Domain ICIC In view of TS 36.300, section 16.1.5: definition of objects / attributes carrying ABS patterns is missing in SA5 specifications. Whether objects / attributes carrying ABS patterns need to be exposed over the Itf-N interface can be considered, depending on proposals from 3GPP members. | None | None | Closed |
| 5 | Controlled implementation of the Time Domain ICIC Definition of objects / attributes carrying information of association between eNBs is missing, particularly for scenarios outlined in the Annex K of TS 36.300. Further actions depend on proposals from 3GPP members. | None | None | Closed |
| 6 | Controlled implementation of the Time Domain ICIC. In view of TS 36.300, section 16.1.5.2, it should be possible to configure ABS patterns via OAM interface, to be considered as preferable by the eNB for non-member UEs in close proximity of the neighbour CSG cell. Further actions depend on proposals from 3GPP members | None | None | Closed |
| 7 | Controlled implementation of the Time Domain ICIC. In view of TS 36.300, section 16.1.5.2, for CSG cells, it should be possible to configure ABS patterns via OAM interface. Further actions depend on proposals from 3GPP members | None | None | Closed |

Table 7.1.2

| "SON centralized and distributed approach must be supported (depending on the SON use case)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.500 [25], 32.501 [26], 32.502 [27], 32.503 [28], 32.505 [29], 32.506 [30], 32.507 [31], 28.627 [32], 28.628 [20], 28.629 [21], 36.300 [33], 36.423 [34] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP provided specifications for certain centralized and distributed SON functions, however there was no study to identify in which scenarios (SON use cases) centralized / distributed / hybrid approach is the most efficient. Future study can be considered, depending on proposals from 3GPP members. | None | None | Closed |

Table 7.1.3

| "Network and Management System should provide possibility to configure certain break points for SON Operations, allowing the operator for manual intervention to proceed with the logic, or to halt / abort it." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.501 [26], 32.502 [27] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The functionality of break points (“stop points”) is specified in TS 32.501, 32.502, applicable to self-configuration only. | None | None | Closed |

Table 7.1.4

| "Network and Management System shall be synchronized in real time with SON initiated network changes. Notifications shall also be available real-time via the CM Northbound Interfaces to NMS." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.301 [35], 32.302 [36] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | In case when distributed SON is changing a parameter, notifications can be configured via the CM Northbound Interface to inform the NM layer of the change according to TS 32.301, 32.302 and related CM specifications. | None | None | Closed |

Table 7.1.5

| "Network and Management System should provide a valuable Reporting Suite for SON activities." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Operations or interfaces for Reporting Suite SON activities, are not specified | None | None | Closed |

Table 7.1.6

| "Network and Management System shall fully support SON as defined in 3GPP standards, inclusive CM Northbound Interface 3GPP BulkCM IRP (CORBA or SOAP based)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Conformance of Network and Management System implementations to 3GPP standards is out of scope of 3GPP | None | None | Closed |

Table 7.1.7

| "Provide an open Northbound Interface for all SON related parameters for interoperability with 3rd party tools." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.658 [37], 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | A set of parameters was introduced in TS 28.658 clause 6.3.25 which contains the definition of the IOC EUtranCellNMCentralizedSON. All SON parameters defined in SA5 are available over open Northbound Interface. | None | None | Closed |

Table 7.1.8

| "Network and Management System should be able to request or report the SON related changes for statistical analysis and historical view." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.301 [35], 32.302 [36] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | In case when distributed SON is changing a parameter, notifications can be configured via the CM Northbound Interface to inform the NM layer of the change | None | None | Closed |

Table 7.1.9

| "It shall be possible to customize SON policies. On the one hand, there shall be flexibility to adjust the SON functionality to the Operator's recommendations. On the other hand, customization shall be a simple process to minimize the manual effort required" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.628 [20] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Customization of policies for some of SON functionalities is specified in TS 28.628: - Load Balancing Optimization Function - Handover (HO) Parameter Optimization Function - Capacity and Coverage Optimization Function - RACH Optimization Function. | None | None | Closed |
| 2 | Customization of policies for Interference Control Function is not specified in TS 26.628; clause 4 entitled “4.4 Interference Control Function” is empty. SA5 specifications do not include any statement whether the Interference Control Function should have any policies | None | None | Closed |

## 7.2	Generic optimization

Abstract of Top OPE Recommendations Clause 4.2: Generic Optimization: 

"In current 2G / 3G Networks parameter optimization is done manually by analyzing drive-test data and performance measurements. An automated parameter optimization has the possibility to reduce the effort for Network optimization and operations significantly.

Network quality and customer satisfaction will be enhanced. 
Network planning and optimizations efforts can be reduced significantly."

Table 7.2.1

| "SON should support the automatic parameter optimization for the following use case: Automatic optimization of coverage and capacity related parameters in dependency of related KPIs and thresholds." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.522 [17], 28.628 [20] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.522, clause 4.5, outlined the following aspects of the coverage and capacity optimization: - Objective and Targets - Parameters to be optimized - Optimization Method - Symptoms of capacity and coverage optimization problems (as per TS 37.320) such as Coverage hole, Weak coverage, Pilot pollution, Overshoot coverage, DL and UL channel coverage mismatch - Inputs that may be used for the identification of the problem scenarios: - UE measurements - Performance measurements - Alarms, other monitoring information e.g. trace data. | None | TS 32.522: SP-110284 (CR0018R1) | Closed |
| 2 | TR 32.836 provided for the centralized Coverage and Capacity Optimization SON function: - The concept of Function cycle and division into stages - Architecture The following Use Cases were identified - Downlink coverage map - Use case 1: Cell coverage adapting to traffic demand - Use case 2: Coverage and accessibility - Use case 3: LTE coverage holes with underlaid UTRAN/GERAN - Use case 4: LTE Connection failure - Use case 5: Radio link quality The measurements needed for Coverage and Capacity Optimization function are identified. The corresponding specifications are provided in the Rel.13 | None | None | Closed |

Table 7.2.2

| "SON should support the automatic parameter optimization for the following use case: Automatic optimization of QoS and GoS related parameters (i.e. adaption of scheduling and / or RACH parameters) in dependency of related KPIs and thresholds" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.522 [17], 32.425 [24], 32.762 [19], 32.766 [38], 28.628 [20], 28.658 [37], 28.659 [39] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Automatic optimization of QoS and GoS in the part of parameters related to scheduling, currently is not addressed in the standard. | None | None | Closed |
| 2 | The following aspects of the distributed RACH Optimization SON function are outlined in the TS 32.522, clause 4.6: - Objective and Targets - Parameters to be optimized - Optimization Method - Problem Detection (out of scope) - Problem Solution (out of scope) - PM For the Targets the corresponding Information Object Class (IOC) definitions provided in the clause 5.3. | None | TS 32.425: CR0067R1 TS 32.522: CR0010R2, CR0011, CR0003R2 | Closed |
| 3 | For RACH optimization by centralized SON function, certain attributes were introduced to the IOC EUtranCellNMCentralizedSON, TS 32.762, clause 6.5. The attributes are marked as related to RACH optimization in the Table 6.5.1.1 of the TS 32.762: - numberOfRaPreambles - pMax - powerRampingStep - preambleInitialReceivedTargetPower - preambleTransMax - responseWindowSize - rootSequenceIndex - sizeOfRAPreamblesGroupA TS 32.766 specifies the corresponding Solution Set definitions. TS 32.425 specifies PM measurements - Distribution of RACH preambles sent - Distribution of RACH access delay | None | TS 32.762: CR0032R4 TR 32.766: CR0026 TS 32.425: SP-110098 (CR0067R1, CR0010R2, CR0003R2), SP-110093 (CR0046, CR0047, CR0048), SP-110095 (CR0055R9) | Closed |

Table 7.2.3

| "SON should support the automatic parameter optimization for the following use case: Automatic optimization of mobility and handover related parameters (i.e. cell individual offsets, down tilts, Event A related parameters) in dependency of related KPIs." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.522 [17], 32.425 [24], 32.762 [19], 32.766 [38], 28.628 [20], 28.658 [37], 28.659 [39] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.522 clause 4.3 defines the following aspects of the distributed Handover (HO) Parameter Optimization Function - Objective and Targets - Parameters To Be Optimized - Optimization Method - Problem Detection - Problem Solution - Architecture - Definition of Logical Functions - Location of Logical Functions | None | TS 32.522: SP-100833 (CR0002R3) SP-110284 (CR0009R2, CR0066R2 ) SP-110634 (CR0066R1, CR0081R2) SP-110721 (CR0070R1, CR0071R1, CR010, CR0011R1) | Closed |
| 2 | TS 32.762 specifies attributes of the IOC EUtranCellNMCentralizedSON that are relevant to centralized HO optimization, see the list in the clause 6.3.25.2 and Table 6.5.1.1. TS 32.766 specifies the corresponding Solution Set definitions. TS 32.425 in clause 4.3.5 provides a list of relevant performance measurements specified in TS 32.425, clause 4.3. | None | TS 32.762: CR0032R4 TR 32.766: CR0026 TS 32.425: SP-100035 (CR011), SP-100262 (CR019), SP-100488 (CR023), SP-110095 (CR035R1), SP-110095 (CR068R1), SP-110095 (CR069R1), SP-110288 (CR079), SP-110707 (CR0089R1), SP-120355 (CR0106R1), SP-120358 (CR0111R1) | Closed |

Table 7.2.4

| "SON should support the automatic parameter optimization for the following use case: Automatic optimization of cells or services in outage based on an unambiguous detection of this outage." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24], 28.628 [20], 28.658 [37] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | NRM parameters related to Cell Outage Compensation are described in 28.658. Detection of outage is based on PM IRP described in 32.425. Coordination with other SON functions such as Capacity and Coverage Optimization, and Energy Saving Management is addressed in 28.628 | None | None | Closed |
| 2 | Optimization of services in outage is not specified in 3GPP | None | None | Closed |

Table 7.2.5

| "Optimization for identified parameters can be done within a value range, defined by the operator. Note: operator and supplier providing a SON solution have to consider that the configuration of a value range eventually could restrict SON functionality leading to less benefits of such a solution. On the other hand without such a definition of a value range SON functionality could lead to negative impacts. It is in interest of supplier and operator to find for every use case the appropriate compromise in form of a well balanced implementation to meet the targets of a specific SON functionality and to avoid side effects. For field solutions the best fitting value range has to be found and to be set as vendor and operator specific parameter(s) if the definition of such a range is applicable for the dedicated use case." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.522 [17], 28.628 [20], 32.511 [40] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.511 contains definition of the attribute maximumDeviationHoTrigger of the IOC EUtranCellSON, that allows the IRPManager to define the maximum allowed absolute deviation of the cell pair specific part of Handover Trigger, from the default point of operation of MRO distributed SON function. | None | TS 32.522: CR0070R1, | Closed |
| 2 | MLB, ICIC, eICIC, Coverage and Capacity Optimization SON functions do not have standardized OAM parameters that would allow IRPManager to set a value range for the parameters to be optimized. | None | None | Closed |

Table 7.2.6

| "Optimization shall be done with respect to KPIs and parameters not directly related to the use case KPI (i.e. other KPIs shall not become worse than defined thresholds (e.g. handover optimization shall be done with respect to capacity related parameters resp. KPIs)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.522 [17], 28.628 [20], |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The Targets defined in TS 32.522, per SON use case, include only KPIs directly related to the use case: - Load Balancing Optimization Function - Handover (HO) Parameter Optimization Function - Capacity and Coverage Optimization Function - RACH Optimization Function | None | None | Closed |

Table 7.2.7

| "Dependency between KPIs resp. definition which KPIs shall be considered in addition to use case KPI(s) shall be configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There is no standard mechanism for the operator to configure which KPIs shall be considered in addition to use case KPI(s). | None | None | Closed |

Table 7.2.8

| "Thresholds for start and end point of parameter optimization shall be configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.628 [20] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | In MRO, maximumDeviationHoTrigger, specifies the upper and lower thresholds limiting the range of changes of the mobility parameters that can be applied by the MRO function. | None | None | Closed |
| 2 | MLB, ICIC, eICIC, Coverage and Capacity Optimization do not have parameters that would allow to configure the thresholds for start and end point of parameter optimization. Future actions can be considered, depending on proposals from 3GPP members. | None | None | Closed |

Table 7.2.9

| "Optimization cycle should be configurable (periodically, event-based)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There is no standard mechanism for the operator to configure the optimization cycle of Coverage and Capacity Optimization, MLB, MRO, ICIC, RACH. | None | None | Closed |

Table 7.2.10

| "Support of centralized / decentralized solution." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | For HO optimization, EM centralized HOO and distributed HOO solution are specified. | None | None | Closed |
| 2 | NM centralized HOO is not specified. | None | None | Closed |
| 3 | CCO centralized solution is specified; decentralized solution is not specified | None | None | Closed |
| 4 | ANR centralized, decentralized and hybrid solutions specified | None | None | Closed |
| 5 | ICIC decentralized solution is specified; centralized solution is not specified | None | None | Closed |
| 6 | eICIC decentralized solution is specified; centralized solution is not specified | None | None | Closed |
| 7 | MLB decentralized solution is specified; centralized solution is not specified | None | None | Closed |

Table 7.2.11

| "Degree of automation configurable by the operator. o Optimization cycle completely automated: yes / no o Automated import of optimised settings: yes / no" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There is no standard mechanism for the operator to configure the degree of automation. | None | None | Closed |

Table 7.2.12

| "Import / export function of network status with history and fallback solution." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The meaning of this recommendation is not clear. | None | None | Closed |

Table 7.2.13

| "OSS should provide standardized interfaces to planning tools / processes." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 28.667 [41], 28.668 [42], 28.669 [43] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This was addressed by the Work Item OAM_RPTi 630012 "Radio Planning Tool (RPT) interface" providing interface to planning tools. Planning processes are out of scope of SA5. | None | None | Closed |

## 7.3	Automatic Neighbour Relation (ANR)

Abstract of Top OPE Recommendations Clause 4.3: ANR 

"The SON use case Automatic Neighbour Cell Configuration and X2 Setup is defined in the 3GPP Release 8 standards inspired strongly by NGMN recommendations. Based on UE Measurements the eNodeB adapts the NR Table.

ANR Algorithm and decision making is located in eNodeB. ANR has the potential to reduce the effort in network planning and configuration changes related to adaptation of adjacent cells that represents one of the most common operations in planning and optimization processes."

Table 7.3.1

| "Operator expect from ANR within Intra-LTE, Inter-LTE and Inter-RAT for all handover types: As ideal target: Full substitution of initial planning of relationships based on planning tools;" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.511 [40], TS 25.484 [44], TS 32.761 [45], TS 32.762 [19], TS 32.763 [46], TS 32.765 [47] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.511 identified Use Cases and Requirements on ANR function management in UTRAN and E-UTRAN TS 25.484 provided Stage 2 for Intra-UTRAN ANR The following specifications provided solution for ANR for intra-system and inter-system, particularly from E-UTRAN to UTRAN, from E-UTRAN to GERAN, and from E-UTRAN to CDMA2000: - TS 32.761 E-UTRAN Network Resource Model (NRM) Integration Reference Point (IRP); Requirements - TS 32.762 E-UTRAN Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS) - TS 32.763 E-UTRAN Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS) - TS 32.765 E-UTRAN Network Resource Model (NRM) Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition | None | None | Closed |

Table 7.3.2

| "Operator expect from ANR within Intra-LTE, Inter-LTE and Inter-RAT for all handover types: Integration with pre-planning of neighbour relationships based on planning tools;" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.667 [41], 28.668 [42], 28.669 [43] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The Radio Planning Tool (RPT) interface is addressed in the TSs 28.667, 28.668, 28.669. | None | None | Closed |

Table 7.3.3

| "Operator expect from ANR within Intra-LTE, Inter-LTE and Inter-RAT for all handover types: Automatic configuration of neighbour relationships inclusive setup of related X2 interfaces;" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Support of automatic configuration of neighbour relationships is described earlier in this clause. Automatic setup of the X2 interfaces is specified in TS 36.423, clause 8.3.3, including exchange of neighbour information based on the Neighbour Information IE (8.3.3.2). | None | None | Closed |

Table 7.3.4

| "Operator expect from ANR within Intra-LTE, Inter-LTE and Inter-RAT for all handover types: Automatic optimization of neighbour relationships" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.511 [40], TS 25.484 [44], TS 32.761 [45], TS 32.762 [19], TS 32.763 [46], TS 32.765 [47] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The following specifications provide for access to all parameters needed for automatic optimization of neighbour relationships: TS 32.511, TS 25.484, TS 32.761, TS 32.762, TS 32.763, TS 32.765 | None | None | Closed |

Table 7.3.5

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. EMS and OSS should provide a general ANR monitoring & control application covering policy control, history log and switch on/off functionality per site." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.761 [45], TS 32.762 [19], TS 32.763 [46], TS 32.765 [47] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The following specifications provide for access to all parameters needed by an OSS application for ANR monitoring and switch on/off functionality per site: TS 32.761, TS 32.762, TS 32.763, TS 32.765 History log can be implemented in the EM or NM (OSS) application; it is not in the scope of the standardization. | None | None | Closed |
| 2 | Policy control for ANR is not implemented. Future actions can be considered, depending on proposals from 3GPP members. | None | None | Closed |

Table 7.3.6

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. Conditional lists in form of white and black lists as defined by 3GPP shall be stored and configurable within the configuration application / EMS and OSS platform. These lists can be read and configured via the northbound interface in operator’s network management level. The ANR functionality informs directly new identified neighbours to the EMS and the OSS." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.658 [37], 32.511 [40] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.511 provides concepts and requirements for Automatic Neighbour Relation (ANR) management; TS28.658 specifies implementation details for northbound interface including blacklist and whitelist | None | None | Closed |

Table 7.3.7

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. Neighbour cell lists shall be autonomously configured and optimized by the system based on UE measurements according to 3GPP’s ANR, with user setting options like: which UE measurements to use for cell list optimization, forbidden adjacency relations, no handover & no remove attributes, etc." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.761 [45], TS 32.762 [19] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | For ANR, user settings are available for no handover & no remove attributes. | None | None | Closed |
| 2 | There are no settings for selection of which UE measurements to use for cell list optimization. | None | None | Closed |

Table 7.3.8

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. ANR functionality is expected in a way that following handover procedure can be done directly after or "on the fly". This means that the time for relationship identification and configuration inclusive setup of X2 is minimized on less than 2 seconds." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The requirement of X2 setup time is less than 2 seconds depends on the implementation of eNBs and currently is not subject to standardization. | None | None | Closed |

Table 7.3.9

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. The system shall support specific ANR measurements and its configuration separated from specific HO measurements to enable early relationship identification. Target of this is to ensure that the relationship configuration time does not endanger the successful following handover." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.331 [48], 36.214 [49], 36.314 [50] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There are no specific ANR measurements / its configuration separated from specific HO measurements. | None | None | Closed |

Table 7.3.10

| "There shall be fast initial ANR data handling and conditional list implementation, where it is possible to set up a scheme of neighbouring cells over multiple Sites with a minimum of UE initiated traffic and customer impact. To face the risks on issues (like lengthy measurement gaps due to ANR or HO failures and call drops due to missing neighbours) several optional features are asked for. It is underlined that these related recommendations shall not put into question that the above SON characteristics of ANR shall be fulfilled. For LTE->3G and LTE->2G neighbour relation configuration some pre-planned information via northbound interface or available information in a Multi-RAT system (like given relations of collocated cells, scrambling codes or ARFCN of likely neighbours) can be used to mitigate side-effects of time consuming UE measurements in an efficient way." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.761 [45], TS 32.762 [19], TS 32.763 [46], TS 32.765 [47] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Network relations can be fully configured via northbound interface when needed. | None | None | Closed |

Table 7.3.11

| "Network and Management System support for Automatic X2-Setup based on handover-relations." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.762 [19] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The Management System is able to configure Automatic X2-Setup using x2WhiteList, x2BlackList parameters, as per TS 32.762. The two lists can particularly be derived from the neighbour relations table. | None | None | Closed |

Table 7.3.12

| "Network and Management System to be able to configure / manage "no X2 flag", "no remove flag" and "no HO flag" (as opposed to eNodeB only per 3GPP." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.762 [19] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This feature is supported in TS 32.762. | None | None | Closed |

Table 7.3.13

| "Due to missing standardized ANR functionality for the direction from 3G or 2G to LTE the system shall support neighbour relation planning for these directions. Future standardization to cover multi vendor scenarios is asked for. The exchange of neighbour relation lists from planning tools or other EMS via northbound interface shall be supported. Within the Multi-RAT system of one supplier the different RAT neighbour relation information shall be considered to achieve automatic neighbour relations also for 3G->LTE and 2G->LTE directions." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

## 7.4	Minimization of Drive Tests (MDT)

Abstract of Top OPE Recommendations Clause 4.4: Minimization of Drive Tests:

"Network operators strongly rely on manual drive-tests to collect the field measurements that are needed to monitor and optimize the performance of their networks. Drive-tests require a huge effort in terms of resources and time. Moreover, drive test can be usually only be done in specific areas (e.g. roads), whereas users and traffic are also distributed on areas not accessible for drive-tests (e.g. indoor). Therefore, it will be highly beneficial to automate the collection of field measurements and to minimize the need for operators to rely on manual drive-tests.

Drive-tests require a huge effort in terms of resources and time so huge potential is given if typical information can be provided by other methods than by today’s cost driving drive tests."

Table 7.4.1

| "According to the overall technical analysis performed, solutions in support for SDT are feasible and expected in the short term evolution of LTE. It is recommended that NGMN solutions for SDT shall: be able to support both user-plane and control plane architectures, depending on the operator’s deployment strategy." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 37.320 [51], 36.300 [33], 36.331 [48], 25.331 [52], 32.422 [53], 36.214 [49], 36.314 [50] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The MDT specifications in TS 37.320 describe functions and procedures to support collection of UE-specific measurements for MDT using Control Plane architecture, for both UTRAN and E-UTRAN. The specifications also include option of Management based MDT. | None | None | Closed |

Table 7.4.2

| "According to the overall technical analysis performed, solutions in support for SDT are feasible and expected in the short term evolution of LTE. It is recommended that NGMN solutions for SDT shall rely on UE involvement for performing, collecting and reporting measurements to the OAM, for both real-time and non real time-reporting, also providing Location and Time information" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 37.320 [51], 36.300 [33], 36.331 [48], 25.331 [52], 32.422 [53], 36.214 [49], 36.314 [50] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The MDT specifications in TS 37.320 describe functions and procedures to support collection of UE-specific measurements for MDT. The measurements are reported either in real time (Immediate MDT) or not in real time (Logged MDT). Time information is included; location information is included when available. | None | None | Closed |

Table 7.4.3

| "According to the overall technical analysis performed, solutions in support for SDT are feasible and expected in the short term evolution of LTE. It is recommended that NGMN solutions for SDT shall: provide efficient handling of a significant number of UEs and the selection of UE based on capability or user profile" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 37.320 [51], 32.422 [53] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | MDT task can be initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. In this case information from significant number of UEs will be collected. The network may use UE capabilities to select terminals for MDT measurements. | None | None | Closed |

Table 7.4.4

| "According to the overall technical analysis performed, solutions in support for SDT are feasible and expected in the short term evolution of LTE. It is recommended that NGMN solutions for SDT shall: effectively support multi-RAT and multivendor scenarios" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 37.320 [51], 36.300 [33], 36.331 [48], 25.331 [52], 32.422 [53], 36.214 [49], 36.314 [50] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The MDT specifications describe functions and procedures to support collection of UE-specific measurements for MDT in multivendor network for both UTRAN and E-UTRAN. | None | None | Closed |

Table 7.4.5

| "In particular the relevant standardization bodies shall address the following aspects according to the recommendation above: 3GPP RAN: protocol extensions for the control-plane architecture (RRC to configure the UE measurements and events and reporting, both for real time and non-real time; network interfaces in order to support measurement collection and reporting in mobility) SA5: Itf-N enhancements to support SDT management and to retrieve UE measurements, both for a control plane and for a user-plane architecture." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 37.320 [51], 36.300 [33], 36.331 [48], 25.331 [52], 32.422 [53], 36.214 [49], 36.314 [50] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP RAN provided for protocol extensions for the control-plane architecture (RRC to configure the UE measurements and events and reporting, both for real time and non-real time; network interfaces in order to support measurement collection and reporting in mobility) 3GPP SA5 provided for Itf-N enhancements to support SDT management and to retrieve UE measurements, both for a control plane and for a user-plane architecture. | None | None | Closed |

## 7.5	Handover (HO) optimization

Table 7.5.1

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization The evaluation metrics and trigger criteria can be defined and customized in OAM interface." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Target provides a clear basis for assessing performance of self-optimization functions. The evaluation metrics for HO parameter optimization (HOO) is defined, see targets of HOO in TS 32.52X/28.62X. | No need for further action | None | Closed |
| 2 | Different self-optimization algorithms may have different trigger conditions for achieving same objectives and targets. The trigger criteria of HOO are vendor specific, however, one standardized trigger criteria is the target of HOO, i.e., when the monitored HOO target values do not meet the targets preset by operator, the HOO function is triggered. | No need for further action | None | Closed |

Table 7.5.2

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization If the performance does not meet the trigger criteria, stops action; otherwise continues." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Different self-optimization algorithms may have different trigger conditions for achieving same objectives and targets. The trigger criteria of HOO are vendor specific, however, one standardized trigger criteria is the target of HOO. It is a common sense in 3GPP that if the performance does not meet the trigger criteria, then action stops, otherwise continues. | No need for further action | None | Closed |

Table 7.5.3

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization If the performance got worse, reverse action be needed." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Logical function block Optimization Fallback Function is defined for reverse action, i.e., fallback, if the performance got worse. | No need for further action | None | Closed |

Table 7.5.4

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization While evaluating, all related performance metrics should be monitored, other performance metrics should not be deteriorated greatly to improve one performance metric" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Different SON functions may have dependencies with each other. The behaviour of one SON function may have influence on other SON functions. SON coordination allows the prevention and resolution of conflicts between SON functions and related multiple performance metrics. Coordination between HandOver parameter Optimization and Load Balancing was addressed in Release-11. | None | None | Closed |
| 2 | Generic SON coordination solution is described in TS 32.522. However, the models and solution sets are not complete. Whether the generic SON coordination solution can support the coordination between HOO and other SON functions (e.g. Capacity and Coverage Optimization) may be studied depending on proposals from 3GPP members. | None | None | Closed |
| 3 | The case when some SON function is located above Itf-N (e.g. NM Capacity and Coverage Optimization), while another SON function is located below Itf-N (e.g. EM HOO or distributed HOO) and the corresponding SON coordination function is below Itf-N is not addressed in Release-11. | None | None | Closed |

Table 7.5.5

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization If the desired metric got better after optimization action, while others become worse but not enough to trigger action, i.e., it is still in acceptable range, this action should be accepted. Otherwise should be reversed in case the metrics scope within 3GPP are not optimal." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Target provides a clear basis for assessing performance of self-optimization functions. Targets need to be carefully specified in terms of a series of performance measurements and/or KPIs, which can be specific, and which can be used also to identify problems. A target should be expressed in terms of a specific value or specific value range. The specific value or desired value range of each target should be set by operators according to their policy and different network situation. | No need for further action | None | Closed |

Table 7.5.6

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization Some KPIs (defined per neighbour relationship) to be considered for HO optimization performance evaluation could be: Rate of failures related to handover" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Rate of failures related to handover is a standardized target for HOO. | No need for further action | None | Closed |

Table 7.5.7

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization Some KPIs (defined per neighbour relationship) to be considered for HO optimization performance evaluation could be: Rate of failures related to handover without RRC state transition" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21], 32.425 [24] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified. | None | None | Closed |

Table 7.5.8

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization Some KPIs (defined per neighbour relationship) to be considered for HO optimization performance evaluation could be: Rate of failures related to handover with RRC state transition" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21], 32.425 [24] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified. | None | None | Closed |

Table 7.5.9

| "In order to avoid multi-vendor inconsistence, it is necessary to specify the evaluation principles of HO optimization Some KPIs (defined per neighbour relationship) to be considered for HO optimization performance evaluation could be: Statistics of RRC Connection Re-establishment" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Statistics of RRC Connection Re-establishment is an already supported performance measurement. However, it is not specified as one performance measurement of HOO. | None | None | Closed |

Table 7.5.10

| "Optimization for identified parameters can be done within a value range, defined by the operator. (see also note in chapter 4.2.3)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The parameter maximumDeviationHoTrigger allows the operator to define the maximum allowed absolute deviation of the cell pair specific part of Handover Trigger, from the default point of operation. | No need for further action | None | Closed |

Table 7.5.11

| "Optimization shall be done with respect to KPIs and parameters not directly related to the use-case KPI (i.e. other KPIs shall not become worse than defined thresholds (e.g. Handover-Optimization shall be done with respect to capacity related parameters resp. KPIs)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Coordination between HOO and Load Balancing Optimization is specified, see content of SON coordination. | None | None | Closed |
| 2 | Generic SON coordination solution is described in TS 32.522. However, the models and solution sets are not complete. Whether the generic SON coordination solution can support the coordination between HOO and other SON functions may be studied depending on proposals from 3GPP members. | None | None | Closed |

Table 7.5.12

| "Dependency between KPIs resp. definition which KPIs shall be considered in addition to HOO KPI(s) (as e.g. HO Success Rate, Call Drop Rate, Cell individual HO failure rates) shall be configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified | None | None | Closed |

Table 7.5.13

| "Thresholds for start and end point of parameter optimization shall be configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Thresholds for start and end point of parameter optimization are not supported because 3GPP thought that HOO algorithm is vendor specific and the outputs (parameters) are also vendor specific. However, The parameter maximumDeviationHoTrigger allows the operator to define the maximum allowed absolute deviation of the cell pair specific part of Handover Trigger, from the default point of operation. | No need for further action | None | Closed |

Table 7.5.14

| "Optimization cycle should be configurable (periodically, event-based)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified. | None | None | Closed |

Table 7.5.15

| "Support of centralized / decentralized solution." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | EM centralized HOO and distributed HOO solution are specified. | None | None | Closed |
| 2 | NM centralized HOO is not specified. | None | None | Closed |

Table 7.5.16

| "Degree of automation configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The meaning of this recommendation is not clear, an LS to NGMN is needed. | None | None | Closed |

Table 7.5.17

| "Import / Export function of network status with history and fallback solution." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The meaning of this recommendation is not clear, an LS to NGMN is needed. | None | None | Closed |

Table 7.5.18

| "OSS should provide standardized interfaces to planning tools/processes." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.521 [54], 32.522 [17], 32.526 [18], 28.627 [32], 28.628 [20], 28.629 [21] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | SON Policy NRM IRP provides standardized interface to any tool/process which is using Itf-N. | No need for further action | None | Closed |

## 7.6 	Load Balancing (LB)

Abstract of Top OPE Recommendations Clause 4.6: Load Balancing

"Load balancing promises the usage of given redundancy in the network to move load from the capacity restricted resource to these ones which have free capacity by sharing load information and appropriate reaction on this.

Using the given redundancy in overlapping cell areas peak load situation can be handled by load balancing functionality delivering a better customer experience and higher revenue of deployed resources. Secondly necessary capacity based on identified overload can be timely shifted or even avoided."

Table 7.6.1

| "In addition to load indicators already defined in 3GPP, for the inter-RAT Load Balancing use cases, NGMN recommends the introduction of new load indicators or the refinement of the existing ones so that: • Load indicators shall take into account the specificities of source and target RATs so that the load or the available capacity is commonly understood in both RATs" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 48.018 [55], TS 36.423 [34] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Management (RIM) protocol will be used to transfer the information via the core between the base stations of different radio technologies as per TS 48.018. For E-UTRAN, the TS 36.423 in clauses 19.2.2.14/15, defines the corresponding eNB/MME Direct Information Transfer procedures. | None | None | Closed |
| 2 | As indicated in the clause 22.4.1.2.2 of TS 36.423, the Cell Capacity Class value, is intended to provide for inter-RAT comparison of radio capacities against each other. However details of Cell Capacity Class value setting via OAM are not specified. | Provide details of configuration Cell Capacity Class value setting via OAM |  | Open |

Table 7.6.2

| "In addition to load indicators already defined in 3GPP, for the inter-RAT Load Balancing use cases, NGMN recommends the introduction of new load indicators or the refinement of the existing ones so that: • Load indicators shall accurately reflect the load and be unambiguously interpreted in a multi vendor environment." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 36.423, clause 9.2.37, defines X2 Radio Resource Status IE and Composite Available Capacity Group. | None | None | Closed |

Table 7.6.3

| "In addition to load indicators already defined in 3GPP, for the inter-RAT Load Balancing use cases, NGMN recommends the introduction of new load indicators or the refinement of the existing ones so that: • For load indicators report over the north bound interface, load indicators shall be defined such as understandable by 3rd parties SON servers." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34], 32.425 [24] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The load indicators specified in TS 36.423 within Radio Resource Status IE, are similar to the information specified with the following difference: - The Radio Resource Status IE carries information of immediate real time values, with Reporting Periodicity configured as one of the following: 1000ms, 2000ms, 5000ms, 10000ms - The PRB utilization counters are accumulated over comparatively long time interval, typically 5 min or 15 min | None | None | Closed |
| 2 | The following load indicators specified in TS 36.423 - Hardware Load Indicator - S1 TNL Load Indicator - Composite Available Capacity Group - ABS Status are not reported over the north bound interface. Study can be considered which load indicators defined in TS 36.423 can be reported over the north bound interface, depending on proposals from 3GPP members. | None | None | Closed |

Table 7.6.4

| "In addition to load indicators, reliable protocols and procedures for load information exchange between RATs shall be defined. The protocols shall be defined such as: • Load information can be exchanged on demand, periodically or based on events. Operators shall be able to configure the type of load reporting to be used based on its own policies." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24], TS 36.423 [34] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Only periodic reports are supported in PM measurements (TS 32.425) and X2 RESOURCE STATUS UPDATE (TS 36.423, 9.1.2.14). | None | None | Closed |

Table 7.6.5

| "In addition to load indicators, reliable protocols and procedures for load information exchange between RATs shall be defined. The protocols shall be defined such as: • Operators shall be able to control the extra signalling generated by the load balancing related information exchange." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | For load reporting over the X2 interface, the Reporting Periodicity IE is defined, that enables negotiation between the eNB and the neighbour, on periodicity of load reports. | None | None | Closed |
| 2 | The reporting periodicity cannot be configured by the operator e.g. via OAM interface. | None | None | Closed |

Table 7.6.6

| "In addition, it is recommended that the load balancing solution include the following functionality: • Load balancing shall have minimal impact on mobility performance while attempting to use a minimum number of handovers or cell re-selections needed to achieve the load balancing." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.300 [33], TS 36.423 [34] |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The TS 36.300 defines the distributed load balancing procedure and TS 36.423 defines the associated signalling over X2. The trade-off between load balancing performance and minimization of handovers or cell re-selections fully depends on the load balancing algorithm(s) which are out of scope of standardization. | None | None | Closed |

Table 7.6.7

| "In addition, it is recommended that the load balancing solution include the following functionality: • Load balancing in mobility scenarios should be coupled with algorithm that automatically adjusts cell individual offset parameter (as per SOCRATES study results)" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.300 [33], TS 36.423 [34] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Load balancing procedure is specified in the TS 36.300, clause 16.1.6. According to the clause 22.4.1.4, the load balancing action may include adjustment of the handover trigger i.e. the cell specific offset that corresponds to the threshold at which a cell initializes the handover preparation procedure. | None | None | Closed |

Table 7.6.8

| "In addition, it is recommended that the load balancing solution include the following functionality: • Load balancing algorithm in mobility scenarios shall also relay on awareness of type of base station to decide whether to enforce handover to macro or to pico cells (as per SOCRATES study results)" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Awareness of the of type of base station is not supported at the X2 interface. | None | None | Closed |

Table 7.6.9

| "In addition, it is recommended that the load balancing solution include the following functionality: • Load balancing shall be designed in such way as to increase overall resource utilization in overlayed heterogeneous networks and reduce investment in capacity." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Specifications of the distributed load balancing function (MLB) in TS 36.423 provide means, such as load reporting, for optimal distribution of capacity over E-UTRAN deployment in certain area. However there are no tools for reporting the load in the overlay network for example in a 3G overlay to the E-UTRAN | None | None | Closed |

Table 7.6.10

| "In terms of load balancing management and interoperability, NGMN recommends the following: • It shall be possible to customize load balancing policies. On the one hand, there shall be flexibility to adjust the load balancing related SON functionality to the Operator's recommendations. On the other hand, customization shall be a simple process to minimize the manual effort required." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.628 [20] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 28.628 and related specifications define SON Policy NRM IRP, particularly Targets for distributed Load Balancing Optimization Function. | None | None | Closed |

Table 7.6.11

| "In terms of load balancing management and interoperability, NGMN recommends the following: • A multi-vendor interface between eNB and OAM needs to be supported to allow for seamless operation in multivendor scenarios." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Load balancing is defined in TS 36.300 as distributed SON function. For this function the standard does not define any OAM configuration parameters. | None | None | Closed |

Table 7.6.12

| "In terms of load balancing management and interoperability, NGMN recommends the following: • Provide an open Northbound Interface for all load balancing related parameters within SON for Interoperability with 3rd party Tools." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.6.13

| "In terms of load balancing management and interoperability, NGMN recommends the following: • It would be desirable to extend the SON functionality to use E2E approach, such that traffic management selection is based not only on load balancing indicators, but also on network topology as to avoid for example MME or SGW relocation in case of intra LTE balancing." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This functionality is not supported. | None | None | Closed |

Table 7.6.14

| "Based on the long term traffic behaviour, it should be possible to direct the traffic in advance by a centralized entity (e.g. periodic events such as sports, fairs, etc.)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The Editors’ note. Meaning of “directing the traffic” is unclear. | None | None | Closed |

Table 7.6.15

| "Cell reselection for idle users should take into account current active user conditions so that if QoS demands increase in a cell it is possible to force cell edge users to camp on strongest neighbour, or to the one that has more resources available." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.300 [33], TS 36.423 [34] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 36.300, clause 22.4.1 “Support for Mobility Load Balancing” includes possibility of modification of reselection configuration. It is supported in TS 36.423. | None | None | Closed |

Table 7.6.16

| "It is recommended that 3GPP include the following in the standardization effort: Complete the standardization work on load indicators to enable multi-vendor and multi RAT support for load balancing Complete the standardization work on load indicators to enable reliable load information exchange in a multi-RAT, multi vendor network configuration" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All topics are addressed earlier in this clause 7.6 |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | All topics are addressed earlier in this clause 7.6. | None | None | Closed |

Table 7.6.17

| "It is recommended that 3GPP include the following in the standardization effort: Extend automatic neighbour relationship capability to include awareness of type of base station (macro/pico/femto) so that: handovers to macro cells are enforced when UE speed exceeds certain threshold handovers to pico happen when UE speed is low and when there is spare pico capacity" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This capability is not supported. | None | None | Closed |

Table 7.6.18

| "It is recommended that 3GPP include the following in the standardization effort: Add capability to selectively offload individual users between 3G/4G RATs based on the decision policy. Standardize network management to/from PCRF interface so that operator policy can operate seamlessly in multivendor environment" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This capability is not supported. | None | None | Closed |

## 7.7	Cell Outage Compensation (COC)

Abstract of Top OPE Recommendations Clause 4.7: Cell Outage Compensation:

"It has to be separated between detection and mitigation.

Cell Outage Detection: Cell outage is detected by statistical analysis, alarm or customer complains. Often, it may not be detected for several hours /days (sleeping cell). This may also only refer to some service in a cell (e.g. sleeping HSDPA, sleeping GPRS). The Goal is that automatic system functionality detects sleeping or poor performing cells.

Cell Outage Compensation: The network is being reconfigured to compensate the loss of service in the respective area. It is ensured that in surrounding area of cell in outage the cells which are not directly involved in COC activities significant quality indicators like call drop rate and average throughputs are not negatively affected. When the failure has been removed an autonomous reconfiguration shall take place. The Goal is:

HW failure of eNodeB unit causes complete outage of a cell. Loss shall be compensated by the network as much as possible until the failure is removed using redundancy in the network.

Revenue loss of cells or services is avoided. Further customer impact is minimized in case of losses. Maintenance activities can / maybe shifted to planned maintenance time periods with following cost savings."

Table 7.7.1

| "The network element can, at the lowest layer, consolidate all available information (alarms, measurements, traces, UE information neighbour cell information, etc.) and create a meaningful alarm that indicates a service affecting problem." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.2

| "Data is provided to a centralized entity that gathers all necessary information from the networks (e.g. alarms, measurements, traces, PM data, probes, neighbour cells, UEs, eNB heartbeat) and consolidates this information to generate structured information on the service state in each individual cell." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.3

| "The centralized entity is multi-vendor capable in that it is possible to connect elements from different suppliers to this entity with no consequent restriction in functionality." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.4

| "Service related problems in the network are detected and alarmed." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.5

| "It is possible to automatically initiate consequent policy-controlled actions such as automated power down and/or traffic blocking of a sleeping cell, Alarm to Operator. It must be possible for the operator to create related policies." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.6

| "The System autonomously compensates for network problems resulting in cell failures. The failure consequences are minimized and the reaction time for the operator can be relaxed." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.7

| "In compensating for these failures, the System interfaces to 2G and 3G network for automatic Inter-RAT (I-RAT) neighbour configuration. Handovers in affected cells of 2G, 3G and LTE technology shall be prevented. New handover relations in both, 2G, 3G and LTE, are configured to mitigate the failure consequences." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.8

| "Additional parameters such as transmission power and antenna settings are also automatically adjusted." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.9

| "In the event of a higher level network element (on which low level elements such as the eNodeB are dependent) failing or becoming unavailable, service outages can be avoided by an appropriate automatic re-parenting of the lower level nodes to other available higher level network elements." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.10

| "In doing so, QoS analysis is applied to manage pooling and load sharing activities" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.11

| "Automatic healing or mitigation mechanisms for several failure classes are in place, decreasing operational effort and mitigating the consequences of unit failures. Node resources are re-configured autonomously and optimized." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.12

| "Fault management and correction is simple and to a significant extent automated, supported, for example, by a correlation function. Parameters such as correlation rule data must be configurable by the operator. Consequent automated actions must also be configurable by the operator." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.13

| "During operation of the System, it may be required to add or remove hardware to or from individual entities or groups of entities (for example, an eNodeB or group of eNodeBs) comprising the system, and it may also be required to increase or decrease the capacity of these entities." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.14

| "It is required to carry out these changes with: As few site visits as possible As little human intervention as possible Where human intervention is required, only minimum technical skills shall be used" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.15

| "Automatic Optimization of mobility and handover related parameters (i.e. cell individual offsets, down tilts, Event A related parameters) in dependency of related KPIs." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.16

| "Automatic Optimization of cells or services in outage based on a unambiguous detection of this outage. Implementation of rule based switch towards planned configuration sets for defined outage scenarios shall be supported by the system" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.17

| "The inventory related instances in network management level "NetworkInventory" (reflecting HW and configuration status) and "ServiceInventory" (reflecting service level) provide standard interface to an instance "CellOutageCompensation" so it can retrieve comprehensive (multi-vendor) network view and information about services to be able to calculate the service impact of reconfiguration processes." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.7.18

| "NEMs should provide interface which would enable an instance "CellOutageCompensation" to orchestrate the reconfiguration process. This in particular should enable the instance "CellOutageCompensation" to define conditions/limits for reconfigurations which can be performed by NEM." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

## 7.8	Common Channel Optimization

Table 7.8.1

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “NGMN recommends that SON solutions for common channel Optimization are implemented according to the following principles. • It is preferable to perform optimization of DL common channel parameters for a group/cluster of cells rather than for each cell in isolation.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.2

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “NGMN recommends that SON solutions for common channel Optimization are implemented according to the following principles. • It should be possible to independently optimize parameters of each common channel. However, it is not precluded that parameters of several common channels are optimized simultaneously by the SON function.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.3

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “NGMN recommends that SON solutions for common channel Optimization are implemented according to the following principles. • The basic parameter which the SON function should be able to adjust for all common channels is the transmit power. It is recognized that antenna tilt variation would generally affect performance of all common channels (both uplink and downlink) and hence should not be adjusted by automated functions for optimizing any individual common channel.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.4

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “The architecture considerations as follows for the definite use cases: • RACH Optimization: For RACH Optimization, it is obvious that the eNB can perform its Optimization independently of the Optimization performed by neighbouring eNBs, hence, it was concluded that a distributed approach whereby the RACH Optimization algorithm resides in the eNB is more appropriate” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.5

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “The architecture considerations as follows for the definite use cases: • DL Common Channel Optimization: in [x]?? reasons are provided for choosing a centralized algorithm for common channel optimization on a slow basis. In addition to the slow adaptation, it might also be necessary to optimize the DL common channel parameters in a more dynamic way to adjust to sudden changes in the radio propagation environment. For such a dynamic adaptation, it seems more appropriate for the SON function to be localized in the eNB i.e. for a distributed approach to be used.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.6

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “Proposal to the standards: • 3GPP is requested to specify the UE measurements for minimization of drive tests in TR 36.805 in such a way that they are re-usable by SON functions for common channel Optimization. For further details please refer to [8].” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.8.7

| Top OPE Recommendations Clause 4.8: Common Channel Optimization “Proposal to the standards: • 3GPP should facilitate the exchange of relevant information (identified in this deliverable) between eNBs for the purpose of common channel optimization using a distributed architecture approach. For further details please refer to [8].” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

## 7.9	Interactions between Home and Macro BTS

Table 7.9.1

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “DL interference mitigation” “Fractional Frequency Reuse and Subframe Reuse” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34], TR 36.921 [103] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 36.423 specifes inter-eNB reporting that can be used for DL interference mitigation such as Relative Narrowband Tx Power. In particular, this signalling can be used in case when Fractional Frequency Reuse is uniformly applied by all eNBs for example according to Distributed Dynamic Frequency Partitioning as outlined in the TR 36.921 Note: this statement is valid for LTE. | None | None | Closed |
| 2 | In the interaction between Home and Macro BTS, there are elements explicitly referring to Fractional Subframe Reuse, such as ABS Pattern Information that can be used by the receiving side for DL interference mitigation. Note: this statement is valid for LTE. | None | None | Closed |
| 3 | In the interaction between Home and Macro BTS, there are no elements explicitly referring to Fractional Frequency Reuse in the DL. Note: this statement is valid for LTE. | None | None | Closed |

Table 7.9.2

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “DL interference mitigation” “Partial co-channel deployment” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP did not specify partial co-channel deployment when only a part of the DL frequency channel is shared between the Home and Macro BTS while other parts are exclusively used by one of them | None | None | Closed |

Table 7.9.3

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation” “Fractional Frequency Reuse and Subframe Reuse” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34], TR 36.921 [103] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 36.423 specifes inter-eNB reporting that can be used for UL interference mitigation such as UL Interference Overload Indication, UL High Interference Indication. In particular, this signalling can be used in case when Fractional Frequency Reuse is uniformly applied by all eNBs for example according to Distributed Dynamic Frequency Partitioning as outlined in the TR 36.921 Note: this statement is valid for LTE. | None | None | Closed |
| 2 | In the interaction between Home and Macro BTS, there are no elements referring to Fractional Subframe Reuse in the UL. Note: this statement is valid for LTE. | None | None | Closed |
| 3 | In the interaction between Home and Macro BTS, there are no elements explicitly referring to Fractional Frequency Reuse in the UL. Note: this statement is valid for LTE. | None | None | Closed |

Table 7.9.4

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation” “Partial co-channel deployment” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP did not specify partial co-channel deployment when only a part of the UL frequency channel is shared between the Home and Macro BTS while other parts are exclusively used by one of them | None | None | Closed |

Table 7.9.5

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “It is recommended that for the case the HBTS is only providing paging services, HBTS configures UE to make measurements on neighbour cells (including HBTS) and use the measurement report as follows: 1) to set the DL transmit power from HBTS so as not to cause excessive interference to MUE 2) To provide MBTS enough information so that it can control the UE UL transmit power to avoid excessive MUE UL interference at the HBTS. For the case HBTS provides data services, a basic solution is to rely on the existing handover procedure to avoid excessive UL and DL interference. “ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This is a recommendation for the power management algorithm in the HTBS which is implementation dependent, out of scope of 3GPP. | None | None | Closed |

Table 7.9.6

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Mobility Optimization between HeNB and Macro eNB: RLF indications for macro BTS to macro BTS handover are also applicable for macro BTS to HBTS handover” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.7

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “SON algorithms focus on the automatic adjustment of the CIO parameter and take into consideration UE speed and cell load” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: Not applicable |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The question is out of scope of SA5 | None | None | Closed |

Table 7.9.8

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UE assisted DL Power Control” “Signalling of power down indication” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: Not applicable |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The question is out of scope of SA5 | None | None | Closed |

Table 7.9.9

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UE assisted DL Power Control” “Signalling of UE MRM from MBTS to HBTS” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: Not applicable |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The question is out of scope of SA5 | None | None | Closed |

Table 7.9.10

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Fractional Frequency Re-use and Subframe Re-use for DL interference mitigation” “Negotiation/signalling of resource partition between MBTS and HBTS “ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: Not applicable |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The question is out of scope of SA5 | None | None | Closed |

Table 7.9.11

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Fractional Frequency Re-use and Subframe Re-use for DL interference mitigation” “Power down indication for a set of Resource Blocks and Subframes” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.12

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Partial Co-channel Deployment for DL interference mitigation” “Negotiation of frequency partitioning between MBTS and HBTS i.e. HBTS system bandwidth” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.13

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation “ “Noise padding “ “Exchange of maximum noise figure allowed and maximum transmit power for UE and/or overload” indicator (backhaul or OTA) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.14

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation” “Fractional Frequency Re-use and Subframe Re-use” “Negotiation/signalling of different Resource Blocks and/or subframes between MBTS and HBTS” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.15

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation” “Fractional Frequency Re-use and Subframe Re-use” “Signalling of MBTS to all HBTS in its coverage about resources reserved for use by HBTS” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.16

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “UL interference mitigation” “Fractional Frequency Re-use and Subframe Re-use” “Indication by MBTS of resource blocks and/or subframes being used by aggressor MUE so that HBTS can avoid using these RBs and/or subframes.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 7.9.17

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Hybrid Interference Management “ “Measurements of neighbour cells to allow HBTS to set correct DL transmit power (FFS if existing measurements are sufficient)” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TRs 36.921 [103], 36.922 [104] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP provided recommendations on Network Listening Mode (NLM), particularly in the TR 36.921 (E-UTRA, FDD) and TR 36.922 (E-UTRA, TDD). There is no need in standardization of these measurements because the measurement results are used internally in the HeNB Note: this statement is valid for LTE. | None | None | Closed |

Table 7.9.18

| Top OPE Recommendations Clause 4.9: Interactions between Home and Macro BTS “Hybrid Interference Management “ “Signalling of UE measurement from HBTS operating in hybrid access mode to MBTS to control UE transmit power” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 36.423 [34] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Implemented in TS 36.423 as RSRP Measurement Report List, clause 9.2.76 Note: this statement is valid for LTE. | None | None | Closed |

## 7.10	SON in CN

Table 7.10

| Top OPE Recommendations Clause 4.10 SON in CN / 4.10.3.1 Proposal to the standards: “In the following some areas are listed it seems beneficial to improve standardization. • Further standardization of S1 and X2 auto establishment. Especially transport and security related configuration is strongly operator and provider dependent today so that standardization could minimize options and lead to further cost efficiency. • Load balancing: load balancing between MME, MME and P-GW, eNB and MME. • Improved understanding of situation in CN nodes based on standardized performance measurements and indicators on the northbound interface between NEM and NMS. • Support of standardized deployment scenarios of single CN nodes like MME in a given cluster. Consideration of eNB allocation, relations to other CN nodes and transport & security issues. Management support via the northbound. • Standardized O&M for CN: examples are SW management, Pool Management, support of PM and CM for important use cases. Consider specific IRPs for core network nodes. • Enable one common view on the network including RAN and CN nodes in multi vendor scenarios and impact model on services. Enabler for this common view is a standardized northbound interface and supporting functionality in RAN and CN NEM and on NMS level. Standardization shall follow recommendations on NGOSS. • It is recommended to consider NGMN recommendations on standardized O&M also for Core Network Management.” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 28.701 [56], TS 28.702 [57], TS 28.703 [58], TS 28.704 [59], TS 28.705 [60], TS 28.706 [61], TS 28.707 [62], TS 28.708 [63], TS 28.709 [64] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The Top OPE Recommendations, Clause 4.10 for SON in CN particularly require “support of PM and CM for important use cases. Consider specific IRPs for core network nodes”. The following SA5 TSs provide specifications for PM in the Core Network: TS 32.406, TS 32.407, TS 32.426. The following SA5 TSs provide specifications for CM in the Core Network: TS 28.701, TS 28.702, TS 28.703, TS 28.704, TS 28.705, TS 28.706, TS 28.707, TS 28.708, TS 28.709. | None | None | Closed |

## 7.11	QoS optimization

Table 7.11.1

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Number of successful sessions per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Implemented in the TS 32.425 as Number of initial E-RABs successfully established in 4.2.1.2 and the Number of additional E-RABs successfully established in 4.2.1.5. | None | None | Closed |

Table 7.11.2

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Number of dropped sessions per QCI“ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Implemented in the TS 32.425 clause 4.2.2.6 as Number of released active E-RABs. | None | None | Closed |

Table 7.11.3

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Cell specific Customer Satisfaction Rate“ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: Not Applicable |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified in 3GPP. | None | None | Closed |

Table 7.11.4

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Min/Avg/Max Throughput per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Partially supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Measurements for average throughput per QCI are implemented in TS 32.425 as IP Throughput in DL in 4.4.6.1 IP Throughput in UL in 4.4.6.2. | None | None | Closed |
| 2 | Min/Max throughput measurements per QCI are not supported in both TS 32.425 and TS 36.314. | None | None | Closed |

Table 7.11.5

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Min/Avg/Max Round Trip Delay per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Neither TS 32.425 nor TS 36.314 support round trip measurements. Note. TS 32.425 clause 4.4.3.1 and TS 36.314 specify Average DL PDCP SDU delay per QCI. | None | None | Closed |

Table 7.11.6

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Packet Loss per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.425 specifies in 4.1.5.2 Packet Uu Loss Rate in the DL per QCI and in 4.1.5.3 Packet Loss Rate in the UL per QCI. | None | None | Closed |

Table 7.11.7

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Mean number of RRC connected users” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified in 3GPP. | None | None | Closed |

Table 7.11.8

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Mean number of RRC connected UEs with data to send per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TS 32.425 [24] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.425, clause 4.4.2.1 specifies the Number of active UEs per QCI, separately for DL and UL. | None | None | Closed |

Table 7.11.9

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Percentage of UEs per cell that is NOT achieving their required GBR and NOT achieving the required SDU Error Ratio per QCI” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified in 3GPP. | None | None | Closed |

Table 7.11.10

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Percentage of UEs for which transfer delay per IP packet was above a particular threshold” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified in 3GPP. | None | None | Closed |

Table 7.11.11

| Top OPE Recommendations Clause 4.11: QoS Optimization “QoS related PM shall be supported” “Percentage of UEs for which average throughput measured at RLC layer for each non-RT QCI was below a particular threshold” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Not specified in 3GPP. | None | None | Closed |

# 8	Performance Management (PM) enhancements

## 8.1	Free configurable measurement and delivery periods

Table 8.1

| Top OPE Recommendations Clause 5.3: "Free configurable measurement and delivery periods for each counter or counter group." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.411 [65], 32.412 [66], 32.416 [67] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.412 defines the operation createMeasurementJob with available granularity period of 5, 15, 30 minutes, 1, 12 or 24 hours. The granularity period specifies the period between two successive measurements. The measurement job also defines the reporting period which is the period between two successive notifications announcing the availability of the measurement file. | None | None | Closed |

## 8.2	Efficient data transfer mechanism

Table 8.2

| Top OPE Recommendations Clause 5.3: "Efficient data transfer mechanism, e.g. simply structured and compact raw data format with a maximum net data rate, e.g. csv (current XML-based 3GPP standard has large overhead; might be improved using e.g. appropriate compression methods; subject for standardization)". |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.431 [68], 32.432 [69], 32.435 [70], 32.436 [71], 32.341 [72], 32.342 [73], 32.346 [74] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.435 defines XML file format definition. Compression of XML is an available option of SA5 standards (File transfer IRP - TS 32.432). TS 32.436 defines Abstract Syntax Notation 1 (ASN.1) file format definition, which is more compact than the XML format. For ASN.1, BER encoding rules are used. | None | None | Closed |

## 8.3	Enhance Management System (EMS) internal post-processing

Table 8.3

| Top OPE Recommendations Clause 5.3: "EMS internal post-processing of raw data without significant delay (near real-time)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: None |  |  | Support status Not applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Product related. Out of scope of SA5 standards. | None | None | Closed |

## 8.4	Automated counter or counter group administration

Table 8.4

| Top OPE Recommendations Clause 5.3: "Automated counter or counter group administration (incl. activation)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.411 [65], 32.412 [66], 32.416 [67] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TS 32.412 defines the management of counter or counter group via the operation createMeasurementJob. | None | None | Closed |

## 8.5	Automated performance data quality management

Table 8.5

| Top OPE Recommendations Clause 5.3: "Automated performance data quality management, e.g. automatic counter restart after outage." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.431 [68], 32.432 [69], 32.435 [70], 32.436 [71] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TSs 32,432, 32.435 and 32.436 specify a suspect flag to be used to indicate whether the data is reliable or not. TS 32.401 contains descriptions of system events that can interrupt measurement collection periods. | None | None | Closed |

## 8.6	Function for simple threshold based on counters and KPIs

Table 8.6

| Top OPE Recommendations Clause 5.3: "Function for simple threshold based on counters and KPIs." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.411 [65], 32.412 [66], 32.416 [67] |  |  | Support status Supported |  |
| Item | Functionality | Action | Related CR(s) | Status |
| 1 | PM IRP defines Threshold Management functionality. See operations in TS 32.412: createThresholdMonitor, deleteThresholdMonitor, listThresholdMonitors, suspendThresholdMonitor, resumeThresholdMonitor. | None | None | Closed |

## 8.7	Function for simple KPI calculation based on counters

Table 8.7

| Top OPE Recommendations Clause 5.3: "Function for simple KPI calculation based on counters." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.410 [75], 32.450 [76], 32.451 [77], 32.454 [78], 32.455 [79] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TSs 32.410, 32.450, 32.451, 32.454, 32.455 defines GERAN, UTRAN, EPC, IMS, E-UTRAN KPIs based on measurements specified in other SA5 TSs. | None | None | Closed |

## 8.8	Automatic identification of network problems and error correction

Table 8.8

| Top OPE Recommendations Clause 5.3: "Automatic identification of network problems and error correction." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.111-1 [6], 32.111-2 [7], 32.111-6 [8] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 32.111-2 defines a correlatedNotification parameter allowing EM to assemble various alarms into an assembly indicating that the alarms in the assembly probably be caused by one network problem. | None | None | Closed |

## 8.9	Standardization of Performance Management (PM)

Table 8.9

| Top OPE Recommendations Clause 5.3: "Standardisation of PM is key to support seamless PM integration in multi vendor scenarios." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.411 [65], 32.412 [66], 32.416 [67], 32.431 [68], 32.432 [69], 32.435 [70], 32.436 [71], 32.401 [80], 32.403 [81], 32.404 [82], 32.405 [83], 32.406 [84], 32.407 [85], 32.408 [86], 32.409 [87], 32.410 [75], 32.425 [24], 32.450 [76], 32.451 [77], 32.454 [78], 32.455 [79] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | 3GPP SA5 provides a full set of PM standards to support multi-vendor scenarios | None | None | Closed |

## 8.10	Real time behaviour of Performance Management (PM)

Table 8.10

| Top OPE Recommendations Clause 5.3: "There is a clear tendency to enable real time behaviour of PM delivery to understand more quickly situation in the network." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.412 [66] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The 5 min granularity period is deemed to be sufficient and is supported by SA5 standards. | None | None | Closed |

## 8.11	UE-based measurements

Table 8.11

| Top OPE Recommendations Clause 5.3: "UE-based measurements (as for MDT) are seen as part of significant information covered by the topic PM." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | MDT has been introduced based on Trace functionality in Rel-10 and Rel-11: 470142 SA5 aspects of Management of UE based network performance measurements (OAM-PM-UE), 510058 Enhanced Management of UE based network performance measurements (OAM-ePM-UE). | None | OAM-PM-UE OAM-ePM-UE | Closed |

# 9	Enhancement of Trace functionality

## 9.1	Trace on Uu, S1 and X2 interfaces

Table 9.1

| Top OPE Recommendations Clause 6.3: "The infrastructure system supports trace functionality as specified in 3GPP 32.421, 32.422, and 32.423. It shall be possible to trace information (standardized or proprietary) on interfaces Uu, S1, X2" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Trace Management is supported from Rel-8 onwards. | None | None | Closed |
| 2 | Tracing of vendor specific extensions is supported (see clause 5.3 in 32.422). | None | None | Closed |

## 9.2	Support for trouble-shooting, optimization, network wide monitoring and trouble detection and customer specific trouble shooting

Table 9.2

| Top OPE Recommendations Clause 6.3: "The trace functionality is seen as basis for trouble-shooting, optimization, network wide monitoring and trouble detection and customer specific trouble shooting" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Cell Trace and/or subscriber equipment trace with additional capability of MDT support a wide range of use cases for troubleshooting, optimization, network wide monitoring and trouble detection and customer specific trouble shooting. | None | None | Closed |
| 2 | Enhancements to MDT with additional measurements related to QoS/QoE should be supported. Under RAN2 responsibility (hence not in scope of SA5 until a RAN2 work item is started). | None | None | Closed |

## 9.3	UE support for transparent collection and reporting of measurements / events

Table 9.3

| Top OPE Recommendations Clause 6.3: "Tracing shall rely on UE support for transparent collection and reporting of measurements / events" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Tracing/MDT supports the collection of UE measurements/events upon activation from the network. | None | None | Closed |

## 9.4	Interface to external 3rd party systems

Table 9.4

| Top OPE Recommendations Clause 6.3: "Interface to external 3rd party systems like Customer Experience Mgmt systems" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This requirement is generic. 3GPP SA5 specifications refer to an interface to TCE used to collect MDT data. TCE Ip address is specified by the Operator that can use the information for Customer Experience Mgmt. | None | None | Closed |

## 9.5	Capture and store real-time data across technologies

Table 9.5

| Top OPE Recommendations Clause 6.3: "User Call Trace applications should be able to capture and store real-time data for specific calls or subscribers across several network elements and technologies (e.g. 2G/3G/LTE)" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92], 52.008 [93] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Supported by signalling activation: (32.422 sect. 4.1.2.1) “In case of home subscriber trace (i.e. in the HPLMN) the Trace Session activation shall go to the HSS / MSC Server / SGSN / MME. | None | None | Closed |

## 9.6	Successful and unsuccessful calls

Table 9.6

| Top OPE Recommendations Clause 6.3: "Tracing and logging of all successful and unsuccessful calls to any of registered numbers" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Not Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | It is not possible to do that for any registered terminating numbers. | None | None | Closed |

## 9.7	High number of users / events

Table 9.7

| Top OPE Recommendations Clause 6.3: "Enhanced functions shall efficiently support the tracing for a high number of users / events with limited impact on the existing network infrastructure" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Limitation in the specifications is due to the Trace Reference coding (3 byte octet string) and the Trace Recording Session Reference coding (2 byte octet string). Limitation in the implementation is mentioned in clause 4.2 of 32.422 due to insufficient resources available in the NE. | Out of scope | None | Closed |

## 9.8	Set call identifying parameter values

Table 9.8

| Top OPE Recommendations Clause 6.3: "Application has to offer the ability to set call identifying parameter values. These settings shall act as call / transaction filters. Each parameter of every protocol has to be available for selection" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.421 [88], 32.422 [53], 32.423 [89], 32.441 [90], 32.442 [91], 32.446 [92] |  |  | Support status Partially Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | There are some filters but not every parameter is available for selection. | None | None | Closed |

# 10	eNodeB Plug & Play - Self commissioning

Table 10.1

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - Automated equipment, radio network and transport network data planning and preparation should be supported" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.501 [26], 32.502 [27], 32.506 [30], 32.508 [94], 32.509 [95] |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.2

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The number of to be planned parameters shall be minimized e.g. by automatic SON functionality as Automatic Neighbour Relation (ANR)." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: 32.511 [40] |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.3

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Self-detection of equipment and appropriate self-configuration related to this equipment should be supported" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.4

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Auto connection to the network should be supported leading to a setup of a secure tunnel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.5

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Auto configuration of SW and planned and default configuration data during self configuration should be supported" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.6

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Automated deployment of licenses during self configuration should be supported: License management should be transparent for the operator and must not cause any negative deployment and deployment impact" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.7

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Automated integration into the access-, core- and network management network should be supported (automatic setup of secure X2, S1-u / S1-MME & OAM channel)" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.8

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Auto Connection with the Authentication Node via Operator’s Certificate repository should be supported." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.9

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Automatic self test and cell/common channel setup should be supported" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.10

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - By auto inventory all changes are available. following the final self test the eNodeB delivers - a state change notification - details on its HW resource configuration (HW resource inventory) - details on its parameter configuration (configuration inventory) as e.g. complete configuration set, SW version etc. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.11

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Depend on operator’s policy, the new eNodeB is put into reserved mode for operator use (e.g. operator tests, from special UEs) or on air immediately." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.12

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Radio neighbours are automatically detected and configured by 3GPP ANR functionality. Automatic X2 setup is done considering security recommendations." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

Table 10.13

| Top OPE Recommendations Clause 7.3: Recommendations "Automatic Self Configuration of eNodeB shall be fully supported. OSS shall support plug and play of eNodeB including automatic setup of secure X2 & S1-u / S1-MME configuration as below: - The following procedural steps assume that no medium at the site is needed for eNodeB specific configuration as e.g. a local maintenance terminal. - Site intervention for configuration shall be minimized. Site intervention should be limited to basic H/W node and necessary transmission H/W installation. Manual capture of node data on site (e.g. H/W serial number) shall be avoided. A process of one time site visit for eNodeB service activation shall be provided." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 |  |  |  |  |

# 11	OSS Standard Itf-N

Table 11.1

| Top OPE Recommendations Clause 8.3: Recommendations "This chapter contains "generic" recommendations only, valid for each type of interface between EMS and NMS. "Plug & Play"  It must be possible to implement the interfaces between network and OSS systems easy and efficient by lowest costs and smallest effort (ideally without any development and/or configuration). Useful  It must deliver efficient support for the OSS business processes. The interface must deliver the needed OSS semantics to support the process. Re-useable / Generic  The interface must be generic enough, to enable the re-use in different integration/business scenarios Flexible / Extensible  It must be possible to extend the interface capabilities (methods and attributes), without breaking the standard Standardized / Open  The interface has to be based on unambiguously standardized specification, which does not allow room for interpretation. The specification and related artefacts must be freely available and useable for everybody. Mature / Stable  The interface must be stable and mature, to avoid expensive changes on implemented interfaces. De-coupled  Changes in the application or in the interface implementation at one of the communication partners may not lead to the need for changes in the application or in the interface implementation of the other communication partners. Evolutionary  OSS interface shall re-use already existing, widely adopted and mature IT standards (e.g. transport protocols) to avoid "reinventing the wheel". Independent  The interface specification must be independent from underlying infrastructure. Upward / Downward Compatible  It must be possible to implement a new version of an interface specification at one of the communication partners, while the other communication partners still use an old version of the interface specification. Interoperable  The interface implementation shall be based on an interoperable portfolio of interfaces / interface specifications to support different OSS business processes using a common architecture and a common information model. Scalable  No performance constraints caused by the interface specification or the implementation. Secure  The interface has to be able to ensure confidentiality and availability of the data, which is transferred by the interface. Reliable / Having Integrity  The interface implementation has to ensure the reliability and the integrity of the data, which is transferred by the interface. Adopted & Verified  Widely adopted and verified, so that every vendor supports it." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All |  |  | Support status Supported (See details in the functionality column) |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | All aforementioned requirements have been further developed during NGCOR Phase 1 project timeframe. Consequently, the support status of 3GPP SA5 specifications to these requirements is provided in deeper details in TR 32.837 - Gap analysis between 3GPP SA5 specifications and NGMN Top Operational Efficiency (OPE) Recommendations. | None | None | Closed |

Table 11.2

| Top OPE Recommendations Clause 8.3.1: IRP Ensembles "The need for MNOs was identified to have a tool enabling them to understand precisely what a given Itf-N interface should be made up of, i.e. instead of the whole bunch of 3GPP IRPs (Integration Reference Points), it appears that only some subset is required for a given Network Management Application Profile. A list of most relevant profiles has been identified by MNOs, providing some first elements about which IRPs should be part of the corresponding IRP Ensembles. Further details are presented in [3]. Further investigation is needed on the following aspects: • Should IRP Ensembles be normative or informative? • Which formalism would be the most appropriate to specify IRP Ensembles? These questions shall be addressed in standardisation bodies and should be in focus of following operator initiatives." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All NRM and Interface IRPs |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Outcome of 3GPP SA5 ad hoc meeting (May 5-7, 2010, Montreal, Canada): "Definition of IRP ensembles will be covered by SA5 in Rel-10 (new WI at SA5#71)". IRP Ensembles can be defined using 3GPP Solution Profiles. 3GPP TS 28.390 has been created to define Solution Profiles. The NGCOR Fault Management Solution Profile has been defined. Additional Solution Profile can be defined as needed. | No action needed. | OAM-FMSP | Closed |

Table 11.3

| Top OPE Recommendations Clause 8.3.2: SON/O&M Implementation Conformance Statement "Some need has been identified for MNOs to have a tool enabling them to check how conformant vendors’ implementations are with respect to 3GPP OA&M / SON IRPs. By “Implementations”, we mean here either EMS systems, NMS systems or NPOT applications. A 3-staged top-down conformance checking methodology is introduced as well as an example of SICS (SON/OA&M Implementation Conformance Statement) proforma. The benefit of such tools for MNOs is to be able to evaluate how high the cost for integrating the implementation in their OSS would be, in terms of CAPEX, OPEX and Time-To-Market. Further details are presented in [2]." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All NRM and Interface IRPs. |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Outcome of 3GPP SA5 ad hoc meeting (May 5-7, 2010, Montreal, Canada): "Conformance statements per IRP ensemble can be automatically produced based on IRP ensemble spec (Word macro) It is not required to produce new SA5 specs for conformance statements but if some companies are interested they can propose a WI". | None | None | ClosedOpen |

Table 11.4

| Top OPE Recommendations Clause 8.3.3: NGOSS/SOA "The convergence of access and core network technologies towards a common IP-based approach will bring many cost saving options. Tomorrow the biggest value of NGN will be to enable the rapid & flexible delivery of new services, through IP transport network, from service delivery platforms to the end user device. Therefore following recommendations are given to enable an NGOSS and SOA compatible network system: Network Operators should support the standardization of end-to-end seamless integrated management system for Network and Service Delivery Platforms; …" |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Out of scope of SA5 standards. | None | None | Closed |

Table 11.5

| Top OPE Recommendations Clause 8.3.3: NGOSS/SOA "The convergence of access and core network technologies towards a common IP-based approach will bring many cost saving options. Tomorrow the biggest value of NGN will be to enable the rapid & flexible delivery of new services, through IP transport network, from service delivery platforms to the end user device. Therefore following recommendations are given to enable an NGOSS and SOA compatible network system: … This should be based on a service-oriented (SOA) aggregation layer as a "blueprint architecture"; … Further details are presented in [4]." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All Interface IRP SOAP Solution Sets. |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | TR 32.824 "Service Oriented Architecture (SOA) Integration Reference Point (IRP) study" (SI FS_OAM_SOA_IRP) concluded that: "SOA provides methods for systems development and integration where systems group functionality around business processes and packages these as interoperable services. A SOA infrastructure allows different applications to exchange data with one another as they participate in business processes. The IRP approach is well suited for operating within a SOA environment (see Section 6). In operator’s environment, the FCAPS types of service, supported by the various IRPs such as AlarmIRP, PMIRP, are one of many key inputs to the aforementioned business processes. It is the conclusion and recommendation of this paper that the various IRPs be evolved further, modified in such that they can fit even better into an SOA infrastructure. Specifically, this paper calls for a new Work Item for Release 9 entitled "SOA for IRP". This Work Item would: • Enhance 32.101 [18] to include the support of SOA infrastructure as part of its Principles and high level requirements. • Enhance 32.102 [19] and 32.150 [20] to include the descriptions of a) the SOA infrastructure and b) the relationship between the SOA infrastructure and the IRP Architecture. • Enhance the relevant Interface IRPs in areas that require amendments for its implementations to improve participation in an SOA infrastructure environment." These recommended actions have been completed. | None | None | Closed |

Table 11.6

| Top OPE Recommendations Clause 8.3.3: NGOSS/SOA "The convergence of access and core network technologies towards a common IP-based approach will bring many cost saving options. Tomorrow the biggest value of NGN will be to enable the rapid & flexible delivery of new services, through IP transport network, from service delivery platforms to the end user device. Therefore following recommendations are given to enable an NGOSS and SOA compatible network system: … This should reuse the fully re-usable and widely deployed 3GPP IRP Framework to manage Wireline Network with possible extensions due to the ongoing alignment process between 3GPP generic NRM and TM Forum SID framework; … Further details are presented in [4]." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: All TS 28.xyz series |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | The 3GPP IRP Framework is fully reusable to manage both wireline and wireless networks. 3GPP NRM IRPs contained in TS 32.xyz series have been revisited to cope with the Umbrella Information Model (TS 28.620), which acts as an output from the joint work between 3GPP and TM Forum. All existing 3GPP Interface IRPs remain unchanged. The outcome is a new series of specifications in 28 series (WI OAM-FMC-IRP). | No additional action needed. | None | Closed |

Table 11.7

| Top OPE Recommendations Clause 8.3.3: NGOSS/SOA "The convergence of access and core network technologies towards a common IP-based approach will bring many cost saving options. Tomorrow the biggest value of NGN will be to enable the rapid & flexible delivery of new services, through IP transport network, from service delivery platforms to the end user device. Therefore following recommendations are given to enable an NGOSS and SOA compatible network system: … Most network elements, IS & service delivery platforms should support a multi-vendor XML-based Web Services standardized interface; … Further details are presented in [4]." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This requirement is addressed to network element vendors. Hence, it’s out of scope of SA5 standards. | None | None | Closed |

Table 11.8

| Top OPE Recommendations Clause 8.3.3: NGOSS/SOA "The convergence of access and core network technologies towards a common IP-based approach will bring many cost saving options. Tomorrow the biggest value of NGN will be to enable the rapid & flexible delivery of new services, through IP transport network, from service delivery platforms to the end user device. Therefore following recommendations are given to enable an NGOSS and SOA compatible network system: … It is recommended that Mobile Network Operators drive the harmonisation of TM Forum Interface Program (TIP) and 3GPP Integration Reference Points (IRP). Further details are presented in [4]." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: |  |  | Support status Not Applicable |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | Out of scope of SA5 standards. | None | None | Closed |

# 12	OSS tool support for optimization and operation

"Tools and application in the north from the OMC plays a significant role in an operator organization to coordinate operator processes: e.g. workflow management tools, optimization tools and data bases. In this chapter the focus is on recommendations regarding these tools as complementary area of the chapter OSS Standard Interface covering the standard northbound interface which is the bridge between the so called OSS tools in the north and the radio and core network infrastructure network element mangers (or OMC).

The main idea of this section is to formulate main use cases and recommendations on related tools in OSS to support standardization."

SA5 scope is on the northbound interface (Itf-N) and the corresponding NGMN recommendations have been addressed in Clause 11 "OSS Standard Itf-N". Consequently, it is considered that the entire chapter on OSS tools is out of scope of the gap analysis between NGMN Top Operational Efficiency (OPE) Recommendations and SA5 specifications.

# 13	Automatic inventory

Table 13.1

| Top OPE Recommendations Clause 10.3: Recommendations "Vendor infrastructure (RAN & Core Network elements) Standardized interface for signalling information about changes performed in the network." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.690 [96], 32.691 [97], 32.692 [98], 32.696 [99], 28.631 [100], 28.632 [101], 28.633 [102] |  |  | Support status Not supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | This requirement cannot be met in TS 32.69x and 28.63x, because no notification or other ways have been defined for the inventory related IOCs to notify the changes. Adding common notifications notifyAttributeValueChange/ notifyObjectCreation/ notifyObjectDeletion for inventory related IOCs may be studied depending on proposals (e.g. a CR against TS 28.632) from 3GPP members. Note: it is assumed that this recommendation applies only to the hardware part of the network. | None | None | Closed |

Table 13.2

| Top OPE Recommendations Clause 10.3: Recommendations "Vendor infrastructure (RAN & Core Network elements) Standardized interface to poll the information about Network Element configuration and components." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.690 [96], 32.691 [97], 32.692 [98], 32.696 [99], 28.631 [100], 28.632 [101], 28.633 [102] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | In Alternative 1 and 2 of TS 32.692/28.632, the unidirectional relations between ManagedFunction and Inventory related IOCs have been defined and the attribute relatedFunction or msFunction in inventory related IOCs represents the ManagedFunction that is associated, and thus the Network Element configuration information can be accessed by the operations in Basic or Bulk CM IRP. Alternative 1 is for NE structure and hardware inventory and the Basic or Bulk CM IRP can be used to obtain the component information. Alternative 2 is for NE structure, hardware, software and license data inventory and FT IRP can be used to obtain the components information. | None | None | Closed |

Table 13.3

| Top OPE Recommendations Clause 10.3: Recommendations "Vendor infrastructure (RAN & Core Network elements) All changes are available via a push or pull mechanism." |  |  |  |  |
| --- | --- | --- | --- | --- |
| Relevant 3GPP specifications: TSs 32.690 [96], 32.691 [97], 32.692 [98], 32.696 [99], 28.631 [100], 28.632 [101], 28.633 [102] |  |  | Support status Supported |  |
| Item | Description | Action | Related CR(s) | Status |
| 1 | For alternative 1 in TS 32.692/28.632: The Basic CM IRP and Bulk CM IRP can be used to obtain the inventory information including the changes. For alternative 2 in TS 32.692/28.632: The FT IRP can be used to obtain the inventory information including the changes. | None | None | Closed |

# 14	Conclusion

The table below gives an overview of the support of NGMN Top Operational Efficiency (OPE) Recommendations by SA5 specifications. Further alignment work may be done if needed via the usual 3GPP process (WIDs, CRs).

Total NGMN recommendations: 207. Total addressed by SA5: 156

Supported: 82

Partially Supported: 28

Not Supported: 27

Not Applicable: 19

Table 14

| Clause | Title | Total NGMN Recomm. | Total addressed by SA5 | Supported | Partially Supported | Not Supported | Not Applicable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Quality and quantity of alarms | 8 | 8 | 6 | 1 | 1 | 0 |
| 5 | Automatic software management | 5 | 5 | 4 | 0 | 1 | 0 |
| 6 | Energy saving | 20 | 20 | 14 | 3 | 0 | 3 |
| 7.1 | OAM Support for SON | 9 | 9 | 3 | 4 | 0 | 2 |
| 7.2 | Generic optimization | 13 | 12 | 3 | 5 | 4 | 1 |
| 7.3 | Automatic Neighbour Relation (ANR) | 13 | 11 | 8 | 2 | 1 | 1 |
| 7.4 | Minimization of Drive Tests (MDT) | 5 | 5 | 5 | 0 | 0 | 0 |
| 7.5 | Handover (HO) optimization | 18 | 18 | 8 | 3 | 5 | 2 |
| 7.6 | Load Balancing (LB) | 18 | 16 | 4 | 5 | 5 | 3 |
| 7.7 | Cell Outage Compensation (COC) | 18 | 0 | - | - | - | - |
| 7.8 | Common Channel Optimization (CCO) | 7 | 0 | - | - | - | - |
| 7.9 | Interactions between Home and Macro BTS | 18 | 2 | 2 | 2 | 2 | 1 |
| 7.10 | SON in CN | 1 | 1 | 0 | 1 | 0 | 0 |
| 7.11 | QoS optimization | 11 | 11 | 4 | 1 | 6 | 0 |
| 8 | Performance Management (PM) enhancements | 11 | 11 | 10 | 0 | 0 | 1 |
| 9 | Enhancement of Trace functionality | 8 | 8 | 5 | 1 | 1 | 1 |
| 10 | eNodeB Plug & Play - Self commissioning | 13 | 0 | - | - | - | - |
| 11 | OSS Standard Itf-N | 8 | 8 | 4 | 0 | 0 | 4 |
| 12 | OSS tool support for optimization and operation | - | - | - | - | - | - |
| 13 | Automatic inventory | 3 | 3 | 2 | 0 | 1 | 0 |
|  |  | 207 | 156 | 82 | 28 | 27 | 19 |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2012-10 |  |  |  |  | First version with document structure and initial contents |  | 0.0.0 |
| 2012-10 |  |  |  |  | Update following SA5#85: update of compliance template | 0.0.0 | 0.0.1 |
| 2013-02 |  |  |  |  | Update following SA5#87: introduction of pCR S5-130315 | 0.0.1 | 0.1.0 |
| 2013-04 |  |  |  |  | Update following SA5#88: introduction of pCR S5-130761 | 0.1.0 | 0.2.0 |
| 2013-09 |  |  |  |  | Update following SA5#90: introduction of pCRs S5-131284, S5-131407, S5-131408 | 0.2.0 | 0.3.0 |
| 2013-10 |  |  |  |  | Update following SA5#91: introduction of pCRs S5-131673, S5-131792, S5-131795, S5-131796 | 0.3.0 | 0.4.0 |
| 2013-10 |  |  |  |  | MCC clean-up | 0.4.0 | 0.4.1 |
| 2013-11 |  |  |  |  | Update following SA5#92: introduction of pCR S5-132171 | 0.4.1 | 0.5.0 |
| 2014-02 |  |  |  |  | Update following SA5#93: introduction of pCRs S5-140188, S5-140267, S5-140268, S5-140294 | 0.5.0 | 0.6.0 |
| 2014-04 |  |  |  |  | Update following SA5#94: introduction of pCRs S5-140748 and S5-140773 | 0.6.0 | 0.7.0 |
| 2014-06 |  |  |  |  | Update following SA5#95: introduction of pCRs S5-143072, S5-143400, S5-143428 | 0.7.0 | 0.8.0 |
| 2014-06 | SA#64 | SP-140329 |  |  | Presented for information | 0.8.0 | 1.0.0 |
| 2014-09 |  |  |  |  | Update following SA5#96: introduction of pCRs S5-144216, S5-144273, S5-144275 and application of S5-144080 | 1.0.0 | 1.1.0 |
| 2014-10 |  |  |  |  | Update following SA5#97: introduction of pCRs S5-145359, S5-145360, S5-145361, S5-145392 | 1.1.0 | 1.2.0 |
| 2014-11 |  |  |  |  | Update following SA5#98: introduction of pCRs S5-146079, S5-146081, S5-146315, S5-146327, S5-146356 | 1.2.0 | 1.3.0 |
| 2015-02 |  |  |  |  | Update following SA5#99: introduction of pCRs S5-151100, S5-151345, S5-151346, S5-151347, S5-151348 | 1.3.0 | 1.4.0 |
| 2015-04 |  |  |  |  | Update following SA5#100: introduction of pCRs S5-152127, S5-152359, S5-152360, S5-152361, S5-152387 | 1.4.0 | 1.5.0 |
| 2015-06 |  |  |  |  | Update following SA5#101: introduction of pCRs S5-153050, S5-153092, S5-153113, S5-153329, S5-153330, S5-153331, S5-153332 | 1.5.0 | 1.6.0 |
| 2015-09 |  |  |  |  | Editorial cleanup by EditHelp | 1.6.0 | 1.6.1 |
| 2015-09 |  |  |  |  | Update following SA5#102: introduction of pCRs S5-154097, S5-154107, S5-154108, S5-154110, S5-154111, S5-154112, S5-154113 | 1.6.1 | 1.7.0 |
| 2015-10 |  |  |  |  | Update following SA5#103: introduction of pCRs S5-155235, S5-155236, S5-155237 | 1.7.0 | 1.8.0 |
| 2015-11 |  |  |  |  | Update following SA5#104: introduction of pCRs S5-156066, S5-156068, S5-156069, S5-156070, S5-156071, S5-156234, S5-156236, S5-156338, S5-156340 | 1.8.0 | 1.9.0 |
| 2015-12 | SA#70 | SP-150683 |  |  | Presented for approval | 1.9.0 | 2.0.0 |
|  |  |  |  |  | Upgrade to Rel-13 | 2.0.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-06 | SA#72 | SP-160413 | 0006 | 1 | F | Fix the status for interactions between Home and Macro BTS | 13.1.0 |