3GPP TSG-SA5 Meeting #111	S5-171084

# 16-20 January 2017, Porto (Portugal)													revision of S5-16xabc

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0483 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Charging enhancement for 3GPP PS Data off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PS_DATA_OFF-CH |  |  |  |  |  | Date: |  |  | 2017-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In order to support the reporting mechanism for the 3GPP PS Data off status, the enhancement on the PGW/GGSN CDR for offline charging over Gz and credit control request for online charing over Gy is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new fields for the 3GPP PS Data off |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging of 3GPP PS Data off cannot be accurated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x(new),5.2.1.3,5.2.1.10.1,5.3.1.2,5.3.1.6.2,6.1.3,6.3.1.2,6.3.2,6.3.3,6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.x	3GPP PS Data off

When 3GPP PS Data off is activated by the user, PGW prevents downlink traffic except for 3GPP PS Data Off Exempt Services. The 3GPP PS Data Off Exempt Services are a set of operator services, defined in TS 22.011 [67].

The UE reports its 3GPP PS Data Off status in PCO (Protocol Configuration Option) to PGW during Initial Attach procedure and report a change of its 3GPP PS Data Off status in PCO by using Bearer Resource Modification procedure, described in TS 23.401 [208].

OCS shall be informed about the activation/inactivation of 3GPP PS Data Off from PGW.

Next change

5.2.1.3	Flow Based bearer Charging (FBC)

IP-CAN bearer charging allows the P-GW to collect charging information related to data volumes sent to and received by the UE/MS, categorised by the QoS applied to the IP-CAN bearer. FBC is supported by the P-GW by the integration of a PCEF. With PCEF, the normal IP-CAN bearer charging is enhanced by the capability to categorise the service data flows within IP-CAN bearer data traffic by rating group or combination of the rating group and service id, i.e., while there is only one uplink an one downlink data volume count per IP-CAN bearer in IP-CAN bearer charging, FBC provides one count per each rating group or combination of the rating group and service id. In case that sponsored connectivity level reporting is active, FBC categorise within IP-CAN bearer data traffic by combination of rating group, sponsor identity and application service provider identity. The level of the reporting is defined per PCC rule. Details of this functionality are specified in TS 23.203 [215] and TS 32.240 [1].

NOTE: 	The P-GW can only include one QoS Information occurrence per service data container. This implies if an operator wishes to be able to separate usage according to QCI and ARP within their billing system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating reporting is used;

-	rating group/service id where rating group/service id reporting is used;

-	rating group/sponsor identity/application service provider identity where sponsored connectivity level reporting charging is used.

IP-CAN bearer specific offline charging is achieved with IP-CAN bearer specific rating group/service identifier defined in clause 5.3.1.1.

According to TS 23.203 [215], FBC shall support different charging models per PCC rule. These charging models may be based on volume and/or time and on number of events matching a specific service data flow template in PCC rule. 
In general the charging of a service data flow shall be linked to the IP-CAN bearer under which the service data flow has been activated.  The following chargeable events are defined for FBC:

-	Start of IP-CAN bearer. Upon encountering this event, a new PGW-CDR for this context is created.

-	Start of service data flow. If service identifier level reporting is required by the PCC rule new counts and time stamps for this combination of the rating group and service id are started. If rating group level reporting is required by the PCC rule needed new counts and time stamps for this rating group are started. If sponsored connectivity level reporting is required by the PCC rule needed new counts and time stamps for this rating group, sponsor identity and application service provider identity are started. The type of counters shall depend on the measurement method configured for the PCC rule. When event based charging applies, the first occurrence of an event matching a service data flow template in PCC rule shall imply that a new count is started. When new events occur, the counter shall be increased. Each event shall be time stamped.

-	Termination of service data flow. If service identifier level reporting is required by the PCC rule and this was the last active service data flow for this combination of the rating group and service id or if rating group level reporting is required by the PCC rule and this was the last active service data flow for this rating group, or if sponsored connectivity level reporting is required by the PCC rule and this was the last active service data flow for this combination of rating group, sponsor identity and application service provider identity, the counters and time stamps are closed and added to the PGW-CDR. For information on how the termination of service data flows is detected, refer to TS 23.203 [72].

-	Access of a multi-access PDN connection becomes unusable: the Termination of service data flow chargeable event applies for all the service data flows carried under the IP-CAN bearer of the unusable access.

-	Access of a multi-access PDN connection becomes usable: the Start of service data flow chargeable event applies for all the service data flows carried under the IP-CAN bearer of the access that has become usable.

-	End of IP-CAN bearer in the P-GW. The PGW-CDR is closed upon encountering this trigger.

-	Serving node (e.g. SGSN/S-GW/ePDG/TWAG) change in the P-GW. New SGSN/S-GW/ePDG/TWAG address is added to PGW-CDR.

-	Expiry of an operator configured time limit per IP-CAN bearer. This event closes the PGW-CDR, and a new one is opened if the IP-CAN bearer is still active.

-	Expiry of an operator configured time limit per rating group. The counters and time stamps are closed and added to the PGW-CDR. A new service data flow container is opened if any matching service data flow is still active.

-	Expiry of an operator configured data volume limit per IP-CAN bearer. This event closes the PGW-CDR, and a new one is opened if the IP-CAN bearer is still active.

-	Expiry of an operator configured data volume limit per rating group. The counters and time stamps are closed and added to the PGW-CDR. A new service data flow container is opened if any matching service data flow is still active.

-	Expiry of an operator configured data event limit per rating group. The counters and time stamps are closed and added to the PGW-CDR. A new service data flow container is opened if any matching service data flow is still active.

-	Change of charging condition: IP-CAN bearer modification (e.g. QoS change, SGSN change, S-GW change, user location change,user CSG information change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data off status,  Serving PLMN Rate Control change, APN Rate Control change), tariff time change or failure handling procedure triggering. When this event is encountered, all current configured counts and time stamps are captured and new counts and time stamps for all active service data flows are started.

-	Intersystem change (e.g. change of radio interface from GSM to UMTS, RAT change) visible in the P-GW. This event closes the PGW-CDR, and a new one is opened if the IP-CAN bearer is still active.

-	PLMN change visible in the P-GW. This event closes the PGW-CDR. A new one is opened if the IP-CAN bearer is still active.

-	MS Timezone change visible in the P-GW. This event closes the PGW-CDR. A new one is opened if the IP-CAN bearer is still active.

-	MO exception data counter receipt in the P-GW: This event closes the CDR. A new one is opened if the IP-CAN bearer is still active.

-	SGSN change in the P-GW. New SGSN address is added to PGW-CDR.

-	Expiry of an operator configured report of service flow data limit per IP-CAN bearer. This event closes the PGW-CDR, and a new one is opened if the IP-CAN bearer is still active.

-	Completion of a time envelope as defined in TS 32.299 [50]. This event closes a service data flow container. Further details are described in clause 5.2.3.4.1 "Triggers for PGW-CDR Charging Information Addition"'. The need for reporting time envelopes may be statically configured for each rating group or dynamically controlled by online charging.

Management intervention may also force trigger a chargeable event.

Relevant service data flows for a certain IP-CAN bearer are determined when FBC is applied. PCC rules are used for this determination. One PCC rule identifies service data flow to be measured but it can also include certain characteristics related to that service data flow.

PCC rules can be activated, deactivated and modified any time during the IP-CAN bearer lifetime. PCC rule activation, deactivation and modification are not chargeable events. However these PCC rule changes may lead to "start of service data flow" and "termination of service data flow" chargeable events.

A PCC rule can contain e.g.:

-	service data flow template (service data flow filters or application identifier) to identify packets belonging to certain service data flow,

-	charging method to identify whether online/offline/both/neither charging interface is used,

-	measurement method for offline charging to identify whether time/volume/events are measured for this service data flow,

-	Charging key (i.e. rating group) for that service data flow,

-	service identifier for that service data flow,

-	Sponsor Identifier (offline charging only),

-	Application Service Provider Identifier (offline charging only),

-	application function record information to correlate the measurement with application level reports,

-	reporting level for the service data flow (rating group, combination of the rating group and service id or combination of the rating group, sponsor identity and application service provider identity),

-	precedence to the situations where two or more PCC rules are overlapping,

-	allowed access type (for NBIFOM control) See clause 5.2.1.7A..

PCC rules can be:

-	pre-defined in P-GW (can be activated either by the PCRF or PCEF itself) or,

-	dynamically provisioned and activated by the PCRF over the Gx interface.

This is specified in TS 23.203 [215] and TS 29.212 [216].

According to TS 23.203 [215] , the PCRF can modify the following charging information in a dynamic PCC rule which is active in the PCEF: Charging key, Service identifier, Sponsor Identifier, Application Service Provider Identifier, Measurement method, and reporting level The PCRF can also modify the allowed access type in the case of NBIFOM.. A change of any of this charging or allowed access type information will trigger a "start of service data flow" chargeable event when a valid counter does not exist corresponding to that changed PCC rule. A change of any of this charging or allowed access type information will trigger a "termination of service data flow" chargeable event when this was the last active service data flow for the counter corresponding to the original PCC rule.

When the CDF is implemented as a separate entity,  all these FBC related chargeable events,  trigger charging events  reporting, for P-GW CDRs to be constructed, enriched or closed by CDF, according to description in clause 5.2.2.

Extended packet inspection can be done in the PCEF with pre-defined PCC rules. The PCEF also have the possibility to output service specific information related to the packet inspection in the CDR.

The capability of P-GW to support ABC is achieved with PCRF providing appropriate PCC rules to the P-GW. Such PCC Rule shall be defined with service data flow template including an Application Identifier for the application which needs to be detected, enforced and charged.

For non-IP type PDN connection in the P-GW:

-	non-3GPP access types are not supported, a multi-access PDN connections do not apply;

-	A single PCC Rule is used with the service data flow descriptor matching the whole non-IP data traffic, and can contain e.g.:

-	service data flow template matching all the packets;

-	charging method to identify whether online/offline/both/neither charging interface is used;

-	measurement method for offline charging to identify whether time/data volume/events are measured for this service data flow;

-	Charging key (i.e. rating group) for that service data flow;

-	service identifier for that service data flow;

-	reporting level for that service data flow:

-	rating group level in case a specific rating group is used for non-IP traffic charging;

-	combination of the rating group and service id in case a specific rating group/service Id is used for non-IP traffic charging.

Next change

5.2.1.10.1	IP-CAN bearer charging

For the purpose of interoperator charging, the P-GW collects charging information per user per IP-CAN bearer. 
In case the P-GW is not aware of IP-CAN bearers, i.e. in case of PMIP based connectivity, P-GW collects charging information per IP-CAN session as it would be one IP-CAN bearer. IP-CAN bearer charging allows the P-GW to collect charging information related to data volumes sent to and received by the UE/MS, categorised by the QCI and ARP applied to the IP-CAN bearer. The user can be identified by MSISDN and/or IMSI, while the IP-CAN bearer can be determined by a unique identifier generated by the P-GW when creating an IP-CAN bearer. This identifier is forwarded to the S-GW/ePDG/ TWAG/SGSN so as to allow correlation of S-GW/ePDG/ TWAG/SGSN IP-CAN bearer CDRs with the matching P-GW charging information in the BD.

The amount of data counted for the IP-CAN bearer shall be the user plane payload at the tunnelling (e.g. GTP-u, GRE Tunnel) interface. Time metering is started when IP-CAN bearer is activated.

NOTE 1:	The control plan address of the P-GW, together with the unique charging identifier assigned by the P-GW, enables the correlation of charging information. The control plane IP address of SGSN or P-GW (acting as GGSN) is the IP address used at Gn/Gp interface. 
The control plane IP address of S-GW or P-GW is the IP address used at S5/S8 interface. 
The control plane IP address of ePDG or P-GW is the IP address used at S2b interface. 
The control plane IP address of TWAG or P-GW is the IP address used at S2a interface.

When Charging per IP-CAN session is active and measurements for IP-CAN bearers are captured in the same CDR as FBC measurements, the following chargeable events are defined:

-	Start of the default bearer for an IP-CAN session when single access is used or start of the first default bearer for a multi-access PDN connection(i.e., when NBIFOM is accepted by PCRF). 
Upon encountering this event, a new CDR for the IP-CAN session is created and the data volume counts 
(i.e., uplink and downlink) are started and captured for the IP-CAN bearer.

NOTE 2:	Start of the default bearer or start of the first default bearer for an IP-CAN session is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Addition of access to a PDN connection. 
Additional volume counts are started and captured for IP-CAN bearers of the access. New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Start of a dedicated bearer for an IP-CAN session. 
Additional volume counts are started and captured for the dedicated bearer.

-	End of dedicated bearer in the P-GW. 
The counters and time stamps for the IP-CAN bearer are closed and resulting container added to the CDR.

-	Removal of access from a multi-access PDN connection. 
The counters and time stamps for the IP-CAN bearers of the removed access are closed and resulting containers added to CDR.

-	Access of a multi-access PDN connection becomes unusable. 
The counters and time stamps for the IP-CAN bearers of the unusable access are closed and resulting containers added to the CDR.

-	Access of a multi-access PDN connection becomes usable. 
New volume counts are started and captured for all bearers of the access that has become usable. These may not be the same as those that were previously active when the access became unusable due to changes in PCC Rules.

-	End of IP-CAN session (i.e. end of the default bearer for a single access PDN connection or end of the last default bearer for a multi-access PDN connection) in the P-GW. 
The counters and time stamps for all IP-CAN bearers and the resulting containers added to the CDR. The CDR is closed.

NOTE 3:	The End of IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Serving node (e.g. SGSN/S-GW/ePDG/TWAG) change in the P-GW. 
New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Expiry of an operator configured time limit for keeping a CDR open. 
This event closes all counters. The resulting containers are added to the CDR and the CDR is closed. 
A new CDR is opened if the IP-CAN session is still active.

NOTE 4:	The expiry of an operator configured time limit for keeping a CDR open event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured time limit per IP-CAN bearer. 
The counters and time stamps for the IP-CAN bearer are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Expiry of an operator configured data volume limit per IP-CAN session. 
This event closes the CDR and a new one is opened if the IP-CAN session is still active.

NOTE 5:	The expiry of an operator configured data volume limit per IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured data volume limit per IP-CAN bearer. 
The counters and time stamps are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Change of charging condition specific to APN-AMBR change. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

-	Change of charging condition specific to IP-CAN bearer modification QoS change. 
When this event is encountered, all counts and time stamps for the modified bearer are captured and new counts and time stamps for the specific bearer are started.

-	Change of charging condition. 
IP-CAN bearer modification except QoS change (e.g. SGSN change, S-GW change, user location change, user CSG information change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data off status), or tariff time change. When this event is encountered, all current configured counts and time stamps are captured and new counts and time stamps for all active bearers are started. 
IP-CAN bearer modification except QoS change events are associated with one access of a multi-access PDN connection and all counts associated with affected access are identified with the specific event. All counts associated with the other access of a multi-access PDN connection are identified as an indirect change of charging condition.

Editor’s Note: The handling of  "Serving PLMN Rate Control change", "APN Rate Control change" events when charging per IP-CAN session applies is FFS.

NOTE 7:	The change of charging condition event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Editor’s Note: whether the special case of user location reporting on dedicated bearer release triggers a change of charging condition for the IP-CAN session is ffs.

-	Intersystem change (e.g. change of radio interface from GSM to UMTS, RAT change) for any connected access visible in the P-GW. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 8:	The intersystem change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	PLMN change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 9:	The PLMN change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	MS Timezone change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 10:	The MS Timezone change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured limit of number of charging condition changes per IP-CAN session. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 11:	The expiry of an operator configured limit of number of charging condition changes event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Management intervention may also force trigger a chargeable event.

NOTE 12:	For non-IP type PDN connection, the following do not apply:

-	Addition of access to a PDN connection;

-	Start of a dedicated bearer for an IP-CAN session;

-	End of dedicated bearer in the P-GW;

-	Removal of access from a multi-access PDN connection;

-	Access of a multi-access PDN connection becomes unusable;

-	Access of a multi-access PDN connection becomes usable.

Next change

5.3.1.2	Flow Based bearer Charging

IP-CAN bearer charging allows the P-GW to collect charging information related to data volumes sent to and received by the UE/MS, categorised by the QoS applied to the IP-CAN bearer. FBC is supported by the P-GW by the integrated PCEF. When the PCEF is present, the normal IP-CAN bearer charging is enhanced by the capability to categorise the service data flows within IP-CAN bearer data traffic by rating group or combination of the rating group and service id. I.e., while there is only one uplink and one downlink data volume count per IP-CAN bearer in IP-CAN bearer charging, FBC may provide one count per each rating group or combination of the rating group and service id. The level of the reporting is defined per PCC rule. Details of this functionality are specified in TS 23.203 [215] and TS 32.240 [1].

NOTE: 	The P-GW can only include one QoS Information occurrence per Multiple Unit Operation. 
This implies if an operator wishes to be able to separate usage according to QCI and ARP within their charging system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating reporting is used;

-	rating group/service id where rating group/service id reporting is used.

Extended packet inspection can be done in the PCEF with pre-defined PCC rules. The PCEF also have the possibility to output service specific information related to the packet inspection in the online charging information.

An Application Function (AF) may provide an external charging identifier to be delivered to the PCEF by the PCRF with a dynamic PCC rule. The PCEF includes this AF correlation information in the online charging information for the rating group and service identifier associated with the dynamic PCC rule when service identifier level reporting is requested.

The capability of P-GW to support ABC is achieved with PCRF providing appropriate PCC Rules to the P-GW. Such PCC Rule shall be defined with service data flow template including an Application Identifier for the application which needs to be detected, enforced and charged.

According to TS 23.203 [215], FBC shall support different charging models per PCC rule. These charging models may be based on volume and/or time and on number of events matching a specific service data flow template in PCC rule. 
In general the charging of a service data flow shall be linked to the IP-CAN bearer under which the service data flow has been activated.  In online charging the PCEF shall request the reservation of units  prior to service delivery.

The following chargeable events are defined for FBC when online charging is activated:

-	Network request for IP-CAN bearer activation before the Initiate IP-CAN bearer Activation message is sent. Associated with the network requested dedicated IP-CAN bearer activation procedure, as defined in 
TS 23.203 [215] and TS 23.060 [201], upon encountering this event, a Debit / Reserve Units Request[Initial], indicating the request for activation of dedicated IP-CAN bearer is sent toward the OCS. For network requested dedicated IP-CAN bearer activation, if known by the PCEF, the PCEF may provide status of UE presence in Presence Reporting Area(s).

-	Start of IP-CAN bearer. Upon encountering this event, a Debit / Reserve Units Request[Initial], indicating the start of the IP-CAN bearer, is sent towards the OCS to authorize the IP-CAN bearer. For network requested dedicated IP-CAN bearer activation, this event triggers a Debit / Reserve Units Request[Update], when the PCEF receives an Update PDP Context Request message with the RAN Procedures Ready flag. If known by the PCEF, the PCEF may provide status of UE presence in Presence Reporting Area(s). PCEF may request quota later when service usage is started.

-	Start of service data flow. In case valid quota does not exist a Debit / Reserve Units Request[Update] isgenerated to request quota. In case charging session does not exist, the request is a Debit/ Reserve Units Request[Initial] sent towards the OCS.
The type of requested quota shall depend on measurement method configured for the PCC rule in case of decentralized unit determination. When event based charging applies, the first occurrence of an event matching a service data flow template in PCC rule shall be considered as the start of a service.

-	Termination of service data flow. If reporting is per rating group and this is the last service data flow utilizing that specific rating group or if reporting is per combination of the rating group and service id and this is the last service data flow utilizing that specific rating group and service id, the required counters are updated. 
Termination of the service data flow itself does not trigger Debit / Reserve Units Request[Update].

--	Access of a multi-access PDN connection becomes unusable: the Termination of service data flow chargeable event applies for all the service data flows carried under the IP-CAN bearer of the unusable access.

-	Access of a multi-access PDN connection becomes usable: the Start of service data flow chargeable event applies for all the service data flows carried under the IP-CAN bearer of the access that has become usable.

-	Expiry of Unused Quota timer for the charging session in PCEF, a Debit / Reserve Units Request[Terminate] is sent to OCS indicating that charging session is terminated, and the IP-CAN bearer is still active. In this case the OCS shall allow for a new session to be started with the same Charging Id.

Editor's Note: Unused Quota timer definition needs to cover "no quota consumed for a period of time".

-	End of IP-CAN bearer. Upon encountering this event, a Debit / Reserve Units Request[Terminate], indicating the end of the IP-CAN bearer, is sent towards the OCS together with the final counts.

-	Ro specific chargeable events (e.g. threshold reached, QHT expires, quota exhaustion, validity time reached, forced re-authorization). Corresponding counts for the rating group(s) are closed and Debit / Reserve Units Request[Update]  is triggered according the rules defined in TS 32.299 [50].

-	Change of charging condition: E.g. QoS change, user location change, user CSG information change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data off status, Serving PLMN Rate Control change, and APN Rate Control change.
When this event is encountered and the corresponding re-authorization trigger is armed, all current counts are captured and sent towards the OCS with a Debit / Reserve Units Request[Update].

-	Tariff time change. When this event is encountered, all current counts are captured and a new counts are started. The counts are sent to the OCS in next Debit / Reserve Units Request.

Management intervention may also force trigger a chargeable event.

PCC rules can be activated, deactivated and modified any time during the IP-CAN bearer lifetime. PCC rule activation, deactivation and modification are not chargeable events. However these PCC rule changes may lead to "start of service data flow" and "termination of service data flow" chargeable events. 

According to TS 23.203 [215] , the PCRF can modify the following charging information in a dynamic PCC rule which is active in the PCEF: Charging key, Service identifier, Measurement method, Service identifier level reporting. The PCRF can also modify the allowed access type in the case of NBIFOM.
Change of Charging key, ServiceIdentifier, IP-CAN type,  measurement method or allowed access type will trigger a "start of service data flow" chargeable event when valid quota does not exist corresponding to that changed PCC rule. Change of Charging key, Service Identifier, IP-CAN type, measurement method,  Service identifier level reporting or allowed access type will trigger a"termination of service data flow'" chargeable event when this is the last service data flow utilizing the quota used for the original PCC rule.

For non-IP type PDN connection in the P-GW:

-	non-3GPP access types are not supported, multi-access PDN connections do not apply;

-	A single PCC Rule is used with the service data flow descriptor matching the whole non-IP data traffic, and can contain e.g.:

-	service data flow template matching all the packets;

-	charging method to identify whether online/offline/both/neither charging interface is used;

-	measurement method to identify whether time/volume/events are measured for this service data flow;

-	Charging key (i.e. rating group) for that service data flow;

-	service identifier for that service data flow,

-	reporting level for that service data flow:

-	rating group level in case a specific rating group is used for non-IP traffic charging.

-	combination of the rating group and service id in case a specific rating group/service Id is used for non-IP traffic charging.

Next change

5.3.1.6.2	Flow Based Charging (FBC)

FBC is supported by the P-GW by the integration of a PCEF. With PCEF, charging is enhanced by the capability to categorise the service data flows within IP-CAN session data traffic by rating group or combination of the rating group and service id. For online charging, FBC provides credit management for each rating group and may provide reporting (i.e., counts) per each rating group or combination of the rating group and service id. The level of the reporting is defined per PCC rule. Details of this functionality are specified in TS 23.203 [215] and TS 32.240 [1].

NOTE 1:	Even though an individual service data flow template is bound to a specific IP-CAN bearer, the assigned rating group or combination of rating group and service id applies to the entire IP-CAN session. 
As a result, data traffic from multiple bearers can be included in the count maintained for the rating group or combination of the rating group and service id. This implies if an operator wishes to be able to separate usage according to IP-CAN bearer within their charging system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

NOTE 2:	The P-GW can only include one QoS Information occurrence per Multiple Unit Operation. 
This implies if an operator wishes to be able to separate usage according to QCI and ARP within their charging system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

NOTE 2a: The P-GW can only include one RAT type per service data container.
This implies if an operator wishes to be able to separate usage according to RAT type within their billing system they will need to ensure that services having different RAT type do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

Extended packet inspection can be done in the PCEF with pre-defined PCC rules. The PCEF also have the possibility to output service specific information related to the packet inspection in the online charging information.

The capability of P-GW to support ABC is achieved with PCRF providing appropriate PCC Rules to the P-GW. Such PCC Rule shall be defined with service data flow template including an Application Identifier for the application which needs to be detected, enforced and charged.

According to TS 23.203 [215], FBC shall support different charging models per PCC rule. These charging models may be based on volume and/or time and on number of events matching a specific service data flow template in PCC rule. 
In online charging the PCEF shall request the reservation of units  prior to service delivery.

The following chargeable events are defined for FBC when online charging and Charging per IP-CAN session is activated:

-	Start of IP-CAN session. Upon encountering this event, a Debit / Reserve Units Request[Initial], indicating the start of the IP-CAN session, is sent towards the OCS to authorize the IP-CAN session. PCEF may request quota later when service usage is started.

-	Start of service data flow. If no valid quota has been granted for the rating group, a Debit / Reserve UnitsRequest[Update]  is generated to request quota. In case charging session does not exist, the request is a Debit  / Reserve Units Request[Initial] sent towards the OCS. The type of requested quota shall depend on measurement method configured for the PCC rule in case of decentralized unit determination. When event based charging applies, the first occurrence of an event matching a service data flow template in PCC rule shall be considered as the start of a service.

-	Access change of service data flow. 
If service identifier level reporting is required by the PCC rule and all service data flows for this combination of the rating group and service id are changing from one access to another, or if rating group level reporting is required by the PCC rule and all service data flows for this rating group are changing from one access to another, corresponding counts are closed and Debit / Reserve Units Request[Update] is triggered.

-	Termination of service data flow. If reporting is per rating group and this is the last service data flow utilizing that specific rating group or if reporting is per combination of the rating group and service id and this is the last service data flow utilizing that specific rating group and service id, the required counters are updated. Termination of the service data flow itself does not trigger Debit / Reserve Units Request[Update].

- 	Expiry of Unused Quota timer for the charging session in PCEF, a Debit / Reserve Units Request[Terminate] is sent to OCS indicating that charging session is terminated, and the IP-CAN bearer is still active. In this case the OCS shall allow for a new session to be started with the same Charging Id.

-	End of IP-CAN session. Upon encountering this event, a Debit / Reserve Units Request[Terminate], indicating the end of the IP-CAN session, is sent towards the OCS together with the final counts.

-	Ro specific chargeable events (e.g. threshold reached, QHT expires, quota exhaustion, validity time reached, forced re-authorization). Corresponding counts for the rating group(s) are closed and Debit / Reserve Units Request[Update] is triggered according the rules defined in TS 32.299 [50].

-	Change of charging condition specific to APN-AMBR change: When this event is encountered and the corresponding re-authorization trigger is armed, all current counts are captured and sent towards the OCS with a  Debit / Reserve Units Request[Update].

-	Change of charging condition other than QoS change: E.g. user location change, user CSG information change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data off status. When this event is encountered and the corresponding re-authorization trigger is armed, all current counts are captured and sent towards the OCS with a Debit / Reserve Units Request[Update]. 
Change of charging condition other than QoS change events are associated with one access of a multi-access PDN connection and all counts associated with affected access are identified with the specific event. All counts associated with the other access of a multi-access PDN connection are identified as an indirect change of charging condition.

Editor’s Note: The handling of "Serving PLMN Rate Control change", "APN Rate Control change" events when charging per IP-CAN session applies is FFS.

-	Tariff time change. When this event is encountered, all current counts are captured and new counts are started. 
The counts are sent to the OCS in next Debit / Reserve Units Request.

Management intervention may also force trigger a chargeable event.

PCC rules can be activated, deactivated and modified any time during the IP-CAN session lifetime. PCC rule activation, deactivation and modification are not chargeable events. However these PCC rule changes may lead to "start of service data flow'" and "termination of service data flow'" chargeable events.

According to TS 23.203 [215], the PCRF can modify the following charging information in a dynamic PCC rule which is active in the PCEF: Charging key, Service identifier, Measurement method, Service identifier level reporting. The PCRF can also modify the allowed access type in the case of NBIFOM.
A change of allowed access type in the case of NBIFOM will trigger an "access change for service data flow" chargeable event when all service data flows for the associated counter are changed from one access to another and takes priority over change of charging information.
Change of Charging key, ServiceIdentifier, IP-CAN type or measurement method will trigger a "start of service data flow'" chargeable event when valid quota does not exist corresponding to that changed PCC rule. Change of Charging key, Service Identifier, IP-CAN type or measurement method, or Service identifier level reporting will trigger a "termination of service data flow" chargeable event when this is the last service data flow utilizing the quota used for the original PCC rule.

Removal of an access from a PDN connection is not a trigger for FBC. Subsequent PCC rule handling, may lead to the "access change for a service data flow" or "termination of service data flow" events for the service data flows that were active on the removed access.

For non-IP type PDN connection:

-	non-3GPP access types are not supported;

-	multi-access PDN connections do not apply;

-	A single PCC Rule is used with the service data flow descriptor matching the whole non-IP data traffic, and can contain e.g.:

-	service data flow template matching all the packets;

-	charging method to identify whether online/offline/both/neither charging interface is used;

-	measurement method for offline charging to identify whether time/data volume/events are measured for this service data flow;

-	Charging key (i.e. rating group) for that service data flow;

-	service identifier for that service data flow;

-	reporting level for that service data flow:

-	rating group level in case a specific rating group is used for non-IP traffic charging;

-	combination of the rating group and service id in case a specific rating group/service Id is used for non-IP traffic charging.

Next change

6.1.3	FBC IP-CAN bearer charging data in P-GW (PGW-CDR)

If FBC is enabled and the collection of CDR data is enabled then the P-GW data specified in table 6.1.3.1 shall be available for each IP-CAN bearer when charging per IP-CAN session is not active, or for each IP-CAN session when charging per IP-CAN session is active.

Table 6.1.3.1: P-GW IP-CAN bearer data (PGW-CDR)

| Field | Category | Description |  |
| --- | --- | --- | --- |
| Record Type | M | P-GW IP-CAN bearer record. |  |
| Retransmission | OC | This parameter, when present, indicates that information from retransmitted Charging Data Request has been used in this CDR. |  |
| Served IMSI | C | IMSI of the served party, if available. |  |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEI | OC | The IMEI or IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |  |
| Served 3GPP2 MEID | OC | MEID of the served party's terminal equipment for 3GPP2 access. |  |
| Served MN NAI | OC | Mobile Node Identifier in NAI format (based on IMSI), if available. |  |
| P-GW Address used | M | The control plane IP address of the P-GW used. |  |
| P-GW Address IPv6 | OC | The control plane IPv6 address, in case of IPv4v6 dual stack, of the P-GW used. |  |
| Charging ID | M | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. This field holds the unique Charging Id in the PMIP case. When charging per IP-CAN session is active, this field holds the Charging Id of the EPS default bearer. It is the same as the PDN Connection Charging Id. |  |
| Charging per IP-CAN Session Indicator | Oc | This field indicates whether charging per IP-CAN session is active. If this field is not present, charging per IP-CAN session is not active. |  |
| PDN Connection Charging Id | OM | This field holds the explicit Charging Id for the PDN connection. When NBIFOM is supported, the PGW will assign an explicit PDN Connection Charging ID for the PDN connection. Otherwise the field holds the Charging Id of the EPS default bearer. This field is used to identify different records belonging to same PDN connection. |  |
| Serving node Address | M | List of SGSN/S-GW/TWAG control plane IP addresses, or the ePDG address used during this record. |  |
| Serving node IPv6 Address | OC | List of SGSN/S-GW/TWAG control plane IPv6 addresses, in case of IPv4v6 dual stack, used during this record. |  |
| Serving node Type | M | List of serving node types in control plane (SGSN, SGW, ePDG, AGW, TWAG). The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |  |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the P-GW. |  |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |  |
| PDP/PDN Type | OM | PDP/PDN type, i.e. IPv4, IPv6, IPv4v6 , or PDP type PPP. |  |
| PDP/PDN Type extension | OM | This field indicates Non-IP PDN type. |  |
| SGi PtP Tunnelling Method | OC | This field indicates whether SGi PtP tunnelling method based on UDP/IP or other methods are used for this PDN connection when non-IP PDN connection. |  |
| SCS/AS Address | OC | This field holds the Address of SCS/AS for SGi PtP tunnelling. |  |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, i.e. IPv4 address when PDP/PDN Type is IPv4 or IPv6 prefix when PDP/PDN Type is IPv6 or IPv4v6, or assigned IP address if any when PDP/PDN Type extension is Non-IP. This parameter shall be present except: - when both the PDP type is PPP and dynamic IP-CAN bearer address assignment is used or - when PDP/PDN Type extension is Non-IP, and no IP address has been assigned by the PGW for the PDN connection. |  |
| Served PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. |  |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDP/PDN type is IPv4v6. |  |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if IPv4 address is static when PDN Type is IPv4, or if IPv6 address is static when PDN Type is IPv6 or IPv4v6. |  |
| Dynamic Address Flag extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |  |
| List of Traffic Data Volumes | OC | A list of changes in charging conditions for all bearers within the IP-CAN session. Each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Applicable only for IP-CAN bearer charging when IP-CAN session charging is active. See table 6.1.3.1 for a description of List of Traffic Data Volumes handling. |  |
| List of Service Data | OM | A list of changes in charging conditions for all service data flows within this IP-CAN bearer categorized per rating group or per combination of the rating group and service id or per combination of rating group, sponsor identity and application service provider identity. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Online charging information (PS Furnish Charging Information) may be added per each service data flow container in case it is sent by the OCS. External charging identifiers may be added per each service data flow container in case sent by the PCRF. Failure-Handling: This field shall be present in case P-GW triggers the Failure-Handling procedure. It shall indicate the Failure Handling scenario and the instant the Failure Action is triggered (see annex B). Scenarios: Continue/New Session; Continue/Ongoing Session; Retry&Terminate/Ongoing Session; Terminate/Ongoing Session. When charging per IP-CAN session is active, see table 6.1.3.2 for a description of List of Service Data handling. |  |
| Record Opening Time | M | Time stamp when IP-CAN bearer is activated in this P-GW or record opening time on subsequent partial records. |  |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |  |
| Last MS Time Zone | OC | This field contains the UE Time Zone the UE is located at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| Duration | M | Duration of this record in the P-GW. |  |
| Cause for Record Closing | M | The reason for the release of record from this P-GW. |  |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a single cause is applicable. . |  |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a set of causes is applicable. |  |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |  |
| Node ID | OM | Name of the recording entity. |  |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |  |
| APN Selection Mode | OM | An index indicating how the APN was selected. |  |
| Served MSISDN | OC | The primary MSISDN of the subscriber, if available. |  |
| User Location Information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |  |
| Last User Location Information | OC | This field contains the User Location Information of the UE at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| User Location Information Time | OC | This field contains the time at which the user location information was acquired. |  |
| User CSG information | OC | This field contains the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |  |
| 3GPP2 User Location information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.212 [71] for 3GPP2 access, if available. |  |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN) (SSID and, when available, BSSID of the access point), as defined in TS 29.274 [210] for S2a GTP-based case, or 29.275 [211] for S2a PMIP-based case, if available. |  |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |  |
| Presence Reporting Area Information | OC | This grouped field contains the Presence Reporting Area Identifier for the UE PDN connection and the initial status of the UE presence in the Presence Reporting Area(s), if available. |  |
| Charging Characteristics | M | The Charging Characteristics applied to the IP-CAN bearer. |  |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |  |
| IMS Signalling Context | OC | Included if the IP-CAN default bearer IM-CN Subsystem Signalling Flag is set, see TS 23.060 [201] is used for IMS signalling. |  |
| Serving node PLMN Identifier | OM | This fields holds the PLMN Identifier (MCC and MNC) serving the UE during this record, as received in the "Serving Network” IE over S5/S8 or retrieved from the RAI received over Gn/Gp. When the Serving node is TWAG, this field includes the PLMN identifier used for UE authentication, i.e. the VPLMN in roaming case, and the HPLMN in non-roaming case. |  |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |  |
| PS Furnish Charging Information | OC | Online charging session specific information |  |
| CAMEL Information | OC | Set of CAMEL information related to IP-CAN bearer, if available. This field applies only for GPRS. When charging per IP-CAN session is active, this field is not applicable. |  |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. When NBIFOM is accepted by PCRF, this field holds the first default bearer’s RAT type when charging per IP-CAN session is active. |  |
| Start Time | OC | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session starts. |  |
| Stop Time | OC | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session is terminated. |  |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |  |
| QoS Information | OC | This field indicates the APN-AMBR uplink and downlink information for the IP-CAN session. Applicable only when charging per IP-CAN session is active. |  |
| NBIFOM Support | OC | This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session. |  |
| NBIFOM Mode | OC | This field indicates the NBIFOM mode selected for the PDN connection, when NBIFOM is supported for the IP-CAN session. This field is only present when NBIFOM is accepted. |  |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer, i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |  |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME during this record |  |
| APN Rate Control | Oc | This field holds the APN Rate Controls enforced in the PGW during this record. |  |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |  |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |  |
| 3GPP PS Data off Status | Oc | This field holds the 3GPP PS Data off Status. |  |

Table 6.1.3.2 contains special handling for the fields in the List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active and IP-CAN bearer charging is performed.

Table 6.1.3.2: List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| Data Volume Uplink | OC | This field includes the number of octets transmitted during the use of the packet data services in the uplink direction. |
| Data Volume Downlink | OC | This field includes the number of octets transmitted during the use of the packet data services in the downlink direction. |
| Change Condition | M | This field defines the reason for closing the container as specified in clause 5.2.1.10.1. |
| Change Time | M | This field is a time stamp, which defines the moment when the volume container is closed. |
| User Location Information | OC | This field contains the location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE is located and used during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "user location change". |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area(s). |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer. First container for each QCI/ARP pair includes this field. In following containers this field is present if previous change condition is "QoS change". This field includes: QCI and ARP. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Charging Id | OC | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. Charging Id is generated by P-GW at IP-CAN bearer activation and is included in all containers in order to identify the containers which pertain to the IP-CAN bearer. |
| RAT Type | OC | This field contains the RAT type for the current IP-CAN bearer. |
| Access Availability Change Reason | OC | This field indicates the reason why the availability of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable. This field is only present when one access of a multi-access PDN connection becomes unusable. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. |
| 3GPP PS Data off Status | OC | This field contains the status of the UE’s 3GPP PS Data off Status. |

Table 6.1.3.3 contains special handling for fields in the List of Service Data when charging per IP-CAN session is active. The complete set of fields in the List of Service Data is defined in TS 32.298 [51].

Table 6.1.3.3: List of Service Data in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| AF Record Information | OC | As specified in TS 32.298 [51]. |
| Charging Rule Base Name | OC | As specified in TS 32.298 [51]. |
| ADC Rule Base Name | OC | Not applicable to PGW-CDR. |
| Data Volume Downlink | OC | As specified in TS 32.298 [51]. |
| Data Volume Uplink | OC | As specified in TS 32.298 [51]. |
| Event Based Charging Information | OC | As specified in TS 32.298 [51]. |
| Local Sequence Number | OC | As specified in TS 32.298 [51]. |
| PS Furnish Charging Information | OC | As specified in TS 32.298 [51]. |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. This field includes: QCI and ARP. For non-GBR QCI values, this field also includes the MBR for the downlink and uplink direction. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Rating Group | M | As specified in TS 32.298 [51]. |
| Report Time | M | As specified in TS 32.298 [51]. |
| Result Code | OC | As specified in TS 32.298 [51]. |
| Service Condition Change | M | As specified in TS 32.298 [51]. Only those values specified for the events in clause 5.3.2.X apply. |
| Service Identifier | OC | As specified in TS 32.298 [51]. |
| Service Specific Info | OC | As specified in TS 32.298 [51]. |
| Serving Node Address | OC | As specified in TS 32.298 [51]. |
| Time of First Usage | OC | As specified in TS 32.298 [51]. |
| Time of Last Usage | OC | As specified in TS 32.298 [51]. |
| Tme Usage | OC | As specified in TS 32.298 [51]. |
| User Location Information | OC | As specified in TS 32.298 [51]. |
| 3GPP2 User Location Information | OC | As specified in TS 32.298 [51]. |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area(s). |
| RAT Type | OC | This field contains the RAT type for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. |
| Sponsor Identity | OC | As specified in TS 32.298 [51]. |
| Application Service Provider Identity | OC | As specified in TS 32.298 [51]. |
| Time Quota Mechanism | OC | As specified in TS 32.298 [51]. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| 3GPP PS Data off Status | OC | This field contains the status of the UE’s 3GPP PS Data off Status. |

Next change

6.3.1.2	Definition of the PS Information

PS specific charging information is provided within the PS Information. The fields of the PS Information are indicated with the node (MME, S-GW, ePDG, TWAG, P-GW, TDF) from which the information is sent.

The detailed structure of the PS Information can be found in table 6.3.1.2.1.

Table 6.3.1.2.1: Structure of the PS Information

| Information Element | Category | Description |
| --- | --- | --- |
| Charging Id | OC | This field holds the Charging Id for this IP-CAN bearer (this together with the P-GW Address constitutes a unique identifier for the IP-CAN bearer). This field holds the unique Charging Id in the PMIP case. When charging per IP-CAN session is active, PGW use this field to hold the Charging Id of the EPS default bearer. It is the same as the PDN Connection Charging Id. |
| Charging per IP-CAN Session Indicator | Oc | This field indicates whether charging per IP-CAN session is active. If this field is not present, charging per IP-CAN session is not active. |
| Node Id | OC | This fields holds the name of the Node |
| PDN Connection Charging Id | OC | This field holds the explicit Charging Id for the PDN connection. When NBIFOM is supported, the PGW will assign an explicit PDN Connection Charging ID for the PDN connection. Otherwise the field holds the Charging Id of the EPS default bearer. This field is used to identify different records belonging to same PDN connection. |
| PDP/PDN Type | OC | This field holds the type of IP-CAN bearer, e.g. IP or PPP, or PDN type (i.e. IPv4, IPv6 or IPv4v6, or Non-IP). |
| SGi PtP Tunnelling Method | OC | This field indicates whether SGi PtP tunnelling method based on UDP/IP or other methods are used for this PDN connection when a non-IP PDN type. |
| SCS/AS Address | OC | This field holds the Address of SCS/AS for SGi PtP tunnelling. |
| PDP/PDN Address | OC | This field holds the IP address of the served IMSI allocated for the PDP context / PDN connection, i.e. IPv4 address or IPv6 prefix, or assigned IP address if any when PDP/PDN Type is Non-IP. This parameter shall be present except: - when both the PDP type is PPP and dynamic PDP address assignment is used or, - when PDP/PDN Type is Non-IP, and no IP address has been assigned by the PGW for the PDN connection. It may occur twice within the PS Information field when PDN type is IPv4v6: first occurrence with IPv6 prefix, second occurrence with IPv4 address. |
| PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. |
| Dynamic Address Flag | OC | This field indicates whether served PDP/PDN address is dynamically allocated. This field is missing if address is static. |
| Dynamic Address Flag Extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |
| Negotiated QoS Profile | OC | This field holds the authorized QoS applied to IP-CAN bearer. See NOTE. This field indicates the bandwith limitation, applied to TDF session in TDF case. When charging per IP-CAN session is active in P-GW, this field indicates the APN-AMBR applied to the IP-CAN session. |
| Serving Node Address | OC | This field holds the SGSN/S-GW/TWAG IP address that is used by the control plane for the handling of control messages, or the AGW IP address, or the ePDG address, or the MME address. It may be used to identify the PLMN to which the user is attached. It may occur twice in the case when the serving node has IPv4v6 dual stack control plane. |
| Serving Node Type | OC | This field holds the type of the serving node (SGSN/S-GW/ePDG/AGW/TWAG from PGW/TDF, or SGSN/MME from SGW). |
| SGW Change | OC | This field is present if this is first Charging Data Request after a change from another serving node (from SGW/ePDG/ TWAG/HSGW). |
| PGW Address | OC | This field holds the IP-address of the P-GW that generated the Charging Id. It may occur twice in the case when the P-GW has IPv4v6 dual stack control plane. |
| TDF Address | OC | The control plane IP address of the TDF used. It may occur twice in the case when the TDF has IPv4v6 dual stack control plane. |
| SGW Address | OC | This field holds the IP-address of the S-GW used. It may occur twice in the case when the P-GW has IPv4v6 dual stack control plane. |
| ePDG Address | OC | This field holds the IP-address of the ePDG used. It may occur twice in the case when the ePDG has IPv4v6 dual stack control plane. |
| TWAG Address | OC | This field holds the IP-address of the TWAG used. It may occur twice in the case when the TWAG has IPv4v6 dual stack control plane. |
| MME Number for MT SMS | OC | This field holds the MME international PSTN/ISDN E.164 number used for "SMS over MME Charging". |
| MME Name | OC | This field holds the operation Identity of the MME used for "SMS over MME Charging". |
| MME Realm | OC | This field holds the operation Realm Identity of the MME used for "SMS over MME Charging". |
| CG Address | OC | This field holds the Charging Gateway IP address if available. Only used in EPC Online Charging. |
| IMSI MCC MNC | OC | This field holds the MCC and MNC extracted from the user's IMSI (first 5 or 6 digits, as applicable from the presented IMSI). |
| IMSI Unauthenticated Flag | OC | This field is present when IMSI is provided in Subscription Id and this IMSI is unauthenticated (due to emergency bearer service situations). |
| PGW MCC MNC | OC | This field holds the MCC-MNC of the network the P-GW belongs to. |
| TDF MCC MNC | OC | PLMN identifier (MCC MNC) of the TDF. |
| NSAPI | OC | This field Identifies a particular IP-CAN bearer for the associated PDN and MSISDN/IMSI from creation to deletion. See NOTE. |
| Called Station Id | OC | This field contains the identifier of the access point (APN) the user is connected to. |
| Session Stop Indicator | OC | This field indicates to the OCS that the last IP-CAN bearer of a session is released and that the IP-CAN session has been terminated. This field only indicates to the OCS that the TDF session is terminated in TDF case. |
| Selection Mode | OC | This field contains the Selection mode for this APN received in the Create PDP context request, on E-UTRAN initial attach and UE requested PDN connectivity, or Attach in WLAN on GTP S2a. |
| Charging Characteristics | OC | This field contains the Charging Characteristics for this IP-CAN bearer received in the Create IP-CAN bearer Request Message (only available in R99 and later releases). |
| Charging Characteristics Selection Mode | OC | This field holds information about how the "Charging Characteristics" was selected. |
| Serving Node MCC MNC | OC | This field holds the MCC and MNC serving the UE: extracted from the RAI within the IP-CAN bearer activation or Update messages received over Gn/Gp, or provided by the SGW in the "Serving Network" IE over S5/S8 or by AGW/TWAG (For TWAN, PLMN identifier used for UE authentication, i.e. the VPLMN in roaming case, and the HPLMN in non-roaming case). |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node MCC MNC" has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| MS Time Zone | OC | This field indicates the offset between universal time and local time in steps of 15 minutes of where the MS currently resides. |
| Charging Rule Base Name | OC | This field indicates the name of a pre-defined group of PCC rules residing at the PCEF. It may occur several times within the PS Information field. |
| ADC Rule Base Name | OC | This field indicates the name of a pre-defined group of ADC rules in TDF case. |
| User Location Info | OC | This field indicates details of where the UE is currently located (e.g. SAI, TAI, RAI, CGI, ECGI or access-specific user location information). |
| User Location Info Time | OC | This field contains the time at which the user location information was acquired. |
| User CSG Information | OC | This field indicates details of the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| 3GPP2 User Location Info | OC | This field holds the 3GPP2 User Location Info of where the UE is currently located (i.e. 3GPP2 BSID: Cell-Id, SID, NID), as defined in TS 29.212 [216] |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN) (SSID and, when available, BSSID of the access point), as defined in TS 29.274 [210] for S2a GTP-based case, or 29.275 [211] for S2a PMIP-based case, if available. |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |
| Presence Reporting Area Information | OC | This field contains part of the Presence Reporting Area Information of UE as defined in TS 29.212 [216], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| RAT Type | OC | This field indicates which Radio Access Technology (RAT) is currently serving the UE as defined in TS 29.061 [205]. When NBIFOM is accepted by PCRF, this field holds the first default bearer’s RAT type when charging per IP-CAN session is active. |
| Furnish Charging Information | OC | This field contains the PS Furnish Information Elements. |
| Offline Charging | OC | This field contains the Offline Charging parameters to control offline charging. |
| PDP Context Type | OC | This field indicates the type of a PDP context (i.e. Primary or Secondary). This field is present when using the Gn/Gp reference point only. |
| Traffic data volumes | OC | This field holds the containers associated to a charging condition change on an IP-CAN bearer. This is included when triggers conditions are met (Qos change, tariff time change ...). It may occur several times within the PS Information field. |
| Service data container | OC | This field holds the container associated to a service condition change on a service data flow (categorized per rating group or per combination of the rating group and service id) within this IP-CAN bearer. For TDF case, this field holds the container associated to a charging condition change on an application traffic (categorized per rating group or per combination of the rating group and service id) within this TDF session. It may occur several times within the PS Information field. |
| User Equipment Info | OC | This field holds the identification of the terminal (IMEI or IMEISV…) It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor's Note: Alignement with online charging is needed. |
| Terminal Information | OC | This field holds the identification of the terminal (IMEI or IMEISV, 3GPP2-MEID..).It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor's Note : this parameter is only for offline charging and usage for online charging instead of User-Equipment-info is ffs |
| Start Time | OC | Timestamp when User IP-CAN/TDF session starts. |
| Stop Time | OC | Timestamp when User IP-CAN/TDF session terminates. |
| Change Condition | OC | This field holds the reason for sending Charging Data Request from the PCN Nodes. |
| Diagnostics | OC | This field holds a more detailed reason for the release of theIP-CAN bearer, when a single cause is applicable, and complements the "Change Condition" information. When charging per IP-CAN session is active, PGW uses this field to hold a more detailed reason for the release of the IP-CAN session, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the connection, when a set of causes is applicable. When charging per IP-CAN session is active, PGW uses this field to hold a more detailed reason for the release of the IP-CAN session, when a set of causes is applicable. |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |
| NBIFOM Support | OC | This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session. |
| NBIFOM Mode | OC | This field indicates the NBIFOM mode selected for the PDN connection, when NBIFOM is supported for the IP-CAN session. This field is only present when NBIFOM is accepted. |
| CP CIoT EPS Optimisation indicator | OC | This field indicates whether CP CIoT EPS Optimisation is used during data transfer with the UE (i.e. Control Plane NAS PDU via S11-U between SGW and MME) or not (i.e. User Plane via S1-U between SGW and eNB), if enabled. |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer , i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME. |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |
| APN Rate Control | Oc | This field holds the list of APN Rate Controls enforced in the PGW. |
| 3GPP PS Data off Status | Oc | This field holds the 3GPP Data off Status when UE’s 3GPP Data off status is Activated or Deactivated. |

NOTE:	For network requested secondary IP-CAN bearer activation, the CCR[Initial] does not contain a value of NSAPI, whilst the Negotiated QoS profile reflects the requested QoS profile used in the activation request.

Editor’s Note: The handling of  "CP CIoT EPS Optimisation" when charging per IP-CAN session applies is FFS.

Next change

6.3.2	Detailed message format for offline charging

The following clause specifies per Operation Type the charging data that are sent by MME, S-GW, ePDG, TWAG, P-GW and TDF.

The Operation Types are listed in the following order: S (Start)/I (Interim)/S (Stop)/E (Event). Therefore, when all Operation Types are possible it is marked as SISE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. SIS or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. SI-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Charging Data Request message for PS offline charging.

Table 6.3.2.1: Supported fields in Charging Data Request message

| Information Element | Node Type | S-GW | ePDG | P-GW | TWAG | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Realm |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Domain |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| User Name |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Token |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Service Information with PS and IMS Information |  |  |  |  |  |  |
| Subscription Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| IMSI Unauthenticated Flag |  | SIS- | - | SIS- | - | - |
| Node Functionality |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Id |  | SIS- | SIS- | SIS- | SIS- | - |
| Charging per IP-CAN Session Indicator |  | - | - | S--- | - | - |
| Node Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDN Connection Charging Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Type |  | SIS- | SIS- | SIS- | SIS- | - |
| SGi PtP Tunnelling Method |  | - | - | SIS- | - | - |
| SCS/AS Address |  | - | - | SIS- | - | - |
| PDP/PDN Address |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Address prefix length |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag Extension |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node Address |  | SIS- | - | SIS- | - | SIS- |
| Serving Node Type |  | SIS- | - | SIS- |  | SIS- |
| SGW Change |  | S--- | S--- | - | S--- | - |
| P-GW Address |  | SIS- | - | SIS- | SIS- | SIS- |
| TDF Address |  | - | - | - | - | SIS- |
| SGW Address |  | SIS- | - | - | - | - |
| ePDG Address |  | - | SIS- | - | - | - |
| TWAG Address |  | - | - | SIS- | - | - |
| CG Address |  | - | - | - | - | - |
| IMSI MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PGW MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| TDF MCC MNC |  | - | - | - | - | SIS- |
| NSAPI |  | - | - | SIS- | - | - |
| Called Station Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Session Stop Indicator |  | - | - | --S- | - | --S- |
| Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node MCC MNC |  | SIS- | - | SIS- | - | SIS- |
| MS Time Zone |  | SIS- | - | SIS- | - | SIS- |
| Charging Rule Base Name |  | - | - | SIS- | - | - |
| ADC Rule Base Name |  | - | - | - | - | SIS- |
| User Location Info |  | SIS- | - | SIS- | - | SIS |
| User Location Info Time |  | SIS- | - | SIS- | - | - |
| User CSG Information |  | SIS- | - | SIS- | - | SIS- |
| 3GPP2 User Location Info |  | SIS- | - | SIS- | - | SIS- |
| TWAN User Location Information |  | - | - | SIS- | SIS- | - |
| UWAN User Location Information |  | - | SIS- | SIS- | - | - |
| Presence Reporting Area Information |  | SIS- | - | SIS- | - | SIS- |
| RAT Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Furnish Charging Information |  | - | - | SIS- | - | - |
| Offline Charging |  | - | - | - | - | - |
| QoS Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Traffic data volumes |  | -IS- | -IS- | - | -IS- | - |
| Service data container |  | - | - | -IS- | - | -IS- |
| User Equipment Info |  | - | - | - | - | S--- |
| Terminal-Information |  | S--- | S--- | S--- | S--- | S--- |
| Start time |  | S--- | S--- | S--- | S--- | S--- |
| Stop time |  | --S- | --S- | --S- | --S- | --S- |
| Change Condition |  | -IS- | -IS- | -IS- | -IS- | -IS- |
| Diagnostics |  | --S- | --S- | --S- | --S- | --S- |
| Enhanced Diagnostics |  | --S- | --S- | --S- | --S- | - |
| Low Priority Indicator |  | S--- | - | S--- | - | - |
| CN Operator Selection Entity |  | SIS- | - | SIS- | - | - |
| NBIFOM Support |  | - | - | S--- | - | - |
| NBIFOM Mode |  | - | - | S--- | - | - |
| CP CIoT EPS Optimisation Indicator |  | SIS- | - | - | - | - |
| UNI PDU CP Only Flag |  | SIS- | - | SIS- | - | - |
| Serving PLMN Rate Control |  | SI-- | - | SI-- | - | - |
| APN Rate Control |  | - | - | SI-- | - | - |
| MO exception data counter |  | SIS- | - | SIS- | - | - |
| 3GPP PS Data off Status |  | - | - | SIS- | - | - |

Table 6.3.2.1a illustrates the basic structure of the supported fields in the Charging Data Request message for "SMS over MME Charging".

Table 6.3.2.1a: Supported fields in Charging Data Request message for "SMS over MME Charging"

| Information Element | Node Type | MME |
| --- | --- | --- |
|  | Supported Operation Types | S/I/S/E |
| Session Identifier |  | ---E |
| Originator Host |  | ---E |
| Originator Realm |  | ---E |
| Destination Domain |  | ---E |
| Operation Type |  | ---E |
| Operation Number |  | ---E |
| Operation Identifier |  | ---E |
| User Name |  | ---E |
| Destination Host |  | ---E |
| Operation Interval |  | ---E |
| Origination State |  | ---E |
| Origination Timestamp |  | ---E |
| Proxy Information |  | ---E |
| Route Information |  | ---E |
| Operation Token |  | ---E |
| Service Information with PS, IMS, MMS and SMS Information |  |  |
| Subscription Id |  | ---E |
| Node Functionality |  | ---E |
| Node Id |  | ---E |
| MME Number for MT SMS |  | ---E |
| MME Name |  | ---E |
| MME Realm |  | ---E |
| Serving Node Type |  | ---E |
| SGSN Address |  | ---E |
| Charging Characteristics |  | ---E |
| Charging Characteristics Selection Mode |  | ---E |
| 3GPP User Location Info |  | ---E |
| User Location Info Time |  | ---E |
| RAT Type |  | ---E |
| Terminal Information |  | ---E |
| Recipient Address |  | ---E |
| SMSC Address |  | ---E |
| Originator Address |  | ---E |
| Submission Time |  | ---E |
| Message Id |  | ---E |
| CN Operator Selection Entity |  | ---E |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Charging Data Response message for PS offline charging, and "SMS over MME Charging".

Table 6.3.2.2: Supported fields in Charging Data Response message

| Information Element | Node Type | S-GW | ePDG | TWAG | P-GW | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Result |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Domain |  |  |  |  |  |  |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Error Reporting Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |

Next change

6.3.3	Detailed Message Format for online charging

The following table specifies per Operation type the charging data that are sent P-GW and TDF network element for:

The Operation types are listed in the following order: I [initial]/U [update]/T [terminate]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire filed is not allowed in a node the entire cell is marked as "-".

Note that not for all structured fields the individual field members are listed in the table. Detailed descriptions of the fields are provided in TS 32.299 [50].

Table 6.3.3.1 illustrates the basic structure of the supported fields in the Debit / Reserve Units Request for PS online charging.

Table 6.3.3.1: Supported fields in Debit / Reserve Units Request message

| Information Element | Node Type | P-GW | TDF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | IUT- | IUT- |
| Originator Host |  | IUT- | IUT- |
| Originator Domain |  | IUT- | IUT- |
| Destination Domain |  | IUT- | IUT- |
| Operation Identifier |  | IUT- | IUT- |
| Operation Token |  | IUT- | IUT- |
| Operation Type |  | IUT- | IUT- |
| Operation Number |  | IUT- | IUT- |
| Destination Host |  | IUT- | IUT- |
| User Name |  | IUT- | IUT- |
| Origination State |  | IUT- | IUT- |
| Origination Timestamp |  | IUT- | IUT- |
| Subscriber Identifier |  | IUT- | IUT- |
| Termination Cause |  | --T- | --T- |
| Requested Action |  | IUT- | IUT- |
| Multiple Operation |  | IU-- | IU-- |
| Multiple Unit Operation |  | IUT- | IUT- |
| Subscriber Equipment Number |  | IUT- | IUT- |
| Route Information |  | IUT- | IUT- |
| Service Information |  | IUT- | IUT- |
| Service Information with IMS and PS Information |  |  |  |
| Node Functionality |  | IUT- | IUT- |
| Charging Id |  | IUT- | - |
| Node Id |  | IUT- | IUT- |
| PDN Connection Charging Id |  | IUT- | IUT- |
| PDP/PDN Type |  | IUT- | - |
| SGi PtP Tunnelling Method |  | IUT- | - |
| SCS/AS Address |  | IUT- | - |
| PDP/PDN Address |  | IUT- | IUT- |
| PDP/PDN Address prefix length |  | IUT- | IUT- |
| Dynamic Address Flag |  | IUT- | IUT- |
| Dynamic Address Flag Extension |  | IUT- | IUT- |
| QoS Information |  | IUT- | IUT- |
| Serving Node Address |  | IUT- | IUT- |
| Serving Node Type |  | IUT- | IUT- |
| SGW Change |  | - | - |
| P-GW Address |  | IUT- | IUT- |
| TDF Address |  | - | IUT- |
| CG Address |  | IUT- | IUT- |
| IMSI MCC MNC |  | IUT- | IUT- |
| IMSI Unauthenticated Flag |  | IUT- | - |
| PGW MCC MNC |  | IUT- | IUT- |
| TDF MCC MNC |  | - | IUT- |
| NSAPI |  | IUT- | - |
| Called Station Id |  | IUT- | IUT- |
| Session Stop Indicator |  | --T- | --T- |
| Selection Mode |  | IUT- | IUT- |
| Charging Characteristics |  | IUT- | IUT- |
| Charging Characteristics Selection Mode |  | IUT- | IUT- |
| Serving Node MCC MNC |  | IUT- | IUT- |
| MS Time Zone |  | IUT- | IUT- |
| Charging Rule Base Name |  | IUT- | - |
| ADC Rule Base Name |  | - | IUT- |
| User Location Info |  | IUT- | IUT- |
| User Location Info Time |  | IUT- | - |
| User CSG Information |  | IUT- | IUT- |
| 3GPP2 User Location Info |  | IUT- | IUT- |
| TWAN User Location Information |  | IUT- | IUT- |
| UWAN User Location Information |  | IUT- | - |
| Presence Reporting Area Information |  | IUT- | IUT- |
| RAT Type |  | IUT- | IUT- |
| Furnish Charging Information |  | - | - |
| Offline Charging |  | IUT- | IUT- |
| PDP Context Type |  | IUT- | - |
| Traffic data volumes |  | - | - |
| Service data container |  | - | - |
| User Equipment Info |  | - | - |
| Terminal-Information |  | - | - |
| Start time |  | - | - |
| Stop time |  | - | - |
| Change Condition |  | - | - |
| Diagnostics |  | --T- | --T- |
| Enhanced Diagnostics |  | --T- | - |
| CN Operator Selection Entity |  | - | - |
| NBIFOM Support |  | I--- | - |
| NBIFOM Mode |  | I--- | - |
| CP CIoT EPS Optimisation Indicator |  | -- | - |
| UNI PDU CP Only Flag |  | IUT- | - |
| Serving PLMN Rate Control |  | IU-- | - |
| APN Rate Control |  | IU-- | - |
| MO exception data counter |  | - | - |
| 3GPP PS Data off Status |  | IUT- | - |

Table 6.3.3.2 illustrates the basic structure of the supported fields in the Debit / Reserve Units Response for PS online charging.

Table 6.3.3.2: Supported fields in Debit / Reserve Units Response Message

| Information Element | Node Type | P-GW | TDF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | IUT- | IUT- |
| Operation Result |  | IUT- | IUT- |
| Originator Host |  | IUT- | IUT- |
| Originator Domain |  | IUT- | IUT- |
| Operation Identifier |  | IUT- | IUT- |
| Operation Type |  | IUT- | IUT- |
| Operation Number |  | IUT- | IUT- |
| Operation Failover |  | IUT- | IUT- |
| Multiple Unit Operation |  | IUT- | IUT- |
| Operation Failure Action |  | IUT- | IUT- |
| Redirection Host |  | IUT- | IUT- |
| Redirection Host Usage |  | IUT- | IUT- |
| Redirection Cache Time |  | IUT- | IUT- |
| Route Information |  | IUT- | IUT- |
| Failed parameter |  | IUT- | IUT- |
| Service Information |  | IUT- | IUT- |
| Service Information with PS Information |  |  |  |
| Furnish Charging Information |  | IUT- | IUT- |
| Offline Charging |  | I---- | I---- |
| Presence Reporting Area Information |  | I--- | - |

Next change

6.5	Bindings for EPC offline charging

This clause aims to describe the mapping between the Diameter Accounting AVP and CDR parameter for EPC offline charging.

Table 6.5.1 describes the mapping of the Diameter Accounting AVP to the CDR parameter of ePDG-CDR, TWAG-CDR, SGW-CDR, PGW-CDR and TDF-CDR in EPC offline charging.

Table 6.5.1: Bindings of Accounting AVP to ePDG-/TWAG-/SGW-/PGW-/TDF-CDR parameter

| Diameter Accounting AVP |  | ePDG-/SGW-/PGW-/TDF-CDR parameter |
| --- | --- | --- |
| Called-Station-Id |  | Access Point Name Network Identifier |
| APN-Rate-Control |  | APN Rate Control |
| AF-Correlation-Information |  | AF record information |
| 3GPP-Selection-Mode |  | APN Selection Mode |
| Change-Condition |  | Change Condition |
| Change-Time |  | Change Time |
| 3GPP-Charging-Characteristics |  | Charging Characteristics |
| Charging-Characteristics-Selection-Mode |  | Charging Characteristics Selection Mode |
| 3GPP-Charging-Id |  | Charging Id |
| Charging-Per-IP-CAN-Session-Indicator |  | Charging per IP-CAN Session Indicator |
| Charging-Rule-Base-Name |  | Charging Rule Base Name |
| Change-Condition |  | Cause for Record Closing |
| CN-Operator-Selection-Entity |  | CN Operator Selection Entity |
| CP-CIoT-EPS-Optimisation-Indicator |  | CP CIoT EPS Optimisation Indicator |
| Accounting-Input-Octets |  | Data Volume Uplink |
| Accounting-Output-Octets |  | Data Volume Downlink |
| Diagnostics |  | Diagnostics |
| Dynamic-Address-Flag |  | Dynamic Address Flag |
| Dynamic-Address-Flag-Extension |  | Dynamic Address Flag extension |
| Enhanced-Diagnostics |  | Enhanced Diagnostics |
| ePDG-Address |  | ePDG Address Used |
| ePDG-Address |  | ePDG IPv6 Address |
| IMSI-Unauthenticated-Flag |  | IMSI Unauthenticated Flag |
| 3GPP-User-Location-Info |  | Last User Location Information |
| 3GPP-MS-Time-Zone |  | Last MS Time Zone |
| Traffic-Data-Volumes |  | List of Traffic Data Volumes |
| Service-Data-Container |  | List of Service Data |
| RRC-Cause-Counter |  | MO exception data counter |
| 3GPP-MS-Time-Zone |  | MS Time Zone |
| Node ID |  | Node ID |
| PDN-Connection-Charging-ID |  | PDN Connection Charging Id |
| 3GPP-PDP-Type |  | PDP/PDN Type |
| 3GPP-PDP-Type |  | PDP/PDN Type extension |
| 3GPP-GGSN-MCC-MNC |  | PGW PLMN Identifier, TDF PLMN Identifier |
| GGSN-Address |  | PGW Address Used |
| GGSN-Address |  | PGW IPv6 Address |
| TDF-IP-Address | TDF Address Used |  |
| TDF-IP-Address | TDF IPv6 Address |  |
| Qos-Information |  | EPC QoS Information/Qos Information |
| 3GPP-RAT-Type |  | RAT Type |
| Rating-Group |  | Rating Group |
| Node-Functionality |  | Record Type |
| Change-Time |  | Report Time |
| Result-Code |  | Result Code |
| SCS-AS-Address |  | SCS/AS Address |
| Change-Condition |  | Service Condition Change |
| Service-Identifier |  | Service Identifier |
| Subscription-Id |  | Served IMSI |
| Terminal-Information |  | Served IMEI |
| Subscription-Id |  | Served MN NAI |
| Subscription-Id |  | Served MSISDN |
| Terminal-Information |  | Served 3GPP2 MEID |
| 3GPP-SGSN-MCC-MNC |  | Serving node PLMN Identifier |
| PDP-Address |  | Served PDP/PDN Address |
| PDP-Address |  | Served PDP/PDN Address extension |
| PDP-Address-Prefix-Length |  | Served PDP/PDN Address prefix length |
| Service-Specific-Info |  | Service Specific Info |
| SGSN-Address |  | Serving Node Address |
| SGSN-Address |  | Serving IPv6 Node Address |
| Serving-Node-Type |  | Serving Node Type |
| Serving-PLMN-Rate-Control |  | Serving PLMN Rate Control |
| SGi-PtP-Tunnelling-Method |  | SGi PtP Tunnelling Method |
| SGW-Address |  | S-GW Address used |
| SGW-Address |  | S-GW IPv6 Address |
| SGW Change |  | S-GW Change |
| Start time |  | Start Time |
| Stop-time |  | Stop Time |
| Time-First-Usage |  | Time of First Usage |
| Time–Last-Usage |  | Time of Last Usage |
| Time-Usage |  | Time Usage |
| TWAG-Address |  | TWAG Address Used |
| TWAG-Address |  | TWAG IPv6 Address |
| TWAN-User-Location-Info |  | TWAN User Location Information |
| UNI-PDU-CP-Only-Flag |  | UNI PDU CP Only Flag |
| 3GPP-User-Location-Info |  | User Location Information |
| User-Location-Info-Time |  | User Location Information Time |
| User-CSG-Information |  | User CSG Information |
| UWAN-User-Location-Info |  | UWAN User Location Information |
| Presence-Reporting-Area-Information |  | Presence Reporting Area Information |
| 3GPP2-BSID |  | 3GPP2 User Location information |
| Low-Priority-Indicator |  | Low Priority Indicator |
| NBIFOM-Support |  | NBIFOM Support |
| NBIFOM-Mode |  | NBIFOM Mode |
| Access-Availability-Change-Reason |  | Access Availability Change Reason |
| 3GPP-PS-Data-off-Status |  | 3GPP PS Data off Status |

Table 6.5.2 describes the mapping of the Diameter Accounting AVP to the CDR parameter of S-SMO/S-SMT-CDR in EPC offline charging for SMS.

Table 6.5.2: Bindings of Accounting AVP to S-SMO/S-SMT-CDR parameter

| Diameter Acounting AVP | S-SMO/S-SMT-CDR parameter |
| --- | --- |
| 3GPP-Charging-Characteristics | Charging Characteristics |
| Charging-Characteristics-Selection-Mode | Charging Characteristics Selection Mode |
| Recipient-Address | Destination Number |
| Submission-Time | Event Time Stamp |
| Message Id | Message Reference |
| MME-Name | MME Name |
| MME-Number-for-MT-SMS | Recording Entity |
| MME-Realm | MME Realm |
| Node ID | Node ID |
| Originator-Address | Originating Address |
| Node-Functionality | Record Type |
| Subscription-Id | Served IMSI |
| Terminal-Information | Served IMEI |
| Subscription-Id | Served MSISDN |
| Serving-Node-Type | Serving Node Type |
| SGSN-Address | Serving Node Address |
| SGSN-Address | Serving IPv6 Node Address |
| SMSC-Address | Service Centre |
| 3GPP-User-Location-Info | User Location Information |
| User-Location-Info-Time | User Location Information Time |

NOTE:	The whole set of ePDG-/TWAG-/MME-/SGW-/PGW-/TDF-CDR parameters is described in the TS 32.298 [51].

End of change