3GPP TS 32.298 V15.0.0 (2017-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Charging management;

Charging Data Record (CDR) parameter description

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

charging, management, protocol, CDR, ASN.1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	15

1	Scope	16

2	References	17

3	Definitions, symbols and abbreviations	21

3.1	Definitions	21

3.2	Symbols	21

3.3	Abbreviations	21

4	Architecture considerations	23

5	CDR parameters and abstract syntax	24

5.0	General	24

5.1	CDR parameter description	24

5.1.1	Generic CDR parameters	24

5.1.1.0	Introduction	24

5.1. 1.1.0A	3GPP PS Data Off Status	24

5.1.1.1	Serving Network Identity	24

5.1.1.2	Service Context Id	24

5.1.1.3	Subscription Identifier	24

5.1.1.4	Service Specific Info	25

5.1.1.5	Service Specific Type	25

5.1.1.6	Service Specific Data	25

5.1.1.7	Subscriber Equipment Number	25

5.1.2	Bearer level CDR parameters	25

5.1.2.0	General	25

5.1.2.1	CS domain CDR parameters	25

5.1.2.1.0	Introduction	25

5.1.2.1.1	Additional Charging Information	25

5.1.2.1.2	AoC parameters/change of AoC parameters	25

5.1.2.1.3	Basic Service/change of service/ISDN Basic Service	25

5.1.2.1.4	Call duration	26

5.1.2.1.5	Call reference	28

5.1.2.1.6	Calling/called/connected/translated number	28

5.1.2.1.7	Calling Party Number	28

5.1.2.1.8	CAMEL call leg information	28

5.1.2.1.9	CAMEL information	29

5.1.2.1.10	CAMEL initiated CF indicator	29

5.1.2.1.11	CAMEL modified Service Centre	29

5.1.2.1.12	CAMEL SMS Information	29

5.1.2.1.13	Cause for termination	30

5.1.2.1.14	Channel Coding Accepted/Channel Coding Used	30

5.1.2.1.15	Data volume	30

5.1.2.1.16	Default call/SMS handling	30

5.1.2.1.17	Destination Subscriber Number	30

5.1.2.1.18	Diagnostics	30

5.1.2.1.19	EMS-Digits	31

5.1.2.1.20	EMS-Key	31

5.1.2.1.21	Entity number	31

5.1.2.1.22	Equipment id	31

5.1.2.1.23	Equipment type	31

5.1.2.1.24	Event time stamps	31

5.1.2.1.25	Fixed Network User Rate	32

5.1.2.1.26	Free format data	32

5.1.2.1.27	Free format data append indicator	32

5.1.2.1.28	GsmSCF address	32

5.1.2.1.29	Guaranteed Bit Rate	32

5.1.2.1.30	HSCSD parameters/Change of HSCSD parameters	33

5.1.2.1.31	Incoming/outgoing trunk group	33

5.1.2.1.32	Interrogation result	33

5.1.2.1.33	IMEI Check Event	33

5.1.2.1.34	IMEI Status	33

5.1.2.1.35	JIP Parameter	34

5.1.2.1.36	JIP Query Status Indicator	34

5.1.2.1.37	JIP Source Indicator	34

5.1.2.1.38	LCS Cause	34

5.1.2.1.39	LCS Client Identity	34

5.1.2.1.40	LCS Client Type	34

5.1.2.1.41	LCS Priority	34

5.1.2.1.42	LCS QoS	34

5.1.2.1.43	Level of CAMEL service	35

5.1.2.1.44	Location/change of location	35

5.1.2.1.45	Location Estimate	35

5.1.2.1.46	Location Extension	35

5.1.2.1.47	Location Routing Number (LRN)	35

5.1.2.1.48	Location Type	35

5.1.2.1.49	LRN Query Status Indicator	35

5.1.2.1.50	LRN Source Indicator	36

5.1.2.1.51	Maximum Bit Rate	36

5.1.2.1.52	Measure Duration	36

5.1.2.1.53	Message reference	36

5.1.2.1.54	MLC Number	36

5.1.2.1.55	Mobile station classmark/change of classmark	36

5.1.2.1.56	MOLR Type	36

5.1.2.1.57	MSC Address	36

5.1.2.1.58	MSC Server Indication	37

5.1.2.1.59	Network Call Reference	37

5.1.2.1.60	Notification to MS user	37

5.1.2.1.61	Number of DP encountered	37

5.1.2.1.62	Number of forwarding	37

5.1.2.1.63	Old /new location	37

5.1.2.1.64	Partial Record Type	37

5.1.2.1.65	Positioning Data	37

5.1.2.1.66	Positioning Data	37

5.1.2.1.67	Privacy Override	37

5.1.2.1.68	Radio channel requested/radio channel used/change of radio channel	37

5.1.2.1.69	Rate Indication	38

5.1.2.1.70	Reason for Service Change	38

5.1.2.1.71	Record extensions	38

5.1.2.1.72	Record type	38

5.1.2.1.73	Recording Entity	38

5.1.2.1.74	Redial attempt	38

5.1.2.1.74A	Related ICID	38

5.1.2.1.75	Roaming number	38

5.1.2.1.76	Routing number	38

5.1.2.1.77	Sequence number	39

5.1.2.1.78	Served IMEI	39

5.1.2.1.79	Served IMSI	39

5.1.2.1.80	Served MSISDN	39

5.1.2.1.81	Service centre address	39

5.1.2.1.82	Service Change Initiator	39

5.1.2.1.83	Service key	39

5.1.2.1.84	Short message service result	39

5.1.2.1.85	Speech version supported/Speech version used	39

5.1.2.1.86	Supplementary service(s)	39

5.1.2.1.87	Supplementary service action	40

5.1.2.1.88	Supplementary service action result	40

5.1.2.1.89	Supplementary service parameters	40

5.1.2.1.90	Supplementary service(s)	40

5.1.2.1.91	System type	40

5.1.2.1.92	Transparency indicator	40

5.1.2.1.93	Update result	40

5.1.2.2	PS domain CDR parameters	40

5.1.2.2.A	Introduction	40

5.1.2.2.B	Void	41

5.1.2.2.0	3GPP2 User Location Information	41

5.1.2.2.0aA	Access Availability Change Reason	41

5.1.2.2.0A	Access Line Identifier	41

5.1.2.2.1	Access Point Name (APN) Network/Operator Identifier	41

5.1.2.2.1A	APN Rate Control	41

5.1.2.2.2	APN Selection Mode	41

5.1.2.2.3	CAMEL Charging Information	41

5.1.2.2.4	CAMEL Information	41

5.1.2.2.5	Cause for Record Closing	43

5.1.2.2.6	Cell Identifier	43

5.1.2.2.7	Charging Characteristics	44

5.1.2.2.8	Charging Characteristics selection mode	44

5.1.2.2.9	Charging ID	45

5.1.2.2.9A	CN Operator Selection Entity	45

5.1.2.2.9Aa	CP CIoT EPS Optimisation Indicator	45

5.1.2.2.9B	Charging per IP-CAN Session Indicator	45

5.1.2.2.10	Destination Number	45

5.1.2.2.11	Diagnostics	45

5.1.2.2.12	Duration	45

5.1.2.2.13	Dynamic Address Flag	46

5.1.2.2.13A	Dynamic Address Flag Extension	46

5.1.2.2.13Aa	Enhanced Diagnostics	46

5.1.2.2.13B	EPC QoS Information	46

5.1.2.2.13C	ePDG Address Used	46

5.1.2.2.13D	ePDG IPv6 Address	46

5.1.2.2.14	Event Time Stamps	46

5.1.2.2.15	Void	46

5.1.2.2.15A	Fixed User Location Information	46

5.1.2.2.16	GGSN Address Used	46

5.1.2.2.16A	Void	47

5.1.2.2.17	IMS Signalling Context	47

5.1.2.2.18	IMSI Unauthenticated Flag	47

5.1.2.2.18A	IP-CAN session Type	47

5.1.2.2.18B	IP-Edge Address IPv6	47

5.1.2.2.18C	IP-Edge Address Used	47

5.1.2.2.18D	IP-Edge Operator Identifier	47

5.1.2.2.18E	Last MS Time Zone	47

5.1.2.2.18F	Last User Location Information	47

5.1.2.2.19	LCS Cause	47

5.1.2.2.20	LCS Client Identity	47

5.1.2.2.21	LCS Client Type	48

5.1.2.2.22	LCS Priority	48

5.1.2.2.23	LCS QoS	48

5.1.2.2.24	List of Service Data	48

5.1.2.2.25	List of Traffic Data Volumes	51

5.1.2.2.26	Local Record Sequence Number	54

5.1.2.2.27	Location Estimate	54

5.1.2.2.28	Location Method	54

5.1.2.2.29	Location Type	54

5.1.2.2.29A	Low Priority Indicator	54

5.1.2.2.29B	NBIFOM Mode	54

5.1.2.2.29C	NBIFOM Support	54

5.1.2.2.30	Measurement Duration	54

5.1.2.2.31	Message reference	54

5.1.2.2.32	MLC Number	54

5.1.2.2.32A	MME Name	54

5.1.2.2.32B	MME Realm	55

5.1.2.2.33	MS Network Capability	55

5.1.2.2.34	MS Time Zone	55

5.1.2.2.35	Network Initiated PDP Context	55

5.1.2.2.36	Node ID	55

5.1.2.2.37	Notification to MS user	55

5.1.2.2.37A	Originating Address	55

5.1.2.2.37B	P-GW Address IPv6	55

5.1.2.2.38	P-GW Address Used	55

5.1.2.2.39	P-GW PLMN Identifier	55

5.1.2.2.40	PDN Connection Charging ID	55

5.1.2.2.41	PDP Type	55

5.1.2.2.42	PDP/PDN Type	56

5.1.2.2.42A	PDP/PDN Type Extension	56

5.1.2.2.43	Positioning Data	56

5.1.2.2.43A	Presence Reporting Area Information	56

5.1.2.2.44	Privacy Override	56

5.1.2.2.45	PS Furnish Charging Information	56

5.1.2.2.46	QoS Requested/QoS Negotiated	56

5.1.2.2.47	RAT Type	57

5.1.2.2.48	Record Extensions	57

5.1.2.2.49	Record Opening Time	57

5.1.2.2.50	Record Sequence Number	57

5.1.2.2.51	Record Type	57

5.1.2.2.52	Recording Entity Number	57

5.1.2.2.52A	Retransmission	57

5.1.2.2.53	RNC Unsent Downlink Volume	57

5.1.2.2.54	Routing Area Code/Location/Cell Identifier/Change of location	57

5.1.2.2.54A	S-GW Address IPv6	58

5.1.2.2.55	S-GW Address Used	58

5.1.2.2.56	S-GW Change	58

5.1.2.2.57	Served 3GPP2 MEID	58

5.1.2.2.57A	Served Fixed Subscriber Id	58

5.1.2.2.58	Served IMEI	58

5.1.2.2.58A	SCS/AS Address	58

5.1.2.2.59	void	58

5.1.2.2.60	Served IMSI	58

5.1.2.2.60A	Served IP-CAN session Address	58

5.1.2.2.60B	Served IP-CAN session Address Extension	58

5.1.2.2.61	Served MN NAI	59

5.1.2.2.62	Served MSISDN	59

5.1.2.2.63	Served PDP Address	59

5.1.2.2.64	Served PDP/PDN Address	59

5.1.2.2.64A	Served PDP/PDN Address Extension	59

5.1.2.2.64B	Served PDP/PDN Address prefix length	59

5.1.2.2.65	Service Centre Address	59

5.1.2.2.66	Serving Node Address	59

5.1.2.2.66A	Serving Node IPv6 Address	59

5.1.2.2.67	Serving Node PLMN Identifier	59

5.1.2.2.68	Serving Node Type	59

5.1.2.2.68A	Serving PLMN Rate Control	60

5.1.2.2.68B	SGi PtP Tunnelling Method	60

5.1.2.2.69	SGSN Address	60

5.1.2.2.69A	Void	60

5.1.2.2.70	SGSN Change	60

5.1.2.2.71	Short Message Service (SMS) Result	60

5.1.2.2.72	Start Time	60

5.1.2.2.73	Stop Time	60

5.1.2.2.73aA 	TDF Address Used	60

5.1.2.2.73bA 	TDF IPv6 Address Used	60

5.1.2.2.73cA 	TDF PLMN Identifier	60

5.1.2.2.73cAa	Traffic Steering Policy Identifier Uplink	61

This field contains traffic steering policy identifier in the uplink direction as specified in TS 23.203[203].	61

5.1.2.2.73cAb	Traffic Steering Policy Identifier Downlink	61

5.1.2.2.73dA	TWAG Address Used	61

5.1.2.2.73eA	TWAG IPv6 Address	61

5.1.2.2.73A	TWAN User Location Information	61

5.1.2.2.73B	UNI PDU CP Only Flag	61

5.1.2.2.74	User CSG Information	61

5.1.2.2.75	User Location Information	61

5.1.2.2.75A	User Location Information Time	61

5.1.2.2.76	Void	61

5.1.2.2.77	UWAN User Location Information	61

5.1.2.3	Void	61

5.1.2.4	CP data transfer domain CDR parameters	62

5.1.2.4.1	Introduction	62

5.1.2.4.2	Access Point Name (APN) Network Identifier	62

5.1.2.4.3	APN Rate Control	62

5.1.2.4.4	Cause for Record Closing	62

5.1.2.4.5	Charging Characteristics	62

5.1.2.4.6	Charging Characteristics selection mode	63

5.1.2.4.7	Charging ID	63

5.1.2.4.8	Diagnostics	64

5.1.2.4.9	Duration	64

5.1.2.4.10	 External-Identifier	64

5.1.2.4.11	List of NIDD Submissions	64

5.1.2.4.12	Local Record Sequence Number	64

5.1.2.4.13	Node ID	65

5.1.2.4.14	RAT Type	65

5.1.2.4.15	Record Extensions	65

5.1.2.4.16	Record Opening Time	65

5.1.2.4.17	Record Sequence Number	65

5.1.2.4.18	Record Type	65

5.1.2.4.19	Retransmission	65

5.1.2.4.20	SCEF ID	65

5.1.2.4.21	Served IMSI	65

5.1.2.4.22	Served MSISDN	65

5.1.2.4.23	Serving Node Identity	65

5.1.2.4.24	Serving Node PLMN Identifier	66

5.1.2.4.25	Serving PLMN Rate Control	66

5.1.3	Subsystem level CDR parameters	67

5.1.3.0	General	67

5.1.3.1	IMS CDR parameters	67

5.1.3.1.0	Introduction	67

5.1.3.1.1	Access Correlation ID	67

5.1.3.1.2	Access Network Information	67

5.1.3.1.2aA 	Access Transfer Type	67

5.1.3.1.2A 	Additional Access Network Information	67

5.1.3.1.3	Alternate Charged Party Address	67

5.1.3.1.3A	AoC Information	67

5.1.3.1.4	Application Provided Called Parties	67

5.1.3.1.5	Application Servers Information	67

5.1.3.1.6	Application Servers involved	67

5.1.3.1.7	Void	67

5.1.3.1.8	Bearer Service	67

5.1.3.1.9	Called Party Address	68

5.1.3.1.10	Carrier Select Routing	68

5.1.3.1.11	Cause for Record Closing	68

5.1.3.1.11A 	Cellular Network Information	68

5.1.3.1.12	Content Disposition	68

5.1.3.1.13	Content Length	68

5.1.3.1.14	Content Type	68

5.1.3.1.15	Event	68

5.1.3.1.16	Expires	68

5.1.3.1.16aA	FE Identifier List	69

5.1.3.1.16A	From Address	69

5.1.3.1.17	GGSN Address	69

5.1.3.1.18	GPRS Charging ID	69

5.1.3.1.18A	Void	69

5.1.3.1.19	IMS Charging Identifier	69

5.1.3.1.20	IMS Communication Service Identifier	69

5.1.3.1.20A	IMS Emergency Indicator	69

5.1.3.1.20B	IMS Visited Network Identifier	70

5.1.3.1.21	Incomplete CDR Indication	70

5.1.3.1.21A	Initial IMS Charging Identifier	70

5.1.3.1.21Aa	Instance Id	70

5.1.3.1.21Aaa	Inter-UE Transfer	70

5.1.3.1.21B	IP Realm Default Indication	70

5.1.3.1.21C	ISUP Cause	70

5.1.3.1.21Ca 	List of Access Network Info Change	70

5.1.3.1.21D 	List of Access Transfer Information	70

5.1.3.1.22	List of Associated URI	71

5.1.3.1.23	List of Called Asserted Identity	71

5.1.3.1.23A	List of Called Identity Changes	71

5.1.3.1.24	List of Calling Party Address	71

5.1.3.1.25	List of Early SDP Media Components	71

5.1.3.1.26	List of Inter Operator Identifiers	72

5.1.3.1.27	List of Message Bodies	72

5.1.3.1.27A	List of NNI Information	72

5.1.3.1.28	List of SDP Media Components	72

5.1.3.1.28A	List of Reason Header	73

5.1.3.1.28B	Local GW Inserted Indication	73

5.1.3.1.29	Local Record Sequence Number	73

5.1.3.1.30	Media Initiator Flag	73

5.1.3.1.31	Media Initiator Party	73

5.1.3.1.31a	MS Time Zone	73

5.1.3.1.31aA	MSC Address	73

5.1.3.1.31A	Neighbour Node Address	73

5.1.3.1.31B	NNI Type	73

5.1.3.1.31C	Void	73

5.1.3.1.32	Node Address	73

5.1.3.1.33	Number Portability Routing	73

5.1.3.1.33A	Void	73

5.1.3.1.34	Online Charging Flag	73

5.1.3.1.35	Originator	74

5.1.3.1.35A	Outgoing Session ID	74

5.1.3.1.36	Private User ID	74

5.1.3.1.37	Real Time Tariff Information	74

5.1.3.1.38	Record Closure Time	74

5.1.3.1.39	Record Extensions	74

5.1.3.1.40	Record Opening Time	74

5.1.3.1.41	Record Sequence Number	74

5.1.3.1.42	Record Type	74

5.1.3.1.42A	Related IMS Charging Identifier	74

5.1.3.1.42B	Related IMS Charging Identifier Generation Node	74

5.1.3.1.42A	Relationship Mode	75

5.1.3.1.43	Requested Party Address	75

5.1.3.1.44	Retransmission	75

5.1.3.1.45	Role of Node	75

5.1.3.1.45A	Route header received	75

5.1.3.1.45B	Route header transmitted	75

5.1.3.1.46	SDP Answer Timestamp	75

5.1.3.1.47	SDP Media Components	75

5.1.3.1.48	SDP Media Description:	76

5.1.3.1.49	SDP Media Name	76

5.1.3.1.50	SDP Offer Timestamp	76

5.1.3.1.51	SDP Session Description	76

5.1.3.1.52	SDP Type	76

5.1.3.1.53	Served Party IP Address	76

5.1.3.1.54	Service Delivery End Time Stamp	76

5.1.3.1.54A	Service Delivery End Time Stamp Fraction	76

5.1.3.1.55	Service Delivery Start Time Stamp	76

5.1.3.1.55A	Service Delivery Start Time Stamp Fraction	77

5.1.3.1.56	Service ID	77

5.1.3.1.57	Service Reason Return Code	77

5.1.3.1.58	Service Request Timestamp	77

5.1.3.1.58A	Service Request Timestamp Fraction	77

5.1.3.1.58B	Session Direction	77

5.1.3.1.59	Session ID	77

5.1.3.1.60	Session Priority	77

5.1.3.1.61	SIP Method	77

5.1.3.1.62	SIP Request Timestamp	77

5.1.3.1.63	SIP Request Timestamp Fraction	77

5.1.3.1.64	SIP Response Timestamp	78

5.1.3.1.65	SIP Response Timestamp Fraction	78

5.1.3.1.66	S-CSCF Information	78

5.1.3.1.66A	Status	78

5.1.3.1.66B	TAD Identifier	78

5.1.3.1.67	Tariff Information	78

5.1.3.1.68	Tariff XML	78

5.1.3.1.68A	Transcoder Inserted Indication	78

5.1.3.1.68B	Transit IOI List	78

5.1.3.1.69	Trunk Group ID Incoming/Outgoing	78

5.1.3.1.69A	User Location Information	78

5.1.3.1.70	VLR Number	78

5.1.4	Service level CDR parameters	79

5.1.4.1	MMS CDR parameters	79

5.1.4.1.0	Introduction	79

5.1.4.1.1	3GPP MMS Version	79

5.1.4.1.2	Access Correlation	79

5.1.4.1.3	Acknowledgement Request	79

5.1.4.1.4	Attributes List	79

5.1.4.1.5	Billing Information	79

5.1.4.1.6	Charge Information	79

5.1.4.1.7	Content Type	80

5.1.4.1.8	Delivery Report Requested	80

5.1.4.1.9	Duration of Transmission	80

5.1.4.1.10	Earliest Time of Delivery	80

5.1.4.1.11	Forward Counter	80

5.1.4.1.12	Forwarding Address	80

5.1.4.1.13	Forwarding MMS Relay/Server Address	80

5.1.4.1.14	Limit	80

5.1.4.1.15	Linked ID	80

5.1.4.1.16	Local Record Sequence Number	80

5.1.4.1.17	Managing Address	80

5.1.4.1.18	Message Class	81

5.1.4.1.19	Message Distribution Indicator	81

5.1.4.1.20	Message ID	81

5.1.4.1.21	Message Reference	81

5.1.4.1.22	Message selection	81

5.1.4.1.23	Message Size	81

5.1.4.1.24	MMBox Storage Information	81

5.1.4.1.25	MM component list	81

5.1.4.1.26	MM Date and Time	82

5.1.4.1.27	MM Listing	82

5.1.4.1.28	MM Status Code	82

5.1.4.1.28A	MS Time Zone	82

5.1.4.1.29	MSCF Information	82

5.1.4.1.30	Originator Address	82

5.1.4.1.31	Originator MMS Relay/Server Address	82

5.1.4.1.32	Priority	82

5.1.4.1.33	Quotas	82

5.1.4.1.34	Quotas requested	82

5.1.4.1.35	Read Reply Requested	82

5.1.4.1.36	Read Status	82

5.1.4.1.37	Recipient Address	83

5.1.4.1.38	Recipient MMS Relay/Server Address	83

5.1.4.1.39	Recipients Address List	83

5.1.4.1.40	Record Extensions	83

5.1.4.1.41	Record Time Stamp	83

5.1.4.1.42	Record Type	83

5.1.4.1.43	Reply Charging	83

5.1.4.1.44	Reply Charging ID	83

5.1.4.1.45	Reply Charging Size	83

5.1.4.1.46	Reply Deadline	83

5.1.4.1.47	Report allowed	84

5.1.4.1.48	Request Status code	84

5.1.4.1.49	Routeing Address	84

5.1.4.1.50	Routeing Address List	84

5.1.4.1.51	Sender Address	84

5.1.4.1.52	Sender Visibility	84

5.1.4.1.53	Service code	84

5.1.4.1.54	Start	84

5.1.4.1.55	Status Text	84

5.1.4.1.56	Submission Time	84

5.1.4.1.57	Time of Expiry	84

5.1.4.1.58	Totals	84

5.1.4.1.59	Totals requested	85

5.1.4.1.60	Upload Time	85

5.1.4.1.61	VAS ID	85

5.1.4.1.62	VASP ID	85

5.1.4.2	LCS CDR parameters	85

5.1.4.2.0	Introduction	85

5.1.4.2.1	Home GMLC Identity	85

5.1.4.2.2	LCS Client Identity	85

5.1.4.2.3	LCS Client Type	85

5.1.4.2.4	LCS Priority	85

5.1.4.2.5	Location Estimate	85

5.1.4.2.6	Location Type	85

5.1.4.2.7	Positioning Data	85

5.1.4.2.8	Provider Error	85

5.1.4.2.9	Requesting GMLC Identity	85

5.1.4.2.10	Result code	85

5.1.4.2.11	Target IMSI	86

5.1.4.2.12	Target MSISDN	86

5.1.4.2.13	User Error	86

5.1.4.2.14	Visited GMLC Identity	86

5.1.4.3	PoC CDR parameters	86

5.1.4.3.0	Introduction	86

5.1.4.3.1	Called Party Address	86

5.1.4.3.2	Charged Party	86

5.1.4.3.3	List of Talk Burst Exchange	87

5.1.4.3.4	Number of participants	87

5.1.4.3.5	Participant Access Priority	87

5.1.4.3.6	Participants involved	87

5.1.4.3.7	PoC controlling address	87

5.1.4.3.8	PoC Event Type	87

5.1.4.3.9	PoC group name	87

5.1.4.3.10	PoC session id	87

5.1.4.3.11	PoC session initiation type	88

5.1.4.3.12	PoC session type	88

5.1.4.3.13	User location info	88

5.1.4.3.14	User Participating Type	88

5.1.4.4	MBMS CDR parameters	88

5.1.4.4.0	Introduction	88

5.1.4.4.1	CN IP Multicast Distribution	88

5.1.4.4.2	MBMS 2G 3G Indicator	88

5.1.4.4.2A	MBMS Data Transfer Start	88

5.1.4.4.2B	MBMS Data Transfer Stop	88

5.1.4.4.3	MBMS GW Address	88

5.1.4.4.4	MBMS Service Area	88

5.1.4.4.5	MBMS Service Type	88

5.1.4.4.6	MBMS Session Identity	89

5.1.4.4.7	Required MBMS Bearer Capabilities	89

5.1.4.4.8	TMGI	89

5.1.4.5	MMTel CDR parameters	89

5.1.4.5.0	Introduction	89

5.1.4.5.1	Associated Party Address	89

5.1.4.5.2	List of Supplementary services	89

5.1.4.5.3 	Number Of Diversions	90

5.1.4.5.4 	Participant Action Type	90

5.1.4.5.5	Service Mode	90

5.1.4.5.6 	Service Type	90

5.1.4.5.7	Void	90

5.1.4.6	SMS CDR parameters	90

5.1.4.6.0	Introduction	90

5.1.4.6.1	Event Timestamp	90

5.1.4.6.1A	External Identifier	90

5.1.4.6.2	Local Record Sequence Number	90

5.1.4.6.3	Message Class	90

5.1.4.6.4	Message Reference	90

5.1.4.6.5	Message Size	91

5.1.4.6.6	MTC IWF Address	91

5.1.4.6.7	Originator IMSI	91

5.1.4.6.8	Originator Info	91

5.1.4.6.9	Originator MSISDN	91

5.1.4.6.10	Originator Other Address	91

5.1.4.6.11	Originator Received Address	91

5.1.4.6.12	Originator SCCP Address	91

5.1.4.6.13	RAT Type	91

5.1.4.6.14	Recipient IMSI	91

5.1.4.6.15	Recipient Info	91

5.1.4.6.16	Recipient MSISDN	92

5.1.4.6.17	Recipient Other Address	92

5.1.4.6.18	Recipient Received Address	92

5.1.4.6.19	Recipient SCCP Address	92

5.1.4.6.20	Record Type	92

5.1.4.6.21	Record Extensions	92

5.1.4.6.22	Served IMEI	92

5.1.4.6.23	SM Data Coding Scheme	92

5.1.4.6.24	SM Delivery Report Requested	92

5.1.4.6.25	SM Destination Interface	92

5.1.4.6.26	SM Device Trigger Indicator	92

5.1.4.6.27	SM Device Trigger information	92

5.1.4.6.28	SM Discharge Time	93

5.1.4.6.29	SM DT Priority Indication	93

5.1.4.6.30	SM DT Reference Number	93

5.1.4.6.31	SM DT Validity Period	93

5.1.4.6.32	SM Message Type	93

5.1.4.6.33	SM Originator Interface	93

5.1.4.6.34	SM Originator Protocol Id	93

5.1.4.6.35	SM Priority	93

5.1.4.6.36	SM Recipient Protocol Id	93

5.1.4.6.37	SM Reply Path Requested	93

5.1.4.6.38	SMS Application Port ID	93

5.1.4.6.39	SM Sequence Number	94

5.1.4.6.40	SM Serving Node	94

5.1.4.6.41	Void	94

5.1.4.6.42	Void	94

5.1.4.6.43	SM Status	94

5.1.4.6.44	SM Total Number	94

5.1.4.6.45	SM User Data Header	94

5.1.4.6.45A	SMS Node Address	94

5.1.4.6.45B	SMS Result	94

5.1.4.6.46	Submission Time	94

5.1.4.6.47	UE Time Zone	94

5.1.4.6.48	User Location Info	94

5.1.4.7	ProSe CDR parameters	94

5.1.4.7.0	Introduction	94

5.1.4.7.0A	Announcing PLMN ID	94

5.1.4.7.1	Announcing UE HPLMN Identifier	94

5.1.4.7.2	Announcing UE VPLMN Identifier	95

5.1.4.7.3	Application ID	95

5.1.4.7.4	Cause for Record Closing	95

5.1.4.7.5	Direct Discovery Model	95

5.1.4.7.5A	Discoveree UE HPLMN Identifier	95

5.1.4.7.5B	Discoveree UE VPLMN Identifier	95

5.1.4.7.5C	Discoverer UE HPLMN Identifier	95

5.1.4.7.5D 	Discoverer UE VPLMN Identifier	95

5.1.4.7.6	Layer two Group ID	95

5.1.4.7.6A	List of Application Specific Data	95

5.1.4.7.6B	List of Coverage Info	95

5.1.4.7.6C	List of Radio Parameter Sets	95

5.1.4.7.7	List of Reception Data Containers and List of Transmission Data Containers	96

5.1.4.7.7A	List of Transmitters	96

5.1.4.7.8	Monitored PLMN Identifier	96

5.1.4.7.9	Monitoring UE PLMN Identifier	97

5.1.4.7.10	Monitoring UE Identifier	97

5.1.4.7.11	Monitoring UE VPLMN Identifier	97

5.1.2.7.12	Node ID	97

5.1.4.7.13	PC Three Control Protocol Cause	97

5.1.4.7.14	PC Three EPC Control Protocol Cause	97

5.1.4.7.15	ProSe Application ID	97

5.1.4.7.16	ProSe Event Type	97

5.1.4.7.17	ProSe Function ID	97

5.1.4.7.18	ProSe Function IP Address	97

5.1.4.7.19	ProSe Function PLMN Identifier	97

5.1.4.7.20	ProSe Group IP multicast address	97

5.1.4.7.21	ProSe Reason for Cancellation	97

5.1.4.7.22	ProSe Request Timestamp	98

5.1.4.7.22A	ProSe Target Layer-2 ID	98

5.1.4.7.23	ProSe UE ID	98

5.1.4.7. 23A 	ProSe UE-to-Network Relay UE ID	98

5.1.4.7.24	Proximity Alert Indication	98

5.1.4.7.25	Proximity Alert Timestamp	98

5.1.4.7.26	Proximity Cancellation Timestamp	98

5.1.4.7.27	Proximity Request Renewal Info Block List	98

5.1.4.7.28	Range Class	98

5.1.4.7.29	Reason for Cancellation	98

5.1.4.7.30	Record Type	99

5.1.4.7.30A	Relay IP address	99

5.1.4.7.31	Requested Application Layer User ID	99

5.1.4.7.32	Requested PLMN Identifier	99

5.1.4.7.33	Requestor Application Layer User ID	99

5.1.4.7.34	Requestor EPC ProSe User ID	99

5.1.4.7.35	Requestor PLMN Identifier	99

5.1.4.7.36	Role Of ProSe Function	99

5.1.4.7.37	Role Of UE	99

5.1.4.7.38	Source IP address	99

5.1.4.7.38a	Target IP address	99

5.1.4.7.38A	Time of First Reception	99

5.1.4.7.38B	Time of First Transmission	99

5.1.4.7.39	Time Window	99

5.1.4.7.40	UE Location	100

5.1.4.7.41	Validity Period	100

5.1.4.7.42	WLAN Link Layer ID	100

5.1.4.8	Monitoring Event CDR parameters	100

5.1.4.8.0	Introduction	100

5.1.4.8.1	Accuracy	100

5.1.4.8.2	Chargeable Party Identifier	100

5.1.4.8.3	Event Timestamp	100

5.1.4.8.4	List of Locations	100

5.1.4.8.5	List of Monitoring Event Report Data	100

5.1.4.8.6	Local Record Sequence Number	101

5.1.4.8.7	Location Type	101

5.1.4.8.8	Maximum Detection Time	101

5.1.4.8.9	Maximum Number of Reports	101

5.1.4.8.10	Monitored User	101

5.1.4.8.11	Monitoring Duration	101

5.1.4.8.12	Monitoring Event Config Status	102

5.1.4.8.13	Monitoring Event Configuration Activity	102

5.1.4.8.14	Monitoring Type	102

5.1.4.8.15 	Node ID	102

5.1.4.8.16	Reachability Configuration	102

5.1.2.8.17	Record Opening Time	102

5.1.2.8.18	Record Type	102

5.1.2.8.19	 Retransmission	102

5.1.4.8.20	SCEF ID	102

5.1.4.8.21	SCEF Reference ID	102

5.2	CDR abstract syntax specification	103

5.2.1	Generic ASN.1 definitions	103

5.2.2	Bearer level CDR definitions	112

5.2.2.0	General	112

5.2.2.1	CS domain CDRs	112

5.2.2.2	PS domain CDRs	132

5.2.2.3	Void	150

5.2.2.4	CP data transfer domain CDRs	150

5.2.3	Subsystem level CDR definitions	153

5.2.3.0	Introduction	153

5.2.3.1	IMS CDRs	153

5.2.4	Service level CDR definitions	167

5.2.4.0	General	167

5.2.4.1	MMS CDRs	167

5.2.4.2	LCS CDRs	179

5.2.4.3	PoC CDRs	182

5.2.4.4	MBMS CDRs	185

5.2.4.5	MMTel CDRs	186

5.2.4.6	SMS CDRs	189

5.2.4.7	ProSe CDRs	193

5.2.4.8	Monitoring Event CDRs	197

6	CDR encoding rules	201

6.0	Introduction	201

6.1	3GPP standardized encodings	201

6.2	Encoding version indication	201

Annex A (informative):	Void	202

Annex B (informative):	Bibliography	203

Annex C (informative):	ASN.1 Cross-reference listing and fully expanded sources	204

Annex D (informative):	Change history	205

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in 3GPP networks (GSM/UMTS/EPS). The 3GPP core network charging architecture and principles are specified in document TS 32.240 [1], which provides an umbrella for other charging management documents that specify:

- the content of the CDRs per domain and subsystem (offline charging);

- the content of real-time charging events per domain/subsystem (online charging);

- the functionality of online and offline charging for those domains and subsystems;

- the interfaces that are used in the charging framework to transfer the charging information 
(i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the CDR parameters, the abstract syntax and encoding rules for all the CDR types that are defined in the charging management TSs described above. Therefore, it is only applicable to offline charging. The mechanisms used to transfer the CDRs from the generating node to the operator's Billing Domain (e.g. the billing system or a mediation device) are specified in TS 32.297 [52]. Further details with respect to the operator's Billing Domain for offline charging are out of scope of 3GPP standardisation.

Note that a generic Diameter application for online charging in 3GPP networks is specified in TS 32.299 [50]. Furthermore, 3GPP TSs are being created to standardise some technical aspects of the operator's Billing Domain for online charging, i.e. the Online Charging System (OCS).

All terms, definitions and abbreviations used in the present document, that are common across 3GPP TSs, are defined in the 3GPP Vocabulary, TR 21.905 [100]. Those that are common across charging management in 3GPP domains or subsystems are provided in the umbrella document TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

Furthermore, requirements that govern the charging work are specified in TS 22.115 [101].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] 	Void.

[13] 	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging".

[14] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32]	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

[34]	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35]	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging".

[36]	Void.

[37]	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) charging".

[38]	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring Event charging".

[39]	void

[40]	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

[41] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	Void.

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Records (CDR) file format and transfer".

[53]- [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102]	3GPP TS 22.002: "Circuit Bearer Services (BS) supported by a Public Land Mobile Network (PLMN)".

[103]  	3GPP TS 22.004: "General on supplementary services".

[104] 	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[105] – [199]	void

[200]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[201]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[202]	3GPP TS 23.060: "General Packet Radio Service (GPRS) Service description; Stage 2".

[203]	3GPP TS 23.203: "Policy and Charging control architecture".

[204]	3GPP TS 23.207: "End-to-end Quality of Service (QoS) concept and architecture".

[205]	Void.

[206]	3GPP TS 23.140: "Multimedia Messaging Service (MMS); Functional description; Stage 2".

[207]	3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service; UDI/RDI fallback and service modification; Stage 2".

[208]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[209]  	3GPP TS 24.080: "Mobile radio Layer 3 supplementary service specification; Formats and coding".

[210]	3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[211]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM); Protocol specification".

[212]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[213]	3GPP TS 27.001: "General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS)".

[214]  	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[215]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[216]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[217]	3GPP TS 29.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL); CAMEL Application Part (CAP) specification".

[218]  	3GPP TS 29.140: "Multimedia Messaging Service (MMS); MM10 interface Diameter based protocol; Stage 3".

[219]	3GPP TS 29.207: "Policy control over Go interface".

[220]	3GPP TS 29.212: "Policy and Charging control over Gx reference point".

[221]	3GPP TS 29.214: "Policy and Charging Control; Reference points".

[222]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[223]	3GPP TS 29.274: "Evolved GPRS Tunnelling Protocol for Control Plane (GTPv2-C); Stage 3".

[224]	3GPP TS 29.275: " Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols;    Stage 3".

[225]	3GPP TS 29.658: "SIP Transfer of IP Multimedia Service Tariff Information".

[226] 	3GPP TS 36.413 "Evolved Universal Terrestrial Radio Access (E-UTRA); S1 Application Protocol (S1AP)".

[227]	3GPP TS 49.031: "Location Services (LCS); Base Station System Application Part LCS Extension (BSSAP-LE)".

[228] 	3GPP TS 32.015: "Telecommunication management; Charging management; Charging data description for the Packet Switched (PS) domain".

[229] 	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services".

[230] 	3GPP TS 29.338: "Diameter based protocols to support SMS capable MMEs".

[231]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications".

[232] - [234] 	void

[235]	3GPP TS 23.303: "Proximity-based services (ProSe)".

[236]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects".

[237] - [240] 	void

[241]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[242]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh Interface; Signalling flows and message contents".

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[245]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[246] - [299] 	void

[300]	ITU-T Recommendation X.680 | ISO/IEC 8824-1: "Information technology; Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[301]	ITU-T Recommendation X.690 | ISO/IEC 8825-1: "Information technology - ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

[302]	ITU-T Recommendation X.691 | ISO/IEC 8825-2: "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[303]	ITU-T Recommendation X.693 | ISO/IEC 8825-4: "Information technology - ASN.1 encoding rules: XML encoding rules (XER)".

[304]	ITU-T Recommendation X.2ab CMIP.

[305]	ITU-T Recommendation X.721 ISO/IEC 10165-2: " Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[306]	ITU-T Recommendation X.2cd ACSE: "…".

[307]	ITU-T Recommendation Q.773: "Transaction capabilities formats and encoding".

[308]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[309]	ITU-T Recommendation Q.767: "Application of the ISDN user part of CCITT signalling system No. 7 for international ISDN interconnections".

[310]	ETS 300 196: "Digital Subscriber Signalling System No. one (DSS1) protocol".

[311]	OMA Location Working Group "Mobile Location Protocol Specification", [http://www.openmobilealliance.org].

[312]	ETSI GSM 05.01: "Digital Cellular Telecommunications System (Phase 2+); Physical Layer on the Radio Path; General Description".

[313]	ETSI GSM 08.08: "European Digital Cellular Telecommunication System (Phase 2); Mobile-Services Switching Centre - Base Station System (MSC - BSS) Interface Layer 3 Specification".

[314]	ETSI TS 283 034 v2.2.0: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Network Attachment Sub-System (NASS); e4 interface based on the DIAMETER protocol".

[315]	ITU-T Recommendation X.121: "…".

[316] – [399]	void

[400]	IETF RFC 822 (1982): "Standard for the format of arpa internet text messages".

[401]	IETF RFC 3261(2002): "SIP: Session Initiation Protocol".

[402]	IETF RFC 3966 (2004): "The tel URI for Telephone Numbers".

[403]	IETF RFC 3265 (2002): "Session Initiation Protocol (SIP)-Specific Event Notification".

[404]	IETF RFC 7315 (2014): "Private Header (P-Header) Extensions to the Session Initiation Protocol (SIP) for the 3rd-Generation Partnership Project (3GPP)".

[405]	IETF RFC 2486 (1999): "The Network Access Identifier".

[406]	IETF RFC 4566 (2006): "SDP: Session Description Protocol".

[407]	IETF RFC 5031 (2008): "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[408]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[409] – [600]	void

[601]	Broadband Forum TR-134: "Broadband Policy Control Framework (BPCF)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100], TS 32.240 [1] and TS 32.297 [42] as well as the following apply:

Billing Domain: part of the operator network, which is outside the core network, which receives and processes CDR files from the core network charging functions. It includes functions that can provide billing mediation and billing or other (e.g. statistical) end applications. It is only applicable to offline charging (see "Online Charging System" for equivalent functionality in online charging).

Charging Data Record (CDR): formatted collection of information about a chargeable event (e.g. time of call set-up, duration of the call, amount of data transferred, etc) for use in billing and accounting. For each party to be charged for parts of or all charges of a chargeable event a separate CDR is generated, i.e. more than one CDR may be generated for a single chargeable event, e.g. because of its long duration, or because more than one charged party is to be charged.

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered.

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with bearer/session/service control is required.

## 3.2	Symbols

For the purposes of the present document, the following symbols as specified in TR 21.905 [100], TS 32.240 [1], 
TS 32.297 [52], TS 23.060 [202] and the following apply:

Bx	The Interface between a Charging Gateway Function (CGF) and the Billing Domain (BD)

Ga	Interface between a node transmitting CDRs (i.e. CDF) and a CDR receiving functionality (CGF)

Gn	Interface between two GSNs within the same PLMN.

Gp	Interface between two GSNs in different PLMNs. The Gp interface allows support of GPRS network services across areas served by the co-operating GPRS PLMNs.

Rf	Offline Charging Reference Point between a Charging Trigger Function (CTF) and the Charging Data Function (CDF)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [100] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [100].

3GPP	3rd Generation Partnership Project

ADC	Application Detection and Control

ASN.1	Abstract Syntax Notation One

BD	Billing Domain

BER	Basic Encoding Rules

CS	Circuit Switched

CDF	Charging Data Function

CDIV	Communication Diversion

CDR	Charging Data Record

CGF	Charging Gateway Function

CIoT		Cellular Internet of Things

CP	Control Plane

CPCN	Control Plane data transfer Charging Node (MME, SCEF, IWK-SCEF)

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

CTF	Charging Trigger Function

GPRS	General Packet Radio Service

ISC	IMS Service Control

IM	IP Multimedia

IMS	IM Subsystem

IMS-AGW	IMS Access Media Gateway

ISO	International Organisation for Standardisation

ITU	International Telecommunication Union

IP	Internet Protocol

IWK-SCEF	Interworking SCEF

LAN	Local Area Network

LCS	LoCation Service

MCC	Mobile Country Code

MME	Mobility Management Entity

MMS	Multimedia Messaging Service

MMTEL	MultiMedia Telephony

MNC	Mobile Network Code

NetLoc	Network provided Location information

NIDD	Non-IP Data Delivery

NNI	Network to Network Interface

PCN	Packet switched Core network Node (SGSN, S–GW, P–GW, TDF)

PER	Packed Encoding Rules

P-GW	PDN GateWay

PCC	Policy and Charging Control

PLMN	Public Land Mobile Network

PS	Packet Switched

RG	Residential Gateway

RDI	Restricted Digital Information

S-GW	Serving GateWay

SCUDIF	Service Change and UDI/RDI Fallback

SMS	Short Message Service

TDF	Traffic Detection Function

TrGW	Transition GateWay

UDI	Unrestricted Digital Information

TWAG	Trusted WLAN Access Gateway

TWAN	Trusted WLAN Access Network

UMTS	Universal Mobile Telecommunications System

UWAN	Untrusted Wireless Access Network

WLAN	Wireless LAN

XER	XML Encoding Rules

XML	eXtensible Mark-up Language

# 4	Architecture considerations

Figure 4.1 provides a high level view of the parts of the charging architecture that are relevant for the present document.  The arrows depict the direction of the charging information flow, where Rf carries charging events, Ga carries CDRs and Bx carries CDR files.

3GPP network

CN Domain

Service node

Subsystem

C

D

F

C

G

F

Billing Domain

Rf

Ga

Bx

Figure 4.1:  Logical offline charging architecture

The present document specifies the parameters, abstract syntax and encoding rules for all 3GPP defined CDR types as applicable to the Bx interface, i.e. the CDR files.

CDF and CGF may or may not be integrated with each others, the core network or service nodes, or the BD. 
The possibilities for integration or distribution of these functions are described for each domain, subsystem or service in the respective domain/subsystem/service specific charging TS.  In the distributed case, the 3GPP standardized reference points/interfaces depicted above, shall be used.

Refer to TS 32.240 [1] for a complete description of the charging architecture. 
Refer to TS 32.297 [52] for the description of the CGF's file based interface to the BD.

# 5	CDR parameters and abstract syntax

## 5.0	General

This clause specifies the parameters and the abstract syntax of the CDRs defined for 3GPP charging management in references TS 32.250 [10] to TS 32.275 [35]. In doing this, the ASN.1 specified by X.680 [300] is utilized as the notational tool.

This clause is organised in two parts:

- 	the first part describes the CDR parameters;

- 	the second part specifies the abstract syntax of the CDRs as seen in the CDR files transferred across the Bx interface.

Each part is further subdivided into a number of subclauses that contain generic, bearer level, service level, and subsystem level CDR parameters and abstract syntax definitions. Word processing features, such as formatting options, have also been used to enhance human readability.

The complete set of all CDR syntax definitions is replicated in annex A in a machine processable format. Technically, the contents of this clause and annex A are completely identical. In case of deviations between this clause and annex A due to errors in the present document, the annex shall prevail.

Note that the encoding rules for the abstract syntax specified in this clause, are detailed in clause 6.

## 5.1	CDR parameter description

Editor's note:  this subclause needs to be aligned with clause 5.2 concerning the allocation of parameters to the domains versus making them generic.

### 5.1.1	Generic CDR parameters

#### 5.1.1.0	Introduction

This subclause contains the description of generic CDR parameters, where the term "generic" implies that these parameters are applicable to CDR types of more than one domain/service/subsystem.

##### 5.1. 1.1.0A	3GPP PS Data Off Status

This field holds the Status of UE’s 3GPP PS Data Off.

#### 5.1.1.1	Serving Network Identity

This field contains a SGSN PLMN Identifier (Mobile Country Code and Mobile Network Code), of the SGSN that was used during the Location request. In case the SGSN changes during the transaction, only the ID of the SGSN that was used at the beginning of the transaction is included in the CDR.

The MCC and MNC are coded as described for 'Routing Area Identity' in TS 29.060 [215].

#### 5.1.1.2	Service Context Id

This field contains the Service Context Id from Diameter Accounting, if Diameter Accounting was used and the field included. It identifies the service, sub-system or domain and release that the CDR is applicable to. The contents are described in TS 32.299 [50].

#### 5.1.1.3	Subscription Identifier

This field identifies the charged party . The contents are coded in a similar fashion as for the Subscription-Id AVP in TS 32.299 [50].

#### 5.1.1.4	Service Specific Info

This grouped field holds the sub-fields "service specific data" and "service specific type" if and as provided by an Application Server or PCEF only for pre-defined PCC rules or TDF only for pre-defined ADC rules.

#### 5.1.1.5	Service Specific Type

This field holds the type of the Service Specific Data parameter.

#### 5.1.1.6	Service Specific Data

This field contains the value of service specific data.

#### 5.1.1.7	Subscriber Equipment Number

The Subscriber Equipment Number field contains the identification of the mobile device (e.g. IMEI) that the subscriber is using.

### 5.1.2	Bearer level CDR parameters

#### 5.1.2.0	General

This clause contains the description of the CDR parameters that are specific to the bearer level CDR types. 
This comprizes the CDR types from the Circuit Switched (CS) domain (TS 32.250 [10]), the Packet Switched (PS) domain, i.e. GPRS (TS 32.251 [11]).

#### 5.1.2.1	CS domain CDR parameters

##### 5.1.2.1.0	Introduction

This clause contains the description of the CDR parameters that are specific to the CS domain CDR types as specified in TS 32.250 [10].

##### 5.1.2.1.1	Additional Charging Information

This field consists of two parts, a charge indicator and additional charging parameters. The charge indicator is derived from the information contained within the ISUP "backward call indicator" and may be used to store a charge indicator (charge/no charge) received from another network node. The additional charging parameters are non-standard and intended to permit the inclusion of further charging information received from Intelligent Network and/or Value Added Service nodes.

##### 5.1.2.1.2	AoC parameters/change of AoC parameters

The AoC parameter field contains the set of charge advice (AoC) parameters sent to the MS on call set-up. If further sets of parameters are sent during the call, as a result of a tariff switch-over for example, then this may be recorded in the Change of AoC Parameter field including the time at which the change occurred.

It should be noted that the Change of AoC Parms. field is optional and not required if partial records are generated on tariff switch-over.

The AoC parameters are defined in TS 22.024 [104].

##### 5.1.2.1.3	Basic Service/change of service/ISDN Basic Service

The basic service field contains the code of the basic service employed on call set-up. Any alteration to the basic service during the connection may be recorded in the change of service field including the time at which the change took place.

The change of service field is optional and may be omitted if partial records are created whenever the basic service is changed.

The coding of basic services is defined in detail in TS 29.002 [214].

In the case of the transit record the GSM basic service employed is generally not available. However, if the device on which the call originates/terminates is connected via ISDN digital subscriber signalling then the appropriate ISDN basic service code may be recorded in the record. One possible example includes the direct connection of an ISDN PABX to an MSC/VLR.

##### 5.1.2.1.4	Call duration

This field contains the relevant call duration in seconds. For incomplete calls (call attempts) the relevant duration is the call holding time from the seizure to the release of the traffic channel. For complete (answered) calls this is the chargeable duration from answer to release of the traffic channel. For partial records this is the duration of the individual partial record and not the cumulative duration of the call.

It should be noted that the time stamps may be expressed in terms of tenths of seconds or even milliseconds and, as a result, the calculation of the call duration may result in the rounding or truncation of the measured duration to a whole number of seconds.

Whether or not rounding or truncation is to be used is considered to be outside the scope of the present document subject to the following restrictions:

1)	A call duration of zero seconds shall not be accepted.

2)	The same method of truncation/rounding shall be applied to both single and partial records.

If CAMEL is invoked for the call and a control relationship is existing, the call might continue after a RELEASE or a DISCONNECT from the called party side received by the gsmSSF. The call duration of the incoming leg is stored in the main body of the call record. For each outgoing leg the call duration is stored in the respective 'CAMELInformation' module. If a call leg does not reach answer status and attempt charging is enabled a 'CAMELInformation' module containing the holding time is generated.

An example of how to use the call duration and the timestamps is given in figure 5.1.2.1.4.1. It shows a CAMEL controlled mobile originated follow-on scenario. The uppermost arrow  marks the over all duration of the call that is to be measured and stored in the main body of the respective MOC record. The duration before t5 (incoming leg) or t4 (outgoing leg) needs not to be stored since the call is answered later on. The call duration in the first outgoing leg module contains the time interval from t4 to t6 (period ). The call duration measurement of the second outleg is started with t9 and ended with t10 (interval ).

Since the last outgoing leg is not answered, the respective module contains the holding time starting with t11 and ending with t13 (period ).

(The timestamps t1, t2, t3, t7, t8 and t12 are mentioned for completion reasons only.)

| Point in time | Signalling message sent/received trigger detection point encountered | Duration logging |
| --- | --- | --- |
| t1 | SETUP; TDP(control) |  |
| t2 | IAM | seizure of outg. leg 1 |
| t3 | ACM |  |
| t4 | ANSWER | start of call duration (outg. leg 1) |
| t5 | CONNECT | start of call duration (inc. leg) |
| t6 | RELEASE; EDP(control) | stop of call duration (outg. leg 1) |
| t7 | IAM | seizure of outg. leg 2 |
| t8 | ACM |  |
| t9 | ANSWER | start of call duration (outg. leg 2) |
| t10 | RELEASE; EDP(control) | stop of call duration (outg. leg 2) |
| t11 | IAM | seizure of outg. leg 3 start of holding time (outg. leg 3) |
| t12 | ACM |  |
| t13 | RELEASE; EDP(control) | stop of holding time (outg. leg 3) |

Figure 5.1.2.1.4.1: Call duration measurement in follow-on scenarios

##### 5.1.2.1.5	Call reference

This field uniquely identifies a call or transaction on one side of the interface (i.e. 'A' or 'B' side) and is derived from the transaction identifier of TS 24.008 [208]. It is also used to identify all partial records and transactions belonging to the same connection.

For the avoidance of doubt, there is no global call reference defined within GSM and the call reference field cannot be used to combine, for example, the MOC and MTC records of a mobile-to-mobile connection.

##### 5.1.2.1.6	Calling/called/connected/translated number

In general an Recommendation E.164 [308] number but may also include other numbering plans e.g. ITU-T Recommendation X.121 [315]. Each of these fields includes the type of number and number plan as specified in detail in TS 24.008 [208]. Where appropriate, these fields may also contain the presentation and screening information also specified in TS 24.008 [208].

The called number is the number received from the mobile station on mobile originated call set-up as defined in TS 24.008 [208]. Similarly, the calling number is the number received from the network on mobile terminated call set-up. In case of CAMEL initiated Call Forward (CF), the called (forwarded-to) number is returned by CAMEL.

The translated number is the result of any digit translation performed by the MSC on the called number received from the mobile station on mobile originated call set-up. This parameter is not included in the CDR if no digit translation has taken place.

The connected number is the number of the actual party reached as defined in TS 24.008 [208]. Although this is normally identical to the called number it may differ. This parameter is not included if identical to the called number.

The following examples are intended to explain the use of these fields:

EXAMPLE 1:	Called Number = Connected Number

Normal call from a mobile subscriber to a mobile subscriber or to a PSTN subscriber.

EXAMPLE 2:	Called Number != Connected Number

In case of routing to a PABX with Automatic Call Distribution or to an ISDN Basic Access with several devices attached. The connected number is that of the party actually reached. N.B. The recording of the actual number connected may be limited by the capability of intermediate signalling connections.

EXAMPLE 3:	MTC record for Call Forwarding ("A" -> "B" -> "C")

In case of call forwarding, the connected number recorded in the MTC record of the "B" subscriber is that of the forwarded-to party or "C" subscriber. The calling party field contains the number of the "A" subscriber.

EXAMPLE 4:	Translated Number

This field is only present if digit translation is applied by the MSC to the called number received from the mobile station. Examples include abbreviated dialling codes and service numbers.

##### 5.1.2.1.7	Calling Party Number

This field contains Calling Party Number modified by CAMEL service.

##### 5.1.2.1.8	CAMEL call leg information

This field contains a set of CAMEL information IEs according to the number of outgoing CAMEL call legs.

##### 5.1.2.1.9	CAMEL information

This field contains a list of parameters with information related to one CAMEL outgoing call leg. This parameter list is an Information Element (IE) used in the CAMEL Call Leg Information field.

As a network option, parameters that are identical to the corresponding values in the top level structure of the record are not recorded again. That means whenever a value is not mentioned in this set the value provided in the basic record is valid instead. This might lead to an empty or even absent structure, if no parameter was modified.

##### 5.1.2.1.10	CAMEL initiated CF indicator

The purpose of this field is to distinguish CAMEL call forwarding service scenarios from standard GSM call forwarding scenarios.

From the Basic Call State Model (BCSM)'s point of view this field is set to 'CF' whenever the Originating CAMEL Subscription Information (O_CSI) was applied after terminating CAMEL call processing had been taken place changing the call destination. For the avoidance of doubt: this flag does not depend on other modified call parameter(s) (e.g.: redirection information, etc.) received in the CAP_CONNECT message of the Terminating CAMEL Subscription Information (T_CSI) service.

This flag also indicates that another record might be generated, one containing the charging information related to the terminating CAMEL service and one containing the charging information related to the originating CAMEL service.

##### 5.1.2.1.11	CAMEL modified Service Centre

This field contains SMS-C address modified by CAMEL service. If this field is present the field Service Centre contain SMS-C address before CAMEL modification.

##### 5.1.2.1.12	CAMEL SMS Information

This field contains following CAMEL information for mobile originated and terminated SMS:

-	Default SMS handling:

This field indicates whether or not a CAMEL encounters default SMS handling. This field shall be present only if default SMS handling has been applied.

-	Free format data:

See clause 5.1.2.1.26.

-	Calling Party Number:

This field contains Calling Party Number modified by CAMEL service.

-	CAMEL modified Service Centre:

This field contains SMS-C address modified by CAMEL service.

NOTE 1:	This field is only applicable for originated SMS.

-	CAMEL Destination Subscriber Number

This field contains short message Destination Number modified by CAMEL service.

NOTE 2:	This field is only applicable for originated SMS.

-	SMS Reference Number:

This field contains the SMS Reference Number assigned to the Short Message by the MSC.

##### 5.1.2.1.13	Cause for termination

This field contains a generalised reason for the release of the connection including the following:

-	normal release;

-	CAMEL initiated call release;

-	partial record generation;

-	partial record call re-establishment;

-	unsuccessful call attempt;

-	abnormal termination during the stable phase;

-	unauthorized network originating a location service request;

-	unauthorized client requesting a location service;

-	position method failure at a location service execution;

-	unknown or unreachable LCS client at a location service request.

A more detailed reason may be found in the diagnostics field.

##### 5.1.2.1.14	Channel Coding Accepted/Channel Coding Used

A list of traffic channel codings for HSCSD connections accepted/negotiated by the MS.

These parameters are only present in the CDRs for HSCSD connections.

##### 5.1.2.1.15	Data volume

This field includes the number of 64 octet segments transmitted during the use of data services if known.

##### 5.1.2.1.16	Default call/SMS handling

This field indicates whether or not a CAMEL encountered default call/SMS handling. This field shall be present only if default call/SMS handling has been applied. Parameter is defined in HLR as part of CAMEL subscription information.

##### 5.1.2.1.17	Destination Subscriber Number

This field contains Destination/Called Subscriber Number modified by CAMEL service. If not modified then this field may contain original Destination Number also when CAMEL is not active.

##### 5.1.2.1.18	Diagnostics

This field includes a more detailed technical reason for the release of the connection and may contain one of the following:

-	a MAP error from TS 29.002 [214];

-	a Cause from TS 24.008 [208];

-	a Cause from TS 29.078 [217];

-	a Cause from ITU-T Recommendation Q.767 [309];

-	a LCS diagnostics according TS 29.002 [214].

The diagnostics may also be extended to include manufacturer and network specific information.

##### 5.1.2.1.19	EMS-Digits

This parameter only applies to location for an emergency services call in North America and gives the North American Emergency Services Routing Digits as defined in TS 29.002 [214].

##### 5.1.2.1.20	EMS-Key

This parameter only applies to location for an emergency services call in North America and gives the North American Emergency Services Routing Key as defined in TS 29.002 [214].

##### 5.1.2.1.21	Entity number

This field contains the Recommendation E.164 [308] number assigned to the entity (MSC, VLR, HLR etc.) that produced the record. For further details concerning the structure of MSC and location register numbers see TS 23.003 [200].

##### 5.1.2.1.22	Equipment id

This field contains a local identifier used to distinguish between equipment of the same equipment type e.g. the number of the conference circuit employed if more than one is available.

##### 5.1.2.1.23	Equipment type

This field contains the type of common equipment employed e.g. conference circuit for multi-party service.

##### 5.1.2.1.24	Event time stamps

These fields contain the event time stamps relevant for each of the individual record types.

The call records may contain three significant call handling time stamps:

-	the time at which the resource in question was seized	(Seizure time);

-	the time at which the call was answered or at which charging commences	(Answer time);

-	the time at which the resource was released	(Release time).

For both Mobile Originated and Mobile Terminated calls, the Seizure time is the time at which the traffic channel is allocated i.e. the time at which the ASSIGN COMMAND message is sent to the MS.

For Mobile Originated calls the Answer time is the time at which the CONNECT message is sent to the calling party. For Mobile Terminated calls the time at which the CONNECT message is received from the called party. However, if the subscriber has subscribed to the advice of charge charging level service, then the answer time shall be derived from the time at which the FACILITY message is received from the MS containing the acknowledgement of receipt of the AOC parameters. Similarly, if the AOC parameters are changed during the call then the change time recorded for a subscriber with AOC charging level is the receipt of the FACILITY message from the MS. For a subscriber with AOC information level the change time recorded is the time at which the FACILITY is sent to the MS. Finally, in case of call re-establishment the answer time is the time at which the new traffic channel is allocated by the MSC i.e. when the ASSIGN COMMAND is sent to the MS.

The Release time is the time at which the connection is released by either party i.e. a DISCONNECT or RELEASE is sent by the network or a DISCONNECT is received from the MS. In the case of a radio link failure, the release time is the time at which the failure was detected by the MSC.

For unsuccessful call attempts the Seizure time is mandatory. The Release time is optional and the call duration recorded is the call holding time i.e. the difference between the two.

For successful calls the Answer time is mandatory and both the Seizure and Release times are optional. The call duration recorded is the chargeable duration i.e. the difference between the Answer and Release time stamps.

The event records include the following time stamps:

-	HLR-int time:	The receipt of a MAP_SEND_ROUTING_INFO request by the HLR;

-	Loc.Upd. time:	The receipt of a MAP_UPDATE_LOCATION_AREA request by the VLR or the receipt of 	a MAP_UPDATE_LOCATION request by the HLR;

-	SS-Action:	The receipt of a supplementary service request by the VLR;

e.g. MAP_REGISTER_SS, MAP_INVOKE_SS

-	SMS-MO:		The receipt of an RP_DATA message from the MS containing an SMS_SUBMIT PDU;

-	SMS-MT:		The transmission of an RP_DATA message to the MS containing an SMS_DELIVER 	PDU;

-	LCS:	The time the LR was processed.

It should be noted that the events listed above are only examples in order to demonstrate the principles and that the list is by no means exhaustive.

All time-stamps include a minimum of date, hour, minute and second.

##### 5.1.2.1.25	Fixed Network User Rate

This field indicates the user data rate applied for the connection in the fixed network. In UMTS, it shall be present for all bearer services as specified in TS 22.002 [102]. In GSM, this parameter is part of the HSCSD connection parameters, see clause 5.1.2.1. 30.

##### 5.1.2.1.26	Free format data

This field contains charging information sent by the gsmSCF in the Furnish Charging Information (FCI) messages as defined in TS 29.078 [217]. The data can be sent either in one FCI message or several FCI messages with append indicator. This data is transferred transparently in the CAMEL clauses of the relevant call records. 'Free format data' sent to the legID=1 is always stored in the top level of the respective record. 'Free format data' sent to the legID >1 is stored in the appropriate CAMEL call leg information field.

If the FCI is received more then once during one continuing incoming/outgoing CAMEL call leg, the append indicator defines whether the FCI information is appended to previous FCI and stored in the relevant record or the information of the last FCI received is stored in the relevant record (the previous FCI information shall be overwritten).

In the event of partial output the currently valid 'Free format data' is stored in the partial record.

##### 5.1.2.1.27	Free format data append indicator

This field contains an indicator whether free format data is to be appended to free format data stored in previous partial CDR. This field is needed in CDR post-processing to sort out valid free format data for that call leg from sequence of partial records. Creation of partial records is independent on received FCIs and thus valid free format data may be divided to different partial records.

If field is missing then free format data in this CDR replaces all received free format data in previous CDRs. Append indicator is not needed in the first partial record. In following partial records indicator shall get value true if all FCIs received during that partial record have append indicator. If one or more of the received FCIs for that call leg during the partial record do not have append indicator then this field shall be missing.

##### 5.1.2.1.28	GsmSCF address

This field identifies the CAMEL server serving the subscriber. Address is defined in HLR as part of CAMEL subscription information.

##### 5.1.2.1.29	Guaranteed Bit Rate

This field contains the Guaranteed Bit Rate based on the FNUR for transparent and Wanted AIUR for non-transparent CS data services based on the described mapping in TS 27.001 [213]. The Guaranteed Bit Rate may be used to facilitate admission control based on available resources, and for resource allocation within UMTS. The bit-rate of the UMTS bearer service shall guarantee to the user or applications refer TS 22.002 [102].

Operator may choose any of the possible values less or equal to wanted AIUR (Air Interface User Rate). 
(If WAIUR is less or equal to 14,4 kbit/s then Guaranteed Bit Rate and Maximum Bit Rate shall be set to 14,4 kbit/s).

##### 5.1.2.1.30	HSCSD parameters/Change of HSCSD parameters

The basic HSCSD parameters are negotiated between the MS and the network at call set-up time. They comprise of the following parameters:

-	the FNUR (Fixed Network User Rate) (optionally);

-	the total AIUR (Air Interface User Rate) requested by the MS (for non-transparent HSCSD connections only);

-	a list of the channel codings accepted by the MS;

-	the maximum number of traffic channels accepted by the MS (this is noted in the channels requested field);

-	the channel coding and the number of traffic channels actually used for the call.

In case the network or user initiated modification procedure takes place during the call, the AIUR requested, the channel coding used and the number of traffic channel requested/used might be recorded in the Change of HSCSD parameters field including the time at which the change occurred and which entity requested the change.

It should be noted that the Change of HSCSD Parameters field is optional and not required if partial records are generated when a Change of HSCSD Parameters takes place.

##### 5.1.2.1.31	Incoming/outgoing trunk group

The incoming trunk group describes the trunk on which the call originates as seen from the MSC. For mobile originated calls this will generally be a BSS trunk. Similarly, the outgoing trunk group describes the trunk on which the call leaves the MSC.

For 3G, this parameter may not be available. When available, this parameter shall be supplied in the CDRs.

##### 5.1.2.1.32	Interrogation result

This field contains the result of the HLR interrogation attempt as defined in the MAP (TS 29.002 [214]).

NOTE:	This field is only provided if the attempted interrogation was unsuccessful.

##### 5.1.2.1.33	IMEI Check Event

This field identifies the type of event that caused the IMEI check to take place:

-	Mobile originating call attempt;

-	Mobile terminating call attempt;

-	Mobile originating SMS;

-	Mobile terminating SMS;

-	Supplementary service actions performed by the subscriber;

-	Location update.

##### 5.1.2.1.34	IMEI Status

This field contains the result of the IMEI checking procedure:

-	Greylisted;

-	Blacklisted;

-	Non-whitelisted.

##### 5.1.2.1.35	JIP Parameter

This Jurisdiction Information Parameter (JIP) is populated if received via one of the methods listed as JIP Source. The field shall identify the actual originating exchange and may be equal to 6 or 10 digits for North America Region (NAR). Note that this field may not apply for international areas, as it is not currently used. Additionally, it is also possible to use the LRN as the JIP if it properly identifies the originating switch.

##### 5.1.2.1.36	JIP Query Status Indicator

This field indicates the status of Location Routing Number (LRN) query as follows:

1.		Number Portability Data Base (NPDB) returns LRN or NULL response (free of any error).

2.		No response was received to the query; the query timed out.

4.		Protocol error in received response message.

5.		Error detected in response data.

6.		Query rejected

9.		No query performed

99.		Query unsuccessful, reason unknown

If the JIP is equal to the LRN, then the JIP query status shall be the same as the LRN query status. If not, this field shall be set to one of the values listed above.

##### 5.1.2.1.37	JIP Source Indicator

This indicator shall be populated if the Jurisdiction Information Parameter is derived. Identifies the method in which the value was derived. Shall be set to the values listed in the LRN Source Indicator.

##### 5.1.2.1.38	LCS Cause

The LCS Cause parameter provides the reason for an unsuccessful location request according TS 49.031 [227].

##### 5.1.2.1.39	LCS Client Identity

This field contains further information on the LCS Client identity:

-	Client External ID;

-	Client Dialled by MS ID;

-	Client Internal ID.

##### 5.1.2.1.40	LCS Client Type

This field contains the type of the LCS Client as defined in TS 29.002 [214].

##### 5.1.2.1.41	LCS Priority

This parameter gives the priority of the location request as defined in TS 49.031 [227]

##### 5.1.2.1.42	LCS QoS

This information element defines the Quality of Service for a location request as defined in TS 49.031 [227].

##### 5.1.2.1.43	Level of CAMEL service

This field describes briefly the complexity of CAMEL invocation:

-	'Basic' means that CAMEL feature is invoked during the set-up phase (e.g. to modify the destination) of the call only;

-	'Online charging' means that CAMEL supported AoC parameter were sent to the mobile station (the Send Charging Information message, SCI, is received from the gsmSCF);

-	The flag 'call duration supervision' is set whenever the call duration supervision is applied in the gsmSSF of the VPLMN (apply charging message is received from the gsmSCF).

##### 5.1.2.1.44	Location/change of location

The location field contains a combination of the Location Area Code (LAC), Cell Identity (CI) and MCC+MNC of the cell in which the served party is currently located. Any change of location may be recorded in the change of location field including the time at which the change took place.

The change of location field is optional and not required if partial records are generated when the location changes.

The LAC and CI are both 2 octet quantities and coded according to TS 24.008 [208].

For SMS over SGs (defined in TS 36.413 [226]), the LAC field contains the Tracking Area Code and the Cell Identity contains the 16 least significant bits.

##### 5.1.2.1.45	Location Estimate

The Location Estimate field is providing an estimate of a geographic location of a target MS according to TS 29.002 [214].

##### 5.1.2.1.46	Location Extension

The Location Externsion field contains the 12 most significant bits from the Cell Identity field. This is used when SMS over SGs (defined in TS 36.413 [226]) is applied and the access is E-UTRAN.

##### 5.1.2.1.47	Location Routing Number (LRN)

This field contains Ten-digit Location Routing Number (LRN) for the Number Portability feature. It is populated if received via one of the methods listed as "LRN Source". It identifies the new location of a ported subscriber. For North America Region (NAR) this may be a 10-digit E.164 number. For Europe, other formats may apply.

If more than 10 digits are received, the first ten digits received are recorded. If fewer than 10 digits are received, the information is left justified in the field and padded with 0xF.

##### 5.1.2.1.48	Location Type

This field contains the type of the location as defined in TS 29.002 [214].

##### 5.1.2.1.49	LRN Query Status Indicator

This field indicates the status of Location Routing Number (LRN) query as follows:

1.		Number Portability Data Base (NPDB) returns LRN or NULL response (free of any error);

2.		No response was received to the query; the query timed out;

4.		Protocol error in received response message;

5.		Error detected in response data;

5.		Query rejected;

9.		No query performed;

99.		Query unsuccessful, reason unknown.

It is populated if an NP query was performed.

##### 5.1.2.1.50	LRN Source Indicator

This field indicates whether the Location Routing Number is obtained from LRN NP database or it came in incoming signalling or switching system data.

It is populated if routing information for a ported subscriber is received from one of the methods listed below. 
It shall be equal to one of the following enumerated values:

1.	LRN NP Database;

2.	SwitchingSystemData;

3.	Incomingsignaling;

9.	Unknown.

##### 5.1.2.1.51	Maximum Bit Rate

This field contains the Maximum Bit Rate based on the FNUR (Fixed Network User Rate) for transparent and WAIUR(Wanted Air Interface User Rate) for non-transparent CS data services based on the described mapping in TS 27.001 [213]. The parameter can be used to make code reservations in the downlink of the radio interface for the UMTS bearer service (BS20 and BS30) refer TS 22.002 [102]. Its purpose is:

- to limit the delivered bit-rate to applications or external networks with such limitations,

- to allow maximum wanted user bit-rate to be defined for applications able to operate with different rates (e.g. applications with adapting codecs).]

Maximum bit rate is set to the highest value  WAIUR (If WAIUR is less or equal to 14.4 kbit/s then Guaranteed Bit Rate and Maximum Bit Rate shall be set to 14.4 kbit/s).

##### 5.1.2.1.52	Measure Duration

This field contains the duration for the section of the location measurement corresponding to the location request and the location report messages.

##### 5.1.2.1.53	Message reference

This field contains a unique message reference number allocated by the mobile station when transmitting a short message to the service centre. This field corresponds to the TP-Message-Reference element of the SMS_SUBMIT PDU defined in TS 23.040 [201].

##### 5.1.2.1.54	MLC Number

This parameter refers to the ISDN (E.164[308]) number of an MLC.

##### 5.1.2.1.55	Mobile station classmark/change of classmark

This MS classmark field contains the mobile station classmark employed by the served MS on call set-up as defined in TS 24.008 [208] (see mobile station classmark 2, mobile station classmark 3). Any alteration in the classmark during the connection may be recorded in the change of classmark field and will include the time at which the change took place.

It should be noted that the change of classmark field is optional and not required if partial records are created when the classmark is altered.

##### 5.1.2.1.56	MOLR Type

The MOLR-Type identifier refers to the type of MO-LR that was invoked as defined in TS 24.080 [209].

##### 5.1.2.1.57	MSC Address

This field contains the Recommendation E.164 [308] number assigned to the MSC that produced the record. For further details concerning the structure of MSC numbers see TS 23.003 [200].

##### 5.1.2.1.58	MSC Server Indication

This field contains an indicator whether the CAMEL subscription information is active. The parameter is present for the VT-CSI in the VMSC and not present for the T-CSI in the GMSC.

This indication should be used for differentiation between the validity of the record content for T-CSI in the GMSC and VT-CSI in the VMSC.

##### 5.1.2.1.59	Network Call Reference

Whenever CAMEL is applied, this field is used for correlation of call records outputted from the originating MSC (when applicable), the GMSC and the terminating MSC, and a network optional call record from the gsmSCF.

##### 5.1.2.1.60	Notification to MS user

This field contains the privacy notification to MS user that was applicable when the LR was invoked as defined in TS 29.002 [214].

##### 5.1.2.1.61	Number of DP encountered

This field indicates how often CAMEL armed detection points (TDP and EDP) were encountered and is a measure of signalling between serving network and CAMEL service and complements 'Level of CAMEL service' field. Detection points from all applied CAMEL services for a single call leg and processed in the same gsmSSF shall be counted together.

##### 5.1.2.1.62	Number of forwarding

This field, if provided via ISUP signalling, contains the number of times a call has been forwarded prior to the interrogation of the HLR and is defined in TS 29.002 [214].

##### 5.1.2.1.63	Old /new location

These fields contain the location of a mobile subscriber before and after a location update. In case of VLR location update the location information consists of a VMSC number and location area code and MCC+MNC, with identity of the cell or the SAC for new location. In case of HLR location update the field contains the VMSC number and the VLR number.

##### 5.1.2.1.64	Partial Record Type

This field indicates the event that caused the generation of a partial record.

##### 5.1.2.1.65	Positioning Data

This information element is providing positioning data associated with a successful or unsuccessful location attempt for a target MS according TS 49.031 [227].

##### 5.1.2.1.66	Positioning Data

This information element is providing positioning data associated with a successful or unsuccessful location attempt for a target MS according TS 49.031 [227].

##### 5.1.2.1.67	Privacy Override

This parameter indicates if MS privacy is overridden by the LCS client when the GMLC and VMSC/SGSN for an MT-LR are in the same country as defined in TS 29.002 [214].

##### 5.1.2.1.68	Radio channel requested/radio channel used/change of radio channel

The radio channel requested field contains the type of channel requested by the user. The following values are permitted:

-	full rate;

-	half rate;

-	dual mode half rate preferred;

-	dual mode full rate preferred.

The radio channel used field indicates the type of traffic channel actually employed for the connection i.e. either full rate (Bm) or half rate (Lm) as described in GSM 05.01  via CR change to 45.001 [312]. Any change in the type of channel used may be recorded in the change of radio channel used field including the time at which the change occurred and the speech version used after the change of radio channel.

##### 5.1.2.1.69	Rate Indication

This parameter specifies the rate adaptation that was used for the connection. The field is constructed from the information in the parameters "rate adaptation" and "other rate adaptation" signalled between the MS/UE and the network, see TS 24.008 [208].

The format of this field is a single octet with the following format:

-	Bits 0-1: the Rate Adaptation field as defined in TS 24.008 [208];

- Bits 2-3: the Other Rate Adaptation field as defined in TS 24.008 [208];

-	Bits 4-7: not used.

##### 5.1.2.1.70	Reason for Service Change

This field contains the type of  service change  requested by the subscriber or performed by the network. Possible values include:

-	subscriber initiated;

-	network initiated;

-	call setup fallback;

-	call setup change order.

For further details see TS 23.172 [207].

##### 5.1.2.1.71	Record extensions

The field enables network operators and/ or manufacturers to add their own extensions to the standard record definitions.

##### 5.1.2.1.72	Record type

The field identifies the type of the record e.g. mobile originated, mobile terminated etc.

##### 5.1.2.1.73	Recording Entity

This field contains the E.164 [308] number assigned to the entity (MSC, VLR, HLR etc.) that produced the record. For further details concerning the structure of MSC and location register numbers see TS 23.003 [200].

##### 5.1.2.1.74	Redial attempt

This field indicates that a call is the result of a redial attempt to switch from speech to multimedia or vice-versa.

##### 5.1.2.1.74A	Related ICID

This field contains the related IMS Charging ID for the IMS call leg in case of SRVCC as received from IMS domain.

##### 5.1.2.1.75	Roaming number

The roaming number field contains the mobile station roaming number as defined in TS 23.003 [200] and coded according to TS 29.002 [214].

##### 5.1.2.1.76	Routing number

The routing number field of the HLR interrogation record contains either a mobile station roaming number or, in case of call forwarding, a forwarded-to number.

##### 5.1.2.1.77	Sequence number

This field contains a running sequence number employed to link the partial records generated for a particular connection.

##### 5.1.2.1.78	Served IMEI

This fields contains the international mobile equipment identity (IMEI) or IMEISV of the equipment served. 
The term "served" equipment is used to describe the ME involved in the transaction recorded e.g. the called ME in case of an MTC record.

The structure of the IMEI, IMEISV is specified in TS 23.003 [200] and the encoding defined in TS 29.274 [223].

##### 5.1.2.1.79	Served IMSI

This fields contains the international mobile subscriber identity (IMSI) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded e.g. the calling subscriber in case of an MOC record.

The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.2.1.80	Served MSISDN

This fields contains the mobile station ISDN number (MSISDN) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded e.g. the called subscriber in case of an MTC record. In case of multi-numbering the MSISDN stored in a MOC record will be the primary MSISDN of the calling party.

The structure of the MSISDN is defined in TS 23.003 [200].

##### 5.1.2.1.81	Service centre address

This field contains a Recommendation E.164 [308] number identifying a particular service centre e.g. short message service centre (see TS 23.040 [201]).

##### 5.1.2.1.82	Service Change Initiator

This field indicates that the owner of this CDR is the initiator of the service change.

##### 5.1.2.1.83	Service key

This field identifies the CAMEL service logic applied. Service key is defined in HLR as part of CAMEL subscription information.

##### 5.1.2.1.84	Short message service result

This field contains the result of an attempt to deliver a short message either to a service centre or to a mobile subscriber (see TS 29.002 [214]). Note that this field is only provided if the attempted delivery was unsuccessful.

##### 5.1.2.1.85	Speech version supported/Speech version used

The speech version supported field contains the speech version supported by the MS with the highest priority. 
The speech version used field contains the speech codec version assigned for that call. The coding is according GSM 08.08 [313] speech version identifier with the extension bit 8 set to 0.

It should be noted that the change of radio channel field is optional and not required if partial records are generated.

##### 5.1.2.1.86	Supplementary service(s)

The supplementary service field in the Supplementary Service record type contains the code of the supplementary service on which the action was performed.

The supplementary services field in the MOC/MTC records contains the codes of the supplementary services invoked as a result of, or during, a connection.

The coding of supplementary service is described in detail in TS 29.002 [214].

##### 5.1.2.1.87	Supplementary service action

This field contains the type of supplementary service action requested by the subscriber or performed by the network. Possible values include:

-	registration;

-	erasure;

-	activation;

-	deactivation;

-	interrogation;

-	invocation.

For further details see TS 22.004 [103].

##### 5.1.2.1.88	Supplementary service action result

This field contains the result of an attempted supplementary service action (see TS 29.002 [214]). Note that this field is only provided if the SS-action was at least partially unsuccessful.

##### 5.1.2.1.89	Supplementary service parameters

This field contains the parameters associated with a supplementary service action requested by the subscriber. For further details of the parameters involved see the GSM 02.8n series of documents.

##### 5.1.2.1.90	Supplementary service(s)

The supplementary service field in the Supplementary Service record type contains the code of the supplementary service on which the action was performed.

The supplementary services field in the MOC/MTC records contains the codes of the supplementary services invoked as a result of, or during, a connection.

The coding of supplementary service is described in detail in TS 29.002 [214].

##### 5.1.2.1.91	System type

This field indicates the use of GERAN, UTRAN (or a value of unknown). This field is present when either the UTRAN or GERAN air-interface is used on call set-up. For an open CDR in a 2G NE (responsible for the CDR), the field is not present (even if the call is handed off to a 3G air interface). For a CDR in a 3G NE (responsible for the CDR), the value unknown shall be used after handover.

##### 5.1.2.1.92	Transparency indicator

This field indicates whether the basic service was employed in transparent or non-transparent mode. It should also be noted that this field is only relevant for those services which may be operated in both transparent and non-transparent modes.

##### 5.1.2.1.93	Update result

This field contains the result of the location update request as defined in the MAP (TS 29.002 [214]). Note that this field is only provided if the attempted update was unsuccessful.

#### 5.1.2.2	PS domain CDR parameters

##### 5.1.2.2.A	Introduction

This subclause contains the description of the CDR parameters that are specific to the PS domain CDR types as specified in TS 32.251 [11].

##### 5.1.2.2.B	Void

##### 5.1.2.2.0	3GPP2 User Location Information

This field holds the 3GPP2 User Location Information. It contains the 3GPP2-BSID as described in TS 29.212 [220].

The parameter is provided to the PGW/TDF during IP-CAN/TDF session establishment/modification, through PCC procedures for non-3GPP Accesses, as defined in the TS 23.203 [203].

##### 5.1.2.2.0aA	Access Availability Change Reason

This field indicates the reason why the availability of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable as defined in TS 29.212 [220].

##### 5.1.2.2.0A	Access Line Identifier

This field contains the Access line id (physical and logical circuit ID) serving the 3GPP UE or fixed user (i.e. Fixed device or RG).

This field contains the Access line id (physical and logical circuit ID):

-	serving the 3GPP UE or fixed device, behind the RG when in bridge mode or in routed mode without NAT, or

-	of the RG when in routed mode with NAT.

##### 5.1.2.2.1	Access Point Name (APN) Network/Operator Identifier

These fields contain the actual connected Access Point Name Network/Operator Identifier determined either by MS, SGSN/MME or modified by CAMEL service. An APN can also be a wildcard, in which case the SGSN/MME selects the access point address.

Following TS 23.003 [200], the APN field is specified in the CDR by two variable strings. The first is the APN Network Identifier (NI portion) and the second is the APN Operator Identifier (OI portion). The APN NI may contain one or more label as described in TS 23.003 [200]. The APN OI is composed of three labels. The first and second labels together shall uniquely identify the PLMN operator (e.g. "mnc<operator mnc>.mcc<operator mcc>.gprs").

To represent the APN NI and OI in the PCN CDRs, the "dot" notation shall be used.

See TS 23.003 [200] and TS 23.060 [202] for more information about APN format and access point decision rules.

##### 5.1.2.2.1A	APN Rate Control

This field contains the APN Rate Control as specified in TS 23.401 [245], which is used during the record for the PDN connection to the PGW.

##### 5.1.2.2.2	APN Selection Mode

This field indicates how the SGSN/MME selected the APN to be used. The values and their meaning are as specified in  TS 29.060 [215] for GTP case and in  TS 29.274 [223] for eGTP case.

##### 5.1.2.2.3	CAMEL Charging Information

This field contains the CAMEL Information as defined for the PDP context from the SGSN as the copy including Tag and Length from the SGSN's CDR (S-CDR).

##### 5.1.2.2.4	CAMEL Information

This field includes following CAMEL information elements for PDP context (S-CDR), Attach/Detach session 
(M-CDR), Mobile originated SMS (S-SMO-CDR) and Mobile terminated SMS (S-SMT-CDR)  if corresponding CAMEL service is activated.

- CAMEL Access Point Name NI (S-CDR):

This field contains the network identifier part of APN before modification by the CSE.

- CAMEL Access Point Name OI (S-CDR):

This field contains the operator identifier part of APN before modification by the CSE.

- CAMEL Calling Party Number (S-SMO-CDR, S-SMT-CDR):

This field contains the Calling Party Number modified by the CAMEL service.

- CAMEL Destination Subscriber Number (S-SMO-CDR):

This field contains the short message Destination Number modified by the CAMEL service.

- CAMEL SMSC Address (S-SMO-CDR):

This field contains the SMSC address modified by the CAMEL service.

- SCF address (S-CDR, M-CDR, S-SMO-CDR, S-SMT-CDR):

This field identifies the CAMEL server serving the subscriber. Address is defined in HLR as part of CAMEL subscription information.

- Service key (S-CDR, M-CDR, S-SMO-CDR, S-SMT-CDR):

This field identifies the CAMEL service logic applied. Service key is defined in HLR as part of CAMEL subscription information.

- Default Transaction/SMS Handling (S-CDR, M-CDR, S-SMO-CDR, S-SMT-CDR):

This field indicates whether or not a CAMEL encountered default GPRS- or SMS-handling. This field shall be present only if default call handling has been applied. Parameter is defined in HLR as part of CAMEL subscription information.

- Free Format Data (S-CDR, M-CDR, S-SMO-CDR, S-SMT-CDR):

This field contains charging information sent by the gsmSCF in the Furnish Charging Information GPRS messages as defined in TS 29.078 [217]. The data can be sent either in one FCI message or several FCI messages with append indicator. This data is transferred transparently in the CAMEL clauses of the relevant call records.

If the FCI is received more then once during one CAMEL call, the append indicator defines whether the FCI information is appended to previous FCI and stored in the relevant record or the information of the last FCI received is stored in the relevant record (the previous FCI information shall be overwritten).

In the event of partial output the currently valid "Free format data" is stored in the partial record.

- FFD Append Indicator (S-CDR, M-CDR):

This field contains an indicator whether CAMEL free format data is to be appended to free format data stored in previous partial CDR. This field is needed in CDR post processing to sort out valid free format data for that call leg from sequence of partial records. Creation of partial records is independent of received FCIs and thus valid free format data may be divided to different partial records.

If field is missing then free format data in this CDR replaces all received free format data in previous CDRs. Append indicator is not needed in the first partial record. In following partial records indicator shall get value true if all FCIs received during that partial record have append indicator. If one or more of the received FCIs for that call leg during the partial record do not have append indicator then this field shall be missing.

- Level of CAMEL services (S-CDR, M-CDR):

This field describes briefly the complexity of CAMEL invocation. Categories are the same as in circuit switched services and measure of resource usage in VPLMN requested by HPLMN.

-	"Basic" means that CAMEL feature is invoked during the PDP context activation phase only (e.g. to modify APN_NI/APN_OI).

-	"Call duration supervision" means that PDP context duration or volume supervision is applied in the gprsSSF of the VPLMN (Apply Charging message is received from the gsmSCF).

-	Number of DPs encountered (S-CDR, M-CDR):

This field indicates how many armed CAMEL detection points (TDP and EDP) were encountered and complements "Level of CAMEL service" field.

- smsReferenceNumber (S-SMO-CDR, S-SMT-CDR)

This parameter contains the SMS Reference Number assigned to the Short Message by the SGSN.

##### 5.1.2.2.5	Cause for Record Closing

This field contains a reason for the release of the CDR. In case of Rf interface is used, it is derived from Change-Condition AVP at PS-information AVP level defined in TS 32.299 [40], when received. The following is included:

-	normal release: IP-CAN bearer release or detach, TDF session release, IP-CAN session release for IP-Edge; It corresponds to "Normal Release" in Change-Condition AVP.

-	data volume limit; It corresponds to "Volume Limit" in Change-Condition AVP.

-	time (duration) limit; It corresponds to "Time Limit" in Change-Condition AVP.

-	maximum number of changes in charging conditions; It corresponds to "Max Number of Changes in Charging conditions " in Change-Condition AVP.

-	For SGSN: intra SGSN intersystem change (change of radio interface from GSM to UMTS or vice versa);

-	For P-GW, TDF and S-GW: Radio Access Technology (RAT) change; It corresponds to "RAT Change" in Change-Condition AVP.

-	abnormal termination (IP-CAN bearer or MM context or TDF session, or IP-CAN session); It corresponds to "Abnormal Release" in Change-Condition AVP.

-	For SGSN: SGSN change;

-	For S-GW, ePDG, TWAG: S-GW change; It corresponds to "S-GW Change" in Change-Condition AVP and is used for inter serving node change of all types.

-	Timezone change; It corresponds to "UE TimeZone Change" in Change-Condition AVP.

-	SGSN or S-GW PLMN change; It corresponds to "Serving Node PLMN Change" in Change-Condition  AVP.

-	For P-GW: APN-AMBR change: It corresponds to "Qos Change" in Change-Condition AVP.

-	For S-GW, P-GW: MO exception data counter receipt: It corresponds to " MO exception data counter receipt" in Change-Condition AVP.

-	unauthorized network originating a location service request;

-	unauthorized client requesting a location service;

-	position method failure at a location service execution;

-	unknown or unreachable LCS client at a location service request;

-	management intervention (request due to O&M reasons); It corresponds to "Management Intervention" in Change-Condition AVP.

A more detailed reason may be found in the diagnostics field or Enhanced Diagnostics field.

##### 5.1.2.2.6	Cell Identifier

For GSM, the Cell Identifier is defined as the Cell Id, reference 24.008 [208], and for UMTS it is defined as the Service Area Code in TS 25.413 [212].

##### 5.1.2.2.7	Charging Characteristics

The Charging Characteristics field allows the operator to apply different kind of charging methods in the CDRs. 
A subscriber may have Charging Characteristics assigned to his subscription. These characteristics can be supplied by the HLR/HSS according to the rules specified in Annex A of TS 32.251 [11].

Charging Characteristics used by IP-Edge [PCEF] or TDF in Convergent Fixed-Mobile Operator scenario are always configured in IP-Edge [PCEF] or TDF respectively, as described in annex D of TS 32.251 [11].

This information can be used by the PCNs/ IP-Edge [PCEF] to activate charging generation and e.g. control the closure of the CDR or the traffic volume containers (see clause 5.1.2.2.25) and applied charging characteristics is included in CDRs transmitted to nodes handling the CDRs via the Ga reference point. It can also be used in nodes handling the CDRs (e.g., the CGF or the billing system) to influence the CDR processing priority and routing. These functions are accomplished by specifying the charging characteristics as sets of charging profiles and the expected behaviour associated with each profile. 
The interpretations of the profiles and their associated behaviours can be different for each PLMN operator and are not subject to standardisation. In the present document only the charging characteristic formats and selection modes are specified.

The functional requirements for the Charging Characteristics are further defined in normative Annex A of 
TS 32.251 [11], including an example for the definitions of the trigger profiles associated with each CDR type.

The format of charging characteristics field is depicted in Figure 4. Each Bx (x =0..15) refers to a specific behaviour defined on a per-Operator basis, indicated as active when set to "1" value. See Annex A of 
TS 32.251 [11] for guidance on how behaviours could be defined.

Figure 5.1.2.2.7.1: Charging Characteristics flags

##### 5.1.2.2.8	Charging Characteristics selection mode

This field indicates the charging characteristic type that the PCNs applied to the CDR. In the SGSN the allowed values are:

-	Home default;

-	Visiting default;

-	Roaming default;

-	APN specific;

-	Subscription specific.

In the S-GW/P-GW/TDF the allowed values are:

-	Home default;

-	Visiting default;

-	Roaming default;

-	Serving node supplied.

NOTE: 	The value 'Serving Node Supplied' is used if the CC what was received from e.g. S-GW is used i.e. the one what comes during bearer activation.

Further details are provided in TS 32.251 [11] Annex A.

In the IP-Edge [PCEF] and TDF in Convergent Fixed-Mobile Operator scenario, the allowed values are:

-	Home default;

-	Visiting default;

-	Fixed default;

Further details are provided in TS 32.251 [11] Annex D.

##### 5.1.2.2.9	Charging ID

This field is a charging identifier, which can be used together with P-GW address to identify all records produced in SGSN(s), S-GW and P-GW involved in a single IP-CAN bearer. Charging ID is generated by P-GW at IP-CAN bearer activation and transferred to bearer requesting SGSN/S-GW. At inter-SGSN/S-GW change the charging ID is transferred to the new SGSN/S-GW as part of each active IP-CAN bearer.

In case of PMIP-based connectivity, the Charging Id is generated per PDN connection.

Different P-GWs allocate the charging ID independently of each other and may allocate the same numbers. 
The CGF and/or BS may check the uniqueness of each charging ID together with the P-GWs address and optionally (if still ambiguous) with the record opening time stamp.

##### 5.1.2.2.9A	CN Operator Selection Entity

This field defines which entity (UE or Network) has selected the Serving Core Network in Network Sharing situations.

##### 5.1.2.2.9Aa	CP CIoT EPS Optimisation Indicator

This field contains the indication on whether Control Plane CIoT EPS optimisation is used by the PDN connection during data transfer with the UE (i.e. Control Plane NAS PDU viaS11-U between SGW andMME) or not (i.e. User Plane via S1-U between SGW and eNB).

##### 5.1.2.2.9B	Charging per IP-CAN Session Indicator

This field indicates whether charging per IP-CAN session is active or not.

##### 5.1.2.2.10	Destination Number

This field contains short message Destination Number requested by the user. See TS 32.250 [10].

##### 5.1.2.2.11	Diagnostics

This field includes a more detailed technical reason for the releases of the connection. 
The diagnostics may also be extended to include manufacturer and network specific information.

##### 5.1.2.2.12	Duration

This field contains the relevant duration in seconds for IP-CAN bearer (S-CDR, SGW-CDR, PGW-CDR), 
IP-CAN session (IPE-CDR),  and attachment (M-CDR). In case of TDF-CDR, this field contains the relevant duration in seconds for TDF session.

It is the duration from Record Opening Time to record closure. For partial records this is the duration of the individual partial record and not the cumulative duration.

It should be noted that the internal time measurements may be expressed in terms of tenths of seconds or even milliseconds and, as a result, the calculation of the duration may result in the rounding or truncation of the measured duration to a whole number of seconds.

Whether or not rounding or truncation is to be used is considered to be outside the scope of the present document subject to the following restrictions:

1)	A duration of zero seconds shall be accepted providing that the transferred data volume is greater than zero.

2)	The same method of truncation/rounding shall be applied to both single and partial records.

##### 5.1.2.2.13	Dynamic Address Flag

This field indicates that PDN address has been dynamically allocated for that particular IP-CAN bearer (PDN connection). This field is missing if address is static. Dynamic address allocation might be relevant for charging e.g. as one resource offered and possible owned by network operator.

##### 5.1.2.2.13A	Dynamic Address Flag Extension

This field indicates that the IPv4 address has been dynamically allocated for that particular IP-CAN bearer (PDN connection) of PDN type IPv4v6, and the dynamic IPv6 prefix is indicated in Dynamic Address Flag. This field is missing if IPv4 address is static. Dynamic address allocation might be relevant for charging e.g. as one resource offered and possible owned by network operator.

##### 5.1.2.2.13Aa	Enhanced Diagnostics

This field includes a more detailed technical reason with a set of causes for the release of the connection and may contain the following:

-	RAN/NAS cause from TS 29.274 [223];

NOTE: The RAN/NAS cause contains a set of different causes. The Enhanced Diagnostics is defined to allow extensions to other types of release causes in the future.

##### 5.1.2.2.13B	EPC QoS Information

This field contains the APN-AMBR for the IP-CAN session. It is used in the PGW-CDR only when charging per IP-CAN session is active.

##### 5.1.2.2.13C	ePDG Address Used

This field is the serving ePDG IP Address for the Control Plane. If both an IPv4 and an IPv6 address of the ePDG is available, the ePDG shall include the IPv4 address in the CDR.

##### 5.1.2.2.13D	ePDG IPv6 Address

This field is the serving ePDG IPv6 Address for the Control Plane, when both IPv4 and IPv6 addresses of the ePDG are available.

##### 5.1.2.2.14	Event Time Stamps

These fields contain the event time stamps relevant for each of the individual record types.

All time-stamps include a minimum of date, hour, minute and second.

##### 5.1.2.2.15	Void

##### 5.1.2.2.15A	Fixed User Location Information

This field contains the UE location in a fixed broadband access network. The location of the subscriber may include Access line id (physical and logical circuit ID) defined in ETSI TS 283 034 [314], SSID and BSSID of the Access Point, defined in IEEE Std 802.11-2012 [408].

##### 5.1.2.2.16	GGSN Address Used

This field is the current serving GGSN/P-GW IP Address for the Control Plane. If both an IPv4 and an IPv6 address of the GGSN/P-GW are available, the SGSN shall include the IPv4 address in the CDR.

##### 5.1.2.2.16A	Void

(Void)

##### 5.1.2.2.17	IMS Signalling Context

Indicates if the IP-CAN bearer is used for IMS signalling. It is only present if the IP-CAN bearer is an IMS signalling bearer. A IP-CAN bearer for IMS signalling is determined via the "IM CN Subsystem Signalling Flag" conveyed via the "Activate PDP context request" message from the MS to the network (refer to TS 24.008 ).

##### 5.1.2.2.18	IMSI Unauthenticated Flag

This field indicates that provided "Served IMSI" is not authenticated, and relates to an emergency bearer established with IMSI as identifier (refer to TS 23.060 [202] and TS 29.274 [223]). This field is missing if IMSI is authenticated, or if IMSI is not provided as identifier.

##### 5.1.2.2.18A	IP-CAN session Type

This field defines the IP-CAN session type, e.g. IP, or PPP. PDP type format is used: See TS 29.060 [215].

##### 5.1.2.2.18B	IP-Edge Address IPv6

This field is the IP-Edge IPv6 Address used for the Control Plane, when both IPv4 and IPv6 addresses of the IP-Edge are available.

##### 5.1.2.2.18C	IP-Edge Address Used

This field is the IP-Edge IP Address used for the Control Plane. If both an IPv4 and an IPv6 addresses of the IP-Edge are available, the field shall include the IPv4 address.

##### 5.1.2.2.18D	IP-Edge Operator Identifier

This field is the PMLN Identifier (Mobile Country Code and Mobile Network Code) of the Convergent Fixed-Mobile Operator owning the IP-Edge located in Fixed Broadband Access.

The MCC and MNC are coded as described for "User Location Info" in TS 29.274 [223].

##### 5.1.2.2.18E	Last MS Time Zone

This field contains the "Time Zone" provided by the SGSN/MME and transferred to the S-GW/P-GW during the IP-CAN bearer deactivation. It is derived from 3GPP-MS-TimeZone AVP provided within PS-Information AVP, both defined in TS 32.299 [50], when received on Rf closure.

##### 5.1.2.2.18F	Last User Location Information

This field contains the User Location Information as described in clause in 5.1.2.2.75.

The field is provided by the SGSN/MME and transferred to the S-GW/P-GW during the IP-CAN bearer deactivation. It is derived from 3GPP-User-Location-Info AVP provided at PS-Information AVP level, both defined in TS 32.299 [50], when received on Rf closure.

##### 5.1.2.2.19	LCS Cause

The LCS Cause parameter provides the reason for an unsuccessful location request according TS 49.031 [227].

##### 5.1.2.2.20	LCS Client Identity

This field contains further information on the LCS Client identity:

- Client External ID;

- Client Dialled by MS ID;

-	Client Internal ID.

##### 5.1.2.2.21	LCS Client Type

This field contains the type of the LCS Client as defined in TS 29.002 [214].

##### 5.1.2.2.22	LCS Priority

This parameter gives the priority of the location request as defined in TS 49.031 [227].

##### 5.1.2.2.23	LCS QoS

This information element defines the Quality of Service for a location request as defined in TS 49.031 [227].

##### 5.1.2.2.24	List of Service Data

This list includes one or more service data containers. Depending on the reporting level of PCC/ADC rules, one service data container either includes charging data for one rating group or for one rating group and service id combination. Each service data container may include the following fields:

-	AF-Record-Information.

-	Charging Rule Base Name .

-	ADC Rule Base Name.

-	Data Volume Downlink.

-	Data Volume Uplink.

-	Event Based Charging Information.

-	Local Sequence Number.

-	PS Furnish Charging Information.

-	EPC Qos Information.

-	Rating Group.

-	Report Time.

-	Result Code.

-	Service Condition Change.

-	Service Identifier.

-	Service Specific Info.

-	Serving Node Address.

-	Time of First Usage.

-	Time of Last Usage.

-	Time Quota Mechanism.

-	Time Usage.

-	user location information.

-	3GPP2 User Location Information.

-	UWAN User Location Information.

-	Sponsor Identity.

-	Application Service Provider Identity.

-	Presence Reporting Area Status.

-	User CSG Information

-	RAT Type

-	Serving PLMN Rate Control

-	APN Rate Control

-	Related Change of Service Condition

Rating Group is the identifier of rating group. This field is mandatory. The parameter corresponds to the Charging Key as specified in TS 23.203 [203].

Charging Rule Base Name is the reference to group of PCC rules predefined at the PCEF. This field is included if any of the PCC rules, which usage is reported within this service data container, was activated by using the Charging Rule Base Name as specified in TS 29.212 [220]. In case multiple Charging Rule Base Names activate PCC rules, which usage is reported within this service data container, the P-GW/IPE-CDR shall include only one occurrence to the service data container.

ADC Rule Base Name is the reference to group of ADC rules predefined at the TDF. This field is included if any of the ADC rules, which usage is reported within this service data container, was activated by using the ADC Rule Base Name as specified in TS 29.212 [220]. In case multiple ADC Rule Base Names activate ADC rules, which usage is reported within this service data container, the TDF shall include only one occurrence to the service data container.

Result Code contains the result code after the interconnection with the OCS. This field may be added to the service data container if online and offline charging are both used for same rating group. The result code in service data container is the value of the Result-Code AVP received within last CCA message in corresponding MSCC AVP to this service data container.

Local Sequence Number is a service data container sequence number. It starts from 1 and is increased by 1 for each service date container generated within the lifetime of this IP-CAN bearer/TDF session.

Time of First Usage is the time stamp for the first IP packet to be transmitted and mapped to the current service data container. For envelope reporting controlled by the Time Quota Mechanism, this indicates the time stamp for the first IP packet to be transmitted that causes an envelope to be opened – see TS 32.299 [50].

Time of Last Usage is the time stamp for the last IP packet to be transmitted and mapped to the current service data container. For envelope reporting, controlled by the Time Quota Mechanism, this indicates the time stamp for an envelope to be closed – see TS 32.299 [50] for conditions for envelope closure.

Time Usage contains the effective used time within the service data container recording interval.

Service Condition Change defines the reason for closing the service data container (see TS 32.251 [11]), such as tariff time change, IP-CAN bearer modification (e.g. QoS change, S-GW change, user location change, user CSG information change), access change of service data flow, indirect service condition change, service usage thresholds, service idled out, termination or failure handling procedure. When one of the "CGI/SAI, ECGI or TAI or RAI Change" are reported as user location change, the dedicated value in service Condition Change is set instead of the generic "user location change" value. This field is specified as bitmask for support of multiple change trigger (e.g. S-GW and QoS change). This field is derived from Change-Condition AVP at Service-Data-Container AVP level defined in TS 32.299 [50] when received on Rf. Each value is mapped to the corresponding value in "ServiceConditionChange" field. When simultaneous change triggers are met, multiple Change-Condition values are set in field bitmask.When no Change-Condition AVP is provided, the "recordClosure" value is set for the service data container. For envelope reporting, the Service Condition Change value shall always take the value "envelopeClosure". The mechanism for creating the envelope is identified within the Time Quota Mechanism field.

EPC Qos Information in service specific service data containers contains the QoS applied for the service.   This is included in the first service data container. In following container EPC QoS information is present if previous change condition is "QoS change". The P-GW/TDF/IPE-CDR shall include only one EPC QoS Information occurrence to one service data container.

Serving Node Address contains the valid serving node (e.g.SGSN/S-GW) control plane IP address during the service data container recording interval.

Data Volume Uplink and Downlink, includes the number of octets transmitted during the service data container recording interval in the uplink and/or downlink direction, respectively. In case of GTP based tunnelling, the amount of data counted in P-GW shall be based on the payload of the GTP-U protocol. In case of PMIP based protocol, the amount of data counted in P-GW shall be based on the payload of the GRE tunnel. In case of DSMIPv6 based protocol, the amount of data counted in P-GW shall be based on the payload of the tunnelling layer. As minimum behaviour, the full payload shall be included. The amount of data counted in TDF shall be based on full payload of the data transferred. The amount of data counted in IP-Edge shall be based on full payload of the data transferred.

Report Time is a time stamp, which defines the moment when the service data container is closed.

Service Identifier is an identifier for a service. The service identifier may designate an end user service, a part of an end user service or an arbitrarily formed group thereof. This field is only included if reporting is per combination of the rating group and service id.

PS Furnish Charging Information includes charging information per rating group in case it is sent by OCS.

User location information contains the user location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE was located during the service data container recording interval. This is included in the service data container only if previous container's change condition is "user location change" or one of the "CGI/SAI, ECGI or TAI or RAI Change". Note the user location information in PGW-CDR/TDF-CDR main level contains the location where the UE was when PGW-CDR/TDF-CDR was opened, and the "Last user location information" in PGW-CDR main level contains the location where the UE was when PGW-CDR is closed.

Presence Reporting Area Status contains the status of the UE presence (i.e. indication on whether the UE is inside or outside) in the Presence Reporting Area(s) identified by "Presence Reporting Area identifier(s)" contained in PGW-CDR main level. This is included in the service data container when the initial status is reported after opening of PGW-CDR, or if previous container’s change condition is "Presence in Presence Reporting Area Change".

3GPP2 User Location Information contains the 3GPP2 user location (i.e. 3GPP2-BSID: Cell-Id, SID, NID) where the UE was located during the service data container recording interval. This is included in the service data container only if previous container's change condition is "user location change". Note the "3GPP2 user location information" in PGW-CDR/TDF-CDR main level contains the location where the UE was when PGW-CDR/TDF-CDR was opened.

UWAN User Location Information contains the Untrusted Wireless Access Network (UWAN) user location which includes the UE local IP address and optionally UDP source port number (if NAT is detected). It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE.

User CSG information contains the status of the user accessing a CSG cell (CSG ID within the PLMN, Access mode and indication on CSG membership) during the service data container recording interval. This is included in the service data container only if previous container’s change condition is "user CSG information change". Note the "user CSG information" in PGW-CDR main level contains the "user CSG information" when PGW-CDR was opened.

AF-Record-Information includes the "AF Charging Identifier" (ICID for IMS) and associated flow identifiers generated by the AF and received by the P-GW over Gx interfaces as defined in TS 29.212 [220]. In case usage of PCC rules, which usage is reported within the container, has different AF-Record-Information then the P-GW shall include all occurrences to the service data container.

Event Based Charging Information includes the number of events and associated timeStamps (each event is timestamped) during the service data container recording interval.

Time Quota Mechanism contains two further subfields and is included if envelope reporting is required:

Time Quota Type identifies the mechanism by which time based usage should be reported – as defined in TS 32.299 [50].

Base Time Interval identifies the length of the base time interval, for controlling the reporting of time based usage, in seconds.

Service Specific Info holds service specific data for a pre-defined PCC or a predefined ADC rule that is used for enhanced packet filtering.

Sponsor Identity identifies the sponsor willing to pay for the operator's charge for connectivity.

Application Service Provider Identity identifies application service provider that is delivering a service to an end user.

RAT Type contains the RAT type for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included.

NOTE: 	Sponsor Identity and Application Service Provider Identity are not used together with Service Identification reporting. Furthermore, neither the Sponsor Identity nor Application Service Provider Identity is applicable to the TDF.

Related Change of Service Condition indicates the reason a related container was closed when the current container is indirectly closed and the supplemental information for the event. This information is applicable when charging per IP-CAN session is active for a multi-access PDN connection.

Serving PLMN Rate Control This field contains the Serving PLMN Rate Control applied by MME during the transfer of the data volume captured by the container (applicable only to the PGW-CDR). This is included in the service data container only if previous container's change condition is "Serving PLMN Rate Control change ". Note the Serving PLMN Rate Control field in PGW-CDR main level contains the Serving PLMN Rate Control when PGW-CDR was opened.

APN Rate Control This field contains the APN Rate Control applied by PGW during the transfer of the data volume captured by the container (applicable only to the PGW-CDR). This is included in the service data container only if previous container's change condition is "APN Rate Control change ". Note the APN Rate Control field in PGW-CDR main level contains the APN Rate Control when PGW-CDR was opened.

##### 5.1.2.2.25	List of Traffic Data Volumes

This list applicable in S-CDR, SGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR, includes one or more containers.

This list applicable in PGW-CDR when charging per IP-CAN session is active and IP-CAN bearer charging is being performed for the session.

In SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR, containers are per QCI/ARP pair. This means that if QoS control within one IP-CAN bearer is applicable in S-GW, P-GW, ePDG and TWAG, there can be several containers open at same time one per each applied QCI/ARP pair.

Each container includes the following fields:

Data Volume Uplink, Data Volume Downlink, Change Condition and Change Time.

Data Volume Uplink includes the number of octets transmitted during the use of the packet data services in the uplink direction. In MBMS charging, this field is normally to be set to zero, because MBMS charging is based on the volume of the downlink data. The counting of uplink data volumes is optional. In S-CDR this field is not present when the SGSN has successfully established Direct Tunnel between the RNC and the GGSN.

Data Volume Downlink includes the number of octets transmitted during the use of the packet data services in the downlink direction. In S-CDR this field is not present when the SGSN has successfully established Direct Tunnel between the RNC and the GGSN.

Change Condition defines the reason for closing the container (see TS 32.251 [11]), such as tariff time change, QoS change or closing of the CDR. This field is derived from Change-Condition AVP Traffic-Data-Volumes AVP level defined in TS 32.299 [40] when received on Rf. Each value is mapped to the corresponding value in "ChangeCondition" field. When no Change-Condition AVP is provided, the "recordClosure" value is set for the container. For User Location Change, when one of the "CGI/SAI, ECGI or TAI or RAI Change" are reported as user location change, the dedicated value in service Condition Change is set instead of the generic "user location change" value.

Change Time is a time stamp, which defines the moment when the volume container is closed or the CDR is closed. All the active IP-CAN bearers do not need to have exactly the same time stamp e.g. due to same tariff time change (variance of the time stamps is implementation and traffic load dependent, and is out of the scope of standardisation).

User Location Information contains the location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE is located and used during the transfer of the data volume captured by the container (applicable only to the SGW-CDR). This is included in the Traffic data container only if previous container's change condition is "user location change". Note the user location information in SGW-CDR main level contains the location where the UE was when SGW-CDR was opened, and the "Last user location information" in SGW-CDR main level contains the location where the UE was when SGW-CDR is closed.

UWAN User Location Information contains the Untrusted Wireless Access Network (UWAN) user location which includes the UE local IP address and optionally UDP source port number (if NAT is detected). It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE.

Presence Reporting Area Status contains the status of the UE presence (i.e. indication on whether the UE is inside or outside) in the Presence Reporting Area(s) identified by "Presence Reporting Area identifier(s)" contained in SGW-CDR main level. This is included in the Traffic data container when the initial status is reported after opening of SGW-CDR, or if previous container’s change condition is "Presence in Presence Reporting Area Change".

User CSG information contains the status of the user accessing a CSG cell (CSG ID within the PLMN, Access mode and indication on CSG membership) during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container’s change condition is "user CSG information change". Note the "user CSG information" in S-CDR/SGW-CDR main level contains the "user CSG information" when S-CDR/SGW-CDR was opened.

EPC QoS Information In case of IP-CAN bearer specific container this contains authorized QoS for the IP-CAN bearer. First container for each QCI/ARP pair includes this field. In following containers this field is present if previous change condition is "QoS change". This field is applicable only in SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR, and TWAG-CDR.

In S-CDR first container includes following optional fields: QoS Requested and QoS Negotiated. In following containers QoS Negotiated is present if previous change condition is "QoS change". In addition to the QoS Negotiated parameter the QoS Requested parameter is present in following containers if the change condition is "QoS change" and the QoS change was initiated by the MS via a IP-CAN bearer modification procedure.

Table 5.1.2.2.25.1 illustrates an examplefor S-CDR but same principles are applicable also for SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR. There are five containers (sets of volume counts) caused by one QoS change, one location change, one tariff time change and one Direct Tunnel establishment (direct tunnel change applicable in S-CDR only). When CDR is opened the subscriber is in CGI1.

Table 5.1.2.2.25.1: Example list of traffic data volumes

| QoS Requested = QoS1 QoS Negotiated = QoS1 Data Volume Uplink = 1 Data Volume Downlink = 2 Change Condition = QoS change Time Stamp = TIME1 | QoS Requested = QoS2 (if requested by the MS) QoS Negotiated = QoS2 Data Volume Uplink = 5 Data Volume Downlink = 6 Change Condition = Tariff change Time Stamp = TIME2 | Data Volume Uplink = 10 Data Volume Downlink = 3 Change Condition = CGI/SAI Change Time Stamp = TIME3 |
| --- | --- | --- |

| Data Volume Uplink = 3 Data Volume Downlink = 4 User Location Info = CGI2 Change Condition = Direct Tunnel establishment Occurrence Time Stamp = TIME4 | Change Condition = Record closed Time Stamp = TIME5 |
| --- | --- |

First container includes initial QoS values and corresponding volume counts. Second container includes new QoS values and corresponding volume counts before tariff time change. Third container includes the indication of location change and corresponding volume counts before the location change and after the tariff time change. Fourth container includes volume counts after the location change and contains the indication of Direct Tunnel establishment. Last container includes no volume count as it refers to Direct Tunnel establishment. The total volume counts can be itemised as shown in Table 5.1.2.2.25.2 (tariff1 is used before and tariff2 after the tariff time change):

Table 5.1.2.2.25.2: Itemised list of total volume count corresponding to table 5.1.2.2.25.1

|  |  | Container |
| --- | --- | --- |
| QoS1+Tariff1 | uplink = 1, downlink = 2 | 1 |
| QoS2+Tariff1 | uplink = 5, downlink = 6 | 2 |
| QoS2+Tariff2 | uplink = 13, downlink = 7 | 3+4 |
| QoS1 | uplink = 1, downlink = 2 | 1 |
| QoS2 | uplink = 18, downlink = 13 | 2+3+4 |
| Tariff1 | uplink = 6, downlink = 8 | 1+2 |
| Tariff2 | uplink = 13, downlink = 7 | 3+4 |
| CGI1 | uplink = 16, downlink = 11 | 1+2+3 |
| CGI2 | uplink = 3, downlink = 4 | 4 |
| No Direct Tunnel | uplink = 19, downlink = 15 | 1+2+3+4 |
| Direct Tunnel | -, - | 5 |

The amount of data counted in the S-GW shall be the payload of the user plane at the S1-U/S4/S2interface. Therefore the data counted already includes the IP PDP bearer protocols i.e. IP or PPP.

The data volume counted in the SGSN is dependent on the system. For GSM SGSN the data volume is the payload of the SNDCP PDUs at the Gb interface. For UMTS-SGSN it is the GTP-U PDUs at the Iu-PS interface. Therefore, in both systems, the data counted already includes the overheads of any PDP bearer protocols.

In GSM, in order to avoid that downstream packets transmitted from the old SGSN to the new SGSN at inter SGSN RA update induce the increase of the PDP CDR downstream volume counters in both SGSN the following rules must be followed:

-	For PDP contexts using LLC in unacknowledged mode: an SGSN shall update the PDP CDR when the packet has been sent by the SGSN towards the MS;

For PDP contexts using LLC in acknowledged mode, a GSM-SGSN shall only update the PDP CDR at the reception of the acknowledgement by the MS of the correct reception of a downstream packet. In other worlds, for inter SGSN RA update, the new SGSN shall update the PDP CDR record when a downstream packet sent by the old SGSN is received by the MS and acknowledged by the MS towards the new SGSN through the RA update complete message.

In UMTS, the not transferred downlink data can be accounted for in the S-CDR with "RNC Unsent Downlink Volume" field, which is the data that the RNC has either discarded or forwarded during handover. Data volumes retransmitted (by RLC or LLC) due to poor radio link conditions shall not be counted.

The following additional fields are applicable in the P-GW CDR when charging per IP-CAN session is active and IP-CAN bearer charging is being performed for the session:

Charging Id In case of IP-CAN bearer specific container this contains the charging Id assigned to the bearer.

RAT Type This field contains the RAT type for the current IP-CAN bearer.

Access Availability Change Reason indicates the reason why the avalibility of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable as defined in TS 29.212 [220].

Related Change of Charging Condition indicates the reason a related container was closed when the current container is indirectly closed and the supplemental information for the event. This information is applicable when charging per IP-CAN session is active for a multi-access PDN connection.

Diagnostics In case of IP-CAN bearer specific container this contains the Diagnostics as per clause 5.1.2.2.11 associated to the bearer.

Enhanced Diagnostics In case of IP-CAN bearer specific container this contains the Enhanced Diagnostics as per clause 5.1.2.2.13Aa associated to the bearer.

CP CIoT EPS optimisation indicator This field contains the indication on whether Control Plane CIoT EPS optimisation is used for the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "change in user plane to UE". Note the CP CIoT EPS Optimisation indicator field in SGW-CDR main level contains the CP CIoT EPS optimisation indicator value when SGW-CDR was opened.

Serving PLMN Rate Control This field contains the Serving PLMN Rate Control applied by MME during the transfer of the data volume captured by the container (applicable only to the SGW-CDR). This is included in the Traffic data container only if previous container's change condition is " Serving PLMN Rate Control change ". Note the Serving PLMN Rate Control field in SGW-CDR main level contains the Serving PLMN Rate Control when SGW-CDR was opened.

##### 5.1.2.2.26	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially for each partial CDR (or whole CDR) including all CDR types. The number is unique within one node, which is identified either by field Node ID or by record-dependent node address (SGSN address, S-/P-GW address, TDF address, IP-Edge Address, Recording Entity).

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.2.2.27	Location Estimate

The Location Estimate field is providing an estimate of a geographic location of a target MS according to TS 29.002 [214].

##### 5.1.2.2.28	Location Method

The Location Method identifier refers to the argument of LCS-MOLR that was invoked as defined in TS 24.080 [209].

##### 5.1.2.2.29	Location Type

This field contains the type of the location as defined in TS 29.002 [214].

##### 5.1.2.2.29A	Low Priority Indicator

This field indicates if the PDN connection has a low priority, i.e. for Machine Type Communication.

##### 5.1.2.2.29B	NBIFOM Mode

This field indicates the NBIFOM mode selected by PCRF for a multi-access PDN connection, i.e. UE initiated or Network initiated as defined in TS 29.212 [220].

##### 5.1.2.2.29C	NBIFOM Support

This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session as defined in TS 29.212 [220].

##### 5.1.2.2.30	Measurement Duration

This field contains the duration for the section of the location measurement corresponding to the Perform_Location_Request and Perform_Location_Response by the SGSN.

##### 5.1.2.2.31	Message reference

This field contains a unique message reference number allocated by the Mobile Station (MS) when transmitting a short message to the service centre. This field corresponds to the TP-Message-Reference element of the SMS_SUBMIT PDU defined in TS 23.040 [201].

##### 5.1.2.2.32	MLC Number

This parameter refers to the ISDN (ITU-T Rec. E.164 [308]) number of a GMLC.

##### 5.1.2.2.32A	MME Name

This field contains the Diameter Identity of the serving MME.

##### 5.1.2.2.32B	MME Realm

This field contains the Diameter Realm Identity of the serving MME.

##### 5.1.2.2.33	MS Network Capability

This MS Network Capability field contains the MS network capability value of the MS network capability information element of the served MS on PDP context activation or on GPRS attachment as defined in TS 24.008 [208].

##### 5.1.2.2.34	MS Time Zone

This field contains the 'Time Zone' IE provided by the SGSN/MME and transferred to the S-GW/P-GW/TDF during the IP-CAN bearer activation/modification procedure as specified in TS 29.060 [215] and in TS 29.212 [220].

##### 5.1.2.2.35	Network Initiated PDP Context

This field in S-CDR indicates that PDP context is network initiated. The field is missing in case of mobile activated PDP context.

##### 5.1.2.2.36	Node ID

This field contains an optional, operator configurable, identifier string for the node that had generated the CDR. 
The Node ID may or may not be the DNS host name of the node.

##### 5.1.2.2.37	Notification to MS user

This field contains the privacy notification to MS user that was applicable when the LR was invoked as defined in TS 29.002 [214].

##### 5.1.2.2.37A	Originating Address

This field is the Originating Address of the SME as defined in TS 23.040 [201].

##### 5.1.2.2.37B	P-GW Address IPv6

This field is the P-GW IPv6 Address used for the Control Plane, when both IPv4 and IPv6 addresses of the P-GW are available.

##### 5.1.2.2.38	P-GW Address Used

These field is the serving P-GW IP Address for the Control Plane. If both an IPv4 and an IPv6 address of the P-GW is available, the P-GW shall include the IPv4 address in the CDR.

##### 5.1.2.2.39	P-GW PLMN Identifier

This field is the P-GW PMLN Identifier (Mobile Country Code and Mobile Network Code).

The MCC and MNC are coded as described for "User Location Info" in TS 29.274 [223].

##### 5.1.2.2.40	PDN Connection Charging ID

This field defines the PDN connection (IP-CAN session) Charging identifier to identify different records belonging to same PDN connection. For a PDN connection that is limited to use one single access at a time this field includes Charging Id of first IP-CAN bearer activated. Together with P-GW address this uniquely identifies the PDN connection.

For application based charging by the TDF:

-	In case of GTP based connectivity, an "EPS default bearer Charging Identifier"

-	In case of PMIP based connectivity, an "unique Charging Id"

is assigned by the P-GW and transferred to the TDF via the PCRF for the TDF session.

##### 5.1.2.2.41	PDP Type

This field defines the PDP type, e.g. IP, or PPP as per TS 29.060 [215] .

##### 5.1.2.2.42	PDP/PDN Type

This field defines the bearer type, e.g. IP, or PPP as per TS 29.060 [215] .

##### 5.1.2.2.42A	PDP/PDN Type Extension

This field defines the PDN type as per TS 29.061 [216] for Non-IP PDN Type.

##### 5.1.2.2.43	Positioning Data

This information element is providing positioning data associated with a successful or unsuccessful location attempt for a target MS according TS 49.031 [227].

##### 5.1.2.2.43A	Presence Reporting Area Information

This field contains the Presence Reporting Area Information: Presence Reporting Area identifier(s) and the indication on whether the UE is inside or outside the Presence Reporting Area(s), as described in TS 29.274 [223].

##### 5.1.2.2.44	Privacy Override

This parameter indicates if the LCS client overrides MS privacy when the GMLC and VMSC/SGSN for an MT-LR are in the same country as defined in TS 29.002 [214].

##### 5.1.2.2.45	PS Furnish Charging Information

This field includes following information elements for IP-CAN bearer (PGW-CDR) , for TDF session (TDF-CDR) , or for the IP-CAN session (IPE-CDR):

- PS Free Format Data

This field contains charging information sent by the OCS in the Diameter Credit-Control Credit-Control-Answer messages as defined in TS 32.251 [11]. The data can be sent either in one Diameter Credit-Control Credit-Control-Answer message or several Diameter Credit-Control Credit-Control-Answer messages with append indicator. This data is transferred transparently in the PS Furnish Charging Information field of the relevant call records.

If the PS Free Format Data is received more than once during one IP-CAN bearer for which an offline session is established, the append indicator defines whether the PS Free Format Data is appended to previous received PS Free Format Data and stored in the relevant record or the information of the last PS Free Format Data received is stored in the relevant record (the previous PS Free Format Data information shall be overwritten).

In the event of partial output the currently valid "PS Free format data" is stored in the partial record.

- PS FFD Append Indicator:

This field contains an indicator whether PS free format data is to be appended to the PS free format data stored in previous partial CDR. This field is needed in CDR post processing to sort out valid PS free format data for that IP-CAN bearer from sequence of partial records. Creation of partial records is independent of received PS Free Format Data and thus valid PS  free format data may be divided to different partial records.

If field is missing then the PS free format data in this CDR replaces all received PS free format data in previous CDRs. Append indicator is not needed in the first partial record. In following partial records indicator shall get value true if all PS Free Format Data received during that partial record have append indicator. If one or more of the received PS Free Format Data for that PDP Context during the partial record do not have append indicator then this field shall be missing.

##### 5.1.2.2.46	QoS Requested/QoS Negotiated

Quality of Service Requested contains the QoS desired by MS at IP-CAN bearer activation. QoS Negotiated indicates the applied QoS accepted by the network.

If a pre-Release '99 only capable terminal is served, the applicable QoS parameters and their encoding in the CDRs are specified in TS 32.015 [228].

In all other cases, the applicable QoS attributes are defined in the "Quality of Service profile" in TS 23.060 [202], and their encoding in the CDR corresponds to the "Quality of Service profile" specified in TS 29.060 [215].

##### 5.1.2.2.47	RAT Type

Holds the value of RAT Type, as provided to S-GW and P-GW, specified in TS 29.061 [216] and also provided to the TDF as specified in TS 29.212 [220].

The field is provided by the SGSN/MME and transferred to the S-GW/P-GW during the IP-CAN bearer activation/modification and transferred to the TDF during the TDF session establishment/modification.

##### 5.1.2.2.48	Record Extensions

This field enables network operators and/or manufacturers to add their own recommended extensions to the standard record definitions. This field contains a set of "management extensions" as defined in X.721 [305]. 
This is conditioned upon the existence of an extension.

##### 5.1.2.2.49	Record Opening Time

This field contains the time stamp when the MS is attached to a SGSN (M-CDR) or IP-CAN bearer is activated in SGSN/S-GW/P-GW (S-CDR, SGW-CDR, PGW-CDR) or TDF session is established, or IP-CAN session is established (IPE-CDR), or record opening time on subsequent partial records (see TS 32.250 [4] for exact format).

Record opening reason does not have a separate field. For SGW/PGW/TDF/IPE -CDRs and M-CDR it can be derived from the field "Sequence number"; i.e. either a missing field or a value one (1) means activation of IP-CAN bearer (SGW/PGW-CDR) or GPRS attachment (M-CDR) or TDF session establishment (TDF-CDR) , or IP-CAN session establishment (IPE-CDR). For the S-CDR the field "SGSN change" also needs to be taken into account.

##### 5.1.2.2.50	Record Sequence Number

This field contains a running sequence number employed to link the partial records generated in the SGSN/SGW/PGW for a particular MM context or IP-CAN bearer, or IP-CAN session, or TDF session (characterised with the same Charging ID and PGW address pair). For M-CDR or S-CDR the sequence number always restarts from one (1) after an inter-SGSN routing area update, see field "SGSN change". The Record Sequence Number is missing if the record is the only one produced in the SGSN/SGW/PGW/TDF/IP-Edge (e.g. inter-SGSN routing area update can result to two M-CDR or two S-CDRs without sequence number and field "SGSN change" present in the second record).

##### 5.1.2.2.51	Record Type

The field identifies the type of the record e.g. S-CDR, SGW-CDR, PGW-CDR, M-CDR, S-SMO-CDR, TDF-CDR, IPE-CDR and S-SMT-CDR.

##### 5.1.2.2.52	Recording Entity Number

This field contains the E.164 number assigned to the entity that produced the record. For further details see TS 23.003 [200].

##### 5.1.2.2.52A	Retransmission

This parameter, when present, indicates that information from retransmitted Diameter ACRs has been used in this CDR.

##### 5.1.2.2.53	RNC Unsent Downlink Volume

This field contains the unsent downlink volume that the RNC has either discarded or forwarded to 2G-SGSN and already included in S-CDR. This field is present when RNC has provided unsent downlink volume count at RAB release and can be used by a downstream system to apply proper charging for this PDP context.

##### 5.1.2.2.54	Routing Area Code/Location/Cell Identifier/Change of location

These fields can occur only in SGSN generated CDRs. The location information contains a combination of the Routing Area Code (RAC) and an optional Cell Identifier of the routing area and cell in which the served party is currently located. In GSM the Cell Identifier is defined by the Cell Identity (CI) and in UMTS by the Service Area Code (SAC). Any change of location (i.e. Routing Area change) may be recorded in the change of location field including the time at which the change took place.

The location field contains a combination of the location area code (LAC), cell identity (CI) and MCC+MNC of the cell in which the served party is currently located.

The change of location field is optional and not required if partial records are generated when the location changes.

The RAC and (optionally) CI are coded according to 3G TS 24.008 [208] and the SAC according  TS 25.413 [212].

##### 5.1.2.2.54A	S-GW Address IPv6

This field is the S-GW IPv6 Address used for the Control Plane, when both IPv4 and IPv6 addresses of the S-GW are available.

##### 5.1.2.2.55	S-GW Address Used

These field is the serving S-GW IP Address for the Control Plane. If both an IPv4 and an IPv6 address of the S-GW is available, the S-GW shall include the IPv4 address in the CDR.

##### 5.1.2.2.56	S-GW Change

This field is present only in the SGW-CDR ,ePDG-CDR or TWAG-CDR to indicate that this is the first record after an inter serving node change (change from SGW, ePDG, TWAG, HSGW).

##### 5.1.2.2.57	Served 3GPP2 MEID

This field contains the Mobile Equipment Identity of the user's terminal in 3GPP2 access, and the content is defined in  TS 29.272 [222].

##### 5.1.2.2.57A	Served Fixed Subscriber Id

This field contains the subscriber identity, as defined in Broadband Forum TR 134 [601], used by the Fixed User (i.e. Fixed Device or RG) for Subscriber IP session in fixed broadband access network.

##### 5.1.2.2.58	Served IMEI

This field contains the International Mobile Equipment Identity (IMEI) of the equipment served, if available. The term "served" equipment is used to describe the ME involved in the transaction recorded e.g. the called ME in the case of a network initiated PDP context.

The structure of the IMEI is specified in TS 23.003 [200] and the encoding defined in TS 29.274 [223].

##### 5.1.2.2.58A	SCS/AS Address

This field contains the Address of SCS/AS.

##### 5.1.2.2.59	void

##### 5.1.2.2.60	Served IMSI

This field contains the International Mobile Subscriber Identity (IMSI) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded e.g. the calling subscriber in case of a mobile initiated PDP context.

The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.2.2.60A	Served IP-CAN session Address

This field contains the IP address for the IP-CAN session. This is a network layer address i.e. of type IPv4 address or IPv6 prefix. The address for each IP-CAN session type is allocated either temporarily or permanently (see "Dynamic Address Flag"). This parameter shall be present except when both the IP-CAN session type is PPP and dynamic address assignment is used.

##### 5.1.2.2.60B	Served IP-CAN session Address Extension

This field contains the IPv4 address for the IP-CAN session when dual-stack IPv4 IPv6 is used, and the IPv6 prefix is included in Served IP-CAN session Address or Served IP-CAN Address.

##### 5.1.2.2.61	Served MN NAI

This field contains the Mobile identifier of the served user, in NAI format based on IMSI, as defined TS 23.003 [200].

##### 5.1.2.2.62	Served MSISDN

This field contains the Mobile Station (MS) ISDN number (MSISDN) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded. In case of multi-numbering the MSISDN stored in a CDR will be the primary MSISDN of the calling party.

The structure of the MSISDN is defined in TS 23.003 [200].

##### 5.1.2.2.63	Served PDP Address

This field contains the PDP address of the served IMSI. This is a network layer address i.e. of type IPv4 address or IPv6 prefix. The address for each PDP type is allocated either temporarily or permanently (see "Dynamic Address Flag"). This parameter shall be present except when both the PDP type is PPP and dynamic PDP address assignment is used.

##### 5.1.2.2.64	Served PDP/PDN Address

This field contains the IP address for the PDN connection (PDP context, IP-CAN bearer). This is a network layer address i.e. of type IPv4 address or IPv6 prefix. The address for each Bearer type is allocated either temporarily or permanently (see "Dynamic Address Flag"). This parameter shall be present except when both the Bearer type is PPP and dynamic address assignment is used.

##### 5.1.2.2.64A	Served PDP/PDN Address Extension

This field contains the IPv4 address for the PDN connection (PDP context, IP-CAN bearer) when dual-stack IPv4 IPv6 is used, and the IPv6 prefix is included in Served PDP Address or Served PDP/PDN Address.

##### 5.1.2.2.64B	Served PDP/PDN Address prefix length

This field contains the prefix length of an IPv6 typed Served PDP/PDN Address. The field needs not available for prefix length of 64 bits, as in this case the 64 bit prefix length default shall be assumed.

##### 5.1.2.2.65	Service Centre Address

This field contains a E.164 number identifying a particular service centre e.g. Short Message Service (SMS) centre (see TS 23.040 [201]).

##### 5.1.2.2.66	Serving Node Address

These fields contain one or several control plane IP addresses of SGSN, MME, ePDG, HSGW, TWAG or S-GW, which have been connected during the record.

If both an IPv4 and an IPv6 address of the SGSN/S-GW/MME/ePDG/HSGW/TWAG are available, the S-GW/P-GW/TDF shall include the IPv4 address in the CDR.

##### 5.1.2.2.66A	Serving Node IPv6 Address

These fields contain one or several control plane IPv6 addresses, in case of IPv4v6 dual stack, of SGSN, MME, ePDG, HSGW, TWAG or S-GW, which have been connected during the record, when both IPv4 and IPv6 addresses of the node are available.

##### 5.1.2.2.67	Serving Node PLMN Identifier

This field contains  the PLMN Identifier (Mobile Country Code and Mobile Network Code) serving the UE.

The MCC and MNC are coded as described for 'Routing Area Identity' in TS 29.060 [75].

##### 5.1.2.2.68	Serving Node Type

These fields contain one or several serving node types in control plane of S-GW or P-GW, which have been connected during the record. The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence.

For Originated and Terminated SMS CDRs, this field contains the Node Type which generates the CDRs, i.e SGSN or MME.

##### 5.1.2.2.68A	Serving PLMN Rate Control

This field contains the Serving PLMN Rate Control as specified in TS 23.401 [245], which is used during the record for the PDN connection to the PGW.

##### 5.1.2.2.68B	SGi PtP Tunnelling Method

This field indicates whether SGi PtP tunnelling method is based on UDP/IP or other methods for a non-IP PDN type PDN connection.

##### 5.1.2.2.69	SGSN Address

These fields contain one or several IP addresses of SGSN. The IP address of the SGSN can be either control plane address or user plane address.

The S-CDR fields contain single address of current SGSN.

The M-CDR fields only contain the address of the current SGSN.

If both an IPv4 and an IPv6 address of the SGSN are available, the SGSNs shall include the IPv4 address in the CDR.

##### 5.1.2.2.69A	Void

(Void).

##### 5.1.2.2.70	SGSN Change

This field is present only in the S-CDR to indicate that this is the first record after an inter-SGSN routing area update.

##### 5.1.2.2.71	Short Message Service (SMS) Result

This field contains the result of an attempt to deliver a short message either to a service centre or to a mobile subscriber (see TS 29.002 [214]). Note that this field is only provided if the attempted delivery was unsuccessful.

##### 5.1.2.2.72	Start Time

This field contains the time when the IP-CAN session starts at the S-GW/P-GW/IP-Edge or TDF session starts at TDF, available in the CDR for the first bearer in an IP-CAN session in case of S-GW/P-GW/IP-Edge or, alternatively, available in the CDR for the TDF session start.

##### 5.1.2.2.73	Stop Time

This field contains the time when the IP-CAN session is terminated at the S-GW/P-GW/IP-Edge or TDF session terminated at the TDF, available in the CDR for the last bearer in an IP-CAN session in case of S-GW/P-GW/IP-Edge or, alternatively, available in the CDR for the TDF session stop.

##### 5.1.2.2.73aA 	TDF Address Used

This field is the serving TDF IP Address for the Control Plane. If  both an IPv4 and an IPv6 addresses of the TDF are available, the TDF shall include the IPv4 address in the CDR. It contains the TDF-IP-Address as described in TS 29.212 [220].

##### 5.1.2.2.73bA 	TDF IPv6 Address Used

This field is the serving TDF IPv6 Address for the Control Plane, when both IPv4 and IPv6 addresses of the TDF are available.

##### 5.1.2.2.73cA 	TDF PLMN Identifier

This field is the TDF PMLN Identifier (Mobile Country Code and Mobile Network Code).

##### 5.1.2.2.73cAa	Traffic Steering Policy Identifier Uplink

This field contains traffic steering policy identifier in the uplink direction as specified in TS 23.203[203].

5.1.2.2.73cAb	Traffic Steering Policy Identifier Downlink

This field contains traffic steering policy identifier in the downlink direction as specified in TS 23.203[203].

##### 5.1.2.2.73dA	TWAG Address Used

This field is the serving TWAG IP Address for the Control Plane. If both an IPv4 and an IPv6 address of the TWAG is available, the TWAG shall include the IPv4 address in the CDR.

##### 5.1.2.2.73eA	TWAG IPv6 Address

This field is the serving TWAG IPv6 Address for the Control Plane, when both IPv4 and IPv6 addresses of the TWAG are available.

##### 5.1.2.2.73A	TWAN User Location Information

This field holds the UE location in a Trusted WLAN Access Network (TWAN), i.e BSSID and SSID of the access point.

##### 5.1.2.2.73B	UNI PDU CP Only Flag

This field contains an indication on whether this PDN connection is applied with "Control Plane Only flag", i.e. transfer using Control Plane NAS PDUs only, when Control Plane CIoT EPS Optimisation is enabled. This field is missing if both, user plane and control plane UNI for PDU transfer (i.e.  S1-U and S11-U from S-GW) are allowed, when Control Plane CIoT EPS Optimisation is enabled.

##### 5.1.2.2.74	User CSG Information

This field contains the "User CSG Information" status of the user accessing a CSG cell: it comprises CSG ID within the PLMN, Access mode and indication on CSG membership for the user when hybrid access applies, as defined in TS 29.060 [215] for GPRS case, and in TS 29.274 [223] for EPC case.

##### 5.1.2.2.75	User Location Information

This field contains the User Location Information as described in

-	TS 29.060 [215] for GTP case (e.g. CGI, SAI, RAI),

-	TS 29.274 [223] for eGTP case (e.g. CGI, SAI, RAI TAI and ECGI) and

-	TS 29.275 [224] for PMIP case.

The field is provided by the SGSN/MME and transferred to the S-GW/P-GW/TDF during the IP-CAN bearer activation/modification and/or TDF session establishment/modification.

##### 5.1.2.2.75A	User Location Information Time

This field contains the time at which the UE was last known to be in the location which is reported during bearer deactivation or UE detach procedure.

##### 5.1.2.2.76	Void

##### 5.1.2.2.77	UWAN User Location Information

This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected). It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE.

#### 5.1.2.3	Void

#### 5.1.2.4	CP data transfer domain CDR parameters

##### 5.1.2.4.1	Introduction

This subclause contains the description of the CDR parameters that are specific to the CP data transfer domain CDR types as specified in TS 32.253 [13].

##### 5.1.2.4.2	Access Point Name (APN) Network Identifier

These fields contain the actual connected Access Point Name Network Identifier. APN selection by the MME for  SCEF based delivery mechanism selection for non-IP data delivery is specified in TS 23.682 [243]

The APN Network Identifier (NI portion) is part of APN, which format is specified in TS 23.003 [200]. To represent the APN NI in the CPCN CDRs, the "dot" notation shall be used.

##### 5.1.2.4.3	APN Rate Control

This field contains the APN Rate Control as specified in TS 29.128 [244], which is used during the record for the PDN connection to the SCEF.

##### 5.1.2.4.4	Cause for Record Closing

This field contains a reason for the release of the CDR. In case of Rf interface is used, it is derived, when received, from  Change-Condition AVP at PS-information AVP level defined in TS 32.299 [50] used for CP data transfer. The following is included:

-	normal release: PDN connection to SCEF release; It corresponds to "Normal Release" in Change-Condition AVP.

-	abnormal termination (PDN connection to SCEF); It corresponds to "Abnormal Release" in Change-Condition AVP.

-	data volume limit; It corresponds to "Volume Limit" in Change-Condition AVP.

-	time (duration) limit; It corresponds to "Time Limit" in Change-Condition AVP.

-	maximum number of NIDD submissions. It corresponds to "Maximum number of NIDD submissions" in Change-Condition AVP.

-	Serving Node change; It corresponds to "Serving Node Change" in Change-Condition AVP and is used for MME change.

-	For SCEF, PLMN change; It corresponds to "PLMN Change" in Change-Condition AVP.

-	For SCEF: APN Rate Control Change: It corresponds to "APN Rate Control Change" in Change-Condition AVP.

-	Serving PLMN Rate Control Change: It corresponds to "Serving PLMN Rate Control Change" in Change-Condition AVP.

-	RAT type Change: It corresponds to "RAT Change " in Change-Condition AVP.

-	management intervention (request due to O&M reasons); It corresponds to "Management Intervention" in Change-Condition AVP.

A more detailed reason may be found in the Diagnostics field.

##### 5.1.2.4.5	Charging Characteristics

The Charging Characteristics field allows the operator to apply different kind of charging methods in the CDRs. 
A subscriber may have Charging Characteristics assigned to his subscription. These characteristics can be supplied by the HSS and selected according to the rules specified in Annex X of TS 32.253 [13].

This information can be used by the CPCNs (SCEF, IWK-SCEF, MME) to activate charging generation and e.g. control the closure of the CDR or the traffic volume containers, and applied charging characteristics is included in CDRs transmitted via the Ga/Rf reference point. It can also be used in nodes handling the CDRs (e.g., the CGF or the billing system) to influence the CDR processing priority and routing. These functions are accomplished by specifying the charging characteristics as sets of charging profiles and the expected behaviour associated with each profile. 
The interpretations of the profiles and their associated behaviours can be different for each PLMN operator and are not subject to standardisation. In the present document only the charging characteristic formats and selection modes are specified.

The format of charging characteristics field is depicted in Figure 5.1.2.4.5.1. Each Bx (x =0..15) refers to a specific behaviour defined on a per-Operator basis, indicated as active when set to "1" value. See Annex X of TS 32.253 [13]. for guidance on how behaviours could be defined.

Figure 5.1.2.4.5.1: Charging Characteristics flags

##### 5.1.2.4.6	Charging Characteristics selection mode

This field indicates the charging characteristic type that the CPCNs (SCEF, IWK-SCEF, MME) applied to the CDR. In the MME the allowed values are:

-	Home default;

-	Roaming default;

-	APN specific;

-	Subscription specific.

In the IWK-SCEF/SCEF the allowed values are:

-	Home default;

-	Roaming default;

-	Serving node supplied.

Further details are provided in TS 32.253 [13] Annex X.

##### 5.1.2.4.7	Charging ID

This field is a charging identifier, which can be used together with SCEF Identity to identify all records produced in CPCN Nodes involved in a single PDN connection to a SCEF. Charging ID is generated by SCEF at PDN connection establishment and transferred to IWK-SCEF/MME., The charging ID is transferred to the new MME, at MME change during the lifetime of the PDN connection.

Different SCEFs allocate the charging ID independently of each other and may allocate the same numbers. 
The CGF and/or BS may check the uniqueness of each charging ID together with the SCEFs address and optionally (if still ambiguous) with the record opening time stamp.

##### 5.1.2.4.8	Diagnostics

This field includes a more detailed technical reason for the releases of the connection. 
The diagnostics may also be extended to include manufacturer and network specific information.

##### 5.1.2.4.9	Duration

This field contains the relevant duration in seconds for PDN connection to SCEF in CPCN Node CDRs.

It is the duration from Record Opening Time to record closure. For partial records this is the duration of the individual partial record and not the cumulative duration.

It should be noted that the internal time measurements may be expressed in terms of tenths of seconds or even milliseconds and, as a result, the calculation of the duration may result in the rounding or truncation of the measured duration to a whole number of seconds.

Whether or not rounding or truncation is to be used is considered to be outside the scope of the present document subject to the following restrictions:

1)	A duration of zero seconds shall be accepted providing that the transferred data volume is greater than zero.

2)	The same method of truncation/rounding shall be applied to both single and partial records.

##### 5.1.2.4.10	 External-Identifier

This field contains the external identifier of the UE, which identifies a subscription associated to an IMSI, as specified in TS 23.003 [200].

##### 5.1.2.4.11	List of NIDD Submissions

This list applicable in CPDT-SCE-CDR and CPDT-SNN-CDR, includes one or more containers.

Each container includes the following fields:

-	Submission Timestamp;

-	Event Timestamp;

-	Data Volume Uplink.

-	Data Volume Downlink.

-	Service Change Condition.

-	Submission Result Code.

SubmissionTimestamp is a time stamp, which defines the moment when the data transfer request NIDD submission was submitted to the CPCN Node.

Event Timestamp is a time stamp, which defines the moment when the event triggered the generation of charging information from the CPCN Node, for the NIDD submission.

Data Volume Uplink and Downlink includes the number of octets transmitted during the NIDD submission.

Service Change Condition contains the reason for the NIDD submission container, e.g. NIDD submission response receipt, NIDD submission response sending, NIDD delivery to the UE, NIDD submission timeout, NIDD delivery from the UE error.

Submission Result Code defines the result of NIDD submission.

##### 5.1.2.4.12	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially for each partial CDR (or whole CDR) including all CDR types. The number is unique within one node, which is identified either by field Node ID or by record-dependent node identities (SCEF F, MME Identity).

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.2.4.13	Node ID

This field contains an optional, operator configurable, identifier string for the node that had generated the CDR. 
The Node ID may or may not be the DNS host name of the node.

##### 5.1.2.4.14	RAT Type

This field contains the Radio Access Technology (RAT) type used for the NIDD submissions.

##### 5.1.2.4.15	Record Extensions

This field enables network operators and/or manufacturers to add their own recommended extensions to the standard record definitions. This field contains a set of "management extensions" as defined in X.721 [305]. 
This is conditioned upon the existence of an extension.

##### 5.1.2.4.16	Record Opening Time

This field contains the time stamp when the PDN connection to SCEF is activated in CPDT-SCE-CDR, CPDT-IWK-CDR, CPDT-MME-CDR, or record opening time on subsequent partial records.

Record opening reason does not have a separate field. For CPDT-SCE-CDR, CPDT-IWK-CDR and CPDT-MME-CDR, it can be derived from the field "Sequence number"; i.e. either a missing field or a value one (1) means activation of PDN connection to SCEF.

##### 5.1.2.4.17	Record Sequence Number

This field contains a running sequence number employed to link the partial records generated in the SCEF/IWK-SCEF/MME for a particular PDN connection to SCEF (characterised with the same Charging ID and SCEF ID pair). The Record Sequence Number starts from one (1). The Record Sequence Number is missing if the record is the only one produced in the SCEF/IWK-SCEF/MME.

##### 5.1.2.4.18	Record Type

The field identifies the type of the record i.e. CPCN-SCE-CDR and CPCN-SNN-CDR.

##### 5.1.2.4.19	Retransmission

This parameter, when present, indicates that information from retransmitted Diameter ACRs has been used in this CDR.

##### 5.1.2.4.20	SCEF ID

This field contains the Diameter Identity of the SCEF serving the PDN connection.

##### 5.1.2.4.21	Served IMSI

This field contains the International Mobile Subscriber Identity (IMSI) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded.

The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.2.4.22	Served MSISDN

This field contains the Mobile Station (MS) ISDN number (MSISDN) of the served party. The term "served" party is used to describe the mobile subscriber involved in the transaction recorded. In case of multi-numbering the MSISDN stored in a CDR will be the primary MSISDN.

The structure of the MSISDN is defined in TS 23.003 [200].

##### 5.1.2.4.23	Serving Node Identity

This field contains the Diameter Identity of the serving node (i.e. MME) connected during the record for the PDN connection to the SCEF.

##### 5.1.2.4.24	Serving Node PLMN Identifier

This field contains the PLMN Identifier (Mobile Country Code and Mobile Network Code) serving the UE.

##### 5.1.2.4.25	Serving PLMN Rate Control

This field contains the Serving PLMN Rate Control as specified in TS 29.128 [244], which is used during the record for the PDN connection to the SCEF.

### 5.1.3	Subsystem level CDR parameters

#### 5.1.3.0	General

This subclause contains the description of the CDR parameters that are specific to the subsystem level CDR types. 
This comprises the CDR types from the CN IM subsystem (TS 32.260 [20]).

#### 5.1.3.1	IMS CDR parameters

##### 5.1.3.1.0	Introduction

This clause contains the description of each field of the IMS CDRs specified in TS 32.260 [20].

##### 5.1.3.1.1	Access Correlation ID

This field holds the charging identifier of the access network. For GPRS access, this shall be the GPRS Charging ID, for EPS, this shall be the charging ID and for other access networks this shall be the Access Network Charging Identifier Value.

##### 5.1.3.1.2	Access Network Information

Holds the content of one of the SIP P-header "P-Access-Network-Info". In SIP, as per RFC 7315 [404], the content of the "P-Access-Network-Info" header is known as the access-net-spec. When multiple access-net-spec values are transported in a single P-Access-Network-Info header in comma-separated format, then only one access-net-spec value is included.

##### 5.1.3.1.2aA 	Access Transfer Type

This field indicates the type of access transfer performed for IMS service continuity, for instance PS-to-PS in case of SRVCC.

##### 5.1.3.1.2A 	Additional Access Network Information

Holds the content of an additional  SIP P-header "P-Access-Network-Info" when it is available. In SIP, as per RFC 7315 [404], the content of the "P-Access-Network-Info" header is known as the access-net-spec. When multiple access-net-spec values are transported in a single P-Access-Network-Info header in comma-separated format, then only one access-net-spec value is included.

##### 5.1.3.1.3	Alternate Charged Party Address

Holds the address of an alternate charged party determined by an AS at IMS session initiation.

##### 5.1.3.1.3A	AoC Information

AoC information is the AoC related Charging information transfered to the CDF, as defined in TS 32.280 [21].

##### 5.1.3.1.4	Application Provided Called Parties

Holds a list of the Called party address(es), if the address(es) are determined by an AS (SIP URL, E.164…).

##### 5.1.3.1.5	Application Servers Information

This is a grouped CDR field containing the fields: "Application Server Involved", "Application Provided Called Parties" and “Status”.

##### 5.1.3.1.6	Application Servers involved

Holds the ASs (if any) identified by the SIP URLs.

##### 5.1.3.1.7	Void

##### 5.1.3.1.8	Bearer Service

Holds the used bearer service for the PSTN leg.

##### 5.1.3.1.9	Called Party Address

In the context of an end-to-end SIP transaction (except for SIP REGISTER and SIP SUBSCRIBE transactions) this field holds the address of the party (Public User ID or Public Service ID) to whom the SIP transaction is posted. The Called Party Address shall be populated with the SIP URI (according to RFC3261 [401]) or Tel URI (according to RFC3966 [402]) contained in the outgoing Request-URI of the request (e.g. after ENUM query or after AS interaction). Called Party Address could also be populated with an URN (according to RFC5031 [407]) for an emergency SIP session.

For a registration procedure this field holds the party (Public User ID) to be registered. In this case, the Called Party Address field is obtained from the "To" SIP header of the SIP request. For a subscription procedure this field holds the address of the resource for which the originator wants to receive notifications of change of states. In this case, the Called Party Address field is obtained from the outgoing Request-URI of the SIP request.

##### 5.1.3.1.10	Carrier Select Routing

This item holds information on carrier select routing, received by S-CSCF during ENUM/DNS processes. 
The parameter corresponds to the CarrierSelectRoutingInformation AVP.

##### 5.1.3.1.11	Cause for Record Closing

This field contains a reason for the release of the CDR including the following:

- normal release: end of session;

- partial record generation: time (duration) limit, maximum number of changes in charging conditions (e.g. maximum number in 'List of Message Bodies' exceeded) or service change (e.g. change in media components, Access Transfer);

- abnormal termination;

- management intervention (request due to O&M reasons).

- CCF initiated record closure;

A more detailed reason may be found in the Service Reason Return Code field.

##### 5.1.3.1.11A 	Cellular Network Information

Holds the content of one SIP header "Cellular-Network-Info". As per TS 24.299 [210], a User Agent (UA) supporting one or more cellular radio access technology (e.g. E-UTRAN) but using a non-cellular IP-CAN to access the IM CN subsystem can use this header field to relay information to its service provider about the radio cell identity of the cellular radio access network on which the UE most recently camped.

##### 5.1.3.1.12	Content Disposition

This sub-field of Message Bodies holds the content disposition of the message body inside the SIP signalling, Content-disposition header field equal to "render", indicates that "the body part should be displayed or otherwise rendered to the user". Content disposition values are: session, render, inline, icon, alert, attachment, etc.

##### 5.1.3.1.13	Content Length

This sub-field of Message Bodies holds the size of the data of a message body in bytes.

##### 5.1.3.1.14	Content Type

This sub-field of Message Bodies holds the MIME type of the message body, Examples are: application/zip, image/gif, audio/mpeg, etc.

##### 5.1.3.1.15	Event

The Event parameter holds the content of the "Event" header defined in RFC 3265 [403],

##### 5.1.3.1.16	Expires

The Expires parameter holds the content of the "Expires" header.

##### 5.1.3.1.16aA	FE Identifier List

This parameter holds the FE Identifier List of the P-Charging-Vector header, as received in the FE-Identifier-List AVP as defined in TS 32.299 [50].

##### 5.1.3.1.16A	From Address

This field holds the information from the SIP From Header.

##### 5.1.3.1.17	GGSN Address

This parameter holds the control plane IP address of the GGSN that handles one or more media component(s) of a IMS session. If GPRS is used to access the IMS, the GGSN address is used together with the GPRS charging ID as the access part of the charging correlation vector. The charging correlation vector is comprised of an access part and an IMS part, which is the IMS Charging Identifier. For further information regarding the composition of the charging correlation vector refer to the appropriate clause in TS 32.240 [1].

##### 5.1.3.1.18	GPRS Charging ID

This parameter holds the GPRS charging ID (GCID) which is generated by the GGSN for a GPRS PDP context. There is a 1:1 relationship between the GCID and the PDP context. If GPRS is used to access the IMS, the GCID is used together with the GGSN address as the access part of the charging correlation vector that is comprised of an access part and an IMS part, which is the IMS Charging Identifier.

For further information regarding the composition of the charging correlation vector refer to the appropriate clause in TS 32.240 [1].

##### 5.1.3.1.18A	Void

##### 5.1.3.1.19	IMS Charging Identifier

This parameter holds the IMS charging identifier (ICID) as generated by the IMS node for the SIP session/transaction. The value of the ICID parameter is identical with the 'icid-value' parameter defined in TS 24.229 [210]. The 'icid-value' is a mandatory part of the P-Charging-Vector and coded as a text-based UTF-8 charset (as are all SIP messages). For further information regarding the composition and usage of the P-Charging-Vector refer to TS 32.260 [20], TS 24.229 [210] and RFC 7315 [404].

The ICID value is globally unique across all 3GPP IMS networks for a time period of at least one month, implying that neither the node that generated this ICID nor any other IMS node reuse this value before the uniqueness period expires.  The one month minimum uniqueness period counts from the time of release of the ICID, i.e. the ICID value no longer being used. This can be achieved by using node specific information, e.g. high-granularity time information and/or topology/location information.  The exact method how to achieve the uniqueness requirement is an implementation issue.

At each SIP session unrelated method, both initial and subsequent (e.g., REGISTER, NOTIFY, MESSAGE etc.), a new, session unrelated ICID is generated at the first IMS network element that processes the method. This ICID value is contained in the SIP request and SIP response of that SIP transaction and must be valid for the duration of the transaction.

At each SIP session establishment a new, session specific ICID is generated at the first IMS network element that processes the session-initiating SIP INVITE message. This ICID is then used in all subsequent SIP messages for that session (e.g., SIP 200 OK, SIP (RE-)INVITE, SIP BYE etc.) until the session is terminated.

##### 5.1.3.1.20	IMS Communication Service Identifier

This parameter holds the IMS Communication Service Identifier (ICSI) as contained in the P-Asserted-Service header of a SIP request to identify an IMS Communication Service as defined in TS 24.229 [210].

##### 5.1.3.1.20A	IMS Emergency Indicator

This field indicates the IMS session or registration is an IMS emergency session or emergency registration.This field is missing if IMS session/registration is not detected as an IMS emergency session/registration.

##### 5.1.3.1.20B	IMS Visited Network Identifier

Holds the SIP P-header "P-Visited-Network-ID". with the value according to 3GPP TS 24.229 [210].

-	For the roaming architecture for voice over IMS with local breakout, the value of IMS visited network identifier is a pre-provisioned string that identifies the network of the P-CSCF at the home network.

-	For the roaming architecture for voice over IMS with home routed traffic, IMS visited network identifier is a string that identifies the visited network of the UE including an indication that the P-CSCF is located in the home network.

##### 5.1.3.1.21	Incomplete CDR Indication

This field provides additional diagnostics when the CCF detects missing ACRs.

##### 5.1.3.1.21A	Initial IMS Charging Identifier

This parameter holds the Initial IMS charging identifier (ICID) as generated by the IMS node for the initial SIP session created for IMS service continuity.

##### 5.1.3.1.21Aa	Instance Id

An Instance Id is defined as a URN generated by the device that uniquely identifies a specific device amongst all other devices. The Instance Id is transported in the sip.instance feature tag in the Contact header of a SIP request associated with the served user.

##### 5.1.3.1.21Aaa	Inter-UE Transfer

This field indicates that Inter-UE transfer has been performed for IMS service continuity and present only in that case.

##### 5.1.3.1.21B	IP Realm Default Indication

This field indicates whether the IP realm used for the SDP media component is the Default IP realm or not.

##### 5.1.3.1.21C	ISUP Cause

When session is released via ISUP, this field indicates the reason the call was released.

##### 5.1.3.1.21Ca 	List of Access Network Info Change

This group field may occur several times in the CDR and each occurrence holds information on subsequent changes in one or two SIP P-header(s) "P-Access-Network-Info" together with the time the location was acquired.

Each element of the list may include the following fields:

-	Access Network Information;

-	Additional Access Network Information;

- 	Access ChangeTime.

##### 5.1.3.1.21D 	List of Access Transfer Information

This grouped field may occur several times in the CDR and each occurrence holds information on a particular access transfer. The field is present only if access transfer procedure has been performed for IMS service continuity.

Each element of the list represents an access transfer and may include the following fields:

-	Access Transfer Type;

-	Inter-UE Transfer;

-	Access Network Information;

-	Additional Access Network Information;

-	Subscriber Equipment Number;

-	Instance Id;

- 	Related IMS Charging Identifier;

- 	Related IMS Charging Identifier Generation Node;

- 	Access Transfer Time.

##### 5.1.3.1.22	List of Associated URI

The list of non-barred public user identities (SIP URIs and/or Tel URIs) associated to the public user identity under registration. The list of identities is obtained from the P-Associated-URI header of a SIP 200 OK response to a SIP REGISTER request.

##### 5.1.3.1.23	List of Called Asserted Identity

This field holds the address or addresses (SIP URI and/or Tel URI according to RFC 3261 [401] and RFC 3966 [402] respectively) of the party (Public User ID or Public Service ID) of the finally asserted called party.

These address/addresses are obtained from the P-Asserted-Identity SIP header field of the 2xx responses corresponding to a SIP request either initiating a dialog or a standalone transaction.

This field shall be present when the P-Asserted-Identity SIP header field is available in the SIP 2xx response. In case no P-Asserted-Identity is known, this list shall include one item (of type SIP URI) with the value "unknown".

##### 5.1.3.1.23A	List of Called Identity Changes

This field holds the set of terminating identity address changes after IMS session establishment and the associated time stamp for each.

These addresses are obtained from the From SIP header field of a SIP UPDATE request or SIP RE-INVITE request.

##### 5.1.3.1.24	List of Calling Party Address

The address or addresses (Public User ID or Public Service ID) of the party requesting a service or initiating a session. This field may hold the SIP URI (according to RFC 3261 [401]), the Tel URI (according to RFC 3966 402]) or both the SIP URI and the Tel URI of the calling party. The address is obtained from the P-Asserted-Identity header of a non-REGISTER SIP request, either initiating a dialog or a standalone transaction.

##### 5.1.3.1.25	List of Early SDP Media Components

This is a grouped field which may occur several times in one CDR.  This field describes session, media parameters and timestamps related to media components set to active according to SDP signalling exchanged during a SIP session establishment and before the final successful or unsuccessful SIP ANSWER to the initial SIP INVITE message is received. Once a media component has been set to active, subsequent status changes shall also be registered.

This field applies only to SIP session related cases, but it may be present both in event CDRs (unsuccessful session establishment) and session CDRs (successful session establishment).

The List of Early SDP Media Components contains the following elements:

- SDP Offer Timestamp;

- SDP Answer Timestamp;

- SDP Media Components;

- Media Initiator flag;

- SDP Session Description.

These fields are described in the appropriate subclause.

##### 5.1.3.1.26	List of Inter Operator Identifiers

This list holds the identification of the pair of originating network and terminating network if exchanged via SIP signalling, as recorded in the Inter Operator Identifier (IOI) AVP as described in TS 32.299 [50].  It may occur several times in one CDR. For further information on the IOI exchange via SIP signalling please refer to TS 24.229 [210].

##### 5.1.3.1.27	List of Message Bodies

This grouped field comprising several sub-fields describing the data that may be conveyed end-to-end in the body of a SIP MESSAGE.  Since several message bodies may be exchanged via SIP-signalling, this grouped field may occur several times.

The List of Message Bodies contains the following elements:

- Content Type;

- Content Disposition;

- Content Length;

- Originator.

They are described in the appropriate subclause.  Message bodies with the "Content-Type" field set to application/sdp and the "Content-Disposition" field set to session are not included in the "Message Bodies" field.

##### 5.1.3.1.27A	List of NNI Information

This grouped field comprising several sub-fields holds information about the NNI used for interconnection and roaming. This field may occur more than once in a CDR if more NNI are involved e.g. when support of transit routing is collocated with the IBCF.

The List of NNI Information contains the following elements:

-	Session Direction;

-	NNI Type;

-	Relationship Mode;

-	Neighbour Node Address.

These field elements are described in the appropriate subclause.

##### 5.1.3.1.28	List of SDP Media Components

This is a grouped field which may occur several times in one CDR and the content should be filled as described in TS 32.260 [20] clause 5.1.3.

The field is present only in a SIP session related case.

The List of SDP Media Components contains the following elements:

-	SIP Request Timestamp;

-	SIP Response Timestamp;

-	SDP Media Components;

-	Media Initiator flag;

-	SDP Session Description.

-	Media Initiator Party.

The Media Initiator Party is only used for PoC charging.

These field elements are described in the appropriate subclause.

##### 5.1.3.1.28A	List of Reason Header

This parameter contains the content of the Reason-header in the SIP BYE and SIP CANCEL, and may contain multiple entries if there are multiple Reason-headers within a SIP BYE or SIP CANCEL

##### 5.1.3.1.28B	Local GW Inserted Indication

This field indicates if the local GW (TrGW, IMS-AGW) is inserted or not for the SDP media component.

##### 5.1.3.1.29	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially for each partial CDR (or whole CDR) including all CDR types. The number is unique within the CCF.

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.3.1.30	Media Initiator Flag

This field indicates if the called party has requested the session modification and it is present only if the initiator was the called party.

##### 5.1.3.1.31	Media Initiator Party

This field indicates initiating party who has requested the session modification in PoC charging.

##### 5.1.3.1.31a	MS Time Zone

This field contains the 'Time Zone' IE provided as part of the NetLoc enhancement for an ICS user as specified in TS 23.292 [229].

##### 5.1.3.1.31aA	MSC Address

This field contains the Recommendation E.164 [308] number assigned to the MSC that produced the record. For further details concerning the structure of MSC numbers see TS 23.003 [200].

##### 5.1.3.1.31A	Neighbour Node Address

This field holds the control plane IP address of the neighbouring network contact point that handles the service request in case of interconnection and roaming.

##### 5.1.3.1.31B	NNI Type

This field indicates whether the type of used NNI is non-roaming, roaming with loopback routing, or roaming without loopback routing. The loopback indication is either sent by the S-CSCF in forward direction within the initial SIP request or sent by TRF in backward direction and received by the ATCF, AS and P-CSCF in the final SIP response.

##### 5.1.3.1.31C	Void

##### 5.1.3.1.32	Node Address

This item holds the address of the node providing the information for the CDR. This may either be the IP address or the FQDN of the IMS node generating the accounting data. This parameter corresponds to the Origin-Host AVP.

##### 5.1.3.1.33	Number Portability Routing

This item holds information on number portability routing, received by S-CSCF during ENUM/DNS processes. 
The parameter corresponds to the NumberPortabilityRoutingInformation AVP.

##### 5.1.3.1.33A	Void

##### 5.1.3.1.34	Online Charging Flag

This field indicates the Online Charging Request was sent based on the provided ECF address from the SIP P-header "P-Charging-Function-Addresses". The parameter corresponds to the Online-Charging-Flag AVP.

NOTE: No proof that online charging action has been taken

##### 5.1.3.1.35	Originator

This sub-field of the "List of Message Bodies" indicates the originating party of the message body.

##### 5.1.3.1.35A	Outgoing Session ID

For a SIP session the Session-ID contains the SIP CALL ID as defined in the Session Initiation Protocol RFC 3261 [401]. When the AS acts as B2BUA, the outgoing session is identified by the Outgoing Session ID which contains the SIP CALL ID.

##### 5.1.3.1.36	Private User ID

Holds the used Network Access Identifier of the served party according to RFC 2486 [405]. This parameter corresponds to the User-Name AVP.

##### 5.1.3.1.37	Real Time Tariff Information

This is a field containing the real time tariff information that may be exchanged in the SIP transaction, encoded in a XML body as described in the TS 29.658 [225]. The RTTI information may be captured in the charging information and it is operator configurable as whether it is used in its original XML format or mapped on a detailed structure of parameters. The RTTI information XML schema in XML format is given in the TS 29.658 [225]. The Tariff Information structure of parameters is provided in the TS 32.280 [40].

The Real Time Tariff Information contains one of the following elements:

- Tariff XML;

- Tariff Information.

These field elements are described in the appropriate subclause.

##### 5.1.3.1.38	Record Closure Time

A Time stamp reflecting the time the CCF closed the record.

##### 5.1.3.1.39	Record Extensions

A set of operator/manufacturer specific extensions to the record, conditioned upon existence of an extension.

##### 5.1.3.1.40	Record Opening Time

A time stamp reflecting the time the CCF opened this record. Present only in SIP session related case.

##### 5.1.3.1.41	Record Sequence Number

This field contains a running sequence number employed to link the partial records generated by the CCF for a particular session (characterised with the same Charging ID and GGSN address pair). The Record Sequence Number is not present if the record is the only one produced in the CCF for a session. The Record Sequence Number starts from one (1).

##### 5.1.3.1.42	Record Type

Identifies the type of record. The parameter is derived from the  Node-Functionality AVP, defined in TS 32.299 [40].

##### 5.1.3.1.42A	Related IMS Charging Identifier

This field holds the Related IMS Charging Identifier when the session is the target access leg in an SRVCC handover. The Related IMS Charging Identifier contains the IMS charging identifier generated for the source access leg.

##### 5.1.3.1.42B	Related IMS Charging Identifier Generation Node

This field holds the identifier of the node that generated the Related IMS charging identifier.

##### 5.1.3.1.42A	Relationship Mode

This field indicates whether the other functional entity (e.g. contact point of the neighbouring network) is regarded as part of the same trust domain.

##### 5.1.3.1.43	Requested Party Address

This field holds the address of the party (Public User ID or Public Service ID) to whom the SIP transaction was originally posted. The Requested Party Address shall be populated with the SIP URI or Tel URI (according to RFC 3261 [401] and RFC 3966 [402] respectively) contained in the incoming Request-URI of the request. Requested Party Address could also be populated with an URN (according to RFC5031 [407]) for an emergency SIP session.

This field is only present if different from the Called Party Address parameter.

##### 5.1.3.1.44	Retransmission

This parameter, when present, indicates that information from retransmitted Diameter ACRs has been used in this CDR.

##### 5.1.3.1.45	Role of Node

The field indicates whether the IMS node (except MRFC) is serving the Originating or the Terminating party. The role can be:

-	Originating (IMS node serving the calling party);

-	Terminating (IMS node serving the called party).

##### 5.1.3.1.45A	Route header received

This field contains the information in the topmost route header in a received initial SIP INVITE and non-session related SIP MESSAGE request.

##### 5.1.3.1.45B	Route header transmitted

This field contains the information in the route header representing the destination in a transmitted initial SIP INVITE and non-session SIP MESSAGE request.

##### 5.1.3.1.46	SDP Answer Timestamp

This parameter contains the time of the response to the SDP Offer.

##### 5.1.3.1.47	SDP Media Components

This is a grouped field comprising several sub-fields associated with one media component.  Since several media components may exist for a session in parallel these sub-fields may occur several times (as much times as media are involved in the session). The sub-fields are present if medium (media) is (are) available in the SDP data which is provided in the ACR received from the IMS node.

The SDP media component contains the following elements:

-	SDP media name;

-	SDP media description;

-	GPRS Charging ID;

-	Local GW Inserted indication;

-	IP Realm Default indication;

-	Transcoder Inserted indication.

These field elements are described in the appropriate subclause.

##### 5.1.3.1.48	SDP Media Description:

This field holds the attributes of the media as available in the SDP data tagged with "i=", "c=","b=","k=", "a=". Only the attribute lines relevant for charging are recorded. To be recorded "SDP lines" shall be recorded in separate "SDP Media Description" fields, thus multiple occurrence of this field is possible. Always complete "SDP lines" are recorded per field.

This field corresponds to the SDP-Media-Description AVP.

Example: "c=IN IP4 134.134.157.81"

For further information on SDP please refer to RFC4566 [406].

Note: session unrelated procedures typically do not contain SDP data.

##### 5.1.3.1.49	SDP Media Name

This field holds the name of the media as available in the SDP data tagged with "m=". Always the complete "SDP line" is recorded.

This field corresponds to the SDP-Media-Name AVP.

Example: "m=video 51372 RTP/AVP 31"

For further information on SDP please refer to RFC 4566 [406].

##### 5.1.3.1.50	SDP Offer Timestamp

This parameter contains the time of the SDP Offer.

##### 5.1.3.1.51	SDP Session Description

Holds the Session portion of the SDP data exchanged between the User Agents if available in the SIP transaction.

This field holds the attributes of the media as available in the session related part of the SDP data tagged with "c=" and "a=" (multiple occurrence possible). Only attribute lines relevant for charging are recorded.

The content of this field corresponds to the SDP-Session-Description AVP of the ACR message.

Note: session unrelated procedures typically do not contain SDP data.

##### 5.1.3.1.52	SDP Type

This field identifies if the SDP media component was an SDP offer or an SDP answer.

##### 5.1.3.1.53	Served Party IP Address

This field contains the IP address of either the calling or called party, depending on whether the P-CSCF is in touch with the calling or called network.

##### 5.1.3.1.54	Service Delivery End Time Stamp

This field records the time at which the service delivery was terminated. It is Present only in SIP session related case.

The content of this field corresponds to the SIP-Request-Timestamp AVP of a received ACR[Stop] message indicating a session termination.

##### 5.1.3.1.54A	Service Delivery End Time Stamp Fraction

This parameter contains the milliseconds fraction in relation to Service Delivery End Time Stamp.

##### 5.1.3.1.55	Service Delivery Start Time Stamp

This field holds the time stamp reflecting either:

-	a successful session set-up: this field holds the start time of a service delivery (session related service)

-	a delivery of a session unrelated service: the service delivery time stamp

-	an unsuccessful session set-up and an unsuccessful session unrelated request: this field holds the time the network entity forwards the unsuccessful indication (SIP RESPONSE with error codes 3xx, 4xx, 5xx) towards the requesting User direction.

The content of this field corresponds to the SIP-Response-Timestamp AVP.

For partial CDRs this field remains unchanged.

##### 5.1.3.1.55A	Service Delivery Start Time Stamp Fraction

This parameter contains the milliseconds fraction in relation to Service Delivery Start Time Stamp.

##### 5.1.3.1.56	Service ID

This field identifies the service the MRFC is hosting. For conferences the conference ID is used here.

##### 5.1.3.1.57	Service Reason Return Code

Provides the returned cause code for the service request (both successful and failure). This parameter corresponds to the Cause-Code AVP.

##### 5.1.3.1.58	Service Request Timestamp

This field contains the time stamp which indicates the time at which the service was requested (SIP Request" message) and is present for session related and session unrelated procedures. The content of this item is derived from the SIP-Request-Timestamp AVP. If the SIP-Request-Timestamp AVP is not supplied by the network entity this field is not present.

For partial CDRs this field remains unchanged.

This field is present for unsuccessful service requests if the ACR message includes the SIP-Request-Timestamp AVP.

##### 5.1.3.1.58A	Service Request Timestamp Fraction

This parameter contains the milliseconds fraction in relation to Service Request Timestamp.

##### 5.1.3.1.58B	Session Direction

This field indicates whether the NNI is used for an inbound or outbound service request on the control plane in case of interconnection and roaming.

##### 5.1.3.1.59	Session ID

The Session identification. For a SIP session the Session-ID contains the SIP CALL ID as defined in the Session Initiation Protocol RFC 3261 [401]. When the AS acts as B2BUA, the incoming Session-ID leg is covered.

##### 5.1.3.1.60	Session Priority

This field contains the priority level of the session. The value of the parameter is derived from Resource-Priority header field and the rules for the translation depend on operator policy described in TS 24.229[210].

##### 5.1.3.1.61	SIP Method

Specifies the SIP-method for which the CDR is generated. Only available in session unrelated cases.

##### 5.1.3.1.62	SIP Request Timestamp

This parameter contains the time of the SIP request (usually a (Re)Invite).

##### 5.1.3.1.63	SIP Request Timestamp Fraction

This parameter contains the milliseconds fraction in relation to the SIP Request Timestamp.

##### 5.1.3.1.64	SIP Response Timestamp

This parameter contains the time of the response to the SIP request. If an SDP is exchanged via SIP messages, then this parameter contains appropriately the time of SIP 200 OK acknowledging an SIP INVITE or of SIP ACK including a SDP ANSWER.

##### 5.1.3.1.65	SIP Response Timestamp Fraction

This parameter contains the milliseconds fraction in relation to the SIP Response Timestamp.

##### 5.1.3.1.66	S-CSCF Information

This field contains Information related to the serving CSCF, e.g. the S-CSCF capabilities upon registration event or the S-CSCF address upon the session establishment event. This field is derived from the Server-Capabilities AVP if present in the ACR received from the I-CSCF.

##### 5.1.3.1.66A	Status

Holds the abnormal status information of specific ASs (if any) when AS(s) respond 4xx/5xx or time out to S-CSCF during an IMS session.

##### 5.1.3.1.66B	TAD Identifier

This field indicates the type of access network (CS or PS) through which the session shall be terminated.

##### 5.1.3.1.67	Tariff Information

This field holds the tariff mapped in the Tariff Information structure. The corresponding structure of the Tariff Information can be found in the TS 32.299 [50]. The formatting from real time tariff information to Tariff Information structure is described in TS 32 280 [40].

##### 5.1.3.1.68	Tariff XML

This field holds the tariff formatted in the XML schema as specified in the TS 29.658 [225].

##### 5.1.3.1.68A	Transcoder Inserted Indication

This field indicates if a transcoder is inserted or not for the SDP media component.

##### 5.1.3.1.68B	Transit IOI List

This parameter holds the Transit-IOI List of the P-Charging-Vector header, as recorded in the Transit-IOI-List AVP as defined in TS 32.299 [50]. Multiple occurrences of this field, shall be in chronological order, i.e. the value in the SIP request is listed first. If only a value for the SIP response is available, the Transit IOI List for the SIP request shall be included with the value "unknown". For further information on the Transit IOI exchange via SIP signalling please refer to TS 24.229 [210].

##### 5.1.3.1.69	Trunk Group ID Incoming/Outgoing

Contains the outgoing trunk group ID for an outgoing session/call or the incoming trunk group ID for an incoming session/call.

##### 5.1.3.1.69A	User Location Information

This field contains the User Location Information using PCC mechanisms as specified in TS 23.203 [203] or the location retrieval via Sh interface by AS as specified in TS 29.328 [242].

##### 5.1.3.1.70	VLR Number

This field contains the Recommendation E.164 [308] number assigned to the VLR that produced the record. For further details concerning the structure of VLR numbers see TS 23.003 [200].

### 5.1.4	Service level CDR parameters

#### 5.1.4.1	MMS CDR parameters

##### 5.1.4.1.0	Introduction

This clause contains the description of each field of the MMS CDRs specified in TS 32.270 [30].

##### 5.1.4.1.1	3GPP MMS Version

The MMS version of the originator MMS Relay/Server as defined in TS 23.140 [206].

##### 5.1.4.1.2	Access Correlation

If the parameter is provided and is not an empty string, it is a unique identifier delivered by the used access network domain of the originator or recipient MMS User Agent. It may be used for correlation of the MMS CDRs with the corresponding MSC server CDRs in CS domain or GSN CDRs in PS domain. It is an empty string if the parameter is not delivered by the access network.

##### 5.1.4.1.3	Acknowledgement Request

This Boolean value indicates whether (value TRUE) or not (value FALSE) a response has been requested in a request at the MM4 reference point.

##### 5.1.4.1.4	Attributes List

This field contains a list of information element names that are used in the MM1_mmbox_view.REQ, which request corresponding information elements from the MMs to be conveyed in the MM1_mmbox_view.RES.  The list of known information element names are those currently defined for the MM1_retrieve.RES and MM1_notification.REQ.  In the absence of the Attributes list information element, the MMS Relay/Server shall, by default and if available, select these information elements from each viewed MM: Message ID, Date and time, Sender address, Subject, Message size, MM State, and MM Flags.

##### 5.1.4.1.5	Billing Information

This field contains transparent charging information provided by the MSCF to the MMS R/S for use by the billing system to properly bill the user for the service being supplied as defined in TS 29.140 [218]. Only the format, but not the content of the "Billing information" field is defined.

##### 5.1.4.1.6	Charge Information

This field consists of two parts, the charged party and the charge type.

The Charged Party is an indication on which party is expected to be charged for an MM e.g. the sending, receiving, both parties or neither. This indicator is only applicable to MM7 CDRs (for VASP-originated MMs). It may be provided by the VASP when submitting an MM.

The Charge Type indicates the type of subscription (i.e. postpaid or prepaid). This indicator is derived from the subscription parameters and only applicable to MM1 CDRs.

The Charged Parties are as follows:

-	Sender: This indicates the sending party is expected to be charged ('normal' charging model);

-	Recipient: This indicates the receiving party is expected to be charged ('reverse' charging model). This model implies there is a commercial agreement between the Recipient and the VASP;

-	Both: This indicates both the sending and the receiving parties are expected to be charged ('shared' charging     model);

-	Neither: This indicates neither the sending nor the receiving parties are expected to be charged ('free of charge' charging model).

The Charge types are as follows:

-	Postpaid;

-	Prepaid.

##### 5.1.4.1.7	Content Type

The Content Type of the MM as defined in TS 23.140 [206].

##### 5.1.4.1.8	Delivery Report Requested

This is an indication of type Boolean whether (value TRUE) or not (value FALSE) the originator/forwarding MMS User Agent has requested a delivery report in the MM1_submit.REQ/MM1_forward.REQ.

##### 5.1.4.1.9	Duration of Transmission

This field contains the relevant time in seconds. The Duration of Transmission is the time from the beginning to the end of the MM transfer between the MMS User Agent and the MMS Relay/Server; e.g. for streaming purposes.

Note that the CDRs purposely do not contain any information about the duration of storage on the MMS Relay/Server. If such information is required it can be calculated by post-processing systems from the CDR timestamps. For instance, the total duration of storage on the originator MMS Relay/Server could be calculated by taking the difference between the 'Record Time Stamp' of the O1S-CDR and the 'Record Time Stamp' of the OMD-CDR.

##### 5.1.4.1.10	Earliest Time of Delivery

This field contains either the earliest time to deliver message or the number of seconds to wait before delivering the message.

##### 5.1.4.1.11	Forward Counter

A Counter indicating the number of times the particular MM was forwarded as defined in TS 23.140 [206].

##### 5.1.4.1.12	Forwarding Address

This field contains a forwarding MMS User Agent address. The MMS supports the use of E-Mail addresses (RFC 822 [400]), MSISDN (E.164[308]) or IP addresses.

##### 5.1.4.1.13	Forwarding MMS Relay/Server Address

This field contains one or more addresses of the forwarding MMS Relay/Server. The address is either an IP address or a domain name.

##### 5.1.4.1.14	Limit

This field contains a number that may be provided in the MM1_mmbox_view.REQ to specify a limit for the number of MMs the information elements to which shall be returned in the MM1_mmbox_view.RES.

##### 5.1.4.1.15	Linked ID

This field identifies a correspondence to a previous valid message delivered to the VASP

##### 5.1.4.1.16	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially including all CDR types. The number is unique within one node, which is identified either by field Node ID or by record-dependent MMS Relay/Server.

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.4.1.17	Managing Address

This field contains the managing MMS User Agent address i.e. the MMS User Agent that sends and receives transactions related to the MMBox management . The MMS supports the use of E-Mail addresses (RFC 822) [400], MSISDN (E.164[308]) or IP address.

##### 5.1.4.1.18	Message Class

A class of messages such as personal, advertisement, information service etc. For more information see TS 23.140 [206].

##### 5.1.4.1.19	Message Distribution Indicator

This is an indication of type Boolean whether (value TRUE) or not (value FALSE) the VASP has indicated the content of the MM is intended for redistribution.

##### 5.1.4.1.20	Message ID

This field specifies the MM Message ID of the MM as defined in TS 23.140 [206]. The concrete syntax of this MM Message ID is given by the body of the field introduced by the string "X-Mms-Message-ID:" in the concrete syntax of the message MM4_Forward.REQ. All CDRs pertaining to the same MM must employ the same value of this parameter, i.e. the value initially assigned by the originator MMS Relay/Server upon submission of the MM by the Originator MMS User Agent.

##### 5.1.4.1.21	Message Reference

A reference as specified in TS 23.140 [206], e.g. URI, for the MM that can be used for retrieving the MM from the recipient MMS Relay/Server.

##### 5.1.4.1.22	Message selection

Messages which are to be viewed may be selected by a list of Message References or by a selection based on MM State and/or MM Flags keywords.

##### 5.1.4.1.23	Message Size

This field contains the number of octets of the MM that is calculated as specified in TS 23.140 [206].

##### 5.1.4.1.24	MMBox Storage Information

This field includes following storage information elements for the MMBox containing the MM State, MM Flags, Store Status, Store Status Text and Stored Message Reference.

- MM State;

This field contains the state of the MM.

- MM Flags:

This field contains the keyword flags of the MM.

- Store Status:

This field contains an appropriate status value of the stored MM, e.g. stored, error-transient-mailbox-full,…

- Store Status Text;

This field includes a more detailed technical description of the store status at the point in time when the CDR is generated.

- Stored Message Reference;

A reference of the newly stored MM.

##### 5.1.4.1.25	MM component list

The MM component list is a set of subject and media components from type of media formats including the size of all elements in octets. For a complete description of media formats that may be supported by MMS, refer to IANA [xx].

##### 5.1.4.1.26	MM Date and Time

The date and time field contains the time stamp relevant for the handling of the MM by the recipient MMS Relay/ Server (read, deleted without being read, etc.). The time-stamp includes at a minimum: date, hour, minute and second.

##### 5.1.4.1.27	MM Listing

This field contains a list of information elements from the MMs returned within the MM1_mmbox_view.RES.  The listing shall consist of the following information elements, separately grouped for each MM returned in the list:

-	Message reference: a unique reference to an MM;

-	Information elements corresponding to those requested in the Message Selection information element on the MM1_mmbox_view.REQ.

##### 5.1.4.1.28	MM Status Code

This field contains an appropriate status value of the delivered MM (e.g. retrieved, rejected, etc.).

##### 5.1.4.1.28A	MS Time Zone

This field contains the 'Time Zone' IE provided for the MMS User Agent as specified in TS 29.060 [215].

##### 5.1.4.1.29	MSCF Information

This is a grouped field comprising several the following sub-fields associated with the invocation of the MSCF for advanced addressing:

-	Billing Information;

-	Routeing address List.

These field elements are described in the appropriate subclause.

##### 5.1.4.1.30	Originator Address

This field contains an originator MMS User Agent address. The MMS supports the use of E-Mail addresses (RFC 822 [400]) or MSISDN (E.164 [308]).

##### 5.1.4.1.31	Originator MMS Relay/Server Address

This field contains an address of the originator MMS Relay/Server. This address is composed of a mandatory IP address and/or an optional domain name.

##### 5.1.4.1.32	Priority

The priority (importance) of the message, see TS 23.140 [206].

##### 5.1.4.1.33	Quotas

The quotas of the MMBox in messages and/or octets identified with Messages or Octets

##### 5.1.4.1.34	Quotas requested

This is an indication that the Managing User Agent has requested the current message and/or size quotas.

##### 5.1.4.1.35	Read Reply Requested

A Boolean value indicating whether the originator MMS User Agent has requested a read-reply report (value TRUE) or not (value FALSE).

##### 5.1.4.1.36	Read Status

See TS 23.140 [206]: Status of the MM, e.g. Read, Deleted without being read.

##### 5.1.4.1.37	Recipient Address

This field contains a recipient MMS User Agent address. The MMS supports the use of E-Mail addresses (RFC 822 [400]), MSISDN (E.164 [308]) or Service provider specific addresses (short code).

##### 5.1.4.1.38	Recipient MMS Relay/Server Address

This field contains an address of the recipient MMS Relay/Server. This address is composed of a mandatory IP address and/or an optional domain name.

##### 5.1.4.1.39	Recipients Address List

This field contains a list of recipient MMS User Agent addresses.

##### 5.1.4.1.40	Record Extensions

The field enables network operators and/or manufacturers to add their own extensions to the standard record definitions.

##### 5.1.4.1.41	Record Time Stamp

This field indicates the date and time when the CDR was produced.

##### 5.1.4.1.42	Record Type

The field identifies the type of the record, see TS 32.250 [10].

##### 5.1.4.1.43	Reply Charging

This field indicates whether the originator of the MM is willing to take over the charge for the sending of a reply-MM to their submitted MM from the recipient(s). In this case the originator MMS Relay/Server marks the MM as no charge (reply-charged).

In the Originator MM1 Submission CDR (O1S-CDR) this parameter indicates whether the originator MMS User Agent has requested reply-charging (value TRUE) or not (value FALSE).

In the Recipient MM1 Notification Request record (R1NRq -CDR) it indicates whether a reply to this particular original MM is free of charge (value TRUE) or not (value FALSE).

In the MM7 Submission CDR (7S-CDR) this parameter indicates whether the originator MMS VASP has requested reply-charging (value TRUE) or not (value FALSE).

##### 5.1.4.1.44	Reply Charging ID

This field is present in the CDR only if the MM is a reply-MM to an original MM. The Reply Charging ID is the Message ID of the original MM.

##### 5.1.4.1.45	Reply Charging Size

In the Originator MM1 Submission CDR (O1S-CDR), in case of reply-charging, this field indicates the maximum size for reply-MM(s) granted to the recipient(s) as specified by the originator MMS User Agent.

In the Recipient MM1 Notification Request CDR (R1NRq-CDR), in case of reply-charging, this field indicates the maximum size of a reply-MM granted to the recipient as specified in the MM1_notification.REQ.

In the MM7 Submission CDR (7S-CDR), in case of reply-charging, this field indicates the maximum size for reply-MM(s) granted to the recipient(s) as specified by the originator MMS VASP.

##### 5.1.4.1.46	Reply Deadline

In the Originator MM1 Submission CDR (O1S-CDR), in case of reply-charging, this field indicates the latest time of submission of replies granted to the recipient(s) as specified by the originator MMS User Agent.

In the Recipient MM1 Notification Request CDR (R1NRq-CDR), in case of reply-charging, this field indicates the latest time of submission of a reply granted to the recipient as specified in the MM1_notification.REQ.

In the MM7 Submission CDR (7S-CDR), in case of reply-charging, this field indicates the latest time of submission of replies granted to the recipient(s) as specified by the originator MMS VASP.

##### 5.1.4.1.47	Report allowed

A Boolean value indicating, if present whether sending of a delivery report is permitted (value TRUE) or not (value FALSE).

##### 5.1.4.1.48	Request Status code

The status of the MM as reflected in the corresponding MM4 message (e.g. error service denied, error network problem, error unsupported message, etc.). For further details see TS 23.140 [206].

##### 5.1.4.1.49	Routeing Address

The field contains a recipient address for routeing of a multimedia message. For a complete description of the routeing address, refer to TS 29.140 [218].

##### 5.1.4.1.50	Routeing Address List

This field contains a list of routeing addresses.

##### 5.1.4.1.51	Sender Address

The address of the MMS User Agent as used in the MM1_notification_REQ/MM1_retrieve.RES. This parameter is present in the CDR even if address hiding was requested, resulting in the sender address is not being included in the above messages.

##### 5.1.4.1.52	Sender Visibility

This Boolean value indicates whether the originator MMS User Agent has requested her address to be hidden from the recipient (value TRUE) or not (value FALSE).

##### 5.1.4.1.53	Service code

This field contains charging information provided by the VASP to the MMS R/S for use by the billing system to properly bill the user for the service being supplied. The usage of the "service code" is, in the release, open to any usage envisioned by the operators, service providers or MMS Relay/Server vendors. In this release only the format, but not the content of the "service code" field is defined.

##### 5.1.4.1.54	Start

This field contains a number that may be used in the MM1_mmbox_view.REQ to index the first MM to be viewed, relative to the selected set of MMs, allowing partial views to be requested

##### 5.1.4.1.55	Status Text

This field includes a more detailed technical status of the message at the point in time when the CDR is generated..

##### 5.1.4.1.56	Submission Time

The submission time field contains the time stamps relevant for the submission of the MM. The time-stamp includes a minimum of date, hour, minute and second.

##### 5.1.4.1.57	Time of Expiry

This field contains the desired date or the number of seconds to expiry of the MM, if specified by the originator MMS User Agent.

##### 5.1.4.1.58	Totals

The total number of messages and/or octets for the MMBox, identified with Messages or Octets.

##### 5.1.4.1.59	Totals requested

This is an indication that the Managing User Agent has requested the current total number of messages and/or size contained by the MMBox.

##### 5.1.4.1.60	Upload Time

The upload time field contains the time stamps relevant for the upload of the MM. The time-stamp includes a minimum of date, hour, minute and second.

##### 5.1.4.1.61	VAS ID

This field specifies the identification of the  VASP as defined in TS 23.140 [206].

##### 5.1.4.1.62	VASP ID

This field specifies the identification of the originating application as defined in TS 23.140 [206].

#### 5.1.4.2	LCS CDR parameters

##### 5.1.4.2.0	Introduction

This clause contains the description of each field of the LCS CDRs specified in TS 32.271 [31].

##### 5.1.4.2.1	Home GMLC Identity

This field contains the IP address of the Home GMLC (H-GMLC) involved in the location request.

##### 5.1.4.2.2	LCS Client Identity

This field contains further information on the LCS Client identity as defined in TS 29.002 [214].

##### 5.1.4.2.3	LCS Client Type

This field contains the type of the LCS Client as defined in TS 29.002 [214].

##### 5.1.4.2.4	LCS Priority

This parameter gives the priority of the location request as defined in TS 49.031 [227].

##### 5.1.4.2.5	Location Estimate

The Location Estimate field is providing an estimate of a geographic location of a target MS according to TS 29.002 [214].

##### 5.1.4.2.6	Location Type

This field contains the type of the location as defined in TS 29.002 [214].

##### 5.1.4.2.7	Positioning Data

This information element is providing positioning data associated with a successful or unsuccessful location attempt for a target MS according TS 49.031 [227].

##### 5.1.4.2.8	Provider Error

This parameter is used to indicate a protocol related type of error as defined in TS 29.002 [214].

##### 5.1.4.2.9	Requesting GMLC Identity

This field contains the IP address of the Requesting GMLC (R-GMLC) involved in the location request.

##### 5.1.4.2.10	Result code

This field indicates the result of the request or individual positioning as defined in OMA Mobile Location Protocol [311].

##### 5.1.4.2.11	Target IMSI

This field contains the International Mobile Subscriber Identity (IMSI) of the targeted party. The term "targeted" party is used to describe the mobile subscriber involved in the transaction recorded e.g. the subscriber whose location is requested in case of mobile terminated location request.

The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.4.2.12	Target MSISDN

This field contains the Mobile Station ISDN Number (MSISDN) of the targeted party. The term "targeted" party is used to describe the mobile subscriber involved in the transaction recorded e.g. the subscriber whose location is requested in case of mobile terminated location request.

In case of multi-numbering the MSISDN stored in a LCS CDR will be the primary MSISDN of the requesting party.

The structure of the MSISDN is defined in TS 23.003 [200].

##### 5.1.4.2.13	User Error

This parameter is sent by the responder when the location request has failed or cannot proceed and if present, takes one of the following values defined in TS 29.002 [214]:

-	System Failure;

-	Data Missing;

-	Unexpected Data Value;

-	Facility Not Supported;

-	Unidentified Subscriber;

-	Illegal Subscriber;

-	Illegal Equipment;

-	Absent Subscriber (diagnostic information may also be provided);

-	Unauthorised requesting network;

-	Unauthorised LCS Client with detailed reason;

-	Position method failure with detailed reason.

##### 5.1.4.2.14	Visited GMLC Identity

This field contains the IP address of the Visited GMLC (V-GMLC) involved in the location request.

#### 5.1.4.3	PoC CDR parameters

##### 5.1.4.3.0	Introduction

This clause contains the description of each field of the PoC CDRs specified in TS 32.272 [32].

##### 5.1.4.3.1	Called Party Address

Called Party Address is of type UTF8String. It indicates address (Public User ID, SIP URL, E.164, etc.) of the participants involved in the PoC session.

##### 5.1.4.3.2	Charged Party

This field indicates the party accepting the charge for the session, whether participating in the session or not. The contents are obtained from the Charged-Party AVP in offline charging.

##### 5.1.4.3.3	List of Talk Burst Exchange

This list contains a number of containers consisting of the following fields:

Change Condition

Change Time

Number of participants

Number of received talk bursts

Number of talk bursts

Received talk burst volume

Received talk bursts time

Talk burst volume

Talk bursts time

Number of talk bursts and Number of received talk bursts indicate the number of talk bursts sent and received respectively by the charged party (for the participating PoC functions) or for the whole session (for the controlling PoC function).

Talk burst volume and Received talk burst volume indicate the total data volume for talk bursts sent and received respectively by the charged party (for the participating PoC functions) or for the whole session (for the controlling PoC function).

Talk burst Time and Received talk burst time indicate the total duration of talk bursts sent and received respectively by the charged party (for the participating PoC functions) or for the whole session (for the controlling PoC function).

Change Time is a time stamp, which defines the moment when the container is closed or the CDR is closed.

Change Condition indicates the reason for closing the container and the addition of a new container.

Number of participants indicates the number of attached participants involved in the talk burst exchange within a container.

##### 5.1.4.3.4	Number of participants

For PoC, this field indicates the number of active participants within the PoC session. For MMtel Charging, this field indicates the number of active participants attached in the MMTel conference.

##### 5.1.4.3.5	Participant Access Priority

This field indicates the access priority for each participant involved in the PoC session.

##### 5.1.4.3.6	Participants involved

This field indicates the participants involved in the PoC session.

The field is of type grouped. It contains the participant address (Called party address), the participant access priority and User Participating Type.

##### 5.1.4.3.7	PoC controlling address

This field contains the address of the server performing the controlling PoC function.

##### 5.1.4.3.8	PoC Event Type

This field contains the PoC session unrelated charging event type.

##### 5.1.4.3.9	PoC group name

This field indicates the name of a group used for the PoC session.

##### 5.1.4.3.10	PoC session id

This field uniquely identifies the overall PoC session.

##### 5.1.4.3.11	PoC session initiation type

The field is of type Enumerated. It identifies the type of the PoC session initiation.

The identifier can be one of the following:

- Pre-established

- On-demand

##### 5.1.4.3.12	PoC session type

The field identifies the type of the PoC session.

##### 5.1.4.3.13	User location info

This field contains any available location information for the charged party. The field is coded as per the 3GPP-User-Location-Info RADIUS VSA defined in TS 29.061 [216].

##### 5.1.4.3.14	User Participating Type

Indicates the User Participating Type participating in the PoC session i.e. Normal, NW PoC Box, UE PoC Box.

#### 5.1.4.4	MBMS CDR parameters

##### 5.1.4.4.0	Introduction

This clause contains the description of each field of the MBMS CDRs specified in TS 32.273 [33].

##### 5.1.4.4.1	CN IP Multicast Distribution

This field is used to indicate if IP multicast distribution to UTRAN is used for the MBMS user plane data.

##### 5.1.4.4.2	MBMS 2G 3G Indicator

- The MBMS 2G 3G Indicator is used to indicate the radio access type that can receive the MBMS bearer service.

##### 5.1.4.4.2A	MBMS Data Transfer Start

The field contains the absolute time stamp of the data delivery start. The value indicates the time in seconds for the radio resources set up relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1/2**32 second.

This field is only valid for E-UTRAN access type.

##### 5.1.4.4.2B	MBMS Data Transfer Stop

The field contains the absolute time stamp of the data delivery stop. The value indicates the time in seconds for the release of radio resources relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1/2**32 second.

This field is only valid for E-UTRAN access type.

##### 5.1.4.4.3	MBMS GW Address

This parameter holds the IP-address of the MBMS GW that generated the Charging Id when MBMS GW is stand-alone.

##### 5.1.4.4.4	MBMS Service Area

The field indicates the area over which the MBMS bearer service has to be distributed.

##### 5.1.4.4.5	MBMS Service Type

The field is used to indicate the type of MBMS bearer service: multicast or broadcast.

##### 5.1.4.4.6	MBMS Session Identity

This field together with TMGI identifies a transmission of a specific MBMS session.

##### 5.1.4.4.7	Required MBMS Bearer Capabilities

The field contains the minimum bearer capabilities the UE needs to support.

##### 5.1.4.4.8	TMGI

The field contains the Temporary Mobile Group Identity allocated to a particular MBMS bearer service. TMGI use and structure is specified in  TS 23.003 [200].

#### 5.1.4.5	MMTel CDR parameters

##### 5.1.4.5.0	Introduction

This subclause contains the description of each of the CDR fields needed to support the charging of MMTel services as specified in TS 32.275 [35].

##### 5.1.4.5.1	Associated Party Address

This field holds the address (SIP URI or Tel URI) of the user, for MMTel supplementary service this field is used for :   the "forwarding party" for CDIV, the "transferor" for ECT, the "Pilot Identity" for FA and the "Initiator party" for 3PTY, as specified in TS 32.275 [35]. The content is obtained from the Associated-Party-Address AVP.

##### 5.1.4.5.2	List of Supplementary services

This list includes several MMTel Supplementary services. Each Supplementary Service may  contain  the following fields as specified in TS 32.275 [35] :

-	Service Type;

-	Service Mode;

-	Number Of Diversions;

-	Associated Party Address;

-	Service ID;

-	Change Time;

-	Number Of Participants;

-	Participant Action Type;

-	AoC information.

Service Type  is defined in clause 5.1.4.5.6

Service Mode is defined in clause 5.1.4.5.5

Number Of Diversions  is defined in clause 5.1.4.5.3

Associated Party Address is defined in clause 5.1.4.5.1

Service ID is an identifier of the conference.

Change Time is a time stamp, which defines the moment when the conference participant has an action (e.g. creating the conference, joining in the conference, being invited into the conference or quiting the conference) triggering the Accounting Request message to CDF in MMTel Charging.

Number Of Participants indicates the number of attached participants involved in the conference.

Participant Action Type indicates the participant's action type during the conference. It is just for Billing Domain's information in each CDR, e.g. creating the conference, joining in the conference, being invited into the conference and quiting the conference.CUG Information indicates the "CUG interlock code" used during the "Closed User Group" communication.

AoC information is defined in clause 5.1.3.1.3A.

##### 5.1.4.5.3 	Number Of Diversions

This field identifies the number of diversions related to a CDIV service as defined in TS 32.275 [35] and TS 24.604 [211]. When counting the number of diversions, all types of diversion are included.

##### 5.1.4.5.4 	Participant Action Type

This field indicates the participant's action type during the conference. The content is obtained from the Participants-Action-Type AVP in TS 32.299 [50].

##### 5.1.4.5.5	Service Mode

This field of Supplementary service indicates the mode for MMTel supplementary services (e.g  CDIV, CB and ECT). The content is obtained from the Service-Mode AVP and described in TS 32.299 [50].

Service Mode values   1024 are reserved for specific Network/Manufacturer variants.

##### 5.1.4.5.6 	Service Type

This field identifies the MMTel supplementary service type as defined in TS 32.275 [35]. . The content is obtained from the MMTel-SService-Type AVP and described in TS 32.299 [50].

Service Type values   1024 are reserved for specific Network/Manufacturer variants

##### 5.1.4.5.7	Void

#### 5.1.4.6	SMS CDR parameters

##### 5.1.4.6.0	Introduction

This clause contains the description of each field of the SMS CDRs specified in TS 32.274 [34].

##### 5.1.4.6.1	Event Timestamp

This field contains the timestamp of the event that triggered the generation of charging information for the SMS transaction.

##### 5.1.4.6.1A	External Identifier

This field contains theE xternalI dentifier of the UE, which identifies a subscription associated to an IMSI, as specified in TS 23.003 [200].

##### 5.1.4.6.2	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially The number is allocated sequentially including all CDR types. The number is unique within the CDF.

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.4.6.3	Message Class

This field contains a class of messages such as personal, advertisement, information service. For more information see TS 23.140 [206].

##### 5.1.4.6.4	Message Reference

This field contains the identity used to identify a Short Message in the SMS node associated with entity that submitted it, and corresponds to the TP-Message-Reference (TP-MR) as defined in TS 23.040 [201].

##### 5.1.4.6.5	Message Size

This field contains the length of the user data part of the Short Message, corresponding to the TP-User-Data-Length (TP-UDL) as defined in TS 23.040 [201].

##### 5.1.4.6.6	MTC IWF Address

This field contains the MTC IWF address.

##### 5.1.4.6.7	Originator IMSI

This field contains IMSI of the originator of the Short Message. The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.4.6.8	Originator Info

This field contains a set of information on the originator of the Short Message, and includes following elements:

-	Originator IMSI

-	Originator MSISDN

-	Originator Other Address

-	Originator SCCP Address

-	Originator Received Address

-	SM Originator Interface

-	SM Originator Protocol Id

These fields are described in the appropriate subclause.

##### 5.1.4.6.9	Originator MSISDN

This field contains MSISDN (E.164 number [308]) of the originator of the Short Message.

##### 5.1.4.6.10	Originator Other Address

This field contains the addresse of an originator of the Short Message other than IMSI and MSISDN: e.g short code, email.

##### 5.1.4.6.11	Originator Received Address

This field contains the original address of the originator of the Short Message, as received by the SMS node.

##### 5.1.4.6.12	Originator SCCP Address

This field contains the SCCP calling address used to receive the Short Message at the SMS node.

##### 5.1.4.6.13	RAT Type

This field contains the Radio Access Technology (RAT) type used for the SMS transaction.

##### 5.1.4.6.14	Recipient IMSI

This field contains IMSI of aRecipient of the Short Message. The structure of the IMSI is defined in TS 23.003 [200].

##### 5.1.4.6.15	Recipient Info

This field contains a set of information on a Recipient of the Short Message, and includes following elements:

-	Recipient IMSI

-	Recipient MSISDN

-	Recipient Other Address

-	Recipient Received Address

-	Recipient SCCP Address

-	SM Destination Interface

-	SM Recipient Protocol Id

These fields are described in the appropriate subclause.

##### 5.1.4.6.16	Recipient MSISDN

This field contains MSISDN (E.164 number [308]) of a Recipient of the Short Message.

##### 5.1.4.6.17	Recipient Other Address

This field contains the addresse of a Recipient of the Short Message other than IMSI and MSISDN: e.g short code, email....

##### 5.1.4.6.18	Recipient Received Address

This field contains the original address of the originator of the Short Message, as received by the SMS node.

##### 5.1.4.6.19	Recipient SCCP Address

This field contains the SCCP called address used by the SMS node to onward deliver the Short Message.

##### 5.1.4.6.20	Record Type

The field identifies the type of the record, see TS 32.250 [10].

##### 5.1.4.6.21	Record Extensions

The field enables network operators and/or manufacturers to add their own extensions to the standard record definitions.

##### 5.1.4.6.22	Served IMEI

This fields contains the international mobile equipment identity (IMEI) or IMEISV of the equipment served. The term "served" equipment is used to describe the UE involved in the SMS transaction recorded.

The structure of the IMEI, IMEISV is specified in TS 23.003 [200].

##### 5.1.4.6.23	SM Data Coding Scheme

This field contains the data coding scheme used within the Short Message, and corresponds to TP-DCS header.

##### 5.1.4.6.24	SM Delivery Report Requested

This field contains an indication whether a delivery report is requested by the Short Message originator.

##### 5.1.4.6.25	SM Destination Interface

This field contains the information describing the interface on which the Short Message is to be delivered by the SMS node.

##### 5.1.4.6.26	SM Device Trigger Indicator

This field contains  indication whether the Short Message submission/delivery to/from  SMS-SC is related  to Device Trigger, and which Device trigger action is requested: request, replace or recall.

##### 5.1.4.6.27	SM Device Trigger information

This field contains the set of information related to SMS submission to SMS-SC for Device Trigger, and includes following elements:

-	MTC IWF Address

-	SM DT Reference Number

-	SM Serving Node

-	SM DT Validity Period

-	SM DT Priority Indication

-	SMS Application Port ID

These fields are described in the appropriate subclause.

##### 5.1.4.6.28	SM Discharge Time

This field contains the time associated with the event being reported in the Short Message Status field as defined in TS 23.040 [201]. This information is only applicable to delivery report charging procedures

##### 5.1.4.6.29	SM DT Priority Indication

This field holds the priority of the device trigger request received via T4 reference point, as specified in TS 29.337 [231].

##### 5.1.4.6.30	SM DT Reference Number

This field contains the Reference Number related to the device trigger request received via T4 reference point, as specified in TS 29.337 [231].

##### 5.1.4.6.31	SM DT Validity Period

This field contains the validity period of the device trigger request received via T4 reference point, as specified in TS 29.337 [231].

##### 5.1.4.6.32	SM Message Type

This field contains the message type that triggered the generation of charging information: submission, delivery report, SM Service Request, T4 Device Trigger, or SM Device Trigger, or MO-SMS T4 submission.

##### 5.1.4.6.33	SM Originator Interface

This field contains the information describing the interface on which the Short Message was received by the SMS node

##### 5.1.4.6.34	SM Originator Protocol Id

This field contains the TP-PROTOCOL-ID (TP-PID) as defined in TS 23.040 [201] describing the protocol used for the Short Message by originator.

##### 5.1.4.6.35	SM Priority

This field contains any priority information associated with a Short Message, as defined in TS 23.040 [201].

##### 5.1.4.6.36	SM Recipient Protocol Id

This field contains the TP-PROTOCOL-ID (TP-PID) as defined in TS 23.040 [201], describing the protocol used for the Short Message to the recipient.

##### 5.1.4.6.37	SM Reply Path Requested

This field contains an indication of whether a reply Short Message to an original Short Message shall follow the same path and corresponds to the TP-Reply-Path (TP-RP) flag.

##### 5.1.4.6.38	SMS Application Port ID

This field holds the Application Port ID of  triggering application for the device trigger request received via T4 reference point, as specified in TS 29.337 [231] , or the Application port ID associated with the UE on MO delivery to the SCS.

##### 5.1.4.6.39	SM Sequence Number

This field contains the sequence number of the SMS within the concatenated short message when part of concatenated short message.

##### 5.1.4.6.40	SM Serving Node

This field contains  the serving node identity, i.e. SGSN/MME/MSC identity serving the UE, received from MTC-IWF via T4 reference point for device trigger, as specified in TS 29.337 [231].

##### 5.1.4.6.41	Void

##### 5.1.4.6.42	Void

##### 5.1.4.6.43	SM Status

This field contains the information from the TP-Status field in a Status-Report TPDU as defined in TS 23.040 [201]. This information is only applicable to delivery report charging procedures.

##### 5.1.4.6.44	SM Total Number

This field contains the total number of short messages when the SMS is part of concatenated short message.

##### 5.1.4.6.45	SM User Data Header

This field contains the user data header extracted from the user data of the SM, corresponding to the user data header (TP-UDH) is specified in TS 23.040 [201].

##### 5.1.4.6.45A	SMS Node Address

This field contains the Address of the SMS Node that produced the record: assigned E.164 number.

##### 5.1.4.6.45B	SMS Result

The field contains the result of an attempt for a Short Message transaction (submission or delivery) at SMS Service Center, when unsuccessful.

##### 5.1.4.6.46	Submission Time

This field contains the timestamp of when the submitted Short Message arrived at the originating SMS Node, obtained from the TP-Service-Center-Time-Stamp (TP-SCTS) as defined in TS 23.040 [201].

##### 5.1.4.6.47	UE Time Zone

This field contains the “Time zone”  as specified in TS 29.060 [215], provided for the Mobile User during the SMS transaction.

##### 5.1.4.6.48	User Location Info

This field contains the information about the location of the subscriber during the SMS transaction. (3GPP user location...).

#### 5.1.4.7	ProSe CDR parameters

##### 5.1.4.7.0	Introduction

This clause contains the description of each field of the ProSe CDRs specified in TS 32.277 [37].

##### 5.1.4.7.0A	Announcing PLMN ID

This field contains PLMN identity of the serving PLMN which signalled the carrier frequency, when this serving PLMN is not the HPLMN or VPLMN, if available.

##### 5.1.4.7.1	Announcing UE HPLMN Identifier

This field contains identifier of Annoucing UE HPLMN (MCC and MNC).

##### 5.1.4.7.2	Announcing UE VPLMN Identifier

This field contains PLMN identity (MCC and MNC) of VPLMN for announcing UE. This field correspondes to Monitored PLMN ID in match report request, as defined in TS 23.303[235] clause 5.3.4. In this case it’s the same with Announcing UE HPLMN Identifier when non-roaming.

##### 5.1.4.7.3	Application ID

This field carries a globally unique identifier identifying a specific 3rd party application, as upper layer of ProSe.

##### 5.1.4.7.4	Cause for Record Closing

This field contains a reason for the release of the CDR. In case of Rf interface is used, it is derived from Change-Condition AVP at ProSe-information AVP level defined in TS 32.299 [50], when received. The following is included:-	proximity alerted: It corresponds to "Proximity Alerted" in Change-Condition AVP.

-	time expired with norenewal: It corresponds to "Time Expired With No Renewal" in Change-Condition AVP.

-	requestor cancellation: It corresponds to "Requestor Cancellation" in Change-Condition AVP.

-	time limited: It corresponds to "Time Limit" in Change-Condition AVP.

-	maximum number of reports: It corresponds to "Max Number of reports" in Change-Condition AVP.

-	abnormal release: It corresponds to "Abnormal Release" in Change-Condition AVP.

##### 5.1.4.7.5	Direct Discovery Model

This field indicates model of the Direct Discovery used by the UE, i.e. Model A, Model B.

##### 5.1.4.7.5A	Discoveree UE HPLMN Identifier

This field contains identifier Discoveree of Discoveree UE HPLMN.

##### 5.1.4.7.5B	Discoveree UE VPLMN Identifier

This field contains identifier of Discoveree UE VPLMN.

##### 5.1.4.7.5C	Discoverer UE HPLMN Identifier

This field contains identifier of Discoverer UE HPLMN.

##### 5.1.4.7.5D 	Discoverer UE VPLMN Identifier

This field contains identifier of Discoverer UE VPLMN.

##### 5.1.4.7.6	Layer two Group ID

This field contains the identifier of a ProSe communication group, uniquely represents a specific one to-many ProSe Direct Communication and is included in CDRs for each participantes in the specific group.

##### 5.1.4.7.6A	List of Application Specific Data

This field contains a list of data blocks provided by the application in the UE. The content of each block is application-specific.

##### 5.1.4.7.6B	List of Coverage Info

This field contains a list of coverage status changes with time stamps. When in coverage, additionally includes list of location changes (i.e., ECGI change) and time stamps.

##### 5.1.4.7.6C	List of Radio Parameter Sets

This field contains a list of radio parameter sets configured in the UE for direct communication use. Each set has an associated time stamp of when it became active.

##### 5.1.4.7.7	List of Reception Data Containers and List of Transmission Data Containers

The same structure is used to convey both the List of Reception Data Containers and the List of Transmission Data Containers. Each list includes a list of changes in trigger conditions (e.g. change of PLMN, go out of coverage, come back to coverage, etc.) for a specific Communication. Each change is time stamped. Trigger condition is used to categorize received or transmitted data volumes, respectively, such as per coverage status duration. Each Direct Communication data containers may include the following fields:

-	Local Sequence Number

-	Change Time.

-	Coverage status.

-	UE Location.

-	Data Volume (transmitted or received).

-	Change Condition.

-	VPLMN Identifier.

-	Usage Information Report Sequence Number.

-	Radio Resources Indicator.

-	Radio Frequency.

Local Sequence Number is a service data container sequence number. It starts from 1 and is increased by 1 for each service date container generated within the lifetime of this direct communication.

Change Time includes the time when the container is closed and reported due to ProSe charging condition change..

Coverage status indicates whether UE is served by E-UTRAN or not, i.e. enter coverage, leave coverage.

UE Location	contains the location information of the UE, i.e. ECGI

Data Volume is the amount of data received or transmitted by UE.

Change Condition contains the reason for closing the container, e.g. change of PLMN, go out of coverage, come back to coverage.

VPLMN Identifier contains the indentifier of PLMN (MCC and MNC) that the UE visits.

Usage Information Report Sequence Number contains the sequence number of usage information report, which is used to generate the container.

Radio Resource Indicator identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direction communication.

Radio Frequency identifies the radio frequency used for ProSe direct communication.

##### 5.1.4.7.7A	List of Transmitters

This field contains a list of transmitters detected for the group. The information stored consists of the source IP address and the ProSe UE ID for each transmitter.

##### 5.1.4.7.8	Monitored PLMN Identifier

This field carries Monitored PLMN ID (MCC and MNC) in Match Report request, as defined in TS 23.303[235] clause 5.3.4. It corresponds to the Announcing UE VPLMN Identifier when roaming and Announcing UE HPLMN Identifier when non-roaming.

##### 5.1.4.7.9	Monitoring UE PLMN Identifier

This field contains identifier of monitoring UE PLMN (MCC and MNC).

##### 5.1.4.7.10	Monitoring UE Identifier

This field carries identifier of the party who initiate Monitor/Match report, i.e. IMSI, which corresponds to UE Identifier parameter in Monitor/Match report request, as defined in TS 23.303[235].

##### 5.1.4.7.11	Monitoring UE VPLMN Identifier

This field contains identifier of Monitoring UE VPLMN (MCC and MNC).

##### 5.1.2.7.12	Node ID

This field contains an optional, operator configurable, identifier string for the node that had generated the CDR. 
The Node ID may or may not be the DNS host name of the node.

##### 5.1.4.7.13	PC Three Control Protocol Cause

This field holds the particular reason why a DISCOVERY_REQUEST or Match_Report messages from the UE have been rejected by the ProSe Function.

##### 5.1.4.7.14	PC Three EPC Control Protocol Cause

This field holds the particular reason why a proximity request messages from the UE have been rejected by the ProSe Function.

##### 5.1.4.7.15	ProSe Application ID

This field carries an identity used for ProSe direct discovery, identifying application related information for the ProSe-enabled UE.

##### 5.1.4.7.16	ProSe Event Type

This field indicates ProSe  charging event, i.e. open Annoucing, open Monitoring, open Match Report, restricted Announcing, restricted Monitoring, restricted Match Report, restricted Discovery Request and restricted Discovery Reporting.

##### 5.1.4.7.17	ProSe Function ID

This field holds the FQDN that identifies a ProSe Function.

##### 5.1.4.7.18	ProSe Function IP Address

This field holds the IP-address of the ProSe Function.

##### 5.1.4.7.19	ProSe Function PLMN Identifier

This field holds the PLMN Identifier (MCC and MNC) of the ProSe Function.

##### 5.1.4.7.20	ProSe Group IP multicast address

This field holds The IP multicast address to be used for performing ProSe Direct Communication.

##### 5.1.4.7.21	ProSe Reason for Cancellation

This field contains a reason for proximity request session is canceled. In case of Rf interface is used, it is derived from ProSe-Reason-For-Cancellation AVP at ProSe-Information AVP level defined in TS 32.299 [50], when received. The following is included:

-	Proximity alerted: When ProSe Function determines that two UEs are in proximity, a PROXIMITY_ALERT message is sent to UE. It corresponds to "Proximity alerted" in ProSe-Reason-For-Cancellation AVP.

-	Time expired with no renewal: Allowed time windows is expired and no renewal request is received from UE. It corresponds to "Time expired with no renewal" in ProSe-Reason-For-Cancellation AVP.

-	Requestor cancellation: a CANCEL_PROXIMITY_REQUEST messag is received from UE. It corresponds to "Requestor cancellation" in ProSe-Reason-For-Cancellation AVP.

##### 5.1.4.7.22	ProSe Request Timestamp

This field holds the timestamp when ProSe Request is received from UE.

##### 5.1.4.7.22A	ProSe Target Layer-2 ID

This field carries the identifier of UE, uniquely represents a specific one-to-one ProSe Direct Communication.

##### 5.1.4.7.23	ProSe UE ID

This field carries a link layer identifier assigned by the EPS that uniquely represents the UE in the context of ProSe Direct Communication.

##### 5.1.4.7. 23A 	ProSe UE-to-Network Relay UE ID

The fiels holds a link layer identifier that uniquely represents the ProSe UE-to-Network relay UE in the context of ProSe Direct Communication via UE-to-Network.

##### 5.1.4.7.24	Proximity Alert Indication

This field indicates whether proximity alert has been sent before proximity request cancellation.

##### 5.1.4.7.25	Proximity Alert Timestamp

This field holds the timestamp when proximity alert is sent, to indicate two UEs are in proximity.

##### 5.1.4.7.26	Proximity Cancellation Timestamp

This field holds the timestamp when proximity request cancellation is requested.

##### 5.1.4.7.27	Proximity Request Renewal Info Block List

This field holds a list of information blocks that are added by each of the  Proximity Request renewal messages captured in the CDR. The information block contains information of the renewal request, e.g. timestamp, time window, range class, and UE location. Each Proximity Request Renewal Info Block may include the following fields:

-	ProSe Request Timestamp

-	Time Window

-	Range Class

-	UE Location

ProSe Request Timestamp is the time when ProSe Renewal Request is received from UE.

Time Window is the time interval in minutes during which a proximity renewal request is valid.

Range Class is the range class for a specific proximity renewal request.

UE Location the UE location with the best known accuracy (e.g. Cell ID or geo-location coordinates) at the time fo the renewal request.

##### 5.1.4.7.28	Range Class

This field carries a range class for a specific proximity request, e.g. 50 m, 100 m, 200 m, 500 m, 1000 m, which as "Range Class" defined in TS 24.334 [236].

##### 5.1.4.7.29	Reason for Cancellation

This field indicates the reason for cancellation of an EPC-level Discovery request, i.e. Proximity alerted, Time expired with no renewal, Requestor cancellation.

##### 5.1.4.7.30	Record Type

The field identifies the type of the record i.e. PF-DD-CDR, PF-ED-CDR and PF-DC-CDR.

##### 5.1.4.7.30A	Relay IP address

The field carries the IP address used as ProSe UE-to-Network Relay UE address for performing ProSe Direct Communication via UE-to-Network.

##### 5.1.4.7.31	Requested Application Layer User ID

This field carrys the user identifier designated in 3rd party application for the user who is targeted in proximity request.

##### 5.1.4.7.32	Requested PLMN Identifier

This field contains PLMN identifier (MCC and MNC) of the user who is targeted in proximity request.

##### 5.1.4.7.33	Requestor Application Layer User ID

This field carrys the user identifier designated in 3rd party application for the user who initiate EPC-level ProSe discovery request.

##### 5.1.4.7.34	Requestor EPC ProSe User ID

This field carrys the identifier generated in ProSe Function for UE who initiate EPC-level ProSe Discovery request.

##### 5.1.4.7.35	Requestor PLMN Identifier

This field contains PLMN identifier (MCC and MNC) of the user who initiate proximity request.

##### 5.1.4.7.36	Role Of ProSe Function

This field indicates ProSe Function resides in which PLMN, i.e. HPLMN, VPLMN, Local PLMN.

##### 5.1.4.7.37	Role Of UE

This field indicates role of the UE using ProSe served by the ProSe Function who generates the CDR, e.g. Announcing UE, Monitoring UE.

##### 5.1.4.7.38	Source IP address

This field holds the IP address UE used as source address for performing ProSe Direct Communication.

##### 5.1.4.7.38a	Target IP address

The field holds the IP address used as target address for performing ProSe Direct one-to-one Communication.

##### 5.1.4.7.38A	Time of First Reception

This field contains the time when collection of reception data is started for the group in this CDR, i.e., the first one-to-many direct communication reception started.

##### 5.1.4.7.38B	Time of First Transmission

This field contains the time when collection of transmitted data is started for the group in this CDR, i.e., the first one-to-many direct communication transmission started.

##### 5.1.4.7.39	Time Window

This field specify a time interval in minutes during which a proximity request is valid. The Time Window is in the range of 1 – 1440 minutes.

##### 5.1.4.7.40	UE Location

This field carries the UE location with the best known accuracy (e.g. Cell ID or geo-location coordinates). The UE Location is set to the cell identity part of the Evolved Cell Global Identifier and obtained from the lower layers of the UE. The value of UE Location is with fixed length of 28 bits.

##### 5.1.4.7.41	Validity Period

This field holds the time interval duration in minutes during which user is authorized for using ProSe Direct Discovery functionality (e.g. Announcing, Monitoring, Match reporting).

##### 5.1.4.7.42	WLAN Link Layer ID

This field carrys WLAN link layer identifier.

#### 5.1.4.8	Monitoring Event CDR parameters

##### 5.1.4.8.0	Introduction

This clause contains the description of each field of the Monitoring Event CDRs specified in TS 32.278 [38].

##### 5.1.4.8.1	Accuracy

This field contains desired level of accuracy of the requested location information and is applicable to the "Location Reporting" Monitoring Event type. Accuracy could be at cell level (CGI/ECGI), eNB, TA/RA level.

##### 5.1.4.8.2	Chargeable Party Identifier

This field identifies the entity towards which accounting/charging functionality is performed by the involved 3GPP network elements.

##### 5.1.4.8.3	Event Timestamp

This field contains the timestamp of the event that triggered the generation of charging information for the Monitoring Event action.

##### 5.1.4.8.4	List of Locations

This field identifies the list of cells, eNBs and/or RAI(s)/TAI(s) for determination of the number of UEs in the area and is applicable to the "Number of UEs present in a geographic area" Monitoring Event type.

##### 5.1.4.8.5	List of Monitoring Event Report Data

This list includes charging information for one or more Monitoring Event reports. Each Monitoring Event Report Data container may include the following fields:

-	Event Timestamp

-	SCEF Reference ID

-	SCEF Id

-	Monitoring Event Report Number

-	Chargeable Party Identifier

-	Monitored User

-	Monitoring Type

-	Reachability Information

-	Reported Location

-	Communication Failure Information

-	List of Number Of UE Per Location Reports

Event Timestamp is a time stamp, which defines the moment when the event triggered the generation of charging information for the Monitoring Event report.

SCEF Reference ID is the identifier created by the SCEF, to identify a Monitoring Request. When combined with the SCEF Id, serves as a globally unique identifier for the Monitoring Request.

SCEF Id is the identifier of the SCEF to which the Monitoring Event Report message was sent.

Monitoring Event Report Number contains the number of the report being sent for the specific request from this node. The number is monotonically increasing for each report starting at 1 for each unique request.

Chargeable Party Identifier identifies the entity towards which accounting/charging functionality is performed by the involved 3GPP network elements.

Monitored User identifies the user that is monitored and is applicable to the "Loss of connectivity", "UE reachability", "Location Reporting", "Communication Failure" and "Availability after DDN Failure" monitoring event types.

Monitoring Type identifies the specific Monitoring Event being reporting.

Reachability Information identifies the reachability status of the UE and is applicable to the "UE reachability" Monitoring Event type.

Reported Location indicates the reported 3GPP system specific location information and is applicable to the "Location Reporting" Monitoring Event type.

Communication Failure Information indicates the reported the reason for communication failure and is applicable to the "Communication Failure" Monitoring Event type.

List of Number of UE per Location Reports contains a list of the location information along with the number of UEs found at that location by the MME/SGSN. It is applicable to the "the number of UEs at a given geographic location" Monitoring Event type.

##### 5.1.4.8.6	Local Record Sequence Number

This field includes a unique record number created by this node. The number is allocated sequentially for each CDR including all CDR types. The number is unique within one node, which is identified either by field Node ID or by record-dependent node address.

The field can be used e.g. to identify missing records in post processing system.

##### 5.1.4.8.7	Location Type

This field identifies whether the request is for Current Location or Last Known Location and is applicable to the "Location Reporting" and "Number of UEs present in a geographic area" Monitoring Event type.

##### 5.1.4.8.8	Maximum Detection Time

This field identifies the maximum period of time without any communication with the UE after which the SCEF is to be informed that the UE is considered to be unreachable and is applicable to the "Loss of connectivity" Monitoring Event type. The value is on the order of 1 minute to multiple hours.

##### 5.1.4.8.9	Maximum Number of Reports

This field identifies the maximum number of event reports to be generated until the associated Monitoring Event is considered to expire. A value of one implies a single event report is to be generated which makes it equivalent to a One-time Monitoring Request. This parameter is not applicable to the "Availability after DDN Failure" Monitoring Event type.

##### 5.1.4.8.10	Monitored User

This field identifies the user that is monitored and is applicable to the "Loss of connectivity", "UE reachability", "Location Reporting", "Communication Failure" and "Availability after DDN Failure" Monitoring Event types.

##### 5.1.4.8.11	Monitoring Duration

This field identifies the absolute time at which the related Monitoring Event request is considered to expire.

##### 5.1.4.8.12	Monitoring Event Config Status

This field identifies whether the request was successful or not. When the request is not successful, a specific value is chosen to indicate the error occurred during handling of the Requested action for the Monitoring event.

##### 5.1.4.8.13	Monitoring Event Configuration Activity

This field indicates Monitoring Event Configuration Activity, i.e. create, transfer, update, and delete.

##### 5.1.4.8.14	Monitoring Type

This field identifies the specific Monitoring Event being requested, which can have the following values:

-	Loss of connectivity.

-	UE reachability.

-	Location Reporting.

-	Communication Failure.

-	Availability after DDN Failure

-	Number of UEs present in a geographic area.

##### 5.1.4.8.15 	Node ID

This field contains an optional, operator configurable, identifier string for the node that had generated the CDR. 
The Node ID may or may not be the DNS host name of the node.

##### 5.1.4.8.16	Reachability Configuration

This field contains the details for configuration for UE reachability, including reachability type, maximum latency and maximum response time.

##### 5.1.2.8.17	Record Opening Time

A time stamp reflecting the time the CDF opened this record.

##### 5.1.2.8.18	Record Type

The field identifies the type of the record i.e. ME-CO-CDR and ME-RE-CDR.

##### 5.1.2.8.19		Retransmission

This parameter, when present, indicates that information from retransmitted Accounting Requests have been used in this CDR.

##### 5.1.4.8.20	SCEF ID

This field contains identifier of the SCEF to which the Monitoring Event Report message was sent.

##### 5.1.4.8.21	SCEF Reference ID

When combined with the SCEF ID, this field serves as a globally unique identifier for the Monitoring Event Request.

## 5.2	CDR abstract syntax specification

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

.$GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

ISDN-AddressString,

LCSClientExternalID,

LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic,

UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17)  version15 (15)}

-- from TS 29.002 [214]

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) modules (0) protocol (3)}

-- from Rec. X.2ab[304]

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

-- from Rec. X.721 [305]

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) modules (0) apdus (0) version1 (1) };

-- From Rec. X.2cd[306].  Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227 / ISO 8650 corrigendum and not "ANY"

--

--  Generic Data Types

--

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

-- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

CalledNumber			::= BCDDirectoryNumber

CallingNumber	::= BCDDirectoryNumber

CellId	::= OCTET STRING (SIZE(2))

--

-- Coded according to TS 24.008 [208]

--

ChargeIndicator			::= INTEGER

{

noCharge			(0),

charge				(1)

}

CauseForRecClosing	::= INTEGER

--

-- Cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- sWGChange value is used in both the S-GW, TWAG and ePDG for inter serving node change

--

{

normalRelease					(0),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

sGWChange						(25),

aPNAMBRChange					(26),

mOExceptionDataCounterReceipt	(27),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

CauseForTerm			::= INTEGER

--

-- Cause codes from 16 up to 31 are defined as 'CauseForRecClosing'

-- (cause for record closing).

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214].

--

{

normalRelease						(0),

partialRecord						(1),

partialRecordCallReestablishment	(2),

unsuccessfulCallAttempt				(3),

abnormalRelease						(4),

cAMELInitCallRelease				(5),

unauthorizedRequestingNetwork		(52),

unauthorizedLCSClient				(53),

positionMethodFailure				(54),

unknownOrUnreachableLCSClient		(58)

}

CNIPMulticastDistribution		::= ENUMERATED

{

nO-IP-MULTICAST				(0),

iP-MULTICAST				(1)

}

Diagnostics						::= CHOICE

{

gsm0408Cause								[0] INTEGER,

-- See TS 24.008 [208]

gsm0902MapErrorValue						[1] INTEGER,

--

-- Note: The value to be stored here corresponds to the local values defined in the MAP-Errors

-- and MAP-DialogueInformation modules, for full details see TS 29.002 [214].

--

itu-tQ767Cause							[2] INTEGER,

-- See Q.767 [309]

networkSpecificCause						[3] ManagementExtension,

-- To be defined by network operator

manufacturerSpecificCause				[4] ManagementExtension,

-- To be defined by manufacturer

positionMethodFailureCause				[5] PositionMethodFailure-Diagnostic,

-- see TS 29.002 [214]

unauthorizedLCSClientCause				[6] UnauthorizedLCSClient-Diagnostic,

-- see TS 29.002 [214]

diameterResultCodeAndExperimentalResult	[7] INTEGER

-- See TS 29.338 [230], TS 29.337 [231], TS 29.128 [244]

}

DiameterIdentity		::= OCTET STRING

EnhancedDiagnostics					::= SEQUENCE

{

rANNASCause						[0] OCTET STRING

-- TS 29.274 [223]

}

GSNAddress	::= IPAddress

IPAddress	::= CHOICE

{

iPBinaryAddress	 IPBinaryAddress,

iPTextRepresentedAddress	 IPTextRepresentedAddress

}

IPBinaryAddress	::= CHOICE

{

iPBinV4Address			[0] IPBinV4Address,

iPBinV6Address			    IPBinV6AddressWithOrWithoutPrefixLength

}

IPBinV4Address	::= OCTET STRING (SIZE(4))

IPBinV6Address	::= OCTET STRING (SIZE(16))

IPBinV6AddressWithOrWithoutPrefixLength ::= CHOICE

{

iPBinV6Address             [1] IPBinV6Address,

iPBinV6AddressWithPrefix   [4] IPBinV6AddressWithPrefixLength

}

IPBinV6AddressWithPrefixLength ::= SEQUENCE

{

iPBinV6Address                    IPBinV6Address,

pDPAddressPrefixLength            PDPAddressPrefixLength DEFAULT 64

}

IPTextRepresentedAddress	::= CHOICE

{	--

-- IP address in the familiar "dot" notation

--

iPTextV4Address		[2] IA5String (SIZE(7..15)),

iPTextV6Address		[3] IA5String (SIZE(15..45))

}

LCSCause				::= OCTET STRING (SIZE(1))

--

-- See LCS Cause Value, TS 49.031 [227]

--

LCSClientIdentity 		::= SEQUENCE

{

lcsClientExternalID	[0] LCSClientExternalID OPTIONAL,

lcsClientDialedByMS	[1] AddressString OPTIONAL,

lcsClientInternalID	[2] LCSClientInternalID OPTIONAL

}

LCSQoSInfo				::= OCTET STRING (SIZE(4))

--

-- See LCS QoS IE, TS 49.031 [227]

--

LevelOfCAMELService		::= BIT STRING

{

basic						(0),

callDurationSupervision		(1),

onlineCharging				(2)

}

LocalSequenceNumber ::= INTEGER (0..4294967295)

--

-- Sequence number of the record in this node

-- 0.. 4294967295 is equivalent to 0..2**32-1, unsigned integer in four octets

--

LocationAreaAndCell		::= SEQUENCE

{

locationAreaCode	[0] LocationAreaCode,

cellId				[1] CellId,

mCC-MNC				[2] MCC-MNC OPTIONAL

}

LocationAreaCode		::= OCTET STRING (SIZE(2))

--

-- See TS 24.008 [208]

--

ManagementExtensions	::= SET OF ManagementExtension

MBMS2G3GIndicator		::= ENUMERATED

{

twoG				(0),	-- For GERAN access only

threeG			(1),	-- For UTRAN access only

twoG-AND-threeG	(2) 	-- For both UTRAN and GERAN access

}

MBMSInformation			::= SET

{

tMGI							[1] TMGI OPTIONAL,

mBMSSessionIdentity			[2] MBMSSessionIdentity OPTIONAL,

mBMSServiceType				[3] MBMSServiceType OPTIONAL,

mBMSUserServiceType			[4] MBMSUserServiceType OPTIONAL, -- only supported in the BM-SC

mBMS2G3GIndicator			[5] MBMS2G3GIndicator OPTIONAL,

fileRepairSupported			[6] BOOLEAN OPTIONAL,			  -- only supported in the BM-SC

rAI							[7] RoutingAreaCode OPTIONAL,	  -- only supported in the BM-SC

mBMSServiceArea				[8] MBMSServiceArea OPTIONAL,

requiredMBMSBearerCaps		[9] RequiredMBMSBearerCapabilities OPTIONAL,

mBMSGWAddress				[10] GSNAddress OPTIONAL,

cNIPMulticastDistribution	[11] CNIPMulticastDistribution OPTIONAL,

mBMSDataTransferStart		[12] MBMSTime OPTIONAL,

mBMSDataTransferStop			[13] MBMSTime OPTIONAL

}

MBMSServiceArea		::= OCTET STRING

MBMSServiceType		::= ENUMERATED

{

mULTICAST		(0),

bROADCAST		(1)

}

MBMSSessionIdentity		::= OCTET STRING (SIZE (1))

--

-- This octet string is a 1:1 copy of the contents of the MBMS-Session-Identity

-- AVP specified in TS 29.061 [82]

--

MBMSTime	::= OCTET STRING (SIZE (8))

--

-- This value indicates the time in seconds relative to 00:00:00 on 1 January 1900 (calculated as

-- continuous time without leap seconds and traceable to a common time reference) where binary

-- encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in

-- the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second as

-- specified in TS 29.061 [82].

--

MBMSUserServiceType		::= ENUMERATED

{

dOWNLOAD		(0),

sTREAMING	(1)

}

MCC-MNC		::= OCTET STRING (SIZE(3))

--

-- See TS 24.008 [208]

--

MessageReference		::= OCTET STRING

MSCAddress		::= AddressString

MscNo					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSISDN					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSTimeZone	::= OCTET STRING (SIZE (2))

--

-- 1.Octet: Time Zone and 2. Octet: Daylight saving time, see TS 29.060 [215]

--

NodeAddress ::= CHOICE

{

iPAddress 	[0] IPAddress,

domainName	[1] GraphicString

}

PDPAddressPrefixLength		::=INTEGER (1..64)

--

-- This is an interger indicating the leght of the PDP/PDN IPv6 address prefix

-- and the default value is 64 bits.

--

PositioningData	::= OCTET STRING (SIZE(1..33))

--

-- See Positioning Data IE (octet 3..n), TS 49.031 [227]

--

RecordingEntity 		::= AddressString

RecordType 	::= INTEGER

--

--	Record values 0..17 and 87,89  are CS specific. The contents are defined in TS 32.250 [10]

--

{

moCallRecord			(0),

mtCallRecord			(1),

roamingRecord		(2),

incGatewayRecord		(3),

outGatewayRecord		(4),

transitCallRecord	(5),

moSMSRecord			(6),

mtSMSRecord			(7),

moSMSIWRecord		(8),

mtSMSGWRecord		(9),

ssActionRecord		(10),

hlrIntRecord			(11),

locUpdateHLRRecord	(12),

locUpdateVLRRecord	(13),

commonEquipRecord	(14),

moTraceRecord		(15),	-- used in earlier releases

mtTraceRecord		(16),	-- used in earlier releases

termCAMELRecord	    (17),

--

--	Record values 18..22 are GPRS specific. The contents are defined in TS 32.251 [11]

--

sgsnPDPRecord		(18),

sgsnMMRecord			(20),

sgsnSMORecord		(21),	-- also MME UE originated SMS record

sgsnSMTRecord		(22),	-- also MME UE terminated SMS record

--

--  Record values 23..25 are CS-LCS specific. The contents are defined in TS 32.250 [10]

--

mtLCSRecord			(23),

moLCSRecord			(24),

niLCSRecord			(25),

--

--  Record values 26..28 are GPRS-LCS specific. The contents are defined in TS 32.251 [11]

--

sgsnMTLCSRecord		(26),

sgsnMOLCSRecord		(27),

sgsnNILCSRecord		(28),

--

--  Record values 30..62 are MMS specific. The contents are defined in TS 32.270 [30]

--

mMO1SRecord			(30),

mMO4FRqRecord		(31),

mMO4FRsRecord		(32),

mMO4DRecord			(33),

mMO1DRecord			(34),

mMO4RRecord			(35),

mMO1RRecord			(36),

mMOMDRecord			(37),

mMR4FRecord			(38),

mMR1NRqRecord		(39),

mMR1NRsRecord		(40),

mMR1RtRecord			(41),

mMR1AFRecord			(42),

mMR4DRqRecord		(43),

mMR4DRsRecord		(44),

mMR1RRRecord			(45),

mMR4RRqRecord		(46),

mMR4RRsRecord		(47),

mMRMDRecord			(48),

mMFRecord			(49),

mMBx1SRecord			(50),

mMBx1VRecord			(51),

mMBx1URecord			(52),

mMBx1DRecord			(53),

mM7SRecord			(54),

mM7DRqRecord			(55),

mM7DRsRecord			(56),

mM7CRecord			(57),

mM7RRecord			(58),

mM7DRRqRecord		(59),

mM7DRRsRecord		(60),

mM7RRqRecord			(61),

mM7RRsRecord			(62),

--

--  Record values 63..70, 82, 89..91 are IMS specific.

--  The contents are defined in TS 32.260 [20]

--

sCSCFRecord			(63),

pCSCFRecord			(64),

iCSCFRecord			(65),

mRFCRecord			(66),

mGCFRecord			(67),

bGCFRecord			(68),

aSRecord				(69),

eCSCFRecord			(70),

iBCFRecord			(82),

tRFRecord			(89),

tFRecord				(90),

aTCFRecord			(91),

--

--  Record values 71..75 are LCS specific. The contents are defined in TS 32.271 [31]

--

lCSGMORecord			(71),

lCSRGMTRecord		(72),

lCSHGMTRecord		(73),

lCSVGMTRecord		(74),

lCSGNIRecord			(75),

--

--  Record values 76..79,86 are MBMS specific.

--  The contents are defined in TS 32.251 [11] and TS 32.273 [33]

--

--  Record values 76,77 and 86 are MBMS bearer context specific

--

sgsnMBMSRecord		(76),

ggsnMBMSRecord		(77),

gwMBMSRecord		(86),

--

--  Record values 78 and 79 are MBMS service specific and defined in TS 32.273 [33]

--

sUBBMSCRecord		(78),

cONTENTBMSCRecord	(79),

--

--  Record Values 80..81 are PoC specific. The contents are defined in TS 32.272 [32]

--

pPFRecord			(80),

cPFRecord			(81),

--

--	Record values 84,85 and 92,95,96, 97 are EPC specific.

--  The contents are defined in TS 32.251 [11]

--

sGWRecord			(84),

pGWRecord			(85),

tDFRecord			(92),

iPERecord			(95),

ePDGRecord			(96),

tWAGRecord			(97),

--

--  Record Value 83 is MMTel specific. The contents are defined in TS 32.275 [35]

--

mMTelRecord			(83),

--

--  Record value  87,88 and 89 are CS specific. The contents are defined in TS 32.250 [10]

--

mSCsRVCCRecord		(87),

mMTRFRecord			(88),

iCSRegisterRecord	(99),

--

--  Record values 93 and 94 are SMS specific. The contents are defined in TS 32.274 [34]

--

sCSMORecord			(93),

sCSMTRecord			(94),

--

--  Record values 100, 101 and 102 are ProSe specific. The contents are defined in TS 32.277 [36]

--

pFDDRecord			(100),

pFEDRecord			(101),

pFDCRecord			(102),

--

--  Record values103 and 104 are Monitoring Event specific. The contents are defined in TS

--	32.278 [38]

--

mECORecord			(103),

mERERecord			(104),

--

--  Record values 105 to 106 are CP data transfer specific. The contents are defined in TS

--	32.253 [13]

--

cPDTSCERecord			(105),

cPDTSNNRecord			(106) , --

--  Record values 110 to 111 are SMS specific. The contents are defined in TS

--	32.274 [34]

--

sCDVTT4Record			(110),

sCSMOT4Record			(111)

}

RequiredMBMSBearerCapabilities		::= OCTET STRING (SIZE (3..14))

--

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 5) of the

-- "Quality of service Profile" information element specified in TS 29.060 [75].

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [208]

--

SCSASAddress		::= SET

--

--

--

{

sCSAddress	[1] IPAddress,

sCSRealm		[2] DiameterIdentity

}

ServiceContextID		::= UTF8String

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData		[0] GraphicString OPTIONAL, 
	serviceSpecificType		[1] INTEGER OPTIONAL

}

SMSResult					::= Diagnostics

SmsTpDestinationNumber ::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- the SMS address field the encoding of the octet string is in

-- accordance with the definition of address fields in TS 23.040 [201].

-- This encoding includes type of number and numbering plan indication

-- together with the address value range.

--

SubscriberEquipmentNumber	::= SET

{

subscriberEquipmentNumberType	[0]	SubscriberEquipmentType,

subscriberEquipmentNumberData	[1]	OCTET STRING

}

SubscriberEquipmentType	::= ENUMERATED

--

-- It should be noted that depending on the services, not all equipment types are applicable.

-- For IMS equipment types 0 and 3 are applicable.

--

{

iMEISV			(0),

mAC				(1),

eUI64			(2),

modifiedEUI64	(3)

}

SubscriptionID	::= SET

--

-- used for ExternalIdentifier with SubscriptionIdType = END-User-NAI. See TS 23.003 [200]

--

{

subscriptionIDType	[0]	SubscriptionIDType,

subscriptionIDData	[1]	UTF8String

}

SubscriptionIDType	::= ENUMERATED

{

eND-USER-E164		(0),

eND-USER-IMSI		(1),

eND-USER-SIP-URI		(2),

eND-USER-NAI			(3),

eND-USER-PRIVATE		(4)

}

SystemType	::= ENUMERATED

--

--  "unknown" is not to be used in PS domain.

--

{

unknown				(0),

iuUTRAN				(1),

gERAN				(2)

}

ThreeGPPPSDataOffStatus		::= ENUMERATED

{

active		    (0),

inactive		(1)

}

TimeStamp	::= OCTET STRING (SIZE(9))

--

-- The contents of this field are a compact form of the UTCTime format

-- containing local time plus an offset to universal time. Binary coded

-- decimal encoding is employed for the digits to reduce the storage and

-- transmission overhead

-- e.g. YYMMDDhhmmssShhmm

-- where

-- YY 	= 	Year 00 to 99		BCD encoded

-- MM 	= 	Month 01 to 12 		BCD encoded

-- DD	=	Day 01 to 31		BCD encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

-- ss	=	second 00 to 59		BCD encoded

-- S	=	Sign 0 = "+", "-"	ASCII encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

--

TMGI		::= OCTET STRING

--

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 4)

-- of the "TMGI" information element specified in TS 29.060 [75].

--

.#END

### 5.2.2	Bearer level CDR definitions

#### 5.2.2.0	General

This clause contains the syntax definitions of the CDRs on the bearer level. This comprises the CDR types from the Circuit Switched (CS) domain (TS 32.250 [10]) and the Packet Switched (PS) domain, i.e. GPRS (TS 32.251 [11]).

#### 5.2.2.1	CS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.250 [10].

.$CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version1 (0)}  DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AE-title,

BCDDirectoryNumber,

CallDuration,

CalledNumber,

CallingNumber,

CauseForTerm,

CellId,

ChargeIndicator,

Diagnostics,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MCC-MNC,

MessageReference,

MSCAddress,

MscNo,

MSISDN,

NodeAddress,

ObjectInstance,

PositioningData,

RecordingEntity,

RecordType,

SMSResult,

SmsTpDestinationNumber,

SystemType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

BearerServiceCode

FROM MAP-BS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-BS-Code (20)  version15 (15) }

-- from TS 29.002 [214]

TeleserviceCode

FROM MAP-TS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-TS-Code (19)  version15 (15) }

-- from TS 29.002 [214]

SS-Code

FROM MAP-SS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-SS-Code (15)  version15 (15) }

-- from TS 29.002 [214]

MOLR-Type

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2)  version15 (15)}

-- from TS 24.080 [209]

DefaultCallHandling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11)  version15 (15) }

-- from TS 29.002 [214]

CallReferenceNumber,

NumberOfForwarding

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CH-DataTypes (13)  version15 (15) }

-- from TS 29.002 [214]

AddressString,

BasicServiceCode,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25)  version15 (15) }

-- from TS 29.002 [214]

IMS-Charging-Identifier,

InterOperatorIdentifierList,

TransitIOILists

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

BasicService

FROM Basic-Service-Elements	{itu-t(0) identified-organization (4) etsi (0) 196 basic-service-elements (8) }

-- from "Digital Subscriber Signalling System No. one (DSS1) protocol" ETS 300 196 [310]

DestinationRoutingAddress

FROM CAP-datatypes { itu-t(0) identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) cap-datatypes (52) version8 (7) }

-- from TS 29.078 [217]

;

--

--  CS CALL AND EVENT RECORDS

--

CSRecord	::= CHOICE

--

-- Record values 0..21 are circuit switch specific

--

{

moCallRecord				[0] MOCallRecord,

mtCallRecord				[1] MTCallRecord,

roamingRecord			[2] RoamingRecord,

incGatewayRecord			[3] IncGatewayRecord,

outGatewayRecord			[4] OutGatewayRecord,

transitRecord			[5] TransitCallRecord,

moSMSRecord				[6] MOSMSRecord,

mtSMSRecord				[7] MTSMSRecord,

moSMSIWRecord			[8] MOSMSIWRecord,

mtSMSGWRecord			[9] MTSMSGWRecord,

ssActionRecord			[10] SSActionRecord,

hlrIntRecord				[11] HLRIntRecord,

locUpdateHLRRecord		[12] LocUpdateHLRRecord,

locUpdateVLRRecord		[13] LocUpdateVLRRecord,

commonEquipRecord		[14] CommonEquipRecord,

recTypeExtensions		[15] ManagementExtensions,

termCAMELRecord			[16] TermCAMELRecord,

mtLCSRecord				[17] MTLCSRecord,

moLCSRecord				[18] MOLCSRecord,

niLCSRecord				[19] NILCSRecord,

mSCsRVCCRecord			[20] MSCsRVCCRecord,

mMTRFRecord				[21] MTRFRecord,

iCSRegisterRecord		[22] ICSregisterRecord

}

MOCallRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

callingNumber				[4] CallingNumber OPTIONAL,

calledNumber					[5] CalledNumber OPTIONAL,

translatedNumber				[6] TranslatedNumber OPTIONAL,

connectedNumber				[7] ConnectedNumber OPTIONAL,

roamingNumber				[8] RoamingNumber OPTIONAL,

recordingEntity				[9] RecordingEntity,

mscIncomingTKGP				[10] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell OPTIONAL,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode OPTIONAL,

transparencyIndicator		[15] TransparencyInd OPTIONAL,

changeOfService				[16] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters				[18] AOCParameters OPTIONAL,

changeOfAOCParms				[19] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

changeOfClassmark			[21] ChangeOfClassmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

dataVolume					[26] DataVolume OPTIONAL,

radioChanRequested			[27] RadioChanRequested OPTIONAL,

radioChanUsed				[28] TrafficChannel OPTIONAL,

changeOfRadioChan			[29] ChangeOfRadioChannel OPTIONAL,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

additionalChgInfo			[34] AdditionalChgInfo OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

gsm-SCFAddress				[36] Gsm-SCFAddress OPTIONAL,

serviceKey					[37] ServiceKey OPTIONAL,

networkCallReference			[38] NetworkCallReference OPTIONAL,

mSCAddress					[39] MSCAddress OPTIONAL,

cAMELInitCFIndicator			[40] CAMELInitCFIndicator OPTIONAL,

defaultCallHandling			[41] DefaultCallHandling OPTIONAL,

hSCSDChanRequested			[42] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated			[43] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms			[44] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur							[45] Fnur OPTIONAL,

aiurRequested				[46] AiurRequested OPTIONAL,

chanCodingsAcceptable		[47] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed				[48] ChannelCoding OPTIONAL,

speechVersionSupported		[49] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed			[50] SpeechVersionIdentifier OPTIONAL,

numberOfDPEncountered		[51] INTEGER OPTIONAL,

levelOfCAMELService			[52] LevelOfCAMELService OPTIONAL,

freeFormatData				[53] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[54] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend			[55] BOOLEAN OPTIONAL,

defaultCallHandling-2		[56] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[57] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[58] ServiceKey OPTIONAL,

freeFormatData-2				[59] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[60] BOOLEAN OPTIONAL,

systemType					[61] SystemType OPTIONAL,

rateIndication				[62] RateIndication OPTIONAL,

locationRoutNum				[63] LocationRoutingNumber OPTIONAL,

lrnSoInd						[64] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[65] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[66] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[67] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[68] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

guaranteedBitRate			[70] GuaranteedBitRate OPTIONAL,

maximumBitRate				[71] MaximumBitRate OPTIONAL,

redial						[72] BOOLEAN OPTIONAL,	-- set indicates redial attempt

reasonForServiceChange		[73] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator		[74] BOOLEAN OPTIONAL,

iCSI2ActiveFlag				[75] NULL OPTIONAL,

iMS-Charging-Identifier		[76] IMS-Charging-Identifier OPTIONAL,

privateUserID				[77] GraphicString OPTIONAL

}

MTCallRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

connectedNumber			[5] ConnectedNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

location					[9] LocationAreaAndCell OPTIONAL,

changeOfLocation			[10] SEQUENCE OF LocationChange OPTIONAL,

basicService				[11] BasicServiceCode OPTIONAL,

transparencyIndicator	[12] TransparencyInd OPTIONAL,

changeOfService			[13] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[14] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters			[15] AOCParameters OPTIONAL,

changeOfAOCParms			[16] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark				[17] Classmark OPTIONAL,

changeOfClassmark		[18] ChangeOfClassmark OPTIONAL,

seizureTime				[19] TimeStamp OPTIONAL,

answerTime				[20] TimeStamp OPTIONAL,

releaseTime				[21] TimeStamp OPTIONAL,

callDuration				[22] CallDuration,

dataVolume				[23] DataVolume OPTIONAL,

radioChanRequested		[24] RadioChanRequested OPTIONAL,

radioChanUsed			[25] TrafficChannel OPTIONAL,

changeOfRadioChan		[26] ChangeOfRadioChannel OPTIONAL,

causeForTerm				[27] CauseForTerm,

diagnostics				[28] Diagnostics OPTIONAL,

callReference			[29] CallReferenceNumber,

sequenceNumber			[30] INTEGER OPTIONAL,

additionalChgInfo		[31] AdditionalChgInfo OPTIONAL,

recordExtensions			[32] ManagementExtensions OPTIONAL,

networkCallReference		[33] NetworkCallReference OPTIONAL,

mSCAddress				[34] MSCAddress OPTIONAL,

hSCSDChanRequested		[35] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated		[36] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms		[37] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur						[38] Fnur OPTIONAL,

aiurRequested			[39] AiurRequested OPTIONAL,

chanCodingsAcceptable	[40] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed			[41] ChannelCoding OPTIONAL,

speechVersionSupported	[42] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed		[43] SpeechVersionIdentifier OPTIONAL,

gsm-SCFAddress			[44] Gsm-SCFAddress OPTIONAL,

serviceKey				[45] ServiceKey OPTIONAL,

systemType				[46] SystemType OPTIONAL,

rateIndication			[47] RateIndication OPTIONAL,

locationRoutNum			[48] LocationRoutingNumber OPTIONAL,

lrnSoInd					[49] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[50] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[51] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[52] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[53] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[54] PartialRecordType OPTIONAL,

guaranteedBitRate		[55] GuaranteedBitRate OPTIONAL,

maximumBitRate			[56] MaximumBitRate OPTIONAL,

reasonForServiceChange	[57] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[58] BOOLEAN OPTIONAL,

iCSI2ActiveFlag			[59] NULL OPTIONAL,

iMS-Charging-Identifier	[60] IMS-Charging-Identifier OPTIONAL,

privateUserID			[61] GraphicString OPTIONAL

}

RoamingRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

callingNumber			[3] CallingNumber OPTIONAL,

roamingNumber			[4] RoamingNumber OPTIONAL,

recordingEntity			[5] RecordingEntity,

mscIncomingTKGP			[6] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[7] TrunkGroup OPTIONAL,

basicService				[8] BasicServiceCode OPTIONAL,

transparencyIndicator	[9] TransparencyInd OPTIONAL,

changeOfService			[10] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[11] SEQUENCE OF  SuppServiceUsed OPTIONAL,

seizureTime				[12] TimeStamp OPTIONAL,

answerTime				[13] TimeStamp OPTIONAL,

releaseTime				[14] TimeStamp OPTIONAL,

callDuration				[15] CallDuration,

dataVolume				[16] DataVolume OPTIONAL,

causeForTerm				[17] CauseForTerm,

diagnostics				[18] Diagnostics OPTIONAL,

callReference			[19] CallReferenceNumber,

sequenceNumber			[20] INTEGER OPTIONAL,

recordExtensions			[21] ManagementExtensions OPTIONAL,

networkCallReference		[22] NetworkCallReference OPTIONAL,

mSCAddress				[23] MSCAddress OPTIONAL,

locationRoutNum			[24] LocationRoutingNumber OPTIONAL,

lrnSoInd					[25] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[26] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[27] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[28] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[29] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[30] PartialRecordType OPTIONAL

}

TermCAMELRecord	::= SET

{

recordtype					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN OPTIONAL,

recordingEntity				[3] RecordingEntity,

interrogationTime			[4] TimeStamp,

destinationRoutingAddress	[5] DestinationRoutingAddress,

gsm-SCFAddress				[6] Gsm-SCFAddress,

serviceKey					[7] ServiceKey,

networkCallReference			[8] NetworkCallReference OPTIONAL,

mSCAddress					[9] MSCAddress OPTIONAL,

defaultCallHandling			[10] DefaultCallHandling OPTIONAL,

recordExtensions				[11] ManagementExtensions OPTIONAL,

calledNumber					[12] CalledNumber,

callingNumber				[13] CallingNumber OPTIONAL,

mscIncomingTKGP				[14] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[15] TrunkGroup OPTIONAL,

seizureTime					[16] TimeStamp OPTIONAL,

answerTime					[17] TimeStamp OPTIONAL,

releaseTime					[18] TimeStamp OPTIONAL,

callDuration					[19] CallDuration,

dataVolume					[20] DataVolume OPTIONAL,

causeForTerm					[21] CauseForTerm,

diagnostics					[22] Diagnostics OPTIONAL,

callReference				[23] CallReferenceNumber,

sequenceNumber				[24] INTEGER OPTIONAL,

numberOfDPEncountered		[25] INTEGER OPTIONAL,

levelOfCAMELService			[26] LevelOfCAMELService OPTIONAL,

freeFormatData				[27] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[28] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend			[29] BOOLEAN OPTIONAL,

defaultCallHandling-2		[30] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[31] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[32] ServiceKey OPTIONAL,

freeFormatData-2				[33] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[34] BOOLEAN OPTIONAL,

mscServerIndication			[35] BOOLEAN OPTIONAL,

locationRoutNum				[36] LocationRoutingNumber OPTIONAL,

lrnSoInd						[37] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[38] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[39] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[40] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[41] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[42] PartialRecordType OPTIONAL

}

IncGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL

}

OutGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL
}

TransitCallRecord		::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

mscIncomingTKGP			[2] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[3] TrunkGroup OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

calledNumber				[5] CalledNumber,

isdnBasicService			[6] BasicService OPTIONAL,

seizureTimestamp			[7] TimeStamp OPTIONAL,

answerTimestamp			[8] TimeStamp OPTIONAL,

releaseTimestamp			[9] TimeStamp OPTIONAL,

callDuration				[10] CallDuration,

dataVolume				[11] DataVolume OPTIONAL,

causeForTerm				[12] CauseForTerm,

diagnostics				[13] Diagnostics OPTIONAL,

callReference			[14] CallReferenceNumber,

sequenceNumber			[15] INTEGER OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

locationRoutNum			[17] LocationRoutingNumber OPTIONAL,

lrnSoInd					[18] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[19] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[20] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[21] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[22] JurisdictionInformationParameterQueryStatus OPTIONAL

}

MOSMSRecord				::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msClassmark				[4] Classmark,

serviceCentre			[5] AddressString,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

messageReference			[8] MessageReference,

originationTime			[9] TimeStamp,

smsResult				[10] SMSResult OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

destinationNumber		[12] SmsTpDestinationNumber OPTIONAL,

cAMELSMSInformation		[13] CAMELSMSInformation OPTIONAL,

systemType				[14] SystemType OPTIONAL,

locationExtension		[15] LocationCellExtension OPTIONAL

}

MTSMSRecord				::= SET

{

recordType				[0] RecordType,

serviceCentre			[1] AddressString,

servedIMSI				[2] IMSI,

servedIMEI				[3] IMEI OPTIONAL,

servedMSISDN				[4] MSISDN OPTIONAL,

msClassmark				[5] Classmark,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

deliveryTime				[8] TimeStamp,

smsResult				[9] SMSResult OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

systemType				[11] SystemType OPTIONAL,

cAMELSMSInformation		[12] CAMELSMSInformation OPTIONAL,

locationExtension		[13] LocationCellExtension OPTIONAL

}

MOSMSIWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

recordingEntity		[3] RecordingEntity,

eventTime			[4] TimeStamp,

smsResult			[5] SMSResult OPTIONAL,

recordExtensions	[6] ManagementExtensions OPTIONAL

}

MTSMSGWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

smsResult			[6] SMSResult OPTIONAL,

recordExtensions		[7] ManagementExtensions OPTIONAL

}

SSActionRecord			::= SET

{

recordType			[0] RecordType,

servedIMSI			[1] IMSI,

servedIMEI			[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msClassmark			[4] Classmark,

recordingEntity		[5] RecordingEntity,

location				[6] LocationAreaAndCell OPTIONAL,

basicServices		[7] BasicServices OPTIONAL,

supplService			[8] SS-Code OPTIONAL,

ssAction				[9] SSActionType OPTIONAL,

ssActionTime			[10] TimeStamp,

ssParameters			[11] SSParameters OPTIONAL,

ssActionResult		[12] SSActionResult OPTIONAL,

callReference		[13] CallReferenceNumber,

recordExtensions		[14] ManagementExtensions OPTIONAL,

systemType			[15] SystemType OPTIONAL

}

HLRIntRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN,

recordingEntity			[3] RecordingEntity,

basicService				[4] BasicServiceCode OPTIONAL,

routingNumber			[5] RoutingNumber,

interrogationTime		[6] TimeStamp,

numberOfForwarding		[7] NumberOfForwarding OPTIONAL,

interrogationResult		[8] HLRIntResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

LocUpdateHLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

recordingEntity			[2] RecordingEntity,

oldLocation				[3] Visited-Location-info OPTIONAL,

newLocation				[4] Visited-Location-info,

updateTime				[5] TimeStamp,

updateResult				[6] LocUpdResult OPTIONAL,

recordExtensions			[7] ManagementExtensions OPTIONAL

}

LocUpdateVLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

recordingEntity			[3] RecordingEntity,

oldLocation				[4] Location-info OPTIONAL,

newLocation				[5] Location-info,

msClassmark				[6] Classmark,

updateTime				[7] TimeStamp,

updateResult				[8] LocUpdResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL,

locationExtension		[10] LocationCellExtension OPTIONAL

}

CommonEquipRecord 		::= SET

{

recordType				[0] RecordType,

equipmentType			[1] EquipmentType,

equipmentId				[2] EquipmentId,

servedIMSI				[3] IMSI,

servedMSISDN				[4] MSISDN OPTIONAL,

recordingEntity			[5] RecordingEntity,

basicService				[6] BasicServiceCode OPTIONAL,

changeOfService			[7] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[8] SEQUENCE OF SuppServiceUsed OPTIONAL,

seizureTime				[9] TimeStamp,

releaseTime				[10] TimeStamp OPTIONAL,

callDuration				[11] CallDuration,

callReference			[12] CallReferenceNumber,

sequenceNumber			[13] INTEGER OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

systemType				[15] SystemType OPTIONAL,

rateIndication			[16] RateIndication OPTIONAL,

fnur						[17] Fnur OPTIONAL

}

--

--  OBSERVED IMEI TICKETS

--

ObservedIMEITicket		::= SET

{

servedIMEI			[0] IMEI,

imeiStatus			[1] IMEIStatus,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

location				[6] LocationAreaAndCell	,

imeiCheckEvent		[7] IMEICheckEvent OPTIONAL,

callReference		[8] CallReferenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

--

--  CS LOCATION SERVICE RECORDS

--

MTLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

notificationToMSUser		[12] NotificationToMSUser OPTIONAL,

privacyOverride			[13] NULL OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm,

servedIMEI				[22] IMEI OPTIONAL

}

MOLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

molr-Type				[6] MOLR-Type,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString OPTIONAL,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

location					[12] LocationAreaAndCell OPTIONAL,

locationEstimate			[13] Ext-GeographicalInformation OPTIONAL,

positioningData			[14] PositioningData OPTIONAL,

lcsCause					[15] LCSCause OPTIONAL,

diagnostics				[16] Diagnostics OPTIONAL,

systemType				[17] SystemType OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

causeForTerm				[19] CauseForTerm

}

NILCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

servedIMEI				[6] IMEI OPTIONAL,

emsDigits				[7] ISDN-AddressString OPTIONAL,

emsKey					[8] ISDN-AddressString OPTIONAL,

lcsQos					[9] LCSQoSInfo OPTIONAL,

lcsPriority				[10] LCS-Priority OPTIONAL,

mlc-Number				[11] ISDN-AddressString OPTIONAL,

eventTimeStamp			[12] TimeStamp,

measureDuration			[13] CallDuration OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm

}

--

--  SRVCC RECORDS

--

MSCsRVCCRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

calledNumber					[5] CalledNumber,

recordingEntity				[9] RecordingEntity,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

iMS-Charging-Identifier		[75] IMS-Charging-Identifier OPTIONAL,

iCSI2ActiveFlag				[76] NULL OPTIONAL,

relatedICID					[77] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode	[78] NodeAddress OPTIONAL

}

--

--  MTRF RECORD

--

MTRFRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

roamingNumber			[5] RoamingNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

basicService				[9] BasicServiceCode OPTIONAL,

seizureTime				[10] TimeStamp OPTIONAL,

answerTime				[11] TimeStamp OPTIONAL,

releaseTime				[12] TimeStamp OPTIONAL,

callDuration				[13] CallDuration,

causeForTerm				[14] CauseForTerm,

diagnostics				[15] Diagnostics OPTIONAL,

callReference			[16] CallReferenceNumber,

sequenceNumber			[17] INTEGER OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

partialRecordType		[19] PartialRecordType OPTIONAL

}

--

--  ICS RECORD

--

ICSregisterRecord ::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN,

privateUserID				[3] GraphicString OPTIONAL,

recordingEntity				[4] RecordingEntity,

newLocation					[5] Location-info,

locationExtension			[6] LocationCellExtension OPTIONAL,

updateTime					[7] TimeStamp OPTIONAL,

iMS-Charging-Identifier		[8] IMS-Charging-Identifier OPTIONAL,

interOperatorIdentifiers		[9] InterOperatorIdentifierList OPTIONAL,

transit-IOI-Lists			[10] TransitIOILists OPTIONAL,

updateResult					[11] LocUpdResult OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL

}

--

--  NP Fields

--

LocationRoutingNumber	::= OCTET STRING (SIZE (5))

--

--  The format is selected to meet the existing standards for the wireline in Telcordia

--   Belcore GR-1100-CORE, BAF Module 720.

--

LocationRoutingNumberSourceIndicator	::=	INTEGER

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

LocationRoutingNumberQueryStatus 		::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

JurisdictionInformationParameter 	::= OCTET STRING (SIZE (5))

--

-- /* JIP Parameter */

--

JurisdictionInformationParameterSourceIndicator 	::=	INTEGER

--

-- Identical to LocationRoutingNumberSourceIndicator

--

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

JurisdictionInformationParameterQueryStatus 	::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

--

--  CS DATA TYPES

--

AdditionalChgInfo		::= SEQUENCE

{

chargeIndicator		[0] ChargeIndicator OPTIONAL,

chargeParameters	[1] OCTET STRING OPTIONAL

}

AiurRequested			::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

-- (note that value "4" is intentionally missing

-- because it is not used in TS 24.008 [208])

--

{

aiur09600BitsPerSecond		(1),

aiur14400BitsPerSecond		(2),

aiur19200BitsPerSecond		(3),

aiur28800BitsPerSecond		(5),

aiur38400BitsPerSecond		(6),

aiur43200BitsPerSecond		(7),

aiur57600BitsPerSecond		(8),

aiur38400BitsPerSecond1		(9),

aiur38400BitsPerSecond2		(10),

aiur38400BitsPerSecond3		(11),

aiur38400BitsPerSecond4		(12)

}

AOCParameters			::= SEQUENCE

--

-- See TS 22.024 [104].

--

{

e1					[1] EParameter OPTIONAL,

e2					[2] EParameter OPTIONAL,

e3					[3] EParameter OPTIONAL,

e4					[4] EParameter OPTIONAL,

e5					[5] EParameter OPTIONAL,

e6					[6] EParameter OPTIONAL,

e7					[7] EParameter OPTIONAL

}

AOCParmChange			::= SEQUENCE

{

changeTime			[0] TimeStamp,

newParameters		[1] AOCParameters

}

BasicServices			::= SET OF BasicServiceCode

CallingPartyCategory	::= Category

CallType				::= INTEGER

{

mobileOriginated	(0),

mobileTerminated	(1)

}

CallTypes 				::= SET OF CallType

CAMELDestinationNumber	::= DestinationRoutingAddress

CAMELInformation		::= SET

{

cAMELDestinationNumber		[1] CAMELDestinationNumber OPTIONAL,

connectedNumber				[2] ConnectedNumber OPTIONAL,

roamingNumber				[3] RoamingNumber OPTIONAL,

mscOutgoingTKGP				[4] TrunkGroup OPTIONAL,

seizureTime					[5] TimeStamp OPTIONAL,

answerTime					[6] TimeStamp OPTIONAL,

releaseTime					[7] TimeStamp OPTIONAL,

callDuration					[8] CallDuration OPTIONAL,

dataVolume					[9] DataVolume OPTIONAL,

cAMELInitCFIndicator			[10] CAMELInitCFIndicator OPTIONAL,

causeForTerm					[11] CauseForTerm OPTIONAL,

cAMELModification			[12] ChangedParameters OPTIONAL,

freeFormatData				[13] FreeFormatData OPTIONAL,

diagnostics					[14] Diagnostics OPTIONAL,

freeFormatDataAppend			[15] BOOLEAN OPTIONAL,

freeFormatData-2				[16] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[17] BOOLEAN OPTIONAL

}

CAMELInitCFIndicator	::= ENUMERATED

{

noCAMELCallForwarding		(0),

cAMELCallForwarding			(1)

}

CAMELModificationParameters		::= SET

--

-- The list contains only parameters changed due to CAMEL call handling.

--

{

callingPartyNumber			[0] CallingNumber OPTIONAL,

callingPartyCategory			[1] CallingPartyCategory OPTIONAL,

originalCalledPartyNumber	[2] OriginalCalledNumber OPTIONAL,

genericNumbers				[3] GenericNumbers OPTIONAL,

redirectingPartyNumber		[4] RedirectingNumber OPTIONAL,

redirectionCounter			[5] NumberOfForwarding OPTIONAL

}

CAMELSMSInformation		::= SET

{

gsm-SCFAddress						[1] Gsm-SCFAddress OPTIONAL,

serviceKey							[2] ServiceKey OPTIONAL,

defaultSMSHandling			 		[3] DefaultSMS-Handling OPTIONAL,

freeFormatData       				[4] FreeFormatData OPTIONAL,

callingPartyNumber 					[5] CallingNumber OPTIONAL,

destinationSubscriberNumber			[6] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[7] AddressString OPTIONAL,

smsReferenceNumber					[8] CallReferenceNumber OPTIONAL

}

Category		::= OCTET STRING (SIZE(1))

--

-- The internal structure is defined in Recommendation Q.763.

--

ChangedParameters		::= SET

{

changeFlags		[0] ChangeFlags,

changeList		[1] CAMELModificationParameters OPTIONAL

}

ChangeFlags				::= BIT STRING

{

callingPartyNumberModified			(0),

callingPartyCategoryModified			(1),

originalCalledPartyNumberModified	(2),

genericNumbersModified				(3),

redirectingPartyNumberModified		(4),

redirectionCounterModified			(5)

}

ChangeOfClassmark 		::= SEQUENCE

{

classmark			[0] Classmark,

changeTime			[1] TimeStamp

}

ChangeOfRadioChannel 	::= SEQUENCE

{

radioChannel		[0] TrafficChannel,

changeTime			[1] TimeStamp,

speechVersionUsed	[2] SpeechVersionIdentifier OPTIONAL

}

ChangeOfService 		::= SEQUENCE

{

basicService		[0] BasicServiceCode,

transparencyInd	[1] TransparencyInd OPTIONAL,

changeTime		[2] TimeStamp,

rateIndication	[3] RateIndication OPTIONAL,

fnur				[4] Fnur OPTIONAL

}

ChannelCoding				::= ENUMERATED

{

tchF4800				(1),

tchF9600				(2),

tchF14400			(3)

}

Classmark				::= OCTET STRING

--

-- See Mobile station classmark 2, Mobile station classmark 3, TS 24.008[208]

--

ConnectedNumber			::= BCDDirectoryNumber

DataVolume				::= INTEGER

--

-- The volume of data transferred in segments of 64 octets.

--

Day						::= INTEGER (1..31)

DayClass				::= ObjectInstance

DayClasses				::= SET OF DayClass

DayDefinition			::= SEQUENCE

{

day				[0] DayOfTheWeek,

dayClass			[1] ObjectInstance

}

DayDefinitions			::= SET OF DayDefinition

DateDefinition			::= SEQUENCE

{

month				[0] Month,

day					[1] Day,

dayClass				[2] ObjectInstance

}

DateDefinitions			::= SET OF DateDefinition

DayOfTheWeek			::= ENUMERATED

{

allDays				(0),

sunday				(1),

monday				(2),

tuesday				(3),

wednesday			(4),

thursday				(5),

friday				(6),

saturday				(7)

}

Destinations			::= SET OF AE-title

EmergencyCallIndEnable	::= BOOLEAN

EmergencyCallIndication	::= SEQUENCE

{

cellId			[0] CellId,

callerId			[1] IMSIorIMEI

}

EParameter ::= INTEGER

EquipmentId				::= INTEGER

EquipmentType			::= INTEGER

{

conferenceBridge	(0)

}

FileType				::= INTEGER

{

callRecords			(1),

traceRecords			(9),

observedIMEITicket	(14)

}

Fnur					::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

fnurNotApplicable			(0),

fnur9600-BitsPerSecond		(1),

fnur14400BitsPerSecond		(2),

fnur19200BitsPerSecond		(3),

fnur28800BitsPerSecond		(4),

fnur38400BitsPerSecond		(5),

fnur48000BitsPerSecond		(6),

fnur56000BitsPerSecond		(7),

fnur64000BitsPerSecond		(8),

fnur33600BitsPerSecond		(9),

fnur32000BitsPerSecond		(10),

fnur31200BitsPerSecond		(11)

}

ForwardToNumber 		::= AddressString

FreeFormatData			::= OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FCI message

-- See TS 29.078 [217]

--

GenericNumber			::= BCDDirectoryNumber

GenericNumbers			::= SET OF GenericNumber

Gsm-SCFAddress			::= ISDN-AddressString

--

-- See TS 29.002 [214]

--

GuaranteedBitRate ::= ENUMERATED

{

gbr14400BitsPerSecond (1),		-- BS20 non-transparent

gbr28800BitsPerSecond (2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

gbr32000BitsPerSecond (3),		-- BS30 multimedia

gbr33600BitsPerSecond (4),		-- BS30 multimedia

gbr56000BitsPerSecond (5),		-- BS30 transparent and multimedia

gbr57600BitsPerSecond (6),		-- BS20 non-transparent

gbr64000BitsPerSecond (7)		-- BS30 transparent and multimedia

}

HLRIntResult			::= Diagnostics

HSCSDParmsChange		::= SEQUENCE

{

changeTime				[0] TimeStamp,

hSCSDChanAllocated		[1] NumOfHSCSDChanAllocated,

initiatingParty			[2] InitiatingParty OPTIONAL,

aiurRequested			[3] AiurRequested OPTIONAL,

chanCodingUsed			[4] ChannelCoding,

hSCSDChanRequested		[5] NumOfHSCSDChanRequested OPTIONAL

}

IMEICheckEvent			::= INTEGER

{

mobileOriginatedCall	(0),

mobileTerminatedCall	(1),

smsMobileOriginating	(2),

smsMobileTerminating	(3),

ssAction				(4),

locationUpdate		(5)

}

IMEIStatus				::= ENUMERATED

{

greyListedMobileEquipment		(0),

blackListedMobileEquipment		(1),

nonWhiteListedMobileEquipment	(2)

}

IMSIorIMEI				::= CHOICE

{

imsi				[0] IMSI,

imei				[1] IMEI

}

InitiatingParty			::= ENUMERATED

{

network				(0),

subscriber			(1)

}

LocationCellExtension	::= BIT STRING (SIZE (12))

LocationChange			::= SEQUENCE

{

location			[0] LocationAreaAndCell,

changeTime		[1] TimeStamp

}

Location-info			::= SEQUENCE

{

mscNumber			[1] MscNo OPTIONAL,

location-area		[2] LocationAreaCode,

cell-identification	[3] CellId OPTIONAL,

mCC-MNC				[4] MCC-MNC OPTIONAL

}

LocUpdResult			::= Diagnostics

MaximumBitRate ::= ENUMERATED

{

mbr14400BitsPerSecond	(1),		-- BS20 non-transparent

mbr28800BitsPerSecond	(2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

mbr32000BitsPerSecond	(3),		-- BS30 multimedia

mbr33600BitsPerSecond	(4),		-- BS30 multimedia

mbr56000BitsPerSecond	(5),		-- BS30 transparent and multimedia

mbr57600BitsPerSecond	(6)		-- BS20 non-transparent

}

Month		::= INTEGER (1..12)

MSPowerClasses	::= SET OF RFPowerCapability

NetworkCallReference	::= CallReferenceNumber

--

-- See TS 29.002 [214]

--

NetworkSpecificCode		::= INTEGER

--

-- To be defined by network operator

--

NetworkSpecificServices	::= SET OF NetworkSpecificCode

NumOfHSCSDChanRequested		::= INTEGER

NumOfHSCSDChanAllocated		::= INTEGER

ObservedIMEITicketEnable	::= BOOLEAN

OriginalCalledNumber	::= BCDDirectoryNumber

OriginDestCombinations	::= SET OF OriginDestCombination

OriginDestCombination	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes originId and destinationId

-- respectively. At least one of the two must be present.

--

{

origin					[0] INTEGER OPTIONAL,

destination				[1] INTEGER OPTIONAL

}

PartialRecordTimer	::= INTEGER

PartialRecordType	::= ENUMERATED

{

timeLimit					(0),

serviceChange				(1),

locationChange				(2),

classmarkChange				(3),

aocParmChange				(4),

radioChannelChange			(5),

hSCSDParmChange				(6),

changeOfCAMELDestination	(7)

}

PartialRecordTypes		::= SET OF PartialRecordType

RadioChannelsRequested	::= SET OF RadioChanRequested

RadioChanRequested	::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

halfRateChannel				(0),

fullRateChannel				(1),

dualHalfRatePreferred		(2),

dualFullRatePreferred		(3)

}

RateIndication ::= OCTET STRING(SIZE(1))

ReasonForServiceChange	::= ENUMERATED

{

msubInitiated			(0),

mscInitiated				(1),

callSetupFallBack		(2),

callSetupChangeOrder		(3)

}

RecordClassDestination	::= CHOICE

{

osApplication	[0] AE-title,

fileType			[1] FileType

}

RecordClassDestinations	::= SET OF RecordClassDestination

RecordingMethod	::= ENUMERATED

{

inCallRecord		(0),

inSSRecord		(1)

}

RedirectingNumber	::= BCDDirectoryNumber

RFPowerCapability	::= INTEGER

--

-- This field contains the RF power capability of the Mobile station

-- classmark 1 and 2 of TS 24.008 [208] expressed as an integer.

--

RoamingNumber			::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

RoutingNumber			::= CHOICE

{

roaming				[1] RoamingNumber,

forwarded			[2] ForwardToNumber

}

Service						::= CHOICE

{

teleservice					[1] TeleserviceCode,

bearerService				[2] BearerServiceCode,

supplementaryService			[3] SS-Code,

networkSpecificService		[4] NetworkSpecificCode

}

ServiceDistanceDependencies	::= SET OF ServiceDistanceDependency

ServiceDistanceDependency	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes aocServiceId and zoneId

-- respectively.

--

{

aocService			[0] INTEGER,

chargingZone			[1] INTEGER OPTIONAL

}

SimpleIntegerName			::= INTEGER

SimpleStringName			::= GraphicString

SpeechVersionIdentifier		::= OCTET STRING (SIZE(1))

--

--	see GSM 08.08[313]

--

--	000 0001	GSM speech full rate version 1

--	001 0001	GSM speech full rate version 2	used for enhanced full rate

--	010 0001	GSM speech full rate version 3	for future use

--	000 0101	GSM speech half rate version 1

--	001 0101	GSM speech half rate version 2 	for future use

--	010 0101	GSM speech half rate version 3	for future use

--

SSActionResult				::= Diagnostics

SSActionType				::= ENUMERATED

{

registration				(0),

erasure					(1),

activation				(2),

deactivation				(3),

interrogation			(4),

invocation				(5),

passwordRegistration		(6)

}

SSParameters				::= CHOICE

{

forwardedToNumber	[0] ForwardToNumber,

unstructuredData		[1] OCTET STRING

}

SupplServices				::= SET OF SS-Code

SuppServiceUsed				::= SEQUENCE

{

ssCode					[0] SS-Code,

ssTime					[1] TimeStamp OPTIONAL

}

SwitchoverTime				::= SEQUENCE

{

hour					INTEGER (0..23),

minute					INTEGER (0..59),

second					INTEGER (0..59)

}

TariffId					::= INTEGER

TariffPeriod				::= SEQUENCE

--

-- Note that the value of tariffId corresponds to the attribute tariffId.

--

{

switchoverTime			[0] SwitchoverTime,

tariffId				[1] INTEGER

}

TariffPeriods				::= SET OF TariffPeriod

TariffSystemStatus			::= ENUMERATED

{

available				(0),	-- available for modification

checked					(1),	-- "frozen" and checked

standby					(2),	-- "frozen" awaiting activation

active					(3)		-- "frozen" and active

}

TrafficChannel			::=	ENUMERATED

{

fullRate			(0),

halfRate			(1)

}

TranslatedNumber		::= 	BCDDirectoryNumber

TransparencyInd			::=	ENUMERATED

{

transparent			(0),

nonTransparent		(1)

}

TrunkGroup				::=	 CHOICE

{

tkgpNumber		[0] INTEGER,

tkgpName			[1] GraphicString

}

TSChangeover			::=	SEQUENCE

--

-- Note that if the changeover time is not

-- specified then the change is immediate.

--

{

newActiveTS			[0] INTEGER,

newStandbyTS			[1] INTEGER,

changeoverTime		[2] GeneralizedTime OPTIONAL,

authkey				[3] OCTET STRING OPTIONAL,

checksum				[4] OCTET STRING OPTIONAL,

versionNumber		[5] OCTET STRING OPTIONAL

}

TSCheckError			::=	SEQUENCE

{

errorId				[0] TSCheckErrorId,

fail					ANY DEFINED BY errorId OPTIONAL

}

TSCheckErrorId			::=	CHOICE

{

globalForm			[0] OBJECT IDENTIFIER,

localForm			[1] INTEGER

}

TSCheckResult			::=	CHOICE

{

success				[0] NULL,

fail					[1] SET OF TSCheckError

}

TSCopyTariffSystem		::=	SEQUENCE

{

oldTS				[0] INTEGER,

newTS				[1] INTEGER

}

TSNextChange			::=	CHOICE

{

noChangeover			[0] NULL,

tsChangeover			[1] TSChangeover

}

TypeOfSubscribers		::= ENUMERATED

{

home					(0),	-- HPLMN subscribers

visiting				(1),	-- roaming subscribers

all	(2)

}

TypeOfTransaction		::=	ENUMERATED

{

successful			(0),

unsuccessful			(1),

all					(2)

}

Visited-Location-info		::= SEQUENCE

{

mscNumber			[1] MscNo,

vlrNumber			[2] VlrNo

}

VlrNo		::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

.#END

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

.$GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

CallDuration,

CallingNumber,

CauseForRecClosing,

CellId,

Diagnostics,

DiameterIdentity,

EnhancedDiagnostics,

GSNAddress,

IPAddress,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocalSequenceNumber,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MBMSInformation,

MessageReference,

MSISDN,

MSTimeZone,

PositioningData,

RecordingEntity,

RecordType,

RoutingAreaCode,

SCSASAddress,

ServiceSpecificInfo,

SMSResult,

SmsTpDestinationNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DefaultGPRS-Handling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version15 (15)}

-- from TS 29.002 [214]

IMEI,

IMSI,

ISDN-AddressString,

RAIdentity

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version15 (15)}

-- from TS 29.002 [214]

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CH-DataTypes (13) version15 (15)}

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version15 (15) }

-- from TS 29.002 [214]

LocationMethod

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version15 (15)}

-- from TS 24.080 [209]

;

--

--  GPRS RECORDS

--

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78,79 and 92, 95, 96 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnMTLCSRecord			[25] SGSNMTLCSRecord,

sgsnMOLCSRecord			[26] SGSNMOLCSRecord,

sgsnNILCSRecord			[27] SGSNNILCSRecord,

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord,

gwMBMSRecord			[86] GWMBMSRecord,

tDFRecord				[92] TDFRecord,

iPERecord				[95] IPERecord,

ePDGRecord				[96] EPDGRecord,

tWAGRecord				[97] TWAGRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

s-GWiPv6Address				[48] GSNAddress OPTIONAL,

servingNodeiPv6Address		[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

userLocationInfoTime		[52] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[53] CNOperatorSelectionEntity OPTIONAL,

presenceReportingAreaInfo	[54] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation	[55] OCTET STRING OPTIONAL,

lastMSTimeZone				[56] MSTimeZone OPTIONAL,

enhancedDiagnostics			[57] EnhancedDiagnostics OPTIONAL,

cPCIoTEPSOptimisationIndicator [59] CPCIoTEPSOptimisationIndicator OPTIONAL,

uNIPDUCPOnlyFlag			[60] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl		[61] ServingPLMNRateControl OPTIONAL,

pDPPDNTypeExtension			[62] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter		[63] MOExceptionDataCounter OPTIONAL

}

PGWRecord 	::= SET

--

--	List of traffic volumes is only applicable when Charging per IP-CAN session is active and

--	IP-CAN bearer charging is being performed for the session.

--

--	EPC QoS Information is only applicable when Charging per IP-CAN session is active.

--

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

lowPriorityIndicator				[46] NULL OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

userLocationInfoTime				[53] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		    	[54] CNOperatorSelectionEntity OPTIONAL,

ePCQoSInformation					[55] EPCQoSInformation OPTIONAL,

presenceReportingAreaInfo			[56] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation			[57] OCTET STRING OPTIONAL,

lastMSTimeZone						[58] MSTimeZone OPTIONAL,

enhancedDiagnostics					[59] EnhancedDiagnostics OPTIONAL,

nBIFOMMode							[60] NBIFOMMode OPTIONAL,

nBIFOMSupport						[61] NBIFOMSupport OPTIONAL,

uWANUserLocationInformation			[62] UWANUserLocationInfo OPTIONAL,

sGiPtPTunnellingMethod				[64] SGiPtPTunnellingMethod OPTIONAL,

uNIPDUCPOnlyFlag					[65] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl				[66] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[67] APNRateControl OPTIONAL,

pDPPDNTypeExtension					[68] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter				[69] MOExceptionDataCounter OPTIONAL,

chargingPerIPCANSessionIndicator    [70] ChargingPerIPCANSessionIndicator OPTIONAL,

threeGPPPSDataOffStatus				[71] ThreeGPPPSDataOffStatus OPTIONAL,

sCSASAddress						[72] SCSASAddress OPTIONAL

}

TDFRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

tDFAddress							[53] GSNAddress,

tDFiPv6AddressUsed					[54] GSNAddress OPTIONAL,

tDFPLMNIdentifier					[55] PLMN-Id OPTIONAL,

servedFixedSubsID					[56] FixedSubsID OPTIONAL,

accessLineIdentifier				[57] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation		[59] FixedUserLocationInformation OPTIONAL

}

IPERecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

iPEdgeAddress					[4] GSNAddress,

chargingID						[5] ChargingID,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

iPCANsessionType				[8] PDPType OPTIONAL,

servedIPCANsessionAddress		[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI		   				[29] IMEI OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servedMNNAI						[36] SubscriptionID OPTIONAL,

iPEdgeOperatorIdentifier		[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

servedIPCANsessionAddressExt 	[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt			[47] DynamicAddressFlag OPTIONAL,

iPEdgeiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission   				[52] NULL OPTIONAL,

servedFixedSubsID				[55] FixedSubsID OPTIONAL,

accessLineIdentifier			[56] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation	[57] FixedUserLocationInformation OPTIONAL

}

EPDGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

ePDGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

ePDGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

uWANUserLocationInformation	[53] UWANUserLocationInfo OPTIONAL,

userLocationInfoTime		[54] TimeStamp OPTIONAL

}

TWAGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

tWAGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

tWAGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

tWANUserLocationInformation	[53] TWANUserLocationInfo OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL,

servingNodePLMNIdentifier	[24] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[25] CNOperatorSelectionEntity OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL,

servingNodePLMNIdentifier	[38] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[39] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMORecord	::= SET

--

--   also for MME UE originated SMS record

--

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL,

servingNodeType				[21] ServingNodeType,

servingNodeAddress			[22] GSNAddress OPTIONAL,

servingNodeiPv6Address		[23] GSNAddress OPTIONAL,

mMEName						[24] DiameterIdentity OPTIONAL,

mMERealm					[25] DiameterIdentity OPTIONAL,

userLocationInformation		[26] OCTET STRING OPTIONAL,

retransmission				[27] NULL OPTIONAL,

servingNodePLMNIdentifier	[28] PLMN-Id OPTIONAL,

userLocationInfoTime		[29] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[30] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMTRecord	::= SET

--

--   also for MME UE terminated SMS record

--

{

recordType					 [0] RecordType,

servedIMSI					 [1] IMSI,

servedIMEI					 [2] IMEI OPTIONAL,

servedMSISDN				 [3] MSISDN OPTIONAL,

msNetworkCapability			 [4] MSNetworkCapability OPTIONAL,

serviceCentre				 [5] AddressString OPTIONAL,

recordingEntity				 [6] RecordingEntity OPTIONAL,

locationArea				 [7] LocationAreaCode OPTIONAL,

routingArea					 [8] RoutingAreaCode OPTIONAL,

cellIdentifier				 [9] CellId OPTIONAL,

eventTimeStamp				 [10] TimeStamp,

smsResult					 [11] SMSResult OPTIONAL,

recordExtensions			 [12] ManagementExtensions OPTIONAL,

nodeID						 [13] NodeID OPTIONAL,

localSequenceNumber			 [14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [15] ChargingCharacteristics,

rATType						 [16] RATType OPTIONAL,

chChSelectionMode			 [17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS			 [18] CAMELInformationSMS OPTIONAL,

originatingAddress			 [19] AddressString OPTIONAL,

servingNodeType				 [20] ServingNodeType,

servingNodeAddress			 [21] GSNAddress OPTIONAL,

servingNodeiPv6Address		 [22] GSNAddress OPTIONAL,

mMEName						 [23] DiameterIdentity OPTIONAL,

mMERealm					 [24] DiameterIdentity OPTIONAL,

userLocationInformation		 [25] OCTET STRING OPTIONAL,

retransmission				 [26] NULL OPTIONAL,

servingNodePLMNIdentifier	 [27] PLMN-Id OPTIONAL,

userLocationInfoTime		 [28] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType,

lcsClientIdentity			 [3] LCSClientIdentity,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationType				 [7] LocationType,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

notificationToMSUser		 [13] NotificationToMSUser OPTIONAL,

privacyOverride				 [14] NULL OPTIONAL,

location					 [15] LocationAreaAndCell OPTIONAL,

routingArea					 [16] RoutingAreaCode OPTIONAL,

locationEstimate			 [17] Ext-GeographicalInformation OPTIONAL,

positioningData				 [18] PositioningData OPTIONAL,

lcsCause					 [19] LCSCause OPTIONAL,

diagnostics					 [20] Diagnostics OPTIONAL,

nodeID						 [21] NodeID OPTIONAL,

localSequenceNumber			 [22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [23] ChargingCharacteristics,

chChSelectionMode			 [24] ChChSelectionMode OPTIONAL,

rATType						 [25] RATType OPTIONAL,

recordExtensions			 [26] ManagementExtensions OPTIONAL,

causeForRecClosing			 [27] CauseForRecClosing,

servingNodePLMNIdentifier	 [28] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMOLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationMethod				 [7] LocationMethod,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNNILCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI OPTIONAL,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

servedIMEI					 [7] IMEI OPTIONAL,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] INTEGER OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

--

--  PS DATA TYPES

--

AccessAvailabilityChangeReason		::= INTEGER (0..4294967295)

--

-- 0 (RAN rule indication)   : This value shall be used to indicate that the availability

--                             of an access is changed due to the RAN rule indication.

-- 1 (Access usable/unusable): This value shall be used to indicate that the availability

--                             of an access is changed due to the access is unusable or usable

--                             again.

--

AccessLineIdentifier	::= SEQUENCE

--

-- "Physical Access Id" includes a port identifier and the identity of the access node where the

--  port resides. "logical Access Id" contains a Circuit-ID. Both are defined ETSI TS 283 034 [314]

--

{

physicalAccessID	[0] UTF8String OPTIONAL,

logicalAccessID		[1] OCTET STRING OPTIONAL

}

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

ADCRuleBaseName		::= IA5String

--

-- identifier for the group of charging rules

-- see ADC-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

AdditionalExceptionReports		::= ENUMERATED

{

notAllowed		(0),

allowed			(1)

}

AFChargingIdentifier	::= OCTET STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNRateControl		::= SEQUENCE

--

-- See TS 24.008 [208] for more information

--

{

aPNRateControlUplink	[0] APNRateControlParameters OPTIONAL,

aPNRateControlDownlink	[1] APNRateControlParameters OPTIONAL

}

APNRateControlParameters		::= SEQUENCE

{

additionalExceptionReports	[0] AdditionalExceptionReports OPTIONAL,

rateControlTimeUnit			[1] RateControlTimeUnit OPTIONAL,

rateControlMaxRate			[2] INTEGER OPTIONAL,

rateControlMaxMessageSize	[3] DataVolumeGPRS OPTIONAL -- aPNRateControlDownlink only

}

APNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7] FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8] FreeFormatData OPTIONAL,

fFDAppendIndicator				[9] FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2] ServiceKey OPTIONAL,

defaultSMSHandling						[3] DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7] FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

ChangeCondition	::= ENUMERATED

{

qoSChange						(0),

tariffTime						(1),

recordClosure					(2),

cGI-SAICHange					(6),	-- bearer modification. "CGI-SAI Change"

rAIChange						(7),	-- bearer modification. "RAI Change"

dT-Establishment				(8),

dT-Removal						(9),

eCGIChange						(10),	-- bearer modification. "ECGI Change"

tAIChange						(11),	-- bearer modification. "TAI Change"

userLocationChange				(12),	-- bearer modification. "User Location Change"

userCSGInformationChange		(13), 	-- bearer modification. "User CSG info Change"

presenceInPRAChange				(14),	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

removalOfAccess					(15),	-- NBIFOM "Removal of Access"

unusabilityOfAccess				(16),	-- NBIFOM "Unusability of Access"

indirectChangeCondition			(17),	-- NBIFOM "Indirect Change Condition"

userPlaneToUEChange				(18),	-- bearer modification. "Change of user plane to UE"

servingPLMNRateControlChange	(19) ,

-- bearer modification “Serving PLMN Rate Control Change"

threeGPPPSDataOffStatusChange	(20)   -- "Change of 3GPP PS DataO ff Status"

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR,PGW-CDR, IPE-CDR, TWAG-CDR and ePDG-CDR only

-- userLocationInformation is used only in S-CDR, SGW-CDR and PGW-CDR

-- chargingID used in PGW-CDR only when Charging per IP-CAN session is active

-- accessAvailabilityChangeReason and relatedChangeOfCharCondition applicable only in PGW-CDR

-- cPCIoTOptimisationIndicator is used in SGW-CDR only

--

{

qosRequested					[1] QoSInformation OPTIONAL,

qosNegotiated					[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink			[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink			[4] DataVolumeGPRS OPTIONAL,

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

ePCQoSInformation				[9] EPCQoSInformation OPTIONAL,

chargingID						[10] ChargingID OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

diagnostics						[13] Diagnostics OPTIONAL,

enhancedDiagnostics				[14] EnhancedDiagnostics OPTIONAL,

rATType							[15] RATType OPTIONAL,

accessAvailabilityChangeReason	[16] AccessAvailabilityChangeReason OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL,

relatedChangeOfCharCondition	[18] RelatedChangeOfCharCondition OPTIONAL,

cPCIoTEPSOptimisationIndicator	[19] CPCIoTEPSOptimisationIndicator OPTIONAL,

servingPLMNRateControl			[20] ServingPLMNRateControl OPTIONAL,

threeGPPPSDataOffStatus			[21] ThreeGPPPSDataOffStatus OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging and Application based Charging service data container

-- presenceReportingAreaStatus is used in PGW-CDR Only

--

{

ratingGroup 						[1] RatingGroupId,

chargingRuleBaseName				[2] ChargingRuleBaseName OPTIONAL,

resultCode							[3] ResultCode OPTIONAL,

localSequenceNumber					[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[5] TimeStamp OPTIONAL,

timeOfLastUsage						[6] TimeStamp OPTIONAL,

timeUsage 							[7] CallDuration OPTIONAL,

serviceConditionChange				[8] ServiceConditionChange,

qoSInformationNeg					[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 					[10] GSNAddress OPTIONAL,

datavolumeFBCUplink					[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink				[13] DataVolumeGPRS OPTIONAL,

timeOfReport						[14] TimeStamp,

failureHandlingContinue				[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier					[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation		[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation					[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation				[20] OCTET STRING OPTIONAL,

eventBasedChargingInformation		[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism					[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo					[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

sponsorIdentity						[25] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[26] OCTET STRING OPTIONAL,

aDCRuleBaseName						[27] ADCRuleBaseName OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedChangeOfServiceCondition		[33] RelatedChangeOfServiceCondition OPTIONAL,

servingPLMNRateControl				[35] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[36] APNRateControl OPTIONAL,

threeGPPPSDataOffStatus             [37] ThreeGPPPSDataOffStatus OPTIONAL,

trafficSteeringPolicyIDDownlink     [38] TrafficSteeringPolicyIDDownlink OPTIONAL,

trafficSteeringPolicyIDUplink       [39] TrafficSteeringPolicyIDUplink OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP-CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

ChargingPerIPCANSessionIndicator	::= ENUMERATED

{

inactive		(0),

active               (1)

}

ChargingRuleBaseName	::= IA5String

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

roamingDefault				(4),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

visitingDefault				(5),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

fixedDefault				(6)		-- For TDF and IP-Edge

}

CNOperatorSelectionEntity	::= ENUMERATED

{

servCNSelectedbyUE		(0),

servCNSelectedbyNtw		(1)

}

CPCIoTEPSOptimisationIndicator ::= BOOLEAN

CSGAccessMode		::= ENUMERATED

{

closedMode  (0),

hybridMode  (1)

}

CSGId		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 23.003 [200]. Coded according to TS 29.060 [215] for GTP, and

-- in TS 29.274 [223] for eGTP.

--

CTEID		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier

-- of MBMS GW for user plane, defined in TS 23.246 [207].

--

DataVolumeGPRS		::= INTEGER

--

-- The volume of data transferred in octets.

--

DataVolumeMBMS ::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI							[1] INTEGER,

maxRequestedBandwithUL		[2] INTEGER OPTIONAL,

maxRequestedBandwithDL		[3] INTEGER OPTIONAL,

guaranteedBitrateUL			[4] INTEGER OPTIONAL,

guaranteedBitrateDL			[5] INTEGER OPTIONAL,

aRP							[6] INTEGER OPTIONAL,

aPNAggregateMaxBitrateUL	[7] INTEGER OPTIONAL,

aPNAggregateMaxBitrateDL	[8] INTEGER OPTIONAL

}

EventBasedChargingInformation		::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue		::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FixedSubsID	::= OCTET STRING

--

-- The fixed subscriber Id identifier is defined in Broadband Forum TR 134 [601].

--

FixedUserLocationInformation	::= SEQUENCE

--

-- See format in IEEE Std 802.11-2012 [408] for "SSID" and "BSSID".

--

{

sSID					[0] OCTET STRING OPTIONAL ,

bSSID					[1] OCTET STRING OPTIONAL,

accessLineIdentifier	[2] AccessLineIdentifier OPTIONAL

}

Flows		::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData		::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress		::= IPAddress

MOExceptionDataCounter		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

counterValue		[0] INTEGER,

counterTimestamp	[1] TimeStamp

}

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NBIFOMMode		::= ENUMERATED

{

uEINITIATED				(0),

nETWORKINITIATED		(1)

}

NBIFOMSupport 	::= ENUMERATED

{

nBIFOMNotSupported			(0),

nBIFOMSupported				(1)

}

NetworkInitiatedPDPContext		::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NodeID		::= IA5String (SIZE(1..20))

NumberOfDPEncountered  ::= INTEGER

PDPAddress		::= CHOICE

{

iPAddress				[0] IPAddress

--	eTSIAddress				[1] ETSIAddress

--								has only been used in earlier releases for X.121 format

}

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for encoding details.

--

PDPPDNTypeExtension	::= INTEGER

--

-- This integer is 1:1 copy of the PDP type value as defined in TS 29.061 [215].

--

PLMN-Id		::= OCTET STRING (SIZE (3))

--

--	This is in the same format as octets 2,3,and 4 of the Routing Area Identity (RAI) IE specified

--	in TS 29.060 [215]

--

PresenceReportingAreaElementsList ::= OCTET STRING

--

-- See Presence-Reporting-Area-Elements-List AVP defined in TS 29.212 [220]

--

PresenceReportingAreaInfo	::= SEQUENCE

{

presenceReportingAreaIdentifier	 [0] OCTET STRING,

presenceReportingAreaStatus		 [1] PresenceReportingAreaStatus OPTIONAL,

presenceReportingAreaElementsList[2] PresenceReportingAreaElementsList OPTIONAL

}

PresenceReportingAreaStatus ::= ENUMERATED

{

insideArea   (0),

outsideArea  (1) ,

inactive 	 (2)

}

PSFurnishChargingInformation		::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RateControlTimeUnit ::= INTEGER

{	unrestricted	(0),

minute			(1),

hour			(2),

day				(3),

week			(4)

}

RatingGroupId		::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0... 4294967295)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RATType		::= INTEGER (0..255)

--

-- Ihis integer is 1:1 copy of the RAT type value as defined in TS 29.061 [215].

--

RelatedChangeOfCharCondition	::= SEQUENCE

{

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

rATType							[15] RATType OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL

}

RelatedChangeOfServiceCondition	::= SEQUENCE

{

userLocationInformation				[20] OCTET STRING OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedServiceConditionChange		[33] ServiceConditionChange OPTIONAL

}

ResultCode		::= INTEGER

--

-- charging protocol return value, range of 4 byte (0... 4294967295)

-- see Result-Code AVP as used in 32.299 [40]

--

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1),	-- bearer modification:

-- apply to Gn-SGSN /SGW Change

sGSNPLMNIDChange 						 (2),	-- bearer modification

tariffTimeSwitch 						 (3),	-- tariff time change

pDPContextRelease 						 (4),	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9),	-- IP flow termination.From "Service Stop" in

-- Change-Condition AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11),	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12),	-- DCCA quota reauthorization

dCCATimeExhausted 						(13),	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14),	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15),	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17),	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18),	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19),	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20),	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21),	-- bearer modification. "CGI-SAI Change"

rAIChange								(22),	-- bearer modification. "RAI Change"

dCCAServiceSpecificUnitExhausted		(23),	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From "Service Data

--	Time Limit" Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From "Service Data

--	Volume Limit" Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29),	-- bearer modification. "ECGI Change"

tAIChange								(30),	-- bearer modification. "TAI Change"

userLocationChange						(31),	-- bearer modification. "User Location Change"

userCSGInformationChange				(32),	-- bearer modification. "User CSG info Change"

presenceInPRAChange						(33), 	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

accessChangeOfSDF						(34),	-- "access change of service data flow"

indirectServiceConditionChange			(35),	-- NBIFOM: "indirect service condition change"

servingPLMNRateControlChange			(36), 	-- bearer modification. "Serving PLMNRate

-- Control Change"

aPNRateControlChange					(37)	-- bearer modification. "APN Rate ControlChange

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5),

tWAN		(6)

}

ServingPLMNRateControl		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

sPLMNDLRateControlValue	[0] INTEGER,

sPLMNULRateControlValue	[1] INTEGER

}

SGiPtPTunnellingMethod		::= ENUMERATED

{

uDPIPbased		(0),

others			(1)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange		::= BOOLEAN

--

-- present if first record after inter serving node change (SGW, ePDG, TWAG, HSGW)

--

TimeQuotaMechanism		::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] INTEGER

}

TimeQuotaType		::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

TrafficSteeringPolicyIDDownlink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-DL AVP as defined in TS 29.212[220]

TrafficSteeringPolicyIDUplink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-UL AVP as defined in TS 29.212[220]

TWANUserLocationInfo		::= SEQUENCE

{

sSID		[0] OCTET STRING,					-- see format in IEEE Std 802.11-2012 [408]

bSSID		[1] OCTET STRING OPTIONAL			-- see format in IEEE Std 802.11-2012 [408]

}

UNIPDUCPOnlyFlag	::= BOOLEAN

UserCSGInformation		::= SEQUENCE

{

cSGId						[0] CSGId,

cSGAccessMode				[1] CSGAccessMode,

cSGMembershipIndication		[2] NULL OPTIONAL

}

UWANUserLocationInfo		::= SEQUENCE

{

uELocalIPAddress	[0] IPAddress,

uDPSourcePort		[1] OCTET STRING (SIZE(2)) OPTIONAL,

sSID				[2] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

bSSID				[3] OCTET STRING OPTIONAL	-- see format in IEEE Std 802.11-2012 [408]

}

.#END

#### 5.2.2.3	Void

#### 5.2.2.4	CP data transfer domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CP data transfer CDR types defined in TS 32.253 [13].

.$CPDTChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) cpdtChargingDataTypes (13) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

- BEGIN

-- EXPORTS everything

- IMPORTS

CallDuration,

DiameterIdentity,

Diagnostics,

LocalSequenceNumber,

ManagementExtensions,

MSISDN,

RecordType,

ServiceContextID,

- SubscriptionID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

IMEI,

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version15 (15)}

-- from TS 29.002 [214]

AccessPointNameNI,

APNRateControl,

ChargingID,

ChargingCharacteristics,

ChChSelectionMode,

DataVolumeGPRS,

NodeID,

PLMN-Id,

ServingPLMNRateControl,

- RATType

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

;

--

--  CP data transfer RECORDS

--

CPDTRecord		::= CHOICE

--

-- Record values 105 to 106 are CP data transfer specific

--

{

cPDTSCERecord			[105] 	CPDTSCERecord,

cPDTSNNRecord			[106] 	CPDTSNNRecord

}

CPDTSCERecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

servedIMSI						[2] IMSI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

chargingID						[4] ChargingID,

serviceContextID				[5] ServiceContextID OPTIONAL,

nodeID							[6] NodeID OPTIONAL,

recordOpeningTime				[7] TimeStamp,

duration						[8] CallDuration,

accessPointNameNI				[9] AccessPointNameNI OPTIONAL,

sCEFID							[10] DiameterIdentity OPTIONAL,

chargingCharacteristics			[11] ChargingCharacteristics,

chChSelectionMode				[12] ChChSelectionMode OPTIONAL,

servingNodeIdentity				[13] DiameterIdentity OPTIONAL,

servingPLMNRateControl			[14] ServingPLMNRateControl OPTIONAL,

listOfNIDDsubmission			[15] SEQUENCE OF NIDDsubmission OPTIONAL,

causeForRecClosing				[16] CPCauseForRecClosing,

diagnostics						[17] Diagnostics OPTIONAL,

localSequenceNumber				[18] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[19] INTEGER OPTIONAL,

recordExtensions				[20] ManagementExtensions OPTIONAL,

externalIdentifier				[21] SubscriptionID OPTIONAL,

aPNRateControl					[22] APNRateControl OPTIONAL,

rATType							[23] RATType OPTIONAL,

servingNodePLMNIdentifier		[24] PLMN-Id OPTIONAL,

servedIMEI						[25] IMEI OPTIONAL

}

CPDTSNNRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

servedIMSI						[2] IMSI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

chargingID						[4] ChargingID,

serviceContextID				[5] ServiceContextID OPTIONAL,

nodeID							[6] NodeID OPTIONAL,

recordOpeningTime				[7] TimeStamp,

duration						[8] CallDuration,

accessPointNameNI				[9] AccessPointNameNI OPTIONAL,

sCEFID							[10] DiameterIdentity OPTIONAL,

chargingCharacteristics			[11] ChargingCharacteristics,

chChSelectionMode				[12] ChChSelectionMode OPTIONAL,

servingNodeIdentity				[13] DiameterIdentity OPTIONAL,

servingPLMNRateControl			[14] ServingPLMNRateControl OPTIONAL,

listOfNIDDsubmission			[15] SEQUENCE OF NIDDsubmission OPTIONAL,

causeForRecClosing				[16] CPCauseForRecClosing,

diagnostics						[17] Diagnostics OPTIONAL,

localSequenceNumber				[18] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[19] INTEGER OPTIONAL,

recordExtensions				[20] ManagementExtensions OPTIONAL,

externalIdentifier				[21] SubscriptionID OPTIONAL,

aPNRateControl					[22] APNRateControl OPTIONAL,

rATType							[23] RATType OPTIONAL,

servingNodePLMNIdentifier		[24] PLMN-Id OPTIONAL,

servedIMEI						[25] IMEI OPTIONAL

}

--

--  CP data transfer DATA TYPES

--

CPCauseForRecClosing	::= INTEGER

{

normalRelease					(0),

abnormalRelease					(1),

volumeLimit						(2),

timeLimit						(3),

maxNIDDsubmissions				(4),

servingNodeChange				(5),

pLMNChange						(6),

servingPLMNRateControlChange	(7),

aPNRateControlChange			(8),

rATTypeChange					(9),

managementIntervention			(10)

}

NIDDsubmission			::= SEQUENCE

{

submissionTimestamp			[0] TimeStamp OPTIONAL,

eventTimestamp				[1] TimeStamp OPTIONAL,

dataVolumeGPRSUplink		[2] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink		[3] DataVolumeGPRS OPTIONAL,

submissionResultCode		[4] SubmissionResultCode OPTIONAL,

serviceChangeCondition		[5] ServiceChangeCondition OPTIONAL

}

ServiceChangeCondition	::= BIT STRING

{

nIDDsubmissionResponseReceipt		(0),

nIDDsubmissionResponseSending	(1),

nIDDdeliveryToUE				(2),

nIDDdeliveryFromUEerror			(3),

nIDDsubmissionTimeout			(4)

}

SubmissionResultCode		::= INTEGER

--

-- Result-Code AVP and Experimental-Result AVP Values as specified in TS 29.128 [244]

-- for MO/MT data transfer

--

.#END

### 5.2.3	Subsystem level CDR definitions

#### 5.2.3.0	Introduction

This subclause contains the syntax definitions of the CDRs on the subsystem level. At present, only the IM subsystem is defined in 3GPP, thus this subclause comprises the CDR types specified for the IMS in TS 32.260 [20].

#### 5.2.3.1	IMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.260 [20].

.$IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

NodeAddress,

MSCAddress,

MSTimeZone,

RecordType,

ServiceContextID,

ServiceSpecificInfo,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

;

--

--  IMS RECORDS

--

IMSRecord ::= CHOICE

--

-- Record values 63-69, 82, 89, ,90, 91 are IMS specific

--

{

sCSCFRecord			[63] SCSCFRecord,

pCSCFRecord			[64] PCSCFRecord,

iCSCFRecord			[65] ICSCFRecord,

mRFCRecord			[66] MRFCRecord,

mGCFRecord			[67] MGCFRecord,

bGCFRecord			[68] BGCFRecord,

aSRecord				[69] ASRecord,

eCSCFRecord			[70] ECSCFRecord,

iBCFRecord			[82] IBCFRecord,

tRFRecord			[89] TRFRecord,

tFRecord				[90] TFRecord,

aTCFRecord			[91] ATCFRecord

}

SCSCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

nNI-Information     						[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator					[52] NULL OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

PCSCFRecord		::= SET

--

-- This record is also applicable for P-CSCF with collocated ATCF

-- ATCF-specific fields which are not applicable to P-CSCF are indicated.

--

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

sIP-Method								[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress								[4] NodeAddress OPTIONAL,

session-Id								[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address			[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID							[8] GraphicString OPTIONAL,

serviceRequestTimeStamp					[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp			[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp				[11] TimeStamp OPTIONAL,

recordOpeningTime						[12] TimeStamp OPTIONAL,

recordClosureTime						[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber				[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing					[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication				[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier					[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress								[22] NodeAddress OPTIONAL,

serviceReasonReturnCode					[23] UTF8String OPTIONAL,

list-Of-Message-Bodies					[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation						[26] INTEGER OPTIONAL,

list-Of-Associated-URI					[27] ListOfInvolvedParties OPTIONAL,

event									[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID					[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 		[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier		[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL, -- ATCF only

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

servedPartyIPAddress					[50] ServedPartyIPAddress OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator					[52] NULL OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,
	list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL, -- ATCF only

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

-- ATCF only

relatedICID								[107] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode				[108] NodeAddress OPTIONAL,

fEIdentifierList                        [109] FEIdentifierList OPTIONAL

}

ICSCFRecord		::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority					[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction	[37] Milliseconds OPTIONAL,

transit-IOI-List					[45] GraphicString OPTIONAL,

userLocationInformation			[47] OCTET STRING OPTIONAL,

mSTimeZone 						[48] MSTimeZone OPTIONAL,

fromAddress						[51] OCTET STRING OPTIONAL,

iMSEmergencyIndicator			[52] NULL OPTIONAL,

listOfReasonHeader				[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation	[56] OCTET STRING OPTIONAL,

routeHeaderTransmitted				[60] OCTET STRING OPTIONAL,

s-CSCF-Information					[61] S-CSCF-Information OPTIONAL,

cellularNetworkInformation			[64] OCTET STRING OPTIONAL,

fEIdentifierList                    [65] FEIdentifierList OPTIONAL

}

MRFCRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,
	list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

service-Id								[70] Service-Id OPTIONAL,

requested-Party-Address					[71] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[72] ListOfInvolvedParties OPTIONAL,

fEIdentifierList                        [73] FEIdentifierList OPTIONAL

}

MGCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL, 
	list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

trunkGroupID								[80] TrunkGroupID OPTIONAL,

bearerService							[81] TransmissionMedium OPTIONAL,

iSUPCause								[82] ISUPCause OPTIONAL,

fEIdentifierList                        [83] FEIdentifierList OPTIONAL

}

BGCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

transit-IOI-List							[45] GraphicString OPTIONAL,

nNI-Information     						[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

ASRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting						[35] CarrierSelectRouting OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL,

online-charging-flag						[43] NULL OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

serviceSpecificInfo						[100] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

requested-Party-Address					[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[102] ListOfInvolvedParties OPTIONAL,

alternateChargedPartyAddress				[103] UTF8String OPTIONAL,

outgoingSessionId						[104] Session-Id OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL,

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

tADS-Identifier							[109] TADIdentifier OPTIONAL,

vlr-Number								[110] MSCAddress OPTIONAL,

msc-Address								[111] MSCAddress OPTIONAL,

threeGPPPSDataOffStatus					[112] ThreeGPPPSDataOffStatus OPTIONAL,

fEIdentifierList                        [113] FEIdentifierList OPTIONAL

}

ECSCFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

fEIdentifierList                        [65] FEIdentifierList OPTIONAL

}

IBCFRecord		::= SET

--

-- This record is also applicable for IBCF with collocated ATCF

-- ATCF-specific fields which are not applicable to IBCF are indicated.

--

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

sIP-Method								[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress								[4] NodeAddress OPTIONAL,

session-Id								[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address			[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID							[8] GraphicString OPTIONAL, -- ATCF only

serviceRequestTimeStamp					[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp			[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp				[11] TimeStamp OPTIONAL,

recordOpeningTime						[12] TimeStamp OPTIONAL,

recordClosureTime						[13] TimeStamp OPTIONAL,

interOperatorIdentifiers					[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[	15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber						[16] INTEGER OPTIONAL,

causeForRecordClosing					[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication				[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier					[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress								[22] NodeAddress OPTIONAL, -- ATCF only

serviceReasonReturnCode					[23] UTF8String OPTIONAL,

list-Of-Message-Bodies					[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation						[26] INTEGER OPTIONAL,

event									[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID					[31] SEQUENCE OF SubscriptionID OPTIONAL, -- ATCF only

list-Of-Early-SDP-Media-Components 		[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier		[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority							[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL, -- ATCF only

list-Of-Called-Asserted-Identity		[42] ListOfInvolvedParties OPTIONAL, -- ATCF only	realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

transit-IOI-List						[45] GraphicString OPTIONAL,

nNI-Information							[46] SEQUENCE OF NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL, 
	list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL, -- ATCF only

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

-- ATCF only

fEIdentifierList                        [107] FEIdentifierList OPTIONAL

}

TRFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction			[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] SEQUENCE OF NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone 								[48] MSTimeZone OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

fEIdentifierList                        [64] FEIdentifierList OPTIONAL

}

ATCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation				[29] OCTET STRING OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

list-of-Requested-Party-Address			[41] ListOfInvolvedParties OPTIONAL,

list-Of-Called-Asserted-Identity			[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

userLocationInformation					[47] OCTET STRING OPTIONAL,

mSTimeZone								[48] MSTimeZone OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL, 
	list-Of-AccessNetworkInfoChange			[62] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

initialIMS-Charging-Identifier			[105] IMS-Charging-Identifier OPTIONAL,

list-Of-AccessTransferInformation		[106] SEQUENCE OF AccessTransferInformation OPTIONAL,

fEIdentifierList                        [107] FEIdentifierList OPTIONAL

}

TFRecord		::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node							[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address					[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers				[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber					[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components			[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions						[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

serviceContextID						[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting				[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting					[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

applicationServersInformation			[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address					[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 		[42] ListOfInvolvedParties OPTIONAL,

nNI-Information							[46] NNI-Information OPTIONAL,

fromAddress								[51] OCTET STRING OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

routeHeaderReceived						[59] OCTET STRING OPTIONAL,

routeHeaderTransmitted					[60] OCTET STRING OPTIONAL,

listOfCalledIdentityChanges				[63] SEQUENCE OF CalledIdentityChange OPTIONAL,

fEIdentifierList                        [64] FEIdentifierList OPTIONAL

}

--

--  IMS DATA TYPES

--

AccessCorrelationID		::= CHOICE

{

gPRS-Charging-Id					[2] INTEGER (0..4294967295),

accessNetworkChargingIdentifier	[4] GraphicString

}

AccessNetworkInfoChange		::= SEQUENCE

{

accessNetworkInformation 			 [0] OCTET STRING OPTIONAL,

additionalAccessNetworkInformation	 [1] OCTET STRING OPTIONAL,

accessChangeTime                 	 [2] TimeStamp OPTIONAL,

cellularNetworkInformation			 [3] OCTET STRING OPTIONAL

}

AccessTransferType	::= ENUMERATED

{

pSToCS (0),

cSToPS (1) ,

pSToPS (2),

cSToCS (3)

}

AccessTransferInformation		::= SEQUENCE

{

accessTransferType 		 			 [0] AccessTransferType OPTIONAL,

accessNetworkInformation 			 [1] OCTET STRING OPTIONAL,

additionalAccessNetworkInformation	 [2] OCTET STRING OPTIONAL,

inter-UE-Transfer					 [3] NULL OPTIONAL,

relatedICID							 [4] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode			 [5] NodeAddress OPTIONAL,

accessTransferTime                 	 [6] TimeStamp OPTIONAL,

subscriberEquipmentNumber			 [7] SubscriberEquipmentNumber OPTIONAL,

instanceId							 [8] OCTET STRING OPTIONAL,

cellularNetworkInformation			 [9] OCTET STRING OPTIONAL

}

ACRInterimLost		::= ENUMERATED

{

no		(0),

yes		(1),

unknown	(2)

}

AoCCostInformation		::= SEQUENCE

{

accumulatedCost		[0] REAL,

incrementalCost		[1] REAL,

currencyCode			[2] INTEGER

}

AoCInformation ::= SET

{

tariffInformation		[0] TariffInformation OPTIONAL,

aoCCostInformation		[1] AoCCostInformation OPTIONAL

}

ApplicationServersInformation	::= SEQUENCE

{

applicationServersInvolved 			[0] NodeAddress OPTIONAL,

applicationProvidedCalledParties		[1] SEQUENCE OF InvolvedParty OPTIONAL,

sTatus								[2] Status OPTIONAL

}

CalledIdentityChange	::= SEQUENCE

{

calledIdentity	[0]	InvolvedParty OPTIONAL,

changeTime		[1] TimeStamp OPTIONAL

}

CarrierSelectRouting	::= GraphicString

CauseForRecordClosing	::= ENUMERATED

{

serviceDeliveryEndSuccessfully	(0),

unSuccessfulServiceDelivery		(1),

timeLimit						(3),

serviceChange					(4), -- e.g. change in media due to Re-Invite,

-- Access Transfer

managementIntervention			(5)  -- partial record generation reasons to be added

--  Additional codes are for further study

}

- Early-Media-Components-List	::= SEQUENCE

{

sDP-Offer-Timestamp 		[0] TimeStamp OPTIONAL,

sDP-Answer-Timestamp		[1] TimeStamp OPTIONAL,

sDP-Media-Components		[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag		[3] NULL OPTIONAL,

sDP-Session-Description	[4] SEQUENCE OF GraphicString OPTIONAL,

sDP-Type					[5] SDP-Type OPTIONAL

}

FEIdentifierList ::= SEQUENCE OF GraphicString

IMS-Charging-Identifier	::= OCTET STRING

IMSCommunicationServiceIdentifier	::= OCTET STRING

Incomplete-CDR-Indication	::= SET

{

aCRStartLost		[0] BOOLEAN,		-- TRUE if ACR[Start] was lost, FALSE otherwise

aCRInterimLost	[1] ACRInterimLost,

aCRStopLost		[2] BOOLEAN			-- TRUE if ACR[Stop] was lost, FALSE otherwise

}

InterOperatorIdentifierList ::= SEQUENCE OF InterOperatorIdentifiers

InterOperatorIdentifiers ::= SEQUENCE

{

originatingIOI	[0] GraphicString OPTIONAL,

terminatingIOI	[1] GraphicString OPTIONAL

}

InvolvedParty ::= CHOICE

{

sIP-URI		[0] GraphicString, -- refer to rfc3261 [401]

tEL-URI		[1] GraphicString,	-- refer to rfc3966 [402]

uRN		[2] GraphicString,	-- refer to rfc5031 [407]

iSDN-E164 	[3] GraphicString	-- refer to ITU-T Recommendation E.164[308]

}

ISUPCause	::= SEQUENCE

{

iSUPCauseLocation 		[0] INTEGER OPTIONAL,

iSUPCauseValue			[1] INTEGER OPTIONAL,

iSUPCauseDiagnostics		[2] OCTET STRING OPTIONAL

}

ListOfInvolvedParties	::= SEQUENCE OF InvolvedParty

ListOfReasonHeader	::= SEQUENCE OF ReasonHeaderInformation

Media-Components-List	::= SEQUENCE

--

--	MediaInitiatorParty is used to identify the initiator of the media

--	multi-participants session e.g. in AS PoC Server

--

{

sIP-Request-Timestamp 			[0] TimeStamp OPTIONAL,

sIP-Response-Timestamp			[1] TimeStamp OPTIONAL,

sDP-Media-Components			[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag				[3] NULL OPTIONAL,

sDP-Session-Description			[4] SEQUENCE OF GraphicString OPTIONAL,

mediaInitiatorParty				[5] InvolvedParty OPTIONAL,

sIP-Request-Timestamp-Fraction	[6] Milliseconds OPTIONAL,

sIP-Response-Timestamp-Fraction	[7] Milliseconds OPTIONAL,

sDP-Type						[8] SDP-Type OPTIONAL

}

MessageBody	::= SEQUENCE

{

content-Type				[0] GraphicString,

content-Disposition		[1] GraphicString OPTIONAL,

content-Length			[2] INTEGER,

originator				[3] InvolvedParty OPTIONAL

}

Milliseconds	::= INTEGER (0..999)

NNI-Information		::= SEQUENCE

{

sessionDirection		[0] SessionDirection OPTIONAL,

nNIType				[1] NNIType OPTIONAL,

relationshipMode		[2] RelationshipMode OPTIONAL,

neighbourNodeAddress	[3] IPAddress OPTIONAL

}

NNIType		::= ENUMERATED

{

non-roaming					(0),

roaming-without-loopback		(1),

roaming-with-loopback		(2)

}

NumberPortabilityRouting		::= GraphicString

RateElement		::= SEQUENCE

{

unitType				[0] INTEGER,

unitValue			[1] REAL,

unitCost				[2] REAL,

unitQuotaThreshold	[3] REAL

}

RealTimeTariffInformation ::= CHOICE

{

tariffInformation		[0] TariffInformation,

tariffXml				[1] UTF8String

}

ReasonHeaderInformation ::= GraphicString

RelationshipMode ::= ENUMERATED

{

trusted		(0),

non-trusted	(1)

}

Role-of-Node	::= ENUMERATED

{

originating	(0),

terminating	(1)

}

S-CSCF-Information	::= SEQUENCE

{

mandatoryCapabilities 	[0] SEQUENCE OF GraphicString OPTIONAL,

optionalCapabilities	[1] SEQUENCE OF GraphicString OPTIONAL,

serverName 				[2] GraphicString OPTIONAL

}

SDP-Media-Component ::=  SEQUENCE

{

sDP-Media-Name        			[0] GraphicString OPTIONAL,

sDP-Media-Descriptions			[1] SDP-Media-Description OPTIONAL,

accessCorrelationID					AccessCorrelationID OPTIONAL,	-- not used in MGCF

-- [2] is used by gPRS-Charging-Id

-- [4] is used by accessNetworkChargingIdentifier

localGWInsertedIndication		[5] BOOLEAN OPTIONAL,

iPRealmDefaultIndication		[6] BOOLEAN OPTIONAL,

transcoderInsertedIndication	[7] BOOLEAN OPTIONAL

}

SDP-Media-Description	::= SEQUENCE OF GraphicString

ServedPartyIPAddress	::=  IPAddress

Service-Id	::= GraphicString

Session-Id	::= GraphicString

--

-- rfc3261 [401]: example for SIP CALL-ID: f81d4fae-7dec-11d0-a765-00a0c91e6bf6@foo.bar.com

--

SessionDirection	::= ENUMERATED

{

inbound		(0),

outbound		(1)

}

SessionPriority	::= ENUMERATED

--

-- PRIORITY-4 is the highest priority and Priority-0 is the lowest priority.

--

{

pRIORITY-0 (0),

pRIORITY-1 (1),

pRIORITY-2 (2),

pRIORITY-3 (3),

pRIORITY-4 (4)

}

SIP-Method	::= GraphicString

SDP-Type	::= ENUMERATED

{

sDP-offer	(0),

sDP-answer	(1)

}

- Status		::= ENUMERATED

{

fourxx			(0),

fivexx			(1),

time-out			(2)

}

TADIdentifier		::= ENUMERATED

{

cS		(0),

pS		(1)

}

TariffInformation ::= SEQUENCE

{

currencyCode			[0] INTEGER,

scaleFactor			[1] REAL,

rateElements			[2] SEQUENCE OF RateElement OPTIONAL

}

TransitIOILists ::= SEQUENCE OF GraphicString

TransmissionMedium ::= SEQUENCE

{

tMR	[0] OCTET STRING (SIZE (1)) OPTIONAL, -- required TM, refer to Q.763

tMU	[1] OCTET STRING (SIZE (1)) OPTIONAL  -- used TM, refer to Q.763

}

TrunkGroupID ::= CHOICE

{

incoming	[0] GraphicString,

outgoing	[1] GraphicString

}

.#END

### 5.2.4	Service level CDR definitions

#### 5.2.4.0	General

This subclause contains the syntax definitions of the CDRs on the service level. This comprises the CDR types from the MMS (TS 32.270 [30]), the LCS (TS 32.271 [31]), PoC (TS 32.272 [32]), MBMS (TS 32.273 [33]), and MMTel (TS 32.275 [35]) services.

#### 5.2.4.1	MMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.270 [30].

.$MMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mmsChargingDataTypes (5) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

GSNAddress,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MscNo,

MSISDN,

MSTimeZone,

RecordType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

ChargingID,

PLMN-Id,

RATType

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13)  version15 (15)}

-- from TS 29.002 [214]

;

--

--  MMS RECORDS

--

MMSRecordType	::= CHOICE

--

-- Record values 30..62 are MMS specific

--

{

mMO1SRecord				[30] MMO1SRecord,

mMO4FRqRecord			[31] MMO4FRqRecord,

mMO4FRsRecord			[32] MMO4FRsRecord,

mMO4DRecord				[33] MMO4DRecord,

mMO1DRecord				[34] MMO1DRecord,

mMO4RRecord				[35] MMO4RRecord,

mMO1RRecord				[36] MMO1RRecord,

mMOMDRecord				[37] MMOMDRecord,

mMR4FRecord				[38] MMR4FRecord,

mMR1NRqRecord			[39] MMR1NRqRecord,

mMR1NRsRecord			[40] MMR1NRsRecord,

mMR1RtRqRecord			[41] MMR1RtRecord,

mMR1ARecord				[42] MMR1ARecord,

mMR4DRqRecord			[43] MMR4DRqRecord,

mMR4DRsRecord			[44] MMR4DRsRecord,

mMR1RRRecord			[45] MMR1RRRecord,

mMR4RRqRecord			[46] MMR4RRqRecord,

mMR4RRsRecord			[47] MMR4RRsRecord,

mMRMDRecord				[48] MMRMDRecord,

mMFRecord				[49] MMFRecord,

mMBx1SRecord			[50] MMBx1SRecord,

mMBx1VRecord			[51] MMBx1VRecord,

mMBx1URecord			[52] MMBx1URecord,

mMBx1DRecord			[53] MMBx1DRecord,

mM7SRecord				[54] MM7SRecord,

mM7DRqRecord			[55] MM7DRqRecord,

mM7DRsRecord			[56] MM7DRsRecord,

mM7CRecord				[57] MM7CRecord,

mM7RRecord				[58] MM7RRecord,

mM7DRRqRecord			[59] MM7DRRqRecord,

mM7DRRsRecord			[60] MM7DRRsRecord,

mM7RRqRecord			[61] MM7RRqRecord,

mM7RRsRecord			[62] MM7RRsRecord

}

MMO1SRecord	::= SET

{

recordType					[0] RecordType,

originatorMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

originatorAddress			[4] MMSAgentAddress,

recipientAddresses			[5] MMSAgentAddresses,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

contentType					[7] ContentType,

mmComponentType				[8] MMComponentType OPTIONAL,

messageSize					[9] DataVolume,

messageClass				[10] MessageClass OPTIONAL,

chargeInformation			[11] ChargeInformation OPTIONAL,

submissionTime				[12] TimeStamp OPTIONAL,

timeOfExpiry				[13] WaitTime OPTIONAL,

earliestTimeOfDelivery		[14] WaitTime OPTIONAL,

durationOfTransmission		[15] INTEGER OPTIONAL,

requestStatusCode			[16] RequestStatusCodeType OPTIONAL,

deliveryReportRequested		[17] BOOLEAN OPTIONAL,

replyCharging				[18] BOOLEAN OPTIONAL,

replyDeadline				[19] WaitTime OPTIONAL,

replyChargingSize			[20] DataVolume OPTIONAL,

priority					[21] PriorityType OPTIONAL,

senderVisibility			[22] BOOLEAN OPTIONAL,

readReplyRequested			[23] BOOLEAN OPTIONAL,

statusText					[24] StatusTextType,

recordTimeStamp				[25] TimeStamp,

localSequenceNumber			[26] LocalSequenceNumber OPTIONAL,

recordExtensions			[27] ManagementExtensions OPTIONAL,

mMBoxstorageInformation		[28] MMBoxStorageInformation OPTIONAL,

mscfInformation				[29] MSCFInformation OPTIONAL,

sGSNPLMNIdentifier			[30] PLMN-Id OPTIONAL,

rATType						[31] RATType OPTIONAL,

mSTimeZone 					[32] MSTimeZone OPTIONAL

}

MMO4FRqRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress,

recipientMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress,

recipientAddresses			[6]  MMSAgentAddresses,

contentType					[7]  ContentType,

mmComponentType				[8]  MMComponentType OPTIONAL,

messageSize					[9]  DataVolume,

messageClass				[10] MessageClass OPTIONAL,

submissionTime				[11] TimeStamp,

timeOfExpiry				[12] WaitTime OPTIONAL,

deliveryReportRequested		[13] BOOLEAN,

priority					[14] PriorityType OPTIONAL,

senderVisibility			[15] BOOLEAN,

readReplyRequested			[16] BOOLEAN,

acknowledgementRequest		[17] BOOLEAN,

forwardCounter				[18] INTEGER OPTIONAL,

forwardingAddress			[19] MMSAgentAddresses OPTIONAL,

recordTimeStamp				[20] TimeStamp,

localSequenceNumber			[21] LocalSequenceNumber OPTIONAL,

recordExtensions			[22] ManagementExtensions OPTIONAL

}

MMO4FRsRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

recipientMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMO4DRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress OPTIONAL,

recipientAddress			[6]  MMSAgentAddress,

mmDateAndTime				[7] TimeStamp,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMO1DRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageID					[4]  OCTET STRING,

mms3GPPVersion				[5]  OCTET STRING OPTIONAL,

originatorAddress			[6]  MMSAgentAddress OPTIONAL,

recipientAddress			[7]  MMSAgentAddress,

mmStatusCode				[8] MMStatusCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp OPTIONAL,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[12] PLMN-Id OPTIONAL,

rATType						[13] RATType OPTIONAL,

mSTimeZone 					[14] MSTimeZone OPTIONAL

}

MMO4RRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress OPTIONAL,

recipientAddresses			[6]  MMSAgentAddresses OPTIONAL,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

readStatus					[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber		   	[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMO1RRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageID					[4]  OCTET STRING,

mms3GPPVersion				[5]  OCTET STRING OPTIONAL,

originatorAddress			[6]  MMSAgentAddress OPTIONAL,

recipientAddress			[7]  MMSAgentAddress OPTIONAL,

readStatus					[8] MMStatusCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp OPTIONAL,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[12] PLMN-Id OPTIONAL,

rATType						[13] RATType OPTIONAL,

mSTimeZone 					[14] MSTimeZone OPTIONAL

}

MMOMDRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

recipientMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

messageSize					[4] DataVolume OPTIONAL,

mmStatusCode				[5] MMStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMR4FRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress,

originatorMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress,

recipientAddresses			[6]  MMSAgentAddresses,

contentType					[7]  ContentType,

mmComponentType				[8]  MMComponentType OPTIONAL,

messageSize					[9]  DataVolume,

messageClass				[10] MessageClass OPTIONAL,

submissionTime				[11] TimeStamp,

timeOfExpiry				[12] WaitTime OPTIONAL,

deliveryReportRequested		[13] BOOLEAN,

priority					[14] PriorityType OPTIONAL,

senderVisibility			[15] BOOLEAN,

readReplyRequested			[16] BOOLEAN,

requestStatusCode			[17] RequestStatusCodeType,

statusText					[18] StatusTextType,

acknowledgementRequest		[19] BOOLEAN,

forwardCounter				[20] INTEGER OPTIONAL,

forwardingAddress			[21] MMSAgentAddresses OPTIONAL,

recordTimeStamp				[22] TimeStamp,

localSequenceNumber			[23] LocalSequenceNumber OPTIONAL,

recordExtensions			[24] ManagementExtensions OPTIONAL

}

MMR1NRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

senderAddress				[4] MMSAgentAddress,

recipientAddress			[5] MMSAgentAddress,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

messageClass				[7] MessageClass OPTIONAL,

mmComponentType				[8] MMComponentType OPTIONAL,

messageSize					[9] DataVolume,

timeOfExpiry				[10] WaitTime OPTIONAL,

messageReference 			[11] OCTET STRING,

deliveryReportRequested		[12] BOOLEAN OPTIONAL,

replyCharging				[13] BOOLEAN OPTIONAL,

replyDeadline				[14] WaitTime OPTIONAL,

replyChargingSize			[15] DataVolume OPTIONAL,

mmStatusCode				[16] MMStatusCodeType OPTIONAL,

statusText					[17] StatusTextType OPTIONAL,

recordTimeStamp				[18] TimeStamp OPTIONAL,

localSequenceNumber			[19] LocalSequenceNumber OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

mscfInformation				[21] MSCFInformation OPTIONAL,

vaspID						[22] OCTET STRING OPTIONAL,

vasID						[23] OCTET STRING OPTIONAL,

sGSNPLMNIdentifier			[24] PLMN-Id OPTIONAL,

rATType						[25] RATType OPTIONAL,

mSTimeZone 					[26] MSTimeZone OPTIONAL

}

MMR1NRsRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress,

messageID					[2]  OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

accessCorrelation			[4] AccessCorrelation OPTIONAL,

reportAllowed				[5] BOOLEAN OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR1RtRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

senderAddress				[4] MMSAgentAddress OPTIONAL,

recipientAddress			[5] MMSAgentAddress,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

contentType					[7] ContentType,

mmComponentType				[8] MMComponentType OPTIONAL,

messageClass				[9] MessageClass OPTIONAL,

submissionTime				[10] TimeStamp,

messageSize					[11] DataVolume OPTIONAL,

deliveryReportRequested		[12] BOOLEAN OPTIONAL,

priority					[13] PriorityType OPTIONAL,

readReplyRequested			[14] BOOLEAN OPTIONAL,

mmStatusCode				[15] MMStatusCodeType OPTIONAL,

statusText					[16] StatusTextType OPTIONAL,

replyDeadline				[17] WaitTime OPTIONAL,

replyChargingSize			[18] DataVolume OPTIONAL,

durationOfTransmission		[19] INTEGER OPTIONAL,

timeOfExpiry				[20] WaitTime OPTIONAL,

recordTimeStamp				[21] TimeStamp OPTIONAL,

localSequenceNumber			[22] LocalSequenceNumber OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

messageReference 			[24] OCTET STRING,

vaspID						[25] OCTET STRING OPTIONAL,

vasID						[26] OCTET STRING OPTIONAL,

sGSNPLMNIdentifier			[27] PLMN-Id OPTIONAL,

rATType						[28] RATType OPTIONAL,

mSTimeZone 					[29] MSTimeZone OPTIONAL

}

MMR1ARecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

accessCorrelation			[4] AccessCorrelation OPTIONAL,

reportAllowed				[5] BOOLEAN OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR4DRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

originatorAddress			[5] MMSAgentAddress,

recipientAddress			[6] MMSAgentAddress,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMR4DRsRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMR1RRRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

originatorAddress			[4] MMSAgentAddress,

accessCorrelation			[5] AccessCorrelation OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR4RRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

originatorAddress			[5] MMSAgentAddress,

recipientAddress			[6] MMSAgentAddress,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMR4RRsRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMRMDRecord		::= SET

{

recordType					[0] RecordType,

originatorMmsRSAddress		[1] MMSRSAddress,

recipientMmsRSAddress		[2] MMSRSAddress OPTIONAL,

messageID					[3] OCTET STRING,

messageSize					[4] DataVolume,

mmStatusCode				[5] MMStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMFRecord		::= SET

{

recordType					[0] RecordType,

forwardingMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

forwardingAddress			[3] MMSAgentAddress,

recipientAddresses			[4] MMSAgentAddresses,

chargeInformation			[5] ChargeInformation OPTIONAL,

timeOfExpiry				[6] WaitTime OPTIONAL,

earliestTimeOfDelivery		[7] WaitTime OPTIONAL,

deliveryReportRequested		[8] BOOLEAN OPTIONAL,

readReplyRequested			[9] BOOLEAN OPTIONAL,

messageReference 			[10] OCTET STRING,

mmStatusCode				[11] MMStatusCodeType OPTIONAL,

statusText					[12] StatusTextType OPTIONAL,

recordTimeStamp				[13] TimeStamp OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

mMBoxstorageInformation		[16] MMBoxStorageInformation OPTIONAL

}

MMBx1SRecord	::= SET

{

recordType				[0]  RecordType,

mmsRelayAddress			[1]  IPAddress,

managingAddress			[2]  MMSAgentAddress,

accessCorrelation		[3]  AccessCorrelation OPTIONAL,

contentType				[4]  ContentType OPTIONAL,

messageSize				[5]  DataVolume OPTIONAL,

messageReference		[6]  OCTET STRING OPTIONAL,

mmState					[7]  OCTET STRING OPTIONAL,

mmFlags					[8]  OCTET STRING OPTIONAL,

storeStatus				[9]  StoreStatus OPTIONAL,

storeStatusText			[10] StatusTextType OPTIONAL,

sequenceNumber			[11] INTEGER OPTIONAL,

timeStamp				[12] TimeStamp OPTIONAL,

recordExtensions		[13] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier		[14] PLMN-Id OPTIONAL,

rATType					[15] RATType OPTIONAL,

mSTimeZone 				[16] MSTimeZone OPTIONAL

}

MMBx1VRecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

attributesList				[4] AttributesList OPTIONAL,

messageSelection			[5] MessageSelection OPTIONAL,

start						[6] INTEGER OPTIONAL,

limit						[7] INTEGER OPTIONAL,

totalsRequested				[8] BOOLEAN OPTIONAL,

quotasRequested				[9] BOOLEAN OPTIONAL,

mmListing					[10] AttributesList OPTIONAL,

requestStatusCode			[11] RequestStatusCodeType OPTIONAL,

statusText					[12] StatusTextType OPTIONAL,

totals						[13] Totals OPTIONAL,

quotas						[14] Quotas OPTIONAL,

sequenceNumber				[15] INTEGER OPTIONAL,

timeStamp					[16] TimeStamp OPTIONAL,

recordExtensions			[17] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[18] PLMN-Id OPTIONAL,

rATType						[19] RATType OPTIONAL,

mSTimeZone 					[20] MSTimeZone OPTIONAL

}

MMBx1URecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

recipientsAddressList		[4] MMSAgentAddresses,

messageClass				[5] MessageClass OPTIONAL,

uploadTime					[6] TimeStamp OPTIONAL,

timeOfExpiry				[7] WaitTime OPTIONAL,

earliestTimeOfDelivery		[8] WaitTime OPTIONAL,

priority					[9] PriorityType OPTIONAL,

mmState						[10] OCTET STRING OPTIONAL,

mmFlags						[11] OCTET STRING OPTIONAL,

contentType					[12] ContentType OPTIONAL,

messageSize					[13] DataVolume OPTIONAL,

messageReference			[14] OCTET STRING OPTIONAL,

requestStatusCode			[15] RequestStatusCodeType OPTIONAL,

statusText					[16] StatusTextType OPTIONAL,

sequenceNumber				[17] INTEGER OPTIONAL,

timeStamp					[18] TimeStamp OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[20] PLMN-Id OPTIONAL,

rATType						[21] RATType OPTIONAL,

mSTimeZone 					[22] MSTimeZone OPTIONAL

}

MMBx1DRecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageReference			[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

sequenceNumber				[7] INTEGER OPTIONAL,

timeStamp					[8] TimeStamp OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[20] PLMN-Id OPTIONAL,

rATType						[21] RATType OPTIONAL,

mSTimeZone 					[22] MSTimeZone OPTIONAL

}

MM7SRecord	::= SET

{

recordType						[0] RecordType,

originatorMmsRSAddress			[1] MMSRSAddress,

linkedID						[2] OCTET STRING OPTIONAL,

vaspID							[3] OCTET STRING,

vasID							[4] OCTET STRING,

messageID						[5] OCTET STRING,

originatorAddress				[6] MMSAgentAddress,

recipientAddresses				[7] MMSAgentAddresses,

serviceCode						[8] OCTET STRING OPTIONAL,

contentType						[9] ContentType,

mmComponentType					[10] MMComponentType OPTIONAL,

messageSize						[11] DataVolume,

messageClass					[12] MessageClass OPTIONAL,

chargeInformation				[13] ChargeInformation OPTIONAL,

submissionTime					[14] TimeStamp OPTIONAL,

timeOfExpiry					[15] WaitTime OPTIONAL,

earliestTimeOfDelivery			[16] WaitTime OPTIONAL,

deliveryReportRequested			[17] BOOLEAN OPTIONAL,

readReplyRequested				[18] BOOLEAN OPTIONAL,

replyCharging					[19] BOOLEAN OPTIONAL,

replyDeadline					[20] WaitTime OPTIONAL,

replyChargingSize				[21] DataVolume OPTIONAL,

priority						[22] PriorityType OPTIONAL,

messageDistributionIndicator	[23] BOOLEAN OPTIONAL,

requestStatusCode				[24] RequestStatusCodeType OPTIONAL,

statusText						[25] StatusTextType OPTIONAL,

recordTimeStamp					[26] TimeStamp,

localSequenceNumber				[27] LocalSequenceNumber OPTIONAL,

recordExtensions				[28] ManagementExtensions OPTIONAL,

mscfInformation					[29] MSCFInformation OPTIONAL

}

MM7DRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress,

linkedID				[2] OCTET STRING OPTIONAL,

replyChargingID			[3] OCTET STRING OPTIONAL,

originatorAddress		[4] MMSAgentAddress,

recipientAddress		[5] MMSAgentAddress,

mmComponentType			[6] MMComponentType OPTIONAL,

messageSize				[7] DataVolume,

contentType				[8] ContentType,

priority				[9] PriorityType OPTIONAL,

recordTimeStamp			[10] TimeStamp OPTIONAL,

localSequenceNumber		[11] LocalSequenceNumber OPTIONAL,

recordExtensions		[12] ManagementExtensions OPTIONAL

}

MM7DRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress,

messageID				[2] OCTET STRING,

recipientAddress		[3] MMSAgentAddress,

serviceCode				[4] OCTET STRING OPTIONAL,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

MM7CRecord		::= SET

{

recordType				[0] RecordType,

originatorMmsRSAddress	[1] MMSRSAddress,

vaspID					[2] OCTET STRING,

vasID					[3] OCTET STRING,

messageID				[4] OCTET STRING,

originatorAddress		[5] MMSAgentAddress,

serviceCode				[6] OCTET STRING OPTIONAL,

requestStatusCode		[7] RequestStatusCodeType OPTIONAL,

statusText				[8] StatusTextType OPTIONAL,

recordTimeStamp			[9] TimeStamp OPTIONAL,

localSequenceNumber		[10] LocalSequenceNumber OPTIONAL,

recordExtensions		[11] ManagementExtensions OPTIONAL

}

MM7RRecord		::= SET

{

recordType				[0] RecordType,

originatorMmsRSAddress	[1] MMSRSAddress,

vaspID					[2] OCTET STRING,

vasID					[3] OCTET STRING,

messageID				[4] OCTET STRING,

originatorAddress		[5] MMSAgentAddress,

serviceCode				[6] OCTET STRING OPTIONAL,

contentType				[7] ContentType,

submissionTime			[8] TimeStamp OPTIONAL,

timeOfExpiry			[9] WaitTime OPTIONAL,

earliestTimeOfDelivery	[10] WaitTime OPTIONAL,

requestStatusCode		[11] RequestStatusCodeType OPTIONAL,

statusText				[12] StatusTextType OPTIONAL,

recordTimeStamp			[13] TimeStamp OPTIONAL,

localSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordExtensions		[15] ManagementExtensions OPTIONAL

}

MM7DRRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

mmDateAndTime			[5] TimeStamp OPTIONAL,

mmStatusCode			[6] MMStatusCodeType,

mmStatusText			[7] StatusTextType OPTIONAL,

recordTimeStamp			[8] TimeStamp OPTIONAL,

localSequenceNumber		[9] LocalSequenceNumber OPTIONAL,

recordExtensions		[10] ManagementExtensions OPTIONAL

}

MM7DRRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

MM7RRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

mmDateAndTime			[5] TimeStamp OPTIONAL,

readStatus				[6] MMStatusCodeType,

mmStatusText			[7] StatusTextType OPTIONAL,

recordTimeStamp			[8] TimeStamp OPTIONAL,

localSequenceNumber		[9] LocalSequenceNumber OPTIONAL,

recordExtensions		[10] ManagementExtensions OPTIONAL

}

MM7RRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

--

--  MMS DATA TYPES

--

AccessCorrelation ::= CHOICE

{

circuitSwitched			[0]	CircuitSwitchedAccess,

packetSwitched			[1]	PacketSwitchedAccess

}

AttributesList			::= SEQUENCE

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

messageID		[0]  OCTET STRING,

dateAndTime		[1]  TimeStamp,

senderAddress	[2]  MMSRSAddress,

subject			[3]  OCTET STRING,

messageSize		[4]  DataVolume ,

mmFlags			[5]  OCTET STRING,

mmState			[6]  MMState

}

ChargeInformation	::= SEQUENCE

--

-- one of the two following parameters must be present

--

{

chargedparty		[0]  ChargedParty OPTIONAL,

chargetype			[1]  ChargeType OPTIONAL

}

ChargedParty	::= ENUMERATED

{

sender				(0),

recipient			(1),

both				(2),

neither				(3),

notspecifiedbyVASP	(99)

}

ChargeType			::= ENUMERATED

{

postpaid				(0),

pre-paid				(1)

}

CircuitSwitchedAccess ::= SEQUENCE

{

mSCIdentifier			[0]  MscNo,

callReferenceNumber		[1]  CallReferenceNumber

}

ContentType			::= OCTET STRING

DataVolume			::= INTEGER

--

-- The volume of data transfered in octets.

--

DeltaSeconds		::= OCTET STRING (SIZE(8))

MediaComponent	::= SEQUENCE

{

mediaType		[0]  OCTET STRING,

mediaSize		[1]  DataVolume

}

MediaComponents	::= SET OF MediaComponent

MessageClass		::= ENUMERATED

{

personal			(0),

advertisement		(1),

information-service	(2),

auto				(3)

}

MessageSelection ::= INTEGER

MMBoxStorageInformation   	::= SET

{

mmState					[0]  MMState,

mmFlag					[1]  OCTET STRING,

storeStatus				[2]  StoreStatus,

storeStatusText			[3]  StatusTextType,

storedMessageReference	[4]  OCTET STRING

}

MMComponentType		::= SEQUENCE

{

subject		[0]  SubjectComponent,

media		[1]  MediaComponents

}

MMSAgentAddress		::= SEQUENCE

--

-- mMSRecipeintType is only included when this datatype is used to identify recipients.

--

{

mMSAgentAddressData	[0]  MMSAgentAddressData,

mMSRecipientType	[1]  SEQUENCE OF MMSRecipientType OPTIONAL

}

MMSAgentAddresses	::= SET OF MMSAgentAddress

MMSAgentAddressData	::= CHOICE

{

eMail-address	[0]  OCTET STRING,

mSISDN			[1]  MSISDN,

shortCode		[2]  OCTET STRING

}

MMSRecipientType	::= ENUMERATED

{

tO				(0),

cC				(1),

bCC				(2)

}

MMSRSAddress		::= SEQUENCE

--

-- usage of SEQUENCE instead of CHOICE allows both address types to be present at the same time

--

{

domainName		[0]  OCTET STRING OPTIONAL,

iPAddress		[2]  IPAddress OPTIONAL

}

MMState		::= ENUMERATED

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

draft			(0),

sent			(1),

new				(2),

retrieved		(3),

forwarded		(4)

}

MMStatusCodeType		::= ENUMERATED

{

retrieved					(0),

forwarded					(1),

expired						(2),

rejected					(3),

deferred					(4),

unrecognised				(5),

read						(6),

deletedWithoutBeingRead 	(7)

}

MSCFInformation		::= SET

{

billingInformation		[0]  OCTET STRING OPTIONAL,

routeingAddressList		[1]  RouteingAddressList OPTIONAL

}

PacketSwitchedAccess ::= SEQUENCE

{

gSNAddress		[0]  GSNAddress,

chargingID		[1] ChargingID

}

PriorityType	::= ENUMERATED

{

low			(0),

normal		(1),

high		(2)

}

Quotas	::= SEQUENCE

{

numberOfMessages	[0] INTEGER OPTIONAL,

numberOfOctets		[1] INTEGER OPTIONAL

}

RequestStatusCodeType	::= INTEGER

--

-- cause codes 0 to 15 are used as defined for 'CauseForTerm'

-- (cause for termination) and 16 to 20 are as defined for 'CauseForRecClosing'

--

{

normalRelease				(0),	-- ok

abnormalRelease				(4),	-- error unspecified

serviceDenied				(30),

messageFormatCorrupt		(31),

sendingAddressUnresolved	(32),

messageNotFound				(33),

networkProblem				(34),

contentNotAccepted		 	(35),

unsupportedMessage			(36)

}

RouteingAddress			::= SEQUENCE

--

-- usage of SEQUENCE instead of CHOICE allows several address types

-- to be present at the same time

--

{

eMail-address		[0] OCTET STRING,

mSISDN				[1] MSISDN OPTIONAL,

shortCode			[2] OCTET STRING OPTIONAL

}

RouteingAddressList	::= SET OF MMSAgentAddress

StatusTextType		::= OCTET STRING

StoreStatus	::= INTEGER

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

stored							(0),

errorTransientFailure			(1),

errorTransientMailboxFull		(2),

errorTransientNetworkProblems	(3),

errorPermanentFailure			(4),

errorPermanentPermissionDenied	(5),

errorPermanentMessageFormat		(6),

errorPermanentMessageNotFound	(7)

}

SubjectComponent	::= SEQUENCE

{

subjectType		[0] OCTET STRING,

subjectSize 	[1] DataVolume

}

Totals	::= SEQUENCE

{

numberOfMessages		[0] INTEGER OPTIONAL,

numberOfOctets			[1] INTEGER OPTIONAL

}

WaitTime		::= CHOICE

{

http-date		[0]	TimeStamp,

delta-seconds	[1]	DeltaSeconds

}

.#END

#### 5.2.4.2	LCS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.271 [31].

.$LCSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) lcsChargingDataTypes (6) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

IPAddress,

LCSClientIdentity,

LocalSequenceNumber,

ManagementExtensions,

MSISDN,

PositioningData,

RecordingEntity,

RecordType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

PLMN-Id

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25)  version15 (15) }

-- from TS 29.002 [214]

AddressString,

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

;

--

--  LCS RECORDS

--

LCSRecord	::= CHOICE

--

-- Record values 71..75 are LCS specific

--

{

lCSGMORecord		[71] LCSGMORecord,

lCSRGMTRecord		[72] LCSRGMTRecord,

lCSHGMTRecord		[73] LCSHGMTRecord,

lCSVGMTRecord		[74] LCSVGMTRecord,

lCSGNIRecord		[75] LCSGNIRecord

}

LCSGMORecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

servedIMSI					[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

servingEntity				[6] ServingEntity OPTIONAL,

locationEstimate			[7] Ext-GeographicalInformation OPTIONAL,

positioningData				[8] PositioningData OPTIONAL,

userError					[9] UserError OPTIONAL,

providerError				[10] ProviderError OPTIONAL,

recordTimeStamp				[11] TimeStamp,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

LCSRGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

homeGMLCIdentity			[12] IPAddress OPTIONAL

}

LCSHGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

requestingGMLCIdentity		[12] IPAddress OPTIONAL,

visitedGMLCIdentity			[13] IPAddress OPTIONAL,

servingNetworkIdentity		[14] PLMN-Id OPTIONAL

}

LCSVGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

homeGMLCIdentity			[12] IPAddress OPTIONAL

}

LCSGNIRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

servedIMSI					[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

servingEntity				[6] ServingEntity OPTIONAL,

resultCode					[7] ResultCodeType OPTIONAL,

recordTimeStamp				[8] TimeStamp,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL

}

--

--  LCS DATA TYPES

--

ProviderError	::= INTEGER

--

-- see ITU-T Q.733 [307] for invoke problem codes

--

ResultCodeType			::= INTEGER (0..MAX)

--

-- Result codes as defined in OMA-MLP Specifications [311]

--

ServingEntity ::= AddressString

UserError	::= OCTET STRING (SIZE (1))

--

-- see TS 29.002 [214] for error code values

--

.#END

#### 5.2.4.3	PoC CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.272 [32].

.$POCChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) pocChargingDataTypes (7) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

LocalSequenceNumber,

ManagementExtensions,

NodeAddress,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

GSNAddress,

NodeID

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

IMS-Charging-Identifier,

Incomplete-CDR-Indication,

InterOperatorIdentifiers,

InvolvedParty,

MessageBody,

Media-Components-List,

Session-Id,

SIP-Method

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

--

--  POC RECORDS

--

POCRecord	::= CHOICE

--

-- Record values 80..81 are PoC specific

--

{

pPFRecord			[80] PPFRecord,

cPFRecord			[81] CPFRecord

}

PPFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

nodeAddress						[3] NodeAddress	OPTIONAL,

session-Id						[4] Session-Id OPTIONAL,

calling-Party-Address			[5] InvolvedParty OPTIONAL,

called-Party-Address			[6] InvolvedParty OPTIONAL,

servedParty						[7] GraphicString OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp OPTIONAL,

recordOpeningTime				[11] TimeStamp OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[15] INTEGER OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[20] NodeAddress OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE OF MessageBody OPTIONAL,

userLocationInfo				[23] OCTET STRING OPTIONAL,

poCInformation					[24] POCInformation	OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

CPFRecord		::=	SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

nodeAddress						[3] NodeAddress OPTIONAL,

session-Id						[4] Session-Id OPTIONAL,

calling-Party-Address			[5] InvolvedParty OPTIONAL,

called-Party-Address			[6] InvolvedParty OPTIONAL,

servedParty						[7] GraphicString OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp OPTIONAL,

recordOpeningTime				[11] TimeStamp OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[15] INTEGER OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[20] NodeAddress OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE OF MessageBody OPTIONAL,

userLocationInfo				[23] OCTET STRING OPTIONAL,

poCInformation					[24] POCInformation OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

--

--  PoC DATA TYPES

--

AccessPriority			::= ENUMERATED

{

pre-emptive 		   (0),

high			       (1),

normal 			 	   (2),

low				       (3)

}

CauseForRecordClosing	::= ENUMERATED

{

normalRelease					(0),

abnormalRelease					(1),

serviceChange					(2), -- e.g. change in media due to Re-Invite

volumeLimit						(3),

timeLimit						(4),

numberofTalkBurstLimit			(5),

maxChangeCond					(6),

sessionTypeChange				(7),

managementIntervention			(8)

}

ChangeCondition			::= ENUMERATED

{

serviceChange					(0), -- e.g. change in media due to Re-Invite

volumeLimit						(1),

timeLimit						(2),

numberofTalkBurstLimit			(3),

numberofActiveParticipants		(4),

tariffTime						(5)

}

ListofTalkBurstExchange			::= SET

{

number-Of-Talk-Bursts			 [1] INTEGER OPTIONAL,

talk-Burst-Volume				 [2] INTEGER OPTIONAL,		-- measured in octets

talk-Bursts-Time				 [3] CallDuration OPTIONAL,

number-Of-Received-Talk-Bursts	 [4] INTEGER OPTIONAL,

received-Talk-Burst-Volume		 [5] INTEGER OPTIONAL,		-- measured in octets

received-Talk-Burst-Time		 [6] CallDuration OPTIONAL,

changeCondition					 [7] ChangeCondition OPTIONAL,

changeTime						 [8] TimeStamp,

numberofParticipants			 [9] INTEGER OPTIONAL

}

ParticipatingType				::=  ENUMERATED

{

normal				(0),

nW-PoC-Box			(1),

uE-PoC-Box			(2)

}

POCEventType					::=  ENUMERATED

{

normal					(0),

instantPersonalAlert				(1),

pOCGroupAdvertisement				(2),

earlySessionSettingup				(3),

pOCTalkBurst				(4)

}

POCInformation			::= SET

{

pOCSessionType				[1] POCSessionType OPTIONAL,

numberofParticipants		[2] INTEGER OPTIONAL,

listofParticipants			[3] SEQUENCE OF POCParticipant OPTIONAL,

listofTalkBurstExchange		[4] SEQUENCE OF ListofTalkBurstExchange OPTIONAL,

pOCControllingAddress		[5] UTF8String OPTIONAL,

pOCGroupName				[6] UTF8String OPTIONAL,

pOCSessionId				[7] UTF8String OPTIONAL,

pOCSessionInitiationType	[8] POCSessionInitType OPTIONAL,

pOCEventType				[9] POCEventType OPTIONAL

}

POCParticipant			::= SET

{

called-party-address			[1]	InvolvedParty,

participant-access-priority		[2]	AccessPriority OPTIONAL,

user-participating-type			[3]	ParticipatingType OPTIONAL

}

POCSessionInitType		::= ENUMERATED

{

pre-established			(0),

on-demand	  			(1)

}

POCSessionType			::= ENUMERATED

{

one-to-one-session				(0),

chat-group-session				(1),

pre-arranged-group-session		(2),

ad-hoc-group-session			(3)

}

.#END

#### 5.2.4.4	MBMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.273 [33].

.$MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mbmsChargingDataTypes (8) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

Diagnostics,

GSNAddress,

LocalSequenceNumber,

ManagementExtensions,

MBMSInformation,

MSISDN,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

AccessPointNameNI,

ChangeOfMBMSCondition,

NodeID,

PDPAddress,

PDPType

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

Media-Components-List

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

--

--  MBMS RECORDS

--

MBMSRecord	::= CHOICE

--

-- Record values 78..79 are MBMS specific

--

{

sUBBMSCRecord			[78] SUBBMSCRecord,

cONTENTBMSCRecord		[79] CONTENTBMSCRecord

}

SUBBMSCRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

ggsnAddress					[2] GSNAddress OPTIONAL,

accessPointNameNI			[3] AccessPointNameNI OPTIONAL,

servedPDPAddress			[4] PDPAddress OPTIONAL,

listOfTrafficVolumes		[5] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[6] TimeStamp,

duration					[7] CallDuration,

causeForRecClosing			[8] MBMSCauseForRecClosing,

diagnostics					[9] Diagnostics OPTIONAL,

recordSequenceNumber		[10] INTEGER OPTIONAL,

nodeID						[11] NodeID OPTIONAL,

recordExtensions			[12] ManagementExtensions OPTIONAL,

localSequenceNumber			[13] LocalSequenceNumber OPTIONAL,

servedMSISDN				[14] MSISDN OPTIONAL,

bearerServiceDescription	[15] Media-Components-List OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

serviceContextID			[17] ServiceContextID OPTIONAL

}

CONTENTBMSCRecord	::= SET

{

recordType					[0] RecordType,

contentProviderId			[1] GraphicString,

listofDownstreamNodes		[2] SEQUENCE OF GSNAddress,

accessPointNameNI			[3] AccessPointNameNI OPTIONAL,

servedPDPAddress			[4] PDPAddress OPTIONAL,

listOfTrafficVolumes		[5] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[6] TimeStamp,

duration					[7] CallDuration,

causeForRecClosing			[8] MBMSCauseForRecClosing,

diagnostics					[9] Diagnostics OPTIONAL,

recordSequenceNumber		[10] INTEGER OPTIONAL,

nodeID						[11] NodeID OPTIONAL,

recordExtensions			[12] ManagementExtensions OPTIONAL,

localSequenceNumber			[13] LocalSequenceNumber OPTIONAL,

recipientAddressList		[14] SEQUENCE OF MSISDN,

bearerServiceDescription	[15] Media-Components-List OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

serviceContextID			[17] ServiceContextID OPTIONAL,

servedpdpPDNType			[18] PDPType OPTIONAL

}

--

--  MBMS DATA TYPES

--

MBMSCauseForRecClosing	::= INTEGER

--

-- cause codes 0 to 15 are defined as used in 'CauseForTerm'

-- (cause for termination) and 16 to 20 are as defined for 'CauseForRecClosing'

-- (cause for record closing)

--

{

normalRelease					(0),

abnormalRelease					(4),

volumeLimit						(16),

timeLimit						(17),

maxChangeCond					(19),

managementIntervention			(20),

listofDownstreamNodeChange		(59)

}

.#END

#### 5.2.4.5	MMTel CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.275 [35].

.$MMTelChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mMTelChargingDataTypes (9) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

LocalSequenceNumber,

ManagementExtensions,

NodeAddress,

RecordType,

ServiceContextID,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

AoCInformation,

CarrierSelectRouting,

CauseForRecordClosing,

Early-Media-Components-List,

FEIdentifierList,

IMS-Charging-Identifier,

IMSCommunicationServiceIdentifier,

Incomplete-CDR-Indication,

InterOperatorIdentifierList,

InvolvedParty,

ListOfInvolvedParties,

ListOfReasonHeader,

Media-Components-List,

MessageBody,

Milliseconds,

NumberPortabilityRouting,

RealTimeTariffInformation,

ReasonHeaderInformation,

Role-of-Node,

Service-Id,

Session-Id,

SessionPriority,

SIP-Method,

TransitIOILists

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

--

--  MMTel RECORDS

--

MMTelServiceRecord	::= CHOICE

--

-- Record values 83 are MMTel specific

--

{

mMTelRecord			[83] MMTelRecord

}

MMTelRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierList OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

serviceRequestTimeStampFraction		[37] Milliseconds OPTIONAL,

serviceDeliveryStartTimeStampFraction	[38] Milliseconds OPTIONAL,

serviceDeliveryEndTimeStampFraction		[39] Milliseconds OPTIONAL,

online-charging-flag					[43] NULL OPTIONAL,

realTimeTariffInformation				[44] SEQUENCE OF RealTimeTariffInformation OPTIONAL,

transit-IOI-Lists						[53] TransitIOILists OPTIONAL,

iMSVisitedNetworkIdentifier				[54] OCTET STRING OPTIONAL,

listOfReasonHeader						[55] ListOfReasonHeader OPTIONAL,

additionalAccessNetworkInformation		[56] OCTET STRING OPTIONAL,

instanceId								[57] OCTET STRING OPTIONAL,

subscriberEquipmentNumber				[58] SubscriberEquipmentNumber OPTIONAL,

cellularNetworkInformation				[64] OCTET STRING OPTIONAL,

requested-Party-Address					[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity		[102] ListOfInvolvedParties OPTIONAL,

outgoingSessionId						[104] Session-Id OPTIONAL,

mMTelInformation						[110] MMTelInformation OPTIONAL,

threeGPPPSDataOffStatus					[112] ThreeGPPPSDataOffStatus OPTIONAL,

fEIdentifierList                        [113] FEIdentifierList OPTIONAL

}

--

--  MMTel DATA TYPES

--

MMTelInformation			::= SET

{

listOfSupplServices	    [0] SEQUENCE OF SupplService OPTIONAL

}

ParticipantActionType ::= ENUMERATED

{

cREATE-CONF         (0),

jOIN-CONF           (1),

iNVITED-INTO-CONF   (2),

qUIT-CONF           (3)

}

SupplService		::= SET

{

serviceType					[0] ServiceType,

serviceMode					[1] ServiceMode OPTIONAL,

numberOfDiversions		 	[2] INTEGER OPTIONAL,

associated-Party-Address	[3] InvolvedParty OPTIONAL,

serviceId					[4] Service-Id OPTIONAL,

changeTime					[5] TimeStamp,

numberOfParticipants		[6] INTEGER OPTIONAL,

participantActionType		[7] ParticipantActionType OPTIONAL,

cUGInformation				[8]	OCTET STRING OPTIONAL,

aoCInformation				[9]	SEQUENCE OF AoCInformation OPTIONAL

}

ServiceType		::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{

oIPresentation		(0),

oIRestriction		(1),

tIPresentation		(2),

tIRestriction		(3),

hOLD				(4),

cBarring			(5),

cDIVersion			(6),

cWaiting			(8),

mWaitingIndic		(9),

cONF				(10),

fLexibleAlerting	(11),

cCBS				(12),

cCNR				(13),

mCID				(14),

cAT					(15),

cUG					(16),

pNM					(17),

cRS					(18),

aoC					(19),

eCT					(20)

}

ServiceMode ::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{	cFunCond			(0),

cFbusy				(1),

cFnoReply			(2),

cFnotLogged			(3),

deflection			(4),

notReach			(5),

iCBarring			(6),

oCBarring			(7),

aCRejection 		(8),

eCTBlind			(9),

eCTConsultative		(10),

threePTY			(11),

aoC-S				(12),

aoC-D				(13),

aoC-E				(14)

}

.#END

#### 5.2.4.6	SMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.274 [34].

.$SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

DataVolume

FROM CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version1 (0)}

DiameterIdentity,

LocalSequenceNumber,

ManagementExtensions,

MessageReference,

MSISDN,

MSTimeZone,

NodeAddress,

RecordType,

SMSResult,

SubscriptionID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

PLMN-Id,

RATType

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

AddressString,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

MessageClass,

PriorityType

FROM MMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mmsChargingDataTypes (5) asn1Module (0) version1 (0)}

;

--

--  SMS RECORDS

--

SMSRecordType	::= CHOICE

--

-- Record values 93,94, 110 and 111 are SMS specific.

--

{

sCSMORecord				 [93] SCSMORecord,

sCSMTRecord				 [94] SCSMTRecord,

sCDVTT4Record			[110] SCDVTT4Record,

sCSMOT4Record			[111] SCSMTRecord

}

SCSMORecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

sMTotalNumber 				[7] INTEGER OPTIONAL,

sMSequenceNumber 			[8] INTEGER OPTIONAL,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

localSequenceNumber			[22] LocalSequenceNumber OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL

}

SCSMOT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

mTCIWFAddress				[20] NodeAddress OPTIONAL,

sMSApplicationPortID		[21] INTEGER OPTIONAL,

externalIdentifier			[22] SubscriptionID OPTIONAL,

localSequenceNumber			[23] LocalSequenceNumber OPTIONAL,

recordExtensions			[24] ManagementExtensions OPTIONAL

}

SCSMTRecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

recipientInfo				[2] RecipientInfo OPTIONAL,

originatorInfo				[3] OriginatorInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

submissionTime				[5]	TimeStamp OPTIONAL,

eventtimestamp				[6]	TimeStamp,

sMPriority					[7]	PriorityType OPTIONAL,

messageReference			[8] MessageReference OPTIONAL,

sMTotalNumber 				[9] INTEGER OPTIONAL,

sMSequenceNumber			[10] INTEGER OPTIONAL,

messageSize					[11] DataVolume OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

sMdeliveryReportRequested	[13] BOOLEAN OPTIONAL,

sMDataCodingScheme			[14] INTEGER OPTIONAL,

sMMessageType				[15] SMMessageType OPTIONAL,

sMReplyPathRequested		[16] NULL OPTIONAL,

sMUserDataHeader			[17] OCTET STRING OPTIONAL,

sMSStatus					[18] SMSStatus OPTIONAL,

sMDischargeTime				[19] TimeStamp OPTIONAL,

userLocationInfo			[20] OCTET STRING OPTIONAL,

rATType						[21] RATType OPTIONAL,

uETimeZone					[22] MSTimeZone OPTIONAL,

sMSResult					[23] SMSResult OPTIONAL,

sMDeviceTriggerInformation	[25] SMDeviceTriggerInformation OPTIONAL,

localSequenceNumber			[26] LocalSequenceNumber OPTIONAL,

recordExtensions			[27] ManagementExtensions OPTIONAL

}

SCDVTT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

eventtimestamp				[2]	TimeStamp,

originatorInfo				[3] OriginatorInfo OPTIONAL,

recipientInfo				[4] RecipientInfo OPTIONAL,

sMDeviceTriggerIndicator	[5] SMDeviceTriggerIndicator OPTIONAL,

sMDeviceTriggerInformation	[6] SMDeviceTriggerInformation OPTIONAL,

sMSResult					[7] SMSResult OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL

}

--

--  SMS DATA TYPES

--

OriginatorInfo	::= SEQUENCE

--

-- OriginatorInfo is used for information about Originator of a Short Message

--

{

originatorIMSI				[0] IMSI OPTIONAL,

originatorMSISDN			[1] MSISDN OPTIONAL,

originatorOtherAddress		[2] SMAddressInfo OPTIONAL,

-- used if type different from IMSI and MSISDN

originatorSCCPAddress		[3] AddressString OPTIONAL,

originatorReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMOriginatorInterface		[5] SMInterface OPTIONAL,

sMOriginatorProtocolID		[6] OCTET STRING OPTIONAL

}

RecipientInfo	::= SEQUENCE

--

-- RecipientInfo is used for information about Recipient of a Short Message

--

{

recipientIMSI				[0] IMSI OPTIONAL,

recipientMSISDN				[1] MSISDN OPTIONAL,

recipientOtherAddress		[2] SMAddressInfo OPTIONAL,

-- used if type different from IMSI and MSISDN

recipientSCCPAddress		[3] AddressString OPTIONAL,

recipientReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMDestinationInterface		[5] SMInterface OPTIONAL,

sMRecipientProtocolID		[6] OCTET STRING OPTIONAL

}

SMAddressDomain		::= SEQUENCE
{

sMDomainName			[0] GraphicString OPTIONAL,

threeGPPIMSI-MCC-MNC	[1] PLMN-Id OPTIONAL

}

SMAddressInfo		::= SEQUENCE
{

sMAddressType		[0] SMAddressType OPTIONAL,

sMAddressData		[1] GraphicString OPTIONAL,

sMAddressDomain		[2] SMAddressDomain OPTIONAL

}

SMAddressType	::= ENUMERATED

{

emailAddress			(0),

mSISDN					(1),

iPv4Address				(2),

iPv6Address				(3),

numericShortCode		(4),

alphanumericShortCode	(5),

other					(6),

iMSI					(7)

}

SMDeviceTriggerIndicator	::= ENUMERATED

{

notDeviceTrigger		(0),

deviceTriggerRequest	(1),

deviceTriggerReplace	(2),

deviceTriggerRecall		(3)

}

SMDeviceTriggerInformation	::= SEQUENCE

--

-- SMDeviceTriggerInformation is used for information on device triggering from T4

-- as specified in TS 29.337[231]

--

{

mTCIWFAddress			[0] NodeAddress OPTIONAL,

sMDTReferenceNumber		[1] INTEGER OPTIONAL,

sMServingNode			[2] SMServingNode OPTIONAL,

sMDTValidityPeriod		[3] INTEGER OPTIONAL,

sMDTPriorityIndication	[4] SMDTPriorityIndication OPTIONAL,

sMSApplicationPortID	[5] INTEGER OPTIONAL

}

SMDTPriorityIndication	::= ENUMERATED

{

nonpriority	(0),

priority	(1)

}

SMInterface		::= SEQUENCE
{

interfaceId			[0] GraphicString OPTIONAL,

interfaceText		[1] GraphicString OPTIONAL,

interfacePort		[2] GraphicString OPTIONAL,

interfaceType		[3] SMInterfaceType OPTIONAL

}

SMInterfaceType	::= ENUMERATED

{

unkown					(0),

mobileOriginating		(1),

mobileTerminating		(2),

applicationOriginating	(3),

applicationTerminating	(4),

deviceTrigger			(5)

}

SMMessageType	::= ENUMERATED

{

submission			(0),

deliveryReport		(1),

sMServiceRequest	(2),

delivery			(3),

t4DeviceTrigger		(4),

sMDeviceTrigger		(5)

}

SMServingNode		::= SEQUENCE
{

sGSNName			[0] DiameterIdentity OPTIONAL,

sGSNRealm			[1] DiameterIdentity OPTIONAL,

sGSNNumber			[2] AddressString OPTIONAL,

mMEName				[3] DiameterIdentity OPTIONAL,

mMERealm			[4] DiameterIdentity OPTIONAL,

mMENumberForMTSMS	[5] AddressString OPTIONAL,

mSCNumber			[6] AddressString OPTIONAL,

iPSMGWNumber		[7] AddressString OPTIONAL,

iPSMGWName			[8] DiameterIdentity OPTIONAL

}

SMSStatus		::= OCTET STRING (SIZE(1))

.#END

#### 5.2.4.7	ProSe CDRs

This subclause contains the abstract syntax definitions that are specific to the ProSe CDR types defined in TS 32.277 [36].

.$ProSeChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) proseChargingDataType (11) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

- BEGIN

-- EXPORTS everything

- IMPORTS

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

- IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

ChargingCharacteristics,

ChChSelectionMode,

DataVolumeGPRS,

NodeID,

PLMN-Id

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

;

--

--  ProSe RECORDS

--

ProSeRecordType		::= CHOICE

--

-- Record values 100..102 are ProSe specific

--

{

pFDDRecord			[100] PFDDRecord,

pFEDRecord			[101] PFEDRecord,

pFDCRecord			[102] PFDCRecord

}

PFDDRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

proSeRequestTimestamp			[8] TimeStamp OPTIONAL,

roleofUE						[9] ProSeUERole OPTIONAL,

pCThreeControlProtocolCause		[10] INTEGER OPTIONAL,

roleofProSeFunction				[11] ProSeFunctionRole OPTIONAL,

proSeApplicationID				[12] UTF8String OPTIONAL,

proSeEventType					[13] ProSeEventType OPTIONAL,

nodeID							[14] NodeID OPTIONAL,

proseFunctionId					[15] UTF8String OPTIONAL,

announcingUEHPLMNIdentifier		[16] PLMN-Id OPTIONAL,

announcingUEVPLMNIdentifier		[17] PLMN-Id OPTIONAL,

monitoringUEHPLMNIdentifier	[18] PLMN-Id OPTIONAL,

monitoringUEVPLMNIdentifier		[19] PLMN-Id OPTIONAL,

monitoredPLMNIdentifier			[20] PLMN-Id OPTIONAL,

applicationID					[21] UTF8String OPTIONAL,

directDiscoveryModel			[22] UTF8String OPTIONAL,

validityPeriod					[23] INTEGER OPTIONAL,

monitoringUEIdentifier			[24] IMSI OPTIONAL,

discovererUEHPLMNIdentifier		[25] PLMN-Id OPTIONAL,

discovererUEVPLMNIdentifier		[26] PLMN-Id OPTIONAL,

discovereeUEHPLMNIdentifier		[27] PLMN-Id OPTIONAL,

discovereeUEVPLMNIdentifier		[28] PLMN-Id OPTIONAL,

announcingPLMNID				[29] PLMN-Id OPTIONAL

}

PFEDRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

proSeRequestTimestamp			[8] TimeStamp OPTIONAL,

roleofUE						[9] ProSeUERole OPTIONAL,

pCThreeEPCControlProtocolCause	[10] INTEGER OPTIONAL,

proseFunctionPLMNIdentifier		[11] PLMN-Id OPTIONAL,

proseFunctionId					[12] UTF8String OPTIONAL,

recordOpeningTime				[13] TimeStamp OPTIONAL,

recordClosureTime				[14] TimeStamp OPTIONAL,

applicationID					[15] UTF8String OPTIONAL,

requestorApplicationLayerUserID [16] UTF8String OPTIONAL,

wLANLinkLayerID					[17] UTF8String OPTIONAL,

requestorEPCProSeUserID			[18] UTF8String OPTIONAL,

requestedApplicationLayerUserID	[19] UTF8String OPTIONAL,

requestedPLMNIdentifier			[20] PLMN-Id OPTIONAL,

timeWindow						[21] INTEGER OPTIONAL,

rangeClass						[22] RangeClass OPTIONAL,

uELocation						[23] OCTET STRING OPTIONAL,

proximityAlertIndication		[24] ProximityAlertIndication OPTIONAL,

proximityAlertTimestamp			[25] TimeStamp OPTIONAL,

proximityCancellationTimestamp	[26] TimeStamp OPTIONAL,

reasonforCancellation			[27] ReasonforCancellation OPTIONAL,

causeForRecClosing				[28] ProSeCauseForRecClosing,

proximityRequestRenewalInfoBlockList	[29] SEQUENCE OF ProximityRequestRenewalInfoBlock OPTIONAL

}

PFDCRecord 	::= SET

{

-- General CDR information

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

nodeID							[8] NodeID OPTIONAL,

proseFunctionPLMNIdentifier		[9] PLMN-Id OPTIONAL,

proseFunctionId					[10] UTF8String OPTIONAL,

recordOpeningTime				[11] TimeStamp OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

-- Common ProSe information. The same data is provided in all currently open group-level CDRs

listOfCoverageInfo				[13] SEQUENCE OF CoverageInfo OPTIONAL,

listOfRadioParameterSet			[14] SEQUENCE OF RadioParameterSetInfo OPTIONAL,

-- Group-specific information. This data could be different in each open group-level CDR

proSeUEID						[15] OCTET STRING OPTIONAL,

sourceIPaddress					[16] IPAddress OPTIONAL,

layerTwoGroupID				[17] OCTET STRING OPTIONAL,

proSeGroupIPmulticastaddress	[18] IPAddress OPTIONAL,

timeOfFirstTransmission			[19] TimeStamp OPTIONAL,

timeOfFirstReception			[20] TimeStamp OPTIONAL,

listOfTransmitters				[21] SEQUENCE OF TransmitterInfo OPTIONAL,

listOfTransmissionData			[22] SEQUENCE OF ChangeOfProSeCondition OPTIONAL,

listOfReceptionData				[23] SEQUENCE OF ChangeOfProSeCondition OPTIONAL,

causeForRecClosing				[24] ProSeCauseForRecClosing,

listOfAppSpecificData			[25] SEQUENCE OF AppSpecificData,

targetIPaddress				[26] IPAddress OPTIONAL,

relayIPaddress					[27] IPAddress OPTIONAL,

proSeUE-to-NetworkRelay UEID	[28] OCTET STRING OPTIONAL,

proSeTargetLayerTwoID			[29] OCTET STRING OPTIONAL

}

--

--  ProSe DATA TYPES

--

ChangeOfProSeCondition			::= SEQUENCE

--

-- Used for transmitted and received data container

--

{

changeConditionTimestamp	[0] TimeStamp OPTIONAL,

coverageStatus					[1] CoverageStatus OPTIONAL,

uELocation						[2] OCTET STRING OPTIONAL,

dataVolume				[3] DataVolumeGPRS OPTIONAL,

serviceChangeCondition			[4] ServiceChangeCondition OPTIONAL,

localSequenceNumber				[5] LocalSequenceNumber OPTIONAL,

usageInformationReportSequenceNumber	[6] INTEGER OPTIONAL,

radioResourcesInd				[7] RadioResourcesIndicator OPTIONAL,

radiofrequency					[8] RadioFrequency OPTIONAL,

vPLMNIdentifier					[9] PLMN-Id OPTIONAL

}

AppSpecificData	::= OCTET STRING

CoverageInfo	::= SEQUENCE

{

coverageStatus	[0] CoverageStatus,

timeStamp		[1] TimeStamp OPTIONAL,

listOfLocation	[2] SEQUENCE OF LocationInfo OPTIONAL

}

CoverageStatus				::= ENUMERATED

{

outOfCoverage				(0),

inCoverage					(1)

}

LocationInfo	::= SEQUENCE

{

uELocation		[0] OCTET STRING OPTIONAL,

timeStamp		[1] TimeStamp OPTIONAL

}

RadioFrequency	::= OCTET STRING

--

-- Format of the value is according to the carrierFreq-r12 ASN.1 data type described in TS

-- 36.331 [241].

--

RadioParameterSetInfo	::= SEQUENCE

--

-- Format of the params value is according to the ProsePreconfiguration-r12 ASN.1 data type

-- described in TS 36.331 [241].

--

{

timeStamp	[0] TimeStamp OPTIONAL,

params		[1] OCTET STRING

}

RadioResourcesIndicator	::= INTEGER

{

operatorProvided	(1),

configured			(2)

}

ServiceChangeCondition	::= BIT STRING

{

pLMNchange 					(0),

coverageStatusChange		(1),

locationChange				(2)

}

ProSeCauseForRecClosing		::= ENUMERATED

{

proximityAlerted			(0),

timeExpiredWithNoRrenewal	(1),

requestorCancellation		(2),

timeLimited					(3),

maxNumberOfReports			(4),

abnormalRelease				(5)

}

ProSeEventType				::= ENUMERATED

{

openAnnouncing					(0),

openMonitoring					(1),

openMatchReport					(2),

restrictedAnnouncing 			(3),

restrictedMonitoring 			(4),

restrictedMatchReport			(5),

restrictedDiscoveryRequest 		(6),

restrictedDiscoveryReporting	(7)

}

ProSeFunctionRole			::= ENUMERATED

{

hPLMN			(0),

vPLMN			(1),

localPLMN		(2)

}

ProSeUERole					::= ENUMERATED

{

annoucingUE					(0),

monitoringUE				(1),

requestorUE					(2),

requestedUE					(3),

discovererUE				(4),

discovereeUE		 		(5)

}

ProximityAlertIndication	::= ENUMERATED

{

alerted						(0),

noAlert						(1)

}

ProximityRequestRenewalInfoBlock			::= SEQUENCE

--

-- Used for EPC-level discovery charging

--

{

proSeRequestTimestamp			[0] TimeStamp OPTIONAL,

timeWindow						[1] INTEGER OPTIONAL,

rangeClass						[2] RangeClass OPTIONAL,

uELocation						[3] OCTET STRING OPTIONAL

}

RangeClass					::= ENUMERATED

{

reserved					(0),

fiftyMeter					(1),

onehundredMeter				(2),

twohundredMeter				(3),

fivehundredMeter			(4),

onethousandMeter			(5)

}

ReasonforCancellation		::= ENUMERATED

{

proximityAlerted			(0),

timeExpiredWithNoRrenewal	(1),

requestorCancellation		(2)

}

TransmitterInfo		::= SEQUENCE

{

sourceIPaddress	[0] IPAddress,

proSeUEID		[1] OCTET STRING

}

.#END

#### 5.2.4.8	Monitoring Event CDRs

This subclause contains the abstract syntax definitions that are specific to the Monitoring Event CDR types defined in TS 32.278 [38].

.$MONTEChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) mONTEChargingDataType (12) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

- BEGIN

-- EXPORTS everything

- IMPORTS

DiameterIdentity,

LocalSequenceNumber,

ManagementExtensions,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

- IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version15 (15) }

-- from TS 29.002 [214]

NodeID,

UserCSGInformation

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

;

--

--  Monitoring Event RECORDS

--

MERecordType		::= CHOICE

--

-- Record values 103..104 are Monitoring Event specific

--

{

mECORecord			[103] MECORecord,

mERERecord			[104] MERERecord

}

MECORecord	::= SET

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

serviceContextID						[2] ServiceContextID OPTIONAL,

nodeID									[3] NodeID OPTIONAL,

recordTimeStamp							[4] TimeStamp OPTIONAL,

eventTimestamp							[5] TimeStamp OPTIONAL,

monitoringEventConfigurationActivity	[6] MonitoringEventConfigurationActivity OPTIONAL,

sCEFReferenceID							[7] SCEFReferenceID OPTIONAL,

sCEFID									[8] DiameterIdentity OPTIONAL,

monitoringType							[9] MonitoringType OPTIONAL,

maximumNumberofReports					[10] INTEGER OPTIONAL,

monitoringDuration						[11] TimeStamp OPTIONAL,

chargeablePartyIdentifier				[12] UTF8String OPTIONAL,

monitoredUser							[13] IMSI OPTIONAL,

maximumDetectionTime					[14] INTEGER OPTIONAL,

localRecordSequenceNumber				[15] LocalSequenceNumber OPTIONAL,

reachabilityConfiguration				[16]	ReachabilityConfiguration OPTIONAL,

locationType							[17] LocationType OPTIONAL,

accuracy								[18] Accuracy OPTIONAL,

listofLocations							[19] SEQUENCE OF EPSLocationInfo OPTIONAL,

monitoringEventConfigStatus				[20] MonitoringEventConfigStatus OPTIONAL,

recordExtensions						[21] ManagementExtensions OPTIONAL

}

MERERecord	::= SET

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

serviceContextID						[2] ServiceContextID OPTIONAL,

nodeID									[3] NodeID OPTIONAL,

recordTimeStamp							[4] TimeStamp OPTIONAL,

localRecordSequenceNumber				[5] LocalSequenceNumber OPTIONAL,

listofMonitoringEventReportData			[6] SEQUENCE OF MonitoringEventReportData OPTIONAL,	recordExtensions						[7] ManagementExtensions OPTIONAL

}

--

--  Monitoring Event DATA TYPES

--

Accuracy			::= ENUMERATED

--

-- Note: value "3" is not used in this specification: it is provided to reflect

-- the full list specified in TS 29.336 Accuracy AVP

--

{

cGIECGI				(0),

eNB					(1),

lATARA				(2),

pRA					(3)

}

CauseType		::= ENUMERATED

{

radioNetworkLayer		(0),

transportLayer			(1),

nAS						(2),

protocol					(3),

miscellaneous			(4)

}

CommunicationFailureInfo		::= SEQUENCE

{

causeType	[0] CauseType OPTIONAL,

s1APCause	[1] INTEGER OPTIONAL,

rANAPCause	[2] INTEGER OPTIONAL,

bSSGPCause	[3] INTEGER OPTIONAL,

gMMCause		[4] INTEGER OPTIONAL,

sMCause		[5] INTEGER OPTIONAL

}

CurrentLocationRetrieved		::= ENUMERATED

{

activeLocationRetrieval		(0)

}

EPSLocationInfo		::= SEQUENCE

--

-- Only one element is present.

--

{

mMELocationInformation	[0] MMELocationInformation OPTIONAL,

sGSNLocationInformation	[1] SGSNLocationInformation OPTIONAL

}

LocationType				::= ENUMERATED

{

currentLocation				(0),

lastKnownLocation			(1)

}

MMELocationInformation		::= SEQUENCE

{

eUTRANCellGlobalIdentity		[0] OCTET STRING OPTIONAL,

trackingAreaIdentity			[1] OCTET STRING OPTIONAL,

geographicalInformation		[2] OCTET STRING OPTIONAL,

geodeticInformation			[3] OCTET STRING OPTIONAL,

currentLocationRetrieved		[4] CurrentLocationRetrieved OPTIONAL,

ageOfLocationInformation		[5] INTEGER OPTIONAL,

userCSGInformation			[6] UserCSGInformation OPTIONAL,

eNodeBID						[7] OCTET STRING OPTIONAL

}

MonitoringEventConfigStatus		::= SEQUENCE

{

serviceResult	[0] ServiceResult OPTIONAL,

sCEFReferenceID	[1] SCEFReferenceID OPTIONAL,

sCEFID			[2] DiameterIdentity OPTIONAL

}

MonitoringEventConfigurationActivity				::= ENUMERATED

{

create				(0),

transfer			(1),

update				(2),

delete				(3)

}

MonitoringEventReportData		::= SEQUENCE

{

eventTimestamp							[0] TimeStamp OPTIONAL,

sCEFReferenceID							[1] SCEFReferenceID OPTIONAL,

sCEFID									[2] DiameterIdentity OPTIONAL,

monitoringEventReportNumber				[3] INTEGER OPTIONAL,

chargeablePartyIdentifier				[4] UTF8String OPTIONAL,

monitoredUser							[5] IMSI OPTIONAL,

monitoringType							[6] MonitoringType OPTIONAL,

reachabilityInformation					[7] ReachabilityType OPTIONAL,

reportedLocation							[8] EPSLocationInfo OPTIONAL,

communicationFailureInformation 			[9] SEQUENCE OF CommunicationFailureInfo OPTIONAL,

listOfNumberOfUEPerLocationReport		[10] SEQUENCE OF NumberOfUEPerLocationReport OPTIONAL

}

MonitoringType				::= ENUMERATED

--

-- Note: value "3" and "4" are not used in this specification: they are provided to reflect the full

-- list specified in TS 29.336 Monitoring-Type AVP.

--

{

lossOfConnectivity						(0),

ueReachability							(1),

locationReporting						(2),

changeOfIMSIIMEISVAssociation			(3),

roamingStatus							(4),

communicationFailure					(5),

availabilityAfterDDNFailure				(6),

numberOfUEPerLocation					(7)

}

NumberOfUEPerLocationReport		::= SEQUENCE

{

ePSLocationInformation	[0] EPSLocationInfo OPTIONAL,

uECount					[1] INTEGER OPTIONAL

}

ReachabilityConfiguration		::= SEQUENCE

{

reachabilityType			[0] ReachabilityType OPTIONAL,

maximumLatency				[1] INTEGER OPTIONAL,

maximumResponseTime			[2] INTEGER OPTIONAL

}

ReachabilityType			::= ENUMERATED

{

reachabilityforSMS				(0),

reachabilityforData				(1)

}

SGSNLocationInformation		::= SEQUENCE

{

cellGlobalIdentity			[0] OCTET STRING OPTIONAL,

locationAreaIdentity			[1] OCTET STRING OPTIONAL,

serviceAreaIdentity			[2] OCTET STRING OPTIONAL,

routingAreaIdentity			[3] OCTET STRING OPTIONAL,

geographicalInformation		[4] OCTET STRING OPTIONAL,

geodeticInformation			[5] OCTET STRING OPTIONAL,

currentLocationRetrieved		[6] CurrentLocationRetrieved OPTIONAL,

ageOfLocationInformation		[7] INTEGER OPTIONAL,

userCSGInformation			[8] UserCSGInformation OPTIONAL

}

SCEFReferenceID	::= INTEGER (0..4294967295)

--

-- 0..4294967295 is equivalent to 0..2**32-1

--

ServiceResult		::= SEQUENCE

{

vendorId			[0] INTEGER OPTIONAL,

serviceResultCode	[1] INTEGER OPTIONAL

}

.#END

# 6	CDR encoding rules

## 6.0	Introduction

TS 32.297 [52] specifies the file based protocol for the "Bx" interface between the CDR generating node, i.e. the Charging Gateway Functionality, and the operator's Billing Domain (BD) (refer to TS 32.240 [1] for details on the charging architecture). The following subclauses define

- the various CDR encodings that are standardised within 3GPP,

- a method how to indicate the encoding applied to the CDRs,

- a version indication of the encoded CDRs.

The latter two items can be used by the system(s) in the BD to easily detect the encoding version used. See TS 32.297 [52] for a detailed description on how this information is used on the Bx interface.

## 6.1	3GPP standardized encodings

The contents of the CDRs sent on the Bx interface are defined by the ASN.1 language clause 5. A number of transfer syntaxes, or encodings, is specified for use in 3GPP systems as follows. For the CDR transfer via the Bx interface, as defined in TS 32.297 [52], the Basic Encoding Rules (ITU-T Recommendation X.690 [301]) encoding must be supported by all 3GPP systems. Optionally, other additional CDR encodings, i.e. Packed Encoding Rules (ITU-T Recommendation X.691 [302]) and XML Encoding Rules (ITU-T Recommendation XER [303]) may also be offered.

The encoding applied to the CDRs is indicated by means of the "Data Record Format" parameter. The following "Data Record Format" values are used:

- "1" signifies the use of Basic Encoding Rules (BER);

- "2" signifies the use of unaligned basic Packed Encoding Rules (PER);

- "3" signifies the use of aligned basic Packed Encoding Rules (PER);

- "4" signifies the use of XML Encoding Rules (XER).

## 6.2	Encoding version indication

An indication of the version of the CDR definition and encoding must be included in the CDR files transferred via the Bx interface specified in TS 32.297 [52]. This version indication consists of a Release Identifier and a Version Identifier.

For CDRs specified in references in middle tier Charging TS 32.250 [10] to TS 32.275 [35], applying the syntax as described in clause 5 of the present document, the version indicator "6", signifying 3GPP Rel-6, shall be applied. The Version Identifier shall carry the value of the middle digit of the version number of the present document, i.e. "0" for the first version under change control, and values "1" and following for any subsequent, modified version as appropriate.

Annex A (informative):
Void

Annex B (informative):
Bibliography

a)	The 3GPP charging specifications

- 	3GPP TS 32.276: "Telecommunication management; Charging management; Voice Call Service Charging".

-	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

-	3GPP TS 32.293: "Telecommunication management; Charging management; Proxy Function".

-	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

-	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

b)	Common 3GPP specifications

-	3GPP TS 22.101: "Service aspects; Service Principles".

c)	other Domain and Service specific 3GPP / ETSI / ITU specifications

-

c)	Network Management related specifications

Annex C (informative):
ASN.1 Cross-reference listing and fully expanded sources

The ASN.1 Cross-reference listing and the fully expanded ASN.1 sources of the Charging protocol are provided for information at http://www.3gpp.org/ftp/Specs/archive/32_series/32.298/ASN.1/

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2009 | SP-45 | SP-090541 | 0103 | - | Add MBMS GW address | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0105 | - | Rel-9 CR 32.298 correction of number portability and carrier select information | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0106 | - | Add "Closed User Group (CUG)" for MMTel Charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0107 | - | Add 3PTY MMTel supplementary service charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0108 | - | CDR parameter for RTTI support in IMS offline charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0110 | - | Set of Corrections in ASN1 description for IMS CDRs | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0112 | - | Set of Corrections in ASN1 description for EPC CDRs | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0114 | - | Correction on Charging Characteristics Format | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090537 | 0115 | - | Emergency bearer service consideration for charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0117 | - | Correction to MO and MT SMS CDRs for SMS over SGs | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0119 | - | Remove CAMEL Charging Information from SGW CDR | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0121 | - | Addition of IP multicast delivery related contents in MBMS information | A | 9.0.0 | 9.1.0 |
| Dec 2009 | SP-46 | SP-090720 | 0123 | - | Correction of PDP/PDN Type | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0125 | - | Alignment with TS 32.251 for "Volume Limit" and "Time Limit" in Change-Condition AVP | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0127 | - | Alignment with TS 32.251 for "User location Change" Condition in ServiceConditionChange and ChangeCondition | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0129 | - | Correction of interOperatorIdentifiers information alignment with TS 32.260 | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0131 | - | Clarify "Change Condition" setting for containers level and "Cause for record Closing" for CDR level for P-GW and S-GW. | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0133 | - | Correction on priority session treatment - alignment with TS 22.153 | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0134 | - | Editorial clean-up | D | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0135 | - | Add CSG parameters for CSG based offline charging | B | 9.1.0 | 9.2.0 |
| Mar 2010 | SP-47 | SP-100040 | 136 | - | Correction of the Role of Node charging parameter definition | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 137 | - | Old/New location description for Location update VLR record - Alignment with TS 32.250. | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 138 | - | Correction on Session Id for AS acting as B2BUA | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 141 | - | Correction on MMTel CDR description for Early SDP- Alignment with TS 32.260 | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 143 | - | Correction in MMTel Charging for session priority - Alignment with TS 32.260 | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 144 | - | Correction on SDP handling in IMS Charging | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100044 | 145 | - | Add "Personal Network management" MMTel supplementary service charging description | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100044 | 146 | - | Add "Customized Ringing Signal (CRS)" MMTel supplementary service charging description | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 147 | - | Correction for offline Charging from PGW - 3GPP2 User location | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100042 | 148 | - | Add Originating Address in SGSNSMTRecord | B | 9.3.0 | 10.0.0 |
| Jun 2010 | SP-48 | SP-100266 | 150 | - | Correction on ASN.1 definitions | A | 10.0.0 | 10.1.0 |
| Jun 2010 | SP-48 | SP-100266 | 152 | - | Charging information for Emergency IMS Sessions | A | 10.0.0 | 10.1.0 |
| Oct 2010 | SP-49 | SP-100496 | 155 | - | Correction for Dual IP addresses associated to one PDN connection | A | 10.1.0 | 10.2.0 |
| Oct 2010 | SP-49 | SP-100496 | 158 | - | Correction on SDP-Type | A | 10.1.0 | 10.2.0 |
| Dec 2010 | SP-50 | SP-100754 | 161 | 2 | Add the missing RecordType for GWMBMSRecord | A | 10.2.0 | 10.3.0 |
| Dec 2010 | SP-50 | SP-100757 | 164 | 2 | Add missing Charging Data Record (CDR) tag for MMTelRecord | A | 10.2.0 | 10.3.0 |
| Dec 2010 | SP-50 | SP-100758 | 166 | 2 | Add missing timestamp granularity | A | 10.2.0 | 10.3.0 |
| Dec 2010 | SP-50 | SP-100758 | 170 | - | Correction of Data Volume Uplink & Downlink in the "List of Service Data" parameter | A | 10.2.0 | 10.3.0 |
| Mar 2011 | SP-51 | SP-110109 | 171 | 2 | Correction on ICSI availability - Align with SA2 TS 23.228 | F | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110105 | 172 | 3 | Adding CDR fields needed for Machine Type Communication | B | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110109 | 177 | 1 | Addition of IARI in IMS charging information, alignment with TS 22.115 and TS 23.228 | B | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110108 | 179 | 1 | Correction on ASN.1 | A | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110109 | 181 | 1 | Correction on Subscriber role | F | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110108 | 182 | 1 | Introduction of new CDRs for SRVCC feature in enhanced MSC server | A | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110108 | 183 | 1 | Corrections in ASN.1 description | A | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110112 | 184 | - | Add 'Advice Of Charge (AoC)' MMTel supplementary service Charging description - Align with 32.275 | B | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110109 | 185 | 1 | MMTel Charging enhancement for alignment with generic AS Charging description in TS 32.260 | C | 10.3.0 | 10.4.0 |
| Mar 2011 | SP-51 | SP-110108 | 187 | 1 | Correction on availability of Called Asserted Identity | A | 10.3.0 | 10.4.0 |
| May 2011 | SP-52 | SP-110281 | 193 | - | Correction with reference to Access Correlation ID | F | 10.4.0 | 10.5.0 |
| May 2011 | SP-52 | SP-110281 | 196 | 1 | Correction of RAT-Type AVP, alignment with TS 29.212, Gx interface | F | 10.4.0 | 10.5.0 |
| May 2011 | SP-52 | SP-110404 | 198 | 1 | Correction on Qos information - Alignment with TS 29.212 | A | 10.4.0 | 10.5.0 |
| May 2011 | SP-52 | SP-110294 | 199 | 1 | CDRs enhancement for OMR Charging introduction | B | 10.4.0 | 10.5.0 |
| May 2011 | SP-52 | SP-110280 | 201 | 1 | Correction in SCC AS CDR for IMS service continuity | A | 10.4.0 | 10.5.0 |
| May 2011 | SP-52 | SP-110281 | 205 | 1 | Correction on IMS Application Reference Identifier (IARI) in IMS Charging | F | 10.4.0 | 10.5.0 |
| Sep 2011 | SP-53 | SP-110528 | 208 | - | Correction on PDN connection identifier for Charging | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110528 | 211 | 1 | Solve Editor's Note on Charging Id | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110530 | 213 | 1 | Correction on MT-LR CDR - Alignment with TS 23.271 | F | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110530 | 214 | 1 | Correction for IARI - Alignment with TS 24.229 | F | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110529 | 219 | 1 | Alignment of the occurrence condition for IMSI with TS 32.251 | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110528 | 222 | - | Correction on RAT Type - Align with CT3 TS 29.061 | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110528 | 225 | - | Correction on pdpPDNtype for PGW | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110528 | 227 | 1 | Removal of placeholder duplication for ASN.1 source code | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110528 | 274 | - | Correction for dynamic address flags associated to PDN connection of PDP/PDN type IPv4v6 | A | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | -- | -- | -- | Editorial correction of misimplementation of CR 0153 in SP-100496 from SA#49 (move of 5.1.2.2.64A from clause 5.1.2.1 to 5.1.2.2). | -- | 10.5.0 | 10.6.0 |
| Sep 2011 | SP-53 | SP-110541 | 0238 | 1 | Addition of Sponsored Data Connectivity charging – Align with TS 23.203 | B | 10.6.0 | 11.0.0 |
| Dec 2011 | SP-54 | SP-110708 | 0308 | 1 | Correction on PDP/PDN Address definition - Alignment with TS 23.401 | A | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110708 | 0301 | 1 | Correction on RatingGroupId and ResultCode range | A | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110709 | 0290 | 1 | Correction on MSC-SRVCC CDRs for Suppl services and location | A | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110712 | 0276 | 2 | Add Transit IOI to IMS Offline Charging | B | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110710 | 0304 | 1 | Correction on ASN.1 syntax – alignment with TS 29.002 | A | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110710 | 0311 | -- | Correction on PDP/PDN Address definition - Alignment with TS 23.401 | A | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110711 | 0302 | 2 | Remove the Size Limitation to ChargingRuleBaseName | C | 11.0.0 | 11.1.0 |
| Mar 2012 | SP-55 | SP-120047 | 0321 | 1 | Correction for E-UTRAN location (TAI and E-CGI) on Location Update (VLR) record | A | 11.1.0 | 11.2.0 |
| Mar 2012 | SP-55 | SP-120048 | 0313 | 1 | Clarification on “SGSN Change” in PGW CDRs | A | 11.1.0 | 11.2.0 |
| Mar 2012 | SP-55 | SP-120049 | 0318 | 1 | Add Status in IMS Charging CDR | B | 11.1.0 | 11.2.0 |
| Mar 2012 | SP-55 | SP-120055 | 0320 | 1 | Correction on Charging for Mobile Terminating Roaming Forwarding (MTRF) – alignment with TS 23.018 | A | 11.1.0 | 11.2.0 |
| June-2012 | SP-56 | SP-120362 | 0323 | 1 | Correction of Serving Node Type, alignment with 29.274 | F | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120360 | 0325 | 1 | Correction of CDRs for SRVCC | A | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120374 | 0328 | 2 | Enhancing IMS charging for RAVEL | B | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120360 | 0331 | 2 | Correction on SGW and PGW Address reporting, alignment with 29.212 | A | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120397 | 0332 | 1 | Add charging parameters for NetLoc | B | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120359 | 0336 | 1 | Correction of List of Message Bodies | A | 11.2.0 | 11.3.0 |
| Sep-2012 | SP-57 | SP-120646 | 0340 | 1 | Rename Service-type AVP | A | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120576 | 0341 | - | Introduction of Loopback indicator in BGCF CDR for RAVEL | B | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120561 | 0345 | - | Remove Authorised-Qos from P-CSCF CDR | A | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120576 | 0346 | 1 | Add TRF CDR to Offline Charging | B | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120575 | 0353 | 1 | Addition of MS Timezone for NetLoc | B | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120566 | 0355 | 1 | Correction of calling party handling | C | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120561 | 0359 | 1 | Corrections to ASN.1 Syntax Definitions | A | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120627 | 0360 | 1 | Reference list correction to align with the corrected TS 29.212 title | F | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120562 | 0362 | 1 | Correction of Called-Party-Address AVP | A | 11.3.0 | 11.4.0 |
| Dec-2012 | SP-58 | SP-120785 | 0369 | 2 | Emergency Indicator introduction in P-CSCF CDR | A | 11.4.0 | 11.5.0 |
|  | SP-58 | SP-120784 | 0373 | - | Corrections of GenericChargingDataTypes and CSChargingDataTypes modules ASN.1 syntax definitions | A | 11.4.0 | 11.5.0 |
|  | SP-58 | SP-120786 | 0375 | - | Corrections of GPRSChargingDataTypes module ASN.1 syntax definitions | A | 11.4.0 | 11.5.0 |
|  | SP-58 | SP-120789 | 0376 | - | Corrections of MMSChargingDataTypes module ASN.1 syntax definitions | F | 11.4.0 | 11.5.0 |
|  | SP-58 | SP-120793 | 0378 | 3 | Offline Charging description for ATCF | B | 11.4.0 | 11.5.0 |
|  | SP-58 | SP-120792 | 0379 | 1 | Introduction SMS CDRs description for SMS over MME Charging | B | 11.4.0 | 11.5.0 |
| Dec-2012 | SP-58 | SP-120793 | 0380 | 1 | Introduction ASN.1 description for combined IBCF and ATCF CDR | B | 11.4.0 | 11.5.0 |
| Dec-2012 | SP-58 | SP-120789 | 0382 | 2 | Correction on charging for IMS transit functions | F | 11.4.0 | 11.5.0 |
| Mar-2013 | SP-59 | SP-130062 | 0366 | 3 | Multiple sets of inter operator identifiers in IMS CDRs for IMS roaming | F | 11.5.0 | 11.6.0 |
|  |  | SP-130054 | 0384 | 1 | Related ICID Corrections for SRVCC Charging Correlation | F |  |  |
|  |  | SP-130051 | 0387 | - | Emergency Indicator introduction in S-CSCF and I-CSCF CDR | A |  |  |
|  |  | SP-130052 | 0389 | - | Correction on PDPAddressPrefixLength | A |  |  |
| Jun-2013 | SP-60 | SP-130279 | 0392 | - | Remove RTTI from TRF and TF CDRs | F | 11.6.0 | 11.7.0 |
|  |  | SP-130270 | 0393 | 1 | Addition of IMS Visited Network Identifier | F |  |  |
|  |  |  | 0397 | 1 | Introduction of Charging for access to Trusted WLAN Access Network in EPC - over S2a | F |  |  |
|  |  | SP-130271 | 0396 | 1 | Adjustment on IMEI - alignment with TS 29.274 | F | 11.7.0 | 12.0.0 |
|  |  | SP-130271 | 0398 | 1 | Add SIP Reason Header Information to CDR for IMS Offline Charging | B |  |  |
| Sep-2013 | SP-61 | SP-130435 | 0404 | 1 | Additional Access Network Information Field | B | 12.0.0 | 12.1.0 |
|  |  |  | 0405 | - | retransmission indication in PS CDRs | F |  |  |
| Dec-2013 | SP-62 | SP-130676 | 0410 | 1 | Correction on missing Serving Network in PS CDRs for Network Sharing | A | 12.1.0 | 12.2.0 |
|  |  | SP-130619 | 0411 | 2 | Addition of Instance Id for IMS Charging | B |  |  |
|  |  | SP-130620 | 0412 | 1 | Requirements for Application Based Charging functionality | B |  |  |
|  |  | SP-130677 | 0417 | 1 | Correction on Serving Node PLMN description in EPC CDRs for Network Sharing | A |  |  |
|  |  | SP-130671 | 0422 | - | Correction on inconsistencies for MMTel Charging | A |  |  |
|  |  | SP-130620 | 0423 | 1 | Addition of TDF CDR for Application Based Charging functionality | B |  |  |
|  |  | SP-130627 | 0425 | 1 | Correction for Route Header for IMS Interconnection Charging | A |  |  |
| Mar-2014 | SP-63 | SP-140034 | 0428 | 1 | Correction for User Location Info Time | A | 12.2.0 | 12.3.0 |
|  |  | SP-140045 | 0429 | - | Introduction of new SC-SMO and SC-SMT CDRs description | B |  |  |
|  |  | SP-140033 | 0440 | 1 | Correction for S-GW change cause for record closing | A |  |  |
|  |  | SP-140037 | 0442 | 1 | Charging management for IMS Centralized Services (ICS) | C |  |  |
| Jun-2014 | SP-64 | SP-140337 | 0443 | 1 | To add field definitions and make clarifications for application based charging in alignment with TS 32.251 | B | 12.3.0 | 12.4.0 |
|  |  |  | 0444 | 2 | Clarifications for ASN.1 related to TDF based charging | B |  |  |
|  |  | SP-140341 | 0445 | 1 | Introduce IPE-CDR and complete TDF-CDR description | B |  |  |
|  |  | SP-140334 | 0450 | 1 | Removal of IMS charging identifier from PGW CDR | A |  |  |
|  |  | SP-140336 | 0451 | - | Introduce Core Network Operator selection origin for Shared Networks | B |  |  |
|  |  | SP-140339 | 0452 | 1 | Introduction of charging information for CHIPS - align with TS 32.251 | B |  |  |
|  |  | SP-140346 | 0454 | 1 | Correction for TADS indication in ASN.1 | A |  |  |
|  |  | SP-140343 | 0456 | 1 | Correction to support multiple Transit IOI Lists in AS, TF and MMTel CDRs | A |  |  |
| 2014-07 | - | - | - | - | Rapporteur/MCC: General editorial changes and clean-up. | - | 12.4.0 | 12.4.1 |
| 2014-09 | SP-65 | SP-140565 | 0470 | 1 | Introduction of Presence Reporting Areas for Charging | B | 12.4.1 | 12.5.0 |
|  |  | SP-140561 | 0475 | 1 | Removal of CDIVN service | A |  |  |
|  |  | SP-140562 | 0476 | 1 | Correction for expanded ASN.1 sources code generation | A |  |  |
|  |  | SP-140563 | 0477 | 1 | Introduction of ASN.1 Cross-reference listing | B |  |  |
|  |  | SP-140564 | 0478 | 1 | Corrections for alignment between charging specifications | F |  |  |
|  |  | SP-140563 | 0479 | 1 | Introduction of report the most up to date User Location Information | B |  |  |
|  |  | SP-140567 | 0484 | 1 | Introduce Charging Characteristics in Convergent scenario | B |  |  |
|  |  |  | 0485 | - | Introduce Traffic Data Volumes in IPE-CDR | B |  |  |
|  |  | SP-140563 | 0486 | 1 | Complete ePDG offline charging description | B |  |  |
|  |  | SP-140568 | 0488 | 1 | Correction on inconsistent defined parameter for NetLoc | A |  |  |
|  |  | SP-140567 | 0489 | 2 | Corrections to include missing fixed user location information for NSWO | F |  |  |
|  |  | SP-140563 | 0490 | - | Correction of Subscriber Equipment Number and Instance Id for privacy concerns | F |  |  |
|  |  |  | 0492 | 1 | Removal of I-WLAN solution | A |  |  |
| 2014-12 | SP-66 | SP-140802 | 0498 | 1 | Correction on User CSG Information in containers description for EPC offline Charging | A | 12.5.0 | 12.6.0 |
|  |  | SP-140804 | 0499 | 1 | Correction on Inter Node Change in SGW and ePDG offline charging | F |  |  |
|  |  | SP-140805 | 0500 | 1 | Additional corrections for removal of compiler errors | F |  |  |
|  |  | SP-140804 | 0501 | 1 | Additional corrections for removal of I-WLAN solution | F |  |  |
|  |  | SP-140803 | 0505 | 1 | Correction of List of SDP media Component field definition- align with 32260 | A |  |  |
|  |  |  | 0508 | - | Consistency correction of SDP information occurrence in BGCF CDR | A |  |  |
| 2015-03 | SP-67 | SP-150064 | 0513 | 1 | Correction for unavailable fields in E-CSCF CDR | A | 12.6.0 | 12.7.0 |
|  |  |  | 0517 | - | Corrections for IPv6 Address Usage in PGW and SGW CDRs | A |  |  |
|  |  | SP-150067 | 0518 | - | Corrections for IPv6 Address Usage in ePDG and TDF CDRs | F |  |  |
|  |  | SP-150066 | 0520 | - | Correction for charging based on MBMS Data Transfer Time | A |  |  |
|  |  | SP-150065 | 0523 | 1 | Inconsistency correction of subscriber role | A |  |  |
|  |  | SP-150069 | 0524 | 1 | Introduction of CDR parameters for Prose Charging | B |  |  |
| 2015-06 | SP-68 | SP-150332 | 0525 | 1 | Alignment of Direct Communications CDR with PC3ch protocol | F | 12.7.0 | 12.8.0 |
|  |  |  | 0526 | - | Addition of Prose Function ID description | F |  |  |
|  |  | SP-150318 | 0527 | 1 | Introduction of multiple Release causes in EPC Charging | B | 12.8.0 | 13.0.0 |
|  |  | SP-150326 | 0528 | 1 | Correction for ProSe Charging | D |  |  |
| 2015-09 | SP-69 | SP-150428 | 0529 | 1 | Parameter details on enhancements for IMS Service Continuity | B | 13.0.0 | 13.1.0 |
|  |  | SP-150422 | 0532 | 1 | Update of Reference RFC7315 | A |  |  |
|  |  | SP-150417 | 0533 | - | Introduction of multiple Release causes in ePDG offline charging | B |  |  |
|  |  | SP-150425 | 0538 | 1 | Introduce ISUP release cause to MGCF CDR | B |  |  |
|  |  | SP-150458 | 0540 | 1 | Correction of monitored PLMN Identifier parameter incorrect naming | A |  |  |
| 2015-12 | SP-70 | SP-150696 | 0543 | 1 | Correction for Access Network Information fields due to update to RFC 7315 | A | 13.1.0 | 13.2.0 |
|  |  | SP-150707 | 0545 | - | Correction on source code for ProSe Charging | A |  |  |
|  |  | SP-150698 | 0551 | 1 | Correction on GPRS-Charging-Id value type | C |  |  |
|  |  | SP-150700 | 0553 | 1 | Correction of Presence reporting area charging ASN.1 definition – alignement with 32.251 | A |  |  |
|  |  | SP-150698 | 0554 | 1 | Update NNI-Type ASN.1 for loopback | B |  |  |
|  |  | SP-150703 | 0555 | 1 | Introduction of TWAG offline charging – charging information | B |  |  |
|  |  | SP-150701 | 0558 | 1 | Introduction of ULI TZ Changes in IMS offline and online charging | B |  |  |
|  |  | SP-150698 | 0560 | 1 | Charging support for Terminating Identification Presentation feature changes in terminating identity | B |  |  |
|  |  |  | 0564 | 3 | Correction on CS Location Information in SIP AS CDR | B |  |  |
|  |  | SP-150706 | 0566 | 1 | Add NBIFOM related charging information | B |  |  |
|  |  | SP-150698 | 0568 | 1 | Cell information received with untrusted WLAN access information | B |  |  |
| 2016-03 | SP-71 | SP-160040 | 0569 | 1 | Charging Id assignment for NBIFOM | B | 13.2.0 | 13.3.0 |
| 2016-03 | SP-71 | SP-160034 | 0570 | - | Correction for UE identification associated with inter-UE transfer | F | 13.2.0 | 13.3.0 |
| 2016-03 | SP-71 | SP-160040 | 0571 | 1 | Correction for Access Availability Change Reason | F | 13.2.0 | 13.3.0 |
| 2016-03 | SP-71 | SP-160037 | 0572 | 1 | ULI for untrusted wireless access network correction | F | 13.2.0 | 13.3.0 |
| 2016-03 | SP-71 | SP-160035 | 0574 | 1 | Introduction of CDR parameters for MONTE Charging | B | 13.2.0 | 13.3.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-06 | SA#72 | SP-160416 | 0575 | - | F | Correction of cell information received with untrusted WLAN access information – alignment with TS 24.229 | 13.4.0 |
| 2016-06 | SA#72 | SP-160412 | 0576 | 1 | F | Correction for the editor’s notes about Monitoring-Type AVP | 13.4.0 |
| 2016-06 | SA#72 | SP-160410 | 0582 | 1 | A | Correction for Access Network Information in BGCF CDR – align with TS 32.260 | 13.4.0 |
| 2016-06 | SA#72 | SP-160416 | 0584 | 1 | F | Corrections ASN.1 syntax errors for expanded source generation | 13.4.0 |
| 2016-06 | SA#72 | SP-160420 | 0586 | - | B | Completion of access change of service data flow for NBIFOM | 13.4.0 |
| 2016-06 | SA#72 | SP-160420 | 0587 | 1 | B | Completion of change of charging condition for NBIFOM | 13.4.0 |
| 2016-06 | SA#72 | SP-160411 | 0588 | 3 | B | Introduce CP Data transfer CDRs parameters and ASN.1 | 13.4.0 |
| 2016-06 | SA#72 | SP-160411 | 0590 | 1 | B | Introduce non-IP PDN and CP CIoT opt in CDRs description | 13.4.0 |
| 2016-09 | SA#73 | SP-160621 | 0593 | 1 | F | Correction on APN Rate Control – Alignment with TS 23.401 | 13.5.0 |
| 2016-09 | SA#73 | SP-160621 | 0595 | 1 | F | Correction of trigger conditions description for NIDD submission | 13.5.0 |
| 2016-09 | SA#73 | SP-160622 | 0596 | - | F | Correction on ASN.1 syntax for IMS, SMS and MONTE CDRs | 13.5.0 |
| 2016-09 | SA#73 | SP-160621 | 0597 | 1 | F | Correction on Non-IP PDP type - alignement with TS 29.061 | 13.5.0 |
| 2016-09 | SA#73 | SP-160622 | 0598 | - | F | Correction on CPDT CDRs ASN.1 description | 13.5.0 |
| 2016-09 | SA#73 | SP-160621 | 0599 | 1 | F | Correction on Control Plane CIoT EPS Optimisation Indicator in PGW - alignement with 23.401 | 13.5.0 |
| 2016-09 | SA#73 | SP-160621 | 0600 | 1 | F | Correction on "MO exception data" RRC establishment cause in offline charging – alignement with TS 23.401 | 13.5.0 |
| 2016-09 | SA#73 | SP-160623 | 0592 | 1 | F | Correction on the SubscriberEquipmentType – align with TS 32.299 | 14.0.0 |
| 2016-09 | SA#73 | SP-160623 | 0594 | 1 | B | Complement of Charging per IP-CAN Session | 14.0.0 |
| 2016-12 | SA#74 | SP-160847 | 0601 | - | F | Correction on P-CSCF and IBCF for ATCF specific fields | 14.1.0 |
| 2016-12 | SA#74 | SP-160858 | 0606 | - | A | Correction on OMR attributes per media in IMS Charging | 14.1.0 |
| 2016-12 | SA#74 | SP-160845 | 0612 | - | A | Correction on Requested Party Address for Emergency IMS session | 14.1.0 |
| 2016-12 | SA#74 | SP-160846 | 0614 | 1 | A | Correction on ASN.1 in PS domain CDRs | 14.1.0 |
| 2016-12 | SA#74 | SP-160844 | 0615 | 1 | B | Addition of charging support for Mulitiple PRAs | 14.1.0 |
| 2016-12 | SA#74 | SP-160847 | 0616 | 1 | F | Correction of value in SM Message Type | 14.1.0 |
| 2017-03 | SA#75 | SP-170144 | 0617 | 1 | B | Charging enhancement for 3GPP PS Data off | 14.2.0 |
| 2017-03 | SA#75 | SP-170133 | 0618 | 1 | B | Addition of the fields for ProSe Charging | 14.2.0 |
| 2017-03 | SA#75 | SP-170129 | 0619 | 1 | B | Addition of multiple PRAs support for AULC | 14.2.0 |
| 2017-03 | SA#75 | SP-170137 | 0621 | 1 | A | Correction on the APN Rate Control and SCS/AS Address | 14.2.0 |
| 2017-03 | SA#75 | SP-170132 | 0622 | - | F | Correction of CauseForRecClosing and CauseForTerm | 14.2.0 |
| 2017-03 | SA#75 | SP-170131 | 0623 | 1 | A | Correction of RelatedChangeOfServiceCondition | 14.2.0 |
| 2017-06 | SA#76 | SP-170501 | 0626 | 1 | B | Introduction of 3GPP Data Off status indication in AS charging | 14.3.0 |
| 2017-06 | SA#76 | SP-170514 | 0627 | 1 | F | Correction in ASN.1 | 14.3.0 |
| 2017-06 | SA#76 | SP-170498 | 0630 | 1 | B | Implement IMS visited network identifier for S8HR | 14.3.0 |
| 2017-06 | SA#76 | SP-170497 | 0631 | 1 | B | Addition of the fields for ProSe one-to-one communication Charging | 14.3.0 |
| 2017-06 | SA#76 | SP-170499 | 0632 | - | B | Deletion of the AULC support from TDF | 14.3.0 |
| 2017-09 | SA#77 | SP-170649 | 0633 | 2 | B | Introduce Device Trigger and SMS MO via T4 charging | 14.4.0 |
| 2017-09 | SA#77 | SP-170648 | 0635 | 2 | B | Addition of the fields for ProSe Direct discovery for public safety use | 14.4.0 |
| 2017-09 | SA#77 | SP-170656 | 0640 | 2 | A | Correction on handling of Private and Public user ID for IMS charging | 14.4.0 |
| 2017-09 | SA#77 | SP-170647 | 0641 | 3 | B | Addition of FE Identifier List to IMS Charging | 14.4.0 |
| 2017-09 | SA#77 | SP-170650 | 0643 | 1 | B | Charging enhancement for eFMSS | 15.0.0 |