3GPP TS 32.299 V14.6.0 (2018-01)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Charging management;

Diameter charging applications

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

charging, management, protocol, Diameter

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	13

1	Scope	14

2	References	15

3	Definitions, symbols and abbreviations	17

3.1	Definitions	17

3.2	Symbols	18

3.3	Abbreviations	18

4	Architecture considerations	20

4.1	High level architecture	20

4.1.0	General	20

4.1.1	Charging related transfer requirements	21

5	3GPP charging applications requirements	22

5.1	Offline charging scenarios	22

5.1.1	Basic principles	22

5.1.1.0	Introduction	22

5.1.1.1	Event based charging	23

5.1.1.2	Session based charging	24

5.1.2	Basic operation	26

5.2	Online charging scenarios	27

5.2.0	Introduction	27

5.2.1	Basic principles	27

5.2.2	Charging scenarios	28

5.2.2.0	Introduction	28

5.2.2.1	Immediate Event Charging (IEC)	29

5.2.2.1.1	Decentralized Unit Determination and Centralized Rating	29

5.2.2.1.2	Centralized Unit Determination and Centralized Rating	31

5.2.2.1.3	Decentralized Unit Determination and Decentralized Rating	33

5.2.2.1.4	Further options	34

5.2.2.2	Event Charging with Unit Reservation (ECUR)	35

5.2.2.2.1	Decentralized Unit Determination and Centralized Rating	35

5.2.2.2.2	Centralized Unit Determination and Centralized Rating	37

5.2.2.2.3	Decentralized Unit Determination and Decentralized Rating	39

5.2.2.3	Session charging with Reservation	41

5.2.2.3.1	Decentralized Unit Determination and Centralized Rating	41

5.2.2.3.2	Centralized Unit Determination and Centralized Rating	43

5.2.2.3.3	Decentralized Unit Determination and Decentralized Rating	45

5.2.3	Basic operations	47

5.3	Other requirements	50

5.3.1	Re-authorization	50

5.3.2	Threshold based re-authorization triggers	50

5.3.3	Termination action	50

5.3.4	Account expiration	50

6	3GPP charging applications – Protocol aspects	51

6.1	Basic principles for Diameter offline charging	51

6.1.0	Introduction	51

6.1.1	Event based charging	52

6.1.2	Session based charging	53

6.1.3	Offline charging error cases - Diameter procedures	55

6.1.3.1	CDF connection failure	55

6.1.3.2	No reply from CDF	55

6.1.3.3	Duplicate detection	55

6.1.3.4	CDF detected failure	55

6.2	Message contents for offline charging	56

6.2.1	Summary of offline charging message formats	56

6.2.1.1	General	56

6.2.1.2	Structure for the Accounting message formats	56

6.2.2	Accounting-Request message	57

6.2.3	Accounting-Answer (ACA) message	59

6.3	Basic principles for Diameter online charging	61

6.3.1	Online Specific Credit-Control application requirements	61

6.3.2	Diameter description on the Ro reference point	61

6.3.2.1	Basic principles	61

6.3.3	Immediate Event Charging (IEC)	62

6.3.4	Event Charging with Unit Reservation (ECUR)	64

6.3.5	Session Charging with Unit Reservation (SCUR)	66

6.3.6	Error cases and scenarios	68

6.3.6.0	Introduction	68

6.3.6.1	Duplicate detection	68

6.3.6.2	Reserve Units / Debit Units operation failure	68

6.3.7	Support of tariff changes during an active user session	68

6.3.7.1	Support of tariff changes using the tariff switch mechanism	68

6.3.7.2	Support of tariff changes using Validity-Time AVP	68

6.3.8	Support of re-authorization	69

6.3.9	Support of failure handling	69

6.3.10	Support of failover	69

6.3.11	Credit pooling	69

6.4	Message formats for online charging	70

6.4.1	Summary of online charging message formats	70

6.4.1.1	General	70

6.4.1.2	Structure for the Credit-Control message formats	70

6.4.2	Credit-Control-Request message	71

6.4.3	Credit-Control-Answer message	76

6.4.4	Re-Auth-Request message	80

6.4.5	Re-Auth-Answer message	81

6.4.6	Capabilities-Exchange-Request message	81

6.4.7	Capabilities-Exchange-Answer message	82

6.4.8	Device-Watchdog-Request message	82

6.4.9	Device-Watchdog-Answer message	82

6.4.10	Disconnect-Peer-Request message	82

6.4.11	Disconnect-Peer-Answer message	82

6.4.12	Abort-Session-Request message	82

6.4.13	Abort-Session -Answer message	82

6.5	Other procedural description of the 3GPP charging applications	83

6.5.1	Re-Authorization	83

6.5.1.1	Idle timeout	83

6.5.1.2	Change of charging conditions	83

6.5.1.3	Reporting quota usage	83

6.5.1.4	Quota consumption	84

6.5.2	Threshold based Re-Authorization triggers	84

6.5.3	Termination action	84

6.5.4	Quota consumption time	85

6.5.5	Service termination	85

6.5.6	Envelope reporting	85

6.5.6.1	Envelope reporting in Online Charging	85

6.5.6.2	Envelope reporting in Offline Charging	85

6.5.6.3	Envelope reporting - Quota consumption time	86

6.5.6.4	Envelope reporting - Combinational quota	86

6.5.7	Combinational quota	87

6.5.8	Online control of offline charging information	87

6.5.9	Support of multiple service	87

6.6	Bindings of the operation to protocol application	88

6.6.0	General	88

6.6.1	Bindings of Charging Data Transfer to Accounting	88

6.6.2	Bindings of Debit / Reserve Units to Credit-Control	89

6.7	Securing Diameter messages	89

7	Summary of used Attribute Value Pairs	90

7.1	Diameter AVPs	90

7.1.0	General	90

7.1.1	Accounting-Input-Octets AVP	91

7.1.2	Void	91

7.1.3	Accounting-Output-Octets AVP	91

7.1.4	Void	92

7.1.5	Acct-Application-Id AVP	92

7.1.6	Auth-Application-Id AVP	92

7.1.7	Called-Station-Id AVP	92

7.1.8	Event-Timestamp AVP	92

7.1.9	Multiple-Services-Credit-Control AVP	92

7.1.10	Rating-Group AVP	93

7.1.11	Result-Code AVP	93

7.1.12	Service-Context-Id AVP	94

7.1.13	Service-Identifier AVP	94

7.1.14	Used-Service-Unit AVP	94

7.1.15	User-Name AVP	95

7.1.16	Vendor-Id AVP	95

7.1.17	User-Equipment-Info AVP	95

7.2	3GPP specific AVPs	96

7.2.0	General	96

7.2.0A	Access-Network-Info-Change AVP	104

7.2.0aA	3GPP-PS-Data-Off-Status AVP	104

7.2.1	Access-Network-Information AVP	104

7.2.1A	Access-Transfer-Information AVP	105

7.2.1B	Access-Transfer-Type AVP	105

7.2.2	Account-Expiration AVP	105

7.2.3	Accumulated-Cost AVP	105

7.2.4	Adaptations AVP	105

7.2.5	Additional-Content-Information AVP	105

7.2.5A	Additional-Exception-Reports AVP	106

7.2.6	Additional-Type-Information AVP	106

7.2.7	Address-Data AVP	106

7.2.8	Address-Domain AVP	106

7.2.9	Address-Type AVP	106

7.2.10	Addressee-Type AVP	106

7.2.11	AF-Correlation-Information AVP	107

7.2.12	Alternate-Charged-Party-Address AVP	107

7.2.12aA	Announcement-Identifier AVP	107

7.2.12aB	Announcement-Information AVP	107

7.2.12aC	Announcement-Order AVP	107

7.2.12aD	Announcing-PLMN-ID AVP	107

7.2.12A	Announcing-UE-HPLMN-Identifier AVP	108

7.2.12B	Announcing-UE-VPLMN-Identifier AVP	108

7.2.13	AoC-Cost-Information AVP	108

7.2.14	AoC-Format AVP	108

7.2.15	AoC-Information AVP	108

7.2.16	AoC-Request-Type AVP	108

7.2.17	AoC-Service AVP	108

7.2.18	AoC-Service-Obligatory-Type AVP	109

7.2.19	AoC-Service-Type AVP	110

7.2.20	AoC-Subscription-Information AVP	110

7.2.20A	APN-Rate-Control   AVP	110

7.2.20B	APN-Rate-Control-Downlink   AVP	110

7.2.20C	 APN-Rate-Control-Uplink   AVP	110

7.2.21	Applic-ID AVP	111

7.2.22	Application-Provided-Called-Party-Address AVP	111

7.2.23	Application-Server AVP	111

7.2.24	Application-Server-Information AVP	111

7.2.24A	Application-Specific-Data AVP	111

7.2.25	Associated-Party-Address AVP	112

7.2.26	Associated-URI AVP	112

7.2.27	Authorised-QoS AVP	112

7.2.28	Aux-Applic-Info AVP	112

7.2.29	Base-Time-Interval AVP	112

7.2.29A	Basic-Service-Code AVP	112

7.2.29B	Bearer-Capability AVP	112

7.2.30	Bearer-Service AVP	112

7.2.30A	BSSID AVP	112

7.2.31	Called-Asserted-Identity AVP	113

7.2.31A	Called-Identity AVP	113

7.2.31B	Called-Identity-Change AVP	113

7.2.32	Called-Party-Address AVP	113

7.2.33	Calling-Party-Address AVP	113

7.2.34	Carrier-Select-Routing-Information AVP	114

7.2.35	Cause-Code AVP	115

7.2.35A	Cellular-Network-Information AVP	116

7.2.36	CG-Address AVP	116

7.2.37	Change-Condition AVP	116

7.2.38	Change-Time AVP	117

7.2.38A	Charge-Reason-Code AVP	117

7.2.39	Charged-Party AVP	117

7.2.39A	Charging-Characteristics-Selection-Mode AVP	118

7.2.39B	Charging-Per-IP-CAN-Session-Indicator AVP	118

7.2.40	Class-Identifier AVP	118

7.2.41	Client-Address AVP	118

7.2.41A	CN-Operator-Selection-Entity AVP	118

7.2.42	Content-Class AVP	118

7.2.43	Content-Disposition AVP	119

7.2.44	Content-Length AVP	119

7.2.45	Content-Size AVP	119

7.2.46	Content-Type AVP	119

7.2.46aA	Coverage-Status AVP	119

7.2.46aaA	Coverage-Info AVP	119

7.2.46abA	CP-CIoT-EPS-Optimisation-Indicator AVP	119

7.2.46acA	CPDT-Information AVP	120

7.2.46A	CSG-Access-Mode AVP	120

7.2.46B	CSG-Membership-Indication AVP	120

7.2.47	Current-Tariff AVP	121

7.2.48	CUG-Information AVP	121

7.2.49	Data-Coding-Scheme AVP	121

7.2.50	DCD-Information AVP	121

7.2.51	Deferred-Location-Event-Type AVP	121

7.2.52	Delivery-Report-Requested AVP	121

7.2.53	Destination-Interface AVP	121

7.2.54	Diagnostics AVP	122

7.2.54A	Discoveree-UE-HPLMN-Identifier AVP	122

7.2.54B	Discoveree-UE-VPLMN-Identifier AVP	122

7.2.54C	Discoverer-UE-HPLMN-Identifier AVP	122

7.2.54D	Discoverer-UE-VPLMN-Identifier AVP	122

7.2.55	Domain-Name AVP	122

7.2.56	DRM-Content AVP	122

7.2.57	Dynamic-Address-Flag AVP	122

7.2.57A	Dynamic-Address-Flag-Extension AVP	122

7.2.58	Early-Media-Description AVP	123

7.2.58A	Enhanced-Diagnostics AVP	123

7.2.59	Envelope AVP	123

7.2.60	Envelope-End-Time AVP	124

7.2.61	Envelope-Reporting AVP	125

7.2.62	Envelope-Start-Time AVP	125

7.2.62A	  EPDG-Address AVP	125

7.2.63	Event AVP	125

7.2.64	Event-Charging-TimeStamp AVP	125

7.2.65	Event-Type AVP	125

7.2.66	Expires AVP	125

7.2.66A	FE-Identifier-List AVP	125

7.2.67	File-Repair-Supported AVP	127

7.2.67aA	Forwarding-Pending AVP	127

7.2.67A	From-Address AVP	127

7.2.68	GGSN-Address AVP	127

7.2.69	IM-Information AVP	127

7.2.70	Incremental-Cost AVP	127

7.2.70A	Instance-Id AVP	127

7.2.71	Interface-Id AVP	127

7.2.72	Interface-Port AVP	128

7.2.73	Interface-Text AVP	128

7.2.74	Interface-Type AVP	128

7.2.74aA	Inter-UE-Transfer AVP	128

7.2.74A	IMS-Application-Reference-Identifier AVP	128

7.2.75	IMS-Charging-Identifier AVP	128

7.2.76	IMS-Communication-Service-Identifier AVP	128

7.2.76A	IMS-Emergency-Indicator AVP	128

7.2.77	IMS-Information AVP	129

7.2.77A	IMS-Visited-Network-Identifier AVP	130

7.2.78	IMSI-Unauthenticated-Flag AVP	130

7.2.79	Incoming-Trunk-Group-ID AVP	130

7.2.79A	Initial-IMS-Charging-Identifier AVP	130

7.2.80	Inter-Operator-Identifier AVP	130

7.2.80A	IP-Realm-Default-Indication AVP	130

7.2.80B	ISUP-Cause AVP	130

7.2.80C	ISUP-Cause-Diagnostics AVP	131

7.2.80D	ISUP-Cause-Location AVP	131

7.2.80E	ISUP-Cause-Value AVP	131

7.2.80F	ISUP-Location-Number AVP	131

7.2.80Fa	Language AVP	131

7.2.80G	Layer-2-Group-ID AVP	131

7.2.81	LCS-APN AVP	131

7.2.82	LCS-Client-Dialed-By-MS AVP	131

7.2.83	LCS-Client-External-ID AVP	131

7.2.84	LCS-Client-ID AVP	131

7.2.85	LCS-Client-Name AVP	133

7.2.86	LCS-Client-Type AVP	133

7.2.87	LCS-Data-Coding-Scheme AVP	133

7.2.88	LCS-Format-Indicator AVP	133

7.2.89	LCS-Information AVP	133

7.2.90	LCS-Name-String AVP	133

7.2.91	LCS-Requestor-ID AVP	134

7.2.92	LCS-Requestor-ID-String AVP	134

7.2.92A	Local-GW-Inserted-Indication AVP	134

7.2.93	Local-Sequence-Number AVP	134

7.2.94	Location-Estimate AVP	134

7.2.95	Location-Estimate-Type AVP	134

7.2.95A	Location-Info AVP	134

7.2.96	Location-Type AVP	135

7.2.97	Low-Balance-Indication AVP	135

7.2.97A	Low-Priority-Indicator AVP	135

7.2.97B	MBMS-Charged-Party AVP	135

7.2.98	MBMS-GW-Address AVP	135

7.2.99	MBMS-Information AVP	135

7.2.100	MBMS-User-Service-Type AVP	136

7.2.101	Media-Initiator-Flag AVP	137

7.2.102	Media-Initiator-Party AVP	137

7.2.103	Message-Body AVP	137

7.2.104	Message-Class AVP	137

7.2.105	Message-ID AVP	137

7.2.106	Message-Size AVP	137

7.2.107	Message-Type AVP	138

7.2.108	MM-Content-Type AVP	138

7.2.109	MMBox-Storage-Requested AVP	138

7.2.110	MMS-Information AVP	139

7.2.111	MMTel-Information AVP	139

7.2.111A	 MMTel-SService-Type AVP	139

7.2.111Aa	 Monitored-PLMN-Identifier AVP	140

7.2.111AaA	Monitoring-Event-Configuration-Activity AVP	140

7.2.111AaB	Monitoring-Event-Functionality AVP	140

7.2.111AaC	Monitoring-Event-Information AVP	140

7.2.111AaD	Monitoring-Event-Report-Data AVP	141

7.2.111AaE	Monitoring-Event-Report-Number AVP	141

7.2.111Ab	Monitoring-UE-HPLMN-Identifier AVP	141

7.2.111Ac	Monitoring-UE-Identifier AVP	141

7.2.111Ad	Monitoring-UE-VPLMN-Identifier AVP	141

7.2.111B	MSC-Address AVP	141

7.2.111C	MTC-IWF-Address AVP	141

7.2.111D	Neighbour-Node-Address AVP	141

7.2.111E	Network-Call-Reference-Number AVP	142

7.2.112	Next-Tariff AVP	142

7.2.112aA	NIDD-Submission  AVP	142

7.2.112A	NNI-Information AVP	142

7.2.112B	NNI-Type AVP	143

7.2.113	Node-Functionality AVP	143

7.2.114	Node-Id AVP	143

7.2.115	Number-Of-Diversions AVP	143

7.2.116	Number-Of-Messages-Sent AVP	143

7.2.117	Number-Of-Participants AVP	143

7.2.118	Number-Of-Received-Talk-Bursts AVP	144

7.2.119	Number-Of-Talk-Bursts AVP	144

7.2.120	Number-Portability-Routing-Information AVP	144

7.2.121	Offline-Charging AVP	144

7.2.122	Online-Charging-Flag AVP	144

7.2.123	Originating-IOI AVP	145

7.2.124	Originator AVP	145

7.2.125	Originator-Address AVP	145

7.2.126	Originator-Interface AVP	146

7.2.127	Originator-Received-Address AVP	146

7.2.128	Originator-SCCP-Address	146

7.2.128A	Outgoing-Session-Id AVP	146

7.2.129	Outgoing-Trunk-Group-ID AVP	146

7.2.130	Participants-Involved AVP	146

7.2.131	Participant-Group AVP	147

7.2.132	Participant-Access-Priority AVP	147

7.2.133	Participant-Action-Type AVP	147

7.2.134	Void	147

7.2.135	Void	147

7.2.135A	PC3-Control-Protocol-Cause AVP	147

7.2.135B	PC3-EPC-Control-Protocol-Cause AVP	147

7.2.136	PDN-Connection-Charging-ID AVP	148

7.2.137	PDP-Address AVP	148

7.2.137A	PDP-Address-Prefix-Length AVP	148

7.2.138	PDP-Context-Type AVP	148

7.2.138A	Play-Alternative AVP	148

7.2.139	PoC-Change-Condition AVP	148

7.2.140	PoC-Change-Time AVP	148

7.2.141	PoC-Controlling-Address AVP	149

7.2.142	PoC-Event-Type AVP	150

7.2.143	PoC-Group-Name AVP	150

7.2.144	PoC-Information AVP	150

7.2.145	PoC-Server-Role AVP	150

7.2.146	PoC-Session-Id AVP	150

7.2.147	PoC-Session-Initiation-Type AVP	151

7.2.148	PoC-Session-Type AVP	151

7.2.149	PoC-User-Role AVP	151

7.2.150	PoC-User-Role-IDs AVP	151

7.2.151	PoC-User-Role-Info-Units AVP	151

7.2.152	Positioning-Data AVP	151

7.2.153	Preferred-AoC-Currency AVP	151

7.2.154	Priority AVP	152

7.2.154aA	Privacy-Indicator AVP	152

7.2.154A	ProSe-3rd-Party-Application-ID AVP	152

7.2.154Aa	ProSe-Direct-Communication-Reception-Data-Container AVP	152

7.2.154B	ProSe-Direct-Communication-Transmission-Data-Container AVP	152

7.2.154C	ProSe-Direct-Discovery-Model AVP	153

7.2.154D	ProSe-Event-Type AVP	153

7.2.154E	ProSe-Function-IP-Address AVP	153

7.2.154F	ProSe-Function-PLMN-Identifier AVP	153

7.2.154G	ProSe-Functionality AVP	153

7.2.154H	ProSe-Group-IP-Multicast-Address AVP	153

7.2.154I	ProSe-Information AVP	153

7.2.154J	ProSe-Range-Class AVP	154

7.2.154K	ProSe-Reason-For-Cancellation AVP	155

7.2.154L	ProSe-Request-Timestamp AVP	155

7.2.154M	 ProSe-Role-Of-UE AVP	155

7.2.154N	ProSe-Source-IP-Address AVP	155

7.2.154O	ProSe-UE-ID AVP	155

7.2.154Oa 	ProSe-UE-to-Network-Relay-UE-ID AVP	155

7.2.154Ob 	ProSe-Target-Layer-2-ID AVP	155

7.2.154P	Proximity-Alert-Indication AVP	156

7.2.154Q	Proximity-Alert-Timestamp AVP	156

7.2.154R	Proximity-Cancellation-Timestamp AVP	156

7.2.155	PS-Append-Free-Format-Data AVP	156

7.2.156	PS-Free-Format-Data AVP	156

7.2.157	PS-Furnish-Charging-Information AVP	156

7.2.158	PS-Information AVP	156

7.2.159	Quota-Consumption-Time AVP	158

7.2.160	Quota-Holding-Time AVP	158

7.2.160aA	Quota-Indicator AVP	158

7.2.160A	Radio-Frequency AVP	158

7.2.160B	Radio-Parameter-Set-Info AVP	158

7.2.160C	Radio-Parameter-Set-Values AVP	159

7.2.160D	Radio-Resources-Indicator AVP	159

7.2.160E	  Rate-Control-Max-Message-Size AVP	159

7.2.160F	  Rate-Control-Max-Rate AVP	159

7.2.160G	Rate-Control-Time-Unit AVP	159

7.2.161	Rate-Element AVP	159

7.2.162	Read-Reply-Report-Requested AVP	160

7.2.163	Void	160

7.2.164	Real-Time-Tariff-Information AVP	161

7.2.164A	Reason-Header AVP	161

7.2.165	Received-Talk-Burst-Time AVP	161

7.2.166	Received-Talk-Burst-Volume AVP	161

7.2.167	Recipient-Address AVP	161

7.2.168	Recipient-Info AVP	162

7.2.169	Recipient-Received-Address AVP	162

7.2.170	Recipient-SCCP-Address	162

7.2.171	Refund-Information AVP	162

7.2.171A	Relationship-Mode AVP	162

7.2.171Aa	Related-Change-Condition-Information AVP	163

7.2.171Ab	Related-Trigger AVP	163

7.2.171B	Related-IMS-Charging-Identifier AVP	163

7.2.171C	Related-IMS-Charging-Identifier-Node AVP	163

7.2.171D	Relay-IP-address AVP	163

7.2.172	Remaining-Balance AVP	163

7.2.173	Reply-Applic-ID AVP	164

7.2.174	Reply-Path-Requested AVP	164

7.2.175	Reporting-Reason AVP	165

7.2.176	Requested-Party-Address AVP	166

7.2.176A	Requested-PLMN-Identifier AVP	166

7.2.176B	Requestor-PLMN-Identifier AVP	166

7.2.177	Role-Of-Node AVP	166

7.2.177aA	Role-Of-ProSe-Function AVP	166

7.2.177A	Route-Header-Received AVP	166

7.2.177B	Route-Header-Transmitted AVP	166

7.2.178	Scale-Factor AVP	167

7.2.178A SCS-Address AVP	167

7.2.178B SCS-AS-Address AVP	167

7.2.178C SCS-Realm AVP	167

7.2.179	SDP-Answer-Timestamp AVP	167

7.2.180	SDP-Media-Component AVP	167

7.2.181	SDP-Media-Description AVP	168

7.2.182	SDP-Media-Name AVP	168

7.2.183	SDP-Offer-Timestamp AVP	168

7.2.184	SDP-Session-Description AVP	168

7.2.185	SDP-TimeStamps AVP	168

7.2.186	SDP-Type AVP	168

7.2.186A	Session-Direction AVP	168

7.2.187	Served-Party-IP-Address AVP	168

7.2.188	Void	169

7.2.189	Service-Data-Container AVP	169

7.2.190	Service-ID AVP	169

7.2.191	Service-Generic-Information AVP	169

7.2.192	Service-Information AVP	170

7.2.193	Service-Mode AVP	171

7.2.194	Service-Specific-Data AVP	171

7.2.195	Service-Specific-Info AVP	171

7.2.196	Service-Specific-Type AVP	171

7.2.197	Void	171

7.2.197a 	Serving-Node-Identity	171

7.2.198	Serving-Node-Type AVP	172

7.2.198A	SGi-PtP-Tunnelling-Method AVP	172

7.2.199	SGSN-Address AVP	172

7.2.199A	SGW-Address AVP	172

7.2.200	SGW-Change AVP	172

7.2.201	SIP-Method AVP	172

7.2.202	SIP-Request-Timestamp AVP	172

7.2.203	SIP-Request-Timestamp-Fraction AVP	172

7.2.204	SIP-Response-Timestamp AVP	173

7.2.205	SIP-Response-Timestamp-Fraction AVP	173

7.2.205A	SM-Device-Trigger-Indicator AVP	173

7.2.205B	SM-Device-Trigger-Information AVP	173

7.2.206	SM-Discharge-Time AVP	173

7.2.207	SM-Message-Type AVP	173

7.2.208	SM-Protocol-Id AVP	174

7.2.208A	SM-Sequence-Number AVP	174

7.2.209	SM-Status AVP	174

7.2.210	SM-User-Data-Header AVP	174

7.2.211	SMS-Information AVP	174

7.2.212	SMS-Node AVP	175

7.2.212A	SMS-Result AVP	175

7.2.213	SM-Service-Type AVP	176

7.2.214	SMSC-Address AVP	176

7.2.214A	Start-of-Charging AVP	176

7.2.215	Start-Time AVP	176

7.2.215A	Status-AS-Code AVP	176

7.2.216	Stop-Time AVP	176

7.2.217	Submission-Time AVP	177

7.2.218	Subscriber-Role AVP	177

7.2.219	Supplementary-Service AVP	177

7.2.219A	TAD-Identifier AVP	177

7.2.220	Talk-Burst-Exchange AVP	177

7.2.221	Talk-Burst-Time AVP	178

7.2.222	Talk-Burst-Volume AVP	178

7.2.222A	Target-IP-Address AVP	178

7.2.223	Tariff-Information AVP	178

7.2.224	Tariff-XML AVP	178

7.2.224A	Teleservice AVP	178

7.2.225	Terminating-IOI AVP	178

7.2.225A	Time-First-Reception AVP	179

7.2.225B	Time-First-Transmission AVP	179

7.2.226	Time-First-Usage AVP	179

7.2.226A	Time-Indicator AVP	179

7.2.227	Time-Last-Usage AVP	179

7.2.228	Time-Quota-Mechanism	179

7.2.229	Time-Quota-Threshold AVP	180

7.2.230	Time-Quota-Type AVP	180

7.2.231	Time-Stamps AVP	180

7.2.232	 Time-Usage AVP	180

7.2.233	Traffic-Data-Volumes AVP	180

7.2.233A	Transcoder-Inserted-Indication AVP	181

7.2.233B	Transit-IOI-List AVP	181

7.2.233C	Transmitter-Info AVP	181

7.2.234	Token-Text AVP	181

7.2.235	Trigger AVP	181

7.2.236	Trigger-Type AVP	181

7.2.237	Trunk-Group-ID AVP	185

7.2.237A	Void	185

7.2.237B	Void	185

7.2.237Ba	TWAG-Address AVP	185

7.2.237C	TWAN-User-Location-Info AVP	186

7.2.238	Type-Number AVP	186

7.2.238A	UNI-PDU-CP-Only-Flag AVP	186

7.2.239	Unit-Cost AVP	186

7.2.240	Unit-Quota-Threshold AVP	186

7.2.240a	Unused-Quota-Timer AVP	187

7.2.240A	User-CSG-Information AVP	187

7.2.240B	Usage-Information-Report-Sequence-Number AVP	187

7.2.241	User-Participating-Type AVP	187

7.2.242	User-Session-Id AVP	187

7.2.242aaA	UWAN-User-Location-Info AVP	187

7.2.242aA	Variable-Part AVP	188

7.2.242aB	Variable-Part-Order AVP	188

7.2.242aC	Variable-Part-Type AVP	188

7.2.242aD	Variable-Part-Value AVP	188

7.2.242A	VCS-Information AVP	188

7.2.242B	VLR-Number AVP	189

7.2.243	Volume-Quota-Threshold AVP	189

7.2.244	Void	189

7.2.245	Void	189

7.2.246	Void	189

7.2.247	Void	189

7.2.248	Void	189

7.2.249	Void	189

7.2.250	Void	190

7.3	3GPP2 specific AVPs	191

7.4	ETSI specific AVPs	191

7.5	oneM2M specific AVPs	191

Annex A (informative):	Bibliography	192

Annex B (informative):	Change history	194

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in GSM/UMTS networks. The GSM/UMTS core network-charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management documents that specify.

-	The content of the CDRs' per domain and subsystem (offline charging);

-	The content of real-time charging messages per domain / subsystem (online charging);

-	The functionality of online and offline charging for those domains and subsystems;

-	The interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies in detail the Diameter based offline and online charging applications for 3GPP networks. It includes all charging parameters, scenarios and message flows..

All terms, definitions and, abbreviations used in the present document, that are common across 3GPP TSs, are defined in TR 21.905 [100]. Those that are common across charging management in GSM/UMTS domains, services or subsystems are provided in the umbrella document TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

Furthermore, requirements that govern the charging work are specified in TS 22.115 [101].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles ".

[2] - [10]	Void.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] 	Void.

[13]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging".

[14] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200]	3GPP TS 23.207: "End to end quality of service concept and architecture".

[201]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[202]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3".

[203]	3GPP TS 29.207: "Policy control over Go interface".

[204]	3GPP TS 29.229: "Cx and Dx Interfaces based on the Diameter protocol; Protocol Details".

[205]	Void.

[206]	3GPP TS 29.230: "3GPP specific codes and identifiers".

[207]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[208]	3GPP TS 23.140: " Multimedia Messaging Service (MMS); Functional description; Stage 2".

[209]	OMA "Multimedia Messaging Service; Encapsulation Protocol".

[210]	OMNA WSP Content Type Numbers 
http://www.openmobilealliance.org/tech/omna/omna-wsp-content-type.aspx

[211]	OMA-CP-POC: "OMA PoC Control Plane".

[212]	 Void.

[213]	3GPP TS 29.140: "MM10 interface based on Diameter protocol; Stage 3".

[214]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point; Stage 3".

[215]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[216]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[217]	3GPP TS 22.142: "Value Added Services (VAS) for Short Message Service (SMS) requirements".

[218]	3GPP TS 23.203: "Policy and Charging control architecture".

[219]	3GPP TS 29.272: " Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[220]	3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[221]	3GPP TS 29.329: "Sh Interface based on the Diameter protocol;Protocol details".

[222]	3GPP TS 29.658: "SIP Transfer of IP Multimedia Service Tariff Information".

[223] 	OMA-DDS-Charging_Data: "Charging Data".

[224] 	3GPP TS 23.003: "Numbering, Addressing and Identification".

[225] 	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[226] 	3GPP TS 29.274: "Evolved GPRS Tunnelling Protocol for Control Plane (GTPv2-C); Stage 3".

[227]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[228] 	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[229] 	3GPP TS 29.173: "Location Services (LCS);Diameter-based SLh interface for Control Plane LCS".

[230] 	3GPP TS 29.272: "Evolved Packet System (EPS);Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[231]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications ".

[232] 	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[233]	3GPP TS 29.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL); CAMEL Application Part (CAP) specification".

[234]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[235]	3GPP TS 23.303: "Proximity-based services (ProSe)".

[236]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects".

[237]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS   AAA Interfaces".

[238]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2)".

[239]	3GPP TS 29.345: "Inter-Proximity-services (ProSe) Function signalling aspects".

[240] 	Void.

[241]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[242]	Void.

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[245]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[246]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security ".

[247] - [299]	Void.

[300]	ETSI TS 283 034 v2.2.0: "Telecommunications and Internet  converged Services and Protocols for Advanced Networking (TISPAN); Network Attachment Sub-System (NASS); e4 interface based on the DIAMETER protocol".

[301] 	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[302] - [400]	Void.

[401]	IETF RFC 6733 (2012): "Diameter Base Protocol".

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403]	Void.

[404]	IETF RFC 7315 (2014): "Private Extensions to the Session Initiation Protocol (SIP) for the 3rd Generation Partnership Projects (3GPP)".

[405]	IETF RFC 3261 (2002): "SIP: Session Initiation Protocol".

[406]	IETF RFC 4566 (2006): "SDP: Session Description Protocol".

[407]	IETF RFC 4005 (2005): "Diameter Network Access Server Application".

[408]	IETF RFC 3264 (2002): "An Offer/Answer Model with the Session Description Protocol (SDP) ".

[409]	IEEE 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[410]	IETF RFC 3066 (2001): "Tags for the Identification of Languages".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [100].

middle tier TS: term used for the 3GPP charging TSs that specify the domain / subsystem / service specific, online and offline, charging functionality. These are all the TSs in the numbering range from TS 32.250 to TS 32.27x, e.g. TS 32.250 [10] for the CS domain, or TS 32.270 [30] for the MMS service. Currently, there is only one "tier 1" TS in 3GPP, which is the TS 32.240 that specifies the charging architecture and principles. Finally, there are a number of top tier TSs in the 32.29x numbering range ([50] ff) that specify common charging aspects such as parameter definitions, encoding rules, the common billing domain interface or common charging applications.

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with session/service control is required

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Rf	Offline Charging Reference Point between a 3G network element and the CDF.

Ro	Online Charging Reference Point between a 3G network element and the OCS.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACA	ACcounting-Answer

ACR	ACcounting-Request

ADC	Application Detection and Control

AoC	Advice of Charge

AS	Application Server

ASA	Abort-Session-Answer

ASR	Abort-Session- Request

AVP	Attribute Value Pair

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CEA	Capabilities-Exchange-Answer

CER	Capabilities-Exchange-Request

CGI	Cell Global Identification

CI	Cost-Information

CIoT	Cellular Internet of Things

CP	Control Plane

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

DBPA	Diameter Base Protocol Accounting

DCD	Dynamic Content Delivery

DPA	Disconnect-Peer-Answer

DPR	Disconnect-Peer-Request

DRM	Digital Rights Management

DWA	Device-Watchdog-Answer

DWR	Device-Watchdog-Request

ECGI	E-UTRAN Cell Global Identifier

ECUR	Event Charging with Unit Reservation

FQDN	Fully Qualified Domain Name

FUI	Final-Unit-Indication

HSGW	HRPD Serving GateWay

GSU	Granted-Service-Unit

IEC	Immediate Event Charging

IM	Instant Messaging

IMS	IP Multimedia Subsystem

IMS-AGW	IMS Access Media Gateway

IWK-SCEF	Interworking SCEF

MSCC	Multiple Services Credit-Control

NetLoc	Network provided Location information

NIDD	Non-IP Data Delivery

NNI	Network to Network Interface

OCS	Online Charging System

ProSe	Proximity-based Services

RAA	Re-Auth-Answer

RAI	Routeing Area Identity

RAR	Re-Auth-Request

RAVEL	Roaming Architecture for VoicE over IMS with Local breakout

SAI	Service Area Identifier

SCCP	Signalling Connection Control Part

SCEF	Service Capability Exposure Function

SDP	Session Description Protocol

TAI	Tracking Area Identity

TDF	Traffic Detection Function

TrGW	Transition GateWay

TWAG	Trusted WLAN Access Gateway

TWAN	Trusted WLAN Access Network

UWAN	Untrusted Wireless Access Network

VCS	Voice Call Service

# 4	Architecture considerations

## 4.1	High level architecture

### 4.1.0	General

The Rf and the Ro are reference points from the Charging Trigger Function (CTF) to the Charging Data Function (CDF) and the Online Charging Function (OCF) respectively, and are intended for the transport of charging events. Rf is used for offline charging whereas Ro is used for online charging. 
Figure 4.1.0.1 and figure 4.1.0.2 depict the position of the Rf and Ro reference points within the overall 3GPP online and offline charging architecture.

- CTF:	Charging Trigger Function

CDF:	Charging Data Function

CGF:	Charging Gateway Function

BD:	Billing Domain. This may also be a billing mediation device / post-processing system.

- Figure 4.1.0.1: Logical ubiquitous offline charging architecture

- CTF:	Charging Trigger Function

OCF:	Online Charging Function

CGF:	Charging Gateway Function

BD:	Billing Domain. This may also be a billing mediation device / post-processing system.

- Figure 4.1.0.2: Logical ubiquitous online charging architecture

Different mappings of the ubiquitous offline charging functions, CTF, CDF and CGF, onto physical implementations are possible. Further details of the configuration refer toTS 32.240 [1]. Details of the implementation options per domain / subsystem / service (usually a subset of the overall possible variants described above) are specified in the respective middle tier TSs.

### 4.1.1	Charging related transfer requirements

Each CTF would have CDF and OCF address list to which it can send its charging events and/or charging requests. 
The list is organized in address priority order. If the primary charging function is not available (e.g. out of service) then the CTF shall send the charging information to the secondary charging function and so on.

Within the scope of this release, each Network Element that generates charging information sends the information only to the charging entities of the same PLMN, and not to charging entities in other PLMNs. 
To implement roaming unbundling for EU roaming regulation III, an architectural solution known as the Single IMSI architecture has been defined. In this architecture a specific Service-NE (known as a Proxy Function) uses the Ro reference point for sending charging information to an OCF in another network. The details of this architecture are defined in Annex B of TS 32.240 [1].

To implement PS domain online charging for roaming context, PCEF in the VPLMN uses the Gy reference point, and TDF in the VPLMN uses the Gyn reference point, both based on Ro interface and specified in TS 32.251[11], for sending charging information to an OCS in the HPLMN. A simplified profile is provided in Annex E of TS 32.251[11] for deployment purposes.

Each CDF in the PLMN may know of other CDFs' network addresses (e.g., for redundancy reasons, to be able to recommend another CDF address with the Redirection Request message). This is achieved by OAM&P configuration facilities that enables each CDF to have a configurable list of peer CDF addresses.

# 5	3GPP charging applications requirements

## 5.1	Offline charging scenarios

### 5.1.1	Basic principles

#### 5.1.1.0	Introduction

Offline charging for both events and sessions between CTF and the CDF is performed using the Rf reference point as defined in TS 32.240 [1].

Two basic scenarios are used:

-	Event based charging;

-	Session based charging.

#### 5.1.1.1	Event based charging

In scenario figure 5.1.1.1.1, CTF asks the CDF to store event related charging data.

- Figure 5.1.1.1.1: Event based charging

1)	Request for resource usage: UE-A requests the desired resource from the Network Element.

2)	Content/Service Delivery: the Network Element delivers the content/service.

3)	Charging Data Generation: the CTF generates charging data related to service delivery.

4)	Record Charging Data Request: the CTF requests the CDF to store event related charging data for CDR generation purposes.

5)	Process Request: CDF stores received information. Whether the CDR is generated or not depends on CDR generation configuration.

6)	Record Charging Data Response: the CDF informs the CTF that charging data was stored.

#### 5.1.1.2	Session based charging

In scenario figure 5.1.1.2.1, CTF asks the CDF to store session related charging data.

- Figure 5.1.1.2.1: Session based charging

1)	Request for resource usage: UE-A requests the desired session from the Network Element.

2)	Session ongoing: the Network Element establish the session.

3)	Charging Data Generation: the CTF generates charging data related to session.

4)	Record Charging Data Request: the CTF requests the CDF to store session related charging data for CDR generation purposes.

5)	Process Request: CDF stores received information. Whether the CDR is generated or not depends on CDR generation configuration.

6)	Record Charging Data Response: the CDF informs the CTF that charging data was stored.

7)	Charging Data Generation: the CTF generates charging data related to session due of e.g. intermediate timer expiry.

8)	Record Charging Data Request: the CTF requests the CDF to store session related charging data for CDR generation purposes.

9)	Process Request: CDF stores received information. Whether the CDR is generated or not depends on CDR generation configuration.

10)	Record Charging Data Response: the CDF informs the CTF that charging data was stored.

11)	Session release: the session is released.

12)	Charging Data Generation: the CTF generates charging data related to session due of session termination.

13)	Record Charging Data Request: the CTF requests the CDF to store session related charging data for CDR generation purposes.

14)	Process Request: CDF stores received information. Whether the CDR is generated or not depends on CDR generation configuration.

15)	Record Charging Data Response: the CDF informs the CTF that charging data was stored.

### 5.1.2	Basic operation

Event and session based Charging are performed by the use of the "Charging Data Transfer" operation:

-	"Charging Data Request"; sent from CTF  CDF
After detecting a chargeable event, the CTF sends a Charging Data Request to the CDF.

-	"Charging Data Response"; sent from CDF  CTF
The CDF replies with a Charging Data Response, which informs the CTF that charging data was received.

Table 5.1.2.1 and table 5.1.2.2 describe the content of these operations.

- Table 5.1.2.1: Charging Data Request content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Originator Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Destination Domain | M | This field contains the realm of the operation destination. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Operation Identifier | OM | The field corresponds to the unique operation identification. |
| User Name | OC | The field contains the identification of the service user. |
| Destination Host | OC | This field contains the identification of the destination point of the operation. |
| Operation Interval | OC | This field contains the proposal for instructions to produce records continuously during a session. |
| Origination State | OC | This field contains the state associated to the source point of the operation. |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Operation Token | OM | This field identifies the domain, subsystem or service and release. |
| Service information | OM | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |

- Table 5.1.2.2: Charging Data Response content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Operation Result | M | This field identifies the result of the operation. |
| Originator Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Operation Identifier | OM | The field corresponds to the unique operation identification. |
| Operation Interval | OC | This field contains the instructions to produce records continuously during a session. |
| Error Reporting Host | OC | If proxies exist between the accounting client and the accounting server this field contains the identity of the proxy that sent a response other than 2001 (Success). |
| Origination State | OC | This field contains the state associated to the source point of the service. |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Proxy Information | OC | This field contains the parameter of the proxy. |

## 5.2	Online charging scenarios

### 5.2.0	Introduction

Online charging for both events and sessions between CTF and the OCF is performed using the Ro reference point. The Ro reference point supports integrity protection and authentication for the case that the CTF is outside the operator domain.

### 5.2.1	Basic principles

There are two sub-functions for online charging that affect online charging principles and require a more detailed description: rating and unit determination. Both rating and unit determination can be implemented centralized, i.e. on the OCF, or decentralized, that is, on the CTF.

Unit determination refers to the calculation of the number of non-monetary units (service units, data volume, time and events) that shall be assigned prior to starting service delivery.

-	With Centralized Unit Determination, the OCF determines the number of non-monetary units that a certain service user can consume based on a service key received from the CTF.

-	With the Decentralized Unit Determination approach, the CTF determines itself how many units are required to start service delivery, and requests these units from the OCF.

After checking the service user's account balance, the OCF returns the number of granted units to the CTF. The CTF is then responsible for the supervision of service delivery. Particularly, the CTF shall limit service delivery to the corresponding number of granted units.

Rating refers to the calculation of a price out of the non-monetary units calculated by the unit determination function.

-	With the Centralized Rating approach, the CTF and the OCF exchange information about non-monetary units. The OCF translates these units into monetary units.

-	With the Decentralized Rating approach, the corresponding rating control is performed within the CTF. Consequently, CTF and OCF exchange information about monetary units.

Three cases for online charging can be distinguished: Immediate Event Charging (IEC), Event Charging with Unit Reservation (ECUR) and Session Charging with Unit Reservation (SCUR). These cases are further described in TS 32.240 [1].

### 5.2.2	Charging scenarios

#### 5.2.2.0	Introduction

In order to perform event charging via Ro, the scenarios between the involved entities UE-A, OCF and CTF need to be defined. The charging flows shown in this subclause include scenarios with immediate event charging and event charging with reservation. In particular, the following cases are shown:

1	Immediate Event Charging

a)	Decentralized Unit Determination and Centralized Rating

b)	Centralized Unit Determination and Centralized Rating

c)	Decentralized Unit Determination and Decentralized Rating

2	Event charging with Unit Reservation

a)	Decentralized Unit Determination and Centralized Rating

b)	Centralized Unit Determination and Centralized Rating

c)	Decentralized Unit Determination and Decentralized Rating

3	Session charging with Unit Reservation

a)	Decentralized Unit Determination and Centralized Rating

b)	Centralized Unit Determination and Centralized Rating

c)	Decentralized Unit Determination and Decentralized Rating

The combination of Centralized Unit Determination with Decentralized Rating is not possible.

#### 5.2.2.1	Immediate Event Charging (IEC)

##### 5.2.2.1.1	Decentralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.1.1.1, CTF asks the OCF to assign a defined number of units.

- Figure 5.2.2.1.1.1: IEC - Centralized Rating and Decentralized Unit Determination

1)	Request for resource usage: UE-A requests the desired resource from the Network Element.

2)	Units Determination: depending on the requested service the CTF determines the number of units accordingly.

3)	Debit Units Request: the CTF requests the OCF to assign the defined number of units.

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represents the price for the number of units determined in item 2.

5)	Account Control: provided that the user's credit balance is sufficient, the OCF triggers the deduction of the calculated amount from the subscriber's account.

6)	Debit Units Response: the OCF informs the CTF of the number of granted units.

7)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the number of granted units.

8)	Credit Unit Control (cont.): this function block is optional and a replication of items 2 to 6.

9)	Content/Service Delivery (cont.): the continuation of content delivery occurs in correspondence with the occurrence of item 8.

10)	Session released: Session is released.

##### 5.2.2.1.2	Centralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.1.2.1, CTF asks the OCF to assign units based on the service key specified by the CTF.

- Figure 5.2.2.1.2.1: IEC - Centralized Rating and Centralized Unit Determination

1)	Request for resource usage: The UE-A requests the desired resource or content from the Network Element.

2)	Debit Units Request: depending on the service requested by the UE-A, the CTF determines the service key and forwards the Debit Units Request with service key to the OCF.

3)	Units Determination: the OCF determines the number of non-monetary units needed for the content/service delivery, based on the received service key.

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represent the price for the number of units determined in item 3.

5)	Account Control: provided that the user's credit balance is sufficient, the OCF triggers the deduction of the calculated amount from the subscriber's account.

6)	Debit Units Response: the OCF informs the CTF of the number of granted units. This includes the case where the number of units granted indicates the permission to render the service that was identified by the received service key.

7)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the number of granted units.

8)	Credit Service Control (cont.): this function block is optional and a replication of items 2 to 6.

9)	Content/Service Delivery (cont.): the continuation of content delivery occurs in correspondence with the occurrence of item 8.

10)	Session released: the session is released.

##### 5.2.2.1.3	Decentralized Unit Determination and Decentralized Rating

In  scenario figure 5.2.2.1.3.1, the CTF asks the OCF to assure the deduction of an amount of the specified number of monetary units from the subscriber's account.

- Figure 5.2.2.1.3.1: IEC - Decentralized Rating and Decentralized Unit Determination

1)	Request for resource usage: The UE-A requests the desired content from the Network Element.

2)	Units Determination: depending on the service requested by the UE-A, the CTF determines the number of units accordingly.

3)	Rating Control: the CTF calculates the number of monetary units that represent the price for the number of units determined in item 2.

4)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the calculated number of monetary units from the subscriber's account.

5)	Account Control: provided that the user's credit balance is sufficient, the OCF triggers the deduction of the calculated amount from the subscriber's account.

6)	Debit Units Response: the OCF indicates to the CTF the number of deducted monetary units.

7)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the number of units as specified in items 2 and 3.

8)	Credit Amount Control (cont.): this function block is optional and a replication of items 2 to 6.

9)	Content/Service Delivery (cont.): the continuation of content delivery occurs in correspondence with the occurrence of item 8.

10)	Session released: the session is released.

##### 5.2.2.1.4	Further options

In addition to the flows that are specified in the previous subclauses, the Debit Unit operation may alternatively be carried out concurrently with service delivery, or after completion of service delivery.

#### 5.2.2.2	Event Charging with Unit Reservation (ECUR)

##### 5.2.2.2.1	Decentralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.2.1.1, the CTF requests the reservation of units prior to service delivery. 
An account debit operation is carried out following the conclusion of service delivery.

- Figure 5.2.2.2.1.1: ECUR -  Decentralized Unit Determination and Centralized Rating

1)	Request for resource usage: The UE-A requests the desired content/service from the NE.

2)	Units Determination: depending on the requested service the CTF determines the number of units accordingly.

3)	Reserve Units Request: the CTF requests the OCF to reserve the number of units determined in item 2.

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represents the price for the number of units determined in item 2.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's account balance is sufficient then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of units.

8)	Reserved Units Supervision: simultaneously with the service delivery, the CTF monitors the consumption of the reserved units.

9)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the reserved number of units.

10)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of units from the subscriber's account. In the case that no further units are required for this service, an appropriate indication triggering the release of the remaining reservation is given.

11)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

12)	Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

13)	Debit Units Response: the OCF informs the CTF of the actually deducted units.

14)	Session Release: the session is released.

##### 5.2.2.2.2	Centralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.2.2.1, the CTF requests the OCF to reserve units based on the service key specified by the CTF. An account debit operation is carried out following the conclusion of service delivery.

- Figure 5.2.2.2.2.1: ECUR - Centralized Unit Determination and Centralized Rating

1)	Request for resource usage: The UE-A requests the desired content from the CTF.

2)	Reserve Units Request: depending on the service requested by the UE-A, the CTF determines the service key and forwards the Reserve Units Request with service key to the OCF.

3)	Units Determination: the OCF determines the number of non-monetary units needed for the content/service delivery, based on the received service key.

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represent the price for the number of units determined in item 3.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's account balance is sufficient, then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of units. This includes the case where the number of units reserved indicates the permission to render the service that was identified by the received service key.

8)	Granted Units Supervision: simultaneously with the service delivery, the CTF monitors the consumption of the reserved units.

9)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the reserved number of units.

10)	Debit Units Request: the CTF provides according to previous Reserve Units Response the request to deduct the amount of units corresponding to the consumed number of units.

11)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

12)	Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

13)	Debit Units Response: the OCF informs the CTF of the actually deducted units.

14)	Session Released: the session is released.

##### 5.2.2.2.3	Decentralized Unit Determination and Decentralized Rating

In scenario figure 5.2.2.2.3.1, the CTF request the OCF to assure the reservation of an amount of the specified number of monetary units from the subscriber's account. An account debit operation that triggers the deduction the amount from the subscriber's account is carried out following the conclusion of service delivery.

- Figure 5.2.2.2.3.1: ECUR - Centralized Unit Determination and Centralized Rating

1)	Request for resource usage: The UE-A requests the desired content from the CTF.

2)	Units Determination: depending on the service requested by the UE-A, the CTF determines the number of units accordingly.

3)	Rating Control: the CTF calculates the number of monetary units that represent the price for the number of units determined in item 2.

4)	Reserve Units Request: the CTF requests the OCF to assure the reservation of an amount corresponding to the calculated number of monetary units from the subscriber's account.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's credit balance is sufficient, then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of monetary units.

8)	Budget Control: simultaneously with the service delivery, the CTF monitors the consumption of the granted amount.

9)	Content/Service Delivery: the CTF delivers the content/service at once, in fractions or in individually chargeable items, corresponding to the number of units.

10)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of monetary units from the subscriber's account.

11)	Account Control: the OCF triggers the deduction of the consumed amount from the subscriber's account.

12)	Debit Units Response: the OCF indicates to the CTF the number of deducted monetary units.

13)	Session Released: the session is released.

#### 5.2.2.3	Session charging with Reservation

##### 5.2.2.3.1	Decentralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.3.1.1, the CTF requests the reservation of units prior to session supervision. An account debit operation is carried out following the conclusion of session termination.

- Figure 5.2.2.3.1.1: SCUR - Decentralized Unit Determination and Centralized Rating

1)	Request for resource usage: The UE-A requests session establishment from the CTF.

2)	Units Determination: depending on the requested type of the session the CTF determines the number of units accordingly.

3)	Reserve Units Request: the CTF requests the OCF to reserve the number of units determined in item 2

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represents the price for the number of units determined in item 2.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's account balance is sufficient then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of units.

8)	Reserved Units Supervision: simultaneously with the ongoing session, the CTF monitors the consumption of the reserved units.

9)	Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

10)	Session Release: the session is released

11)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of units from the subscriber's account.

12)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

13)	Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

14)	Debit Units Response: the OCF informs the CTF of the actually deducted units.

##### 5.2.2.3.2	Centralized Unit Determination and Centralized Rating

In scenario figure 5.2.2.3.2.1, the CTF requests the OCF to reserve units based on the service key specified by the CTF. An account debit operation is carried out following the conclusion of session.

- Figure 5.2.2.3.2.1: SCUR - Centralized Unit Determination and Centralized Rating

1)	Request for resource usage: The UE-A requests the session establishment from the CTF.

2)	Reserve Units Request: depending on the requested type of the session by the UE-A, the CTF determines the service key identifier and forwards the Reserve Un with service key its Request to the OCF.

3)	Units Determination: the OCF determines the number of non-monetary units needed for the content/service delivery, based on the received service key.

4)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represent the price for the number of units determined in item 3.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's account balance is sufficient, then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of units. This includes the case where the number of units reserved indicates the permission to render the service that was identified by the received service key.

8)	Granted Units Supervision: simultaneously with the ongoing session, the CTF monitors the consumption of the reserved units.

9)	Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

10)	Session Released: the session is released.

11)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of units from the subscriber's account

12)	Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

13)	Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

14)	Debit Units Response: the OCF informs the CTF of the actually deducted units.

##### 5.2.2.3.3	Decentralized Unit Determination and Decentralized Rating

In scenario figure 5.2.2.3.3.1, the CTF request the OCF to assure the reservation of an amount of the specified number of monetary units from the subscriber's account. An account debit operation that triggers the deduction the amount from the subscriber's account is carried out following the conclusion of session establishment.

- Figure 5.2.2.3.3.1: SCUR - Decentralized Unit Determination and Decentralized Rating

1)	Request for resource usage: The UE-A requests the session establishment from the CTF.

2)	Units Determination: depending on the requested type of the session by the UE-A, the CTF determines the number of units accordingly.

3)	Rating Control: the CTF calculates the number of monetary units that represent the price for the number of units determined in item 2.

4)	Reserve Units Request: the CTF requests the OCF to assure the reservation of an amount corresponding to the calculated number of monetary units from the subscriber's account.

5)	Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

6)	Reservation Control: if the user's credit balance is sufficient, then the corresponding reservation is made.

7)	Reserve Units Response: the OCF informs the CTF of the reserved number of monetary units.

8)	Budget Control: simultaneously with the ongoing session, the CTF monitors the consumption of the granted amount.

9)	Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

10)	Session Released: the session is released.

11)	Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of monetary units from the subscriber's account.

12)	Account Control: the OCF triggers the deduction of the consumed amount from the subscriber's account.

13)	Debit Units Response: the OCF indicates to the CTF the number of deducted monetary units.

### 5.2.3	Basic operations

Online Credit-Control uses two basic logical operations: Debit Units and Reserve Units.

-	"Debit Units Request"; sent from CTF  OCF
After receiving a service request from the subscriber, the CTF sends a Debit Units Request to the OCF. 
The CTF may either specify a service key (centralised unit determination) or the number of units requested (decentralised unit determination). 
For refund purpose, the CTF sends a Debit Units Request to the OCF as well.

-	"Debit Units Response"; sent from OCF  CTF 
The OCF replies with a Debit Units Response, which informs the CTF of the number of units granted as a result of the Debit Units Request. This includes the case where the number of units granted indicates the permission to render the requested service. 
For refund purpose, the OCF replies with a Debit Units Response.

-	"Reserve Units Request"; sent from CTF  OCF
Request to reserve a number of units for the service to be provided by an CTF. In case of centralized unit determination, the CTF specifies a service key in the Reserve Unit Request, and the OCF determines the number of units requested. In case of decentralised unit determination, the number of units requested is specified by the CTF.

-	"Reserve Units Response"; sent from OCF  CTF 
Response from the OCF which informs the CTF of the number of units that were reserved as a result of the "Reserve Units Request".

IEC uses the Direct Debiting One Time Event procedure specified in RFC 4006 [402] and therefore is performed by the use of the logical "Debit Units" operation, as specified in clause 6.3.3.

SCUR and ECUR use both the "Debit Units" and "Reserve Units" operations.

SCUR uses the Session Based Credit-Control procedure specified in RFC 4006 [402]. In session charging with unit reservation, when the "Debit Units" and "Reserve Units" operations are both needed, they shall be combined in one message, as specified in clause 6.3.5.

For SCUR and ECUR the consumed units are deducted from the subscriber's account after service delivery. 
Thus, the reserved and consumed units are not necessarily the same. Using this operation, it is also possible for the CTF to modify the current reservation, including the return of previously reserved units.

Table 5.2.3.1 and table 5.2.3.2 describe the content of these operations.

- Table 5.2.3.1: Debit / Reserve Units Request content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Destination Domain | M | This field contains the realm of the operation destination. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Token | M | This field contains the service identifier. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Destination Host | OC | This field contains the identification of the destination point of the operation. |
| User Name | OC | This field contains the identification of the user. |
| Origination State | OC | This field contains the state associated to the source point of the operation. |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Subscriber Identifier | OM | This field contains the identification of the mobile subscriber (i.e. MSISDN) or the fixed user (i.e. fixed device or residential gateway ) that uses the requested service. |
| Termination Cause | OC | This field contains the termination reason of the service. |
| Requested Action | OC | This field contains the requested action. |
| AoC Type | OC | This field denotes if AoC Information is requested and what type of information is needed. |
| Multiple Operation | OM | This field indicate the occurrence of multiple operations. |
| Multiple Unit Operation | OC | This field contains the parameter for the quota management. |
| Operation Correlation Identifier | OC | This field contains information to correlate messages generated for different components of the service. |
| Subscriber Equipment Number | OC | This field contains the identification of the mobile device (i.e. IMEI) that uses the subscriber. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Service Information | OM | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |

- Table 5.2.3.2: Debit / Reserve Units Response content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Operation Result | M | This field identifies the result of the operation. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Operation Failover | OC | This field contains an indication to the CTF whether or not a failover handling is to be used when necessary. |
| Multiple Unit Operation | OC | This field contains the parameter for the quota management. |
| Cost Information | OC | This field contains the cost information of the service. |
| Low Balance Indication | OC | This field indicates whether the subscriber account balance went below a designated threshold set by his account. |
| Remaining Balance | OC | This field contains the remaining balance of the subscriber. |
| Operation Failure Action | OC | For Credit-Control sessions the content of this field enables the credit-control client to decide what to do if sending credit-control messages to the credit-control server has been temporarily prevented. |
| Operation Event Failure Action | OC | For one time event direct debiting the content of this field enables the credit-control client to decide what to do if sending credit-control messages to the credit-control server has been temporarily prevented. |
| Redirection Host | OC | This field contains the host the request should be forwarded to. |
| Redirection Host Usage | OC | This field identifies the routing entry resulting from the Redirect-Host is to be used. |
| Redirection Cache Time | OC | This field contains the maximum number of seconds the peer and route table entries, created as a result of the Redirect-Host, will be cached. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Failed parameter | OC | This field contains missing and/or unsupported parameter that caused the failure. |
| Service Information | OC | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |

## 5.3	Other requirements

### 5.3.1	Re-authorization

The server may specify an idle timeout associated with a granted quota. Alternatively, the client may have a configurable default value. The expiry of that timer shall trigger a re-authorization request.

Mid-session service events (re-authorization triggers) may affect the rating of the current service usage. 
The server may instruct the Credit-Control client to re-authorize the quota upon a number of different session related triggers that can affect the rating conditions.

When a re-authorization is trigger, the client shall reports quota usage. The reason for the quota being reported shall be notified to the server.

### 5.3.2	Threshold based re-authorization triggers

The server may optionally include an indication to the client of the remaining quota threshold that shall trigger a quota re-authorization.

### 5.3.3	Termination action

The server may specify to the client the behaviour on consumption of the final granted units, or zero units granted in the first place; this is known as termination action.

### 5.3.4	Account expiration

The server may provide to the client the date and time of account expiration, which the client may use depending on operator policy.

# 6	3GPP charging applications – Protocol aspects

## 6.1	Basic principles for Diameter offline charging

### 6.1.0	Introduction

In order to support the offline charging principles described in the present document, the Diameter client and server shall implement at least the following Diameter options listed in RFC 6733 [401], i.e. the basic functionality of Diameter accounting, as defined by the Diameter Base Protocol (RFC 6733 [401]) is re-used.

The charging architecture implementing Diameter adheres to the structure where all communications for offline charging purposes between the CTF (Diameter client) and the CDF (Diameter server) are carried out on the Diameter Rf reference point, where the CTF reports charging information to the Charging Data Function (CDF). The CDF uses this information to construct and format CDRs. The above-mentioned reference points are defined in TS 32.240 [1].

A configurable timer is supported in the CDF to supervise the reception of the ACR[Interim] and/or ACR[Stop]. An instance of the "Timer" is started at the beginning of the accounting session, reset on the receipt of an ACR[Interim] and stopped at the reception of the ACR[Stop]. Upon expiration of the timer, the CDF stops the accounting session with the appropriate error indication.

For offline charging, the CTF implements the accounting state machine described in RFC 6733 [401]. 
The server (CDF) implements the accounting state machine "SERVER, STATELESS ACCOUNTING" as specified in RFC 6733 [401], i.e. there is no order in which the server expects to receive the accounting information.

The offline charging functionality is based on the Network Elements reporting accounting information upon reception of various messages which trigger charging generation, as most of the accounting relevant information is contained in these messages. This reporting is achieved by sending Diameter Accounting Requests (ACR) [start, interim, stop and event] from the Network Elements to the CDF.

Following the Diameter base protocol specification, the following "types" of accounting data may be sent with regard to offline charging:

- START session accounting data.

- INTERIM session accounting data.

- STOP session accounting data.

- EVENT accounting data.

Two cases are currently distinguished for offline charging purposes:

- Event based charging; and

- Session based charging.

ACR types START, INTERIM and STOP are used for accounting data related to successful sessions. In contrast, EVENT accounting data is unrelated to sessions, and is used e.g. for a simple registration or interrogation and successful service event triggered by a Network Element. In addition, EVENT accounting data is also used for unsuccessful session establishment attempts.

The flows and scenarios for the above two described cases are further detailed below.

### 6.1.1	Event based charging

In the case of event based charging, the network reports the usage or the service rendered where the service offering is rendered in a single operation. It is reported using the ACR[Event].

Figure 6.1.1.1 shows the transactions that are required on the Diameter offline interface in order to perform event based charging. The operation may alternatively be carried out prior to, concurrently with or after service/content delivery.

- Figure 6.1.1.1: Event based offline charging

Step 1:	The Network Element receives indication that service has been used/delivered.

Step 2:	The Network Element (acting as client) sends Accounting-Request (ACR) with Accounting-Record-Type AVP set to EVENT_RECORD to indicate service specific information to the CDF (acting as server).

Step 3:	The CDF receives the relevant service charging parameters and processes accounting request.

Step 4:	The CDF returns Accounting-Answer message with Accounting-Record-Type AVP set to EVENT_RECORD to the Network Element in order to inform that charging information was received.

### 6.1.2	Session based charging

Session based charging is the process of reporting usage reports for a session and uses the START, INTERIM and STOP accounting data. During a session, a Network Element may transmit multiple ACR Interims' depending on the proceeding of the session.

Figure 6.1.2.1 shows the transactions that are required on the Diameter offline interface in order to perform session based charging.

- Figure 6.1.2.1: Session based offline charging

Step 1:	The Network Element receives a service request. The service request may be initiated either by the user or the other Network Element.

Step 2:	In order to start accounting session, the Network Element sends a Accounting-Request (ACR) with Accounting-Record-Type AVP set to START_RECORD to the CDF.

Step 3:	The CDF opens a CDR for current session.

Step 4:	The CDF returns Accounting-Answer (ACA) message with Accounting-Record-Type set to START_RECORD to the Network Element and possibly Acct-Interim-Interval AVP (AII) set to non-zero value indicating the desired intermediate charging interval.

Step 5:	When either AII elapses or charging conditions changes are recognized at Network Element (NE), the NE sends an Accounting-Request (ACR) with Accounting-Record-Type AVP set to INTERIM_RECORD to the CDF.

Step 6:	The CDF updates the CDR in question.

Step 7:	The  CDF returns Accounting-Answer (ACA) message with Accounting-Record-Type set to INTERIM_RECORD to the Network Element.

Step 8:	The service is terminated.

Step 9:	The Network Element sends a Accounting-Request (ACR) with Accounting-Record-Type AVP set to STOP_RECORD to the CDF.

Step 10:	The CDF updates the CDR accordingly and closes the CDR.

Step 11:	The CDF returns Accounting-Answer (ACA) message with Accounting-Record-Type set to STOP_RECORD to the Network Element.

### 6.1.3	Offline charging error cases - Diameter procedures

#### 6.1.3.1	CDF connection failure

When the connection towards the primary CDF is broken, the process of sending accounting information should continue towards a secondary CDF (if such a CDF is configured). For further CDF connection failure functionality, see clause "Transport Failure Detection" in the RFC 6733 [401].

If no CDF is reachable the Network Element may buffer the generated accounting data in non-volatile memory. Once the CDF connection is working again, all accounting messages stored in the buffer is sent to the CDF, in the order they were stored in the buffer.

#### 6.1.3.2	No reply from CDF

In case a Network Element does not receive an ACA in response to an ACR, it may retransmit the ACR message. The waiting time until a retransmission is sent, and the maximum number of repetitions are both configurable by the operator. When the maximum number of retransmissions is reached and still no ACA reply has been received, the Network Element executes the CDF connection failure procedure as specified above.

If retransmitted ACRs' are sent, they are marked with the T-flag as described in RFC 6733 [401], in order to allow duplicate detection in the CDF, as specified in the next clause 6.1.3.3.

#### 6.1.3.3	Duplicate detection

A Diameter client marks possible duplicate request messages (e.g. retransmission due to the link fail over process) with the T-flag as described in RFC 6733 [401].

If the CDF receives a message that is marked as retransmitted and this message was already received, then it discards the duplicate message. However, if the original of the re-transmitted message was not yet received, it is the information in the marked message that is taken into account when generating the CDR. The CDRs are marked if information from duplicated message(s) is used.

#### 6.1.3.4	CDF detected failure

The CDF closes a CDR when it detects that expected Diameter ACRs for a particular session have not been received for a period of time. The exact behaviour of the CDF is operator configurable.

## 6.2	Message contents for offline charging

### 6.2.1	Summary of offline charging message formats

#### 6.2.1.1	General

The corresponding Diameter accounting application messages for the Charging Data Transfer operation is Accounting-Request (ACR) and Accounting-Answer (ACA) as specified in the Diameter Base Protocol Accounting (DBPA) application in RFC 6733 [401].

Table 6.2.1.1.1 describes the use of these messages which are adapted for 3GPP offline charging.

- Table 6.2.1.1.1: Offline charging messages reference table

| Command-Name | Source | Destination | Abbreviation |
| --- | --- | --- | --- |
| Accounting-Request | CTF | CDF | ACR |
| Accounting-Answer | CDF | CTF | ACA |
| Capabilities-Exchange-Request | CTF | CDF | CER |
| Capabilities-Exchange-Answer | CDF | CTF | CEA |

Additional Diameter messages (i.e. DPR/DPA, DWR/DWA)are used according to the Diameter Base Protocol Accounting (DBPA) specification in RFC 6733 [401].

#### 6.2.1.2	Structure for the Accounting message formats

The following is the basic structure shared by all offline charging messages. This is based directly on the format of the messages defined in the Diameter Base Protocol Application specification in RFC 6733 [401].

Those Diameter Accounting AVPs that are used for 3GPP offline charging are marked in table 6.2.2.1 and table 6.2.3.1 with a category as specified in TS 32.240 [1].

An AVP in grey strikethrough in the message format (in grey in the tables) is not used by 3GPP.

The following symbols are used in the message format definition:

-	<AVP> indicates a mandatory AVP with a fixed position in the message.

-	{AVP} indicates a mandatory AVP in the message.

-	[AVP] indicates an optional AVP in the message.

-	*AVP indicates that multiple occurrences of an AVP is possible.

### 6.2.2	Accounting-Request message

The ACR messages, indicated by the Command-Code field set to 271 is sent by the CTF to the CDF in order to send charging information for the requested bearer / subsystem /service.

The ACR message format is defined according to the Diameter Base Protocol in RFC 6733 [401] as follows:

<ACR> ::= < Diameter Header: 271, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

[ Acct-Application-Id ]

[ Vendor-Specific-Application-Id ]

[ User-Name ]

[ Destination-Host ]

[ Accounting-Sub-Session-Id ]

[ Acct-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Acct-Interim-Interval ]

[ Accounting-Realtime-Required ]

[ Origin-State-Id ]

[ Event-Timestamp ]

* [ Proxy-Info ]

* [ Route-Record ]

[ Service-Context-Id ]

[ Service-Information ]

* [ AVP ]

NOTE:	Similar information as in subscription_id should be added as 3GPP parameter, IMEI.

Table 6.2.2.1 illustrates the basic structure of a 3GPP Diameter Accounting-Request message as used for 3GPP offline charging.

- Table 6.2.2.1: 3GPP Accounting-Request message contents

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Destination-Realm | M | This field contains the realm of the operator domain. The realm will be addressed with the domain address of the corresponding public URI. |
| Accounting-Record-Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Accounting-Record-Number | M | This field contains the sequence number of the transferred messages. |
| Acct-Application-Id | OM | The field corresponds to the application ID of the Diameter Accounting Application and is defined with the value 3. |
| Vendor-Specific-Application-Id | - | Not used in 3GPP. |
| Vendor-Id | - | Not used in 3GPP. |
| Auth-Application-Id | - | Not used in 3GPP. |
| Acct-Application-Id | - | Not used in 3GPP. |
| User-Name | OC | Contains the user name determined by the domain: bearer, sub-system or service as described in middle tier TS. |
| Destination-Host | OC | This field contains the destination address of the CDF. |
| Accounting-Sub-Session-Id | - | Not used in 3GPP. |
| Accounting-Session-Id | - | Not used in 3GPP. |
| Acct-Multi-Session-Id | - | Not used in 3GPP. |
| Acct-Interim-Interval | OC | This field contains the proposal for instructions to produce records continuously during a session. |
| Accounting-Realtime-Required | - | Not used in 3GPP. |
| Origin-State-Id | OC | This field contains the state associated to the CTF. |
| Event-Timestamp | OC | This field corresponds to the exact time the accounting is requested. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | Oc | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| Service-Context-Id | OM | This field indicates the service and the corresponding 'middle tier' TS. |
| Service-Information | OM | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |
| AVP | Oc | This field contains extended Information. |

NOTE:	A detailed description of the AVPs is provided in clause 7.

### 6.2.3	Accounting-Answer (ACA) message

The ACA messages, indicated by the Command-Code field set to 271 is sent by the CDF to the CTF in order to reply to the ACR.

The ACA message format is defined according to the Diameter Base Protocol in RFC 6733 [401] as follows:

<ACA> ::= < Diameter Header: 271, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

[ Acct-Application-Id ]

[ Vendor-Specific-Application-Id ]

[ User-Name ]

[ Accounting-Sub-Session-Id ]

[ Acct-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ Acct-Interim-Interval ]

[ Failed-AVP ]

[ Accounting-Realtime-Required ]

[ Origin-State-Id ]

[ Event-Timestamp ]

* [ Proxy-Info ]

* [ AVP ]

Table 6.2.3.1 illustrates the basic structure of a 3GPP Diameter Accounting-Answer message as used for offline charging. This message is always used by the CDF as specified below, regardless of the CTF it is received from and the ACR record type that is being replied to.

- Table 6.2.3.1: 3GPP Accounting-Answer (ACA) message content

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Result-Code | M | This field contains the result of the specific query. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Accounting-Record-Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Accounting-Record-Number | M | This field contains the sequence number of the transferred messages. |
| Acct-Application-Id | OM | The field corresponds to the application ID of the Diameter Accounting Application and is defined with the value 3. |
| Vendor-Specific-Application-Id | - | Not used in 3GPP |
| Vendor-Id | - | Not used in 3GPP |
| Auth-Application-Id | - | Not used in 3GPP |
| Acct-Application-Id | - | Not used in 3GPP |
| User-Name | OC | Contains the user name determined by the domain: bearer, sub-system or service as described in middle tier TS. |
| Accounting-Sub-Session-Id | - | Not used in 3GPP |
| Accounting-RADIUS-Session-Id | - | Not used in 3GPP |
| Acct-Multi-Session-Id | - | Not used in 3GPP |
| Error-Message | OC | This field contains a human readable error message. |
| Error-Reporting-Host | OC | This field contains the identity of the Diameter host that sent the Result-Code AVP to a value other than 2001 (Success) if the host setting the Result-Code is different from the one encoded in the Origin-Host AVP. |
| Acct-Interim-Interval | OC | This field contains the instructions to produce records continuously during a session. |
| Accounting-Realtime-Required | - | Not used in 3GPP |
| Failed-AVP | OC | This field contains the AVP that could not be processed sucessfully. This field can occur only once. |
| Origin-State-Id | OC | This field contains the state associated to the source point of the operation. |
| Event-Timestamp | OC | This field contains the time when the operation is requested. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| AVP | OC | This field contains extended Information. |

## 6.3	Basic principles for Diameter online charging

### 6.3.1	Online Specific Credit-Control application requirements

For online charging, the basic functionality as defined by the IETF Diameter Credit-Control Application is used. 
The basic structure follows a mechanism where the online client (CTF) requests resource allocation and reports Credit-Control information to the Online Charging System (OCS).

The usage and values of Validity-Time AVP and the timer "Tcc" are under the sole control of the Credit-Control server (OCS) and determined by operator configuration of the OCS.

The online client implements the state machine described in RFC 4006 [402] for "CLIENT, EVENT BASED" and/or "CLIENT, SESSION BASED". I.e. when the client applies IEC it uses the "CLIENT, EVENT BASED" state machine, and when the client applies ECUR defined in 3GPP it uses the "CLIENT, SESSION BASED" state machine for the first and final interrogations.

The OCS implements the state machine described in RFC 4006 [402] for the "SERVER, SESSION AND EVENT BASED" in order to support Immediate Event Charging and Event Charging with Unit Reservation.

### 6.3.2	Diameter description on the Ro reference point

#### 6.3.2.1	Basic principles

For online charging the Diameter Credit-Control Application (DCCA) defined in RFC 4006 [402] is used with additional AVPs defined in the present document.

Three cases for control of user credit for online charging are distinguished:

- -	Immediate Event Charging IEC; and

- -	event charging with unit reservation (ECUR).

- -	Session Charging with Unit Reservation (SCUR)

In the case of Immediate Event Charging (IEC),the Credit-Control process for events is controlled by the corresponding CC-Requested-Type EVENT_REQUEST that is sent with Credit-Control-Request (CCR) for a given Credit-Control event.

In the case of Event Charging with Unit Reservation (ECUR) the CC-Request-Type INITIAL / TERMINATION_REQUEST are used for charging for a given Credit-Control event, however, where a reservation is made prior to service delivery and committed on execution of a successful delivery.

Session Charging with Unit Reservation is used for Credit-Control of sessions and uses the CC-Request-Type INITIAL / UPDATE and TERMINATION_REQUEST.

The Network Element may apply IEC, where CCR event messages are generated, or ECUR, using CCR Initial and Termination. The decision whether to apply IEC or ECUR is based on the service and/or operator's policy.

### 6.3.3	Immediate Event Charging (IEC)

Figure 6.3.3.1 shows the transactions that are required on the Ro reference point in order to perform event based Direct Debiting operation. The Direct Debiting operation may alternatively be carried out prior to service/content delivery. 
the Network Element shall ensure that the requested service execution is successful, when this scenario is used.

- Figure 6.3.3.1: IEC - Direct Debiting operation

Step 1.	The Network Element receives a service request.
The Direct Debiting operation is performed as described in RFC 4006 [402].

Step 2.	The Network Element performs direct debiting prior to service execution. Network element (acting as DCCA client) sends CCR with CC-Request-Type AVP set to EVENT_REQUEST to indicate service specific information to the OCS (acting as DCCA server). The Requested-Action AVP (RA) is set to DIRECT_DEBITING. If known, the Network Element may include Requested-Service-Unit AVP (RSU) (monetary or non-monetary units) in the request message.

Step 3.	Having transmitted the CCR message the Network Element starts the communication supervision timer 'Tx' (RFC 4006 [402]). Upon receipt of the Credit-Control- Answer (CCA) message the Network Element shall stop timer Tx.

Step 4.	The OCS determines the relevant service charging parameters.

Step 5.	The OCS returns CCA message with CC-Request-Type AVP set to EVENT_REQUEST to the Network Element in order to authorize the service execution
(Granted-Service-Unit AVP (GSU) and possibly Cost-Information AVP (CI) indicating the cost of the service and Remaining-Balance AVP are included in the CCA message).
The CCA message has to be checked by the Network Element accordingly and the requested service is controlled concurrently with service delivery. The Refund-Information AVP may be included in the CCA message in order to be sent during  the REFUND-ACCOUNT mechanism, see below scenario.

Step 6.	Service is being delivered.

NOTE:	It is possible to perform also, CHECK_BALANCE and PRICE_ENQUIRY using above described mechanism RFC 4006 [402].

Figure 6.3.3.2 shows the transactions for refund purpose.

- Figure 6.3.3.2: IEC - Direct Debiting operation for refund purpose

The Direct debiting operation is performed, previously, as described in RFC 4006 [402].

Step 1.	The service charged previously through Direct Debiting Operation is finally proved to be unsuccessfully delivered.

Step 2.	As a consequence, the Network Element performs direct debiting operation in order to perform the related refund. Network element (acting as DCCA client) sends CCR with CC-Request-Type AVP set to EVENT_REQUEST to indicate service specific information to the OCS (acting as DCCA server). The Requested-Action AVP (RA) is set to REFUND-ACCOUNT. 
The Network Element includes Refund-Information AVP if received in the previous IEC CCA.

Step 3.	Having transmitted the CCR message the Network Element starts the communication supervision timer 'Tx' (RFC 4006 [402]). Upon receipt of the Credit-Control- Answer (CCA) message the Network Element shall stop timer Tx.

Step 4.	The OCS reads the AVPs included in the CCR and performs the refund accordingly.

Step 5.	The OCS returns CCA message with CC-Request-Type AVP set to EVENT_REQUEST and the related result code.

### 6.3.4	Event Charging with Unit Reservation (ECUR)

Figure 6.3.4.1 shows the transactions that are required on the Ro reference point in order to perform the ECUR. ECUR is used when event charging needs separate reserve and commit actions.

- Figure 6.3.4.1: ECUR - Session based Credit-Control

Step 1.	The Network Element receives a service request. The service request may be initiated either by the user or the other Network Element.

Step 2.	In order to perform Reserve Units operation for a number of units (monetary or non-monetary units), the Network Element sends a CCR with CC-Request-Type AVP set to INITIAL_REQUEST to the OCS. If known, the Network Element may include Requested-Service-Unit (RSU) AVP (monetary or non monetary units) in the request message.

Step 3.	If the service cost information is not received by the OCS, the OCS determines the price of the desired service according to the service specific information received by issuing a rating request to the Rating Function. If the cost of the service is included in the request, the OCS directly reserves the specified monetary amount. If the credit balance is sufficient, the OCS reserves the corresponding amount from the users account.

Step 4.	Once the reservation has been made, the OCS returns CCA message with CC-Request-Type set to INITIAL_REQUEST to the Network Element in order to authorize the service execution (Granted-Service-Unit and possibly Cost-Information indicating the cost of the service and Remaining-Balance AVP are included in the CCA message).  The OCS may return the Validity-Time (VT) AVP with value field set to a non-zero value. The OCS may indicate in the Low-Balance-Indication AVP that the subscriber account balance has fallen below a predefined treshold of this account.

Step 5.	Content/service delivery starts and the reserved units are concurrently controlled.

Step 6.	When content/service delivery is completed, the Network Element sends CCR with CC-Request-Type AVP set to TERMINATION_REQUEST to terminate the active Credit-Control session and report the used units.

Step 7.	The OCS deducts the amount used from the account. Unused reserved units are released, if applicable.

Step 8.	The OCS acknowledges the reception of the CCR message by sending CCA message with CC-Request-Type AVP indicating TERMINATION_REQUEST (possibly Cost-Information AVP indicating the cumulative cost of the service and Remaining-Balance AVP are included in the CCA message).

NOTE:	This scenario is supervised by corresponding timers (e.g. validity time timer) that are not shown in the figure 6.3.4.1.

### 6.3.5	Session Charging with Unit Reservation (SCUR)

Figure 6.3.5.1 shows the transactions that are required on the Ro reference point in order to perform the SCUR.

- Figure 6.3.5.1: SCUR - Session based CCredit-Control

Step 1.	The Network Element receives a session initiation. The session initiation may be done either by the user or the other Network Element.

Step 2.	In order to perform Reserve Units operation for a number of units (monetary or non-monetary units), the Network Element sends a CCR with CC-Request-Type AVP set to INITIAL_REQUEST to the OCS. If known, the Network Element may include Requested-Service-Unit (RSU) AVP (monetary or non monetary units) in the request message.

Step 3.	If the service cost information is not received by the OCS, the OCS determines the price of the desired service according to the service specific information received by issuing a rating request to the Rating Function. If the cost of the service is included in the request, the OCS directly reserves the specified monetary amount. If the credit balance is sufficient, the OCS reserves the corresponding amount from the users account.

Step 4.	Once the reservation has been made, the OCS returns CCA message with CC-Request-Type set to INITIAL_REQUEST to the Network Element in order to authorize the service execution (Granted-Service-Unit and possibly Cost-Information indicating the cost of the service and Remaining-Balance AVP are included in the CCA message).  The OCS may return the Validity-Time (VT) AVP with value field set to a non-zero value. The OCS may indicate in the Low-Balance-Indication AVP that the subscriber account balance has fallen below a predefined threshold of this account.

Step 5.	Content/service delivery starts and the reserved units are concurrently controlled.

Step 6.	During session delivery, in order to perform Debit Units and subsequent Reserve Units operations, the Network Element sends a CCR with CC-Request-Type AVP set to UPDATE_REQUEST, to report the units used and request additional units, respectively. The CCR message with CC-Request-Type AVP set to UPDATE_REQUEST shall be sent by the Network Element between the INITIAL_REQUEST and TERMINATION_REQUEST either on request of the Credit-Control application within the validity time or if the validity time is elapsed. If known, the Network Element may include Requested-Service-Unit AVP (monetary or non monetary units) in the request message.  The Used-Service-Unit (USU) AVP is complemented in the CCR message to deduct units from both the user's account and the reserved units, respectively.

Step 7.	The OCS deducts the amount used from the account. If the service cost information is not received by the OCS, the OCS determines the price of the desired service according to the service specific information received by issuing a rating request to the Rating Function. If the cost of the service is included in the request, the OCS directly reserves the specified monetary amount. If the credit balance is sufficient, the OCS reserves the corresponding amount from the users account.

Step 8.	Once the deduction and reservation have been made, the OCS returns CCA message with CC-Request-Type set to UPDATE_REQUEST to the Network Element, in order to allow the content/service delivery to continue (new Granted-Service-Unit (GSU) AVP and possibly Cost-Information (CI) AVP indicating the cumulative cost of the service and Remaining-Balance AVP are included in the CCA message). The OCS may include in the CCA message the Final-Unit-Indication (FUI) AVP to indicate the final granted units. The OCS may indicate in the Low-Balance-Indication AVP that the subscriber account balance has fallen below a predefined threshold of this account.

Step 9.	Session delivery continues and the reserved units are concurrently controlled.

Step 10.	The session is terminated at the Network Element.

Step 11. 	The Network Element sends CCR with CC-Request-Type AVP set to TERMINATION_REQUEST to terminate the active Credit-Control session and report the used units.

Step 12.	The OCS deducts the amount used from the account. Unused reserved units are released, if applicable.

Step 13.	The OCS acknowledges the reception of the CCR message by sending CCA message with CC-Request-Type AVP indicating TERMINATION_REQUEST (possibly Cost-Information AVP indicating the cumulative cost of the service and Remaining-Balance AVP are included in the CCA message).

NOTE:	This scenario is supervised by corresponding timers (e.g. validity time timer) that are not shown in figure 6.3.5.1.

### 6.3.6	Error cases and scenarios

#### 6.3.6.0	Introduction

This clause describes various error cases and how these should be handled.

The failure handling behaviour is locally configurable in the Network Element. If the Direct-Debiting-Failure-Handling or Credit-Control-Failure-Handling AVP is not used, the locally configured values are used instead.

#### 6.3.6.1	Duplicate detection

The detection of duplicate request is needed and shall be enabled. To speed up and simplify as much as possible the duplicate detection, the all-against-all record checking should be avoided and just those records marked as potential duplicates need to be checked against other received requests (in real-time ) by the receiver entity.

The Network Element marks the request messages that are retransmitted after a link fail over as possible duplicates with the T-flag as described in RFC 6733 [401]. For optimized performance, uniqueness checking against other received requests is only necessary for those records marked with the T-flag received within a reasonable time window. This focused check is based on the inspection of the Session-Id and CC-Request-Number AVP pairs.

Note that for EBCC the duplicate detection is performed in the Correlation Function that is part of the OCS. 
The OCS that receives the possible duplicate request should mark as possible duplicate the corresponding request that is sent over the 'Rc' reference point. However, this assumption above is for further study and needs to be clarified.

For Credit-Control duplicate detection, please refer to the Diameter Credit-Control.

#### 6.3.6.2	Reserve Units / Debit Units operation failure

In the case of an OCS connection failure, and/or receiving error responses from the OCS, please refer to RFC 6733 [401] and RFC 4006 [402] for failure handling descriptions.

### 6.3.7	Support of tariff changes during an active user session

#### 6.3.7.1	Support of tariff changes using the tariff switch mechanism

After a tariff switch has been reached, all the active user sessions shall report their session usage by the end of the validity period of the current request and receive new quota for resource usage for the new tariff period.

In order to avoid the need for mass simultaneous quota refresh, the traffic usage can be split into resource usage before a tariff switch and resources used after a tariff switch.

The Tariff-Time-Change AVP is used to determine the tariff switch time as described by RFC 4006 [402]. 
In addition to the scenarios described in RFC 4006 [402], the Tariff-Time-Change AVP may also be used in the context of continuously time-based charging.

The Tariff-Change-Usage AVP is used within the Used-Service-Units AVP to distinguish reported usage before and after the tariff time change.

The Tariff-Change-Usage AVP is not used directly within the Multiple-Services-Credit-Control AVP.

#### 6.3.7.2	Support of tariff changes using Validity-Time AVP

Changes to the tariffs pertaining to the service during active user sessions may also be handled using the Validity Time AVP.

NOTE:	RFC 4006 [402] does not directly describe how tariff changes are handled with validity time. 
If validity time is used for tariff time changes it might overload the client and the server.

### 6.3.8	Support of re-authorization

Mid Diameter CC session re-authorizations of multiple active resource quotas within a DCC session can be achieved using a single Diameter Credit-Control- Request/ Credit-Control- Answer message sequence.

The OCS may also re-authorize multiple active resource quotas within a DCC session by using a single Diameter Re-Auth-Request/Answer message sequence.

New quota allocations received by the Network Element override any remaining held quota resources after accounting for any resource usage while the re-authorization was in progress.

### 6.3.9	Support of failure handling

The Credit-Control-Failure-Handling AVP as defined in RFC 4006 [402] determines what to do if the sending of Diameter credit-control messages to the OCS has been temporarily prevented. The usage of Credit-Control-Failure-Handling AVP gives flexibility to have different failure handling for credit-control session.

This AVP may be received from the OCS or may be locally configured. The value received from the OCS in the Diameter CCA message always override any already existing value.

As defined in RFC 4006 [402], the Tx timer is introduced to limit the waiting time in the CTF for an answer to the CCR sent to the OCS. When the Tx timer elapses the CTF takes an action to the end user according to the value of the Credit-Control-Failure-Handling AVP.

It is possible that several concurrent CCR messages are triggered for the same online charging session. In this case, each CCR message shall reset the Tx timer as defined in RFC 4006 [402].

### 6.3.10	Support of failover

As defined in RFC 4006 [402] if a failure occurs during an ongoing credit-control session, the CTF may move the Credit-Control message stream to an alternative OCS if the primary OCS indicated FAILOVER_SUPPORTED in the CC-Session-Failover AVP. In case CC-Session-Failover AVP is set to FAILOVER_NOT SUPPORTED the Credit-Control message stream is not moved to a backup OCS.

For new Credit-Control sessions, failover to an alternative OCS should be performed if possible. For instance, if an implementation of the CTF can determine primary OCS unavailability it can establish the new Credit-Control sessions with a possibly available secondary OCS.

Since the OCS has to maintain session states, moving the credit-control message stream to a backup OCS requires a complex charging context transfer solution. This charging context transfer mechanism by OCS is out of the scope of  the 3GPP standardization work.

### 6.3.11	Credit pooling

Credit pooling shall be supported as described in TS 32.240 [1].

NOTE: 	Credit pooling is not applicable to IEC since there is no quota management between CTF and OCF.

## 6.4	Message formats for online charging

### 6.4.1	Summary of online charging message formats

#### 6.4.1.1	General

The corresponding Diameter Credit-Control application messages for the Debit / Reserve Unit Request operation is Credit-Control-Request (CCR) and for the Debit / Reserve Unit Response operation is Credit-Control-Answer (CCA) as specified in RFC 4006 [402].

The Diameter Credit-Control Application (DCCA) specifies an approach based on a series of "interrogations":

- 1. Initial interrogation.

- 2. Zero, one or more interim interrogations.

- 3. Final interrogation.

In addition to a series of interrogations, also a one time event (interrogation) can be used e.g. in the case when service execution is always successful.

All of these interrogations use CCR and CCA messages. The CCR for the "interim interrogation" and "final interrogation" reports the actual number of "units" that were used, from what was previously reserved. This determines the actual amount debited from the subscriber's account.

Table 6.4.1.1.1 describes the use of these Diameter messages which are adapted for 3GPP online charging.

- Table 6.4.1.1.1: Online charging messages reference table

| Command-Name | Source | Destination | Abbreviation |
| --- | --- | --- | --- |
| Credit-Control-Request | CTF | OCS | CCR |
| Credit-Control-Answer | OCS | CTF | CCA |
| Capabilities-Exchange-Request | CTF | OCS | CER |
| Capabilities-Exchange-Answer | OCS | CTF | CEA |

Additional Diameter messages (i.e. ASR/ASA, DPR/DPA, DWR/DWA, RAR/RAA) are used according to the Diameter Base Protocol Accounting (DBPA) specification in RFC 6733 [401] and to the DCCA specification in RFC 4006 [402].

#### 6.4.1.2	Structure for the Credit-Control message formats

The following is the basic structure shared by all online charging messages. This is based directly on the format of the messages defined in RFC 4006 [402].

Those Diameter accounting AVPs that are used for 3GPP online charging are marked in the table of contents 6.4.2 and 6.4.3 with a category as specified in TS 32.240 [1].

In the definition of the Diameter commands, the AVPs that are specified in the referenced specifications but not used by the 3GPP charging specifications are marked with strikethrough, e.g. [ Acct-Multi-Session-Id ].

The following symbols are used in the message format definitions:

- -	<AVP> indicates a mandatory AVP with a fixed position in the message.

- -	{AVP} indicates a mandatory AVP in the message.

- -	[AVP] indicates an optional AVP in the message.

- -	*AVP indicates that multiple occurrences of an AVP is possible.

### 6.4.2	Credit-Control-Request message

The CCR messages, indicated by the Command-Code field set to 272 is sent by the CTF to the OCF in order to request credits for the request bearer / subsystem /service.

The CCR message format is defined according to RFC 4006 [402] as follows:

<CCR> ::= < Diameter Header: 272, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Application-Id }

{ Service-Context-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ Destination-Host ]

[ User-Name ]

[ CC-Sub-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Origin-State-Id ]

[ Event-Timestamp ]

*[ Subscription-Id ]

[ Service-Identifier ]

[ Termination-Cause ]

[ Requested-Service-Unit ]

[ Requested-Action ]

*[ Used-Service-Unit ]

[ AoC-Request-Type ]

[ Multiple-Services-Indicator ]

*[ Multiple-Services-Credit-Control ]

*[ Service-Parameter-Info ]

[ CC-Correlation-Id ]

[ User-Equipment-Info ]

*[ Proxy-Info ]

*[ Route-Record ]

[ Service-Information ]

*[ AVP ]

Table 6.4.2.1 illustrates the basic structure of a 3GPP Diameter CCR message as used for online charging.

- Table 6.4.2.1: 3GPP CCR message content

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Destination-Realm | M | This field contains the realm of the operator domain. The realm will be addressed with the domain address of the corresponding public URI. |
| Auth-Application-Id | M | The field corresponds to the application ID of the Diameter Credit-Control Application and is defined with the value 4. |
| Service-Context-Id | M | This field indicates the supported protocol version. |
| CC-Request-Type | M | This field defines the transfer type: event for event based charging and initial, update, terminate for session based charging. |
| CC-Request-Number | M | This field contains the sequence number of the transferred messages. |
| Destination-Host | OC | This field contains the destination peer address of the OCS identity. |
| User-Name | OC | Contains the user name determined by the domain: bearer, sub-system or service as described in middle tier TS. |
| CC-Sub-Session-Id | - | Not used in 3GPP. |
| Acct-Multi-Session-Id | - | Not used in 3GPP. |
| Origin-State-Id | OC | This field contains the state associated to the CTF. |
| Event-Timestamp | OC | This field corresponds to the exact time the quota is requested. |
| Subscription-Id | OM | This field contains the identification of the user that is going to access the service in order to be identified by the OCS. |
| Subscription-Id-Type | M | This field determines the type of the identifier, e.g. t value 0 is used for the international E.164 format according to ITU-T E.164 numbering plan. |
| Subscription-Id-Data | M | This field contains the user data content e.g. the MSISDN. |
| Service-Identifier | - | Not used in 3GPP. |
| Termination-Cause | OC | This field contains the reason the Credit-Control session was terminated. |
| Requested-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Requested-Action | OC | The field defines the type of action if the CC-Request-Type indicates EVENT. |
| AoC-Request-Type | Oc | This field denotes if AoC Information is requested and what type of information is needed. |
| Used-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Tariff-Change-Usage | - | Not used in 3GPP. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Multiple-Services-Indicator | OM | This field indicates whether the CTF is capable of handling multiple services independently. |
| Multiple-Services-Credit-Control | OC | This field contains all parameters for the CTF quota management and defines the quotas to allow traffic to flow. |
| Granted-Service-Unit | - | Not used in CCR. |
| Tariff-Time-Change | - | Not used in CCR. |
| CC-Time | - | Not used in CCR. |
| CC-Money | - | Not used in 3GPP . |
| Unit-Value | - | Not used in 3GPP . |
| Value-Digits | - | Not used in 3GPP . |
| Exponent | - | Not used in 3GPP . |
| Currency-Code | - | Not used in 3GPP . |
| CC-Total-Octets | - | Not used in CCR. |
| CC-Input-Octets | - | Not used in CCR. |
| CC-Output-Octets | - | Not used in CCR. |
| CC-Service-Specific-Units | - | Not used in CCR. |
| AVP | - | Not used in 3GPP. |
| Requested-Service-Unit | OC | This field contains the amount of requested service units for a particular category or an indication that units are needed for a particular category, as defined in DCCA [402]. |
| CC-Time | OC | This field contains the amount of requested time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the requested amount of octets to be sent and received. |
| CC-Input-Octets | OC | This field contains the requested amount of octets to be received. |
| CC-Output-Octets | OC | This field contains the requested amount of octets to be sent. |
| CC-Service-Specific-Units | OC | This field contains the requested amount of service specific units, e.g. number of events. |
| AVP | - | Not used in 3GPP. |
| Used-Service-Unit | OC | This field contains the amount of used non-monetary service units measured for a particular category to a particular quota type. |
| Reporting-Reason | OC | Used as defined in clause 7.2. |
| Tariff-Change-Usage | OC | This field identifies the reporting period for the used service unit, i.e. before, after or during tariff change. |
| CC-Time | OC | This field contains the amount of used time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the amount of sent and received octets. |
| CC-Input-Octets | OC | This field contains the amount of received octets. |
| CC-Output-Octets | OC | This field contains the amount of sent octets. |
| CC-Service-Specific-Units | OC | This field contains the amount of service specific units, e.g. number of events. |
| Event-Charging-TimeStamp | Oc | Used as defined in clause 7.2. |
| AVP | - | Not used in 3GPP. |
| Tariff-Change-Usage | - | Not used in 3GPP . |
| Service-Identifier | OC | This field contains identity of the used service. This ID with the Service-Context-ID together forms an unique identification of the service. |
| Rating-Group | OC | This field contains the identifier of a rating group. |
| G-S-U-Pool-Reference | - | Not used in CCR. |
| G-S-U-Pool-Identifier | - | Not used in CCR. |
| CC-Unit-Type | - | Not used in CCR. |
| Unit-Value | - | Not used in CCR. |
| Value-Digits | - | Not used in CCR. |
| Exponent | - | Not used in CCR. |
| Validity-Time | - | Not used in CCR. |
| Result-Code | - | Not used in CCR. |
| Final-Unit-Indication | - | Not used in CCR. |
| Final-Unit-Action | - | Not used in CCR. |
| Restriction-Filter-Rule | - | Not used in CCR. |
| Filter-Id | - | Not used in CCR. |
| Redirect-Server | - | Not used in CCR. |
| Redirect-Address-Type | - | Not used in CCR. |
| Redirect-Server-Address | - | Not used in CCR. |
| Time-Quota-Threshold | - | Not used in CCR. |
| Volume-Quota-Threshold | - | Not used in CCR. |
| Unit-Quota-Threshold | - | Not used in CCR. |
| Quota-Holding-Time | - | Not used in CCR. |
| Quota-Consumption-Time | - | Not used in CCR. |
| Reporting-Reason | OC | Used as defined in clause 7.2. |
| Trigger | OC | Used as defined in clause 7.2. |
| Trigger-Type | OC | Used as defined in clause 7.2. |
| PS-Furnish-Charging-Information | - | Not used in CCR. |
| Refund-Information | OC | Used as defined in clause 7.2. |
| AF-Correlation-Information | OC | Used as defined in clause 7.2. |
| Envelope | OC | Used as defined in clause 7.2. |
| Envelope-Start-Time | M | Used as defined in clause 7.2. |
| Envelope-End-Time | OC | Used as defined in clause 7.2. |
| CC-Total-Octets | OC | Used as defined in clause 7.2. |
| CC-Input-Octets | OC | Used as defined in clause 7.2. |
| CC-Output-Octets | OC | Used as defined in clause 7.2. |
| CC-Service-Specific-Units | OC | Used as defined in clause 7.2 |
| Envelop-Reporting | OC | Used as defined in clause 7.2. |
| Time-Quota-Mechanism | OC | Used as defined in clause 7.2. |
| Service-Specific-Info | OC | Used as defined in clause 7.2. |
| Service-Specific-Type | OC | Used as defined in clause 7.2. |
| Service-Specific-Data | OC | Used as defined in clause 7.2. |
| QoS-Information | OC | This field contains authorized QoS applicable for service data flow, which initially triggers the activation of this MSCC instance. Included in first quota request to rating group if service data flow specific QoS control is in use, see TS 29.212 [215] for more information. For IP-CAN bearer specific Rating Group/Service Identifier this field is not included. For TDF, this field contains bandwidth limitation applicable for application traffic and only the Max-Requested-Bandwidth-UL and the Max-Requested-Bandwidth-DL are used. |
| QoS-Class-Identifier | M | See TS 29.212 [215] for more information. |
| Max-Requested-Bandwidth-UL | OC | See TS 29.212 [215] for more information. |
| Max-Requested-Bandwidth-DL | OC | See TS 29.212 [215] for more information. |
| Guaranteed-Bitrate-UL | OC | See TS 29.212 [215] for more information. |
| Guaranteed-Bitrate-DL | OC | See TS 29.212 [215] for more information. |
| Bearer-Identifier | OC | See TS 29.212 [215] for more information. |
| Allocation-Retention-Priority | OC | See TS 29.212 [215] for more information. |
| Priority-Level | OC | See TS 29.212 [215] for more information. |
| Pre-emption-Capability | OC | See TS 29.212 [215] for more information. |
| Pre-emption-Vulnerability | OC | See TS 29.212 [215] for more information. |
| APN-Aggregate-Max-Bitrate-UL | OC | See TS 29.212 [215] for more information. |
| APN-Aggregate-Max-Bitrate-DL | OC | See TS 29.212 [215] for more information. |
| Announcement-Information | OC | Not used in CCR. |
| 3GPP-RAT-Type | OC | This field contains the RAT Type of the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier. If traffic from multiple access types is included in this report, only one field is included. This field is only applicable when NBIFOM is accepted for the IP-CAN session. See TS 29.061 [207] for more information. |
| Related-Trigger | OC | This field contains the trigger types for a related trigger for another access in a multi-access PDN connection. This field is only included when the Trigger AVP contains a Trigger-Type AVP with value of "indirect change condition" which is applicable when charging per IP-CAN session for a multi-access PDN connection. |
| AVP | - | Not used in CCR . |
| Service-Parameter-Info | - | Not used in 3GPP. |
| Service-Parameter-Type | - | Not used in 3GPP. |
| Service-Parameter-Value | - | Not used in 3GPP. |
| CC-Correlation-Id | OC | This field contains information to correlate CCRs generated for different components of the service, e.g., transport and service level. |
| User-Equipment-Info | OC | This field contains the identification of the identity and terminal capability the subscriber is using for the connection to mobile network if available. |
| User-Equipment-Info-Type | M | This field determines the type of the identifier. The used value is 0 for the international mobile equipment identifier and software version according to TS 23.003[224]. |
| User-Equipment-Info-Value | M | This field contains the user IMEI. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | OC | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| Service-Information | OM | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |
| AVP | OC | This field contains extended Information. |

### 6.4.3	Credit-Control-Answer message

The CCA messages, indicated by the Command-Code field set to 272 is sent by the OCF to the CTF in order to reply to the CCR.

The CCA message format is defined according to RFC 4006 [402] as follows:

<CCA> ::=  < Diameter Header: 272, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Auth-Application-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ User-Name ]

[ CC-Session-Failover ]

[ CC-Sub-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Origin-State-Id ]

[ Event-Timestamp ]

[ Granted-Service-Unit ]

*[ Multiple-Services-Credit-Control ]

[ Cost-Information]

[ Low-Balance-Indication ]

[ Remaining-Balance ]

[ Final-Unit-Indication ]

[ Check-Balance-Result ]

[ Credit-Control-Failure-Handling ]

[ Direct-Debiting-Failure-Handling ]

[ Validity-Time]

*[ Redirect-Host]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ Route-Record ]

[ Failed-AVP ]

[ Service-Information ]

*[ AVP ]

Table 6.4.3.1 illustrates the basic structure of a 3GPP Diameter CCA message as used for online charging. This message is always used by the OCF as specified below, independent of the receiving CTF and the CCR record type that is being replied to.

- Table 6.4.3.1: 3GPP CCA message content

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Result-Code | M | This field contains the result of the specific query. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Auth-Application-Id | M | The field corresponds to the application ID of the Diameter Credit-Control Application and is defined with the value 4. |
| CC-Request-Type | M | This field defines the transfer type: initial, update, terminate for session based charging and event for event based charging. |
| CC-Request-Number | M | This field contains the sequence number of the transferred messages. |
| User-Name | - | Not used in 3GPP. |
| CC-Session Failover | OC | This field contains an indication to the CTF whether or not a failover handling is to be used when necessary. |
| CC-Sub-session-Id | - | Not used in 3GPP. |
| Acct-Multi-Session-Id | - | Not used in 3GPP. |
| Origin-State-Id | - | Not used in 3GPP. |
| Event-Timestamp | - | Not used in 3GPP. |
| Granted-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Tariff-Time-Change | - | Not used in 3GPP. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Multiple-Services-Credit-Control | OC | This field contains all parameters for the CTF quota management and defines the quotas to allow traffic to flow. |
| Granted-Service-Unit | OC | This field contains the amount of granted service units for a particular category. |
| Tariff-Time-Change | OC | This field contains the switch time when the tariff will be changed. |
| CC-Time | OC | This field contains the amount of granted time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the amount for sent and received octets. |
| CC-Input-Octets | OC | This field contains the amount for received octets. |
| CC-Output-Octets | OC | This field contains the amount for sent octets. |
| CC-Service-Specific-Units | OC | This field contains the amount for service specific units, e.g. number of events. |
| AVP | - | Not used in 3GPP . |
| Requested-Service-Unit | - | Not used in CCA. |
| CC-Time | - | Not used in CCA. |
| CC-Money | - | Not used in 3GPP . |
| Unit-Value | - | Not used in 3GPP . |
| Value-Digits | - | Not used in 3GPP . |
| Exponent | - | Not used in 3GPP . |
| Currency-Code | - | Not used in 3GPP . |
| CC-Total-Octets | - | Not used in CCA. |
| CC-Input-Octets | - | Not used in CCA. |
| CC-Output-Octets | - | Not used in CCA. |
| CC-Service-Specific-Units | - | Not used in CCA. |
| AVP | - | Not used in 3GPP. |
| Used-Service-Unit | - | Not used in CCA. |
| Reporting-Reason | - | Not used in CCA. |
| Tariff-Change-Usage | - | Not used in CCA. |
| CC-Time | - | Not used in CCA. |
| CC-Money | - | Not used in 3GPP . |
| Unit-Value | - | Not used in 3GPP . |
| Value-Digits | - | Not used in 3GPP . |
| Exponent | - | Not used in 3GPP . |
| Currency-Code | - | Not used in 3GPP . |
| CC-Total-Octets | - | Not used in CCA. |
| CC-Input-Octets | - | Not used in CCA. |
| CC-Output-Octets | - | Not used in CCA. |
| CC-Service-Specific-Units | - | Not used in CCA. |
| Event-Charging-TimeStamp | - | Not used in CCA. |
| AVP | - | Not used in 3GPP. |
| Tariff-Change-Usage | - | Not used in 3GPP. |
| Service-Identifier | OC | This field contains identity of the used service. This ID with the Service-Context-ID together forms an unique identification of the service. |
| Rating-Group | OC | This field contains the identifier of a rating group. |
| G-S-U-Pool-Reference | OC | Only used in ECUR and SCUR. |
| G-S-U-Pool-Identifier | M | Used as defined in DCCA [402]. |
| CC-Unit-Type | M | Used as defined in DCCA [402]. |
| Unit-Value | M | Used as defined in DCCA [402]. |
| Value-Digits | M | Used as defined in DCCA [402]. |
| Exponent | OC | Used as defined in DCCA [402]. |
| Validity-Time | OC | This field defines the time in order to limit the validity of the granted quota for a given category instance. |
| Result-Code | OC | This field contains the result of the query. |
| Final-Unit-Indication | OC | This field indicates that the Granted-Service-Unit AVP containing the final units for the service. |
| Final-Unit-Action | M | Used as defined in DCCA [402]. |
| Restriction-Filter-Rule | OC | Used as defined in DCCA [402]. |
| Filter-Id | OC | Used as defined in DCCA [402]. |
| Redirect-Server | OC | Used as defined in DCCA [402]. |
| Redirect-Address-Type | M | Used as defined in DCCA [402]. |
| Redirect-Server-Address | M | Used as defined in DCCA [402]. |
| Time-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Volume-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Unit-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Quota-Holding-Time | OC | Used as defined in clause 7.2. |
| Quota-Consumption-Time | OC | Used as defined in clause 7.2. |
| Reporting-Reason | - | Not used in CCA. |
| Trigger | Oc | Used as defined in clause 7.2. |
| Trigger-Type | OC | Used as defined in clause 7.2. |
| PS-Furnish-Charging-Information | OC | Used as defined in clause 7.2. |
| Refund-Information | OC | Used as defined in clause 7.2. |
| AF-Correlation-Information | - | Not used in CCA. |
| Envelope-Reporting | OC | Used as defined in clause 7.2. |
| Time-Quota-Mechanism | OC | Used as defined in clause 7.2. |
| Time-Quota-Type | M | Used as defined in clause 7.2. |
| Base-Time-Interval | M | Used as defined in clause 7.2. |
| AF-Correlation-Information | - | Not used in CCA. |
| Service-Specific-Info | - | Not used in CCA. |
| QoS-Information | - | Not used in CCA. |
| Announcement-Information | OC | This field contains the Announcement service parameters. Each instance specifies a single announcement to be played to the specified user. |
| Announcement-Identifier | M | This field contains a code identifying an announcement to be played. |
| Variable-Part | OC | This field contains the parameters necessary to define playback of a variable within the announcement. |
| Variable-Part-Order | OC | This field identifies the order of the variable to be played back. |
| Variable-Part-Type | M | This field identifies the type of the variable to be played back. |
| Variable-Part-Value | M | This field identifies the value of the variable to be played back. |
| Time-Indicator | OC | This field instructs the announcement to be connected at the specified time before granted quota is exhausted. |
| Quota-Indicator | OC | This field indicates whether granted quota is to be used during announcement setup and playback. |
| Announcement-Order | OC | This field contains the order in which announcements should be connected for playback when there are multiple announcements provided in a single message with the same timing indicator. |
| Play-Alternative | OC | This field indicates the call party toward whom an announcement is directed. |
| Privacy-Indicator | OC | This field indicates whether the announcement is considered private and is only to be played to the requested party. |
| Language | OC | This field contains a code indicating the language of the announcement that should be played. |
| 3GPP-RAT-Type | - | Not used in CCA. |
| Related-Trigger | - | Not used in CCA |
| AVP | - | Not used in 3GPP. |
| Cost-Information | OC | Used as defined in DCCA [402]. |
| Unit-Value | M | Used as defined in DCCA [402]. |
| Value-Digits | M | Used as defined in DCCA [402]. |
| Exponent | OC | Used as defined in DCCA [402]. |
| Currency-Code | M | Used as defined in DCCA [402]. |
| Cost-Unit | OC | Used as defined in DCCA [402]. |
| Low-Balance-Indication | Oc | This field indicates whether the subscriber account balance went below a designated threshold set by his account. |
| Remaining-Balance | OC | This field contains the remaining balance of the subscriber. |
| Unit-Value | M | Used as defined in DCCA [402]. |
| Value-Digits | M | Used as defined in DCCA [402]. |
| Exponent | OC | Used as defined in DCCA [402]. |
| Currency-Code | M | Used as defined in DCCA [402]. |
| Final-Unit-Indication | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Final-Unit-Action | - | Not used in 3GPP. |
| Restriction-Filter-Rule | - | Not used in 3GPP. |
| Filter-Id | - | Not used in 3GPP. |
| Redirect-Server | - | Not used in 3GPP. |
| Redirect-Address-Type | - | Not used in 3GPP. |
| Redirect-Server-Address | - | Not used in 3GPP. |
| Check-Balance-Result | - | Not used in 3GPP. |
| Credit-Control-Failure-Handling | OC | Used as defined in DCCA [402]. |
| Direct-Debiting-Failure-Handling | OC | Used as defined in DCCA [402]. |
| Validity-Time | - | Not used in 3GPP. |
| Redirect-Host | OC | Used by redirect agent function as specified in [401]. Contains the host the request should be forwarded to. |
| Redirect-Host-Usage | OC | Used by redirect agent function as specified in [401]. Dictates how the routing entry resulting from the Redirect-Host is to be used. |
| Redirect-Max-Cache-Time | OC | Used by redirect agent function as specified in [401]. Contains the maximum number of seconds the peer and route table entries, created as a result of the Redirect-Host, will be cached. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | OC | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| Failed-AVP | OC | This field contains the AVP that could not be processed sucessfully. This field can occur only once. |
| Service-Information | OC | This parameter holds the individual service specific parameters as defined in the corresponding 'middle tier' TS. |
| AVP | OC | This field contains extended Information. |

### 6.4.4	Re-Auth-Request message

The DCCA RAR message format is defined according to the Diameter Base Protocol in RFC 6733 [401] and DCCA specification in RFC 4006 [402]:

<RAR> ::= < Diameter Header: 258, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Destination-Host }

{ Auth-Application-Id }

{ Re-Auth-Request-Type }

[ User-Name ]

[ Origin-State-Id ]

*[ Proxy-Info ]

*[ Route-Record ]

[ CC-Sub-Session-Id ]

[ G-S-U-Pool-Identifier ]

[ Service-Identifier ]

[ Rating-Group ]

*[ AVP ]

Table 6.4.4.1 illustrates the basic structure of a Diameter Credit-Control Re-Auth-Request message as used for online charging.

- Table 6.4.4.1: RAR message contents for online charging

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Destination-Realm | M | This field contains the realm of the operator domain. The realm will be addressed with the domain address of the corresponding public URI. |
| Destination-Host | M | This field contains the destination peer address of the OCS identity. |
| Auth-Application-Id | M | The field corresponds to the application ID of the Diameter Credit-Control Application and is defined with the value 4. |
| Re-Auth-Request-Type | M | This field is used to inform the CTF of the action expected upon expiration of the Authorization-Lifetime |
| User-Name | OC | This field contains the username. |
| Origin-State-Id | OC | This field contains the state associated to the CTF. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | OC | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| CC-Sub-Session-Id | - | Not used in 3GPP. |
| G-S-U-Pool-Identifier | OC | This field contains an identifier to indicate the credit pool. |
| Service-Identifier | OC | This field contains identity of the used service. This ID with the Service-Context-ID together forms an unique identification of the service. |
| Rating-Group | OC | This field contains the identifier of a rating group. |
| AVP | OC | This field contains extended Information. |

### 6.4.5	Re-Auth-Answer message

The DCCA RAA message format is defined according to the Diameter Base Protocol in RFC 6733 [401] and DCCA specification in RFC 4006 [402]:

<RAA> ::= < Diameter Header: 258, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ Origin-State-Id ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ Failed-AVP ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Host-Cache-Time ]

*[ Proxy-Info ]

[ CC-Sub-Session-Id ]

[ G-S-U-Pool-Identifier ]

[ Service-Identifier ]

[ Rating-Group ]

*[ AVP ]

Table 6.4.5.1 illustrates the basic structure of a Diameter Credit-Control Re-Auth-Answer message as used for online charging.

- Table 6.4.5.1: RAA message contents for online charging

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Result-Code | M | This field contains the result of the specific query. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| User-Name | OC | This field contains the username. |
| Origin-State-Id | OC | This field contains the state associated to the source point of the operation. |
| Error-Message | OC | This field contains a human readable error message. |
| Error-Reporting-Host | OC | This field contains the identity of the Diameter host that sent the Result-Code AVP to a value other than 2001 (Success) if the host setting the Result-Code is different from the one encoded in the Origin-Host AVP. |
| Failed-AVP | OC | This field contains the AVP that could not be processed sucessfully. This field can occur only once. |
| Redirect-Host | OC | Used by redirect agent function as specified in [401]. Contains the host the request should be forwarded to. |
| Redirect-Host-Usage | OC | Used by redirect agent function as specified in [401]. Dictates how the routing entry resulting from the Redirect-Host is to be used. |
| Redirect-Max-Cache-Time | OC | Used by redirect agent function as specified in [401]. Contains the maximum number of seconds the peer and route table entries, created as a result of the Redirect-Host, will be cached. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| CC-Sub-Session-Id | - | Not used in 3GPP. |
| G-S-U-Pool-Identifier | - | Not used in 3GPP. |
| Service-Identifier | - | Not used in 3GPP. |
| Rating-Group | - | Not used in 3GPP. |
| AVP | OC | This field contains extended Information. |

### 6.4.6	Capabilities-Exchange-Request message

The Capabilities-Exchange-Request message structure shall be as specified in RFC 6733 [401] clause 5.3.1.

### 6.4.7	Capabilities-Exchange-Answer message

The Capabilities-Exchange-Answer message structure shall be as specified in RFC 6733 [401] clause 5.3.2.

### 6.4.8	Device-Watchdog-Request message

The Device-Watchdog-Request message structure shall be as specified in RFC 6733 [401] clause 5.5.1.

### 6.4.9	Device-Watchdog-Answer message

The Device-Watchdog-Answer message structure shall be as specified in RFC 6733 [401] clause 5.5.2.

### 6.4.10	Disconnect-Peer-Request message

The Disconnect-Peer-Request message structure shall be as specified in RFC 6733 [401] clause 5.4.1.

### 6.4.11	Disconnect-Peer-Answer message

The Disconnect-Peer-Answer message structure shall be as specified in RFC 6733 [401] clause 5.4.2.

### 6.4.12	Abort-Session-Request message

The Abort-Session-Request message structure shall be as specified in RFC 6733 [401] clause 8.5.1.

### 6.4.13	Abort-Session -Answer message

The Abort-Session-Answer message structure shall be as specified in RFC 6733 [401] clause 8.5.2.

## 6.5	Other procedural description of the 3GPP charging applications

### 6.5.1	Re-Authorization

#### 6.5.1.1	Idle timeout

The server may specify an idle timeout associated with a granted quota using the Quota-Holding-Time AVP. 
If no traffic associated with the quota is observed for this time, the client shall understand that the traffic has stopped and the quota is returned to the server. The client shall start the quota holding timer when quota consumption ceases.  This is always when traffic ceases, i.e. the timer is re-started at the end of each packet. It applies equally to the granted time quota and to the granted volume quota. The timer is stopped on sending a CCR and re-initialised on receiving a CCA with the previous used value or a new value of Quota-Holding-Time AVP if received.

Alternatively, if this AVP is not present, a locally configurable default value in the client shall be used. 
A Quota-Holding-Time AVP value of zero indicates that this mechanism shall not be used.

#### 6.5.1.2	Change of charging conditions

There are a number of mid-session service events (re-authorization triggers), which could affect the rating of the current service usage, e.g. end user QoS changes or location updates. When allocating resources, the server may instruct the Credit-Control client to re-authorize the quota upon a number of different session related triggers that can affect the rating conditions. The server instruct the Network Element to monitor for such events by using the Trigger AVP containing one or more Trigger-Type AVPs in the CCA command. These events are in addition to the static triggers defined in the service specific document (middle tier TS).

Once the OCS has armed one or more triggers using the Trigger AVP at the Network Element, these triggers shall remain in effect until another Trigger AVP is received for the same Rating Group, where the Network Element shall arm all triggers present in the Trigger AVP and reset all other triggers. The presence of the Trigger AVP without any Trigger-Type AVPs in a CCA allows OCS to disable all the triggers that were armed in a previous Trigger AVP.

NOTE:	This removes the need for the OCS to send trigger information in every CCA message when they have not changed.

When one of the armed triggers happen, a credit re-authorization shall be sent to the server including information related to the service event even if all the granted service units have not been used. The quota is also being reported.

For example, if the Trigger AVP is used, then the client shall only re-authorize the quota for the service usage associated with  events which were included in the last received Trigger AVP.

If the server does not control the events for re-authorization using the Trigger AVP, the Network Element shall only monitor for default events defined in the relevant service specific document (middle tier TS).

#### 6.5.1.3	Reporting quota usage

The Credit-Control client shall report the quota usage under a number of circumstances. When this happens, the reason for the quota being reported is notified to the server through the use of the Reporting-Reason AVP in the CCR. The reason for reporting credit usage can occur directly in the Multiple-Services-Credit-Control AVP, or in the Used-Service-Units AVP, depending on whether it applies for all quota types or a particular quota type respectively. It shall not be used at command level.  It shall always and shall only be sent when usage is being reported.

When the reason is RATING_CONDITION_CHANGE, the Trigger AVP shall also be included to indicate the specific armed trigger events which caused the reporting and re-authorization request.

#### 6.5.1.4	Quota consumption

The consumption of quota is captured using mechanisms described in clause 6.5.1.3.

Volume quota is considered used or consumed in the normal way, corresponding to actual traffic.

The consumption of time quota may be controlled by Quota-Consumption-Time as described in clause 6.5.4, or by extended mechanisms as described in clause 6.5.7.

### 6.5.2	Threshold based Re-Authorization triggers

The server may optionally include as part of the Multiple-Services-Credit-Control AVP, when it is providing a quota, an indication to the client of the remaining quota threshold that shall trigger a quota re-authorization. 
The Time-Quota-Threshold AVP indicates the threshold in seconds when the granted quota is time, and the Volume-Quota-Threshold AVP indicates the threshold in octets when the granted quota is volume. 
The Unit-Quota-Threshold AVP indicates the threshold in service specific units, that are defined in the service specific documents, when the granted quota is service specific.

If the threshold triggers were included along with the quota granted, the Credit-Control client, then, shall seek re-authorization from the server for the quota when the quota contents fall below the supplied threshold. The client shall allow service to continue whilst the re-authorization is progress, until the original quota had been consumed.

### 6.5.3	Termination action

The termination action is sent over the Ro reference point. Two different approaches are specified:

-	The Final-Unit-Indication AVP with Final-Unit-Action TERMINATE does not include any other information. When the user has consumed the final granted units or zero quota has been granted by the OCS, the Network Element shall terminate the service. This is the default handling applicable whenever the client receives an unsupported Final-Unit-Action value. The Network Element shall send CCR message with CC-Request-Type AVP  set to the value UPDATE_REQUEST and report the Used-Service-Unit AVP for the service that has terminated, as defined in RFC 4006 [402].

-	Another termination action consists in re-directing packets corresponding to a terminated service (consumption of the final granted units or zero quota has been granted by the OCS) to an application server. This allows the client to redirect user originated requests to a top-up server so that network access can be re-instated. This functionality is achieved with the server returning a "REDIRECT" and redirect-to URL in the Final-Units-Action AVP of the Multiple-Services-Credit-Control AVP. Upon receiving this result code, the Network Element shall apply the redirection. The URL should be categorized so that the End-User's ability to reach it is guaranteed.

When zero quota has been granted by the OCS, the termination action shall be enforced at the reception of the CCA message.

### 6.5.4	Quota consumption time

The server may optionally indicate to the client that the quota consumption shall be stopped after a period equal to the Quota Consumption Time in which no packets are received or at session termination, whichever is sooner. This is indicated by including the Quota-Consumption-Time AVP in the CCA. The idle period equal to the Quota Consumption Time is included in the reported usage. The quota is consumed normally during gaps in traffic of duration less than or equal to the Quota-Consumption-Time. Quota consumption resumes on receipt of a further packet belonging to the service data flow.

If packets are allowed to flow during a CCR/CCA[Update] exchange, and the Quota-Consumption-Time AVP value in the provided quota is the same as in the previously provided quota, then the Quota-Consumption-Time runs normally through this procedure. For example, if 5 seconds of a 10 second QCT timer have passed when a CCR[Update] is triggered, and the CCA[Update] returns 2 seconds later, then the QCT timer will expire 3 seconds after the receipt of the CCA and the remaining unaccounted 5 seconds of usage will be recorded against the new quota even though no packets were transmitted with the new quota.

In the case of a new quota with the Quota-Consumption-Time AVP, or when packets are blocked during the CCR/CCA [Update] procedure then the Quota-Consumption-Time stops running (if it was running) and quota consumption begins again when the next service data flow packet matching the Charging Rule is received.

If a Quota-Consumption-Time AVP value of zero is provided, or if no Quota-Consumption-Time AVP is present in the CCA, the quota is consumed continuously from the point at which it is granted.

### 6.5.5	Service termination

The OCF may determine that a service requires termination. The OCF may perform this termination synchronously if it has a CCR pending processing by returning CCA with Result-Code AVP with value DIAMETER-AUTHORIZATION-REJECTED. If the OCF does not have a pending request (asynchronous), the OCF may trigger an ASR to terminate the Diameter session related to the service. On reception of an ASR, the CTF shall close the associated Credit-Control session by sending a CCR [Terminate]. The behaviour of the CTF, in relation to the user session, on reception of an ASR is detailed in the middle-tier TS. As an alternative to the ASR, the OCF may trigger a RAR to which the CTF behaves as described in RFC 4006 [402] and the OCF shall return a CCA with Result-Code AVP with value DIAMETER-AUTHORIZATION-REJECTED for the resulting CCR.

### 6.5.6	Envelope reporting

#### 6.5.6.1	Envelope reporting in Online Charging

The OCF may determine the need for additional detailed reports identifying start time and end times of specific activity in addition to the standard quota management provided in RFC 4006 [402]. The OCF controls this by sending a CCA[INITIAL] with Envelope-Reporting AVP. The OCF may request reports for just time, time and volume, time and number of events, or time and volume and number of events, through appropriate values in Envelope-Reporting AVP.

The time envelope of the activity to be reported is determined by the mechanism indicated by the OCF in this CCA[INITIAL], to control the time quota consumption in the CTF.

The selected mechanism is used by the CTF to generate the envelopes, reporting the start and end of each time envelope, and the data volume transferred and / or the number of events that occurred for the duration of the envelope

NOTE:	Envelope reporting is independent of quota management (i.e. there is no interaction).

#### 6.5.6.2	Envelope reporting in Offline Charging

When a particular time quota consumption mechanism is selected by the OCF, the need for reporting detailed time envelopes in offline charging, may be statically configured in the CTF or dynamically controlled by the OCF using Offline-Charging AVP.

The selected time quota consumption mechanism is used by the CTF to generate the envelopes, reporting the start and end of each time envelope, and the data volume transferred and / or the number of events that occurred for the duration of the envelope.

#### 6.5.6.3	Envelope reporting - Quota consumption time

In case the time quota consumption mechanism selected by the OCF is per clause 6.5.4, and the OCF indicated the need for envelope reporting,.the CTF, on receiving the command, monitors the traffic for a period of time controlled by the Quota-Consumption-Time AVP and reports each period as a single envelope for each Quota-Consumption-Time expiry where there was traffic, in Envelope AVP.

#### 6.5.6.4	Envelope reporting - Combinational quota

In case the time quota consumption mechanism selected by the OCF is per clause 6.5.7, and the OCF indicated the need for envelope reporting, the CTF, on receiving the command, monitors the traffic based on information included in Time-Quota-Mechanism AVP, and reports each corresponding envelope in the Envelope AVP in the CCR.

In case of Continuous Time Period (CTP):

-	the first envelope starts on the first traffic, and subsequent envelopes start on the first traffic following the closure of the previous envelope.

-	each envelope ends at expiry of the first base time interval which contains no traffic.

The envelope for CTP includes the last base time interval, i.e. the one which contained no traffic. The end of an envelope can only be determined "retrospectively".

In case of Discrete Time Period (DTP), an envelope corresponds to exactly one base time interval.

### 6.5.7	Combinational quota

The Quota-Consumption-Time mechanism, described in clause 6.5.4, may be extended (and replaced) when granting time based quota to provide potentially more efficient use of the online charging interface, i.e. reduced traffic and the algorithms in the OCF are potentially simpler. The alternative handling mechanisms that are defined in this clause are:

1.	Continuous Time Period (CTP)

2.	Discrete Time Period (DTP)

The mechanisms selected by the OCF is indicated to the CTF with the Time-Quota-Mechanism AVP in the CCA.

The base time interval, specified by the Base-Time-Interval AVP, is a basic unit for consuming quota. Quota is deemed to be consumed at the start of each base time interval. The CTF shall allow traffic to pass for the duration of the base time interval.

For DTP, the base time interval defines the length of the discrete time period. Quota consumption resumes only on the first traffic following the expiry of the DTP.

For CTP, quota consumption continues during consecutive base time intervals in which traffic has occurred up to and including the first base time interval which contains no traffic. Quota consumption shall be stopped after this first base time interval expiry which contained no traffic. Quota consumption resumes only on the first subsequent traffic.

If the CTF receives a Multiple-Services-Credit-Control AVP with both the Quota-Consumption-Time AVP and Time-Quota-Mechanism AVP, then the Time-Quota-Mechanism AVP takes precedence and the CTF shall behave accordingly.

### 6.5.8	Online control of offline charging information

The Offline-Charging AVP is used on the Ro interface by the OCS to control the CTF in relation to the mechanism by which the CTF generates offline charging information, e.g. for flow based charging controls the formation of service data containers. The information contained, within the Offline-Charging AVP, takes precedence over the default configuration at the CTF. If the Offline-Charging AVP is not sent in the CCA, the OCS does not control the offline charging mechanisms and therefore the default configuration at the CTF is employed.

Controls over time usage, defined in clause 6.5.6 and clause 6.5.7, are included.

### 6.5.9	Support of multiple service

The support of multiple services within a single Diameter session in 3GPP is limited to services that are grouped in one of the middle tier TSs specified through the Service-Context-Id AVP.

## 6.6	Bindings of the operation to protocol application

### 6.6.0	General

This clause aims to describe the mapping between the protocol independent messages and parameter with the Diameter messages and AVP utilized on the 3GPP offline and online charging.

### 6.6.1	Bindings of Charging Data Transfer to Accounting

Table 6.6.1.1 describes the bindings of the Charging Data Transfer operation parameter to the DBPA AVP for 3GPP offline charging.

- Table 6.6.1.1: Bindings to Accounting

| Charging Data Transfer element | Diameter Accounting AVP |
| --- | --- |
| Operation Number | Accounting-Record-Number |
| Operation Type | Accounting-Record-Type |
| Operation Identifier | Acct-Application-Id |
| Operation Interval | Acct-Interim-Interval |
| Destination Domain | Destination-Realm |
| Error Reporting Host | Error-Reporting-Host |
| Origination Timestamp | Event-Timestamp |
| Originator Host | Origin-Host |
| Originator Domain | Origin-Realm |
| Origination State | Origin-State-Id |
| Proxy Information | Proxy-Info |
| Operation Result | Result-Code |
| Route Information | Route-Record |
| Service Information | Service-Information |
| Session Identifier | Session-Id |
| Operation Token | Service-Context-Id |
| User Name | User-Name |

### 6.6.2	Bindings of Debit / Reserve Units to Credit-Control

Table 6.6.2.1 describes the bindings of the Debit / Reserve Units operation parameter to the DCCA AVP for 3GPP 0nline charging.

- Table 6.6.2.1: Bindings to Credit-Control

| Debit / Reserve Units element | DCCA AVP |
| --- | --- |
| AoC Type | AoC-Request-Type |
| Cost Information | Cost-Information |
| Destination Domain | Destination-Realm |
| Destination Host | Destination-Host |
| Failed parameter | Failed-AVP |
| Low Balance Indication | Low-Balance-Indication |
| Multiple Operation | Multiple-Services-Indicator |
| Multiple Unit Operation | Multiple-Services-Credit-Control |
| Operation Correlation | CC-Correlation-Id |
| Operation Failover | CC-Session-Failover |
| Operation Failure Action | Credit-Control-Failure-Handling |
| Operation Identifier | Auth-Application-Id |
| Operation Number | CC-Request-Number |
| Operation Result | Result-Code |
| Operation Token | Service-Context-Id |
| Operation Type | CC-Request-Type |
| Origination State | Origin-State-Id |
| Origination Timestamp | Event-Timestamp |
| Originator Domain | Origin-Realm |
| Originator Host | Origin-Host |
| Proxy Information | Proxy-Info |
| Redirection Cache Time | Redirect-Max-Cache-Time |
| Redirection Host | Redirect-Host |
| Redirection Host Usage | Redirect-Host-Usage |
| Remaining Balance | Remaining-Balance |
| Requested Action | Requested-Action |
| Route Information | Route-Record |
| Service Information | Service-Information |
| Session Identifier | Session-Id |
| Subscriber Equipment Number | User-Equipment-Info |
| Subscriber Identifier | Subscription-Id |
| Termination Cause | Termination-Cause |
| User Name | User-Name |

## 6.7	Securing Diameter messages

For secure transport of Diameter messages used for offline and online charging application, see 3GPP TS 33.210 [246].

# 7	Summary of used Attribute Value Pairs

## 7.1	Diameter AVPs

### 7.1.0	General

The use of the Attribute Value Pairs (AVPs) that are defined in the Diameter Protocol is specified in clause 6.2 for offline charging and in clause 6.4 for online charging. The information is summarized in the table 7.1.0.1 in alphabetical order. Detailed specification of some of these AVPs is available after the table and for the others can be found from RFC 6733 [401], RFC 4006 [402] and RFC 4005 [407].

Those Diameter AVPs that are used are marked "M", "OM"or "Oc" in the following table. 
This implies that their content can be used by the CDF for offline and by the OCF for online charging purposes. 
Those Diameter AVPs that are not used are marked "-" in table 7.1.0.1.

- Table 7.1.0.1: Use Of IETF Diameter AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not |
| Accounting-Input-Octets | 363 | OC | - | - | - | Unsigned64 | M | - | - | V |
| Accounting-Output-Octets | 364 | OC | - | - | - | Unsigned64 | M | - | - | V |
| Accounting-Realtime-Required | 483 | - | - | - | - | Enumerated | - | - | - | - |
| Accounting-Record-Number | 485 | M | M | - | - | Unsigned32 | M | - | - | V |
| Accounting-Record-Type | 480 | M | M | - | - | Enumerated | M | - | - | V |
| Accounting-Sub-Session-Id | 287 | - | - | - | - | Unsigned64 | - | - | - | - |
| Acct-Application-Id | 259 | OC | OC | - | - | Unsigned32 | M | - | - | V |
| Acct-Interim-Interval | 85 | OC | OC | - | - | Unsigned32 | M | - | - | V |
| Acct-Multi-Session-Id | 50 | - | - | - | - | Unsigned32 | - | - | - | - |
| Acct-Session-Id | 44 | - | - | - | - | OctetString | - | - | - | - |
| Auth-Application-Id | 258 | - | - | M | M | Unsigned32 | M | - | - | V |
| AVP | * | - | - | - | - | Grouped | - | - | - | - |
| Called-Station-Id | 30 | OC | - | OC | - | UTF8String | M | - | - | V |
| CC-Correlation-Id | 411 | - | - | OC | - | OctetString | - | M | - | V |
| CC-Input-Octets | 412 | - | - | OC | OC | Unsigned64 | M | - | - | V |
| CC-Money | 413 | - | - | - | - | Grouped | - | - | - | - |
| CC-Output-Octets | 414 | - | - | OC | OC | Unsigned64 | M | - | - | V |
| CC-Request-Number | 415 | - | - | M | M | Unsigned32 | M | - | - | V |
| CC-Request-Type | 416 | - | - | M | M | Enumerated | M | - | - | V |
| CC-Service-Specific-Units | 417 | - | - | OC | OC | Unsigned64 | M | - | - | V |
| CC-Session-Failover | 418 | - | - | - | OC | Enumerated | M | - | - | V |
| CC-Sub-Session-Id | 419 | - | - | - | - | Unsigned64 | - | - | - | - |
| CC-Time | 420 | - | - | OC | OC | Unsigned32 | M | - | - | V |
| CC-Total-Octets | 421 | - | - | OC | OC | Unsigned64 | M | - | - | V |
| CC-Unit-Type | 454 | - | - | - | M | Enumerated | M | - | - | V |
| Check-Balance-Result | 422 | - | - | - | - | Enumerated | - | - | - | - |
| Cost-Information | 423 | - | - | - | OC | Grouped | M | - | - | V |
| Cost-Unit | 424 | - | - | - | OC | UTF8String | M | - | - | V |
| Credit-Control | 426 | - | - | - | - | Enumerated | - | - | - | - |
| Credit-Control-Failure-Handling | 427 | - | - | - | OC | Enumerated | M | - | - | V |
| Currency-Code | 425 | - | - | - | M | Unsigned32 | M | - | - | V |
| Destination-Host | 293 | OC | - | OC | - | DiamIdent | M | - | - | V |
| Destination-Realm | 283 | M | - | M | - | DiamIdent | M | - | - | V |
| Direct-Debiting-Failure-Handling | 428 | - | - | - | OC | Enumerated | M | - | - | V |
| Error-Message | 281 | - | - | - | - | UTF8String | - | - | - | - |
| Error-Reporting-Host | 294 | - | OC | - | - | DiamIdent | - | - | - | V,M |
| Event-Timestamp | 55 | OC | OC | OC | - | Time | M | - | - | V |
| Exponent | 429 | - | - | - | OC | Integer32 | M | - | - | V |
| Failed-AVP | 279 | - | - | - | OC | Grouped | M | - | - | V |
| Filter-Id | 11 | - | - | - | OC | UTF8String | M | - | - | V |
| Final-Unit-Action | 449 | - | - | - | OC | Enumerated | M | - | - | V |
| Final-Unit-Indication | 430 | - | - | - | OC | Grouped | M | - | - | V |
| Granted-Service-Unit | 431 | - | - | - | OC | Grouped | M | - | - | V |
| G-S-U-Pool-Identifier | 453 | - | - | - | OC | Unsigned32 | M | - | - | V |
| G-S-U-Pool-Reference | 457 | - | - | - | OC | Grouped | M | - | - | V |
| Multiple-Services-Credit-Control | 456 | - | - | OC | OC | Grouped | M | - | - | V |
| Multiple-Services-Indicator | 455 | - | - | OM | - | Enumerated | M | - | - | V |
| Origin-Host | 264 | M | M | M | M | DiamIdent | M | - | - | V |
| Origin-Realm | 296 | M | M | M | M | DiamIdent | M | - | - | V |
| Origin-State-Id | 278 | OC | OC | OC | - | Unsigned32 | M | - | - | V |
| Proxy-Info | 284 | OC | OC | OC | OC | Grouped | M | - | - | V |
| Proxy-Host | 280 | M | M | M | M | DiamIdent | M | - | - | V |
| Proxy-State | 33 | M | M | M | M | OctetString | M | - | - | V |
| Rating-Group | 432 | OC | - | OC | OC | Unsigned32 | M | - | - | V |
| Redirect-Address-Type | 433 | - | - | - | M | Enumerated | M | - | - | V |
| Redirect-Host | 292 | - | - | - | OC | DiamURI | M | - | - | V |
| Redirect-Host-Usage | 261 | - | - | - | OC | Enumerated | M | - | - | V |
| Redirect-Max-Cache-Time | 262 | - | - | - | OC | Unsigned32 | M | - | - | V |
| Redirect-Server | 434 | - | - | - | OC | Grouped | M | - | - | V |
| Redirect-Server-Address | 435 | - | - | - | M | UTF8String | M | - | - | V |
| Requested-Action | 436 | - | - | OC | - | Enumerated | M | - | - | V |
| Requested-Service-Unit | 437 | - | - | OC | - | Grouped | M | - | - | V |
| Restriction-Filter-Rule | 438 | - | - | - | OC | IPFilterRule | M | - | - | V |
| Result-Code | 268 | - | M | - | M | Unsigned32 | M | - | - | V |
| Route-Record | 282 | OC | - | OC | OC | DiamIdent | M | - | - | V |
| Service-Context-Id | 461 | OM | - | M | - | UTF8String | M | - | - | V |
| Service-Identifier | 439 | OC | - | OC | OC | Unsigned32 | M | - | - | V |
| Service-Parameter-Info | 440 | - | - | - | - | Grouped | - | - | - | - |
| Service-Parameter-Type | 441 | - | - | - | - | Unsigned32 | - | - | - | - |
| Service-Parameter-Value | 442 | - | - | - | - | OctetString | - | - | - | - |
| Session-Id | 263 | M | M | M | M | UTF8String | M | - | - | V |
| Subscription-Id | 443 | OC | - | OM | - | Grouped | M | - | - | V |
| Subscription-Id-Data | 444 | M | - | M | - | UTF8String | M | - | - | V |
| Subscription-Id-Type | 450 | M | - | M | - | Enumerated | M | - | - | V |
| Tariff-Change-Usage | 452 | - | - | OC | - | Enumerated | M | - | - | V |
| Tariff-Time-Change | 451 | - |  | - | OC | Time | M | - | - | V |
| Termination-Cause | 295 |  | - | OC | - | Enumerated | M | - | - | V |
| Unit-Value | 445 | - | - | - | M | Grouped | M | - | - | V |
| Used-Service-Unit | 446 | - | - | OC | - | Grouped | M | - | - | V |
| User-Equipment-Info | 458 | OC | - | OC | - | Grouped | - | M | - | V |
| User-Equipment-Info-Type | 459 | OM | - | M | - | Enumerated | - | M | - | V |
| User-Equipment-Info-Value | 460 | OM | - | M | - | OctetString | - | M | - | V |
| User-Name | 1 | OC | OC | OC | - | UTF8String | M | - | - | V |
| Value-Digits | 447 | - | - | - | M | Integer64 | M | - | - | V |
| Validity-Time | 448 | - | - | - | OC | Unsigned32 | M | - | - | V |
| Vendor-Id | 266 | - | - | - | - | Unsigned32 | - | - | - | - |
| Vendor-Specific-Application-Id | 260 | - | - | - | - | Grouped | - | - | - | - |

NOTE:	Result-Code AVP is defined in Diameter Base Protocol in RFC 6733 [401]. However, new values are used in offline and online charging applications. These additional values are defined below.

### 7.1.1	Accounting-Input-Octets AVP

The Accounting-Input-Octets AVP (AVP code 363) contains the number of octets transmitted during the data container recording interval, reflecting the volume counts for uplink traffic for a data flow.

### 7.1.2	Void

### 7.1.3	Accounting-Output-Octets AVP

The Accounting-Output-Octets AVP (AVP code 364) contains the number of octets transmitted during the data container recording interval, reflecting the volume count for downlink traffic for a data flow.

### 7.1.4	Void

### 7.1.5	Acct-Application-Id AVP

The Acct-Application-Id AVP (AVP code 259) shall contain the value of 3 as defined in RFC 6733 [401] according TS 29.230 [206].

### 7.1.6	Auth-Application-Id AVP

The Auth-Application-Id AVP (AVP code 258) shall contain the value of 4 as defined in RFC 4006 [402] according TS 29.230 [206].

### 7.1.7	Called-Station-Id AVP

The Called-Station-Id AVP (AVP code 30) shall contain the Access Point Name (APN) the user is connected to.

### 7.1.8	Event-Timestamp AVP

The Event-Timestamp AVP (AVP code 55) shall contain the time when the chargeable event is received in the CTF.

### 7.1.9	Multiple-Services-Credit-Control AVP

The Multiple-Services-Credit-Control AVP (AVP code 456) is of type grouped as specified in RFC 4006 [402]. It contains additional 3GPP specific charging parameters.

It has the following ABNF grammar:

<Multiple-Services-Credit-Control> ::=	   < AVP Header: 456 >

[ Granted-Service-Unit ]
   [ Requested-Service-Unit ]
* [ Used-Service-Unit ]
   [ Tariff-Change-Usage ]  
* [ Service-Identifier ]  
   [ Rating-Group ]  
* [ G-S-U-Pool-Reference ]
   [ Validity-Time ]
   [ Result-Code ]
   [ Final-Unit-Indication ]
   [ Time-Quota-Threshold ]
   [ Volume-Quota-Threshold ] 
   [ Unit-Quota-Threshold ]
   [ Quota-Holding-Time ]
   [ Quota-Consumption-Time ]
* [ Reporting-Reason ]
   [ Trigger ]
   [ PS-Furnish-Charging-Information ]

[ Refund-Information ]

* [ AF-Correlation-Information]

* [ Envelope ]
   [ Envelope-Reporting ]
   [ Time-Quota-Mechanism ]
* [ Service-Specific-Info ]

[ QoS-Information ]

* [ Announcement-Information ]

[ 3GPP-RAT-Type ]

[ Related-Trigger ]
* [ AVP ]

### 7.1.10	Rating-Group AVP

The Rating-Group AVP (AVP code 432), is defined in RFC 4006 [402]. It contains the charging key (defined in TS 23.203 [218]). Each quota allocated to a Diameter CC session has a unique Rating Group value as specified in RFC 4006 [402].

### 7.1.11	Result-Code AVP

This subclause defines new Result-Code AVP (AVP code 268) values that shall be supported by all Diameter implementations that conform to the present document. The Result-Code AVP operates as described in RFC 6733 [401] and RFC 4006 [402]. The following result code descriptions are examples of the possible uses for the code:

- Transient Failures (4xxx):

- 4010	DIAMETER_END_USER_SERVICE_DENIED

The OCF denies the service request due to service restrictions (e.g. terminate rating group) or limitations related to the end-user, for example the end-user's account could not cover the requested service.

- 4011	DIAMETER_CREDIT_CONTROL_NOT_APPLICABLE

The OCF determines that the service can be granted to the end user but no further Credit-Control
needed for the service (e.g. service is free of charge or is treated for offline charging).

- 4012	DIAMETER_CREDIT_LIMIT_REACHED

The OCF denies the service request since the end- user's account could not cover the requested service. If the CCR contained used-service-units they are deducted, if possible.

- Permanent Failures (5xxx):

5003	DIAMETER_AUTHORIZATION_REJECTED

The OCF denies the service request in order to terminate the service for which credit is requested. For example this error code is used to inform IP CAN bearer has to be terminated in the CCR message or to inform blacklist the rating group in the Multiple-Service-Credit-Control AVP.

- 5030	DIAMETER_USER_UNKNOWN

The specified end user could not be found in the OCF.

- 5031	DIAMETER_RATING_FAILED

This error code is used to inform the CTF that the OCF cannot rate the service request due to insufficient rating input, incorrect AVP combination or due to an AVP or an AVP value that is not recognized or supported in the rating. For Flow Based Charging this error code is used if the Rating group is not recognized. The Failed-AVP AVP MUST be included and contain a copy of the entire AVP(s) that could not be processed successfully or an example of the missing AVP complete with the Vendor-Id if applicable. The value field of the missing AVP should be of correct minimum length and contain zeroes.

### 7.1.12	Service-Context-Id AVP

The Service-Context-Id AVP is defined in RFC 4006 [402]. It is of type UTF8String and contains a unique identifier of the Diameter Credit-Control service specific document that applies to the request. 
This is an identifier allocated by the service provider/operator, by the service element manufacturer or by a standardization body and MUST uniquely identify a given Diameter Credit-Control service specific document. For offline charging, this identifies the service specific document ('middle tier' TS) on which associated CDRs should based. The format of the Service-Context-Id is:

"extensions".MNC.MCC."Release"."service-context" "@" "domain"

The 3GPP specific values for "service-context" "@" "domain" are:

32251@3gpp.org for PS charging

32253@3gpp.org for CPDT charging

32260@3gpp.org for IMS charging

32270@3gpp.org for MMS service charging

32271@3gpp.org for LCS service charging

32272@3gpp.org for PoC service charging

32273@3gpp.org for MBMS service charging

32274@3gpp.org for SMS service charging

32275@3gpp.org for MMTel service charging

32276@3gpp.org for VCS charging

32277@3gpp.org for ProSe charging

32278@3gpp.org for Monitoring Event charging

32280@3gpp.org for AoC Service Information

32296@3gpp.org for ABMF Rc information

The "Release" indicates the 3GPP Release the service specific document is based upon e.g. 12 for Release 12.

As a minimum, "Release"."service-context" "@" "domain" shall be used. If the minimum is used all operator provisionable parameters (Oc and Om) are optional.

The MNC.MCC identifies the operator implementing the service specific document, which is used to determine the specific requirements for the operator configurable parameters.

The "extensions" is operator specific information to any extensions in a service specific document.

### 7.1.13	Service-Identifier AVP

The Service-Identifier AVP (AVP code 439), is defined in RFC 4006 [402]. For further details, please refer the middle-tier specification.

### 7.1.14	Used-Service-Unit AVP

The Used-Service-Unit AVP (AVP code 446) is of type grouped as specified in RFC 4006 [402]. It contains additional 3GPP specific charging parameters.

It has the following ABNF grammar:

<Used-Service-Unit> ::= 	< AVP Header: 446 >

[ Reporting-Reason ]

[ Tariff-Change-Usage ]

[ CC-Time ]

[ CC-Money ]

[ CC-Total-Octets ]

[ CC-Input-Octets ]

[ CC-Output-Octets ]

[ CC-Service-Specific-Units ]

*[ Event-Charging-TimeStamp ]

*[ AVP ]

### 7.1.15	User-Name AVP

The User-Name AVP (AVP code 1) contains the user name in the format of a NAI according to RFC 6733 [401].

### 7.1.16	Vendor-Id AVP

The Vendor-Id AVP (AVP code 266), as part of the grouped Vendor-Specific-Application-Id AVP, shall contain the value of 10415, which is the IANA registered value for '3GPP' in TS 29.230 [206].

### 7.1.17	User-Equipment-Info AVP

The User-Equipment-Info  AVP (AVP code 458) is of type grouped and contains the identity and capability of the terminal the subscriber is using for the connection to network according to IETF RFC 4006 [402] with the following ABNF grammar:

- User-Equipment-Info ::= < AVP Header: 458 >

{ User-Equipment-Info-Type }

{ User-Equipment-Info-Value }

For PS charging, when the User-Equipment-Info-Type AVP (AVP code 459) is set to IMEISV (0), the value within the User-Equipment-Info-Value AVP (AVP code 460) is of type OctetString and shall be a UTF-8 encoded hexadecimal. The composition of the IMEISV follows the definition in TS 23.003 [224] . If only IMEI is received a filler ‘F’ is used to make it 16 digits.

For IMS charging, when the User-Equipment-Info-Type AVP (AVP code 459) is set to IMEISV (0), the value within the User-Equipment-Info-Value AVP (AVP code 460) is of type OctetString and shall be a UTF-8 encoded decimal. The composition of the IMEISV follows the definition in TS 23.003 [224]. If only IMEI is received the number of digits are truncated to 15.

## 7.2	3GPP specific AVPs

### 7.2.0	General

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table 7.2.0.1 along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in RFC 4005 [407] with the exception that the 'M' flag shall be set.

- Table 7.2.0.1: 3GPP specific AVPs

| AVP Name | AVP Code |  |  |  | Used in |  |  |  |  |  |  |  |  |  |  |  |  | Value Type |  |  | AVP Flag rules |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ACR |  |  | ACA |  |  | CCR |  |  |  |  | CCA |  |  |  |  | Must |  |  | May |  |  | Should not |  |  | Must not |  |  |  |
| 3GPP-Charging-Characteristics | 13 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Charging-Id | 2 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-GGSN-MCC-MNC | 9 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-IMSI | 1 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC | 8 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-MS-TimeZone | 23 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-NSAPI | 10 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-PDP-Type | 3 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-PS-Data-Off-Status | 4406 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  |  |  |  |  |  |  |  |
| 3GPP-RAT-Type | 21 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Selection-Mode | 12 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator | 11 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC | 18 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-User-Location-Info | 22 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Availability-Change-Reason | 2833 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value | 503 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Network-Info-Change | 4401 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Access-Network-Information | 1263 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Access-Transfer-Information | 2709 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Access-Transfer-Type | 2710 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Account-Expiration | 2309 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Accumulated-Cost | 2052 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Accuracy | 3137 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adaptations | 1217 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ADC-Rule-Base-Name | 1095 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Additional-Content-Information | 1207 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Additional-Exception-Reports | 3936 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Additional-Type-Information | 1205 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Address-Data | 897 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Address-Domain | 898 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Addressee-Type | 1208 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Address-Type | 899 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AF-Charging-Identifier | 505 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AF-Correlation-Information | 1276 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Allocation-Retention-Priority | 1034 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternate-Charged-Party-Address | 1280 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcement-Identifier | 3905 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcement-Information | 3904 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcement-Order | 3906 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcing-PLMN-ID | 4408 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcing-UE-HPLMN-Identifier | 3426 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Announcing-UE-VPLMN-Identifier | 3427 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Cost-Information | 2053 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Format | 2310 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Information | 2054 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | X |  | Grouped |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Request-Type | 2055 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Service | 2311 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Service-Obligatory-Type | 2312 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Service-Type | 2313 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| AoC-Subscription-Information | 2314 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| APN-Rate-Control | 3933 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| APN-Rate-Control-Downlink | 3934 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| APN-Rate-Control-Uplink | 3935 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Applic-ID | 1218 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Application-Port-Identifer | 3010 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[231] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Provided-Called-Party-Address | 837 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Application-Server | 836 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Application-Server-ID | 2101 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Service-Provider-Identity | 532 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer[214] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Server-Information | 850 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Application-Service-Type | 2102 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Session-ID | 2103 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Specific-Data | 3458 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Associated-Party-Address | 2035 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Associated-URI | 856 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Authorised-QoS | 849 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Aux-Applic-Info | 1219 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Base-Time-Interval | 1265 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Basic-Service-Code | 3411 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Bearer-Capability | 3412 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Bearer-Service | 854 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| BSSID | - 2716 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Called-Asserted-Identity | 1250 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Called-Identity | 3916 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Called-Identity-Change | 3917 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Called-Party-Address | 832 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Calling-Party-Address | 831 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Carrier-Select-Routing-Information | 2023 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Cause-Code | 861 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Cellular-Network-Information | 3924 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CG-Address | 846 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Change-Condition | 2037 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Change-Time | 2038 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Charge-Reason-Code | 2118 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | X |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Charged-Party | 857 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Charging-Characteristics-Selection-Mode | 2066 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Charging-Per-IP-CAN-Session-Indicator | 4400 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Charging-Rule-Base-Name | 1004 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class-Identifier | 1214 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Client-Address | 2018 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CN-IP-Multicast-Distribution | 921 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CN-Operator-Selection-Entity | 3421 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Communication-Failure-Information | 4300 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [244] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Class | 1220 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Content-Disposition | 828 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Content-ID | 2116 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Provider-ID | 2117 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Length | 827 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Content-Size | 1206 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Content-Type | 826 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Coverage-Info | 3459 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Coverage-Status | 3428 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CP-CIoT-EPS-Optimisation-Indicator | 3930 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CPDT-Information | 3927 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CSG-Access-Mode | 2317 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CSG-Id | 1437 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[219] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSG-Membership-Indication | 2318 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| CUG-Information | 2304 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Current-Tariff | 2056 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Data-Coding-Scheme | 2001 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| DCD-Information | 2115 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Deferred-Location-Event-Type | 1230 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Delivery-Report-Requested | 1216 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Delivery-Status | 2104 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination-Interface | 2002 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Diagnostics | 2039 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Discoveree-UE-HPLMN-Identifier | 4402 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Discoveree-UE-VPLMN-Identifier | 4403 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Discoverer-UE-HPLMN-Identifier | 4404 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Discoverer-UE-VPLMN-Identifier | 4405 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |  |
| Domain-Name | 1200 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| DRM-Content | 1221 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Dynamic-Address-Flag | 2051 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Dynamic-Address-Flag-Extension | 2068 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Early-Media-Description | 1272 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Enhanced-Diagnostics | 3901 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Envelope | 1266 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Envelope-End-Time | 1267 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Envelope-Reporting | 1268 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Envelope-Start-Time | 1269 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| EPDG-Address | 3425 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| EPS-Location-Information | 1496 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [219] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Event | 825 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Event-Charging-TimeStamp | 1258 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Event-Type | 823 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Expires | 888 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| External-Identifier | 3111 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| FE-Identifier-List | 4413 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| File-Repair-Supported | 1224 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Fixed-User-Location-Info | 2825 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Flows | 510 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forwarding-Pending | 3415 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| From-Address | 2708 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| GGSN-Address | 847 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Guaranteed-Bitrate-UL | 1026 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| IM-Information | 2110 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Application-Reference-Identifier | 2601 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Charging-Identifier | 841 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Communication-Service-Identifier | 1281 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Emergency-Indicator | 2322 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Information | 876 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMS-Visited-Network-Identifier | 2713 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IMSI-Unauthenticated-Flag | 2308 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Incoming-Trunk-Group-Id | 852 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Incremental-Cost | 2062 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Initial-IMS-Charging-Identifier | 2321 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Instance-Id | 3402 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Interface-Id | 2003 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Interface-Port | 2004 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Interface-Text | 2005 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Interface-Type | 2006 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Inter-Operator-Identifier | 838 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Inter-UE-Transfer | 3902 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| IP-Realm-Default-Indication | 2603 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ISUP-Cause | 3416 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ISUP-Cause-Diagnostics | 3422 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ISUP-Cause-Location | 3423 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ISUP-Cause-Value | 3424 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| ISUP-Location-Number | 3414 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Language | 3914 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Layer-2-Group-ID | 3429 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-APN | 1231 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Client-Dialed-By-MS | 1233 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Client-External-ID | 1234 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Client-Id | 1232 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Client-Name | 1235 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Client-Type | 1241 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Data-Coding-Scheme | 1236 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Format-Indicator | 1237 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Information | 878 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Name-String | 1238 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Requestor-ID | 1239 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| LCS-Requestor-ID-String | 1240 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Local-GW-Inserted-Indication | 2604 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Local-Sequence-Number | 2063 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Location-Estimate | 1242 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Location-Estimate-Type | 1243 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Location-Info | 3460 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Location-Type | 1244 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Low-Balance-Indication | 2020 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Low-Priority-Indicator | 2602 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Mandatory-Capability | 604 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [204] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max-Requested-Bandwidth-DL | 515 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max-Requested-Bandwidth-UL | 516 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [214] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Maximum-Detection-Time | 3131 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum-Number-of-Reports | 3128 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-2G-3G-Indicator | 907 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-Charged-Party | 2323 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-Data-Transfer-Start | 929 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-Data-Transfer-Stop | 930 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-GW-Address | 2307 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Address |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-Information | 880 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-Service-Area | 903 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-Service-Type | 906 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-Session-Identity | 908 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MBMS-User-Service-Type | 1225 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Media-Initiator-Flag | 882 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Media-Initiator-Party | 1288 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Message-Body | 889 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Message-Class | 1213 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Message-ID | 1210 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Message-Size | 1212 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Message-Type | 1211 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MMBox-Storage-Requested | 1248 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MM-Content-Type | 1203 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| MME-Name | 2402 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [229] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MME-Number-for-MT-SMS | 1645 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [230] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MME-Realm | 2408 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [229] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MMS-Information | 877 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MMTel-Information | 2030 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MMTel-SService-Type | 2031 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitored-PLMN-Identifier | 3430 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Duration | 3130 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Event-Config-Status | 3142 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Event-Configuration-Activity | 3919 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Event-Functionality | 3922 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Monitoring-Event-Information | 3921 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Event-Report-Data | 3920 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Event-Report-Number | 3923 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-Type | 3127 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Monitoring-UE-HPLMN-Identifier | 3431 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Monitoring-UE-Identifier | 3432 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Monitoring-UE-VPLMN-Identifier | 3433 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| MONTE-Location-Type | 3136 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MSC-Address | 3417 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| MSISDN | 701 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [221] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| MTC-IWF-Address | 3406 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| NBIFOM-Mode | 2830 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| NBIFOM-Support | 2831 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Neighbour-Node-Address | 2705 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Network-Call-Reference-Number | 3418 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Next-Tariff | 2057 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | X |  | Grouped |  |  | V/M |  |  | - |  |  | - |  |  |  |  |  |  |
| NIDD-Submission | 3928 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V/M |  |  | - |  |  | - |  |  |  |  |  |  |
| NNI-Information | 2703 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| NNI-Type | 2704 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Node-Functionality | 862 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Node-Id | 2064 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Of-Diversions | 2034 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-Messages-Sent | 2019 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-Messages-Successfully-Exploded | 2111 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-Messages-Successfully-Sent | 2112 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Of-Participants | 885 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Of-Received-Talk-Bursts | 1282 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-Talk-Bursts | 1283 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-UE-Per-Location-Configuration | 4306 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Number-Of-UE-Per-Location-Report | 4307 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [244] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Portability-Routing-Information | 2024 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Offline-Charging | 1278 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Online-Charging-Flag | 2303 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Optional-Capability | 605 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Origin-App-Layer-User-Id | 3600 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | refer [238] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Originating-IOI | 839 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Originator | 864 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Originator-Address | 886 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Originator-Interface | 2009 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Originator-Received-Address | 2027 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Originator-SCCP-Address | 2008 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Outgoing-Session-Id | 2320 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Outgoing-Trunk-Group-Id | 853 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Participant-Access-Priority | 1259 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Participant-Action-Type | 2049 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Participant-Group | 1260 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Participants-Involved | 887 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PC3-Control-Protocol-Cause | 3434 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PC3-EPC-Control-Protocol-Cause | 3435 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PDN-Connection-Charging-ID | 2050 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PDP-Address | 1227 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PDP-Address-Prefix-Length | 2606 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PDP-Context-Type | 1247 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Play-Alternative | 3913 |  |  |  | - |  |  | - |  |  | - |  |  |  |  | X |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Change-Condition | 1261 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Change-Time | 1262 |  |  |  | X |  |  | - |  |  | - |  |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Controlling-Address | 858 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Event-Type | 2025 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Group-Name | 859 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Information | 879 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Server-Role | 883 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Session-Id | 1229 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Session-Initiation-type | 1277 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-Session-Type | 884 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-User-Role | 1252 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| PoC-User-Role-IDs | 1253 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| PoC-User-Role-Info-Units | 1254 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Positioning-Data | 1245 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |  |
| Preferred-AoC-Currency | 2315 |  |  |  | - |  |  | - |  |  | X |  |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |  |
| Presence-Reporting-Area-Identifier | 2821 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | X |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presence-Reporting-Area-Information | 2822 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | X |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presence-Reporting-Area-Elements-List | 2820 |  |  |  | X |  |  | - |  |  | X |  |  |  |  | X |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  |
| Presence-Reporting-Area-Status |  | 2823 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Priority |  | 1209 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Priority-Indication |  | 3006 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Priority-Level |  | 1046 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Privacy-Indicator |  | 3915 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-3rd-Party-Application-ID |  | 3440 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-App-Id |  | 3811 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [239] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ProSe-Direct-Communication-Reception-Data-Container |  | 3461 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Direct-Communication- Transmission-Data-Container |  | 3441 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Direct-Discovery-Model |  | 3442 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Event-Type |  | 3443 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-ID |  | 3602 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [238] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-IP-Address |  | 3444 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-PLMN-Identifier |  | 3457 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Functionality |  | 3445 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Group-IP-Multicast-Address |  | 3446 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Information |  | 3447 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Range-Class |  | 3448 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Reason-For-Cancellation |  | 3449 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| ProSe-Request-Timestamp |  | 3450 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Role-Of-UE |  | 3451 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Source-IP-Address |  | 3452 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Target-Layer-2-ID |  | 4410 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-UE-ID |  | 3453 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-UE-to-Network-Relay-UE-ID |  | 4409 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Validity-Timer |  | 3815 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Alert-Indication |  | 3454 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Alert-Timestamp |  | 3455 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Cancellation-Timestamp |  | 3456 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| PS-Append-Free-Format-Data |  | 867 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| PS-Free-Format-Data |  | 866 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | X |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| PS-Furnish-Charging-Information |  | 865 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| PS-Information |  | 874 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| QoS-Information |  | 1016 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| QoS-Class-Identifier |  | 1028 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Consumption-Time |  | 881 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Holding-Time |  | 871 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Indicator |  | 3912 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Radio-Frequency |  | 3462 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | OctetString |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Radio-Parameter-Set-Info |  | 3463 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |
| Radio-Parameter-Set-Values |  | 3464 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Radio-Resources-Indicator |  | 3465 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| RAI |  | 909 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| RAN-NAS-Release-Cause |  | 2819 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rate-Control-Max-Message-Size |  | 3937 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Control-Max-Rate |  | 3938 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Control-Time-Unit |  | 3939 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Element |  | 2058 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reachability-Information |  | 3140 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Read-Reply-Report-Requested |  | 1222 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Real-Time-Tariff-Information |  | 2305 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  |  |  |  |  |  |  |
| Reason-Header |  | 3401 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  |  |  |  |  |  |  |
| Received-Talk-Burst-Time |  | 1284 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Received-Talk-Burst-Volume |  | 1285 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Recipient-Address |  | 1201 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-Info |  | 2026 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-Received-Address |  | 2028 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-SCCP-Address |  | 2010 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reference-Number |  | 3007 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Refund-Information |  | 2022 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | X |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-Change-Condition-Information |  | 3925 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-Trigger |  | 3926 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-IMS-Charging-Identifier |  | 2711 |  | X |  |  | - |  |  |  |  |  | X |  |  |  |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-IMS-Charging-Identifier-Node |  | 2712 |  | X |  |  | - |  |  |  |  |  | X |  |  |  |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Relationship-Mode |  | 2706 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Relay-IP-address |  | 4411 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Remaining-Balance |  | 2021 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reply-Applic-ID |  | 1223 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Reply-Path-Requested |  | 2011 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reporting-Reason |  | 872 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Requested-Party-Address |  | 1251 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Requested-PLMN-Identifier |  | 3436 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Requesting-EPUID |  | 3816 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Requestor-PLMN-Identifier |  | 3437 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Required-MBMS-Bearer-Capabilities |  | 901 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Role-Of-Node |  | 829 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Role-Of-ProSe-Function |  | 3438 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Route-Header-Received |  | 3403 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Route-Header-Transmitted |  | 3404 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| RRC-Cause-Counter |  | 4318 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Scale-Factor |  | 2059 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCEF-ID |  | 3125 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SCEF-Reference-ID |  | 3124 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SCS-Address |  | 3941 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCS-AS-Address |  | 3940 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCS-Realm |  | 3942 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | DiameterIdentity |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Answer-Timestamp |  | 1275 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Media-Component |  | 843 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Media-Description |  | 845 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Media-Name |  | 844 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Offer-Timestamp |  | 1274 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Session-Description |  | 842 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-TimeStamps |  | 1273 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Type |  | 2036 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Serving-Node |  | 2401 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Serving-Node-Identity |  | 3929 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | DiameterIdentity |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Served-Party-IP-Address |  | 848 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Server-Capabilities |  | 603 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Server-Name |  | 602 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Service-Data-Container |  | 2040 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Generic-Information |  | 1256 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service-Id |  | 855 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Information |  | 873 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | X |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Service-Mode |  | 2032 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Data |  | 863 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Info |  | 1249 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Type |  | 1257 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Serving-Node-Type |  | 2047 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Serving-PLMN-Rate-Control |  | 4310 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Session-Direction |  | 2707 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Session-Priority |  | 650 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SGi-PtP-Tunnelling-Method |  | 3931 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGSN-Address |  | 1228 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGW-Address |  | 2067 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGW-Change |  | 2065 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Method |  | 824 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Request-Timestamp |  | 834 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Request-Timestamp-Fraction |  | 2301 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Response-Timestamp |  | 835 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Response-Timestamp-Fraction |  | 2302 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Device-Trigger-Indicator |  | 3407 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Device-Trigger-Information |  | 3405 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Discharge-Time |  | 2012 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Message-Type |  | 2007 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Protocol-ID |  | 2013 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Sequence-Number |  | 3408 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SM-Service-Type |  | 2029 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SM-Status |  | 2014 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-User-Data-Header |  | 2015 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Information |  | 2000 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Node |  | 2016 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Result |  | 3409 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMSC-Address |  | 2017 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Sponsor-Identity |  | 531 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [214] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SSID |  | 1524 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [237] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Start-of-Charging |  | 3419 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Start-Time |  | 2041 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Status- AS-Code |  | 2702 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Stop-Time |  | 2042 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Submission-Time |  | 1202 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Subscriber-Role |  | 2033 |  | - |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Supplementary-Service |  | 2048 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| TAD-Identifier |  | 2717 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Talk-Burst-Exchange |  | 1255 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Talk-Burst-Time |  | 1286 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Talk-Burst-Volume |  | 1287 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Target-App-Layer-User-Id |  | 3601 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer[238] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Target-IP-Address |  | 4412 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | Address |  |  | VM |  |  | - |  |  | - |  |  | - |  |  |
| Tariff-Information |  | 2060 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | X |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Tariff-XML |  | 2306 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TDF-IP-Address |  | 1091 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Teleservice |  | 3413 |  | - |  |  | - |  |  |  |  |  | X |  |  |  | - |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Terminal-Information |  | 1401 |  | X |  |  | - |  |  |  |  |  | - |  |  |  | - |  | refer [219] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Terminating-IOI |  | 840 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Reception |  | 3466 |  | X |  |  |  |  |  | - |  |  | - | - |  |  |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Transmission |  | 3467 |  | X |  |  |  |  |  | - |  |  | - | - |  |  |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Usage |  | 2043 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Indicator |  | 3911 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Last-Usage |  | 2044 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Quota-Mechanism |  | 1270 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Quota-Threshold |  | 868 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Quota-Type |  | 1271 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Stamps |  | 833 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Usage |  | 2045 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Window |  | 3818 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| TMGI |  | 900 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Token-Text |  | 1215 |  | - |  |  | - |  |  |  |  |  | X | - |  |  |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Total-Number-Of-Messages-Exploded |  | 2113 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Total-Number-Of-Messages-Sent |  | 2114 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Traffic-Data-Volumes |  | 2046 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Transcoder-Inserted-Indication |  | 2605 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Transit-IOI-List |  | 2701 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Transmitter-Info |  | 3468 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Trigger |  | 1264 |  | - |  |  | - |  |  |  |  |  | X | X |  |  |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Trigger-Type |  | 870 |  | - |  |  | - |  |  |  |  |  | X | X |  |  |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Trunk-Group-Id |  | 851 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TWAG-Address |  | 3903 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TWAN-User-Location-Info |  | 2714 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Type-Number |  | 1204 |  | - |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| UDP-Source-Port |  | 2806 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Local-IP-Address |  | 2805 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Reachability-Configuration |  | 3129 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| UNI-PDU-CP-Only-Flag |  | 3932 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Unit-Cost |  | 2061 |  | - |  |  | - |  |  |  |  |  | X | X |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Unit-Quota-Threshold |  | 1226 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Unused-Quota-Timer |  |  | 4407 |  |  | - |  |  | - |  |  | X |  |  | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Usage-Information-Report-Sequence-Number |  | 3439 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| User-CSG-Information |  | 2319 |  | X |  |  | - |  |  |  |  |  | X | X |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| User-Data |  | 606 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| User-Location-Info-Time |  | 2812 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| User-Participating-Type |  | 1279 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| User–Session-Id |  | 830 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| UWAN-User-Location-Info |  | 3918 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| VAS-Id |  | 1102 |  | - |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [213] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Variable-Part |  | 3907 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Variable-Part-Order |  | 3908 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Variable-Part-Type |  | 3909 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Variable-Part-Value |  | 3910 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| VASP-Id |  | 1101 |  | - |  |  | - |  |  |  |  |  | X | - |  |  |  |  | refer [213] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| VCS-Information |  | 3410 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Visited-PLMN-Id |  | 1407 |  | X |  |  | - |  |  |  |  |  | - | - |  |  |  |  | refer [230] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| VLR-Number |  | 3420 |  | X |  |  | - |  |  |  |  |  | X | - |  |  |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Volume-Quota-Threshold |  | 869 |  | - |  |  | - |  |  |  |  |  | - | X |  |  |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| WLAN-Link-Layer-Id |  | 3821 |  | X |  |  | - |  |  |  |  |  | X |  |  |  | - |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |

NOTE:	Per TS 29.230 [206] the following AVP codes are reserved for the present specification:
Before Rel-8: 800 to 899, 1200 to 1399
Rel-8: 2000 to 2199.
Rel-9: 2300 to 2399. 
Rel-10: 2600 to 2699. 
Rel-11: 2700 to 2799. 
Rel-12: 3400 to 3499. 
Rel-13: 3900 to 3999.
Rel-14: 4400 to 4499.

### 7.2.0A	Access-Network-Info-Change AVP

The Access-Network-Info-Change AVP (AVP code 4401) is of type Grouped and holds information on subsequent changes in one or two SIP P-header(s) "P-Access-Network-Info" together with the time it was acquired.

It has the following ABNF grammar:

Access-Network-Info-Change :: =  < AVP Header: 4401>

*	[ Access-Network-Information ]

[ Cellular-Network-Information ]

[ Change-Time ]

### 7.2.0aA	3GPP-PS-Data-Off-Status AVP

The 3GPP-PS-Data-Off-Status AVP (AVP code 4406) is of type Enumerated and indicated the change in 3GPP PS Data off status of UE. The following values are defined:

0 		Active

1 		Inactive

### 7.2.1	Access-Network-Information AVP

The Access-Network-Information AVP (AVP code 1263) is of type OctetString and indicates one instance of the SIP P-header "P-Access-Network-Info". In SIP, as per RFC 7315 [404], the content of the "P-Access-Network-Info" header is known as the access-net-spec. When multiple access-net-spec values are transported in a single "P-Access-Network-Info" header in comma-separated format, then multiple Access-Network-Information AVPs are used with one access-net-spec value included in each AVP.

### 7.2.1A	Access-Transfer-Information AVP

The Access-Transfer-Information AVP (AVP code 2709) is of type Grouped and provides information on access transfer for IMS service continuity.

It has the following ABNF grammar:

Access-Transfer-Information :: =  < AVP Header: 2709>

[ Access-Transfer-Type ]

*	[ Access-Network-Information ]

[ Cellular-Network-Information ]

[ Inter-UE-Transfer ]

[ User-Equipment-Info ]

[ Instance-Id ]

[ Related-IMS-Charging-Identifier ]

[ Related-IMS-Charging-Identifier-Node ]

[ Change-Time ]

### 7.2.1B	Access-Transfer-Type AVP

The Access-Transfer-Type AVP (AVP code 2710) is of type Enumerated and indicates which type of transfer occurred for IMS service continuity. The following values are defined:

0	PS to CS Transfer

1	CS to PS Transfer

2	PS to PS Transfer

3	CS to CS Transfer

### 7.2.2	Account-Expiration AVP

The Account-Expiration AVP (AVP code 2309) is of type Time and indicates the subscriber account expiration date and time of day.

### 7.2.3	Accumulated-Cost AVP

The Accumulated-Cost AVP (AVP code 2052) is of type Grouped and holds the accumulated cost for the ongoing session.

It has the following ABNF grammar:

Accumulated-Cost:: = < AVP Header: 2052 >

{ Value-Digits }

[ Exponent ]

### 7.2.4	Adaptations AVP

The Adaptations AVP (AVP code 1217) is of type Enumerated and indicates whether the originator allows adaptation of the content (default Yes). The values indicating whether adaptations are allowed are:

0	Yes

1	No

### 7.2.5	Additional-Content-Information AVP

The Additional-Content-Information AVP (AVPcode 1207) is of type Grouped and identifies any subsequent content types. It is used to identify each content (including re-occurences) within an MM when the Type-Number AVP or Additional-Type-Information AVP from the Content-Type AVP indicate a multi-part content.

It has the following ABNF grammar:

Additional-Content-Information:: = < AVP Header: 1207 >

[ Type-Number ]

[ Additional-Type-Information ]

[ Content-Size ]

### 7.2.5A	Additional-Exception-Reports AVP

The Additional-Exception-Reports AVP (AVP code 3936) is of type Enumerated and indicates whether additional exception reports are allowed when maximum rate is reached. It has the following values:

0	Not allowed

1	Allowed

### 7.2.6	Additional-Type-Information AVP

The Additional-Type-Information AVP (AVP code 1205) is of type UTF8String and identifies any additional information beyond well-known media types or non-well-known media types.

### 7.2.7	Address-Data AVP

The Address-Data AVP (AVP code 897) is of type UTF8String and indicates the address information and formatted according type of address indicated in the Address-Type AVP and according to MMS encapsulation [209].

### 7.2.8	Address-Domain AVP

The Address-Domain AVP (AVP code 898) is of type Grouped and indicates the domain/network to which the associated address resides. If this AVP is present, at least one of the AVPs described within the grouping shall be included.

It has the following ABNF:

Address-Domain :: =  < AVP Header: 898 >

[ Domain-Name ]

[ 3GPP-IMSI-MCC-MNC ]

### 7.2.9	Address-Type AVP

The Address-Type AVP (AVP code 899) is of type Enumerated and indicates the type of address carried within the Address-Information AVP. It has the following values:

0	e-mail address

1	MSISDN

2	IPv4 Address

3	IPv6 Address

4	Numeric Shortcode

5	Alphanumeric Shortcode

6	Other

7	IMSI

### 7.2.10	Addressee-Type AVP

The Addressee-Type AVP (AVP code 1208) is of type Enumerated and identifies the how the recipient is addressed in the header of an MM. The following values are defined:

0	TO

1	CC

2	BCC

### 7.2.11	AF-Correlation-Information AVP

The AF-Correlation-Information AVP (AVPcode 1276) is of type Grouped and includes the "AF Charging Identifier" (ICID for IMS)  and associated  flow identifiers generated by the AF and received by P-GW over Rx/Gx as defined in TS 29.214 [214] and TS 29.212 [215]. The AF-Correlation-Information is defined per Rating Group or per Rating Group and Service Identifier when Service Identifier level reporting applies.

When several AF sessions (refer to TS 29.214 [214]) are conveyed over the same bearer, this AVP may appear several times per MSCC instance.

It has the following ABNF grammar:

AF-Correlation-Information:: =  < AVP Header: 1276 >

{ AF-Charging-Identifier }

* [ Flows ]

### 7.2.12	Alternate-Charged-Party-Address AVP

The Alternate-Charged-Party-Address AVP (AVP code 1280) is of type UTF8String and  holds  the address of the alternate charged party determined by an AS at IMS session initiation.

### 7.2.12aA	Announcement-Identifier AVP

The Announcement-Identifier AVP (AVP code 3905) is of type Unsigned32. It contains a code identifying an announcement to be played.

### 7.2.12aB	Announcement-Information AVP

The Announcement-Information AVP (AVP code 3904) is of type Grouped and holds the Announcement service parameters. Each Announcement-Information AVP specifies a single announcement to be played to the specified user.

The Announcement-Order AVP contains the order in which announcements should be connected for playback when there are multiple Announcement-Information AVPs provided in a single message with the same timing indicator.

It has the following ABNF grammar:

Announcement-Information:: =  < AVP Header: 3904 >

{ Announcement-Identifier }

*	[ Variable-Part ]

[ Time-Indicator ]

[ Quota-Indicator ]

[ Announcement-Order ]

[ Play-Alternative ]

[ Privacy-Indicator ]

[ Language ]

### 7.2.12aC	Announcement-Order AVP

The Announcement-Order  AVP (AVP code 3906) is of type Unsigned32. It contains the order in which announcements should be connected for playback when there are multiple Announcement-Information AVPs provided in a single message with the same timing indicator.

### 7.2.12aD	Announcing-PLMN-ID AVP

The Announcing-PLMN-ID AVP (AVP code4408) is of type UTF8String and contains PLMN identity of the serving PLMN which signalled the carrier frequency, when this serving PLMN is not the HPLMN nor the VPLMN, if available.

### 7.2.12A	Announcing-UE-HPLMN-Identifier AVP

The Announcing-UE-HPLMN-Identifier AVP (AVP code 3426) is of type UTF8String and contains identifier of annoucing UE HPLMN.

### 7.2.12B	Announcing-UE-VPLMN-Identifier AVP

The Announcing-UE-VPLMN-Identifier AVP (AVP code 3427) is of type UTF8String and contains identifier of annoucing UE VPLMN.

### 7.2.13	AoC-Cost-Information AVP

The AoC-Cost-Information AVP (AVP code 2053) is of type Grouped and holds accumulated and incremental cost infromation for the AoC service

It has the following ABNF grammar:

AoC-Cost-Information:: =  < AVP Header: 2053 >

[ Accumulated-Cost ]

* [ Incremental-Cost ]

[ Currency-Code ]

### 7.2.14	AoC-Format AVP

The AoC-Format AVP (AVP code 2310) is of type Enumerated and holds the format on how the AoC information shall be sent to the UE. It can be one of the following values:

0	MONETARY

1	NON_MONETARY

2	CAI

### 7.2.15	AoC-Information AVP

The AoC-Information AVP (AVP code 2054) is of type Grouped that includes the information required for advice of charge.

It has the following ABNF grammar:

AoC-Information:: =  < AVP Header: 2054 >

[ AoC-Cost-Information ]

[ Tariff-Information ]

[ AoC-Subscription-Information ]

### 7.2.16	AoC-Request-Type AVP

The AoC-Request-Type AVP (AVP code 2055) is of type enumerated and tags if the client is looking for AoCI in conjunction to the Request-Type and Request-Action AVPs. It can be one of the following values:

0	AoC_NOT_REQUESTED

1	AoC_FULL

2	AoC_COST_ONLY

3	AoC_TARIFF_ONLY

### 7.2.17	AoC-Service AVP

The AoC-Service AVP (AVP code 2311) is of type Grouped and holds the pair of AoC Service type and AoC Service obligatory type.

It has the following ABNF grammar:

AoC-Service :: =          < AVP Header: 2311 >

[ AoC-Service-Obligatory-Type ]

[ AoC-Service-Type ]

### 7.2.18	AoC-Service-Obligatory-Type AVP

The AoC-Service-Obligatory-Type AVP (AVP code 2312) is of type Enumerated and holds the information if the AoC information is binding or not. It can be one of the following values:

0	NON_BINDING

1	BINDING

### 7.2.19	AoC-Service-Type AVP

The AoC-Service-Type AVP (AVP code 2313) is of type Enumerated and defines the type of AoC information to be provided to the subscriber. It can be one of the following values:

0	NONE

1	AOC-S

2	AOC-D

3	AOC-E

### 7.2.20	AoC-Subscription-Information AVP

The AoC-Subscription-Information AVP (AVP code 2314) is of type Grouped and holds the subscription and formatting parameters received from HSS.

It has the following ABNF grammar:

AoC-Subscription-Information:: =      < AVP Header: 2314 >

* [ AoC-Service ]

[ AoC-Format ]

[ Preferred-AoC-Currency ]

### 7.2.20A	APN-Rate-Control   AVP

The APN-Rate-Control AVP (AVP code 3933) is of type Grouped and holds the Rate Control parameters applicable to data PDUs transfers in uplink and downlink direction for this APN.

It has the following ABNF grammar:

APN-Rate-Control :: = 	< AVP Header: 3933 >

[ APN-Rate-Control-Uplink ]

[ APN-Rate-Control-Downlink]

### 7.2.20B	APN-Rate-Control-Downlink   AVP

The APN-Rate-Control-Downlink AVP (AVP code 3934) is of type Grouped and holds the APN Rate Control parameters applicable to data PDUs transfers in downlink for this APN.

It has the following ABNF grammar:

APN-Rate-Control-Downlink :: = 	< AVP Header: 3934 >

[ Rate-Control-Time-Unit ]

[ Rate-Control-Max-Rate ]

[ Rate-Control-Max-Message-Size ]

### 7.2.20C		APN-Rate-Control-Uplink   AVP

The APN-Rate-Control-Uplink AVP (AVP code 3935) is of type Grouped and holds the APN Rate Control parameters applicable to data PDUs transfers in uplink for this APN.

It has the following ABNF grammar:

APN-Rate-Control-Uplink :: = 	< AVP Header: 3935>

[ Additional-Exception-Reports ]

[ Rate-Control-Time-Unit ]

[ Rate-Control-Max-Rate ]

### 7.2.21	Applic-ID AVP

The Applic-ID AVP (AVP code 1218) is of type UTF8String and holds the identification of the destination application that the underlying MMS abstract message was addressed to.

### 7.2.22	Application-Provided-Called-Party-Address AVP

The Application-Provided-Called-Party-Address AVP (AVP code 837) is of type UTF8String and holds the called party number (SIP URI, E.164), if it is determined by an application server.

### 7.2.23	Application-Server AVP

The Application-Server AVP (AVP code 836) is of type UTF8String and holds the SIP URL(s) of the AS(s) addressed during the session.

### 7.2.24	Application-Server-Information AVP

The Application-Server-Information AVP (AVP code 850) is of type Grouped and contains information about application servers visited through ISC interface.

It has the following ABNF grammar:

<Application-Server-Information>::=	<AVP Header: 850 >

[ Application-Server ]
									   * [ Application-Provided-Called-Party-Address ]
										[ Status- AS-Code ]

### 7.2.24A	Application-Specific-Data AVP

The Application-Specific-Data AVP (AVP code 3458) is of type OctetString and contains a data block provided by the application in the UE. The content of each block is application-specific.

### 7.2.25	Associated-Party-Address AVP

The Associated-Party-Address AVP (AVP code 2035) is of type UTF8String and is used for MMTel supplementary service. It holds the address (SIP URI or Tel URI) of the user, the MMTel supplementary service is provided to :   the "forwarding party" for CDIV, the "transferor" for ECT, the "Pilot Identity" for Flexible Alerting (FA), the "Initiator party" for 3PTY.

### 7.2.26	Associated-URI AVP

The Associated-URI AVP (AVP code 856) is of type UTF8String and holds a non-barred public user identity (SIP URI or Tel URI) associated to the the public user identity under registration. This identity is obtained from the P-Associated-URI header of a 200 OK SIP response to a REGISTER request. This AVP may appear several times when the P-Associated-URI header contains more than one public user identity.

### 7.2.27	Authorised-QoS AVP

The Authorised-QoS AVP (AVP code 849) is of type UTF8String and holds the Authorised QoS as defined in TS 23.207 [200] / TS 29.207 [203] and applied via the Go reference point. This AVP is not used.

### 7.2.28	Aux-Applic-Info AVP

The Aux-Applic-Info AVP (AVP code 1219) is of type UTF8String and holds additional application/implementation specific control information.

### 7.2.29	Base-Time-Interval AVP

The Base-Time-Interval AVP (AVP code 1265) is of type Unsigned32. It contains the length of the base time interval, for controlling the consumption of time quota, in seconds.

### 7.2.29A	Basic-Service-Code AVP

The Basic-Service-Code AVP (AVP code 3411) is of type Grouped and holds service information utilized by a voice call service. Either a Bearer-Service AVP or a Teleservice AVP is included.

It has the following ABNF grammar:

Basic-Service-Code:: =   < AVP Header: 3411 >

[ Bearer-Service ]

[ Teleservice ]

### 7.2.29B	Bearer-Capability AVP

The Bearer-Capability AVP (AVP code 3412) is of type OctetString and indicates the type of bearer capability connection to the user. The value as described in TS 29.078 [233] shall be used.

### 7.2.30	Bearer-Service AVP

The Bearer-Service AVP (AVP code 854) is of type OctetString and holds the used bearer service for the PSTN leg of an IMS session or a voice call service.

### 7.2.30A	BSSID AVP

The BSSID AVP (AVP code 2716) is of type UTF8String and contains the BSSID of the access point where UE is located in a WLAN Access Network. The format used for BSSID is described in IEEE Std 802.11-2012 [409].

### 7.2.31	Called-Asserted-Identity AVP

The Called-Asserted-Identity AVP (AVP code 1250) is of type UTF8String and holds the address (Public User ID: SIP URI, E.164, etc.) of the finally asserted called party.

The address is obtained from the P-Asserted-Identity SIP header field of the 2xx responses corresponding to a SIP request either initiating a dialog or a standalone transaction. This field may appear several times in the request when the P-Asserted-Identity contains both a SIP URI and a Tel URI.

This field shall be present when the P-Asserted-Identity SIP header field is available in the SIP 2xx response.

### 7.2.31A	Called-Identity AVP

The Called-Identity AVP (AVP code 3916) is of type UTF8String and holds the address (Public User ID: SIP URI, E.164, etc.) of the called party after a change.

The address is obtained from the From SIP header field of a SIP UPDATE request or SIP RE-INVITE request.

### 7.2.31B	Called-Identity-Change AVP

The Called-Identity-Change AVP (AVP code 3917) is of type Grouped and provides the terminating identity address change and associated time stamp.

It has the following ABNF grammar:

Called-Identity-Change :: =  < AVP Header: 3917>

[ Called-Identity ]

[ Change-Time ]

### 7.2.32	Called-Party-Address AVP

The Called-Party-Address AVP (AVP code 832) is of type UTF8String. It holds the address (SIP URI, Tel URI or URN) of the party to whom the call is addressed.

For IMS charging (except for SIP Register and SIP Subscription transactions), it holds the address of the party (Public User ID or Public Service ID) to whom the SIP transaction is posted. The Called-Party-Address AVP shall be populated with the SIP URI or Tel URI contained in the Request-URI of the outgoing request. For a registration procedure, this AVP holds the party (Public User ID) to be registered. In this case, the Called-Party-Address AVP is obtained from the "To" SIP header of the SIP Request. For a subscription procedure this AVP holds the address of the resource for which the originator wants to receive notifications of change of states. In this case, the Called-Party-Address AVP is obtained from the outgoing Request-URI of the SIP Request.

For VCS charging, it holds the address (URN is not applicable) which identifies the party to which a voice call is destined after processing by the Proxy Function. It is converted from the circuit-switched Called Party Number as per TS 29.163 [234] for the Request-URI header. For a mobile originating call, this AVP contains the Called Party Number after processing by the Proxy Function (e.g. number normalization).

### 7.2.33	Calling-Party-Address AVP

The Calling-Party-Address AVP (AVP code 831) is of type UTF8String and, for IMS charging, holds the address (SIP URI or Tel URI) which identifies the party (Public User Identity or Public Service Identity) initiating a SIP transaction. It is obtained from the P-Asserted-Identity header of any non-REGISTER SIP Request, either initiating a dialog or a standalone transaction. This AVP may appear several times when the P-Asserted-Identity header contains both a SIP URI and a Tel URI. In case no P-Asserted-Identity is known, this AVP list shall include one item with the value "unknown".

For VCS charging, it holds the address (SIP URI or Tel URI) which identifies the party initiating a voice call. It is converted from the circuit-switched Calling Party Number as per TS 29.163 [234] for the P-Asserted-Identity header.

### 7.2.34	Carrier-Select-Routing-Information AVP

The Carrier-Select-Routing-Information AVP (AVP code 2023) is of type UTF8String. This AVP holds information on carrier selection performed by S-CSCF/AS. This information is sent over SIP in the Requested URI header.

### 7.2.35	Cause-Code AVP

The Cause-Code AVP (AVP code 861) is of type Integer32 and includes the cause code value from IMS node. It is used in ACR[Stop] and/or ACR[Event] messages. It is also used in the CCR[Terminate] and/or CCR[Event] messages.

Within the cause codes, values  0 are reserved for successful causes while values  1 are used for failure causes. In case of errors where the session has been terminated as a result of a specific known SIP error code, then the SIP error code is also used as the cause code.

- Successful cause code values

0		Normal end of session

The cause "Normal end of session" is used in Accounting-request[stop] message to indicate that an ongoing SIP session has been normally released either by the user or by the network (SIP BYE message initiated by the user or initiated by the network has been received by the IMS node after the reception of the SIP ACK message).

-1		Successful transaction when 200 Final Response

The cause "Successful transaction" is used in Accounting-request[Event] message to indicate a successful SIP transaction (e.g. SIP REGISTER, SIP MESSAGE, SIP NOTIFY, SIP SUBSCRIBE when 200 Final Response). It may also be used by an Application Server to indicate successful service event execution.

-2		End of SUBSCRIBE dialog

The cause "End of SUBSCRIBE dialog" is used to indicate the closure of a SIP SUBSCRIBE dialog . For instance a successful SIP SUBSCRIBE transaction terminating the dialog has been detected by the IMS node (i.e. SIP SUBSCRIBE with expire time set to 0).

-2xx	2xx Final Response

The cause-code "2xx Final Response"(except 200) is used when the SIP transaction is terminated due to an IMS node receiving/initiating a 2xx Final response as described in RFC 3261 [405].

-3xx	3xx Redirection

The cause "3xx Redirection" is used when the SIP transaction is terminated due to an IMS node receiving/initiating a 3xx response as described in RFC 3261 [405].

-3		End of REGISTER dialog

The cause "End of REGISTER dialog" is used to indicate the closure of a SIP REGISTER dialog. For instance a successful SIP REGISTER transaction terminating the dialog has been detected by the IMS node (i.e. SIP REGISTER with expire time set to 0).

- Failure cause code values

1		Unspecified error

The cause "Unspecified error" is used when the SIP transaction is terminated due to an unknown error.

4xx	4xx Request failure

The cause "4xx Request failure" is used when the SIP transaction is terminated due to an IMS node receiving/initiating a 4xx error response as described in RFC 3261 [405].

5xx	5xx Server failure

The cause "5xx Server failure" is used when the SIP transaction is terminated due to an IMS node receiving/initiating a 5xx error response as described in RFC 3261 [405].

6xx	6xx Global failure

The cause "6xx Global failure" is used when the SIP transaction is terminated due to an IMS node receiving/initiating a 6xx error response as described in RFC 3261 [405].

2		Unsuccessful session setup

The cause "Unsuccessful session setup" is used in the Accounting-request[stop] when the SIP session has not been successfully established (i.e. Timer H expires and SIP ACK is not received or SIP BYE is received after reception of the SIP 200 OK final response and SIP ACK is not received) as described in TS 24.229 [202] and in RFC 3261 [405].

3		Internal error

The cause "Internal error" is used when the SIP transaction is terminated due to an IMS node internal error (e.g. error in processing a request/response).

### 7.2.35A	Cellular-Network-Information AVP

The Cellular-Network-Information AVP (AVP code 3924) is of type OctetString and indicates one instance of the SIP header "Cellular-Network-Info".

### 7.2.36	CG-Address AVP

The CG-Address AVP (AVP code 846) is of type Address and holds the IP-address of the charging gateway.

### 7.2.37	Change-Condition AVP

The Change-Condition AVP (AVP code 2037) is of type Integer32,  and indicates the change in charging condition: (Qos change, tariff time change …) which causes:

-	Sending of Accounting-Request from node;

-	Volume counts container closing for an IP-CAN bearer;

-	Service data container closing;

-	ProSe direct communication data container closing;

-	Record closing.

The following  values are defined :

0	Normal Release
The "Normal Release" value is used to indicate IP-CAN session termination , IP-CAN bearer release or Service Data Flow Termination; PDN connection to SCEF release.

1	Abnormal Release

2	Qos Change

3	Volume Limit

4	Time Limit

5	Serving Node Change

6	Serving Node PLMN Change

7	User Location Change

8	RAT Change

9	UE TimeZone Change

10	Tariff Time Change

11	Service Idled Out

12	ServiceSpecificUnitLimit

13	Max Number of Changes in Charging conditions

14	CGI-SAI Change

15	RAI Change

16	ECGI Change

17	TAI Change

18	Service Data Volume Limit

19	Service Data Time Limit

20	Management Intervention

21	Service Stop

22	User CSG Information Change

23	S-GW Change

24	Change of UE Presence in Presence Reporting Area

25	Proximity alerted

26	Time expired with no renewal

27	Requestor cancellation

28 	Maximum number of reports

29	PLMN change

30	Coverage status change

31 Removal of access

32 Unavailability of access

33 Access change of service data flow

34 Indirect change condition

35 Maximum number of NIDD submissions

36 Change in UP to UE

37 Serving PLMN Rate Control Change

38 APN Rate Control Change

39 NIDD Submission Response Receipt

40 NIDD Submission Response Sending

41 NIDD Delivery to UE

42 NIDD Delivery from UE Error

43 NIDD Submission Timeout

44 MO exception data counter

45 Change of 3GPP PS Data off Status

NOTE:	Values 25-30 are applied for ProSe charging only.

### 7.2.38	Change-Time AVP

The Change-Time AVP (AVP code 2038) is of type Time.

In EPC Charging, it holds the time in UTC format when the volume counts associated to the IP-CAN bearer/TDF session, or the service data container, is closed and reported due to charging condition change.

For IMS Charging, it holds the time in UTC format at which the access transfer occurred.

For MMTel Charging, it holds the time in UTC format and it is a time stamp that defines the moment when the conference participant has an action (e.g. creating the conference, joining in the conference, being invited into the conference and quiting the conference) triggering the Accounting Request message to CDF.

In ProSe Charging, it holds the time in UTC format when the volume counts associated to the ProSe group communication container, is closed and reported due to ProSe charging condition change.

### 7.2.38A	Charge-Reason-Code AVP

The Charge-Reason-Code AVP (AVP code 2118) is of type Enumerated and identifies if the Rate-Element corresponds to a specific charge type. It can be one of the following values:

0	UNKNOWN

1	USAGE

2	COMMUNICATION-ATTEMPT-CHARGE

3	SETUP-CHARGE

4	ADD-ON-CHARGE

### 7.2.39	Charged-Party AVP

The Charged-Party AVP (AVP code 857) is of type UTF8String and holds the address (Public User ID: SIP URI, Tel URI, etc.) of the party to be charged.

For Monitoring Event charging, it contains a string that identifies the entity towards which accounting/charging functionality is performed by the involved 3GPP network elements.

### 7.2.39A	Charging-Characteristics-Selection-Mode AVP

The Charging-Characteristics-Selection-Mode AVP (AVP code 2066) is of type Enumerated and indicates how the applied Charging-Characteristics was selected. It may take following values:

0	Serving-Node-Supplied

1	Subscription-specific

2	APN-specific

3	Home-Default

4	Roaming-Default

5	Visiting-Default

### 7.2.39B	Charging-Per-IP-CAN-Session-Indicator AVP

The Charging-Per-IP-CAN-Session-Indicator AVP (AVP code 4400) is of type of Enumerated. It indicates whether the charging per IP-CAN session is active for offline charging. If this AVP is not present, that means charging per IP-CAN session is not active.

The following value is defined:

0	Inactive

1	Active

### 7.2.40	Class-Identifier AVP

The Class-Identifier AVP (AVP code 1214) is of type Enumerated and the values are:

0	Personal

1	Advertisement

2	Informational

3	Auto

### 7.2.41	Client-Address AVP

The Client-Address AVP (AVP code 2018) is of type Address and is the address of the messaging Node which the OCS is connected to.

### 7.2.41A	CN-Operator-Selection-Entity AVP

The CN-Operator-Selection-Entity AVP (AVP code 3421) is of type Enumerated and holds the indication on which entity has selected the Serving Core Network in Network Sharing situations.

It has the following values:

0	The Serving Network has been selected by the UE

1	The Serving Network has been selected by the network

### 7.2.42	Content-Class AVP

The Content-Class AVP (AVP code 1220) is of type Enumerated and classifies the content of the MM to the highest content class to which the MM belongs, as defined in MMS Encapsulation [209]. The classes can be one of the following:

0	text

1	image-basic

2	image-rich

3	video-basic

4	video-rich

5	megapixel

6	content-basic

7	content-rich

### 7.2.43	Content-Disposition AVP

The Content-Disposition AVP (AVP code 828) is of type UTF8String and indicates how the message body or a message body part is to be interpreted (e.g. session, render), as described in RFC 3261 [405].

### 7.2.44	Content-Length AVP

The Content-Length AVP (AVP code 827) is of type Unsigned32 and holds the size of the message-body, as described in RFC 3261 [405].

### 7.2.45	Content-Size AVP

The Content-Size AVP (AVP code 1206) is of type Unsigned32 and indicates the size in bytes of the specified content type.

### 7.2.46	Content-Type AVP

The Content-Type AVP (AVP code 826) is of type UTF8String and holds the media type (e.g. application/sdp, text/html) of the message-body, as described in RFC 3261 [405].

### 7.2.46aA	Coverage-Status AVP

The Coverage-Status AVP (AVP code 3428) is of type Enumerated and indicates whether UE is served by E-UTRAN or not. The following values are defined:

0 		Out of coverage

1 		In coverage

### 7.2.46aaA	Coverage-Info AVP

The Coverage-Info AVP (AVP code 3459) is of type Grouped and provides information on the coverage status (i.e., whether the UE is served by E-UTRAN or not) and the time when the coverage status changed to its current state. When in E-UTRAN coverage, additionally includes a list of location changes (i.e., ECGI change) and associated time for each change.

It has the following ABNF grammar:

Coverage-Info :: = < AVP Header: 3459>

[ Coverage-Status ]

[ Change-Time ]

*  [ Location-Info ]

### 7.2.46abA	CP-CIoT-EPS-Optimisation-Indicator AVP

The CP-CIoT-EPS-Optimisation-Indicator AVP (AVP code 3930) is of type Enumerated, and indicates whether Control Plane CIoT EPS optimisation is used by this PDN connection for data transfer with the UE. The following values are defined:

0	Not Apply

1	Apply

### 7.2.46acA	CPDT-Information AVP

The CPDT-Information AVP (AVP code 3927) is of type Grouped. Its purpose is to allow the transmission of additional CP data transfer service specific information elements.

It has the following ABNF grammar:

CPDT-Information :: = < AVP Header: 3927>

[ External-Identifier ]

[ SCEF-ID ]

[ Serving Node Identity ]

[ SGW-Change ]
[ NIDD-submission ]

### 7.2.46A	CSG-Access-Mode AVP

The CSG-Access-Mode AVP (AVP code 2317) is of type Enumerated and holds the mode in which the CSG cell User is accessing to, operates. It has the following values:

0	Closed mode

1	Hybrid Mode

### 7.2.46B	CSG-Membership-Indication AVP

The CSG-Membership-Indication AVP (AVP code 2318) is of type Enumerated, and indicates the UE is a member of the accessing CSG cell, if the access mode is Hybrid, as described in TS 29.060 [225], and in TS 29.274 [226]. If this indication is not present, this means the UE is a Not member of the CSG cell for hybrid access mode. The following values are defined:

0	Not CSG member

1	CSG Member

### 7.2.47	Current-Tariff AVP

The Current-Tariff AVP (AVP code 2056) is of type Grouped and holds tariff information. The Tariff is a formula for cost calculation given the Used-Service-Unit AVP. The calculated cost is given in the Currency-Code AVP. The formula sums all the rating elements and multiplies the sum by the Scale-Factor AVP.

It has the following ABNF grammar:

Current-Tariff:: =   < AVP Header: 2056 >

[ Currency-Code ]

[ Scale-Factor ]

* [ Rate-Element ]

### 7.2.48	CUG-Information AVP

The CUG-Information AVP (AVP code 2304) is of type OctetString and holds the "CUG Interlock Code" which identifies CUG membership within the Network for "Closed User Group" MMTel supplementary service.

### 7.2.49	Data-Coding-Scheme AVP

The Data-Coding-Scheme AVP (AVP code 2001) is of type Integer32 and contains the data coding scheme of the message. For SM applications the specific coding is as described in TS 23.040 [216].

### 7.2.50	DCD-Information AVP

The DCD-Information AVP (AVP code 2115) is of type Grouped. Its purpose is to allow the transmission of service information elements used for DCD services.

The AVP is defined in OMA-DDS-Charging_Data [223].

### 7.2.51	Deferred-Location-Event-Type AVP

The Deferred-Location-Even-Type AVP (AVP code 1230) is of type UTF8String and holds information related to a deferred location request.

### 7.2.52	Delivery-Report-Requested AVP

The Delivery-Report-Requested AVP (AVP code 1216) is of type Enumerated and indicates whether a delivery report has been requested by the originator or not. The values for whether a report was requested are:

0	No

1	Yes

### 7.2.53	Destination-Interface AVP

The Destination-Interface AVP (AVP code 2002) is type Grouped, which contains information related to the Interface on which the message is to be delivered.

Destination-Interface ::= < AVP Header: 2002 >

[ Interface-Id ]

[ Interface-Text ]

[ Interface-Port ]

[ Interface-Type ]

### 7.2.54	Diagnostics AVP

The Diagnostics AVP (AVP code 2039) is of type Integer32 and provides a more detailed cause value from PCN node. It complements the Change-Condition AVP for Offline Charging.

### 7.2.54A	Discoveree-UE-HPLMN-Identifier AVP

The Discoveree-UE-HPLMN-Identifier AVP (AVP code 4402) of type UTF8String and contains identifier of Discoveree UE HPLMN.

### 7.2.54B	Discoveree-UE-VPLMN-Identifier AVP

The Discoveree-UE-VPLMN-Identifier AVP (AVP code 4403) is of type UTF8String and contains identifier of Discoveree UE VPLMN.

### 7.2.54C	Discoverer-UE-HPLMN-Identifier AVP

The Discoverer-UE-HPLMN-Identifier AVP (AVP code 4404) of type UTF8String and contains identifier of Discoverer UE HPLMN.

### 7.2.54D	Discoverer-UE-VPLMN-Identifier AVP

The Discoverer-UE-VPLMN-Identifier AVP (AVP code 4405) of type UTF8String and contains identifier of Discoverer UE VPLMN.

### 7.2.55	Domain-Name AVP

The Domain-Name AVP (AVP code 1200) is of type UTF8String and represents a fully qualified domain name (FQDN).

### 7.2.56	DRM-Content AVP

The DRM-Content AVP (AVP code 1221) is of type Enumerated and indicates if the MM contains DRM-protected content. The values are:

0	No

1	Yes

### 7.2.57	Dynamic-Address-Flag AVP

The Dynamic-Address-Flag AVP (AVP code 2051)  is of type Enumerated, and indicates whether the PDP context/PDN address is statically or dynamically allocated. If this AVP is not present, this means that the address is statically allocated. The following values are defined:

0	Static

1	Dynamic

### 7.2.57A	Dynamic-Address-Flag-Extension AVP

The Dynamic-Address-Flag-Extension AVP (AVP code 2068) is of type Enumerated, and indicatesthat the IPv4 PDN address has been dynamically allocated for that particular IP CAN bearer (PDN connection) of PDN type IPv4v6, and the dynamic IPv6 address is indicated in Dynamic Address Flag. If this AVP is not present, this means that the address is statically allocated.The following values are defined:

0	Static

1	Dynamic

### 7.2.58	Early-Media-Description AVP

The Early-Media-Description AVP (AVP code 1272) is of type grouped and describes the SDP session, media parameters and timestamps related to media components set to active according to SDP signalling exchanged during a  SIP session establishment before the final successful or unsuccessful SIP answer to the initial SIP INVITE message is received. Once a media component has been set to active, subsequent status changes shall also be registered.

It has the following ABNF grammar:

<Early-Media-Description>:: =	 <AVP Header: 1272>

[ SDP-TimeStamps ]

* [ SDP-Media-Component ]

* [ SDP-Session-Description ]

Media can be considered as inactive in range of situations, such as the listed below according to RFC 3264 [408]:

-	Media marked with "a=inactive" attribute.

-	Media offered with zero bandwith.

In contrast, media with directionality marked as "a=recvonly", "a=sendonly", "a=sendrecv" shall be considered in state  "active" and thus, it may be exchanged in one or both directions.

### 7.2.58A	Enhanced-Diagnostics AVP

The Enhanced-Diagnostics AVP (AVP code 3901) is of type Grouped and provides a set of causes for the release. It complements the Change-Condition AVP for Offline Charging from PCN Nodes.

It has the following ABNF grammar:

Enhanced-Diagnostics :: = 	< AVP Header: 3901>

*[ RAN-NAS-Release-Cause ]

NOTE: The RAN-NAS-Release-Cause AVP is under a grouped AVP to allow extensions to other types of release causes in the future.

### 7.2.59	Envelope AVP

The Envelope AVP (AVP code 1266) is a grouped AVP which reports the start and end time of one time envelope using the Envelope-Start-Time and Envelope-End-Time AVPs. Further details of its usage are described in clause 6.5.6 and clause 6.5.7.

Envelope :: = < AVP Header: 1266>

{ Envelope-Start-Time }

[ Envelope-End-Time ]

[ CC-Total-Octets ]

[ CC-Input-Octets ]

[ CC-Output-Octets ]

[ CC-Service-Specific-Units ]

If an envelope has not been closed at the time of the usage report, then the Envelope-End-Time AVP shall be absent. If an envelope was started before the reporting interval then the Envelope-Start-Time is nevertheless present and contains the same time as previously reported, i.e. the actual time of the start of the envelope. The client shall include the volume reports (the CC-xxxxx-Octets AVPs) or events (CC-Service-Specific-Units AVP) if these were requested in the corresponding Envelope-Reporting AVP. The reported volume is always the volume from the beginning of the time envelope.

In circumstances, in which an envelope is retrospectively deemed to have been closed, e.g. with Quota-Consumption-Time changes in a CCA, then the client shall include the Envelope AVP for the envelope in the next usage report.

Multiple occurrences of this AVP shall be in chronological order, i.e. the first envelope is listed first in CCR.

### 7.2.60	Envelope-End-Time AVP

This Envelope-End-Time AVP (AVP code 1267) is of type Time. It is set to the time of the end of the time envelope.

### 7.2.61	Envelope-Reporting AVP

This Envelope-Reporting AVP (AVP code 1268) is of type Enumerated and is used in the CCA[INITIAL] to indicate whether the client shall report the start and end of each time envelope, in those cases in which quota is consumed in envelopes. It can take the values:

0		DO_NOT_REPORT_ENVELOPES

1		REPORT_ENVELOPES

2		REPORT_ENVELOPES_WITH_VOLUME

3		REPORT_ENVELOPES_WITH_EVENTS

4		REPORT_ENVELOPES_WITH_VOLUME_AND_EVENTS

If this AVP is not included in the CCA[Initial] then the client shall not report the individual envelopes. 
If this AVP is included within the Offline-Charging AVP, the value shall dictate the mechanism by which offline charging information is generated.

### 7.2.62	Envelope-Start-Time AVP

The Envelope-Start-Time AVP (AVP code 1269) is of type Time. It is set to the time of the packet of user data which caused the time envelope to start.

### 7.2.62A			EPDG-Address AVP

The EPDG-Address AVP (AVP code 3425) is of type Address and holds the IP-address of the ePDG Node.

### 7.2.63	Event AVP

The Event AVP (AVP code 825) is of type UTF8String and holds the content of the "Event" header.

### 7.2.64	Event-Charging-TimeStamp AVP

The Event-Charging-TimeStamp AVP (AVP code 1258) is of type Time, and it holds the timestamp of the event reported in the CC-Service-Specific-Units AVP when event based charging applies.

### 7.2.65	Event-Type AVP

The Event-Type AVP (AVP code 823) is of type Grouped and contains information about the type of chargeable telecommunication service/event for which the ACR and/or CCR message(s) is generated.

It has the following ABNF grammar:

<Event-Type>::	=	<AVP Header: 823 >

[ SIP-Method ]

[ Event ]

[ Expires ]

### 7.2.66	Expires AVP

The Expires AVP (AVP code 888) is of type Unsigned32 and holds the content of the "Expires" header, if available.

### 7.2.66A	FE-Identifier-List AVP

The FE-Identifier-List AVP (AVP code 4413) is of type UTF8String and holds the FE-Identifier-List as defined in TS 24.229 [210] of IMS Network Elements that generates charging information.

The FE-Identifier exchange via SIP signalling is defined in TS 24.229 [210].

### 7.2.67	File-Repair-Supported AVP

The File-Repair-Supported AVP (AVP code 1224) is of type Enumerated and indicates whether the MBMS user service supports point-to-point file repair. The following values are supported:

1		SUPPORTED

The MBMS user service does support point-to-point file repair.

2		NOT_SUPPORTED

The MBMS user service does not support point-to-point file repair.

### 7.2.67aA	Forwarding-Pending AVP

The Forwarding-Pending AVP (AVP code 3415) is of type Enumerated and indicates that a forwarded-to-number has been received and the voice call will be forwarded. When it is not present, the voice call is not expected to be forwarded. The values are:

0	Forwarding not pending

1	Forwarding pending

### 7.2.67A	From-Address AVP

The From-Address AVP (AVP code 2708) is of type UTF8String and includes the information from the SIP From header.

### 7.2.68	GGSN-Address AVP

The GGSN-Address AVP (AVP code 847) is of type Address and holds the IP-address of the P-GW that generated the GPRS/EPC Charging ID, as described in [1].

### 7.2.69	IM-Information AVP

The IM-Information AVP (AVP code 2110) is of type Grouped. Its purpose is to allow the transmission of service information elements used for IM services. The AVP is defined in OMA-DDS-Charging_Data [223].

### 7.2.70	Incremental-Cost AVP

The Incremental-Cost AVP (AVP code 2062) is of type Grouped and holds the incremental cost since last AoC interaction for the ongoing session.

It has the following ABNF grammar:

Incremental-Cost:: = < AVP Header: 2062 >

{ Value-Digits }

[ Exponent ]

### 7.2.70A	Instance-Id AVP

The Instance-Id AVP (AVP code 3402) is of type UTF8String and contains a URN generated by the device that uniquely identifies a specific device amongst all other devices. The Instance Id is transported in the Contact header of a SIP request associated with the served user.

### 7.2.71	Interface-Id AVP

The Interface-Id AVP (AVP code 2003) is of type UTF8String and holds the interface identification provided by the messaging node (originator/destination).

### 7.2.72	Interface-Port AVP

The Interface-Port AVP (AVP code 2004) is of type UTF8String and holds the port-identification or contains information about the transport layer port used by the application associated with the charging event.

### 7.2.73	Interface-Text AVP

The Interface-Text AVP (AVP code 2005) is of type UTF8String and is the consolidation information about the application associated with the charging event.

### 7.2.74	Interface-Type AVP

The Interface-Type AVP (AVP code 2006) is of type Enumerated and contains information about type of interface / nature of the transaction in the messaging node for which the charging event occurs. The AVP can take the following values:

0	Unknown

1	MOBILE_ORIGINATING

2	MOBILE_TERMINATING

3	APPLICATION_ORIGINATING

4	APPLICATION_TERMINATION

### 7.2.74aA	Inter-UE-Transfer AVP

The Inter-UE-Transfer AVP (AVP code 3902) is of type Enumerated and contains information about type of the transfer. If this AVP is not present, this means that the type of transfer is Intra-UE transfer. The AVP can take the following values:

0	Intra-UE transfer

1	Inter-UE transfer

### 7.2.74A	IMS-Application-Reference-Identifier AVP

The IMS-Application-Reference-Identifier AVP (AVP code 2601) is of type UTF8String and holds the IMS Application Reference Identifier (IARI) as contained in a SIP request to identify an IMS Application as defined in TS 24.229 [202].

Editor's Note: The SIP parameter from which the IMS Application Reference ID (IARI) is to be extracted requires further investigation in CT1. A mechanism to identify the IARI in use is FFS.

### 7.2.75	IMS-Charging-Identifier AVP

The IMS-Charging-Identifier AVP (AVP code 841) is of type UTF8String and holds the IMS Charging Identifier (ICID) as generated by a IMS node for a SIP session and described in TS 24.229 [204].

### 7.2.76	IMS-Communication-Service-Identifier AVP

The IMS-Communication-Service-Identifier AVP (AVP code 1281) is of type UTF8String and holds the IMS Communication Service Identifier (ICSI) as contained in the P-Asserted-Service header of a SIP request to identify an IMS Communication Service as defined in TS 24.229 [202].

### 7.2.76A	IMS-Emergency-Indicator AVP

The IMS-Emergency-Indicator AVP (AVP code 2322) is of type Enumerated, and indicates the IMS session is an IMS emergency session or IMS registration. If this AVP is not present, this means the IMS session or registration is not detected as an emergency session or registration. The following values are defined:

0	Non Emergency

1	Emergency

### 7.2.77	IMS-Information AVP

The IMS-Information AVP (AVP code 876) is of type Grouped. Its purpose is to allow the transmission of additional IMS service specific information elements.

It has the following ABNF grammar:

IMS-Information :: = 	  < AVP Header: 876>

[ Event-Type ]

[ Role-Of-Node ]

{ Node-Functionality }

[ User-Session-Id ]

[ Outgoing-Session-Id ]

[ Session-Priority ]

*  [ Calling-Party-Address ]

[ Called-Party-Address ]

*  [ Called-Asserted-Identity ]

[ Called-Identity-Change ]

[ Number-Portability-Routing-Information ]

[ Carrier-Select-Routing-Information ]

[ Alternate-Charged-Party-Address ]

*  [ Requested-Party-Address ]

*  [ Associated-URI ]

[ Time-Stamps ]

*  [ Application-Server-Information ]

*  [ Inter-Operator-Identifier ]

*	[ Transit-IOI-List ]

[ IMS-Charging-Identifier ]

*  [ SDP-Session-Description ]

*  [ SDP-Media-Component ]

[ Served-Party-IP-Address ]

[ Server-Capabilities ]

[ Trunk-Group-ID ]

[ Bearer-Service ]

[ Service-Id ]

*  [ Service-Specific-Info ]

*  [ Message-Body ]

[ Cause-Code ]

*  [ Reason-Header ]

*	[ Access-Network-Information ]

[ Cellular-Network-Information ]

*  [ Early-Media-Description ]

[ IMS-Communication-Service-Identifier ]

[ IMS-Application-Reference-Identifier ]

[ Online-Charging-Flag ]

[ Real-Time-Tariff-Information ]

[ Account-Expiration ]

[ Initial-IMS-Charging-Identifier ]

* [ NNI-Information ]

[ From-Address ]

[ IMS-Emergency-Indicator ]

[ IMS-Visited-Network-Identifier ]

*  [ Access-Network-Info-Change ]

*  [ Access-Transfer-Information ]

[ Related-IMS-Charging-Identifier ]

[ Related-IMS-Charging-Identifier-Node ]

[ Route-Header-Received ]

[ Route-Header-Transmitted ]

[ Instance-Id ]

[TAD-Identifier]

[FE-Identifier-List]

### 7.2.77A	IMS-Visited-Network-Identifier AVP

The IMS-Visited-Network-Identifier AVP (AVP code 2713) is of type UTF8String and contains the contents of the SIP P-header "P-Visited-Network-ID". with the value according to 3GPP TS 24.229 [202]

-	For the roaming architecture for voice over IMS with local breakout, the value of IMS-Visited-Network-Identifier AVP is a pre-provisioned string that identifies the network of the P-CSCF at the home network.

-	For the roaming architecture for voice over IMS with home routed traffic, IMS-Visited-Network-Identifier AVP is a string that identifies the visited network of the UE including an indication that the P-CSCF is located in the home network.

### 7.2.78	IMSI-Unauthenticated-Flag AVP

The IMSI-Unauthenticated-Flag AVP (AVP code 2308) is of type Enumerated, and indicates the served IMSI is not authenticated. This may occur when emergency bearer service is provided (refer to TS 23.060 [74] and TS 29.274 [91]). If this flag is not present, this means the served IMSI is authenticated. The following values are defined:

0	Authenticated

1	Unauthenticated

### 7.2.79	Incoming-Trunk-Group-ID AVP

The Incoming-Trunk-Group-ID AVP (AVP code 852) is of type UTF8String and identifies the incoming PSTN leg.

### 7.2.79A	Initial-IMS-Charging-Identifier AVP

The Initial-IMS-Charging-Identifier AVP (AVP code 2321) is of type UTF8String and holds the Initial IMS Charging Identifier (ICID) as generated by a IMS node for the initial SIP session created for IMS service continuity.

### 7.2.80	Inter-Operator-Identifier AVP

The Inter-Operator-Identifier AVP (AVP code 838) is of type Grouped and holds the identification of the network neighbours (originating and terminating) as exchanged via SIP signalling and described in RFC 7315 [404].

It has the following ABNF grammar:

<Inter-Operator-Identifier>:: =  < AVP Header: 838 >

[ Originating-IOI ]

[ Terminating-IOI ]

### 7.2.80A	IP-Realm-Default-Indication AVP

The IP-Realm-Default-Indication-Indication AVP (AVP code 2603) is of type Enumerated and indicates whether the IP realm used for the SDP media component is the Default IP realm or not. The following values are defined:

0	Default IP Realm Not used

1	Default IP realm used

### 7.2.80B	ISUP-Cause AVP

The ISUP-Cause AVP (AVP code 3416) is of type Grouped and indicates the reason a voice call service is released. When used for IMS charging, this AVP indicates the reason a CS call is released.

It has the following ABNF:

- ISUP-Cause ::= <AVP Header: 3416>

[ ISUP-Cause-Location ]
[ ISUP-Cause-Value ]
[ ISUP-Cause-Diagnostics ]

### 7.2.80C	ISUP-Cause-Diagnostics AVP

The ISUP-Cause-Diagnostics AVP (AVP code 3422) is of type OctetString and holds the diagnostics field associated with the release of the voice call service, if available. When used for IMS charging, this AVP holds the diagnostics field associated with the release of a CS call.  Refer to TS 29.078 [233] for supported values.

### 7.2.80D	ISUP-Cause-Location AVP

The ISUP-Cause-Location AVP (AVP code 3423) is of type Unsigned32 and identifies the network in which the event causing the call release. Refer to TS 29.078 [233] for supported values.

### 7.2.80E	ISUP-Cause-Value AVP

The ISUP-Cause-Value AVP (AVP code 3424) is of type Unsigned32 and identifies the reason a voice call service is released. When used for IMS charging, this AVP indicates the reason a CS call is released.  Refer to TS 29.078 [233] for supported values.

### 7.2.80F	ISUP-Location-Number AVP

The ISUP-Location-Number AVP (AVP code 3414) is type OctetString and contains the international E.164 indicating the location number of the served user. It contains an E.164 number as transported in ISUP and is encoded as a TBCD-string. See TS 29.002 [232] for encoding of TBCD-strings. This AVP does not include leading indicators for the nature of address and the numbering plan; it contains only the TBCD-encoded digits of the address.

### 7.2.80Fa	Language AVP

The Language AVP (AVP code 3914) is of type UTF8String and contains a code defined in RFC 3066 [410] indicating the language of the announcement that should be played.

### 7.2.80G	Layer-2-Group-ID AVP

The Layer-2-Group-ID AVP (AVP code 3429) is of type OctetString and carry the identifier of a ProSe communication group, uniquely represents a specific one-to-many ProSe Direct Communication and is included in CDRs for each participantes in the specific group. This identity is referred to as "ProSe Layer-2 Group ID" in TS 23.303 [235].

### 7.2.81	LCS-APN AVP

The LCS-APN AVP (AVP code 1231) is of type UTF8String and contains the APN of the LCS Client.

### 7.2.82	LCS-Client-Dialed-By-MS AVP

The LCS-Client-Dialed-By-MS AVP (AVP code 1233) is of type UTF8String and holds the number of the LCS Client dialled by the UE.

### 7.2.83	LCS-Client-External-ID AVP

The LCS-Client-External-ID AVP (AVP code 1234) is of type UTF8String and holds the identification of the external LCS Client.

### 7.2.84	LCS-Client-ID AVP

The LCS-Client-Id AVP (AVP code 1232) is of type Grouped and holds information related to the identity of an LCS client.

It has the following ABNF grammar:

<LCS-Client-ID>:: = < AVP Header: 1232 >

[ LCS-Client-Type ]

[ LCS-Client-External-ID ]

[ LCS-Client-Dialed-By-MS ]

[ LCS-Client-Name ]

[ LCS-APN ]

[ LCS-Requestor-ID ]

### 7.2.85	LCS-Client-Name AVP

The LCS-Client-Name AVP (AVP code 1235) is of type Grouped and contains the information related to the name of the LCS Client.

It has the following ABNF grammar:

<LCS-Client-Name>:: = < AVP Header: 1235>

[ LCS-Data-Coding-Scheme ]

[ LCS-Name-String ]

[ LCS-Format-Indicator ]

### 7.2.86	LCS-Client-Type AVP

The LCS-Client-Type AVP (AVP code 1241) is of type Enumerated and contains the type of services requested by the LCS Client. It can be one of the following values:

0	EMERGENCY_SERVICES

1	VALUE_ADDED_SERVICES

2	PLMN_OPERATOR_SERVICES

3	LAWFUL_INTERCEPT_SERVICES

### 7.2.87	LCS-Data-Coding-Scheme AVP

The LCS-Data-Coding-Scheme AVP (AVP code 1236) is of type UTF8String and contains the information of the alphabet and the language used.

### 7.2.88	LCS-Format-Indicator AVP

The LCS-Format-Indicator AVP (AVP code 1237) is of type Enumerated and contains the format of the LCS Client name. It can be one of the following values:

0	LOGICAL_NAME

1	EMAIL_ADDRESS

2	MSISDN

3	URL

4	SIP_URL

### 7.2.89	LCS-Information AVP

The LCS-Information AVP (AVP code 878) is of type Grouped. Its purpose is to allow the transmission of additional LCS service specific information elements.

It has the following ABNF grammar:

LCS-Information :: = < AVP Header: 878>

[ LCS-Client-ID ]

[ Location-Type ]

[ Location-Estimate ]

[ Positioning-Data ]

[ 3GPP-IMSI ]

[ MSISDN ]

### 7.2.90	LCS-Name-String AVP

The LCS-Name-String AVP (AVP code 1238) is of type UTF8String and contains the LCS Client name.

### 7.2.91	LCS-Requestor-ID AVP

The LCS-Requestor-ID AVP (AVP code 1239) is of type Grouped and contains information related to the identification of the Requestor.

It has the following ABNF grammar:

<LCS-Requestor-ID>:: = 	< AVP Header: 1239 >

[ LCS-Data-Coding-Scheme ]

[ LCS-Requestor-ID-String ]

### 7.2.92	LCS-Requestor-ID-String AVP

The LCS-Requestor-ID-String AVP (AVP code 1240) is of type UTF8String and contains the identification of the Requestor and can be e.g. MSISDN or logical name.

### 7.2.92A	Local-GW-Inserted-Indication AVP

The Local-GW-Inserted-Indication  AVP (AVP code 2604) is of type Enumerated and indicates if the local GW (TrGW, IMS-AGW) is inserted or not for the SDP media component.

The following values are defined:

0	Local GW Not Inserted

1	Local GW Inserted

### 7.2.93	Local-Sequence-Number AVP

The Local-Sequence-Number  AVP (AVP code 2063) is of type Unsigned32 and holds the service data container sequence number : increased by 1 for each service data container closed.

### 7.2.94	Location-Estimate AVP

The Location-Estimate AVP (AVP code 1242) is of type OctetString and contains an estimate of the location of an MS in universal coordinates and the accuracy of the estimate. Refers to the geographical area description in TS 23.032 [227] for the internal structure and encoding of this AVP.

### 7.2.95	Location-Estimate-Type AVP

The Location-Estimate-Type AVP (AVP code 1243) is of type Enumerated and contains one of the following values:

0	CURRENT_LOCATION

1	CURRENT_LAST_KNOWN_LOCATION

2	INITIAL_LOCATION

3	ACTIVATE_DEFERRED_LOCATION

4	CANCEL_DEFERRED_LOCATION

### 7.2.95A	Location-Info AVP

The Location-Info AVP (AVP code 3460) is of type Grouped and provides information on the location (i.e., ECGI) and associated time for the change to that location.

It has the following ABNF grammar:

Location-Info :: = < AVP Header: 3460>

[ 3GPP-User-Location-Info ]

[ Change-Time ]

### 7.2.96	Location-Type AVP

The Location-Type AVP (AVP code 1244) is of type Grouped and indicates the type of location estimate required by the LCS client.

It has the following ABNF grammar:

Location-Type:: = 			< AVP Header: 1244>

[ Location-Estimate-Type ]

[ Deferred-Location-Event-Type ]

### 7.2.97	Low-Balance-Indication AVP

The Low-Balance-Indication AVP (AVP code 2020) is of type Enumerated and indicates if the subscriber balance went below a designated threshold by its account. This indication can be used to advise the end user about the need to replenish his balance. It can be one of the following values:

0	NOT-APPLICABLE

1	YES

### 7.2.97A	Low-Priority-Indicator AVP

The Low-Priority-Indicator AVP (AVP code 2602) is of type Enumerated and indicates if the PDN connection has a low priority, i.e. for Machine Type Communications. It can be one of the following values:

0	NO

1	YES

### 7.2.97B	MBMS-Charged-Party AVP

The MBMS-Charged-Party AVP (AVP code 2323) is of type Enumerated and indicates which party is being charged by the CDR generated. The following values are supported:

0	Content Provider

1	Subscriber

### 7.2.98	MBMS-GW-Address AVP

The MBMS-GW-Address AVP (AVP code 2307) is of type Address and holds the IP-address. 
This AVP of the MBMS GW that generated the MBMS Charging ID when MBMS GW is stand-alone.

### 7.2.99	MBMS-Information AVP

The MBMS-Information AVP (AVP code 880) is of type Grouped. Its purpose is to allow the transmission of additional MBMS service specific information elements.

It has the following ABNF grammar:

MBMS-Information :: = 	< AVP Header: 880>

[ TMGI ]

[ MBMS-Service-Type ]

[ MBMS-User-Service-Type ]

[ File-Repair-Supported ]

[ Required-MBMS-Bearer-Capabilities ]

[ MBMS-2G-3G-Indicator ]

[ RAI ]

* [ MBMS-Service-Area ]

[ MBMS-Session-Identity ]

[ CN-IP-Multicast-Distribution ]

[ MBMS-GW-Address ]

[ MBMS-Charged-Party ]

* [ MSISDN ]

[ MBMS-Data-Transfer-Start ]

[ MBMS-Data-Transfer-Stop ]

### 7.2.100	MBMS-User-Service-Type AVP

The MBMS-User-Service-Type AVP (AVP code 1225) is of type Enumerated indicates type of service the the MBMS user service that is being delivered. The following values are supported:

1	DOWNLOAD
The MBMS user service of type: download.

2	STREAMING
The MBMS user service is of type: streaming.

### 7.2.101	Media-Initiator-Flag AVP

The Media-Initiator-Flag AVP (AVP code 882) is of type Enumerated and indicates which party has requested the session modification. The default value is '0' indicating the called party initiated the modification:

0		called party

1		calling party

2		unknown

### 7.2.102	Media-Initiator-Party AVP

The Media-Initiator-Party AVP (AVP code 1288) is of type UTF8String. Enumerated in IMS charging, it holds the address (SIP URI or Tel URI) of the party (Public User ID or Public Service ID) who initiates the media action, like adding/removing, connecting/disconnecting the media. The Media Initiator Party shall be populated with the SIP URI or Tel URI contained in the Request-URI of the outgoing request. It is use for PoC charging.

### 7.2.103	Message-Body AVP

The Message-Body AVP (AVP Code 889) is of type Grouped AVP and holds information about the message bodies including user-to-user data.

It has the following ABNF grammar:

<Message-Body>::=		< AVP Header: 889 >

{ Content-Type }
{ Content-Length }
[ Content-Disposition ]
	[ Originator ]

The message bodies shall not include the bodies' of Content-Type = "application-sdp" as these are captured in other AVPs.

### 7.2.104	Message-Class AVP

The Message-Class AVP (AVP code 1213) is of type Grouped.

It has the following ABNF grammar:

Message-Class :: = < AVP Header: 1213 >

[ Class-Identifier ]

[ Token-Text ]

### 7.2.105	Message-ID AVP

The Message-ID AVP (AVP code 1210) is of type UTF8String and holds the identification of the message being charged.

### 7.2.106	Message-Size AVP

The Message-Size AVP (AVP code 1212) is of type Unsigned32. For MMS, it holds the total size in bytes of the MM calculated according to TS 23.140 [208]. For SMS, it holds the total size in octets of SM including any user data header.

### 7.2.107	Message-Type AVP

The Message-Type AVP (AVP code 1211) is of type Enumerated and holds the type of the message according to the MMS transactions e.g. submission, delivery. The following values are defined and are as specified in MMS Encapsulation [209]:

1	m-send-req

2 	m-send-conf

3	m-notification-ind

4	m-notifyresp-ind

5	m-retrieve-conf

6	m-acknowledge-ind

7	m-delivery-ind

8	m-read-rec-ind

9	m-read-orig-ind

10	m-forward-req

11	m-forward-conf

12	m-mbox-store-conf

13	m-mbox-view-conf

14	m-mbox-upload-conf

15	m-mbox-delete-conf

### 7.2.108	MM-Content-Type AVP

The MM-Content-Type AVP (AVP code 1203) is of type Grouped and indicates the overall content type of the MM content and includes information about all the contents of an MM.

It has the following ABNF grammar:

MM-Content-Type :: = 	< AVP Header: 1203 >

[ Type-Number ]

[ Additional-Type-Information ]

[ Content-Size ]

* [ Additional-Content-Information ]

### 7.2.109	MMBox-Storage-Requested AVP

The MMBox-Storage-Requested AVP (AVP code 1248) is of type Enumerated and indicates whether an MMBoxstorage has been requested by the originator MMS User Agent or not. The values for whether an MMBox Storage was requested are:

0		No

1		Yes

### 7.2.110	MMS-Information AVP

The MMS-Information AVP (AVP code 877) is of type Grouped. Its purpose is to allow the transmission of additional MMS service specific information elements.

It has the following ABNF grammar:

MMS-Information :: = 	< AVP Header: 877>

[ Originator-Address ]

* [ Recipient-Address ]

[ Submission-Time ]

[ MM-Content-Type ]

[ Priority ]

[ Message-ID ]

[ Message-Type ]

[ Message-Size ]

[ Message-Class ]

[ Delivery-Report-Requested ]

[ Read-Reply-Report-Requested ]

[ MMBox-Storage-Requested ]

[ Applic-ID ]

[ Reply-Applic-ID ]

[ Aux-Applic-Info ]

[ Content-Class ]

[ DRM-Content ]

[ Adaptations ]

[ VASP-Id ]

[ VAS-Id ]

### 7.2.111	MMTel-Information AVP

The MMTel-Information AVP (AVP code 2030) is of type Grouped. Its purpose is to allow the transmission of additional MMtel  service specific information elements. It holds MMTel supplementary services invoked during MMTel service.

It has the following ABNF grammar:

MMTel-Information :: =  < AVP Header: 2030>

* [ Supplementary-Service ]

### 7.2.111A		MMTel-SService-Type AVP

The MMTel-SService-Type AVP (AVP Code 2031) is of type Unsigned32 and identifies the type of MMTel supplementary service. The following values are defined:

0	Originating Identification Presentation (OIP)

1	Originating Identification Restriction (OIR)

2	Terminating Identification Presentation (TIP)

3	Terminating Identification Restriction (TIR)

4	Communication HOLD (HOLD)

5	Communications Barring (CB )

6	Communication Diversion (CDIV)

8	Communication Waiting (CW)

9	Message Waiting Indication (MWI)

10	Conference (CONF)

11	Flexible Alerting (FA)

12	Completion of Communication to Busy Subscriber (CCBS)

13	Completion of Communications on No Reply (CCNR)

14	Malicious Communication Identification (MCID)

15	Customized Alerting Tone (CAT)

16	Closed User Group (CUG)

17	Personal Network management (PNM)

18	Customized Ringing Signal (CRS)

19	Advice of Charge (AoC)

20	Explicit Communication Transfer (ECT)

- Values   1024  are reserved for specific Network/Manufacturer supplementary services variants.

### 7.2.111Aa		Monitored-PLMN-Identifier AVP

The Monitored-PLMN-Identifier AVP (AVP code 3430) is of type UTF8String and carries Monitored PLMN ID in MATCH_REPORT request. It corresponds to the Announcing UE VPLMN Identifier when roaming and Announcing UE HPLMN Identifier when non-roaming. It is referred to as "Monitored PLMN ID” in TS 23.303 [235].

### 7.2.111AaA	Monitoring-Event-Configuration-Activity AVP

The Monitoring-Event-Configuration-Activity AVP (AVP code3919) is of type Enumerated and indicates Monitoring Event configuration action which triggers the Monitoring Event charging. The AVP may take the values as follows:

0 		create

1 		transfer

2 		update

3		delete

### 7.2.111AaB	Monitoring-Event-Functionality AVP

The Monitoring-Event-Functionality AVP (AVP code 3922) is of type Enumerated and indicates the Monitoring Event functionality to be charged. The AVP may take the values as follows:

0 		Configuration

1 		Reporting

### 7.2.111AaC	Monitoring-Event-Information AVP

The Monitoring-Event-Information AVP (AVP code 3921) is of type Grouped. Its purpose is to allow the transmission of additional Monitoring Event specific information elements.

It has the following ABNF grammar:

Monitoring-Event-Information :: = 	< AVP Header: 3921>

[ Monitoring-Event-Functionality ]

[ Event-Timestamp ]

[ Monitoring-Event-Configuration-Activity ]

[ SCEF-Reference-ID ]

[ SCEF-ID ]

[ Monitoring-Type ]

[ Maximum-Number-of-Reports ]

[ Monitoring-Duration ]

[ Charged-Party ]

[ Maximum-Detection-Time]

[ UE-Reachability-Configuration ]

[ MONTE-Location-Type ]

[ Accuracy ]

* [ Number-Of-UE-Per-Location-Configuration ]

* [ Monitoring-Event-Report ]

### 7.2.111AaD	Monitoring-Event-Report-Data AVP

The Monitoring-Event-Report-Data AVP (AVP code 3920) is of type Grouped. Its purpose is to allow the transmission of charging information associated with an individual Monitoring Event report.

It has the following ABNF grammar:

Monitoring-Event-Report-Data :: = 	< AVP Header: 3920>

[ Event-Timestamp ]

[ SCEF-Reference-ID ]

[ SCEF-ID ]

[ Monitoring-Event-Report-Number ]

[ Charged-Party ]

[ Subscription-Id ]

[ Monitoring-Type ]

[ Reachability-Information ]

[ EPS-Location-Information ]

[ Communication-Failure-Information ]

* [ Number-Of-UE-Per-Location-Report ]

### 7.2.111AaE	Monitoring-Event-Report-Number AVP

The Monitoring-Event-Report-Number AVP (AVP code 3923) is of type Unsigned32 and indicates the number of the report being sent for the specific Monitoring Event request from this node.

### 7.2.111Ab	Monitoring-UE-HPLMN-Identifier AVP

The Monitoring-UE-VPLMN-Identifier AVP (AVP code 3431) is of type UTF8String and contains identifier of monitoring UE HPLMN.

### 7.2.111Ac	Monitoring-UE-Identifier AVP

The Monitoring-UE-Identifier AVP (AVP code 3432) is of type UTF8String and carry identifier of the party who initiate monitor/match report, i.e. IMSI, which corresponds to UE Identifier parameter in monitor/match report request.

### 7.2.111Ad	Monitoring-UE-VPLMN-Identifier AVP

The Monitoring-UE-VPLMN-Identifier AVP (AVP code 3433) is of type UTF8String and contains identifier of monitoring UE VPLMN.

### 7.2.111B	MSC-Address AVP

The MSC-Address AVP (AVP code 3417) is type OctetString and contains the international E.164 address of the MSC that generated the network call reference number. It is encoded as a TBCD-string. See TS 29.002 [232] for encoding of TBCD-strings. This AVP does not include leading indicators for the nature of address and the numbering plan; it contains only the TBCD-encoded digits of the address.

### 7.2.111C	MTC-IWF-Address AVP

The MTC-IWF-Address AVP (AVP code 3406) is of type Address and holds the address of MTC-IWF for Machine Type Communcication, defined in TS 29.337 [231].

### 7.2.111D	Neighbour-Node-Address AVP

The Neighbour-Node-Address AVP (AVP code 2705) is of type Address and holds the control plane IP address of the neighbouring network contact point that handles the service request in case of interconnection and roaming.

### 7.2.111E	Network-Call-Reference-Number AVP

The Network-Call-Reference-Number AVP (AVP code 3418) is type OctetString and contains the reference number generated by the GMSC/MSC for the voice call service. When combined with the identity of the MSC that allocated it can be used to unambiguously identify the call.

### 7.2.112	Next-Tariff AVP

The Next-Tariff AVP (AVP code 2057) is of type Grouped and holds tariff information. The Tariff is a formula for cost calculation given the Used-Service-Unit AVP. The calculated cost is given in the Currency-Code AVP. 
The formula sums all the rating elements and multiplies the sum by the Scale-Factor AVP.

It has the following ABNF grammar:

Next-Tariff :: =  < AVP Header: 2057 >

[ Currency-Code ]

[ Scale-Factor ]

* [ Rate-Element ]

### 7.2.112aA	NIDD-Submission  AVP

The NIDD-Submission AVP (AVP code 3928) is of type Grouped and holds information about the NIDD submission for CP data transfer.

It has the following ABNF grammar:

NIDD-Submission  :: = 	  < AVP Header: 3928>

[ Submission-Timestamp ]

[ Event-Timestamp ]

[ Accounting-Input-Octets ]

[ Accounting-Output-Octets ]

[ Change-Condition ]

Editor’s Note: AVP for result of NIDD submission is ffs.

### 7.2.112A	NNI-Information AVP

The NNI-Information AVP (AVP code 2703) is of type Grouped and holds information about the NNI used for interconnection and roaming.

It has the following ABNF grammar:

NNI-Information :: = 	  < AVP Header: 2703>

[ Session-Direction ]

[ NNI-Type ]

[ Relationship-Mode ]

[ Neighbour-Node-Address ]

### 7.2.112B	NNI-Type AVP

The NNI-Type AVP (AVP code 2704) is of type Enumerated and indicates whether the type of used NNI is non-roaming, roaming without loopback routing or roaming with loopback routing. It has the following values:

- 0	non-roaming

1	roaming without loopback

2	roaming with loopback

### 7.2.113	Node-Functionality AVP

The Node-Functionality AVP (AVP code 862) is of type Enumerated and includes the functionality identifier of the node. The functionality identifier can be one of the following:

0	S-CSCF
1	P-CSCF
2	I-CSCF
3	MRFC	
4	MGCF	
5	BGCF
6	AS
7	IBCF
8	S-GW
9	P-GW

10	HSGW
11	E-CSCF

12	MME

13	TRF

14	TF

15	ATCF

16	Proxy Function

17	ePDG

18	TDF
19	TWAG

20	SCEF

21	IWK-SCEF

### 7.2.114	Node-Id AVP

The Node-Id AVP (AVP code 2064) is of type UTF8String and includes an optional, operator configurable identifier string for the node.

### 7.2.115	Number-Of-Diversions AVP

The Number-of-Diversions AVP (AVP Code 2034) is of type Unsigned32 and holds the number of diversions related to a CDIV service. When counting the number of diversions, all types of diversion are included.

### 7.2.116	Number-Of-Messages-Sent AVP

The Number-Of-Messages-Sent AVP (AVP code 2019) is of type Unsigned32 and indicates the number of SMSs sent by the IMS application, if applicable. It contains the total number of SMS when concatenated short message for SMS Offline Charging.

### 7.2.117	Number-Of-Participants AVP

The Number-Of-Participants AVP (AVP code 885) is of type Unsigned32 and holds the number of invited parties of the multi-party session when included in the initial charging request message, e.g. in PoC, CONFerence and SIMPLE IM. When included in interim / update charging messages, it indicates the number of parties who are currently attached in the session at the time the interim / update messages are sent.

NOTE:	The information to populate this field may be obtained from the TBCP-Talk-Burst-Grant message in PoC case. The information to populate this field may be obtained from the Diameter Accounting Request message in MMTel CONF Charging.

### 7.2.118	Number-Of-Received-Talk-Bursts AVP

The Number-Of-Received-Talk-Bursts AVP (AVP code 1282) is of type Unsigned32 and holds the number of the received talk bursts.

### 7.2.119	Number-Of-Talk-Bursts AVP

The Number-Of-Talk-Bursts AVP (AVP code 1283) is of type Unsigned32 and holds the number of the sent talk bursts.

### 7.2.120	Number-Portability-Routing-Information AVP

The Number-Portability-Routing-Information AVP (AVP code 2024) is of type UTF8String and holds information on routing number received by the node (e.g. S-CSCF) performing number portability look-up (ENUM/DNS). This information is also sent over SIP in the Requested URI header enabling other IMS nodes receiving this header to also report this information.

### 7.2.121	Offline-Charging AVP

The Offline-Charging AVP (AVP code 1278) is a grouped AVP, which is used to set the parameters required to control offline charging.

It has the following ABNF grammar:

Offline-Charging ::=     < AVP Header:1278 >

[ Quota-Consumption-Time ]
		[ Time-Quota-Mechanism ]

[ Envelope-Reporting ]

*	[ Multiple-Services-Credit-Control ]

*	[ AVP ]

At most one of Quota-Consumption-Time AVP or Time-Quota-Mechanism AVP shall be present, if individual instances are not included within the Multiple-Services-Credit-Control AVP.

The Multiple-Services-Credit-Control AVPs, if present, shall contain the Rating-Group AVP to identify the category, optionally one of Quota-Consumption-Time AVP and Time-Quota-Mechanism AVP, and optionally the Envelope-Reporting AVP.

Any values specified in the Offline-Charging AVP take precedence over the configured defaults. The values of the parameters specified at Multiple-Services-Credit-Control level take precedence over the values specified directly at Offline-Charging level. If neither Quota-Consumption-Time AVP nor Time-Quota-Mechanism AVP is included in the Multiple-Services-Credit-Control AVP, then the general reporting requirements dictated by the Quota-Consumption-Time AVP or Time-Quota-Mechanism AVP and Envelope-Reporting AVP directly within the Offline-Charging AVP shall apply.

### 7.2.122	Online-Charging-Flag AVP

The Online-Charging-Flag AVP (AVP code 2303) is of type Enumerated and indicates the Online Charging Request was sent based on the provided ECF address from the SIP P-header "P-Charging-Function-Addresses". It has the following values:

0	ECF address not provided

1	ECF address provided

### 7.2.123	Originating-IOI AVP

The Originating-IOI AVP (AVP code 839) is of type UTF8String (alphanumeric string) and holds the Inter Operator Identifier (IOI) for the originating network as generated by the IMS Network Element which takes responsibility for populating this parameter  in a SIP request as described in RFC 7315 [404] and TS 24.229 [202]. The Originating IOI contains the following values:

- Type 1 IOI

-	IOI of the visited network where the P-CSCF is located for SIP requests directed to the S-CSCF when initiated by the served user

-	IOI of the home network where the S-CSCF is located for SIP requests directed to the TRF in the visited network when "VPLMN routing" is applied in a Roaming Architecture for Voice over IMS with Local breakout.

-	IOI of the home network where the S-CSCF is located for SIP requests directed to the visited network P-CSCF when terminated at the served user..

- Type 2 IOI

-	IOI of the home network of the originating end user where the S-CSCF is located in case a session is initiated from the IMS. In case of redirection by the S-CSCF, Originating-IOI AVP indicates the terminating party's network operator from which the session is redirected.

-	IOI of the visited network when "VPLMN routing" is applied in a Roaming Architecture for Voice over IMS with Local breakout.

-	IOI of the originating network where the MGCF is located in case a session is initiated from the PSTN toward the IMS.

- Type 3 IOI

-	IOI of the home network (originating side or terminating side) where the S-CSCF is located when forwarding a SIP request as described in TS 24.229 [202] to an AS (proxy, terminating UA or redirect server or B2BUA).

-	IOI of the service provider network where the AS is located when an AS (originating UA or B2BUA) initiates a SIP request as described in TS 24.229 [202].

For further details on the Type 1, Type 2 and Type 3 IOIs, please refer to TS 32.240 [1].

### 7.2.124	Originator AVP

The Originator AVP (AVP code 864) is of type Enumerated and indicates the originating party of the message body. The following values are defined:

0	Calling Party

1	Called Party

### 7.2.125	Originator-Address AVP

The Originator-Address AVP (AVP code 886) is of type Grouped. Its purpose is to identify the originator of a message.

It has the following ABNF grammar:

Originator-Address :: = < AVP Header: 886 >

[ Address-Type ]

[ Address-Data ]

[ Address-Domain ]

### 7.2.126	Originator-Interface AVP

The Originator-Interface AVP (AVP code 2009) is the group AVP which contains information related to the Interface on which the message originated.

It has the following ABNF grammar:

Originator-Interface ::= < AVP Header: 2009 >

[ Interface-Id ]

[ Interface-Text ]

[ Interface-Port ]

[ Interface-Type ]

### 7.2.127	Originator-Received-Address AVP

The Originator-Received-Address AVP (AVP code 2027) is of type Grouped. Its purpose is to identify the originator of a message with the original, unmodified address information as received before any address manipulations has taken place in the entity generating the charging information. This field allows correlation of address information with information generated by other nodes  in the message flow.

It has the following ABNF grammar:

Originator-Received-Address :: = < AVP Header: 2027>

[ Address-Type ]

[ Address-Data ]

[ Address-Domain ]

### 7.2.128	Originator-SCCP-Address

The Originator-SCCP-Address AVP (AVP code 2008) is of type Address. It is the "SCCP calling address" used by the messaging node when receiving a message. This is usually the address of the MSC or SGSN/Serving Node that was serving the UE when it submitted the message. It contains either a Point Code (ISPC) or a Global Title, where Global Title represents an E.164 number. The Address Type discriminator in RFC 6733 [401] is set to value 8, E.164, and the address information is UTF8 encoded.

### 7.2.128A	Outgoing-Session-Id AVP

The Outgoing-Session-Id AVP (AVP code 2320) is of type UTF8String and holds the outgoing session identifier for an AS acting as B2BUA. For a SIP session the Outgoing-Session-Id AVP contains the SIP Call ID of the outgoing leg, as defined in RFC 3261 [405].

### 7.2.129	Outgoing-Trunk-Group-ID AVP

The Outgoing-Trunk-Group-ID AVP (AVP code 853) is of type UTF8String and identifies the outgoing PSTN leg.

### 7.2.130	Participants-Involved AVP

The Participants-Involved AVP (AVP code 887) is of type UTF8String and holds the list of address (Public User ID: SIP URI, Tel URI, MSISDN) of the parties who are involved into the PoC session.

### 7.2.131	Participant-Group AVP

The Participant-Group AVP (AVP code 1260) is of type Grouped and holds detailed information, e.g. the address (Public User ID: SIP URI, Tel URI, MSISDN), the access priority parameters, etc, of the party who is involved into the PoC session.

It has the following ABNF grammar:

- < Participant-Group > ::  =	< AVP Header: 1260>

[ Called-Party-Address ]

[ Participant-Access-Priority ]

[ User-Participating-Type ]

### 7.2.132	Participant-Access-Priority AVP

Participant-Access-Priority AVP (AVP code 1259) is of type Enumerated. It is a subfield of Participants-Group AVP to indicate the priority level for users when initiating a new PoC session or participating in a PoC session. The AVP may take the values as follows:

1	Pre-emptive priority:

The highest level priority. A request with pre-emptive priority SHALL cause the current other requests to be revoked immediately, unless they are also with pre-emptive priority

2	High priority: Lower than Pre-emptive priority

3	Normal priority: Normal level. Lower than High priority

4	Low priority: Lowest level priority

### 7.2.133	Participant-Action-Type AVP

The Participant-Action-Type AVP (AVP code 2049) is of type Enumerated and holds the participant's action type during the conference for Billing Domain's information. The following values are defined according to TS 24.605 [219]:

0	CREATE_CONF

1	JOIN_CONF

2	INVITE_INTO_CONF

3	QUIT_CONF

### 7.2.134	Void

### 7.2.135	Void

### 7.2.135A	PC3-Control-Protocol-Cause AVP

The PC3-Control-Protocol-Cause AVP (AVP code 3434) is of type Integer32 and holds the particular reason why a DISCOVERY_REQUEST or MATCH_REPORT messages from the UE have been rejected by the ProSe Function. It is referred to as "PC3 Control Protocol cause value” in TS 24.334 [236].

### 7.2.135B	PC3-EPC-Control-Protocol-Cause AVP

The PC3-EPC-Control-Protocol-Cause AVP (AVP code 3435) is of type Integer32 and holds the particular reason why a proximity request messages from the UE have been rejected by the ProSe Function. It is referred to as "PC3 EPC Control Protocol cause value” in TS 24.334 [236].

### 7.2.136	PDN-Connection-Charging-ID AVP

The PDN-Connection-Charging-ID AVP (AVP code 2050) is of type Unsigned32 and contains the charging identifier to identify different records belonging to same PDN connection. For a PDN connection that is limited to use a single access at a time this field includes Charging Id of first IP-CAN bearer activated. Together with P-GW address this uniquely identifies the PDN connection for charging.

Coding of this AVP is same as 3GPP-Charging-Id coding described in TS 29.061 [207].

### 7.2.137	PDP-Address AVP

The PDP-Address AVP (AVP code 1227) is of type Address and holds the IP-address associated with the IP CAN bearer session (PDP context / PDN connection). The PDP-Address-Prefix-Length AVP needs not be available for IPv6 typed IP-address prefix length of 64 bits.

### 7.2.137A	PDP-Address-Prefix-Length AVP

The PDP-Address-Prefix-Length AVP (AVP code 2606) is of type Unsigned32 and contains the prefix length of an IPv6 typed PDP-Address AVP. The omission of this AVP for an IPv6 typed IP address implicitly means prefix length of 64 bits, as in this case the 64 bit prefix length default shall be assumed.

### 7.2.138	PDP-Context-Type AVP

The PDP-Context-Type AVP (AVP code 1247) is of type Enumerated and indicates the type of a PDP context. The values for  requested are:

0	Primary

1	Secondary

This AVP shall only be present in the CCR[Initial].

### 7.2.138A	Play-Alternative AVP

The Play-Alternative AVP (AVP code 3913) is of type Enumerated and indicates the call party toward whom an announcement is directed.

It has the following values:

0	served party

1	remote party

### 7.2.139	PoC-Change-Condition AVP

The PoC-Change-Condition AVP (AVP code 1261) is of type Enumerated and contains the reason for closing a container and the addition of a new container. The AVP may take the following values:

0	ServiceChange

1	VolumeLimit

2	TimeLimit

3	NumberofTalkBurstLimit

4	NumberofActiveParticipants

5	TariffTime

### 7.2.140	PoC-Change-Time AVP

The PoC-Change-Time AVP (AVP code 1262) is of type Time and is a time stamp that defines the moment when a container is closed or the CDR is closed.

### 7.2.141	PoC-Controlling-Address AVP

The PoC-Controlling-Address AVP (AVP code 858) is of type UTF8String and identifies the PoC server performing the controlling function for the associated PoC session.

### 7.2.142	PoC-Event-Type AVP

The PoC-Event-Type AVP (AVP code 2025) is of type Enumerated and indicates PoC session unrelated charging event. The AVP may take the values as follows:

0 		Normal;

1 		Instant Ppersonal Aalert event;

2 		PoC Group Advertisement event;

3 		Early Ssession Setting-up event;

4 		PoC Talk Burst

### 7.2.143	PoC-Group-Name AVP

The PoC-Group-Name AVP (AVP code 859) is of type UTF8String and identifies a group. Included if the session is a pre-arranged group session or a chat group session. It can be used for PoC and OMA SIMPE IM Charging, or other applications.

### 7.2.144	PoC-Information AVP

The PoC-Information AVP (AVP code 879) is of type Grouped. Its purpose is to allow the transmission of additional PoC service specific information elements.

It has the following ABNF grammar:

PoC-Information :: = 		  < AVP Header: 879>

[ PoC-Server-Role ]
[ PoC-Session-Type ]
[ PoC-User-Role ]
[ PoC-Session-Initiation-type ]
[ PoC-Event-Type ]
[ Number-Of-Participants ]

* [ Participants-Involved ]

* [ Participant-Group ]
* [ Talk-Burst-Exchange ]

[ PoC-Controlling-Address ] 
[ PoC-Group-Name ]
[ PoC-Session-Id ] 
[ Charged-Party ]

NOTE:	In the ABNF definition of PoC-Information AVP, the Participants-Involved AVP is kept only for backward compatibility with Releases before the 3GPP Release 7.

### 7.2.145	PoC-Server-Role AVP

The PoC-Server-Role AVP (AVP code 883) is of type Enumerated and specifies the role of the PoC server. The identifier can be one of the following:

0	Participating PoC Server

1	Controlling PoC Server

### 7.2.146	PoC-Session-Id AVP

The PoC-Session-Id AVP (AVP code 1229) is of type UTF8String. It uniquely identifies an end-to-end PoC session and may be used for correlation between charging information generated by participating and controlling PoC functions. This information is obtained from the "Contact" header of the SIP message received from the controlling PoC function.

NOTE: 	The PoC-Session-Id may not be available in the initial charging interactions for the PoC session.

### 7.2.147	PoC-Session-Initiation-Type AVP

The PoC-Session-Initiation-Type AVP (AVP code 1277) is of type Enumerated and specifies the type of the PoC session initiation. The identifier can be one of the following:

0	Pre-established

1	On-demand

### 7.2.148	PoC-Session-Type AVP

The PoC-Session-Type AVP (AVP code 884) is of type Enumerated and specifies the type of the PoC session. The identifier can be one of the following, refer Appendix C.5.1 in OMA PoC Control Plane specification [211]:

0	1 to 1 PoC session

1	Chat PoC group session

2	Pre-arranged PoC group session

3	Ad-hoc PoC group session

### 7.2.149	PoC-User-Role AVP

The PoC-User-Role AVP (AVP code 1252) is of type Grouped. It specifies the role(s) related information of the PoC User that participating in the PoC Session.

It has the following grammar:

- PoC-User-Role ::  =  <AVP header: 1252>

[ PoC-User-Role-Ids ]
	[ PoC-User-Role-info-Units ]

### 7.2.150	PoC-User-Role-IDs AVP

The PoC-User-Role-IDs AVP (AVP code 1253) is of type UTF8String and identifies the PoC user role.

### 7.2.151	PoC-User-Role-Info-Units AVP

The PoC-User-Role-Info-Units (AVP code 1254) is of type Enumerated and specify the role type details of PoC users. The identifier can be one of the following:

1	Moderator

2	Dispatcher

3	Session-Owner

4	Session-Participant

### 7.2.152	Positioning-Data AVP

The Positioning-Data AVP (AVP code 1245) is of type UTF8String and indicates the usage of each positioning method that was attempted to determine the location estimate either successfully or unsuccessfully.

### 7.2.153	Preferred-AoC-Currency AVP

The Preferred-AoC-Currency AVP (AVP code 2315) is of type Unsigned32 and indicates the preferred currency code that the AoC function would like to get the AoC information. It is specified by using the numeric values defined in the ISO 4217 standard, refer RFC 4006 [402].

### 7.2.154	Priority AVP

The Priority AVP (AVP code 1209) is of type Enumerated and the priority (importance) of the message if specified. For SMS Charging the value "low" is not applicable. The values are:

0	Low

1	Normal

2	High

### 7.2.154aA	Privacy-Indicator AVP

The Privacy-Indicator AVP (AVP code 3915) is of type Enumerated. It is used to indicate whether the announcement is considered private and is only to be played to the requested party. It has the following values:

0	NOT_PRIVATE

1	PRIVATE

### 7.2.154A	ProSe-3rd-Party-Application-ID AVP

The ProSe-3rd-Party-Application-ID AVP (AVP code  3440) is of type UTF8String and carry A globally unique identifier identifying a specific 3rd party application, as upper layer of ProSe. It is referred to as "Applicatoin Identity” in TS 24.334 [236].

### 7.2.154Aa	ProSe-Direct-Communication-Reception-Data-Container AVP

The ProSe-Direct-Communication-Reception-Data-Container AVP (AVP code 3461) is of type Grouped. Its purpose is to allow the transmission of the container to be reported for ProSe Charging. On encountering change on ProSe charging condition, this container  identifies the volume count for receiving within a ProSe group communication.

It has the following ABNF grammar:

ProSe-Direct-Communication-Reception-Data-Container :: =  < AVP Header: 3461>

[ Local-Sequence-Number ]

[ Coverage-Status ]

[ 3GPP-User-Location-Info ]

[ Accounting-Input-Octets ]

[ Change-Time ]

[ Change-Condition ]

[ Visited-PLMN-Id ]

[ Usage-Information-Report-Sequence-Number ]

[ Radio-Resources-Indicator]

[ Radio-Frequency ]

### 7.2.154B	ProSe-Direct-Communication-Transmission-Data-Container AVP

The ProSe-Direct-Communication- Transmission-Data-Container AVP (AVP code 3441) is of type Grouped. Its purpose is to allow the transmission of the container to be reported for ProSe Charging. On encountering change on ProSe charging condition, this container  identifies the volume count for transmitting within a ProSe group communication.

It has the following ABNF grammar:

ProSe-Direct-Communication- Transmission-Data-Container :: =  < AVP Header: 3441>

[ Local-Sequence-Number ]

[ Coverage-status ]

[ 3GPP-User-Location-Info ]

[ Accounting-Output-Octets ]

[ Change-Time ]

[ Change-Condition ]

[ Visited-PLMN-Id ]

[ Usage-Information-Report-Sequence-Number ]

[ Radio-Resources-Indicator ]

[ Radio-Frequency ]

### 7.2.154C	ProSe-Direct-Discovery-Model AVP

The ProSe-Direst-Discovery-Model AVP (AVP code 3442) is of type Enumerated and indicates model of the Direct Discovery used by the UE. It is referred to as "Prose Direct Discovery Model” in TS 23.303 [235].The AVP may take the values as follows:

0 		Model A

1 		Model B

### 7.2.154D	ProSe-Event-Type AVP

The ProSe-Event-Type AVP (AVP code 3443) is of type Enumerated and indicates ProSe  charging event. The AVP may take the values as follows:

0 		Annoucing

1 		Monitoring

2 		Match Report

### 7.2.154E	ProSe-Function-IP-Address AVP

The ProSe-Function-IP-Address AVP (AVP code 3444) is of type Address and holds the IP-address of the ProSe Function.

### 7.2.154F	ProSe-Function-PLMN-Identifier AVP

The ProSe-Function-PLMN-Identifier AVP (AVP code 3457) is of type UTF8String and contains identifier of Prose Function resided PLMN.

### 7.2.154G	ProSe-Functionality AVP

The ProSe-Functionality AVP (AVP code 3445) is of type Enumerated and indicates the ProSe functionality UE is requesting. The AVP may take the values as follows:

0 		Direct discovery

1 		EPC-level discovery

2 		Direct communication

### 7.2.154H	ProSe-Group-IP-Multicast-Address AVP

The ProSe- Group-IP-Multicast-Address AVP (AVP code 3446) is of type Address and holds The IP multicast address to be used for performing ProSe direct communication. It is referred to as "ProSe Group IP multicast address" in TS 23.303 [235].

### 7.2.154I	ProSe-Information AVP

The ProSe-Information AVP (AVP code 3447) is of type Grouped. Its purpose is to allow the transmission of additional ProSe service specific information elements.

It has the following ABNF grammar:

ProSe-Information :: = 	< AVP Header: 3447>

[ Announcing-UE-HPLMN-Identifier ]

[ Announcing-UE-VPLMN-Identifier ]

[ Monitoring-UE-HPLMN-Identifier ]

[ Monitoring-UE-VPLMN-Identifier ]

[ Monitored-HPLMN-Identifier ]

[ Role-Of-ProSe-Function ]

[ ProSe-App-Id ]

[ ProSe-3rd-Party-Application-ID ]

[ Application-Specific-Data ]

[ ProSe-Event-Type ]

[ ProSe-Direct-Discovery-Model ]

[ ProSe-Function-IP-Address ]

[ ProSe-Function-ID ]

[ ProSe-Validity-Timer ]

[ ProSe-Role-Of-UE ]

[ ProSe-Request-Timestamp ] [ PC3-Control-Protocol-Cause ]

[ Monitoring-UE-Identifier ]

[ Prose-Function-PLMN-Identifier ]

[ Requestor-PLMN-Identifier ]

[ Origin-App-Layer-User-Id ]

[ WLAN-Link-Layer-Id ]

[ Requesting-EPUID ]

[ Target-App-Layer-User-Id ]

[ Requested-PLMN-Identifier ]

[ Time-Window ]

[ ProSe-Range-Class ]

[ Proximity-Alert-Indication ]

[ Proximity-Alert-Timestamp ]

[ Proximity-Cancellation-Timestamp ]

[ ProSe-Reason-For-Cancellation ]

[ PC3-EPC-Control-Protocol-Cause ]

[ ProSe-UE-ID ]

[ ProSe-Source-IP-Address ]

[ Layer-2-Group-ID ]

[ ProSe-Group-IP-Multicast-Address ]

* [ Coverage-Info ]

* [ Radio-Parameter-Set-Info ]

* [ Transmitter-Info ]

[ Time-First-Transmission ]

[ Time-First-Reception ]

* [ ProSe-Direct-Communication-Transmission-Data-Container ]

* [ ProSe-Direct-Communication-Reception-Data-Container ]

[ Announcing-PLMN-ID]

[ ProSe-Target-Layer-2-ID ]

[ Relay-IP-address ]

[ ProSe-UE-to-Network-Relay-UE-ID ]

[ Target-IP-Address ]

### 7.2.154J	ProSe-Range-Class AVP

The ProSe-Range-Class AVP (AVP code 3448) is of type Enumerated and carry a range class for a specific proximity request. The AVP may take the values as follows:

0	Reserved

1	50 m

2	100 m

3	200 m

4	500 m

5	1000 m

6-255	Unused

It is referred to as "Range Class" in TS 24.334 [236].

### 7.2.154K	ProSe-Reason-For-Cancellation AVP

The ProSe-Reason-For-Cancellation AVP (AVP code 3449) is of type Enumerated and indicates the reason for cancellation of an EPC-level discovery request. The AVP may take the values as follows:

0 		Proximity Alert sent

1 		Time expired with no renewal

2 		Requestor cancellation

### 7.2.154L	ProSe-Request-Timestamp AVP

The ProSe-Request-Timestamp AVP (AVP code 3450) is of type Time, and it holds the timestamp when ProSe Request is received from UE.

### 7.2.154M		ProSe-Role-Of-UE AVP

The ProSe- Role-Of-UE AVP (AVP code 3451) is of type Enumerated and indicates Role of the UE using ProSe. The AVP may take the values as follows:

0 		Announcing UE

1 		Monitoring UE

2 		Requestor UE

3 		Requested UE

### 7.2.154N	ProSe-Source-IP-Address AVP

The ProSe-Source-IP-Address AVP (AVP code 3452) is of type Address and holds the IP address UE used as source address for performing ProSe direct communication. It is referred to as "source IP address" in TS 23.303 [235].

### 7.2.154O	ProSe-UE-ID AVP

The ProSe-UE-ID AVP (AVP code 3453) is of type OctetString and carry a link layer identifier assigned by the EPS that uniquely represents the UE in the context of ProSe Direct Communication. It is referred to as "ProSe UE ID" in TS 23.303 [235].

### 7.2.154Oa 	ProSe-UE-to-Network-Relay-UE-ID AVP

The ProSe-UE-to-Network-Relay-UE-ID AVP (AVP code4409) is of type OctetString and holds a link layer identifier that uniquely represents the ProSe UE-to-Network relay UE in the context of ProSe Direct Communication via UE-to-Network Relay.

### 7.2.154Ob 	ProSe-Target-Layer-2-ID AVP

The ProSe-Target-Layer-2-ID AVP (AVP code4410) is of type OctetString and carries the identifier of UE that uniquely represents a specific one-to-one ProSe Direct Communication.

### 7.2.154P	Proximity-Alert-Indication AVP

The Proximity-Alert-Indication AVP (AVP code 3454) is of type Enumerated and indicates whether proximity alert has been sent before proximity request cancellation. The AVP may take the values as follows:

0 		Alert

1 		No Alert

### 7.2.154Q	Proximity-Alert-Timestamp AVP

The Proximity-Alert-Timestamp AVP (AVP code 3455) is of type Time, and it holds the timestamp when proximity alert is sent, to indicate two UEs are in proximity.

### 7.2.154R	Proximity-Cancellation-Timestamp AVP

The Proximity-Cancellation-Timestamp AVP (AVP code 3456) is of type Time, and it holds the timestamp when proximity request cancellation is requested.

### 7.2.155	PS-Append-Free-Format-Data AVP

The PS-Append-Free-Format-Data AVP (AVP code 867) is of type enumerated and indicates if the information sent in the PS-Free-Format-Data AVP shall be appended to the PS-free-format-data stored for the online-session. The following values are defined:

0	'Append' 
If this AVP is present and indicates 'Append', the P-GW shall append the received PS free format data to the PS free format data stored for the online charging session.

1	'Overwrite' 
If this AVP is absent or in value 'Overwrite', the P-GW shall overwrite all PS free format data already stored for the online charging session.

The P-GW shall ignore this AVP if no PS free format data is stored for the online charging session.

### 7.2.156	PS-Free-Format-Data AVP

The PS-Free-Format-Data AVP (AVP code 866) is of type OctectString and holds online charging session specific data.

### 7.2.157	PS-Furnish-Charging-Information AVP

The PS-Furnish-Charging-Information AVP (AVP code 865) is of type Grouped. Its purpose is to add online charging session specific information, received via the Ro reference point, onto the Rf reference point in order to facilitate its inclusion in CDRs. This information element may be received in a CCA message via the Ro reference point. In situations where online and offline charging are active in parallel, the information element is transparently copied into an ACR to be sent on the Rf reference point.

It has the following ABNF grammar:

PS-Furnish-Charging-Information :: = < AVP Header: 865>

{ 3GPP-Charging-Id }
{ PS-Free-Format-Data }
[ PS-Append-Free-Format-Data ]

### 7.2.158	PS-Information AVP

The PS-Information AVP (AVP code 874) is of type Grouped. Its purpose is to allow the transmission of additional PS service specific information elements.

It has the following ABNF grammar:

PS-Information :: = 	< AVP Header: 874>

[ 3GPP-Charging-Id ]

[ PDN-Connection-Charging-ID ]
   [ Node-Id ]
   [ 3GPP-PDP-Type ]
* [ PDP-Address ]
   [ PDP-Address-Prefix-Length ]
   [ Dynamic-Address-Flag ]
   [ Dynamic-Address-Flag-Extension ]
   [ QoS-Information ]
 * [ SGSN-Address ]
 * [ GGSN-Address ]

* [ TDF-IP-Address ]

* [ SGW-Address ]

* [ ePDG-Address ]

* [ TWAG-Address ]
   [ CG-Address ]
   [ Serving-Node-Type ]
   [ SGW-Change ]
   [ 3GPP-IMSI-MCC-MNC ]
   [ IMSI-Unauthenticated-Flag ]
   [ 3GPP-GGSN-MCC-MNC ]
   [ 3GPP-NSAPI ]
   [ Called-Station-Id ]
   [ 3GPP-Session-Stop-Indicator ]
   [ 3GPP-Selection-Mode ]
   [ 3GPP-Charging-Characteristics ]
   [ Charging-Characteristics-Selection-Mode ]
   [ 3GPP-SGSN-MCC-MNC ]
   [ 3GPP-MS-TimeZone ]

[ Charging-Rule-Base-Name ] 
   [ ADC-Rule-Base-Name ]

[ 3GPP-User-Location-Info ]

[ User-Location-Info-Time ]

[ User-CSG-Information ]

* [ Presence-Reporting-Area-Information ]

[ 3GPP2-BSID ]

[ TWAN-User-Location-Info ]
	[ UWAN-User-Location-Info ]
   [ 3GPP-RAT-Type ]
   [ PS-Furnish-Charging-Information ]
   [ PDP-Context-Type ] 
   [ Offline-Charging ]

* [ Traffic-Data-Volumes ]

* [ Service-Data-Container ]

[ User-Equipment-Info ]

[ Terminal-Information ]

[ Start-Time ]

[ Stop-Time ]

[ Change-Condition ]

[ Diagnostics ]

[ Low-Priority-Indicator ]

[ NBIFOM-Mode ]

[ NBIFOM-Support ]

[ MME-Number-for-MT-SMS ]

[ MME-Name ]

[ MME-Realm ] 
	[ Logical-Access-ID ]
	[ Physical-Access-ID ]

[ Fixed-User-Location-Info ]

[ CN-Operator-Selection-Entity ]

[ Enhanced-Diagnostics ]

[ SGi-PtP-Tunnelling-Method ]

[ CP-CIoT-EPS-Optimisation-Indicator ]

[ UNI-PDU-CP-Only-Flag ]

[ Serving-PLMN-Rate-Control ]

[ APN-Rate-Control ]

[ Charging-Per-IP-CAN-Session-Indicator ]

[ RRC-Cause-Counter ]

[ 3GPP-PS-Data-Off-Status ]

[ SCS-AS-Address ]

[ Unused-Quota-Timer ]

### 7.2.159	Quota-Consumption-Time AVP

The Quota-Consumption-Time AVP (AVP code 881) is of type Unsigned32 and contains an idle traffic threshold time in seconds. This AVP may be included within the Multiple-Services-Credit-Control AVP when this AVP also contains a Granted-Service-Units AVP containing a CC-Time AVP (i.e. when the granted quota is a time quota).

### 7.2.160	Quota-Holding-Time AVP

The Quota-Holding-Time AVP (AVP code 871) is of type Unsigned32 and contains the quota holding time in seconds.  The client shall start the quota holding timer when quota consumption ceases.  This is always when traffic ceases, i.e. the timer is re-started at the end of each packet. The Credit-Control Client shall deem a quota to have expired when no traffic associated with the quota is observed for the value indicated by this AVP. The timer is stopped on sending a CCR and re-initialised on receiving a CCA with the previous used value or a new value of Quota-Holding-Time if received.

This optional AVP may only occur in a CCA command.  It is contained in the Multiple-Services-Credit-Control AVP.  It applies equally to the granted time quota and to the granted volume quota.

A Quota-Holding-Time value of zero indicates that this mechanism shall not be used. If the Quota-Holding-Time AVP is not present, then a locally configurable default value in the client shall be used.

### 7.2.160aA	Quota-Indicator AVP

The Quota-Indicator AVP (AVP code 3912) is of type Enumerated. It is used to indicate whether granted quota is to be used during announcement setup and playback. It has the following values:

0	QUOTA_IS_NOT_USED_DURING_PLAYBACK

1	QUOTA_IS_USED_DURING_PLAYBACK

### 7.2.160A	Radio-Frequency AVP

The Radio-Frequency AVP (AVP code 3462) is of type OctetString and identifies the radio frequency used for ProSe direct communication. The format of the value is according to the ARFCN-ValueEUTRA-r9 ASN.1 data type described in TS 36.331 [241].

### 7.2.160B	Radio-Parameter-Set-Info AVP

The Radio-Parameter-Set-Info AVP (AVP code 3463) is of type Grouped and provides information on a radio parameter set configured in the UE for direct communication use. Each set has an associated time stamp of when it became active.

It has the following ABNF grammar:

Radio-Parameter-Set-Info :: = < AVP Header: 3463>

[ Radio-Parameter-Set-Values ]

[ Change-Time ]

### 7.2.160C	Radio-Parameter-Set-Values AVP

The Radio-Parameter-Set-Values AVP (AVP code 3464) is of type OctetString and provides the radio parameter set configured in the UE for direct communication. The format of the value is according to the SL-Preconfiguration-r12 ASN.1 data type described in TS 36.331 [241].

### 7.2.160D	Radio-Resources-Indicator AVP

The Radio-Resources-Indicator AVP (AVP code 3465) is of type Integer32 and identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direct communication. The allowed values are:

1	Operator-provided

2	Configured

### 7.2.160E			Rate-Control-Max-Message-Size AVP

The Rate-Control-Max-Message-Size AVP (AVP code 3937) is of type Unsigned32 and holds the maximum data PDU size in octets.

### 7.2.160F			Rate-Control-Max-Rate AVP

The Rate-Control-Max-Rate AVP (AVP code 3938) is of type Unsigned32 and holds the maximum number of data PDUs per time unit indicated in Rate-Control-Time-Unit AVP.

### 7.2.160G	Rate-Control-Time-Unit AVP

The Rate-Control-Time-Unit AVP (AVP code 3939) is of type Unsigned32 and holds a time unit for rate control.It has the following values:

0	Unrestricted

This value is used to indicate the number of data PDUs is not restricted.

1	Minute

2	Hour

3	Day

4	Week

### 7.2.161	Rate-Element AVP

The Rate-Element AVP (AVP code 2058) is of type Grouped and holds simple rate element of one dimension. Possible dimensions are the CC-Unit-Type.

Example: CC-Unit-Type AVP TIME, Unit-Value AVP 6 and Unit-Cost AVP 10 with Exponent AVP 2 should read: 
10 cents per 6 seconds time. The currency is context dependent.

IF CC-Unit-Type AVP is MONEY, this is a fixed fee and Unit-Value is ignored.

It has the following ABNF grammar:

Rate-Element :: =  < AVP Header: 2058 >

{ CC-Unit-Type }

[ Charge- Reason-Code ]

[ Unit-Value ]

[ Unit-Cost ]

[ Unit-Quota-Threshold ]

### 7.2.162	Read-Reply-Report-Requested AVP

The Read-Reply-Report-Requested AVP (AVP code 1222) is of type Enumerated and indicates whether a read reply report has been requested by the originator MMS User Agent or not. The values for whether a report was requested are:

0	No

1	Yes

### 7.2.163	Void

### 7.2.164	Real-Time-Tariff-Information AVP

The Real-Time-Tariff-Information AVP (AVP code 2305) is of type Grouped and holds the tariff information received in real time according to TS 29.658 [222] in the SIP signalling.

It has the following ABNF grammar:

- < Real-Time-Tariff-Information > :: =  < AVP Header: 2305 >

[ Tariff-Information ]

[ Tariff-XML ]

### 7.2.164A	Reason-Header AVP

The Reason-Header AVP (AVP code 3401) is of type UTF8String and contains the content of the Reason-header in the SIP BYE and CANCEL. It may contain multiple entries if there are multiple Reason headers within a SIP BYE or CANCEL.

### 7.2.165	Received-Talk-Burst-Time AVP

The Received-Talk-Burst-Time AVP (AVP code 1284) is of  type Unsigned32 and holds the duration in seconds of  the received talk bursts.

### 7.2.166	Received-Talk-Burst-Volume AVP

The Received-Talk-Burst-Volume AVP (AVP code 1285) is of type Unsigned32 and holds the volume in bytes of  the received talk bursts.

### 7.2.167	Recipient-Address AVP

The Recipient-Address AVP (AVP code 1201) is of type Grouped. Its purpose is to identify the recipient of a message.

It has the following ABNF grammar:

Recipient-Address :: = 	< AVP Header: 1201 >

[ Address-Type ]

[ Address-Data ]

[ Address-Domain ]

[ Addressee-Type ]

### 7.2.168	Recipient-Info AVP

The Recipient-Info AVP (AVP code 2026) is of type Grouped. Its purpose is to group information associated with a recipient. and contains the list of Recipient addresses of the message.

It has the following ABNF grammar:

Recipient-Info :: = 	< AVP Header: 2026 >

[ Destination-Interface ]

* [ Recipient-Address ]

* [ Recipient-Received-Address ]

[ Recipient-SCCP-Address ]

[ SM-Protocol-ID ]

NOTE 1: This Recipient-Info AVP allows charging for messages with multiple recipients by repeating this AVP for every recipient. The Recipient-Info AVP unambigiously associates the grouped information to one specific recipient.

NOTE 2: The SM-Protocol-ID AVP only relates to the recipient when charging MT SMS messages as specified in TS 23.040 [216].

### 7.2.169	Recipient-Received-Address AVP

The Recipient-Received-Address AVP (AVP code 2028) is of type Grouped. Its purpose is to identify the recipient of a message with the original, unmodified address information as received before any address manipulations has taken place in the entity generating the charging information. This field allows correlation of address information with information generated by other nodes  in the message flow.

It has the following ABNF grammar:

Recipient-Received-Address :: = < AVP Header: 2028>

[ Address-Type ]

[ Address-Data ]

[ Address-Domain ]

### 7.2.170	Recipient-SCCP-Address

The Recipient-SCCP-Address AVP (AVP code 2010) is of type Address. It is the "SCCP called address" used by the messaging node when delivering the message. This is usually the address of the MSC or SGSN/Serving Node that is serving the UE when it delivers the message. It contains a Global Title, where Global Title represents an E.164 number, and possibly a Point Code (ISPC). The AddressType discriminator in RFC 6733 [401] is set to value 8, E.164, and the address information is UTF8 encoded.

### 7.2.171	Refund-Information AVP

The Refund-Information AVP (AVP code 2022) is of type OctetString and it conveys relevant information for the OCS application relative to refund mechanism. When refund mechanism is implemented in the OCS this AVP may be included in the CCA of the previous IEC. This AVP shall be used by the CTF in case of a refund scenario and thus shall be included in the CCR for refund if previously received in CCA for IEC.

### 7.2.171A	Relationship-Mode AVP

The Relationship-Mode AVP (AVP code 2706) is of type Enumerated and indicates whether the other functional entity (e.g. contact point of the neighbouring network) is regarded as part of the same trust domain. It has the following values:

0	trusted

1	non-trusted

### 7.2.171Aa	Related-Change-Condition-Information AVP

The Related-Change-Condition-Information AVP (AVP code 3925) is of type Grouped. Its purpose is to allow the transmission of the change condition information that occurred on another access of a multi-access PDN connection that triggered an indirect change condition of the current access. In additional to the identification of the change condition(s) that occurred on the other access, the supplemental data for that event is also included (e.g., user location information).

It has the following ABNF grammar:

Related- Change-Condition- Information :: =  < AVP Header: 3925>

[ SGSN-Address ]

*[ Change-Condition]

[ 3GPP-User-Location-Info ]

[ 3GPP2-BSID ]

[ UWAN-User-Location-Info ]

[ Presence-Reporting-Area-Status ]

[ User-CSG-Information ]

[ 3GPP-RAT-Type ]

### 7.2.171Ab	Related-Trigger AVP

The Related-Trigger AVP (AVP code 3926) is of type Grouped and holds the trigger types for a related trigger for another access in a multi-access PDN connection. This AVP is only included when the Trigger AVP contains a Trigger-Type AVP with value of "indirect change condition" which is applicable charging per IP can session for a multi-accessPDN connection.

It has the following ABNF grammar:

- Related-Trigger  :: = < AVP Header: 3926 >

* [ Trigger-Type ]

### 7.2.171B	Related-IMS-Charging-Identifier AVP

The Related-IMS-Charging-Identifier AVP (AVP code 2711) is of type UTF8String and holds the Related IMS Charging Identifier (ICID) as generated by the Enhanced MSC Server or the P-CSCF for the target access leg of an SRVCC access transfer.

### 7.2.171C	Related-IMS-Charging-Identifier-Node AVP

The Related-IMS-Charging-Identifier-Node AVP (AVP code 2712) is of type Address and holds the identifier of the Enhanced MSC Server or the P-CSCF that generated the Related IMS Charging Identifier (ICID).

### 7.2.171D	Relay-IP-address AVP

The Relay-IP-address AVP (AVP code 4411) is of type Address and holds the ProSe UE-to-Network Relay UE IP address for performing ProSe Direct Communication via UE-to-Network.

### 7.2.172	Remaining-Balance AVP

The Remaining-Balance AVP (AVP code 2021) is of type Grouped and provides information about the remaining account balance of the subscriber.

It has the following ABNF grammar:

Remaining-Balance :: = < AVP Header: 2021 >

{ Unit-Value }

{ Currency-Code }

### 7.2.173	Reply-Applic-ID AVP

The Reply-Applic-ID AVP (AVP code 1223) is of type UTF8String and holds the identifier of a "reply path", i.e. the identifier of the application to which delivery reports, read-reply reports and reply-MMs are addressed.

### 7.2.174	Reply-Path-Requested AVP

The Reply-Path-Requested AVP (AVP code 2011) is of type Enumerated. The possible values are:

0	No Reply Path Set

1	Reply path Set

### 7.2.175	Reporting-Reason AVP

The Reporting-Reason AVP (AVP code 872) is of type Enumerated and specifies the reason for usage reporting for one or more types of quota for a particular category.  It can occur directly in the Multiple-Services-Credit-Control AVP, or in the Used-Service-Units AVP within a CCR command reporting credit usage. It shall not be used at command level. It shall always and shall only be sent when usage is being reported. The following values are defined for the Reporting-Reason AVP:

0	THRESHOLD
This value is used to indicate that the reason for usage reporting of the particular quota type indicated in the Used-Service-Units AVP where it appears is that the threshold has been reached.

1	QHT
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that the quota holding time specified in a previous CCA command has been hit (i.e. the quota has been unused for that period of time).

2	FINAL
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that a service termination has happened, e.g. PDP context, IP CAN bearer termination or service data flow termination.

3	QUOTA_EXHAUSTED
This value is used to indicate that the reason for usage reporting of the particular quota type indicated in the Used-Service-Units AVP where it appears is that the quota has been exhausted.

4	VALIDITY_TIME
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that the credit authorization lifetime provided in the Validity-Time AVP has expired.

5	OTHER_QUOTA_TYPE
This value is used to indicate that the reason for usage reporting of the particular quota type indicated in the Used-Service-Units AVP where it appears is that, for a multi-dimensional quota, one reached a trigger condition and the other quota is being reported.

6	RATING_CONDITION_CHANGE
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that a change has happened in some of the rating conditions that were previously armed (through the Trigger AVP, e.g. QoS, Radio Access Technology,…). The specific conditions that have changed are indicated in an associated Trigger AVP.

7	FORCED_REAUTHORISATION
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that it is there has been a Server initiated re-authorization procedure, i.e. receipt of RAR command

8	POOL_EXHAUSTED
This value is used to indicate that the reason for usage reporting of the particular quota type indicated in the Used-Service-Units AVP where it appears is that granted units are still available in the pool but are not sufficient for a rating group using the pool.

9	UNUSED_QUOTA_TIMER 
This value is used to indicate that the reason for usage reporting of all quota types of the Multiple-Service-Credit-Control AVP where its appears is that the unused quota timer has expired.

The values QHT, FINAL, VALIDITY_TIME, FORCED_REAUTHORISATION, RATING_CONDITION_CHANGE, UNUSED_QUOTA_TIMER apply for all quota types and are used directly in the Multiple-Services-Credit-Control AVP, whereas the values THRESHOLD, QUOTA_EXHAUSTED and OTHER_QUOTA_TYPE apply to one particular quota type and shall occur only in the Used-Service-Units AVP. The value POOL_EXHAUSTED apply to all quota types using the credit pool and occurs in the Used-Service-Units AVP. It may optionally occur in the Multiple-Services-Credit-Control AVP if all quota types use the same pool.

When the value RATING_CONDITION_CHANGE is used, the Trigger AVP shall also be included to indicate the specific events which caused the re-authorization request.

### 7.2.176	Requested-Party-Address AVP

The Requested-Party-Address AVP (AVP code 1251) is of type UTF8 String. It holds the address (SIP URI, Tel URI or URN) of the party to whom the call was originaly addressed.

For IMS charging, it holds the address of the party (Public User ID or Public Service ID) to whom the SIP transaction was originally posted. The Requested--Party-Address AVP shall be populated with the SIP URI, Tel URI or URN contained in the Request-URI of the incoming request. This AVP is only present if different from the Called-Party-Address AVP.

For VCS charging, it holds the address (URN is not applicable) which identifies the party to which a voice call is destined before processing by the Proxy Function. It is converted from the circuit-switched Called Party BCD Number as per TS 29.163 [234] for the Request-URI header. This is included only for a mobile originating call.

### 7.2.176A	Requested-PLMN-Identifier AVP

The Requested-PLMN-Identifier AVP (AVP code 3436) is of type UTF8String and contains PLMN identifier of the user who is targeted in proximity request.

### 7.2.176B	Requestor-PLMN-Identifier AVP

The Requestor-PLMN-Identifier AVP (AVP code 3437) is of type UTF8String and contains PLMN identifier of the user who initiate proximity request.

### 7.2.177	Role-Of-Node AVP

The Role-Of-Node AVP (AVP code 829) is of type Enumerated and specifies whether the IMS (except the MRFC) or VCS node is serving the calling, called, or forwarding party. The identifier can be one of the following:

0	ORIGINATING_ROLE

The node is applying an originating role, serving the calling party.

1	TERMINATING_ROLE

The node is applying a terminating role, serving the called party.

2	FORWARDING_ROLE

The node is applying a originating role, serving the forwarding party.

### 7.2.177aA	Role-Of-ProSe-Function AVP

The Role-Of-ProSe-Function AVP (AVP code 3438) is of type Enumerated and indicates ProSe Function resides in which PLMN. The AVP may take the values as follows:

0 		HPLMN

1 		VPLMN

2 		Local PLMN

### 7.2.177A	Route-Header-Received AVP

The Route-Header-Received AVP (AVP code 3403) is of type UTF8String and includes the information in the topmost route header in a received initial SIP INVITE or non-session related SIP MESSAGE request.

### 7.2.177B	Route-Header-Transmitted AVP

The Route-Header-Transmitted AVP (AVP code 3404) is of type UTF8String and includes the information in the route header representing the destination in a transmitted initial SIP INVITE or non-session related SIP MESSAGE request.

### 7.2.178	Scale-Factor AVP

The Scale-Factor AVP (AVP code 2059) is of type Grouped and holds simple multiplication factor in the same format as Unit-Value.

It has the following ABNF grammar:

Scale-Factor :: = < AVP Header: 2059 >

{ Value-Digits }

[ Exponent ]

### 7.2.178A SCS-Address AVP

The SCS-Address AVP (AVP code 3941) is of type Address and holds the IP-address of SCS/AS.

### 7.2.178B SCS-AS-Address AVP

The SCS-AS-Address AVP (AVP code 3940) is of type Grouped and holds the Address of SCS/AS.

It has the following ABNF grammar:

SCS-AS-Address :: = < AVP Header: 3940>

[ SCS-Realm ]

[ SCS-Address ]

### 7.2.178C SCS-Realm AVP

The SCS-Realm AVP (AVP code 3942) is of type DiameterIdentity and holds the Diameter Realm Identity of SCS/AS.

### 7.2.179	SDP-Answer-Timestamp AVP

The SDP-Answer-Timestamp AVP (AVP code 1275) is of type Time and holds the time in UTC format of the response to the SDP offer.

### 7.2.180	SDP-Media-Component AVP

The SDP-Media-Component AVP (AVP code 843) is of type Grouped and contains information about media used for a IMS session.

It has the following ABNF grammar:

SDP-Media-Component ::=	< AVP Header: 843 >

[ SDP-Media-Name ]

* [ SDP-Media-Description ]

[ Local-GW-Inserted-Indication ]

[ IP-Realm-Default-Indication ]

[ Transcoder-Inserted-Indication ]

[ Media-Initiator-Flag ]

[ Media-Initiator-Party ]

[ 3GPP-Charging-Id ]

[ Access-Network-Charging-Identifier-Value ]

[ SDP-Type ]

NOTE:	When populating the SDP-Media-Component, either the 3GPP-Charging-ID or the Access-Network-Charging-Identifier-Value should be present but not both. The 3GPP-Charging-ID is expected to be used for 3GPP defined IP-CANS (e.g. GPRS) while the Access-Network-Charging-Identifier-Value is used for non-3GPP defined IP-CANs.

### 7.2.181	SDP-Media-Description AVP

The SDP-Media-Description AVP (AVP code 845) is of type UTF8String and holds the content of  SDP lines (i=, c=, b=, k=, a=, etc.) related to a media description, as described in RFC 4566 [406]. The attributes are specifying the media described in the SDP-Media-Name AVP.

### 7.2.182	SDP-Media-Name AVP

The SDP-Media-Name AVP (AVP code 844) is of type UTF8String and holds the content of the SDP "m=" line in   a media description, as described in RFC 4566 [406].

### 7.2.183	SDP-Offer-Timestamp AVP

The SDP-Offer-Timestamp AVP (AVP code 1274) is of type Time and holds the time in UTC format of the SDP offer.

### 7.2.184	SDP-Session-Description AVP

The SDP-Session-Description AVP (AVP code 842) is of type UTF8String and holds the content of  the SDP line (i=, c=, b=, k=, a=, etc.) in the session description, as described in RFC 4566 [406].

### 7.2.185	SDP-TimeStamps AVP

The SDP-TimeStamps AVP (AVP code 1273) is of type Grouped and holds the time of the SDP offer and the SDP answer.

It has the following ABNF grammar:

- SDP-TimeStamps :: =  < AVP Header: 1273 >

[ SDP-Offer-Timestamp ]

[ SDP-Answer-Timestamp ]

### 7.2.186	SDP-Type AVP

The SDP-Type AVP (AVP code 2036) is of type Enumerated and holds information if the SDP media component was of type SDP offer or SDP answer. The values are:

0	SDP Offer

1	SDP Answer

### 7.2.186A	Session-Direction AVP

The Session-Direction AVP  (AVP code 2707) is of type Enumerated and indicates whether the NNI is used for an inbound or outbound service request on the control plane in case of interconnection and roaming.

It has the following values:

0	inbound

1	outbound

### 7.2.187	Served-Party-IP-Address AVP

The Served-Party-IP-Address AVP (AVP code 848) is of type Address and holds the IP address of either the calling or called party, depending on whether the P-CSCF is in touch with the calling or the called party. This AVP is only provided by the P-CSCF.

### 7.2.188	Void

### 7.2.189	Service-Data-Container AVP

The Service-Data-Container AVP (AVP code 2040) is of type Grouped. Its purpose is to allow the transmission of the container to be reported for Flow based Charging or application based charging. On encountering change on charging condition in Flow based Charging, this container  identifies the volume count (separated for uplink and downlink), elapsed time or number of events, per service data flow identified per rating group or combination of the rating group and service id within an IP-CAN bearer. On encountering change on charging condition in application based Charging, this container identifies the volume count (separated for uplink and downlink), elapsed time or number of events, per rating group or combination of the rating group and service identifier within a TDF session.

It has the following ABNF grammar:

Service-Data-Container :: =  < AVP Header: 2040>

[ AF-Correlation-Information ]

[ Charging-Rule-Base-Name ]

[ Accounting-Input-Octets ]

[ Accounting-Output-Octets ]

[ Local-Sequence-Number ]

[ QoS-Information ]

[ Rating-Group ]

[ Change-Time ]

[ Service-Identifier ]

[ Service-Specific-Info ]

[ ADC-Rule-Base-Name ]

[ SGSN-Address ]

[ Time-First-Usage ]

[ Time-Last-Usage ]

[ Time-Usage ]

*[ Change-Condition]

[ 3GPP-User-Location-Info ]

[ 3GPP2-BSID ]

[ UWAN-User-Location-Info ]

[ Sponsor-Identity ]

[ Application-Service-Provider-Identity ]

* [ Presence-Reporting-Area-Information]

[ Presence-Reporting-Area-Status ]

[ User-CSG-Information ]

[ 3GPP-RAT-Type ]

[ Related-Change-Condition-Information ]

[ Serving-PLMN-Rate-Control ]

[ APN-Rate-Control ]

[ 3GPP-PS-Data-Off-Status ]

### 7.2.190	Service-ID AVP

The Service-ID AVP (AVP code 855) is of type UTF8String and identifies the service the MRFC is hosting or AoC service. For conferences the conference ID is used as the value of this parameter.

### 7.2.191	Service-Generic-Information AVP

The Service-Generic-Information AVP (AVP code 1256) is of type Grouped. Its purpose is to allow the transmission of service information elements used for all services.

The AVP is defined in OMA-DDS-Charging_Data [223].

### 7.2.192	Service-Information AVP

The Service-Information AVP (AVP code 873) is of type Grouped. Its purpose is to allow the transmission of additional 3GPP service specific information elements which are not described in this document.

It has the following ABNF grammar:

Service-Information :: = 	< AVP Header: 873>

* [ Subscription-Id ]

[ AoC-Information ]

[ PS-Information ]

[ IMS-Information ]

[ MMS-Information ]

[ LCS-Information ]

[ PoC-Information ]

[ MBMS-Information ]

[ SMS-Information ]

[ VCS-Information ]

[ MMTel-Information ]

[ ProSe-Information ]

[ Service-Generic-Information ]

[ IM-Information ]

[ DCD-Information ]

[ M2M-Information ]

[ CPDT-Information ]

The format and the contents of the fields inside the Service-Information AVP are specified in the middle-tier documents which are applicable for the specific service. Note that the formats of the fields are service-specific, i.e. the format will be different for the various services.

The Subscription-Id AVP in Service-Information AVP is only used on the Rf interface.

Further fields may be included in the Service-Information AVP when new services are introduced.

### 7.2.193	Service-Mode AVP

The Service-Mode AVP (AVP Code 2032) is of type Unsigned32 and provides the mode for CDIV, CB and ECT MMTel supplementary services. The following values are defined:

0	Communication Forwarding Unconditional (CFU)

1	Communication Forwarding Busy (CFB)

2	Communication Forwarding No Reply (CRNR)

3	Communication Forwarding on Not Logged-In  (CFNL)

4	Communication Deflection (CD

5	Communication Forwarding on Subscriber Not Reachable (CFNRc)

6	Incoming Call Barring (ICB)

7	Outgoing Call Barring (OCB)

8	Anonymous Communication Rejection (ACR)

9	Blind Transfer

10	Consultative Transfer

11	Three-Party (3PTY)

12	Advice of Charge -S (AoC-S)

13	Advice of Charge -D (AoC-D)

14	Advice of Charge -E (AoC-E)

Values   1024 are reserved for specific Network/Manufacturer variants

### 7.2.194	Service-Specific-Data AVP

The Service-Specific-Data AVP (AVP Code 863) is of type UTF8String and holds the value of the Service-Specific-Data.

### 7.2.195	Service-Specific-Info AVP

The Service-Specific-Info AVP (AVP Code 1249) is of type Grouped and holds service specific data if and as provided by an Application Server, for IMS charging, a PCEF only for pre-defined PCC rules for PS domain charging, or a Proxy function for VCS charging.

It has the following ABNF grammar:

Service-Specific-Info  ::=	   < AVP Header: 1249 >

[ Service-Specific-Data ]
   											[ Service-Specific-Type ]

### 7.2.196	Service-Specific-Type AVP

The Service-Specific-Type AVP (AVP Code 1257) is of type Unsigned32 and holds the type of the Service-Specific-Data.

### 7.2.197	Void

### 7.2.197a 	Serving-Node-Identity

The Serving-Node-Identity AVP (AVP code 3929) is of type DiameterIdentity and holds the identity of the Serving Node (e.g. MME from the SCEF) that was used during a report.

### 7.2.198	Serving-Node-Type AVP

The Serving-Node-Type AVP (AVP Code 2047) is of type Enumerated and identifies the type of Serving Node. It may take the following values:

0    SGSN

1    PMIPSGW

2    GTPSGW

3    ePDG

4    hSGW

5    MME

6    TWAN

### 7.2.198A	SGi-PtP-Tunnelling-Method AVP

The SGi-PtP-Tunnelling-Method AVP (AVP Code 3931) is of type Enumerated and indicates whether SGi PtP tunnelling method is based on UDP/IP or other methods for a non-IP PDN type PDN connection.The following values are defined:

0	UDP_IP_based

1	Others

### 7.2.199	SGSN-Address AVP

The SGSN-Address AVP (AVP code 1228) is of type Address and holds the IP-address of the SGSN/Serving Node (e.g. S-GW, AGW, ePDG, TWAG from the P-GW/TDF and S4-SGSN, MME from the S-GW) that was used during a report.

### 7.2.199A	SGW-Address AVP

The SGW-Address AVP (AVP code 2067) is of type Address and holds the IP-address of the SGW Node.

### 7.2.200	SGW-Change AVP

The SGW-Change AVP (AVP Code 2065) is of type Enumerated, and indicates this is the first ACR[Start] due to an inter serving node change (change from SGW, ePDG, TWAG, HSGW). If this AVP is not present, this means this ACR[Start] is not due to an inter serving node change.

The following values are defined:

0	ACR_Start_NOT_due_to_SGW_Change

1	ACR_Start_due_to_SGW_Change

### 7.2.201	SIP-Method AVP

The SIP-Method AVP (AVP code 824) is of type UTF8String and holds the name of the SIP Method (INVITE, UPDATE etc.) causing a accounting request to be sent to the CDF or CCR to be sent to the OCF.

### 7.2.202	SIP-Request-Timestamp AVP

The SIP-Request-Timestamp AVP (AVP code 834) is of type Time and holds the time in UTC format of the SIP request (e.g. Invite, Update).

### 7.2.203	SIP-Request-Timestamp-Fraction AVP

The SIP-Request-Timestamp-Fraction AVP (AVP code 2301) is of type Unsigned32 and holds the miliseconds fraction in relation to SIP-Request-Timestamp.

### 7.2.204	SIP-Response-Timestamp AVP

The SIP-Response-Timestamp AVP (AVP code 835) is of type Time and holds the time in UTC format of the response to the SIP request (e.g. 200 OK).

### 7.2.205	SIP-Response-Timestamp-Fraction AVP

The SIP-Response-Timestamp-Fraction AVP (AVP code 2302) is of type Unsigned32 and holds the miliseconds fraction in relation to SIP-Response-Timestamp.

### 7.2.205A	SM-Device-Trigger-Indicator AVP

The SM-Device-Trigger-Indicator AVP (AVP code 3407) is of type Enumerated, and indicates whether the Short Message is related to Device Trigger, and which action is requested by the Device trigger. If this AVP is not present, this means the Short Message is not related to Device Trigger:

0	Not DeviceTrigger

1	Device Trigger request

2	Device Trigger replace

3	Device Trigger recall

### 7.2.205B	SM-Device-Trigger-Information AVP

The SM-Device-Trigger-Information AVP (AVP code 3405) is of type Grouped and holds the specific device trigger details for the Short Message.

It has the following ABNF grammar:

SM-Device-Trigger-Information :: =   < AVP Header: 3405>

[ MTC-IWF-Address ]

[ Reference-Number ]
[ Serving-Node ]

[ Validity-Time ]

[ Priority-Indication ]

[ Application-Port-Identifier ]

### 7.2.206	SM-Discharge-Time AVP

The SM-Discharge-Time AVP (AVP code 2012) is of type Time. It indicates the time associated with the event being reported in the SM-Status AVP. It is only used in scenarios in which the delivery of the Delivery-Report (more properly the Status-Report) is being charged.

For example, if SM-Status has the value 0x00, then the SM-Discharge-Time indicates the time of the delivery of the original Short Message.

The SMS Node shall ensure the correct encoding of this, as the other AVPs using the type Time, since the SMS messages use different formats.

### 7.2.207	SM-Message-Type AVP

The SM-Message-Type AVP (AVP code 2007) is of type Enumerated and indicates the type of the message which caused the charging interaction. The values are given below:

0	SUBMISSION

1	DELIVERY_REPORT

2.	SM Service Request

3	T4 Device Trigger

4	SM Device Trigger

5	MO-SMS T4 submission

### 7.2.208	SM-Protocol-Id AVP

The SM-Protocol-ID AVP (AVP code 2013) is of type OctetString and holds an indication of the protocol used for the SM.

### 7.2.208A	SM-Sequence-Number AVP

The SM-Sequence-Number AVP (AVP code 3408) is of type Unsigned32 and includes the sequence number of the SM within the concatenated short message when applicable for SMS Offline Charging.

### 7.2.209	SM-Status AVP

The SM-Status AVP (AVP code 2014) is of type OctetString. The OctetString is of length 1 octet and contains status information about the delivery of an SM.

### 7.2.210	SM-User-Data-Header AVP

The SM-User-Data-Header AVP (AVP code 2015) is of type OctetString and contains any user data header extracted from the user data part of the SM.  Encoding is as described in TS 23.040 [216]. Any padding bits are not considered part of the header and are omitted.

### 7.2.211	SMS-Information AVP

The SMS-Information AVP (AVP code 2000) is of type Grouped. Its purpose is to allow the transmission of additional SMS service specific information elements.

It has the following ABNF grammar:

SMS-Information :: = < AVP Header: 2000>

[ SMS-Node ]

[ Client-Address ]

[ Originator-SCCP-Address ]

[ SMSC-Address ]

[ Data-Coding-Scheme ]

[ SM-Discharge-Time ]

[ SM-Message-Type ]

[ Originator-Interface ]

[ SM-Protocol-ID ]

[ Reply-Path-Requested ]

[ SM-Status ]

[ SM-User-Data-Header ]

[ Number-Of-Messages-Sent ]

[ SM-Sequence-Number ]

* [ Recipient-Info ]

[ Originator-Received-Address ]

[ SM-Service-Type ]

[ SMS-Result ]

[ SM-Device-Trigger-Indicator ]

[ SM-Device-Trigger-Information ]

[ MTC-IWF-Address ]

[ Application-Port-Identifier ]

[ External-Identifier ]

### 7.2.212	SMS-Node AVP

The SMS-Node AVP (AVP code 2016) is of type Enumerated and identifies the role which the SMS node performs in relation to the charging event. It takes the following values:

0	SMS Router

1	IP-SM-GW

2	SMS Router and IP-SM-GW

3	SMS-SC

### 7.2.212A	SMS-Result AVP

The SMS-Result AVP (AVP code 3409) is of type Unsigned32 and includes the result of an attempt for a Short Message transaction (submission or delivery).

### 7.2.213	SM-Service-Type AVP

The SM-Service-Type AVP (AVP code 2029) is of type Enumerated and indicates the type of SM service that caused the charging interaction. The values as defined in TS 22.142 [217] are given below:

0	VAS4SMS Short Message content processing

1	VAS4SMS Short Message forwarding

2	VAS4SMS Short Message Forwarding multiple subscriptions

3	VAS4SMS Short Message filtering

4	VAS4SMS Short Message receipt

5	VAS4SMS Short Message Network Storage

6	VAS4SMS Short Message to multiple destinations

7	VAS4SMS Short Message Virtual Private Network (VPN)

8	VAS4SMS Short Message Auto Reply

9	VAS4SMS Short Message Personal Signature

10	VAS4SMS Short Message Deferred Delivery

11 .. 99	Reserved for 3GPP defined SM services

100 .. 199	Vendor specific SM services

The SM-Service-Type AVP shall be present if the SM-Message-Type AVP has value 2, SM Service Request.

### 7.2.214	SMSC-Address AVP

The SMSC-Address AVP (AVP code 2017) is of type Address and carries the address of the SMSC, as contained in the SM.

### 7.2.214A	Start-of-Charging AVP

The Start-of-Charging  AVP (AVP Code 3419) is of type Time and holds the time in UTC format which represents the time origin for charging and may be equivalent to the time call setup is initiated or the time the call is answered depending on configuration.

### 7.2.215	Start-Time AVP

The Start-Time  AVP (AVP Code 2041) is of type Time and holds the time in UTC format which represents the start of a user session at the S-GW/P-GW or the start of a voice call in the MSC.

### 7.2.215A	Status-AS-Code AVP

The Status-AS-Code AVP (AVP Code 2702) is of type Enumerated and only present to specify abnormal response code, e.g, 4xx, 5xx or Timeout, etc for specific AS when it responds abnormally to S-CSCF. 
If AS responds SIP 200 OK, this AVP isn't present in Application-Server-Information AVP.

The AVP may take the values as follows:

0 		4xx;

1 		5xx;

2 		Timeout

### 7.2.216	Stop-Time AVP

The Stop-Time AVP (AVP Code 2042) is of type Time and holds the time in UTC format which represents the termination of a user session at the S-GW/P-GW or the termination of a voice call in the MSC.

### 7.2.217	Submission-Time AVP

The Submission-Time AVP (AVP code 1202) is of type Time and indicates the time at which the message was submitted.

### 7.2.218	Subscriber-Role AVP

The Subscriber-Role AVP (AVP code 2033) is of type Enumerated and indicates the role of the subscriber (originating party or terminating party) in MMTel supplementary services. The role can be one of the following:

0	Originating

1	Terminating

This AVP is not used.

### 7.2.219	Supplementary-Service AVP

The Supplementary-Service AVP (AVP code 2048) is of type Grouped and holds the specific supplementary service details for one MMTel supplementary service.

It has the following ABNF grammar:

Supplementary-Service :: =   < AVP Header: 2048>

[ MMTel-SService-Type ]

[ Service-Mode ]
[ Number-Of-Diversions ]

[ Associated-Party-Address ]

[ Service-ID ]

[ Change-Time ]

[ Number-Of-Participants ]

[ Participant-Action-Type ]

[ CUG-Information ]

[ AoC-Information ]

### 7.2.219A	TAD-Identifier AVP

The TAD-Identifier AVP (AVP code 2717) is of type Enumerated and indicates the type of access network (CS or PS) through which the session shall be terminated. It can be one of the following values:

0	CS

1	PS

### 7.2.220	Talk-Burst-Exchange AVP

The Talk-Burst-Exchange AVP (AVP code 1255) is of type Grouped and holds the talk burst related charging data.

It has the following ABNF grammar:

Talk-Burst-Exchange ::	=	< AVP Header: 1255 >

{ PoC-Change-Time }
[ Number-Of-Talk-Bursts ]
[ Talk-Burst-Volume ]
[ Talk-Burst-Time ]
[ Number-Of-Received-Talk-Bursts ]
[ Received-Talk-Burst-Volume ]
[ Received-Talk-Burst-Time ]
[ Number-Of-Participants ]
[ PoC-Change-Condition ]

### 7.2.221	Talk-Burst-Time AVP

The Talk-Burst-Time AVP (AVP code 1286) is of  type Unsigned32 and holds the duration in seconds of the sent talk bursts.

### 7.2.222	Talk-Burst-Volume AVP

The Talk-Burst-Volume AVP (AVP code 1287) is of type Unsigned32 and holds the volume in bytes of  the sent talk bursts.

### 7.2.222A	Target-IP-Address AVP

The Target-IP-Address AVP (AVP code 4412) is of type Address and holds the IP address used as target address for performing ProSe Direct one-to-one Communication.

### 7.2.223	Tariff-Information AVP

The Tariff-Information AVP (AVP code 2060) is of type Grouped and holds a tariff definition either from the local provider or from 3rd party provider. It contains information about the operator and the ID of the service being provided, the current tariff and possible next tariff after tariff switch time. It may also chain to tariffs provided by intermediate operators in the chain.

It has the following ABNF grammar:

Tariff-Information :: =  < AVP Header: 2060 >

{ Current-Tariff }

[ Tariff-Time-Change ]

[ Next-Tariff ]

### 7.2.224	Tariff-XML AVP

The Tariff-XML AVP (AVP code 2306) is of type UTF8String and holds the XML body describing the tariff/add-on charge information exchanged during the SIP transaction, as described in the TS 29.658 [222].

### 7.2.224A	Teleservice AVP

The Teleservice AVP (AVP code 3413) is of type OctetString and holds the used teleservice for the voice call service.

### 7.2.225	Terminating-IOI AVP

The Terminating-IOI AVP (AVP code 840) is of type UTF8String (alphanumeric string) and holds the Inter Operator Identifier (IOI) for the terminating network as generated by the IMS Network Element which takes responsibility for populating this parameter in a SIP response as described in RFC 7315 [404] and TS 24.229 [202].

The Terminating IOI contains the following values:

Type 1 IOI

-	IOI of the home network where the S-CSCF is located for SIP responses directed to

the P-CSCF in the visited network when request was initiated by the served user

-	IOI of the visited network where the TRF is located for SIP responses directed to

the S-CSCF in the home network when request was initiated by the served user and "VPLMN routing" is applied in a Roaming Architecture for Voice over IMS with Local breakout.

-	IOI of the visited network where the P-CSCF is located for SIP responses directed to 
the S-CSCF in the home network when request was terminated at the served user.

Type 2 IOI

-	IOI of the home network of the terminating end user where the S-CSCF is located in case a 
session is initiated toward the IMS. In case of redirection by the S-CSCF, Terminating-IOI AVP indicates the terminating party's network operator to which the session is redirected.

-	IOI of the terminating network where the MGCF is located in case a session is initiated from the IMS toward the PSTN.

Type 3 IOI

-	IOI of the service provider network (originating side or terminating side) where the AS (proxy, terminating UA or redirect server or B2BUA) is located when receiving a SIP request as described in TS 24.229 [202].

-	IOI of the home network operator contacted by an AS when an AS (originating UA or B2BUA) initiates a SIP request as described in TS 24.229 [202].

For further details on the Type 1, Type 2 and Type 3 IOIs, please refer to TS 32.240 [1].

### 7.2.225A	Time-First-Reception AVP

The Time-First-Reception  AVP (AVP code 3456) is of type Time and holds the time in UTC format for the first IP packet received.

### 7.2.225B	Time-First-Transmission AVP

The Time-First-Transmission  AVP (AVP code 3467) is of type Time and holds the time in UTC format for the first IP packet transmitted.

### 7.2.226	Time-First-Usage AVP

The Time-First-Usage AVP (AVP code 2043) is of type Time and holds the time in UTC format for the first IP packet to be transmitted and mapped to the current service data container.

### 7.2.226A	Time-Indicator AVP

The Time-Indicator AVP (AVP code 3911) is of type Unsigned32 and instructs the announcement to be connected at the specified time before granted quota is exhausted, which ranges from zero to a value smaller than the granted quota. A value of zero means at the time quota is exhausted. Absence of this field indicates that the announcement is to be played before the IMS session is allowed to continue.

### 7.2.227	Time-Last-Usage AVP

The Time-Last-Usage AVP (AVP code 2044) is of type Time and holds the time in UTC format for the last IP packet to be transmitted and mapped to the current service data container.

### 7.2.228	Time-Quota-Mechanism

The Time-Quota-Mechanism AVP (AVP code 1270) is of type Grouped.

It has the following syntax:

- Time-Quota-Mechanism ::= < AVP Header: 1270>

{ Time-Quota-Type }

{ Base-Time-Interval }

The OCS may include this AVP in a Multiple-Services-Credit-Control AVP, when granting time quota.

### 7.2.229	Time-Quota-Threshold AVP

The Time-Quota-Threshold AVP (AVP code 868) is of type Unsigned32 and contains a threshold value in seconds. This AVP may be included within the Multiple-Services-Credit-Control AVP when this AVP also contains a Granted-Service-Units AVP containing a CC-Time AVP (i.e. when the granted quota is a time quota).

If received, the Credit-Control client shall seek re-authorization from the server for the quota when the quota contents fall below the supplied threshold. The client shall allow service to continue whilst the re-authorization is progress, until the time at which the original quota would have been consumed.

### 7.2.230	Time-Quota-Type AVP

The Time-Quota-Type AVP (AVP code 1271) is of type Enumerated. It is used to indicate which time quota consumption mechanism shall be used for the associated Rating Group. It has the following values:

0	DISCRETE_TIME_PERIOD

1	CONTINUOUS_TIME_PERIOD

### 7.2.231	Time-Stamps AVP

The Time-Stamps AVP (AVP code 833) is of type Grouped and holds the time stamp of the SIP REQUEST and the time stamp of the response to the SIP REQUEST.

It has the following ABNF grammar:

Time-Stamps ::=	< AVP Header: 833 >

[ SIP-Request-Timestamp ]
[ SIP-Response-Timestamp ]
[ SIP-Request-Timestamp-Fraction ]
[ SIP-Response-Timestamp-Fraction ]

### 7.2.232		Time-Usage AVP

The Time-Usage  AVP (AVP code 2045) is of type Unsigned32 and holds the effective used time within the service data container reporting interval.

### 7.2.233	Traffic-Data-Volumes AVP

The Traffic-Data-Volumes AVP (AVP code 2046) is of type Grouped. Its purpose is to allow the transmission of the IP-CAN bearer container, on encountering change on charging condition for this IP-CAN bearer.This container  reports the volume count (separated for uplink and downlink). The 3GPP-Charging-Id AVP for the IP-CAN bearer is included when charging per IP-CAN session is active.

It has the following ABNF grammar:

Traffic-Data-Volumes :: = 	  < AVP Header: 2046>

[ QoS-Information ]

[ Accounting-Input-Octets ]

[ Accounting-Output-Octets ]

[ Change-condition ]

[ Change-Time ]

[ 3GPP-User-Location-Info ]

[ UWAN-User-Location-Info ]

[ 3GPP-Charging-Id ]

[ Presence-Reporting-Area-Status ]

[ User-CSG-Information ]

[ 3GPP-RAT-Type ]
   [ Access-Availability-Change-Reason ]
   [ Related-Change-Condition-Information ]
   [ Diagnostics ]

[ Enhanced-Diagnostics ]

[ CP-CIoT-EPS-Optimisation-Indicator ]

[ Serving-PLMN-Rate-Control ]

### 7.2.233A	Transcoder-Inserted-Indication AVP

The Transcoder-Inserted-Indication  AVP (AVP code 2605) is of type Enumerated and indicates if a transcoder is inserted or not for the SDP media component. The following values are defined:

0	Transcoder Not Inserted

1	Transcoder Inserted

### 7.2.233B	Transit-IOI-List AVP

The Transit-IOI-List AVP (AVP code 2701) is of type UTF8String and holds the Inter Operator Identifiers (IOI) for the transit networks as generated by the IMS Network Elements which take responsibility for populating this parameter in a SIP request and response as described in RFC 7315 [404] and TS 24.229 [202]. Multiple occurrences of this AVP shall be in chronological order, i.e. the value in the SIP request is listed first. If only a value for the SIP response is available, the Transit-IOI-List for the SIP request shall be included with the value "unknown".

### 7.2.233C	Transmitter-Info AVP

The Transmitter-Info AVP (AVP code 3468) is of type Grouped and provides information on a transmitter detected for direct communication. Each transmitter is identified by a source IP address and ProSe UE ID.

It has the following ABNF grammar:

Transmitter-Info :: = < AVP Header: 3468>

[ ProSe-Source-IP-Address ]

[ ProSe-UE-ID ]

### 7.2.234	Token-Text AVP

The Token-Text AVP (AVP code 1215) is of type UTF8String and contains extension information for the Message-Class AVP.

### 7.2.235	Trigger AVP

The Trigger AVP (AVP code 1264) is of type Grouped and holds the trigger types. The presence of the Trigger AVP without any Trigger-Type AVP in a CCA allows OCS to disable all the triggers. The presence of the Trigger AVP in the CCR identifies the event(s) triggering the CCR.

It has the following ABNF grammar:

- Trigger  :: = < AVP Header: 1264 >

* [ Trigger-Type ]

### 7.2.236	Trigger-Type AVP

The Trigger-Type AVP (AVP code 870) is of type Enumerated and indicates a single re-authorization event type.

When included in the CCA command, the Trigger-Type AVP indicates the events that shall cause the Credit-Control client to re-authorize the associated quota. The client shall not re-authorize the quota when events which are not included in the Trigger AVP occur.

When included in the CCR command indicates the specific event which caused the re-authorization request of the Reporting-Reason with value RATING_CONDITION_CHANGE associated.

It has the following values:

1	CHANGE_IN_SGSN_IP_ADDRESS

This value is used to indicate that a change in the SGSN IP address shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

2	CHANGE_IN_QOS

This value is used to indicate that a change in the end user negotiated QoS shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

NOTE 1:	This should not be used in conjunction with enumerated values 10 to 23.

3	CHANGE_IN_LOCATION

This value is used to indicate that a change in the end user location shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

NOTE 2:	This should not be used in conjunction with enumerated values 30 to 36.

4	CHANGE_IN_RAT

This value is used to indicate that a change in the radio access technology shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

- 5	CHANGE_IN_UE_TIMEZONE
This value is used to indicate that a change in the TimeZone where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

10	CHANGEINQOS_TRAFFIC_CLASS
This value is used to indicate that a change in the end user negotiated traffic class shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

11	CHANGEINQOS_RELIABILITY_CLASS
This value is used to indicate that a change in the end user negotiated reliability class shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

12	CHANGEINQOS_DELAY_CLASS
This value is used to indicate that a change in the end user negotiated delay class shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

13	CHANGEINQOS_PEAK_THROUGHPUT
This value is used to indicate that a change in the end user negotiated peak throughput shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

14	CHANGEINQOS_PRECEDENCE_CLASS
This value is used to indicate that a change in the end user negotiated precedence class shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

15	CHANGEINQOS_MEAN_THROUGHPUT
This value is used to indicate that a change in the end user negotiated mean throughput shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

16	CHANGEINQOS_MAXIMUM_BIT_RATE_FOR_UPLINK
This value is used to indicate that a change in the end user negotiated uplink maximum bit rate shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

17	CHANGEINQOS_MAXIMUM_BIT_RATE_FOR_DOWNLINK
This value is used to indicate that a change in the end user negotiated downlink maximum bit rate shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

18	CHANGEINQOS_RESIDUAL_BER
This value is used to indicate that a change in the end user negotiated residual BER shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

19	CHANGEINQOS_SDU_ERROR_RATIO
This value is used to indicate that a change in the end user negotiated SDU error ratio shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

20	CHANGEINQOS_TRANSFER_DELAY
This value is used to indicate that a change in the end user negotiated transfer delay shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

21	CHANGEINQOS_TRAFFIC_HANDLING_PRIORITY
This value is used to indicate that a change in the end user negotiated traffic handling priority shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

22	CHANGEINQOS_GUARANTEED_BIT_RATE_FOR_UPLINK
This value is used to indicate that a change in the end user negotiated uplink guaranteed bit rate shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

23	CHANGEINQOS_GUARANTEED_BIT_RATE_FOR_DOWNLINK
This value is used to indicate that a change in the end user negotiated downlink guaranteed bit rate shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

24	CHANGEINQOS_APN_AGGREGATE_MAXIMUM_BIT_RATE
This value is used to indicate that a change in the APN aggregate maximum bit rate for the IP-CAN session shall cause the Credit-Control client to ask for a re-authorization of the associated quota. This value is only applicable when charging per IP-CAN session is active.

30	CHANGEINLOCATION_MCC
This value is used to indicate that a change in the MCC of the serving network shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

31	CHANGEINLOCATION_MNC
This value is used to indicate that a change in the MNC of the serving network shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

32	CHANGEINLOCATION_RAC
This value is used to indicate that a change in the RAC where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

33	CHANGEINLOCATION_LAC
This value is used to indicate that a change in the LAC where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

34	CHANGEINLOCATION_CellId
This value is used to indicate that a change in the Cell Identity where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

35	CHANGEINLOCATION_TAC
This value is used to indicate that a change in the TAC where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

36	CHANGEINLOCATION_ECGI
This value is used to indicate that a change in the ECGI where the end user is located shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

40	CHANGE_IN_MEDIA_COMPOSITION
This value is used to indicate that a change in the media composition (as identified within SDP) for an existing SIP session shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

50	CHANGE_IN_PARTICIPANTS_NMB
This value is used specifically for multi participating session to indicate that a change in the number of active participants within a session shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

51	CHANGE_IN_ THRSHLD_OF_PARTICIPANTS_NMB
This value is used specifically to indicate that a change in the threshold of participants number within a session shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

NOTE 3:	The threshold and the granularity of threshold are operator configurable. This should not be used in conjunction with value 50.

- 52	CHANGE_IN_USER_PARTICIPATING_TYPE
This value is used specifically to indicate that a change in the user participating type within a session shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

- 60	CHANGE_IN_SERVICE_CONDITION
This value is used to indicate that a change in rating conditions associated with a service occurs. 
The description of the conditions causing a change are service specific and may be documented in middle-tier specifications or may be configurable.

- 61	CHANGE_IN_SERVING_NODE
This value is used to indicate that a change in serving node shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

- 62	CHANGE_IN_ACCESS_FOR_A_SERVICE_DATA_FLOW
This value is used to indicate that a change in access for a service data flow shall cause the Credit-Control client to ask for a re-authorization of the associated quota.  This value is only applicable when charging per IP-CAN session is active for a multi-access PDN connection.

- 70	CHANGE_IN_USER_CSG_INFORMATION
This value is used to indicate a request of reporting the event that the user enters/leaves a CSG cell. When used in a CCR, at entry to a CSG cell, the User-CSG-Information AVP shall be provided with the event report. When used in a CCR without any User-CSG-Information AVP, it indicates the user leaves the CSG cell.

- 71	CHANGE_IN_HYBRID_SUBSCRIBED_USER_CSG_INFORMATION
This value is used to indicate a request of reporting the event that the user enters/leaves a hybrid cell that the user subscribes to. When used in a CCR, at entry to a hybrid cell where the user is a member, the User-CSG-Information AVP shall be provided with the event report. When used in a CCR without any User-CSG-Information AVP, it indicates the user leaves the hybrid cell he was a member of.

- 72	CHANGE_IN_HYBRID_UNSUBSCRIBED_USER_CSG_INFORMATION
This value is used to indicate a request of reporting the event that the user enters/leaves a hybrid cell that the user does not subscribe to. When used in a CCR, at entry to a hybrid cell where the user is not a member, the User-CSG-Information AVP shall be provided with the event report. When used in a CCR without any User-CSG-Information AVP, it indicates the user leaves the hybrid cell he was not a member of.

73	CHANGE_OF_UE_PRESENCE_IN_PRESENCE_REPORTING_AREA
This value is used to indicate a request of reporting the event that the user enters/leaves the area(s) as indicated in the Presence-Reporting-Area-Information AVP. This includes reporting the initial status at the time the request is initiated. When used in a CCR, the Presence-Reporting-Area-Identifier AVP and Presence-Reporting-Area-Status AVP shall be provided in the Presence-Reporting-Area-Information AVP with the event report. For the UE-dedicated presence area, the Presence-Reporting-Area-Elements-List AVP shall be provided in the Presence-Reporting-Area-Information AVP. When used in a CCA with a list of PRA(s) , the new list of PRA(s) shall supersede the previous provided list. In case this trigger is not included in Trigger-Type AVP, it indicates all PRAs are unsubscribed.74	CHANGE_IN_SERVING_PLMN_RATE_CONTROL
This value is used to indicate that a change in the Serving PLMN Rate Control shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

- 75	CHANGE_IN_APN_RATE_CONTROL
This value is used to indicate that a change in the APN Rate Control shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

- 76	CHANGE_IN_3GPP_PS_DATA_OFF

- This value is used to indicate that a change in the 3GPP_PS_DATA_OFF shall cause the Credit-Control client to ask for a re-authorization of the associated quota.

### 7.2.237	Trunk-Group-ID AVP

The Trunk-Group-ID AVP (AVP code 851) is of type Grouped and identifies the incoming and outgoing PSTN legs.

It has the following ABNF grammar:

Trunk-Group-ID :: =	 <AVP Header: 851>

[ Incoming-Trunk-Group-ID ]
						[ Outgoing-Trunk-Group-ID ]

### 7.2.237A	Void

### 7.2.237B	Void

### 7.2.237Ba	TWAG-Address AVP

The TWAG-Address AVP (AVP code 3903) is of type Address and holds the IP-address of the TWAG Node.

### 7.2.237C	TWAN-User-Location-Info AVP

The TWAN-User-Location-Info AVP (AVP code 2714) is of type Grouped and contains the UE location in a Trusted WLAN Access Network (TWAN):  BSSID and SSID of the access point.

It has the following ABNF grammar:

TWAN-User-Location-Info :: = 	< AVP Header: 2714>

{ SSID }

[ BSSID ]

### 7.2.238	Type-Number AVP

The Type-Number AVP (AVP code1204) is of type Enumerated and identifies the well-known media types. The values are taken from OMNA WSP Content Type Numbers [210].

### 7.2.238A	UNI-PDU-CP-Only-Flag AVP

The UNI-PDU-CP-Only-Flag AVP (AVP code 3932) is of type Enumerated, and indicates whether this PDN connection is applied with "Control Plane Only flag", i.e. using only S11-U from S-GW when Control Plane CIoT EPS Optimization is enabled. If this AVP is not present, this means both user plane and control plane can be used in UNI for PDU transfer, i.e. S1-U and S11-U respectively from S-GW, when Control Plane CIoT EPS Optimization is enabled. The following values are defined:

0	UNI-PDU-both-UP-CP

1	UNI-PDU-CP-Only

### 7.2.239	Unit-Cost AVP

The Unit-Cost AVP (AVP code 2061) is of type Grouped and holds monetary value. It has the same format of Unit-Value AVP.

It has the following ABNF grammar:

Unit-Cost :: = < AVP Header: 2061 >

{ Value-Digits }

[ Exponent ]

### 7.2.240	Unit-Quota-Threshold AVP

The Unit-Quota-Threshold AVP (AVP code 1226) is of type Unsigned32 and contains a threshold value in service specific units. This AVP may be included within the Multiple-Services-Credit-Control AVP when this AVP also contains a Granted-Service-Units AVP containing CC-Service-Specific-Units AVP (i.e. when the granted quota is service specific) or within Rate-Element AVP..

If received in the context of Multiple-Service-Credit-Control AVP, the Credit-Control client shall seek re-authorization from the server for the quota when the quota contents fall below the supplied threshold. 
The client shall allow service to continue whilst the re-authorization is in progress, up to the volume indicated in the original quota.

In the context of the Rating-Element AVP it denotes the durability of a Rating Element within a Tariff. I.e. if the service consumed Unit-Quota-Threshold number of Unit-Types, the next Rating element becomes in effect.

### 7.2.240a	Unused-Quota-Timer AVP

The Unused-Quota-Timer AVP (AVP code 4407) is of type Unsigned32 and contains a threshold time in seconds for the time period without any quota granted or any quota being consumed for any rating group belonging to the Gy session.

### 7.2.240A	User-CSG-Information AVP

The User-CSG-Information AVP (AVP code 2319) is of type Grouped and holds the user "Closed Subscriber Group" information associated to CSG cell access: it comprises CSG ID within the PLMN, access mode and indication on CSG membership for the user when hybrid access applies, as defined in TS 29.060 [225] for GPRS case, and in TS 29.274 [226] for EPC case.

It has the following ABNF grammar:

User-CSG-Information :: = 	< AVP Header: 2319>

{ CSG-Id }

{ CSG-Access-Mode }

[ CSG-Membership-Indication ]

### 7.2.240B	Usage-Information-Report-Sequence-Number AVP

The Usage-Information-Report-Sequence-Number AVP (AVP code 3439) is of type Integer32 and indicates the sequence number of usage information report, which is used to generate the container.

### 7.2.241	User-Participating-Type AVP

The User-Participating-Type AVP(AVP code 1279) is of type Enumerated. It is a subfield of Participants-Group AVP to indicate the user participating type when participating in the PoC session. The AVP may take the values as follows:

0   Normal

1   NW PoC Box

2   UE PoC Box

### 7.2.242	User-Session-Id AVP

The User-Session-Id AVP (AVP code 830) is of type UTF8String and holds the session identifier. For a SIP session the User-Session-Id contains the SIP Call ID, as defined in RFC 3261 [405]. When the AS acts as B2BUA, the incoming Session-ID leg is covered.

### 7.2.242aaA	UWAN-User-Location-Info AVP

The UWAN-User-Location-Info AVP (AVP code 3918) is of type Grouped and contains the UE location in an Unrusted Wireless Access Network (UWAN). The UWAN User Location Information shall include the UE local IP address and optionally UDP source port number (if NAT is detected). It may also include WLAN Location Information the BSSID and SSID of the access point.

It has the following ABNF grammar:

UWAN-User-Location-Info :: = 	< AVP Header: 3918>

{ UE-Local-IP-Address }

[ UDP-Source-Port ]

[ SSID ]

[ BSSID ]

### 7.2.242aA	Variable-Part AVP

The Variable-Part AVP (AVP code 3907) is of type Grouped and specifies the order, type, and value of a variable to be played back within the announcement.

It has the following ABNF grammar:

Variable-Part :: =  < AVP Header: 3907 >

[ Variable-Part-Order ]

{ Variable-Part-Type }

{ Variable-Part-Value }

### 7.2.242aB	Variable-Part-Order AVP

The Variable-Part-Order AVP (AVP code 3908) is of type Unsigned32. It contains the order of the variable part within the announcement to be played back.

### 7.2.242aC	Variable-Part-Type AVP

The Variable-Part-Type AVP (AVP Code 3909) is of type Unsigned32 and holds the type of the variable part.

The following values are defined:

0			Integer

1			Number

2			Time

3			Date

4			Currency

5-127		Reserved for future standardization

128-255	Reserved for operator-defined types

### 7.2.242aD	Variable-Part-Value AVP

The Variable-Part-Value AVP (AVP Code 3910) is of type UTF8String and holds the value of the variable part. Table 7.2.242aD.1 describes the content of the Variable-Part-Value AVP for each of the standardized values of the Variable-Part-Type AVP.

- Table 7.2.242aD.1: Variable-Part-Value AVP Contents

| Type | Value |
| --- | --- |
| Integer | String of digits, which shall be announced as a single number, up to 10 digits. |
| Number | String of digits, which shall be announced as individual digits, up to 24 digits. |
| Time | A string of digits in the form HHMM. |
| Date | A string of digits in the form of YYYYMMDD. |
| Currency | A string of digits in the form of AAAAAABB, where AAAAAA is the integral part and BB is the decimal part. |

### 7.2.242A	VCS-Information AVP

The VCS-Information AVP (AVP code 3410) is of type Grouped. Its purpose is to allow the transmission of additional VCS service specific information elements.

It has the following ABNF grammar:

VCS-Information :: = 	  < AVP Header: 3410>

[ Bearer-Capability ]

[ Network-Call-Reference-Number ]

[ MSC-Address ]
[ Basic-Service-Code ]

[ ISUP-Location-Number ]

[ VLR-Number ]

[ Forwarding-Pending ]

[ ISUP-Cause ]

[ Start-Time ]

[ Start-of-Charging ]

[ Stop-Time ]

[ PS-Free-Format-Data ]

### 7.2.242B	VLR-Number AVP

The VLR-Number AVP (AVP code 3420) is type OctetString and identifies the international E.164 address of the VLR serving the user. It is encoded as a TBCD-string. See TS 29.002 [232] for encoding of TBCD-strings. 
This AVP does not include leading indicators for the nature of address and the numbering plan; it contains only the TBCD-encoded digits of the address.

### 7.2.243	Volume-Quota-Threshold AVP

The Volume-Quota-Threshold AVP (AVP code 869) is of type Unsigned32 and contains a threshold value in octets. This AVP may be included within the Multiple-Services-Credit-Control AVP when this AVP also contains a Granted-Service-Units AVP containing a CC-Total-Octets AVP, CC-Input-Octets AVP or CC-Output-Octets AVP (i.e. when the granted quota is a volume quota).

If received, the Credit-Control client shall seek re-authorization from the server for the quota when the quota contents fall below the supplied threshold. The client shall allow service to continue whilst the re-authorization is progress, up to the volume indicated in the original quota.

### 7.2.244	Void

### 7.2.245	Void

### 7.2.246	Void

### 7.2.247	Void

### 7.2.248	Void

### 7.2.249	Void

### 7.2.250	Void

## 7.3	3GPP2 specific AVPs

For the purpose of offline and online charging, the use of the AVPs defined for 3GPP2 Access is provided in table 7.3.1, and the specification including the detailed definition and description, is referred to.

These AVPs shall be used together with value 5535 (3GPP2) as Vendor-Id.

- Table 7.3.1: 3GPP2 specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  |
| 3GPP2-BSID | 9010 | X | - | X | - | [215] |

## 7.4	ETSI specific AVPs

For the purpose of offline and online charging, the use of the AVPs defined for Fixed Broadband Access is provided in table 7.4.1, and the specification including the detailed definition and description, is referred to.

These AVPs shall be used together with value 13019 (ETSI) as Vendor-Id.

NOTE:	The Fixed-User-Location-Info AVP is a fixed access specific AVP defined by 3GPP and listed in Table 7.2.0.1. The Fixed-User-Location-Info AVP uses the Vendor-Id specified in clause 7.2.0.

- Table 7.4.1: ETSI specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  |
| Logical-Access-ID | 302 | X | - | X | - | [300] |
| Physical-Access-ID | 313 | X | - | X | - | [300] |

## 7.5	oneM2M specific AVPs

For the purpose of offline and online charging, the use of the AVPs defined by oneM2M in oneM2M TS-0004[301] is provided in table 7.5.1, and the specification including the detailed definition and description, is referred to.

These AVPs shall be used together with value 45687 (oneM2M) as Vendor-Id.

- Table 7.5.1: oneM2M specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  |
| M2M-Information | 1011 | X | - | - | - | [301] |

Annex A (informative):
Bibliography

a	The 3GPP charging specifications

-	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

-	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

-	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

-	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

-	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

-	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

-	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

-	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

-	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging".

- 	3GPP TS 32.276: "Telecommunication management; Charging management; Voice Call Service Charging".

-	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

-	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

-	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) encoding rules description".

-	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

-	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

-	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

-	3GPP TS 32.293: "Telecommunication management; Charging management; Proxy Function".

b	Common 3GPP specifications

-	3GPP TS 33.201: "Access domain security".

c	other Domain and Service specific 3GPP / ETSI specifications

-

d	Relevant ITU Recommendations

-

e	Relevant IETF RFCs

-	IETF RFC 959 (1985): "File Transfer Protocol".

-	IETF RFC 1350 (1992): "TFTP Protocol".

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2007 | SP-38 | SP-070745 | 0202 | -- | Add new AVP codes to satisfy OMA charging requirements | C | 8.0.0 | 8.1.0 |
| Dec 2007 | SP-38 | SP-070745 | 0203 | -- | Add new values to PoC-User-Role-info-Units AVP - Align with OMA PoC charging requirements | C | 8.0.0 | 8.1.0 |
| Dec 2007 | SP-38 | SP-070745 | 0204 | -- | Add general description to PoC-Group-Name | B | 8.0.0 | 8.1.0 |
| Dec 2007 | SP-38 | SP-070925 | 0205 | 1 | Introduce Diameter details for SMS charging | B | 8.0.0 | 8.1.0 |
| Mar 2008 | SP-39 | SP-080059 | 0206 | -- | Add IBCF to Node-Functionality AVP list of NEs | B | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080074 | 0207 | -- | Usage of CC-Correlation-Id in online charging - Align with IETF RFC 4006 | C | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080074 | 0208 | -- | Align Number-Of-Messages-Sent AVP in Diameter Binding for SMS charging with new R8 TS 32.274 | C | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080074 | 0209 | -- | Corrections on Diameter AVP for SMS Charging | F | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080074 | 0210 | -- | Add on Number Portability and Carrier Select routing information | B | 8.1.0 | 8.2.0 |
| Jun 2008 | SP-40 | SP-080330 | 0211 | -- | Correction on SCCP-Address AVPs | F | 8.2.0 | 8.3.0 |
| Jun 2008 | SP-40 | SP-080330 | 0212 | -- | Add PoC-Event-Type AVP into PoC-Information | B | 8.2.0 | 8.3.0 |
| Jun 2008 | SP-40 | SP-080330 | 0213 | -- | Correction to PoC-Controlling-Address AVP and PoC-Group-Name AVP | F | 8.2.0 | 8.3.0 |
| Sep 2008 | SP-41 | SP-080466 | 0214 | -- | Correction of inconsistencies in Offline Charging and Online Charging messages | F | 8.3.0 | 8.4.0 |
| Sep 2008 | SP-41 | SP-080330 | 0215 | -- | Correction on AVP codes - Alignment with TS 29.230 | F | 8.3.0 | 8.4.0 |
| Sep 2008 | SP-41 | SP-080330 | 0216 | -- | Multiple SMS destination - Alignment with TS 23.040 | C | 8.3.0 | 8.4.0 |
| Dec 2008 | SP-42 | SP-080706 | 0218 | - | Completion on message tables | F | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080707 | 0219 | - | Service Context Id for MMTEL | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0220 | - | Correction on AVP code allocation | F | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0221 | - | Clarification on AVP descriptions for EPC Charging | F | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0222 | - | Add SMS-SC as SMS node type | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0223 | - | Additional Address Info for SMS charging | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0224 | - | Add charging of SMS services to 32.299 | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080707 | 0225 | - | TS 32.299 AVPs Introduction for MMTel charging | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0226 | - | Correction on References Section | F | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0227 | - | Addition of SDP offer and answer and clarification on media initiator | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0228 | - | Additional non-3GPP access information | F | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080706 | 0229 | - | Add a new value to Trigger-Type AVP | B | 8.4.0 | 8.5.0 |
| Dec 2008 | SP-42 | SP-080852 | 0217 | - | TS 32.299 AVPs for offline charging - Rf interface from S-GW and P-GW | B | 8.4.0 | 8.5.0 |
| Mar 2009 | SP-43 | SP-090206 | 0232 |  | TS 32.299 MMTel information AVP alignment with 32.275 definition | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090206 | 0235 |  | Add CONF charging specific parameters | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090203 | 0230 | 1 | Correction on 'Subscription Id' category used for EPS offline Charging | C | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090203 | 0231 | - | Service-Type and Service-Mode in Supplementary-service AVP : format change | F | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090206 | 0232 | - | SMS AVP structure alignment | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090045 | 0234 |  | Add Serving-Node-Type AVP to PS-Information in 32.299 | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090206 | 0235 | - | AoC Support in Ro | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090045 | 0236 | - | EPS Offline Charging - Complete PS-information AVP description | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090203 | 0237 | - | Multiple subscription-id in service-information for EPS offline Charging | B | 8.5.0 | 8.6.0 |
| Mar 2009 | SP-43 | SP-090206 | 0238 | - | Missing information in PS information AVP for SGW/PGW CDRs in EPS offline charging | B | 8.5.0 | 8.6.0 |
| Jun 2009 | SP-44 | SP-090432 | 0243 | - | Correction of Recipient-Info AVP | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0244 | - | AVP code allocation for DCD Charging | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0245 | - | Rel-8 CR 32.299 alignment with RFC 4006 | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0246 | - | Rel-8 CR 32.299 clarification of ICID | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0247 | - | Remove generic "Non 3GPP specific information" parameter | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0248 | - | Correction on PDP context usage | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0249 | - | Correction on AoC-Information AVP | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0252 | - | Correction on Refund Information | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0253 | - | LCS-information AVP not complete | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0254 | - | PS-information AVP description alignment with 32.251 definition | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090432 | 0257 | - | Correction on EPC Charging | F | 8.6.0 | 8.7.0 |
| Jun 2009 | SP-44 | SP-090294 | 0251 | - | Add MMTel supplementary services FA, CCBS&CCNR, MCID, CAT for MMTel Charging | B | 8.7.0 | 9.0.0 |
| Jun 2009 | SP-44 | SP-090292 | 0255 | - | Rel-9 CR 32.299 addition of online charging flag | B | 8.7.0 | 9.0.0 |
| Jun 2009 | SP-44 | SP-090292 | 0256 | - | Rel-9 CR 32.299 correction of timestamp granularity | B | 8.7.0 | 9.0.0 |
| Sep 2009 | SP-45 | SP-090536 | 0258 | - | Correction on AVP definitions | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0260 | - | Correction on MMS-Information AVP | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0262 | - | Rel-9 CR 32.299 correction and alignment with TS 32.280 | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0264 | - | Error in Number-Portability-Routing-Information AVP definition | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0265 | - | Add "Closed User Group (CUG)" for MMTel Charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0266 | - | Add 3PTY MMTel supplementary service charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090541 | 0267 | - | R9 CR 32299 add MBMS GW address below MBMS information AVP | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090538 | 0268 | - | New AVPs for RTTI support in IMS offline charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0271 | - | Correction on Content Type - Alignment with OMA definition | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090537 | 0272 | - | Correction of time stamp diameter types | F | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0274 | - | Correction of Accounting Input/Output Octets handling | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090537 | 0275 | - | Emergency bearer service consideration for charging | B | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090536 | 0277 | - | Addition of IP multicast delivery indicator below MBMS information AVP | A | 9.0.0 | 9.1.0 |
| Dec 2009 | SP-46 | SP-090720 | 0279 | - | Alignment of Address-Type AVP with 32.274 | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0281 | - | Alignment with TS 32.251 for "Volume Limit" and "Time Limit" in Change-Condition AVP | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0283 | - | Multiple Change-Condition AVP for simultaneous Condition changes | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0284 | - | Editorial clean-up | D | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090722 | 0285 | - | MMTel related AVP applicable for Online Charging | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0287 | - | Correction on priority session treatment | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0289 | - | Alignment with TS 32.251 for "User location Change" Condition in Change-Condition AVP | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0291 | - | Alignment between Change-Condition AVP value with ASN1 ServiceConditionChange value "serviceStop" | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0292 | - | AVP for Account Expiration Information from OCS to IMS Application Servers | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0293 | - | Aligning AoC- Information AVP with RTTI and subscription information | C | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090720 | 0295 | - | Correction of Number Portability and Carrier Select information AVPs | A | 9.1.0 | 9.2.0 |
| Dec 2009 | SP-46 | SP-090721 | 0296 | - | Add CSG parameters for CSG based online and offline charging | B | 9.1.0 | 9.2.0 |
| Mar 2010 | SP-47 | SP-100041 | 0297 | - | Correction on AVP code definitions | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 0299 | - | Correction of Role-of-Node AVP | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 0301 | - | Alignment with TS 32.251 for "Charging Characteristics Selection Mode" parameter | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 0302 | - | Add CSG parameters for CSG based online and offline charging | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100044 | 0303 | - | MMTel related AVP applicable for Online Charging | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100040 | 0305 | - | Correction for offline Charging from PGW - 3GPP2 User location | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 0306 | - | Remove unused Service-Condition-Change AVP | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100041 | 0307 | - | Correction on SDP handling in IMS Charging | F | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100044 | 0308 | - | Add "Personal Network management" MMTel supplementary service charging description | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SP-47 | SP-100044 | 0309 | - | Add "Customized Ringing Signal (CRS)" MMTel supplementary service charging description | B | 9.2.0 | 9.3.0 |
| Jun 2010 | SP-48 | SP-100265 | 0312 | - | Correction on AVP definitions | A | 9.3.0 | 9.4.0 |
| Oct 2010 | SP-49 | SP-100496 | 0314 | - | Correction for Dual IP addresses associated to one PDN connection | A | 9.4.0 | 9.5.0 |
| Oct 2010 | SP-49 | SP-100495 | 0317 | - | Correction on Charging-Rule-Based-Name AVP - Alignment with TS 23.203 | A | 9.4.0 | 9.5.0 |
| Oct 2010 | SP-49 | SP-100496 | 0319 | - | Correction on Event Charging with Reservation | A | 9.4.0 | 9.5.0 |
| Oct 2010 | SP-49 | SP-100497 | 0320 | - | Correction of Reason-Code AVP | F | 9.4.0 | 9.5.0 |
| Dec 2010 | SP-50 | SP-100756 | 0328 | - | Correction of Inter-Operator-Identifier AVP – Align with TS 32.260 | A | 9.5.0 | 9.6.0 |
| Dec 2010 | SP-50 | SP-100757 | 0322 | - | Correction of Trigger-Type AVP | F | 9.5.0 | 9.6.0 |
| Dec 2010 | SP-50 | SP-100758 | 0324 | 2 | Add missing LCS-Format-Indicator AVP value for "SIP_URL" | F | 9.5.0 | 9.6.0 |
| Dec 2010 | SP-50 | SP-100759 | 0325 | 2 | Replace the Authorized-QoS AVP name with Authorised-QoS AVP | F | 9.6.0 | 10.0.0 |
| Mar 2010 | SP-51 | SP-110109 | 0329 | 1 | Add E.164 harmonized address format to the current E.212 in MMS Charging | C | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110105 | 0330 | 3 | Adding CDR fields needed for Machine Type Communication | B | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110108 | 0332 | 2 | Add missing enumeration value for E-CSCF network element in Node-Functionality AVP - Align with 32.298 | A | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110108 | 0336 | 1 | Add internal structure and encoding for the Location-Estimate AVP | A | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110109 | 0340 | 2 | Correction to charging scenarios | D | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110108 | 0344 | 1 | Correction of CSG trigger handling - Alignment with TS 29.212 | A | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110109 | 0345 | 1 | Addition of IARI in IMS charging information, alignment with TS 22.115 and TS 23.228 | B | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110109 | 0347 | 1 | R10 32299 Correction on AVP Subscriber-role | F | 10.0.0 | 10.1.0 |
| Mar 2010 | SP-51 | SP-110112 | 0349 | - | Add 'Advice Of Charge (AoC)' MMTel supplementary service Charging description - Align with 32.275 | B | 10.0.0 | 10.1.0 |
| May 2011 | SP-52 | SP-110281 | 0352 | 1 | Correction to Re-authorization Request Message | F | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110281 | 0356 | 1 | Correction of RAT-Type AVP, alignment with TS 29.212, Gx interface | F | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110404 | 0359 | 1 | Correction on essential supported fields in EPC Online Charging | A | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110404 | 0362 | 1 | Correction on Rf interface for missing information in SGW CDR | A | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110294 | 0363 | 1 | AVPs enhancement for OMR Charging introduction | B | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110280 | 0365 | 1 | Correction in SCC AS CDR for IMS service continuity | A | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110281 | 0366 | 1 | Correction on IMS Application Reference Identifier (IARI) in IMS Charging | F | 10.1.0 | 10.2.0 |
| May 2011 | SP-52 | SP-110317 | 0350 | 1 | Rc Context Definition for Diameter Usage | B | 10.2.0 | 11.0.0 |
| Sep 2011 | SP-53 | SP-110528 | 0370 | - | Correction on PDN connection identifier for Charging | A | 11.0.0 | 11.1.0 |
| Sep 2011 | SP-53 | SP-110528 | 0375 | - | Correction for dynamic address flags associated to PDN connection of PDP/PDN type IPv4v6 | A | 11.0.0 | 11.1.0 |
| Sep 2011 | SP-53 | SP-110530 | 0377 | - | Correction of RAT Type, alignment with TS 29.061 | A | 11.0.0 | 11.1.0 |
| Sep 2011 | SP-53 | SP-110528 | 0381 | 1 | Correction on AVP definition - Align with IETF RFC 3588 | A | 11.0.0 | 11.1.0 |
| Sep 2011 | SP-53 | SP-110541 | 0393 | 1 | Addition of Sponsored Data Connectivity charging - Align with TS 23.203 | B | 11.0.0 | 11.1.0 |
| Dec 2011 | SP-54 | SP-110708 | 0420 | 2 | Correction of Dynamic Address Flag usage for IPv4v6 PDN Connection in PS Information AVP | A | 11.1.0 | 11.2.0 |
| Dec 2011 | SP-54 | SP-110711 | 0412 | 1 | Correction of IPv6 PDP/PDN prefix | A | 11.1.0 | 11.2.0 |
| Dec 2011 | SP-54 | SP-110711 | 0416 | - | Correction of IETF specified AVP usage in 3GPP charging applications | A | 11.1.0 | 11.2.0 |
| Dec 2011 | SP-54 | SP-110712 | 0394 | 2 | Add Transit IOI to IMS Offline Charging | B | 11.1.0 | 11.2.0 |
| Mar 2012 | SP-55 | SP-120048 | 0422 | 1 | Add missing Trigger-Type value to address change of UE Timezone alignment with TS 29.212 | A | 11.2.0 | 11.3.0 |
| Mar 2012 | SP-55 | SP-120049 | 0425 | 1 | Add Status in AVP "Application-Server-Information | B | 11.2.0 | 11.3.0 |
| June-2012 | SP-56 | SP-120362 | 0428 | 1 | Correction of Serving Node Type, alignment with 29.274 | F | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120359 | 0430 | 1 | Correction of AVP usage, alignment with RFC 4006 | A | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120361 | 0433 | 1 | Correction on AVP definition | A | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120362 | 0434 | 1 | Correction of Multiple Service Credit-Control use | F | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120359 | 0440 | 1 | Correction of diameter AVP usages, alignment with RFC 4006 | A | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120360 | 0447 | 1 | Correction on SGW and PGW Address reporting, alignment with 29.212 | A | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120374 | 0448 | 2 | Enhancing IMS charging for RAVEL | B | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120362 | 0449 | 1 | Correction of Termination Action procedure | F | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120397 | 0450 | 1 | Add charging parameters for NetLoc | B | 11.3.0 | 11.4.0 |
| June-2012 | SP-56 | SP-120361 | 0452 | - | Correction on PDP-Address-Prefix-Length | A | 11.3.0 | 11.4.0 |
| Sep-2012 | SP-57 | SP-120646 | 0456 | 1 | Rename Service-type AVP | A | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120561 | 0460 | - | Remove Authorised-Qos from P-CSCF CDR | A | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120566 | 0461 | 1 | Correction on the AVP name | F | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120566 | 0462 | 1 | Supplementation for Reporting-Reason AVP | C | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120564 | 0465 | 1 | Correction of AoC-Information AVP usage | A | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120562 | 0467 | 1 | Correction of Called-Party-Address AVP | A | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120566 | 0469 | 1 | Correction of calling party handling | C | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120646 | 0472 | 2 | Correction on AoC service support | A | 11.4.0 | 11.5.0 |
| Sep-2012 | SP-57 | SP-120627 | 0473 | 1 | Reference list correction to align with the corrected TS 29.212 title | F | 11.4.0 | 11.5.0 |
| Dec-2012 | SP-58 | SP-120789 | 0474 | 1 | Correction on the figure description of Centralized Unit Determination and Centralized Rating | F | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120789 | 0476 | 1 | Clarification of Type 1 and Type 2 IOI Usage for IMS Roaming | F | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120785 | 0479 | 2 | Emergency Indicator introduction in P-CSCF CDR | A | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120801 | 0481 | - | Correction on NNI-Information AVP | F | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120792 | 0482 | 1 | Introduction AVPs description for SMS over MME Charging | B | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120793 | 0483 | 1 | Offline Charging description for ATCF | B | 11.5.0 | 11.6.0 |
| Dec-2012 | SP-58 | SP-120789 | 0484 | - | Correction on charging for IMS transit functions | F | 11.5.0 | 11.6.0 |
| Mar-2013 | SP-59 | SP-130062 | 0488 | 1 | Clarification of Type 1 IOI Usage and Multiple Sets of Inter Operator Identifiers for IMS Roaming | F | 11.6.0 | 11.7.0 |
|  |  | SP-130054 | 0491 | 1 | Related ICID Corrections for SRVCC Charging Correlation | F |  |  |
|  |  | SP-130050 | 0495 | 1 | Re-introduce Authorized-Qos AVP specified as unused | A |  |  |
|  |  | SP-130054 | 0496 | - | Correction of LCS-Client-Type AVP definition | F |  |  |
|  |  | SP-130053 | 0485 | 1 | Correction on Trigger-Type AVP | F | 11.7.0 | 12.0.0 |
|  |  |  | 0489 | 1 | Correction of Minimum Service-Context-Id AVP Content | F |  |  |
| Jun-2013 | SP-60 | SP-130270 | 0502 | 1 | Addition of IMS Visited Network Identifier | A | 12.0.0 | 12.1.0 |
|  |  | SP-130270 | 0506 | - | Correction on AVP definitions | A |  |  |
|  |  | SP-130271 | 0517 | 1 | Adjustment on IMEI - alignment to TS 29.274 | F |  |  |
|  |  | SP-130303 | 0528 | - | Correction of User-Equipment-Info-Value : encoding | A |  |  |
|  |  | SP-130270 | 0530 | 1 | Introduction of Charging for access to Trusted WLAN Access Network in EPC - over S2a | A |  |  |
|  |  | SP-130271 | 0531 | 1 | Add Reason Header AVP | B |  |  |
| Sep-2013 | SP-61 | SP-130443 | 0532 | - | Missing value for ATCF in Node-Functionality AVP | F | 12.1.0 | 12.2.0 |
|  |  | SP-130435 | 0533 | 1 | Correction on on data accounting | F |  |  |
|  |  | SP-130443 | 0537 | - | Missing value for ATCF in Node-Functionality AVP | A |  |  |
|  |  | SP-130435 | 0539 | - | Additional Access Network Information Field | B |  |  |
|  |  | SP-130435 | 0540 | - | Align RAR/RAA description with other messages description and Correction on proxy-info AVP in RAA | F |  |  |
| Dec 2013 | SP-62 | SP-130677 | 0550 | 2 | Correction for use of Destination-Host AVP in ACR | A | 12.2.0 | 12.3.0 |
|  |  | SP-130620 | 0551 | 1 | AVP enhancement for application based charging | B |  |  |
|  |  | SP-130619 | 0552 | 2 | Addition of Instance Id for IMS Charging | B |  |  |
|  |  | SP-130618 | 0554 | - | Correction for User Location Info Time | A |  |  |
|  |  | SP-130671 | 0559 | - | Correction on inconsistencies for MMTel Charging | A |  |  |
|  |  | SP-130620 | 0560 | 1 | AVP definition for application based charging | B |  |  |
|  |  | SP-130627 | 0565 | 1 | Correction for Route Header for IMS Interconnection Charging | A |  |  |
|  |  | SP-130618 | 0567 | - | Correction on Application-Server-Information AVP | A |  |  |
| Mar-2014 | SP-63 | SP-140154 | 0569 | 2 | Addition of Fixed User Subscription Identifier | B | 12.3.0 | 12.4.0 |
|  |  | SP-140033 | 0575 | 1 | Correction of data type for Time-Usage and applicability of Service-Specific-Info and AF-Charging-Identifier | A |  |  |
|  |  | SP-140045 | 0576 | 1 | Introduction of information for SMS offline Charging | B |  |  |
|  |  | SP-140033 | 0581 | - | Correction for S-GW change in Change-Condition AVP | A |  |  |
| Jun-2014 | SP-64 | SP-140336 | 0582 | 1 | Remove clause 6.3 Editor's Notes | F | 12.4.0 | 12.5.0 |
|  |  | SP-140337 | 0583 | 1 | To add AVP definitions and clarifications for application based charging to align with TS 32.251 | B |  |  |
|  |  | SP-140341 | 0584 | - | Introduce Access Line Identifier in PS-Information for Convergent Fixed-Mobile scenario | B |  |  |
|  |  | SP-140336 | 0585 | 1 | Introduce Core Network Operator selection origin for Shared Networks | B |  |  |
|  |  | SP-140342 | 0586 | 2 | Introduction of charging information for Voice Call Service (VCS) | B |  |  |
|  |  | SP-140339 | 0587 | 1 | Introduction of charging information for CHIPS - align with TS 32.251 | B |  |  |
|  |  | SP-140335 | 0589 | - | Renaming of TWAN-SSID and TWAN-BSSID AVPs | A |  |  |
|  |  | SP-140346 | 0591 | 1 | Correction for TADS indication AVP | F |  |  |
|  |  | SP-140343 | 0595 | 1 | Correction to support multiple Transit IOI Lists in IMS Charging | A |  |  |
| Jul-2014 | - | - | - | - | Rapporteur/MCC: General editorial changes and clean-up. | - | 12.5.0 | 12.5.1 |
| Sep-2014 | SP-65 | SP-140565 | 0596 | 1 | Introduction of Presence Reporting Areas for Charging | B | 12.5.1 | 12.6.0 |
|  |  | SP-140561 | 0601 | 1 | Removal of CDIVN service | A |  |  |
|  |  | SP-140564 | 0602 | 1 | Corrections for alignment between charging specifications | F |  |  |
|  |  | SP-140563 | 0603 | 1 | Complete ePDG offline charging description | B |  |  |
|  |  | SP-140567 | 0604 | - | Corrections to include missing fixed user location information for NSWO | F |  |  |
|  |  | SP-140569 | 0608 | 1 | Correction to define missing AVP for MBMS offline charging | A |  |  |
|  |  | SP-140563 | 0609 | 2 | Removal of I-WLAN solution | A |  |  |
| Dec-2014 | SP-66 | SP-140802 | 0614 | - | Correction on User CSG Information in containers description for EPC offline Charging | A | 12.6.0 | 12.7.0 |
|  |  | SP-140804 | 0615 | 1 | Correction on Inter Node Change in SGW and ePDG offline charging | F |  |  |
|  |  | SP-140807 | 0616 | 1 | Clarification of serving node for TDF | F |  |  |
|  |  | SP-140803 | 0618 | - | Correction to Status-Code AVP | A |  |  |
| Mar 2015 | SP-67 | SP-150070 | 0625 | 1 | Correction on IETF references | A | 12.7.0 | 12.8.0 |
|  |  | SP-150066 | 0627 | - | Correction for charging based on MBMS Data Transfer Time | A |  |  |
|  |  | SP-150065 | 0630 | 1 | Inconsistency correction of subscriber role | A |  |  |
|  |  | SP-150069 | 0631 | 1 | Introducing new AVPs for ProSe charging | B |  |  |
|  |  | SP-150062 | 0620 | - | Introduction of reason header in IMS online charging | B | 12.8.0 | 13.0.0 |
| Jun 2015 | SP-68 | SP-150326 | 0632 | 1 | Correction on 3GPP specific Diameter AVP definitions | D | 13.0.0 | 13.1.0 |
|  |  | SP-150328 | 0634 | 1 | Correction on charging information for Voice Call Service (VCS) | A |  |  |
|  |  | SP-150332 | 0636 | - | Correction on Service-Information AVP for ProSe Charging | A |  |  |
|  |  |  | 0638 | 1 | Alignment of Direct Communications CDR with PC3ch protocol | A |  |  |
|  |  | SP-150325 | 0640 | - | Clarify Cause-code for IMS successful transaction | A |  |  |
|  |  | SP-150331 | 0642 | 1 | Correction TDF as Node Functionality | A |  |  |
|  |  | SP-150318 | 0643 | 1 | Introduction of multiple Release causes in EPC Charging | B |  |  |
|  |  | SP-150321 | 0649 | - | Correction for dynamic address flags associated to PDN connection | A |  |  |
|  |  | SP-150323 | 0653 | - | Correction of RAT Type - alignment with TS 29.061 | A |  |  |
|  |  | SP-150326 | 0655 | 1 | Correction for ProSe Charging | D |  |  |
|  |  | SP-150326 | 0656 | 1 | Support for M2M-Information in the Service-Information AVP | B |  |  |
|  |  | SP-150321 | 0657 | - | Harmonisation of clauses for vendor specific AVP | D |  |  |
| Sep 2015 | SP-69 | SP-150425 | 0658 | - | Specification of code point for M2M-Information AVP | F | 13.1.0 | 13.2.0 |
|  |  | SP-150428 | 0659 | 1 | Parameter details on enhancements for IMS Service Continuity | B |  |  |
|  |  | SP-150426 | 0661 | 1 | Correction on AVP for concatenated short messages in SMS Offline Charging | A |  |  |
|  |  | SP-150423 | 0665 | 1 | Correction on Subscriber-Role AVP specified as unused | A |  |  |
|  |  | SP-150422 | 0668 | - | Correction on reference of User-Location-Info-Time AVP | A |  |  |
|  |  | SP-150424 | 0670 | 1 | Correction on AVP definitions | F |  |  |
|  |  | SP-150425 | 0671 | - | Correction on Radio-Resources-Indicator AVP | F |  |  |
|  |  | SP-150427 | 0673 | - | Correction of ABNF for ISUP-Cause AVP | A |  |  |
|  |  | SP-150422 | 0676 | 1 | Update of Reference RFC7315 | A |  |  |
|  |  | SP-150424 | 0678 | 1 | Correction on mapping between Information Elements and AVPs | A |  |  |
|  |  | SP-150425 | 0679 | 1 | Re-use ISUP-Cause AVP for IMS-PSTN Interworking | B |  |  |
| Dec 2015 | SP-70 | SP-150696 | 0682 | - | Correction for Access-Network-Information AVP due to update to RFC 7315 | A | 13.2.0 | 13.3.0 |
|  |  | SP-150704 | 0685 | - | Correction on interoperator identifier for IMS Roaming | A |  |  |
|  |  | SP-150695 | 0691 | 1 | Correction for when User leaves a CSG | A |  |  |
|  |  | SP-150703 | 0692 | 1 | Introduction of TWAG offline charging – charging information | B |  |  |
|  |  | SP-150705 | 0698 | - | Correction on Tariff Information | A |  |  |
|  |  | SP-150701 | 0699 | - | Introduction of ULI TZ Changes in IMS offline and online charging | B |  |  |
|  |  | SP-150816 | 0700 | 1 | Definition of AVPs for Announcement service | B |  |  |
|  |  | SP-150698 | 0702 | 1 | Charging support for Terminating Identification Presentation feature changes in terminating identity | B |  |  |
|  |  | SP-150706 | 0703 | 1 | Introduction of NBIFOM Support and NBIFOM Mode information for PS charging | B |  |  |
|  |  | SP-150693 | 0704 | 1 | Clarification of charging entity location limitation | B |  |  |
|  |  | SP-150698 | 0707 | 1 | Correction on CS Location Information in SIP AS CDR | B |  |  |
|  |  |  | 0711 | 1 | Cell information received with untrusted WLAN access information | B |  |  |
| Mar 2016 | SP-71 | SP-160040 | 0712 | 1 | Charging Id assignment for NBIFOM | B | 13.3.0 | 13.4.0 |
| Mar 2016 | SP-71 | SP-160039 | 0713 | - | Correction for Variable-Part-Type AVP values | F | 13.3.0 | 13.4.0 |
| Mar 2016 | SP-71 | SP-160034 | 0714 | - | Correction for UE identification associated with inter-UE transfer | F | 13.3.0 | 13.4.0 |
| Mar 2016 | SP-71 | SP-160040 | 0715 | 1 | Correction for Access Availability Change Reason | F | 13.3.0 | 13.4.0 |
| Mar 2016 | SP-71 | SP-160037 | 0716 | 1 | ULI for untrusted wireless access network correction | F | 13.3.0 | 13.4.0 |
| Mar 2016 | SP-71 | SP-160035 | 0718 | 1 | Introducing new AVPs for MONTE charging | B | 13.3.0 | 13.4.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-06 | SA#72 | SP-160416 | 0719 | 1 | F | Correction of cell information received with untrusted WLAN access information – alignment with TS 24.229 | 13.5.0 |
| 2016-06 | SA#72 | SP-160416 | 0720 | - | F | Correction to align AVP code for UWAN-User-Location-Info in ABNF with table | 13.5.0 |
| 2016-06 | SA#72 | SP-160412 | 0721 | 1 | F | Correction for the AVP codes for MONTE charging | 13.5.0 |
| 2016-06 | SA#72 | SP-160420 | 0724 | - | B | Completion of access change of service data flow for NBIFOM | 13.5.0 |
| 2016-06 | SA#72 | SP-160420 | 0725 | 1 | B | Completion of change of charging condition for NBIFOM | 13.5.0 |
| 2016-06 | SA#72 | SP-160411 | 0726 | 3 | B | Introduce CP data transfer Rf offline charging | 13.5.0 |
| 2016-06 | SA#72 | SP-160411 | 0734 | 1 | B | Introduce non-IP PDN and CP CIoT opt in Rf offline charging | 13.5.0 |
| 2016-06 | SA#72 | SP-160417 | 0732 | 1 | F | Correction on the AoC-Information AVP and Terminal-Information AVP | 14.0.0 |
| 2016-09 | SA#73 | SP-160623 | 0736 | 1 | B | Complement of Charging per IP-CAN Session | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0739 | 1 | F | Introduction of allocated AVPs code ranges - alignement with 29.230 | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0740 | 1 | A | Correction of trigger conditions description for NIDD submission | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0741 | 1 | A | Correction on AVP Code for Serving PLMN Rate Control - alignement with 29.230 | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0742 | 1 | A | Correction on APN Rate Control - Alignment with TS 23.401 | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0745 | - | A | Correction on Control Plane CIoT EPS Optimisation Indicator in PGW - alignement with 23.401 | 14.1.0 |
| 2016-09 | SA#73 | SP-160621 | 0746 | - | A | Correction on "MO exception data" RRC establishment cause in offline charging – alignement with TS 23.401 | 14.1.0 |
| 2016-12 | SA#74 | SP-160845 | 0752 | 1 | A | Correction on Requested Party Address for Emergency IMS session | 14.2.0 |
| 2016-12 | SA#74 | SP-160851 | 0754 | - | A | Correction on duplicate values for Change-Condition | 14.2.0 |
| 2016-12 | SA#74 | SP-160847 | 0755 | 1 | F | Correction on Access Network Info change with the time in offline | 14.2.0 |
| 2016-12 | SA#74 | SP-160847 | 0756 | 1 | F | Correction on Role-Of-node for AS serving the forwarding party | 14.2.0 |
| 2016-12 | SA#74 | SP-160847 | 0757 | 1 | F | Correction of Number-Portability-Routing-Information | 14.2.0 |
| 2016-12 | SA#74 | SP-160847 | 0758 | 1 | F | Correction of Called-Party-Address and Requested-Party-Address | 14.2.0 |
| 2016-12 | SA#74 | SP-160847 | 0759 | 1 | F | Correction of Change-Time to include Access Transfer Time | 14.2.0 |
| 2017-03 | SA#75 | SP-170144 | 0760 | 1 | B | Charging enhancement for 3GPP PS Data off | 14.3.0 |
| 2017-03 | SA#75 | SP-170133 | 0761 | 1 | B | Addition of the fields for ProSe Charging | 14.3.0 |
| 2017-03 | SA#75 | SP-170129 | 0762 | 1 | B | Addition of multiple PRAs support for AULC | 14.3.0 |
| 2017-03 | SA#75 | SP-170137 | 0764 | 1 | A | Correction on the APN Rate Control and SCS/AS Address AVP | 14.3.0 |
| 2017-03 | SA#75 | SP-170136 | 0770 | 1 | A | Correction removal of Redirect-Address-Type from CCR | 14.3.0 |
| 2017-03 | SA#75 | SP-170138 | 0771 | 1 | B | Replace reference to RFC 3588 by RFC 6733 | 14.3.0 |
| 2017-03 | SA#75 | SP-170138 | 0772 | 1 | B | Introduce clause for Diameter transport security | 14.3.0 |
| 2017-03 | SA#75 | SP-170138 | 0773 | 1 | B | New Command CCF in RFC 6733 | 14.3.0 |
| 2017-03 | SA#75 | SP-170138 | 0774 | - | B | Remove "encr" and 'P' flag description in AVPs tables | 14.3.0 |
| 2017-03 | SA#75 | SP-170134 | 0775 | - | B | Adding Reporting-Reason for Unused Quota timer | 14.3.0 |
| 2017-06 | SA#76 | SP-170498 | 0776 | 1 | B | Implement IMS visited network identifier for S8HR | 14.4.0 |
| 2017-06 | SA#76 | SP-170497 | 0777 | 1 | F | Correction on the description of AVPs | 14.4.0 |
| 2017-06 | SA#76 | SP-170495 | 0780 | 1 | B | Adding of Unused-Quota-Timer to CCA | 14.4.0 |
| 2017-06 | SA#76 | SP-170503 | 0782 | - | A | Correction of ISUP Cause naming | 14.4.0 |
| 2017-06 | SA#76 | SP-170500 | 0783 | - | B | Introduce T4 Device Trigger charging | 14.4.0 |
| 2017-06 | SA#76 | SP-170500 | 0784 | - | B | Introduce MSISDN-less SMS MO via T4 charging | 14.4.0 |
| 2017-06 | SA#76 | SP-170514 | 0785 | 1 | F | Correction of IMEI and IMEISV definition | 14.4.0 |
| 2017-06 | SA#76 | SP-170497 | 0786 | 1 | B | Addition of the AVPs for ProSe Discovery and Communication | 14.4.0 |
| 2017-06 | SA#76 | SP-170506 | 0787 | 1 | B | Introduction of FE Identifier List AVP | 14.4.0 |
| 2017-09 | SA#77 | SP-170653 | 0788 | 1 | F | Clarification on Envelope reporting | 14.5.0 |
| 2018-01 | SA#78 | SP-171005 | 0791 | - | A | Correction where RAN-NAS-Release-Cause is defined as a squence | 14.6.0 |