3GPP TS 32.511 V10.1.0 (2018-01)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Automatic Neighbour Relation (ANR) management;

Concepts and requirements

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

ANR, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	6

4	Concepts and background	6

5	Requirements	6

5.1	Business level requirements	6

5.1.1	Void	6

5.2	Specification level requirements	7

5.2.1	Void	7

5.2.2	Void	7

5.2.3	Void	7

5.2.4	Use cases	7

5.2.4.1	Management of fully automatic ANR function	7

5.2.4.2	Manual start of the ANR function by operator	8

5.2.4.3	Handling of noX2 attribute	9

5.2.5	Requirements	10

5.2.5.1	ANR function management	10

Annex A (informative):	Change history	12

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

32.511:	"Automatic Neighbour Relation (ANR) management; Concepts and requirements".

# 1	Scope

The present document describes the requirements and architecture for the management of Neighbour Cell Relations (NCRs) across the Itf-N.  The NCR management is a key feature of Self Organization Network (SON) [4].

The NCR concept and background information are described in clause 4. 
The requirements for management of NCR are defined in clause 5.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TR 32.816: "Telecommunication management; Study on Management of Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Evolved Packet Core (EPC)".

[5]	3GPP TR 32.501 "Telecommunication management; Self-Configuration of Network Elements; Concepts and  Integration Reference Point (IRP) Requirements".

[6]	3GPP TS 36.300 “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

[7]	3GPP TS 32.301 “Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Requirements”

# 3	Definitions and abbreviations

For the purposes of the present document, the terms and definitions given in TS 32.101 [2], TS 32.102 [3] and TS 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TS 32.101 [2], TS 32.102 [3] and TS 21.905 [1], in that order.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ANR function:	The ANR function is described in 36.300 [6], clause 22.3.2a.

Neighbour Cell Relation:	The Neighbour Cell Relation (NCR) is defined in 36.300 [6] clause 22.3.2a

Searchlist:	List of frequencies and supporting information to be used for neighbour cell measurements. The Searchlist contains entries for E-UTRAN and supported IRATs.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NCR	Neighbour Cell Relation

eNB	eNodeB or evolved NodeB

UC	Use Case

# 4	Concepts and background

The ANR function in the eNB relates to the Use Cases Establishment of new eNB in network and Optimisation of the neighbourhood list in [4].

-	For Establishment of new eNB in network. If the operator so chooses, the OAM system adds and configures NCRs before the eNB goes into operation.

-	For Optimisation of the neighbourhood list, the ANR function deals with automatic NCR additions and removals. It minimizes the need for planning and configuring NCRs. If the operator so chooses, the OAM system adds and configures NCRs or removes NCRs after the eNB goes into operation.

# 5	Requirements

## 5.1	Business level requirements

REQ-ANR-CON-001	NCRs shall be set up and optimized with no or minimal human intervention.

REQ-ANR-CON-002	Initial status of the newly created NCR by ANR function shall be such that HO is allowed, X2 connection setup is allowed, and the NCR is allowed to be removed by ANR.

### 5.1.1	Void

## 5.2	Specification level requirements

### 5.2.1	Void

### 5.2.2	Void

### 5.2.3	Void

### 5.2.4	Use cases

#### 5.2.4.1	Management of fully automatic ANR function

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | The goal is that the IRPManager may add and remove NRs and that it may change attributes of the NCRs |  |
| Actors and Roles (*) | - - IRPManager |  |
| Telecom resources | - - ANR function - - eNB |  |
| Assumptions |  |  |
| Pre conditions | - - The ANR function is active; - - The cell may or may not have Neighbour Cell Relations configured by O&M. - - The eNB has finished Use Case Self-configuration of a new eNodeB [5]. |  |
| Begins when | This Use Case begins when all pre conditions have been met. |  |
| Step 1 (*) (M) | - - If the IRPManager finds out that an unsuitable Neighbour Cell Relation has been added by ANR, the IRPManager may “Blacklist” that particular Neighbour Cell Relation. - - If the IRPManager finds out that a desired Neighbour Cell Relation has not been added by ANR, the IRPManager may “Whitelist” that particular Neighbour Cell Relation. |  |
| Ends when (*) | This Use Case ends when the eNB is taken out of service or when the ANR function is stopped. |  |
| Exceptions |  |  |
| Post Conditions |  |  |
| Traceability (*) | REQ-ANR-CON-001 |  |

#### 5.2.4.2	Manual start of the ANR function by operator

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal (*) | The goal is that the IRPManager may add and remove NCRs and that it may change attributes of the NCRs. |  |
| Actors and Roles (*) | - - IRPManager |  |
| Telecom resources | - - ANR function - - eNB |  |
| Assumptions |  |  |
| Pre conditions | - - The ANR function is not active; - - The eNB may have Neighbour Cell Relations. The NCRs may be configured by O&M or be may have been added by ANR function if ANR function has been active previously. |  |
| Begins when | The Use Case begins when the IRP Manager starts the ANR function. |  |
| Step 1 (*) (M) |  |  |
| Step 2 (*) (M) | The IRPManager may uncheck the noRemove attribute from any present Neighbour Cell Relation. |  |
| Step 3 (*) (M) | - - If the IRPManager finds out that an unsuitable Neighbour Cell Relation has been added by ANR, the IRPManager may “Blacklist” that particular Neighbour Cell Relation. - - If the IRPManager finds out that a desired Neighbour Cell Relation has not been added by ANR, the IRPManager may “Whitelist” that particular Neighbour Cell Relation. |  |
| Ends when (*) | This Use Case ends when the eNB is taken out of service or when the ANR function is stopped. |  |
| Exceptions |  |  |
| Post Conditions |  |  |
| Traceability (*) | REQ-ANR-CON-001 |  |

#### 5.2.4.3	Handling of noX2 attribute

Use Case 1

IRPManager needs to be able to forbid and allow the establishment of X2 interfaces from the source macro eNBs to a target eNB.  IRPManager is aware that the target eNB cannot support X2 connections.  This UC on how noX2 is used relates to node level rather than cell level.

Use Case 2

IRPManager needs to be able to allow and forbid the establishment of X2 interfaces from the source HeNBs to a target macro eNB.  This UC supports the case when a potentially large number of HeNBs in the vicinity of a macro eNB, X2 establishment requests from HeNB might saturate the physical ports of the macro eNB (not in terms of bandwidth saturation but rather the saturation in terms of the number of simultaneous establishment requests supported).  This UC on how noX2 is used relates to node level rather than Cell level.

Editor’s note: The supporting cases need to be further refined

Use Case 3

IRPManager needs to be able to forbid the establishment of the X2 interface from (IRPManager's) operator's eNB to another operator's eNB or to an eNB that belongs to another unwanted PLMN.  This UC supports the case when the IP address of the target eNB cannot be obtained or the X2 handovers to another unwanted PLMN are not allowed.  This UC on how noX2 is used relates to node level rather than cell level.

Editor’s note: The first supporting case (pertaining to the IP address) needs further study

Use Case 4

IRPManager needs to be able to ask for the release of the X2 interface improperly established by eNB.  This UC supports the case when the serving eNB has established an X2 interface (e.g., by ANR) before IRPManager had a chance to forbid the establishment of that X2 interface the IRPManager needs to be able to ask the serving eNB to release the X2 interface to the target eNB.  This UC on how noX2 is used relates to node level rather than cell level.

### 5.2.5	Requirements

#### 5.2.5.1	ANR function management

The business level requirements in section 5.1 are decomposed into the following specification level requirements:

REQ-ANR-FUN-01	An IRPManager shall be able to request that HO be allowed from source cell to target cell.

REQ-ANR-FUN-02	An IRPManager shall be able to request that HO be prohibited from source cell to target cell.

REQ-ANR-FUN-03	An IRPManager shall be able to request that HO be allowed from source cell to target cell and that no other entity than an IRPManager can remove that request. This is termed as HO white-listing.

REQ-ANR-FUN-04	An IRPManager shall be able to request that HO be prohibited from source cell to target cell and that no other entity than an IRPManager can remove that request. This is termed as HO black-listing.

REQ-ANR-FUN-05	An IRPAgent shall inform the IRPManager about success or failure of IRPManager operations to allow HO, prohibit HO, HO white-list and HO black-list.

REQ_ANR-FUN-06	An IRPManager shall be able to request establishment of an X2 connection from one eNB to another eNB.

REQ-ANR-FUN-07	An IRPManager shall be able to request the release of an X2 connection between two eNB’s.

REQ-ANR-FUN-08	An IRPManager shall be able to request that X2 interface from one eNB to another eNB be established if the X2 interface is not established and that the release of X2 interface be prohibited. No other entity than an IRPManager can remove that request.  This is termed as X2 white-listing.

REQ-ANR-FUN-09	An IRPManager shall be able to request that X2 interface from one eNB to another eNB be released if the X2 interface is established and that the establishment of the X2 interface be prohibited. No other entity than an IRPManager can remove that request.  This is termed as X2 black-listing.

REQ-ANR-FUN-10	Operator shall be able to disable/enable one eNB or multiple eNB’s ANR function when needed.

REQ-ANR-FUN-11	void

REQ-ANR-FUN-12	An IRPManager shall be able to add and configure new NCRs in the eNB.

REQ-ANR-FUN-13	An IRPManager shall be able to remove NCRs in the eNB

REQ-ANR-FUN-14	An IRPAgent shall inform the IRPManager about changes to the NCR according to TS 32.301 [7].

REQ-ANR-FUN-15	A Searchlist is needed for each cell. The IRPManager should be able to configure the Searchlist.

REQ-ANR-FUN-16	An IRPAgent shall inform the IRPManager about the newly added and removed NCRs according to TS 32.301 [7].

REQ-ANR-FUN-17	An IRPManager shall be able to retrieve ANR related attribute values on cell level, identifying:

-	Source cell & target cell;

-	NCR status (locked, unlocked);

-	HO status (allowed, prohibited).

Editor’s note:

The ‘locked’ NCR status indicates that the NR shall not be removed by the ANR function.

The ‘unlocked’ NCR status indicates that the NR may be removed by the ANR function.

The ‘allowed’ HO status indicates that handovers are allowed for this NCR.

The ‘prohibited’ HO status indicates that handovers are prohibited for this NCR.

The combination of ‘locked’ NCR status and ‘allowed’ HO status is a ‘whitelisted’ relation.

The combination of ‘locked’ NCR status and ‘prohibited’ HO status is a ‘blacklisted’ relation

REQ-ANR-FUN-18	The IRPAgent shall support a capability allowing the IRPManager to determine whether the X2 interface between two eNodeBs is established or not established.

REQ-ANR-FUN-19	IRPManager shall be able to request that the source eNB be prohibited to use X2 interface for HOs to a target eNB even if the X2 interface exists between the eNBs.  No other entity than an IRPManager can remove that request.  This is termed as X2HO black-listing.

REQ-ANR-FUN-20	The IRPAgent shall support a capability allowing the IRPManager to retrieve the X2 whitelisted and blacklisted eNBs.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2008-12 |  |  |  |  |  | Submitted to SA#42 for information and approval | 8.0.0 |
| 2009-03 | SP-43 | SP-090213 | 001 | 1 |  | Remove solution related terms and inconsistencies in Requirement specification and add clarifications | 8.1.0 |
| 2009-06 | SP-44 | SP-090290 | 002 | -- |  | Clarify requirement REQ-ANR-FUN-18 | 8.2.0 |
| 2009-12 | - | - | - | - |  | Update to Rel-9 version (MCC) | 9.0.0 |
| 2011-03 | - | - | - | - |  | Update to Rel-10 version (MCC) | 10.0.0 |
| 2018-01 | SA#78 | SP-170972 | 0014 | 1 | A | Alignment with traffic specification | 10.1.0 |