# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183211

# 14-18 May 2018, La Jolla (US)

# Source:	China Mobile, Huawei

# Title:	pCR 28.552 Add RRC connection number measurements

# Document for:	Approval

Agenda Item:	6.4.5

1	Decision/action requested

It is asked to discuss and approve this contribution.

2	References

[1]	3GPP TS 32.425 Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network v15.0.0

[2]	3GPP TS 28.552 Management and orchestration of networks and network slicing; NR and NG-RAN performance measurements and assurance v0.2.0

3	Rationale

RRC connection number measurements can be used to evaluate the radio network load, which was defined in E-UTRAN [1]. It is also necessary to define these measurements in TS 28.552[2].

4	Detailed proposal

It is proposed to add a use case “monitoring RRC connection number” in Annex A.

It is proposed to add RRC connection number measurements for gNB-CU.

1st Modified Section

5	Performance measurements and assurance data for gNB

5.1	Performance measurements and assurance data for gNB-CU

5.1.x	RRC connection number

5.1.x.y	Mean number of RRC Connections

- This measurement provides the mean number of RRC Connections during each granularity period.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of RRC connections for each NR cell and then taking the arithmetic mean.

- A single integer value.

- RRC.ConnMean

- GNBCUFunction. NRCellCU (ENGNBCUFunction.NRCellCU for NR option 3)

- Valid for packet switched traffic

- NR and NG-RAN

5.1.x.z	Max number of RRC Connections

- This measurement provides the maximum number of RRC Connections during each granularity period.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of RRC connections for each NR cell and then taking the maximum.

- A single integer value.

- RRC.ConnMax

- GNBCUFunction. NRCellCU (ENGNBCUFunction.NRCellCU for NR option 3)

- Valid for packet switched traffic

- NR and NG-RAN

2nd Modified Section

Annex A (informative):
Use cases for performance measurements and assurance data

A.x Use case of monitoring RRC connection number

The number of the RRC connections need to be monitored as it reflects the load of the radio network, the operators can use this information for dynamic frequency resource allocation or load balance purpose. Moreover, it is an important factor to be evaluated in the radio network capacity enhancement decision-making.

End of Modified Section