# 3GPP TSG SA WG5 (Telecom Management) Meeting #120	S5-185132

# 20 - 24 August 2018, Belgrade, Serbia	Rev. of S5-185132

# Source:	ETRI, KT

# Title:	Add attributes for network slice capacity planning

# Document for:	Approval

Agenda Item:	6.4.3

1	Decision/action requested

The group is asked to discuss and approve the contribution.

2	References

[a]	TS 28.541 v1.2.1: “Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and 3 (Release 15)”.

3	Rationale

This pCR proposes to add attributes in NRM for network slice capacity planning.

4	Detailed proposal

It is proposed to make the following changes to TR 28.541.

1st modified section

6.3.3	ServiceProfile

6.3.3.1	Definition

This <<DataType>> represents the properties of network slice related requirement should be supported by the network slice instance in 5G network.

6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| nSSAI | M | T | T | F | T |
| pLMNId | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| netCapacityPlanningReq | O | T | T | F | T |

6.3.3.3	Attribute constraints

None.

6.3.3.4	Notifications

Not applicable.

6.3.4	SliceProfile

6.3.4.1	Definition

This <<DataType>> represents the properties of network slice subnet related requirement should be supported by the network slice subnet instance in 5G network.

6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| nSSAI | M | T | T | F | T |
| pLMNId | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| subnetCapacityPlanningReq | O | T | T | F | T |

6.3.4.3	Attribute constraints

None.

6.3.4.4	Notifications

Not applicable

2nd modified section

6.4	Attribute definition

6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| netCapacityPlanningReq | This parameter specifies the requirements of network level resource capacity planning requirements. It is a structure containing the following elements: - list of netSliceCapPlanRequirements allowedValues: - list of netCapacityPlanningReq is a list of entries where an entry identifies the network level slice capacity planning requirements. An entry has the following attributes: targetNSSIForCapPlan(list), typeOfNSSI(integer), maxNumOfNSSIAllowed (integer), netTopology(list),linkCapacity, reservedLinkCapacity, usedLinkCapacity, optObjectiveType(integer), optObjectives(integer), optAlgorithmId(string) | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| subnetCapacityPlanningReq | This parameter specifies the requirements of subnetwork level resource capacity planning requirements. It is a structure containing the following elements: - list of subnetSliceCapPlanRequirements allowedValues: - list of subnetCapacityPlanningReq is a list of entries where an entry identifies the subnetwork level slice capacity planning requirements. An entry has the following attributes: subnetTopology(list), linkCapacity, reservedLinkCapacity, usedLinkCapacity, optObjectiveType(integer), optObjectives(integer), optAlgorithmId(string) | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

End of modified sections