3GPP TSG-SA5 Meeting #122	S5-187048

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0006 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of CQI measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/11/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For CQI calculation, the precoding matrix W(i) is different when it is in PMI mode (the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI') or non-PMI mode (the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI'). So in different mode, the result of CQI is also different. These different modes are suitable for different scenarios. The non-PMI mode can use wide band or narrow band beam corresponding to one port, or multiple port. One the other hand, the calculation of CQI values also depends rank. So the definitions of CQI measurements based on modes and ranks are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of wideband CQI distribution in non-PMI mode and average CQI in PMI mode are respectively added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DL quality measurement of RF signal is missing, the DL service quality assessment can’t be fetched. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X CQI Measurement

5.1.1.X.1	Wideband CQI distribution in non-PMI mode on one port

- This measurement provides the distribution of the wideband CQI (Channel Quality Indicator) reported by UEs in the cell which is not configured in PMI mode and CSI-RS is mapped in one port.

- CC.

- This measurement is obtained by incrementing the appropriate measurement bin when a wideband CQI value is reported by a UE in the cell which is not configured in PMI mode and CSI-RS is mapped in one port.  In case only one table is used, table 1 (TS 38.214 Table 5.2.2.1-2) will be provided first.

- A single integer value.

- WBWoPMICQIDist.TableX.BinZ

where X represents the CQI table (1 to 3),  Z represents the CQI value (0 to 15).

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.2  Average CQI in non- PMI mode at multiple port

- This measurement provides the average of the CQI (Channel Quality Indicator) reported by UEs in the cell which is not configured in PMI mode and CSI-RS is mapped in multiple ports.

- CC.

- This measurement is obtained by incrementing the appropriate measurement when a CQI value is reported by a UE in the cell which is not configured in PMI mode, CSI-RS is mapped in multiple ports.  CRI-RS are used for CQI measurement, separate counters are maintained for each rank and CQI table. In case only one table is used table 1 (TS 38.214 Table 5.2.2.1-2) will be provided first.  Average CQI per Table per Rank = WBWoPMITotalCQI.TalbeX.RankY/ WBWoPMINbrCQI.TalbeX.RankY.

- A single integer value.

- The measurement name has the form WBWoPMITotalCQI.TalbeX.RankY which  grand total  CQI values

WBWoPMINbrCQI.TalbeX.RankY which grand the number of CQI per rank.

where X represents the CQI table (1 to 3), Y represents the rank value (1 to 8).

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.3	Average CQI in PMI mode

- This measurement provides the average of the CQI (Channel Quality Indicator) reported by UEs in the cell which is configured in PMI mode.

- CC.

- This measurement is obtained by incrementing the appropriate measurement when a CQI value is reported by a UE in the cell which is configured in PMI mode.  In case multiple ports CRI-RS are used for CQI measurement, separate counters are maintained for each rank and CQI table .In case only one port, rank1 will be provided first. In case only one table is used table 1 (TS 38.214 Table 5.2.2.1-2) will be provided first.  Average CQI per Table per Rank = WithPMITotalCQI.TalbeX.RankY/ WithPMINbrCQI.TalbeX.RankY.

- A single integer value.

- The measurement name has the form WithPMITotalCQI.TalbeX.RankY which  grand total  CQI values

WithPMINbrCQI.TalbeX.RankY which grand the number of CQI per rank.

where X represents the CQI table (1 to 3), Y represents the rank value (1 to 8).

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x	 Monitoring of RF performance

Monitoring of the quality of RF signal in the cell is useful for the purpose of network planning and network coverage quality assessment. Measurements of Channel Quality Indicator (CQI) reported by UEs is a useful metric reflecting RF signal quality.

End of modifications