3GPP TSG-SA5 Meeting #122	S5-187104

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0684 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of 5GS for SMS charging via Ro Rf |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2018-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5GS specific parameters need to be considered for SMS over NAS in offline and online charging from SMS-SC, and IP-SM-GW: 1) The description for the following parameters is still valid for SMS-SC and IP-SM-GW (extracted from 5GS parameters by SMSF): - IMSI (contained in the SUPI) - MSISDN (contained in the GPSI) - IMEI/IMEISV (contained in PEI) 2) The user location information and RAT type for 5GS are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) No change in current specification 2) Refer to 3GPP User location and 3GPP RAT Type specified in TS 29.061 (applicability of 5GS values per CDRs and specified in TS 32.274). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to apply accurate charging for SMS delivered over NAS in 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.6.13, 5.1.4.6.48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.4.6.13	RAT Type

This field contains the Radio Access Technology (RAT) type used for the SMS transaction, as provided to the SMS Node, and specified in TS 29.061 [216] 3GPP RAT Type.

Next change

##### 5.1.4.6.48	User Location Info

This field contains the information about the location of the subscriber during the SMS transaction, as provided to the SMS Node, and specified in TS 29.061 [216] 3GPP user location.

End of changes