3GPP TSG-SA5 Meeting #123	S5-191103

# Montreal,Canada, 21-25 January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0039 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add description of charging information for offline charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Nchf interface for 5G data network offline charging service is not specified in TS 32.255. The charging information exchange between SMF and CHF, the charging informaiton for offline charging is compliant with that for converged charging. Some charging parameters and triggers might be identified for offline charging service. This contribution is to separate the charging information for offline charging information for 5G data connectivity. In addtion, the mixing of charging information for offline and offline charging in TS 32.255 may result in misuses when CTF generates charging events toward to CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separation the charging information of offline charging is given in this contribution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 6.1.1.3, 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.255

6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the SMF as used for 5G data connectivity converged charging.

Table 6.1.1.2.1: Charging Data Request message contents for converged charging

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | M | Described in TS 32.290 [57] |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | OC | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Notify URI | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Container Information | OC | This field holds the 5G data connectivity PDU session container specific information described in clause 6.2. |
| UPF ID | OC | This field holds the UPF identifier used to identify the UPF when quota granted per UPF by CHF for Requested unit and reporting the usage of the UPF for Used unit. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |

Table 6.1.1.2.2: Charging Data Request message contents for offline charging

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | M | Described in TS 32.290 [57] |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | OC | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Notify URI | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Rating Group | M | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Container Information | OC | This field holds the 5G data connectivity PDU session container specific information described in clause 6.2. |
| UPF ID | OC | This field holds the UPF identifier used to identify the UPF when quota granted per UPF by CHF for Requested unit and reporting the usage of the UPF for Used unit. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |

6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G data connectivity converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | M | Described in TS 32.290 [57] |
| Result code | M | Described in TS 32.290 [57] |
| Failed parameter | OC | Described in TS 32.290 [57] |
| Failure Handling | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | Described in TS 32.290 [57] |
| Multiple Unit information | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | M | Described in TS 32.290 [57] |
| UPF ID | OC | This field holds the UPF identifier used for quota granted per UPF by CHF |
| Granted Unit | OC | Described in TS 32.290 [57] (NOTE 1) |
| Validity Time | OC | Described in TS 32.290 [57] (NOTE 1) |
| Final Unit Indication | OC | Described in TS 32.290 [57] (NOTE 1) |
| Time Quota Threshold | OC | Described in TS 32.290 [57] (NOTE 1) |
| Volume Quota Threshold | OC | Described in TS 32.290 [57] (NOTE 1) |
| Unit Quota Threshold | OC | Described in TS 32.290 [57] (NOTE 1) |
| Quota Holding Time | OC | Described in TS 32.290 [57] (NOTE 1) |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Quota Consumption Time | OC | Described in TS 32.290 [57] (NOTE 1) |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |
| NOTE 1: Granted Unit, Validity Time, Final Unit Indication, Time Quota Threshold, Volume Quota Threshold, Unit Quota Threshold, Quota Holding Time and Quota Consumption Time are only included in online charging. |  |  |

End