3GPP TSG-SA5 Meeting #123 	S5-191245

# Montreal,Canada, 21-25 January 2019											Revision of S5-19xxxx

# Source:	Ericsson

# Title:	Discussion paper around KPI for DRB Accessibility

# Document for:	Approval

Agenda Item:	6.4.7

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal

# 2	References

[1]	3GPP TS 28.550 “Management and orchestration; Performance assurance”

[2]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[3]	3GPP TS 28.554 “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”

[4]	S5-191248 “Add DRB Accessibility KPI and Use Case”

[5]	S5-191201 “Add measurements and UC related to setup of UE associated NG signalling connection”

[6]			S5-191202 “Add measurements and UC related to RRC connection establishment”

# 3	Rationale

# This discussion paper gives the background to the DRB Accessibility KPI proposed in S5-19xxxx [4].

In LTE (32.450) an E-RAB Accessibility KPI is defined as:

d)

This KPI multiply the RRC success rate, “core signalling” success rate, and the “ERAB establischment” success rate, into one useful KPI.

For NR, a similar KPI is wanted, but it needs to be adapted to NR. In S5-191248 [4] a “DRB Accessibility KPI” is defined as:

DRB Accessibility KPI = “UE RRC connection success rate” * “UE NGAP connection success rate” * “DRB setup success rate” * 100

For “DRB setup success rate” those are performed with “mapped 5QI” and “S-NSSAI” as subcounters

UE RRC connection success rate  =RRC.ConnEstabSucc. CauseRRC.ConnEstabAtt. Cause (measurements defined in S5-191201 [5])

UE NGAP connection success rate =UECNTX.ConnEstabSucc. CauseUECNTX.ConnEstabAtt. Cause  (measurements defined in S5-191202 [6])

DRB setup success rate, with 5QI as sub-counters = DRB.EstabSucc. 5QIDRB.EstabAtt. 5QI  (measurement in clause 5.1.1.10 TS 28.552)

DRB setup success rate, with S-NSSAI as subcounters = DRB.EstabSucc. SNSSAIDRB.EstabAtt. SNSSAI (measurement in clause 5.1.1.10 TS 28.552)

Note that the proposed DRB Accessibility KPI, not only cover the “initial DRB setup”, also additional DRB setup is included in KPI. This means that not all attempts to setup a DRB are preceeded by an RRC connection setup and NGAP connection setup, since additional DRBs can be setup while the RRC and NGAP connection is already established.

# 4	Detailed proposal

See S5-191248 [4], for a “Accessibility KPI” defined as proposed in the Rationale.